3GPP TSG RAN WG1 #122bis									R1-2507307

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	5

Source:	NEC

Title: 	Discussion on LS on when RRC layer submits periodic CSI inference configuration to lower layer

Document for:	Discussion and Decision

- Introduction

RAN2 LS R1-2506720(R2-2506519) [1] on when RRC layer submits periodic CSI inference configuration to lower layer requests RAN1 to reply which of the following option (i.e. Option 1 or Option 2) is best. In section 2, we discuss this issue.

In RAN2#131, RAN2 can’t achieve consensus on how to correctly capture the following part marked in bold in RAN1 Reply LS (R1-2410898) in RRC running CR:

<omit unrelated text> - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. <omit unrelated text> Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

In more details, RAN2 identified the following two options on when RRC layer submits periodic CSI inference configuration (i.e. CSI-ReportConfig) to lower layer (i.e. PHY layer):

- Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable.

- Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete.

From RAN2 point of view, the consequence of the two options can be described as follows:

- For Option 1, companies in RAN2 think there may be two different potential approaches in PHY layer how to handle non-applicable periodic CSI-ReportConfig. It is for RAN1 information.

  - Approach 1: The UE’s PHY layer will immediately perform inference of periodic CSI, even if the inference configuration is non-applicable. Consequently, the UE may report invalid periodic CSI before the corresponding CSI-ReportConfig becomes applicable.

  - Approach 2: The UE’s PHY layer will ignore the inference configuration of periodic CSI if it is non-applicable. Consequently, the UE will not report periodic CSI before the corresponding CSI-ReportConfig becomes applicable.

- For Option 2, as RRC layer holds on submitting the inference configuration to lower layer until reporting as applicable, the UE’s PHY layer will perform inference of periodic CSI and report only after sending RRCReconfigurationComplete with the corresponding CSI-ReportConfig setting to “applicable”.

From RAN2 point of view, this issue can be solved by option 2 but needs to check with RAN1. RAN2 also discussed option 1 and couldn’t conclude as its consequence is outside scope of RAN2. RAN2 would like to ask RAN1 which option (i.e. option 1 or option 2) is best.

- Discussion

The intention of related RAN1 agreement in [1] is that UE does not perform model inference if it reports inapplicable for the inference configuration. This intention matches the consequence of Option 2.

Moreover, if Option 1 Approach 1 is adopted, UE wastes its resources for prediction and generates useless results. If Option 1 Approach 2 is adopted, it is still useless interaction between PHY and higher layer since there is no measurement and report.

Therefore, we have the following proposal.

Proposal: Send LS reply to confirm Option 2 in R1-2506720(R2-2506519) is the best option.

- Conclusion

In this contribution we discuss the RAN2 LS R1-2506720(R2-2506519) [1] and have the following proposal.

Proposal: Send LS reply to confirm Option 2 in R1-2506720(R2-2506519) is the best option.

- References

- R1-2506720(R2-2506519), LS on when RRC layer submits periodic CSI inference configuration to lower layer