3GPP TSG RAN WG1 #122bis			R1-2507428

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:   10.5.1

Source: 	    KPN N.V., TNO

Title:	                Views on evaluation assumptions for NR ISAC

Document for:   Discussion and decision

- Introduction

This document presents our view on ISAC evaluation KPIs for 5G NR considering the impact of sensing on communication services.

- Discussion

- As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations.

  - For other waveform and reference signals, companies are to share relevant information

  - No UE impacts

The Rel.20 5G Advanced Study on ISAC [1] considers as baseline the usage of existing DL NR waveform and DL NR reference signal, and allowing for companies to report in the evaluations if they use different waveform, i.e. the  description of SID states the following:

It is unclear if “No UE impact” means only no UE participation in sensing operation, no UE specification impact, or no UE experienced performance impact. It should be understood that no UE participation in sensing operation or no UE specification impact do not require evaluations. Performance experienced by the UE however depends on various factors such as sensing and communication loads, signals utilized for sensing and communication, QoS requirements, etc.

Consequently, there may be a possible impact of sensing on communication performance, which should be made explicit in the sensing evaluations. Current RAN1 discussions include and agree upon sensing KPIs, while discussions related to possible impact on communication KPIs or defining comprehensive ISAC KPIs have not yet been addressed [2].

Observation 1: Sensing related KPIs such as e.g. position accuracy, velocity accuracy, false alarm probability, and missed probability detection are agreed as sensing performance metrics for Release 20 5G Advanced NR ISAC in RAN1. However, there are no definitions or discussions about possible impact on communication performance or about combined ISAC KPIs.

One aspect to consider is the limited study time and the necessary additional efforts (to define proper communication metrics and scenarios with co-existence of sensing and communication) as well as dimensioning the evaluation scenario (e.g., the traffic model and UE distribution for communications, and capturing the impact of mutual interference). Thus, instead of defining metrics and scenarios, we can start the discussion where companies report the observed/expected performance impact on communication services as this will help in defining the relevant metrics and scenarios in the future ISAC studies.

Proposal 1: Companies in their sensing evaluations results should provide a statement if there may or may not be an expected impact on communication services performance. Such statement may be descriptive and qualitative or, if available, include the exact performance impact, such as change in user throughput and/or latency as well as the considered evaluation parameters such as the cell load, traffic type and deployment scenario.

Further, based on the used signals for sensing performance evaluations, there may be an impact on the communication KPIs. Consider the two scenarios:

- Existing DL NR waveform and DL NR reference signals are used for sensing with a particular configuration e.g. periodicity, power settings, etc. This particular DL NR waveform and DL NR reference signal configuration might result in an impact on communication services in terms of e.g. latency or DL/UL throughput.

- Other DL signal waveform/reference  signals are used for sensing but somehow interleaved with the NR signal. This would result in reduced resources for communication and thus, there may be an impact on communication services based on the current communication load.

Observation 2: Impact on communication services depend on the type and configuration of DL waveform or reference signals used for sensing purpose.

Proposal 2: Companies in their sensing evaluation results should provide the description of DL NR waveform (or other waveform)  or DL NR reference signal (or other reference signal) that is utilized for sensing and its configuration.

- Conclusion

This contribution has presented KPN’s views on ‘ISAC Evaluations for 5G NR’. The following observations and proposals are therefore made:

Observation 1: Sensing related KPIs such as e.g. position accuracy, velocity accuracy, false alarm probability, and missed probability detection are agreed as sensing performance metrics for Release 20 5G Advanced NR ISAC in RAN1. However, there are no definitions or discussions about possible impact on communication performance or about combined ISAC KPIs.

Observation 2: Impact on communication services depend on the type and configuration of DL waveform or reference signals used for sensing purpose.

Proposal 1: Companies in their sensing evaluations results should provide a statement if there may or may not be an expected impact on communication services performance. Such statement may be descriptive and qualitative or, if available, include the exact performance impact, such as change in user throughput and/or latency as well as the considered evaluation parameters such as the cell load, traffic type and deployment scenario.

Proposal 2: Companies in their sensing evaluation results should provide the description of DL NR waveform (or other waveform)  or DL NR reference signal (or other reference signal) that is utilized for sensing and its configuration.

- References

[1] RP-252819, Revised SID: Study on Integrated Sensing And Communication (ISAC) for NR, 3GPP TSG RAN Meeting #109, Beijing, China, September 15-18, 2025

[2] R1-2506480, Summary#5 on evaluations for NR ISAC, 3GPP TSG RAN WG1 #122, Bengaluru, India, August 25-29, 2025