3GPP TSG RAN WG1 #122bis                         			   		R1-2507447

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	11.4.2

Source:	InterDigital, Inc.

Title:	Modulation, joint channel coding and modulation for 6GR air interface

Document for:	Discussion and Decision

# Introduction

RAN approved a SI on 6G Radio in RAN#108 [1]. The following objectives are included:

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - … - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - …

This contribution discusses modulation as well as joint channel coding and modulation for 6G radio.

Modulation and channel coding have been essential parts of the physical layer in previous 3GPP generations and will continue to be so in the 6G physical layer. Previous generations have been based on separate modulation/demodulation with uniform QAM and channel encoding/decoding. While this modular approach may be simple, it is theoretically suboptimal. To enhance the spectrum efficiency of the 6G system, a previously untapped source of performance gain, called shaping gain, could be taken advantage of by considering constellation shaping in 6G radio.

# Non-uniform QAM modulation schemes

According to the Shannon's capacity theorem, the maximum spectral efficiency over an AWGN channel is achieved when the input signal is Gaussian-distributed. Rectangular QAM with equiprobable symbols deviates from the ideal Gaussian input. Although easy to implement, it comes with a shaping gap, ~1.53 dB loss in AWGN, compared to the Shannon capacity.

Non-uniform QAM modulation, e.g., constellation shaping/modulation shaping, has been showing promising gains compared to uniform, e.g., rectangular and equiprobable, QAM [4]-[12]. Non-uniform QAM improves the spectrum efficiency of a communication system by shaping the signal distribution more like Gaussian, thereby closing the shaping gap to the Shannon capacity. The signal distribution can be changed either by changing the symbol locations (Geometric Constellation Shaping (GCS)) or by changing the symbol probabilities (Probabilistic Constellation Shaping (PCS)), or a combination of both.

Constellation shaping (e.g., PCS and GCS) can recover the shaping gap of uniform QAM.​ PCS reuses the symbol locations of the uniform QAM constellations, but the transmission-probabilities of constellation symbols are shaped with a Gaussian probability function. For example, a 64QAM with non-uniform symbol probability is shown in Figure 1 (b).  PCS jointly trades off transmission entropy and conditional entropy to maximize the mutual information. Uniform QAM maximizes the transmission entropy but also has a large conditional entropy. As a result, the mutual information is worse than for non-uniform QAM. The mutual information gain of PCS over uniform QAM is called the shaping gain.

GCS, on the other hand, keeps the transmission-probabilities uniform, i.e., each symbol is equally likely. Instead, the locations of constellation points in the IQ (in-phase and quadrature) plane are modified. Instead of using a rectangular grid, as in uniform QAM, constellation points are non-uniformly placed in the IQ plane to shape the geometrical constellation symbol locations to be Gaussian-like. For example, a 1D-NUC (64 QAM) is shown in Figure 1 (a).

In contrast to PCS, GCS does not require additional blocks within the transmitter architecture; it only requires enhanced mapping from bits to I/Q symbols. However, as the constellation points are not uniformly distributed, an enhanced demapping may be needed in the receiver. Especially for a non-linear MIMO receiver, GCS may be complicated to implement compared to the simple and efficient demodulator that can be used with rectangular QAM.

PCS reuses the geometry of the QAM constellations, e.g., rectangular QAM, but modifies the probabilities of the transmitted symbols. In an example, the transmission-probabilities of constellation symbols are shaped with a Gaussian probability mass function, where the constellation points that are closer to the origin are shaped to have higher probability than others.

Figure 1 Example of (left) Geometric Constellation Shaping e.g., 1D-NUC, and (right) Probabilistic Constellation Shaping (PCS).

Many academic papers have studied and shown the performance gains of PCS. In [4], uniform QAM and PCS were compared for AWGN. Significant gains are found from probabilistic shaping over uniform input, with sensitivity improvements of up to 0.43 dB for 16 QAM, 0.8 dB for 64 QAM and more than 1 dB for 256 QAM [2].

In a recent study [5], PCS was applied to MIMO. The paper indicated that it is possible to achieve more than 2 dB gains with PCS through interference shaping, breaking the AWGN limit (1.53 dB). In particular, PCS with MIMO achieved more than 2 dB gain for a low antenna correlation scenario and even higher gain (3.5 dB) for a medium antenna correlation scenario [5].

Constellation shaping has been used and implemented in several practical systems. A recent example, that also uses low-density parity check (LDPC) coding, is the widely used digital terrestrial broadcasting standard ATSC 3.0 [13]. For example, ATSC 3.0 adopts 1D-NUC for 1024 QAM and 2D-NUC for 16, 64 and 256 QAM for enhanced RL performance.

In NR, the output bits of the low-density parity check (LDPC) coding are used to generate uniform QAM symbols. Our companion contribution in 11.4.1, we propose to adopt LDPC and polar codes for data and control channel, respectively [6]. If channel (e.g., LDPC) coding will be used also for 6G radio, the combination of PCS and FEC coding can be considered.

One way to incorporate PCS in the communication system is outer PCS, as illustrated in Figure 2. When PCS operation is performed prior to the FEC encoding in the transmitter, it is called outer PCS. In this case, the information bits can be split into two parts; the first part of the bits are shaped and the second part of the bits are not shaped [7]. The bits for shaping go through a distribution matcher (DM), e.g. CCDM or ESS. The output of DM is the shaped (positive) PAM symbols. Since the symbols are shaped (PAM) prior to the FEC encoder, i.e., LDPC, we require a function (amplitude to bits) to convert PAM symbols back to bits and concatenate with non-shaped bits to be sent as input to the LDPC encoder. After the rate matcher (RM), the signed bits are extracted and added to the shaped (positive) PAM symbols to form the positive and negative PAM symbols, and those PAM symbols including I and Q branches are ready to map to the legacy NR QAM (grey mapping). On the receiver side, the inverse DM operation is performed after FEC decoding. Inverse DM operation after the FEC decoding is performed to restore the original sequence of input bits, which may be achieved with relatively low complexity.

Figure 2 An exemplary implementation of outer PCS

# Discussion

## Baseline schemes for performance evaluation

In RAN1#122, the following agreement was made.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Any enhancements for joint channel coding and modulation should be compared against NR modulation.

Observation 1: The baseline for evaluation of benefits of enhanced joint channel coding and modulation schemes can be NR modulation

## Use of PCS to meet spectral efficiency listed in SID

As indicated in the objectives, RAN1 should aim at using a common 6G Radio design, which meets mobile broadband (MBB) service requirements as a high priority. The eMBB use case in 6G is associated with requirements such as enhanced spectral efficiency, improved network capacity, and energy efficiency. Enhanced spectral efficiency is explicitly listed as objective 1c the SID [1].

Many companies have proposed to support higher order modulation, e.g., up to 4096 QAM, in high-SNR scenarios. As discussed above, PCS can provide substantial gains in spectral efficiency, especially in combination with high order modulation.

## Numerical evaluation of joint channel coding and modulation

We have evaluated the shaping gain of PCS over uniform 64-QAM, 256-QAM and 1024-QAM by means of link level (LL) simulations (please see the appendix for simulation assumptions). In our evaluation, we have used outer PCS as shown in Figure 2 without any changes to the NR LDPC encoding/decoding. The shaping decoder (distribution matching) in the receiver uses as input the LDPC output hard decisions (soft bits are not required) with little additional complexity and effort. Further details of the PCS implementation used in the simulation can be found in the Appendix.

Observation 2: PCS can be easily integrated with NR shared channel processing, reusing LDPC encoding and decoding schemes

In our LL simulation, we evaluate MCS Table 4 index 12 (R=722 SE= 4.5234), index 13 (R=822, SE= 4.8164),  index 19 (R=841 SE= 6.5703), index 20 (R=885 SE= 6.9141), index 24 (R=885 SE= 8.3301), and index 25 (R=900.5 SE= 8.7939) in TS 38.214. The channel coding rate when PCS is used needs to be slightly higher than for uniform QAM to achieve the same SE. This is because the PCS (DM) algorithm introduces a so-called matching rate kn where k denotes the input bit length for shaping and n is the output length of PAM symbols for a DM. In practice,  the channel coding rate for PCS needs to be adjusted to maintain the same SE as for uniform QAM for a fair shaping gain comparison.

Figure 3 256 QAM comparison (a) MCS Table 4 Index 19, 256 QAM, SE=6.5703,  (b) MCS Table 4 Index 20, 256 QAM, SE=6.9141

Figure 4 64 QAM comparison (a) MCS Table 4 Index 12, 64 QAM, SE=4.5234 (b) MCS Table 4 Index 13, 64 QAM, SE=4.8164

Figure 5 1024 QAM comparison (a) MCS Table 4 Index 24, 1024 QAM, SE=8.3301 (b) MCS Table 4 Index 25, 1024 QAM, SE=8.7939

Our LL simulation results in Figure 5 show that gains around 0.8 dB at BLER = 0.1 for 1024 QAM can be achieved (for the same SE). The SNR gain at BLER = 0.1 for 256 QAM in AWGN for MCS Table 4 index of 21 and 22 is around 0.8 dB as shown in Figure 3. The SNR gain at BLER = 0.1 for 64QAM in AWGN for MCS Table 4 index of 12 and 13 is around 0.7 dB as shown in Figure 4.

Observation 3: Initial simulation results for PCS show performance gains (e.g., 0.7-0.8 dB) over uniform QAM.

Another benefit of introducing constellation shaping is that it may potentially relax the EVM requirements. RAN4 has defined EVM requirement for each modulation supported. GCS and PCS are essentially new modulation types that may require new EVM requirements to be evaluated and defined. In addition to BLER analysis, we also evaluate the potential EVM (rms EVM) requirement for PCS. In this evaluation, we derive the potential EVM requirement based on required SNR for achieving 95% of the maximum throughput. To be more specific, we use EVMrms≈1SNR  to compute the EVM rms value corresponding to PCS and uniform QAM cases and estimate the EVM relaxation amount based on the EVM difference, where the SNR value is in the linear scale.

The simulation results evaluating EVM are as shown in Table 1, where the ΔEVM value is defined as ∆EVM(%)=EVMPCS(%)-EVMUNI(%), where both EVMPCS(%) and EVMUNI(%) (EVM rms in percentage) for 95% of maximum throughput. Based on our observations, PCS scheme is more robust than uniform QAM with respect to EVM by 0.5 - 0.7%. As EVM for higher order modulations are relatively small, even 0.5% additional margin may represent significant benefits for RF design.

Table 1 EVM tolerance when PCS is enabled

| Modulation Order | SE | (current) 5G NR EVM requirements for uniform QAM | Potential EVM relaxation amount (ΔEVM) |
| --- | --- | --- | --- |
| PCS 64 QAM | 4.8164 | 8% | 0.5% |
| PCS 256 QAM | 6.9141 | 3.5% | 0.7% |
| PCS 1024 QAM | 8.7939 | 2.5% (below 4.2 GHz) 2.8% (above 4.2 GHz) | 0.5% |

Observation 4: Simulation results for PCS show potential 0.5% – 0.7% EVM requirements relaxation over uniform QAM.

To meet the study objective of designing a 6G radio with enhanced spectral efficiency, RAN1 should study non-uniform QAM (PCS). Following Observation 1, the uniform QAM of NR should serve as the baseline.

Proposal 1: Study PCS as candidates for 6G joint coding and modulation with uniform QAM of NR as the baseline.

# Conclusions

Observation 1: The baseline for evaluation of benefits of enhanced joint channel coding and modulation schemes can be NR modulation

Observation 2: PCS can be easily integrated with NR shared channel processing, reusing LDPC encoding and decoding schemes

Observation 3: Initial simulation results for PCS show performance gains (e.g., 0.7-0.8 dB) over uniform QAM.

Observation 4: Simulation results for PCS show potential 0.5% – 0.8% EVM requirements relaxation over uniform QAM.

Proposal 1: Study PCS as candidates for 6G joint coding and modulation with uniform QAM of NR as the baseline.

# Appendix

## PCS implementation

There are many solutions to achieve non-uniform symbol probabilities in PCS. The function which implements the distribution is called distribution matcher (DM) [9],[10]. Two popular DM algorithms are constant composition distribution matching (CCDM) [9] and enumerative sphere shaping (ESS) [10].  CCDM is a specific technique used in PCS to generate sequences with a fixed symbol composition, ensuring the target probability distribution. In practice, CCDM can directly assign the shaped symbol probabilities based on arithmetic coding (AC), which can help design transparency and make it easier to align evaluation assumptions. Therefore, we selected CCDM for baseline evaluation in our LL simulation.

NR supports 22m-QAM, with m=2 (16 QAM),  …, m=5 (1024 QAM). Furthermore, any 22m-QAM can be defined as the product of two independent 2m-PAM constellation [5]. By this definition, any 22m-QAM can be decomposed into two independents 2m-PAM constellations, i.e. I and Q branches. Therefore, PCS can, in principle, shape two 2m-PAM/ m-ary PAM constellations (I and Q branch) independently for any 22m-QAM. Furthermore, the positive and negative sign of m-ary PAM can be assumed equally likely. Therefore, the symbol shaping can in practice be applied only to positive PAM. In this manner, the symbol shaping dimension can be decomposed from a 2D constellation to two 1D constellations. In other words, CCDM maps a binary input sequence to an m-ary PAM symbol sequence, where each symbol appears a specific number of times in the output sequence.

In our LL evaluation, we use outer PCS (as shown in Figure 2) due to its simplicity, e.g., there are no iterations between LDPC decoding and the DM algorithm. In addition, DM on the decoder side can be based on hard decisions (soft bits are not required), reducing computational complexity. For the LL simulation, the m-ary PAM symbol probability needs to be determined, i.e., transmit entropy for a m-ary PAM modulation order for CCDM. In one implementation realization, the transmission entropy and shaping PAM symbol probability based on the Maxwell-Boltzmann (MB) equation and the SNR operating point for an m-ary PAM can be determined. In practice, when SNR is lower, more shaping is required. In our LL simulation, for each MCS index we evaluate, we only choose the SNR at BLER = 0.1, for example, MCS index 12-13 for 64 QAM, the SNR operating point is around 15.1 dB and 15.9 dB for BLER = 0.1, we use this SNR operating point to determine the symbol probability and do not dynamically adjust symbol probability during the simulation.

The next step is to determine the CCDM input-output size (k,n) based on the shaping PAM symbol probability and the input size k. In our simulation, some information bits are used for shaping and the remaining bits are not shaped. More details on how to determine the shaped and non-shaped bits can be found in [7]. For instance, when the code rate is greater than (m-1)/m, some information bits are left unshaped and mapped to PAM sign bits according to [7].

In short, the input bits are split into two parts. The first part is the input bit length k for shaping and the remaining input bits (second part) are not shaped but are used for generating sign bits. After splitting the input bits, the bits of the first part with length k are passed through DM with output n (positive) PAM symbols. The output of CCDM with n (positive) PAM symbols is passed through amplitude to bits conversion and concatenated with the second part of non-shaped bits, for the input of LDPC encoder.

The next step is to determine the LDPC rate matcher output bits E. The rate matcher output bit length E is based on channel coding rate R and the input bit length of LDPC as in NR. Interestingly, the output of LDPC rate matcher bit length E is equal to the product of the DM symbol output length n and the m-ary PAM, i.e. E=m+1n. Now, the output of rate matcher bits can be split into two sequences: a first sequence of coded bits of length E1 from the systematic bits with mn bit length  and a second sequence with length E2=E-E1 from the systematic and parity bits. The second sequence with length E2 is used for sign bits for QAM mapping to QAM symbol generation. More details regarding the QAM symbol generation can be found in [7].

Simulation parameters

The LL parameters are captured in Table 2.

Table 2 Link level parameters

| Parameters | Value |
| --- | --- |
| TB/CB block size | TBS/CB =7400 (no CB segmentation) for 64 QAM, 256 QAM and 1024 QAM |
| Code rate (R) | 64QAM R=[772 822] MCS Table 4 Indices 12, 13 256 QAM R = [841 885] MCS Table 4 Indices 19, 20 1024 QAM R = [853 900.5] MCS Table 4 Indices 24, 25 |
| Coding Scheme | NR LDPC, BGN = 1 |
| Decoding Scheme | Min-Sum Decoder Max iter = 25 |
| CRC | 24 bits |
| Bit Interleaver | Interleaving scheme for LDPC coding in TS 38.212 for Uniform QAM. Bit Interleaver is disabled for PCS. |
| Target BLER | 0.1 (10%), 0.05 (5%) |
| Channel | AWGN |
| HARQ | HARQ is disabled |
| RV | 0 |
| DM algorithm | (AC-)CCDM |
| PCS related parameters | Symbol probability Ps for CCDM Ps= [0.3496 0.2976 0.2142 0.1386] for 64 QAM, R = 772 Ps=[0.3519 0.2991 0.2162 0.1328] for 64 QAM, R = 822 Ps=[0.2066 0.1942 0.1715 0.1424 0.1111 0.0815 0.0562 0.0364] for 256 QAM, R =841 Ps=[0.1676 0.1623 0.1523 0.1383 0.1217 0.1037 0.0856 0.0684] for 256 QAM, R =885 Ps=[0.1158 0.1135 0.1090 0.1026 0.0946 0.0855 0.0757 0.0657 0.0559 0.0466 0.0381 0.0305 0.0239 0.0184 0.0138 0.0102] for 1024 QAM, R =853 Ps=[0.0923 0.0913 0.0893 0.0864 0.0827 0.0782 0.0732 0.0677 0.0620 0.0561 0.0502 0.0445 0.0389 0.0337 0.0289 0.0245] for 1024 QAM, R =900.5 |

# References

RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator).

TS 38.300, “NR and NG-RAN Overall Description; Stage 2,” v 18.6.0, June 2025.

R1-2505827, “Waveform for 6GR air interface, ” RAN1#122, Bangaluru, India, Aug. 2025.

T. Fehenberger, A. Alvarado, G. Böcherer and N. Hanik, "On Probabilistic Shaping of Quadrature Amplitude Modulation for the Nonlinear Fiber Channel," in Journal of Lightwave Technology, vol. 34, no. 21, pp. 5063-5073, 1 Nov.1, 2016, doi: 10.1109/JLT.2016.2594271.

Probabilistic Shaping in MIMO: Going Beyond 1.53dB AWGN Gain With the Non-Linear Demapper https://arxiv.org/abs/2504.18459

R1-2505838, “Channel coding for 6GR air interface, ” RAN1#122, Bangaluru, India, Aug. 2025.

G. Böcherer, F. Steiner and P. Schulte, "Bandwidth Efficient and Rate-Matched Low-Density Parity-Check Coded Modulation," in IEEE Transactions on Communications, vol. 63, no. 12, pp. 4651-4665, Dec. 2015

G. Forney, R. Gallager, G. Lang, F. Longstaff and S. Qureshi, "Efficient Modulation for Band-Limited Channels," in IEEE Journal on Selected Areas in Communications, vol. 2, no. 5, pp. 632-647, September 1984

P. Schulte and G. Böcherer, "Constant Composition Distribution Matching," in IEEE Transactions on Information Theory, vol. 62, no. 1, pp. 430-434, Jan. 2016

A. Amari et al., "Introducing Enumerative Sphere Shaping for Optical Communication Systems With Short Blocklengths," in Journal of Lightwave Technology, vol. 37, no. 23, pp. 5926-5936, 1 Dec.1, 2019

J. G. Andrews, T. E. Humphreys and T. Ji, "6G Takes Shape," in IEEE BITS the Information Theory Magazine, vol. 4, no. 1, pp. 2-24, March 2024

S. Hu, H. Wang and S. Semenov, "Supporting Probabilistic Constellation Shaping in 5G-NR Evolution," in IEEE Transactions on Wireless Communications, vol. 23, no. 4, pp. 3586-3599, April 2024

L. Fay et al., "An Overview of the ATSC 3.0 Physical Layer Specification," in IEEE Transactions on Broadcasting, vol. 62, no. 1, March 2016