3GPP TSG RAN WG 1 Meeting #122-bis	         R1-2507480

Prague, Czech Republic, October 13th – 17th, 2025

Agenda Item:	11.1

Source:	Lenovo

Title:	Overview of 6GR air-interface

Document for:	Discussion

# 1	Introduction

As work on 6G minimum TPR based on ITU-R M.2160 is underway as captured in TR 38.914 [1], the 3GPP RAN#108 plenary concluded and approved the 6G Radio (6GR) SID in RP-251881, [2], which defines the study objectives towards the next-generation radio access technology and its enabling components. This contribution provides further details on high-level 6G vision, design principles, and highlights aspects related to scalable 6GR design, support of minimum spectrum allocation, coverage, MRSS, synchronization signal structure and periodicity, operation of bandwidth/band adaptation, harmonization of TN and NTN, duplexing and control channel design.

# 2	Discussion

## 2.1 Scalable 6GR design

In RAN#122 meeting the following agreement on 6GR scalability was reached:

Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any

In the same vein to the above agreement, we regard the support of different types of devices under a unified RAT as critical to 6G at Day-1. Ensuring a minimum set of common capabilities per device type has the potential to reduce device costs, to ensure a globally aligned minimal set of device types, and to simplify device type specification testing and certification procedures. The device categories may be fixed as 6G Cat. MBB, 6G Cat. IoT-Lo, etc. according to device-specific capabilities on key aspects of peak data rates, battery life, bandwidth, MIMO/antennas support etc. It is furthermore highly desirable to avoid unnecessarily sub-categorizing Day-1 device types without strong market needs or technical necessities. However, being able to allow for emerging device types and capabilities beyond Day-1 of 6G and 7-10 years from now is also imperative, and as such careful consideration must be dedicated for how different device types can coexist and be serviced by the same 6G System.

Figure 2.1-1: Set of device categories expected for 6G Day-1 services.

Considering the variety of use cases and services which are envisioned for 6G, different populations of devices with varying capabilities need to be supported in 6GR from Day-1. The various types of devices may range from simple devices that support simple features and limited capabilities (i.e., low-tier IoT) to more sophisticated devices that support more advanced features and capabilities (e.g., high throughput MBB, FWA). Figure 2.1.1 lists candidate device types based on exemplary characteristics which can be envisioned for 6G.

With reference to Figure 2.1-1, Low-tier IoT devices group successors to LTE-M /NB-IoT enabling applications such as smart meters, health monitoring, and home automation. These devices are expected to meet demanding requirements including extended coverage (up to +10 dB link budget enhancement), battery life of at least 10 years, and narrow bandwidth operation of 5 MHz. Networks may need to accommodate such IoT devices and support densities of at least 106 devices per square kilometer.

High-tier IoT devices will address more demanding applications like robotics, CCTV, and latency-sensitive use cases and represent a 5G NR Rel-17 RedCap evolution. These devices operate on wider bandwidths (e.g., 10–40 MHz) and at higher frequencies (e.g., 2–7 GHz). Device designs may incorporate half-duplex operation, reduced antenna counts (to manage costs), and coverage recovery techniques to achieve MBB-grade coverage. An exemplary list of features/characteristics between the two-tiered IoT devices envisioned is listed in Table 2.1-2.

Table 2.1-2: Comparison of Low-Tier vs High-Tier IoT Device Characteristics

| Feature | Low-Tier IoT (LPWA) | High-Tier IoT (RedCap Evolution) |
| --- | --- | --- |
| Bandwidth | 5 MHz | 10–40 MHz |
| Target Frequency Bands | Sub-1 GHz / low bands | 2–7 GHz |
| Coverage | +10 dB gain (MCL ~155 dB) | MBB-like, with coverage recovery |
| Battery Life | Minimum 10 years | Moderate |

In NR, compared to LTE, no explicit device categories (where each device category defines a combination of uplink and downlink capabilities) have been specified. However, the introduction of explicit device categories in the 6G design has the following advantages:

- A modular design for the different device categories can be supported and a certain device type may be mapped to different device categories.

- Set of mandatory and optional features can be easier defined per device category.

- The capability reporting for the different device categories can be significantly simplified.

On the other hand, preserving a pure capability reporting approach as in NR to categorize devices will be hard to scale, as lesson learned from 5G NR, given the multi-dimensional space of device capabilities in terms of communications, and beyond, i.e., AI, sensing or energy savings capabilities, that 6G devices are expected to encompass. In the light of all of the above, the main questions are therefore how to design the common features across device types, and what additional feature needs to be added or removed to differentiate across device types. Since, 6GR principle is to define a single RAT to support diverse types, the 6GR design of the common channels e.g., SS/PBCH, CORESET#0, MSI, RACH, paging etc., can be common across device type. However, the bandwidth, payload, repetition, and periodicity of some of these common channels can be adapted considering the device category (such as minimum device bandwidth, data rate, reliability, and maximum latency requirements).

Firstly, RAN1 needs to discuss and list out the commonly designed features/parameters/factors across device types. An example of such device type features/attributes can be found in Table 2.1-3.

Table 2.1-3: List of common features & dedicated features/attributes across device types

| Common Features/Attributes | Dedicated Features/Attributes |
| --- | --- |
| Radio Hardware Characteristics: - Number of transmit/receive antennas and RF chains - Supported power classes - Maximum supported MIMO layers | Advanced Functional Capabilities: - AI/M integration - Sensing |
| Radio Access and Spectrum Capabilities: - Supported duplexing modes (e.g., FDD, TDD, half-duplex) - Supported modulation orders - Carrier aggregation and spectrum aggregation capabilities | Energy efficiency and power saving mechanisms |
| Device and Processing Capabilities: - UE processing times (e.g., HARQ timing, scheduling response) - Maximum supported coverage levels (link budget / MCL) | Maximum supported UE bandwidth |
| Mobility and Performance Attributes: - Maximum supported mobility/speed - Reliability requirements (e.g., target BLER, packet error rate) - Latency requirements (e.g., user-plane and control-plane latency) | Service and Traffic related attributes: - Support for Massive Connection density - Peak achievable data rate |
| Service and Traffic related attributes: - Voice |  |

Secondly, RAN1 should discuss the possible value ranges of these attributes across different frequency ranges, bands, and operating bandwidths, while recognizing that some device types may support only a subset of them. For example, the association of duplex modes supported can be optionally left to a device type as not all duplex modes needs to be supported by all device types. In a third step, RAN1 can discuss what can be additionally added to differentiate different device types such as ones described in Table 2.1-3. In a fourth step, the assignment of specific attribute values to each device type may be deferred to discussion in the RAN plenary.

Proposal 1: Study 6G Radio design on how to enable scalable chipset design enabling scalable coverage targets and addressing a variety of service requirements spanning across MBB and IoT over different device types.

Observation 1: Critical questions towards a scalable 6G design are: how to design the common feature set across device types, and what additional feature needs to be added or removed to differentiate across device types.

Proposal 2: Study utilizing explicit device categories to identify in a scalable manner different operational feature sets associated with various device types.

Proposal 3: Evaluate towards a scalable 6G Radio design the following staggered steps:

- Identification of common feature set parameters across device types;

  - Includes common channels e.g., SS/PBCH, CORESET#0, MSI, RACH, paging considering payload restriction due to minimum bandwidth, data rate

  - Common features as listed in Table 2.1-3

- Discussion and evaluation of parameter values/value ranges for common feature set listed in Table 2.1-3 considering each device types;

- Discussion and evaluation of additional dedicated feature set parameters listed in Table 2.1-3 applicable to device types; and

- Association of specific values/value ranges to common and/or dedicated feature set parameter values/value ranges per each device category should be done later at RAN plenary level.

## 2.2 Minimum Spectrum Allocation

The following agreement was made in RAN1#122 regarding the support of minimum bandwidth for a device.

Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

The following proposal was endorsed in the RAN#109 to clarify on design optimization for the minimum spectrum allocation and smallest UE bandwidth.

Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary.

Current 5G NR SS/PBCH signal block structure of 20 PRBs over 4 symbols takes into consideration the minimum system bandwidth and MNO channel bandwidth of 5 MHz. However, latter part of 5G introduced optimization for 3 MHz MNO channel bandwidth by puncturing the PBCH signal to 12 PRBs, that reduced PBCH performance.

For 6G, RAN1 needs to further discuss the direction for the 6G system design when the MNO’s minimum spectrum allocation of 3 MHz while the minimum device bandwidth can be 3 MHz or 5 MHz. Some operator’s raised concern during RAN#109 against optimizing the system design for the 3 MHz carrier bandwidth, while they are fine to support the 3 MHz carrier bandwidth without optimization as there are fewer 3 MHz carriers compared to 5 MHz carriers.

6GR could improve the PBCH performance for the 3 MHz carrier bandwidth by a native design of 3MHz for PBCH rather than compared with the 5G NR design of puncturing PBCH. However, a pragmatic approach, grounded in the practical deployments and much wider globally availability of 5 MHz allocations over the 3 MHz allocations is to optimize the SS/PBCH for the 5 MHz carrier bandwidth. Moreover, such an approach aligns already with the 5 MHz 5G NR design and offers potential benefits to MRSS as well, whereas it does not preclude support of 3 MHz SS/PBCH in 6GR, should practical needs arise based on limited available allocations for some geos/MNOs. In case of the latter, evaluation and consideration of 6GR specific performance/coverage enhancements for 3 MHz SS/PBCH over the 5G NR existing solution are not precluded and may be considered on a need basis.

Observation 2: Globally, practical deployments and availability of 5 MHz allocations is much larger than the one of 3 MHz allocations.

Proposal 4: Study optimizations of the system design for the 5MHz carrier bandwidth in consideration of the following benefits:

- Pragmatic approach aligned with larger 5 MHz vs 3 MHz global avaliability and deployments

Proposal 5: Evaluate based on existing deployments and global availability of 3 MHz carrier whether there is practical and commercial need for further optimization to enhance performance / coverage for 3 MHz carrier.

Given physical propagation laws and the global spectrum availability, we regard existing FR1 and low bands (i.e., sub-1 GHz) as critical for delivering good quality coverage and connectivity for a wide range of devices and services (e.g., IoT, LPWA, MBB, voice), and operational efficiency (i.e., both in terms of energy and spectral efficiency) with low TCO. The emerging FR3 (i.e., the range between FR1 and FR2-1, including around 7 GHz), is regarded as a promising opportunity for spectrum with wider bands available for global coverage. However, FR3 remains in some geos uncertain given existing incumbents and spectrum fragmentation that prevent easy access and utilization of these spectral resources across the world. Nonetheless, for 6G FR3 still remains a powerful candidate spectrum (in particular around 7 GHz) to increase capacity and still preserve satisfactory coverage comparable to FR1 at the cost of focused radio infrastructure investment (i.e., XL-MIMO scale out). Lastly, we regard FR2 as a potential candidate for specific deployments where catering for high capacity and user density is of interest, such as residential FWA.

Proposal 6: The 6G Radio minimum/maximum transmission bandwidth per frequency range that we regard feasible and realizable given spectrum challenges is outlined below in Table 2.2-1.

Table 2.2-1: Overview of feasible carrier bandwidth configurations per frequency range

| Frequency Range | Minimum MNO CBW (MHz) | Minimum Device BW (MHz) | Maximum CBW (MHz) |
| --- | --- | --- | --- |
| 600 MHz – 2 GHz | 3, 5 | 5 | 50 |
| 2 GHz – 6 GHz | 5-10 | 5 | 200 |
| 6 GHz - 24 GHz | - | [20] | 200 |
| 24.25 GHz – 52.6 GHz | 50 | 50 | 400 |

## 2.3 MRSS

6G Radio frame structure should preserve compatibility with 5G NR frame structure for efficient 5G-6G interworking and support of 5G-6G multi-RAT spectrum sharing (MRSS), at least for terrestrial networks, while enabling at same time forward compatibility needed to bring new features, [3]. 6GR study must check the support of many URLLC features designed during 5G NR, i.e., whether any such feature needs to be in 6GR Day-1 or it can be brought in later.

Proposal 7: 6GR Day-1 frame structure design should enable inter-working of 5G-6G multi-RAT spectrum sharing (MRSS) and be forward-compatible to bring new features.

Agreement - Identify the high-level aspects which impact on the NR-6GR MRSS support - Including the lessons learned from LTE-NR DSS

One aspect that impacts the 5G NR-6GR MRSS design is the early RAT identification. An early RAT identification such as identifying a 6G RAT from 5G NR RAT can be done by defining orthogonal synchronization sequences i.e., low cross correlation and transmitting such synchronization signals using a distinct sync raster point (raster points can be up to RAN4 discussion) thereby simplifying UE complexity with respect to identifying 5G NR or 6GR RAT. Also, there can be time domain multiplexing between the 5G NR and 6GR common channel transmission especially SSB burst transmission of 6GR can be offset from that of the 5G NR SSB burst transmission to avoid any overlap.

Proposal 8: RAN1 should discuss how to perform early RAT identification using appropriate synchronization signal design as part of 5G NR-6GR MRSS.

## 2.4 Synchronization Signal Structure and Periodicity

Agreement - Identify the high-level aspects which impact on the 6GR sync signal structure and associated periodicity.

The number of SS/PBCH beams supported for XL-MIMO is increased to sufficiently cover all spatial regions from ~200 AE in 5G NR to ~1000 AE in 6GR, resulting in 3dB beam width change from 1.60 (angle/deg.) for the ~200 AE to 0.41 (angle/deg.) for the ~1000 AE. The conclusion is that beam width for XL-MIMO is reduced by a factor of four such that four times more beams are needed to cover the same area when co-sited with 5G. The time domain location of SS/PBCH symbols and the number of SS/PBCH blocks in a slot by reducing the gaps between SS/PBCH blocks is another important factor to be considered to enable compact SS/PBCH transmission in a SSB burst.

Proposal 9: The number of candidate SS/PBCH beams from XL-MIMO in the upper mid-band spectrum and its impact on the network energy consumption needs to be studied and evaluated.

Another aspect is the time domain symbol and frequency domain resource need to define the SS/PBCH block structure considering various aspects such as the minimum common bandwidth depending on the frequency range, etc..,. Various SS/PBCH block structure can be envisioned considering 5MHz or 20MHz minimum channel bandwidth, increased/reduced time domain symbols. These SS/PBCH block structure needs to be compared against 5G NR SS/PBCH block structure in terms of performance e.g., detection reliability, residual time/frequency errror and network power consumption. The lean FDMed SS/PBCH block maybe suitable at wider bandwidth and higher frequency range. Lastly, SS repetitions within SS/PBCH block maybe suitable to increase the reliability of one shot SSB burst detection considering the increase in the SS/PBCH periodicity.

a) FDM-ed SS/PBCH block       b) SS/PBCH block - SS repetition

Figure 2.4-1: Various SS/PBCH block structure envisioned for 6GR

Significant energy saving at higher frequency range can be achieved by minimizing the number of time domain symbols defining SS/PBCH block structure together with compact SS/PBCH block transmission by reducing the gaps between SS/PBCH blocks in an SSB burst. However, for the lower frequency due to limitation in the minimum supported common carrier bandwidth, increasing the periodicity of SSB transmission is the way to gain network energy saving.

Proposal 10: Study the feasibility of different SSB structures for NES including:

- Lean SSB structure for the upper mid band spectrum

- Improving reliability of SSB detection

In 5G, uniform SSB burst periodicity was defined e.g., 5ms, 10ms…,160ms. However, increased periodicity of SSB burst e.g., 160ms, results in few transmissions of SSBs. Although increasing the periodicity of transmitted cell defining SSB brings in significant energy saving, further evaluations are needed to understand its impact on AGC tracking, cell selection time at UE, mobility, other measurements etc., Also, number of SSBs needed for a UE to get time-frequency synchronization to decode other common channel e.g., paging channel can depend on the UE geometry within a cell e.g., SINR. Also, UE in not good geometry needs three or four SSBs resulting in significant delay and power consumption to achieve finer synchronization. Hence, a joint mechanism ensuring network energy saving while simultaneously minimizing the acquisition time at the device side should be studied.

Some of the above drawbacks can be addressed by clustering/grouping SSB bursts with reduced inter-SSB burst gap between them and increasing the periodicity of these clusters resulting in significant network power saving. One benefit of these clustered transmission is that it can maintain sufficient SSB density in the time domain together with increased periodicity. The significant network power saving comes from the fact that the Cell enter the deep sleep state because of 160ms cluster periodicity while Cell can enter only the light sleep state with uniformly spaced SSB periodicity. Furthermore, number of SSB bursts in a cluster also has an impact on power consumption, reducing from four to three SSB bursts reduces the network energy consumption by 15% (TE consumption of 160ms clustered SSB periodicity with four SSB bursts per cluster compared with three SSB bursts per cluster). Although our evaluation assumption assumes only deep sleep state of one relative power value and 50ms transition state, with longer periodicity, network can enter even lower power saving sate e.g., ultra-deep sleep state with even lower relative power value and longer transition state. Such enhancement to power saving state should also be taken into considering as it reduces significant power consumption for longer periodicity. From the Figure 2.4-2, both maximum and average latency is significantly reduced comparing to similar regularly spaced SSB periodicity. Cluster based common channel structure reduces the need of additional RS overhead in terms of power consumption and spectral efficiency. Hence, cluster based SSB transmission can replace regularly spaced SSBs in many ways and under different cell load conditions and should be evaluated.

Figure 2.4-2: Illustration of SSB burst clusters/groups vs Regular 5G NR SSB.

Proposal 11: Study the feasibility and impact of reducing gaps between SS/PBCH block transmission within a SSB burst.

Proposal 12: Study the feasibility of clustering/grouping of SSBs and other common signal/channel and compare pros and cons in terms of acquisition latency and network energy consumption.

## 2.5 Operation of Bandwidth / Band Adaptation

Agreement - Study and identify the lessons learned from NR BWP framework

Currently, the bandwidth of a BWP is semi-statically configured. 5G NR BWP framework allows the UEs to be switched to another narrower bandwidth part using UE specific DCI signalling. Reduction of the bandwidth of active BWPs is not allowed, thereby affecting the flexibility of adapting the bandwidth of a bandwidth part according to the cell load while achieving network energy savings in the low load situation. Moreover, UE dedicated signalling to configure / reconfigure parameters of a bandwidth-part for UE(s) incur lot of signalling overhead e.g., adapting the bandwidth of a bandwidth part for a group of UE(s) as part of network energy savings in low load situation.

Observation 3: Reconfiguration of bandwidth-part incur huge signalling overhead in 5G NR due to usage of UE dedicated signalling.

Many parameters in the dedicated Radio resource configuration can impact bandwidth-part switching delay due to increased configuration reload time. The switching delay may be reduced by redesigning number of parameters associated to a dedicated bandwidth-part.

Observation 4: In 5G NR, volume of dedicated bandwidth-part radio resource configuration impacts bandwidth-part switching delay.

Proposal 13: RAN1 should re-design bandwidth part considering minimizing switching delay and reducing signalling overhead.

## 2.6 Multiple Access Schemes

Regarding Multiple Access (MA) schemes and associated enhancements, 5G NR Orthogonal Multiple Access (OMA) techniques, i.e., TDMA and OFDMA, should be the baseline for 6G Radio. However, for massive IoT applications and scenarios, contention-based multiple access schemes may be considered to reduce signalling overhead to accommodate sporadic IoT traffic while reducing energy consumption.

Proposal 14: On top of TDMA and OFDMA for 6GR, contention-based multiple access schemes may be considered for IoT applications and scenarios.

## 2.7 Harmonization of TN and NTN

In RAN1#122, following was agreed to pursue a harmonized air-interface design for both Terrestrial Networks (TN) and Non-Terrestrial Networks (NTN).

Agreement For harmonized 6GR design for TN and NTN, RAN1 studies to identify the technical aspects affected by NTN characteristics, as well as lessons learned from NR/IoT NTN

Designing 6G from the outset with NTN provisions to extend TN coverage should be considered for support of critical services (e.g., emergency text, call), mainstream voice (i.e., beyond 5G-Advanced restricted scenario of voice over NB-IoT-NTN GEO Access) as well as and basic data connectivity (e.g., for IoT and limited MBB) services. Since NTN deployment started late within 5G NR and the lifecycle of satellite networks can be different compared to TN, the 6G Day-1 functionality should include a minimum set of features in the unified 6G RAT to support NTN functionality. Building upon the ongoing studies of waveform, modulation, and coding, certain NTN-specific aspects should be included early stages of study. For instance, waveform robustness against high Doppler, adaptive and flexible numerologies to accommodate long propagation delays, and polarization multiplexing  to enhance link capacity and throughput in NTN scenarios are critical.

Equally important is the design of initial access and common channels. 5G NR NTN initial access procedures, designed for terrestrial design, face limitations under typical NTN conditions characterized by large propagation delays, high Doppler shifts, intermittent coverage, and wide beam footprints. 5G NR synchronization (PSS/SSS/PBCH) and RACH procedures were not optimized for such environments, e.g., timing advance windows are insufficient for long delays, Doppler severely degrades correlation-based detection, and common channel configurations may not guarantee reliable access over large footprints. For 6G, studies should focus on NTN-aware synchronization and SSB design robust to Doppler, extended RACH procedures supporting large timing uncertainties, and enhanced common channel structures that ensure consistent broadcast coverage. Additionally, mechanisms for GNSS-free operation, predictive or re-access procedures under intermittent visibility, and possible use of polarization diversity during access should be explored to achieve unified and reliable initial access for both TN and NTN in 6G.

Positioning, Navigation, and Timing (PNT) is expected to play a critical role in 6G. Traditional GNSS-based solutions are not always reliable in challenging environments. At the same time, NTN platforms inherently offer wide coverage and can serve as a complementary or backup source of PNT information. The problem, however, lies in the lack of tight integration between communication and navigation functions. The motivation for 6G is therefore to leverage communication waveforms, synchronization signals, and broadcast channels not only for data transfer but also for precise timing, ranging, and positioning. To achieve this, the first step is to design NTN-aware reference signals and frame structures that enable accurate time transfer and Doppler-resilient ranging. Such design could also be incorporated during the early stages of 6G study.

For future releases, more advanced enhancements can be staged, once the baseline framework has been validated. For example, refinements to common channel design could be carried out to enable adaptive broadcast signaling optimized for orbital dynamics and user distribution, and polarization multiplexing could be further extended to joint spatial polarization multiplexing techniques for capacity scaling. Integration of AI/ML-based link adaptation for delay/Doppler prediction, as well as multi-connectivity across TN and NTN with seamless service continuity, can also be progressively introduced.  Similarly, for PNT system, advanced techniques such as combining TN/NTN anchors, integrating GNSS with 6G signals, or employing AI/ML-based hybrid localization can be studied in later releases to provide seamless and resilient PNT services.

Proposal 15: RAN1 to study at least the following in Rel-20 to achieve a harmonized air-interface design for both TN and NTN:

- Waveform aspects, e.g., low PAPR (DFT-s-OFDM for DL), robustness against Doppler

- Consider NTN constraints such as Doppler (e.g., 48 Khz in S-band for LEO satillites) and delay requirements (e.g., defined in Table 7.1-1 of TR 38.821) for initial access and common channel design, e.g., synchronization signals, PRACH etc.,

- Optimal SSB periodicity and beam sweeping patterns for large footprints and earth moving beams.

- Fast re-access or re-synchronization when beams move out of coverage

- Initial access procedure not relying on GNSS pre-compensation

- NTN-aware reference signals and frame structures that enable accurate time transfer and Doppler-resilient ranging

- Use of circular polarization during synchronization and RACH to improve detection reliability and throughput

## 2.8 Duplexing

In the last RAN1#122 meeting, the following agreement was reached regarding.

Agreement - Study and identify the lessons learned from NR duplex modes - On 6GR duplexing study, RAN1 considers at least following duplex types - FD-FDD - Semi-static TDD - gNB semi-static SBFD - HD-FDD on UE side - Dynamic TDD - Study whether to consider following duplexing types - gNB dynamic SBFD - UE SBFD - gNB FD - Note: Other duplex modes are not precluded

From the above agreement, the supported duplexing modes by RAN1 are FD-FDD, semi-static TDD, gNB semi-static SBFD, HD-FDD on the UE side, and dynamic TDD. Therefore, compared to Day-1 5G-NR, Day-1 6GR should include SBFD slots/symbols, besides DL, UL, and flexible slots/symbols in the slot/symbol format configuration and indication mechanisms.

Proposal 16: Day-1 6GR should support following slot/symbol types:

- DL slot/symbol for DL transmission

- UL slot/symbol for UL transmission

- SBFD slot/symbol with part of frequency resources for DL transmission and part for UL transmission

- Flexible slot/symbol with part of symbols for DL transmission and part for UL transmission

However, with the dynamic TDD, one of the key lessons learned from 5G-NR duplex and 5G-NR, in general, is that excessive flexibility introduces major practical and operational challenges. 5G-NR has supported dynamic TDD since Rel-15, but it has seen limited or no deployment success so far. One contributing factor could be that the relevant inter-gNB and inter-UE CLI handling schemes were only specified later in Rel-16 and may not have been part of the initial 5G-NR deployed features. Another factor is the strong intra-band inter-gNB CLI, which led to the Rel-16 CLI study concluding that dynamic TDD is unsuitable for macro-cell deployments. Note that, to support dynamic TDD, two different symbol format indication mechanisms are currently specified for configuring common “flexible” symbols as either DL or UL for a UE: UE-specific semi-static configuration via tdd-UL-DL-ConfigurationDedicated and dynamic SFI UE-group indication via DCI format 2_0. Among them, the dynamic SFI approach introduces additional PDCCH decoding complexity. Nonetheless, the motivation for dynamic TDD duplex mode remains valid in 6GR - particularly with the expectation of asymmetric and dynamic real-time UL/DL traffic. This is also true for gNB dynamic SBFD duplex mode. Therefore, it is beneficial to include dynamic TDD and gNB dynamic SBFD modes in 6GR duplexing study, though a more deployment-friendly versions may be necessary. This may include an enhanced inter-gNB CLI handling scheme and a less complex, more practical symbol-format indication mechanism to realize at least part of the potentials of dynamic TDD/SBFD.

Proposal 17: RAN1 to study a more practical friendly dynamic TDD and dynamic SBFD modes in 6GR duplexing.

On other hand, UE side SBFD and gNB FD are attractive features to further reduce DL/UL latency, improve coverage and throughput, etc. However, both schemes face same challenge, i.e., self-interference. Currently, UE side SBFD seems to be practically applicable to only advanced and more capable UE types, e.g., FWA, CPE, and in some deployment scenarios and conditions. Nonetheless, hardware and signal processing techniques are advancing rapidly, and they might prove in the near future that UE side SBFD and gNB FD (i.e., overlapping SBFD) are practically applicable, even for less advanced UEs. Therefore, Day-1 6GR duplexing should be designed with forward compatibility mindset to avoid fragmented design as in 5G-NR.

Proposal 18: RAN1 to include UE side SBFD in Day-1 6GR duplexing targeting at least advanced and more capable UE types.

Proposal 19: Day-1 6GR duplexing should be designed with forward compatibility mindset to accommodate future advanced duplexing in Day-2/3 6GR.

## 2.9 Control Channel Design

In 5G NR, the downlink physical layer control channel design was flexible by introducing Control Resource set (CORESET) and search space monitoring with flexible time and frequency placements. The drawback is that the power consumption due to the number of blind decoding attempts to detect the downlink control at the device side. Hence, 6G Radio should study and evaluate new schemes to reduce the blind decoding attempts such as minimizing the downlink control information formats, monitoring periods etc.

Proposal 20: The study of 6GR should include ways to reduce power consumption of downlink control signal from beginning.

5G NR defines multiple uplink control channel formats for the uplink control channel to transmit combination of scheduling requests, ACK/NACK feedback and measurements such as channel state information etc.. At the same time, the uplink control information can also be piggybacked with the uplink data channel. 6G Radio should aim to create a lean design for uplink control channel.

Proposal 21: 6GR should strive for a leaner design of uplink control channels.

# 3	Conclusions

Proposal 1: Study 6G Radio design on how to enable scalable chipset design enabling scalable coverage targets and addressing a variety of service requirements spanning across MBB and IoT over different device types.

Observation 1: Critical questions towards a scalable 6G design are: how to design the common feature set across device types, and what additional feature needs to be added or removed to differentiate across device types.

Proposal 2: Study utilizing explicit device categories to identify in a scalable manner different operational feature sets associated with various device types.

Proposal 3: Evaluate towards a scalable 6G Radio design the following staggered steps:

- Identification of common feature set parameters across device types;

  - Includes common channels e.g., SS/PBCH, CORESET#0, MSI, RACH, paging considering payload restriction due to minimum bandwidth, data rate

  - Common features as listed in Table 2.1-3

- Discussion and evaluation of parameter values/value ranges for common feature set listed in Table 2.1-3 considering each device types;

- Discussion and evaluation of additional dedicated feature set parameters listed in Table 2.1-3 applicable to device types; and

- Association of specific values/value ranges to common and/or dedicated feature set parameter values/value ranges per each device category should be done later at RAN plenary level.

Observation 2: Globally, practical deployments and availability of 5 MHz allocations is much larger than the one of 3 MHz allocations.

Proposal 4: Study optimizations of the system design for the 5MHz carrier bandwidth in consideration of the following benefits:

- Pragmatic approach aligned with larger 5 MHz vs 3 MHz global avaliability and deployments

Proposal 5: Evaluate based on existing deployments and global availability of 3 MHz carrier whether there is practical and commercial need for further optimization to enhance performance / coverage for 3 MHz carrier.

Proposal 6: The 6G Radio minimum/maximum transmission bandwidth per frequency range that we regard feasible and realizable given spectrum challenges is outlined below in Table 2.2-1.

Proposal 7: 6GR Day-1 frame structure design should enable inter-working of 5G-6G multi-RAT spectrum sharing (MRSS) and be forward-compatible to bring new features.

Proposal 8: RAN1 should discuss how to perform early RAT identification using appropriate synchronization signal design as part of 5G NR-6GR MRSS.

Proposal 9: The number of candidate SS/PBCH beams from XL-MIMO in the upper mid-band spectrum and its impact on the network energy consumption needs to be studied and evaluated.

Proposal 10: Study the feasibility of different SSB structures for NES including:

- Lean SSB structure for the upper mid band spectrum

- Improving reliability of SSB detection

Proposal 11: Study the feasibility and impact of reducing gaps between SS/PBCH block transmission within a SSB burst.

Proposal 12: Study the feasibility of clustering/grouping of SSBs and other common signal/channel and compare pros and cons in terms of acquisition latency and network energy consumption.

Observation 3: Reconfiguration of bandwidth-part incur huge signalling overhead in 5G NR due to usage of UE dedicated signalling.

Observation 4: In 5G NR, volume of dedicated bandwidth-part radio resource configuration impacts bandwidth-part switching delay.

Proposal 13: RAN1 should re-design bandwidth part considering minimizing switching delay and reducing signalling overhead.

Proposal 14: On top of TDMA and OFDMA for 6GR, contention-based multiple access schemes may be considered for IoT applications and scenarios.

Proposal 15: RAN1 to study at least the following in Rel-20 to achieve a harmonized air-interface design for both TN and NTN:

- Waveform aspects, e.g., low PAPR (DFT-s-OFDM for DL), robustness against Doppler

- Consider NTN constraints such as Doppler (e.g., 48 Khz in S-band for LEO satillites) and delay requirements (e.g., defined in Table 7.1-1 of TR 38.821) for initial access and common channel design, e.g., synchronization signals, PRACH etc.,

- Optimal SSB periodicity and beam sweeping patterns for large footprints and earth moving beams.

- Fast re-access or re-synchronization when beams move out of coverage

- Initial access procedure not relying on GNSS pre-compensation

- NTN-aware reference signals and frame structures that enable accurate time transfer and Doppler-resilient ranging

- Use of circular polarization during synchronization and RACH to improve detection reliability and throughput

Proposal 16: Day-1 6GR should support following slot/symbol types:

- DL slot/symbol for DL transmission

- UL slot/symbol for UL transmission

- SBFD slot/symbol with part of frequency resources for DL transmission and part for UL transmission

- Flexible slot/symbol with part of symbols for DL transmission and part for UL transmission

Proposal 17: RAN1 to study a more practical friendly dynamic TDD and dynamic SBFD modes in 6GR duplexing.

Proposal 18: RAN1 to include UE side SBFD in Day-1 6GR duplexing targeting at least advanced and more capable UE types.

Proposal 19: Day-1 6GR duplexing should be designed with forward compatibility mindset to accommodate future advanced duplexing in Day-2/3 6GR.

Proposal 20: The study of 6GR should include ways to reduce power consumption of downlink control signal from beginning.

Proposal 21: 6GR should strive for a leaner design of uplink control channels.

# References

[1] 3GPP TR 38.914: Study on 6G Scenarios and requirements, v0.1.0.

[2] RP-251881: 3GPP Study on 6G Radio (FS_6G_Radio), Status: Approved at 3GPP RAN Plenary RAN#108.

[3] R1-2505526: Discussion on numerology and frame structure for 6GR air interface, Source: Lenovo, submitted to RAN1#122.