3GPP TSG RAN WG1 #122 bis			R1-2507496

Prague, Czech, Oct. 13th – 17th, 2025

Agenda item:	10.1.1.1

Source: 	ETRI

Title:	Discussion on inference related aspects of CSI compression

Document for:	Discussion

Introduction

The study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has progressed during Release-18 and the spatial-frequency domain CSI compression using two-sided AI model has been agreed for one of the representative sub-use cases for CSI feedback enhancement[1]. In Release-19, further study for CSI compression have been done to address the trade-off between performance and complexity/overhead and to alleviate/resolve issues related to inter-vendor training collaboration[2] and reached the following agreement[3]:

Based on the study, RAN1 recommends the following with potential RAN1 specification impacts common to all inter-vendor collaboration options. - Recommend Case 0 - Case 2 and 3 can be considered for further enhancement built upon the specification of case 0. - Recommend following specification impacts for the two-sided CSI compression use case - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI / RI determination, payload determination, quantization configuration / codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note: The framework defined in BM and CSI prediction use cases could be reused. - Specify performance monitoring, if needed

In RAN #108, it was agreed to proceed the normative work of Release 20 with the following objectives[4]:

AI/ML framework for two sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_air_Ph2 and NR_AIML_air projects for CSI feedback enhancement [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model selection, activation, deactivation, switching, fallback - ID related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring,. CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

Following is the agenda regarding the AI/ML for NR air interface enhancements in Release-20 for RAN1 #122[5]:

- 10.1 Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements Please refer to RP-251870 for detailed scope of the WI. - 10.1.1 CSI spatial/frequency compression without temporal aspects (“Case 0”) - 10.1.1.1 Inference related aspects Including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports. - 10.1.1.2 Other aspects Including NW and UE data collection for training, performance monitoring, as well as model pairing related issues. - 10.1.2 Inter-vendor training collaboration for two-sided AI/ML models Including specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”), as well as RAN4-triggered issues.

In this contribution, we present our views on inference related aspects of CSI compression for the normative work of AI/ML for NR Air Interface in Release-20.

Discussion

Target CSI type

A following agreement regarding target CSI type was made during RAN1 #122[10].

Agreement: For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider raw channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

The remaining issue under discussion is the domain of the precoding matrix, the spatial-frequency domain or the beam-delay domain. In our view, the spatial-frequency domain is the more suitable choice, as it preserves alignment with the legacy specification.

Proposal 1: For AI/ML-based CSI compression, consider the precoding matrix on the spatial-frequency domain as baseline.

Measurement and report configuration

To support AI/ML-based CSI compression, it is essential to design measurement and report configurations tailored for AI/ML-based CSI compression. We propose to reuse of the legacy CSI framework as much as possible to ensure efficiency and compatibility.

For the measurement configuration for AI/ML-based CSI compression, we suggest reusing the existing CSI framework. This approach maintains consistency with legacy systems and simplifies integration.

Proposal 2: For AI/ML-based CSI compression, reuse the measurement resource configuration in the legacy CSI framework.

For the report configuration, however, it is necessary to define the additional components required to support AI/ML-based CSI compression within the existing CSI framework. Specifically, following aspects are need to be studied for the report configuration:

- Mechanism to indicate to construct CSI feedback payload using AI/ML-based encoder model

- Method to ensure consistency between the NW-side model and the UE-side model

- Method to indicate how to derive CQI (Channel Quality Indicator) and RI (Rank Indicator)

- Configuration to support multiple CSI payload length using the AI/ML-based encoder model.

Proposal 3: For AI/ML-based CSI compression, reuse the report configuration in the legacy CSI framework and further study following aspects:

- Mechanism to indicate to construct CSI feedback payload using AI/ML-based encoder model

- Method to ensure consistency between the NW-side model and the UE-side model

- Method to indicate how to derive CQI (Channel Quality Indicator) and RI (Rank Indicator)

- Configuration to support multiple CSI payload length using the AI/ML-based encoder model.

CQI and RI determination

When the UE performs AI/ML-based CSI feedback, it may also derive the Rank Indicator (RI) and Channel Quality Indicator (CQI) as part of the reporting process. For RI, we propose to follow the same determination method as in the legacy CSI feedback framework. This ensures compatibility and reduces complexity.

CQI derivation, however, presents a new challenge in the context of AI/ML-based CSI feedback. Specifically, the UE lacks knowledge of the reconstructed CSI at the gNB (i.e., the output-CSI-NW), which is necessary for computing post-SNR and, consequently, for accurate CQI estimation. Various CQI derivation approaches were discussed in the study phase as following[6]:

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Each of the above options has its own advantages and disadvantages, and as a result, no definitive conclusion was reached. Given the importance of CQI in overall system performance and the unique constraints introduced by AI/ML-based CSI feedback, we propose continued discussions to enable specification support for CQI determination.

Proposal 4: For AI/ML-based CSI compression, further discuss on the CQI determination method.

Quantization and payload determination

In RAN1 #122, there were discussions on quantization and following agreements were made[10].

Agreement Specify the precoding matrix feedback via two-sided model as follows: For a certain layer and rank , - A precoding matrix is mapped with a latent message , with l = 1, ..., v where l is the layer index. - The mapping is subject to [at least] a pairing ID configured by high-layer signalling. - FFS is common or specific for different layers or ranks - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further - is quantized and mapped to bit sequence before reporting. Agreement For precoding matrix feedback, for a certain layer of rank , the latent message contains real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer and rank is determined by , where is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer and rank is determined by , where is the length of the vector segment, is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - and / or and / or are fixed values or determined from multiple values based on NW configuration or UE reporting. - and / or is layer-common or layer-specific, rank-common or rank-specific. - If configured, and / or and / or above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive , L and . Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: , and are independent on subband and port configurations, - FFS each combination of , and is applicable to a certain set of subband and port configurations

A point of discussion is whether to support scalar quantization, vector quantization, or both scalar and vector quantizations. Each approach has its respective pros and cons: vector quantization generally offers higher flexibility and superior performance, whereas scalar quantization provides greater simplicity in implementation and specification. From our perspective, vector quantization may need to be supported, with scalar quantization treated as a special case within the vector quantization framework.

Proposal 5 : For AI/ML-based CSI compression, prioritize to consider the vector quantization and consider the scalar quantization in the framework of vector quantization, if needed.

When scalar quantization is applied, the required quantization-related parameters are {d, Q}, whereas in the case of vector quantization, the required parameters are {d, L, Q}. For inference configuration, it may be necessary to specify parameter sets associated with quantization. With respect to parameter specification, further discussion is needed on whether the parameters should be rank-specific or rank-common, and whether they should be layer-specific or layer-common. From our perspective, the quantization parameters may be configured in a rank-common and layer-specific manner.

Proposal 6: For AI/ML-based CSI compression, support configuration of the quantization parameters in rank-common and layer-specific manners.

The values of the quantization parameters also require further discussion, as they directly determine the CSI feedback size per layer. From our perspective, the size of CSI feedback per layer should fall within a range of approximately 24 to 256 bits, taking into account the information sizes per layer defined in the current specification for the eTypeII codebook.

In addition to the quantization parameters, quantization codebooks should be configured to support both inference and inter-vendor training collaborations. It should be noted that the UE-side implementation may develop an actual model whose output dimension differs from that of the reference model, and may also employ a quantization codebook distinct from the one used in the reference model. For instance, even if the standardized AI/ML model is specified with scalar quantization, an actual implementation may adopt vector quantization instead.

Nevertheless, the quantization codebook information is essential for enabling inter-vendor training collaboration. In this regard, two potential options for defining quantization codebooks have been proposed, as outlined in the agreement reached at RAN1 #122[10].

Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

The first option is to pre-define quantization codebooks within the specification, such that all vendors develop AI/ML models based on the standardized codebook. The second option is to allow each vendor to define its own trained codebook in accordance with the AI/ML model it develops. From our perspective, the performance of AI/ML-based CSI feedback can be enhanced by leveraging co-trained codebooks; therefore, we prefer the approach of utilizing trained codebooks, with such codebooks exchanged through inter-vendor training collaborations.

Proposal 7: For AI/ML-based CSI compression, consider the trained quantization codebook and its exchanges to enable inter-vendor training collaborations.

For inference report configuration, it was agreed to use paring ID as a following agreement in RAN1 #122[10].

Agreement: For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Payload size is closely related to the quantization parameters, whose values may need to be specified. Vendors are developing AI/ML models that support either the full set or only a subset of possible quantization parameter values. In cases where vendors develop models supporting only a subset of values, the NW and the UE must exchange their supported parameter sets during the model pairing procedure. Otherwise, such exchange of supported quantization parameter values is not required.

Observation 1: If AI/ML models support only a subset of quantization parameter values, the exchange of available quantization parameters is necessary during the model pairing procedure.

During inference configuration, the network configures the quantization parameters for the layer(s). If the parameters are defined in a layer-specific manner, multiple parameters must be configured. Furthermore, if the quantization parameters are defined in a rank-specific manner, multiple sets of parameters must be configured.

Observation 2: More than one quantization parameter sets may be configured during inference configuration, except in the case where the quantization parameters are both layer-common and rank-common.

UCI mapping

During AI/ML-based CSI feedback procedure, UE prepares input(s) (e.g., precoding matrix), processes it through the CSI generation model to obtain latent message(s), converts them into bit sequences for reporting, and subsequently generates the UCI based on these bit sequence(s). UCI mapping refers to the last process of generating uplink control information (UCI) from the bit sequence(s), for uplink transmission. For AI/ML-based CSI feedback, we propose to adopt the type2-r16 UCI mapping as the baseline approach. This may ensure compatibility with existing frameworks while minimizing the need for structural changes for introducing AI/ML-based CSI feedback.

In the Type2-R16 UCI mapping, CSI Part 1 includes an indicator representing the total number of non-zero coefficients summed across all layers, which is used to determine the size of CSI Part 2. However, in AI/ML-based CSI feedback, this type of indication is not applicable. Therefore, a new method for indicating the size of CSI Part 2 needs to be investigated.

Furthermore, in AI/ML-based CSI feedback, CSI Part 2 cannot be split into multiple groups having different meanings. We therefore propose mapping the UCI into either a single group or, at most, two groups—for example, mapping the UCI for layer 0 to Group 0 and the UCI for all other layers to Group 1.

Proposal 8: For AI/ML-based CSI feedback, adopt type2-r16 UCI mapping as the baseline, investigate a new method for indicating the size of CSI Part 2 in CSI Part 1, and map the CSI Part 2 into a single group or, at most, two groups.

CSI processing criteria and timeline

The specification support of CSI processing criteria and timeline is fundamentally related to the AI/ML-based encoder model at the UE side. Therefore, we propose to apply the design principles and procedures already established in beam management or CSI prediction use-cases, those are based the UE-side model, in Release-19. This approach ensures consistency across features, leverages existing design, and minimizes standardization efforts for the specification supports of AI/ML-based CSI compression.

Observation 3: For AI/ML-based CSI compression, the CSI processing criteria and timelines designed in UE-side beam management or CSI prediction use cases can be reused.

Priority rules for CSI reports

For the priority rules for AI/ML-based CSI reports, we propose adopting the priority rules for the legacy type2-r16 CSI reports as the baseline, consistent with the approach suggested above for UCI mapping.

Proposal 9: For AI/ML-based CSI compression, adopt the priority rules for the legacy type2-r16 CSI reports.

Codebook subset restriction (CBSR)

The codebook subset restriction (CBSR) in the legacy CSI feedback framework provides a mechanism to exclude certain beams/precoders at the gNB. With respect to CBSR, the following agreement was reached during the study item phase at RAN1 #113 [7].

Agreement In CSI compression using two-sided model use case, further study the feasibility of at least the following methods to support codebook subset restriction: - input-CSI-NW/output-CSI-UE is in angular-delay domain, beam restriction can be based on legacy SD basis vector-based input CSI in angular domain. - FFS amplitude restriction - FFS if input-CSI-NW/output-CSI-UE is in spatial-frequency domain

In our view, a similar restriction mechanism is also necessary in the AI/ML-based CSI feedback framework to enable network operation with interference control. Nevertheless, the CSI feedback framework employing AI/ML models may differ from the one defined in the current specification. Therefore, the feasibility of applying such restrictions within the AI/ML-based CSI feedback framework requires further study.

Proposal 10: For AI/ML-based CSI compression, investigate the feasibility of introducing codebook subset restriction in the AI/ML-based CSI feedback framework.

Conclusion

In this contribution, ETRI’s views on AI/ML for CSI compression sub-use case were shown and the following observations and proposals were made:

Proposal 1: For AI/ML-based CSI compression, consider the precoding matrix on the spatial-frequency domain as baseline.

Proposal 2: For AI/ML-based CSI compression, reuse the measurement resource configuration in the legacy CSI framework.

Proposal 3: For AI/ML-based CSI compression, reuse the report configuration in the legacy CSI framework and further study following aspects:

- Mechanism to indicate to construct CSI feedback payload using AI/ML-based encoder model

- Method to ensure consistency between the NW-side model and the UE-side model

- Method to indicate how to derive CQI (Channel Quality Indicator) and RI (Rank Indicator)

- Configuration to support multiple CSI payload length using the AI/ML-based encoder model.

Proposal 4: For AI/ML-based CSI compression, further discuss on the CQI determination method.

Proposal 5 : For AI/ML-based CSI compression, prioritize to consider the vector quantization and consider the scalar quantization in the framework of vector quantization, if needed.

Proposal 6: For AI/ML-based CSI compression, support configuration of the quantization parameters in rank-common and layer-specific manners.

Proposal 7: For AI/ML-based CSI compression, consider the trained quantization codebook and its exchanges to enable inter-vendor training collaborations.

Proposal 8: For AI/ML-based CSI feedback, adopt type2-r16 UCI mapping as the baseline, investigate a new method for indicating the size of CSI Part 2 in CSI Part 1, and map the CSI Part 2 into a single group or, at most, two groups.

Proposal 9: For AI/ML-based CSI compression, adopt the priority rules for the legacy type2-r16 CSI reports.

Proposal 10: For AI/ML-based CSI compression, investigate the feasibility of introducing codebook subset restriction in the AI/ML-based CSI feedback framework.

Observation 1: If AI/ML models support only a subset of quantization parameter values, the exchange of available quantization parameters is necessary during the model pairing procedure.

Observation 2: More than one quantization parameter sets may be configured during inference configuration, except in the case where the quantization parameters are both layer-common and rank-common.

Observation 3: For AI/ML-based CSI compression, the CSI processing criteria and timelines designed in UE-side beam management or CSI prediction use cases can be reused.

- References

- 3GPP TR 38.843 V18.0.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), 3GPP RAN #102, Dec. 2023.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #121, May 2025.

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm (Moderator), 3GPP RAN #108, June 2025.

- R1-2505100, Draft Agenda, Chair (China Mobile), 3GPP RAN WG1 #122, August 2025.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #112, Feb. 2023.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #113, May. 2023.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #114-bis, October 2023.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #120-bis, April 2025.

- 3GPP RAN1, Chairman's notes, 3GPP RAN WG1 #122, August 2025.