3GPP TSG RAN WG1 #122bis                                R1-2507553

Prague, Czech, Oct. 13th – 17th, 2025

Source:	Sharp

Title:	Discussions on Inter-vendor training collaboration

Agenda Item:	10.1.2

Document for:	Discussion and Decision

Introduction

In RAN#108[1], a new WI of AI/ML for NR air interface enhancements that mainly includes normative work of CSI compression use case was agreed and the objective for Inter-vendor training collaboration in the WID is copied as follows.

Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

In this contribution, we discuss procedures and specification impacts of Direction A and Direction C with regard to Inter-vendor training collaboration.

Discussions

As concluded in RAN1#120bis, Direction A with sub-option 3a-1/4-1 and Direction C were deemed as feasible to alleviate/resolve Inter-vendor training collaboration complexity[2] and thus recommended to RAN plenary for future work.

Conclusion (RAN1#120bis) - RAN1 concludes that both Direction A and Direction C are feasible. - RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible. - This includes both 3a-1 with and without target CSI sharing.

For Direction C, a standardized model being tested by RAN4 ensures a minimum performance requirement. On top of that, the model structure based on the standardized model, i.e. reference model for Direction A sub-option 3a-1 with exchange of parameters from gNB to UE and re-training of the generation model at UE-side would achieve better performance in terms of model output. For sub-option 4-1, UE vendors would literally re-train the encoder via the exchanged data set from gNB.

Direction A sub-option 3a-1/Direction C

For Direction C, the purpose of a fully specified CSI generation (reconstruction) model is to enable training of compatible CSI reconstruction (generation) model at the NW (UE) side and as aforementioned, Direction A sub-option 3a-1 can be treated as optimization of Direction C, and the major improvement lies on the exchange of parameters. Considering the specified model (Direction C) only meets the minimum performance requirement, when the parameters exchanged are not available at UE-side, UE still can perform CSI compression with the standardized model, i.e. fall back to Direction C.

Proposal 1: For Direction A sub-option 3a-1, if the parameters exchanged from gNB are not available, UE performs CSI feedback (compression) with the specified model provided by RAN4.

Two alternatives are listed for model re-training, shown as follows[3].

Agreement(RAN1#118bis) For the evaluation studies of Direction A, - For encoder parameter exchange from NW-side to UE-side of 3a-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder against the exchanged encoder. - Step 2: UE-side develops actual encoder against the nominal decoder - Alt. 2: UE-side directly develops and trains the actual encoder. - Note: Target CSI is available at UE side - For dataset exchange from NW-side to UE-side {target CSI, CSI feedback} of 4-1, - Alt. 1: UE-side first trains a nominal decoder - Step 1: UE-side develops a nominal decoder using the exchanged dataset. - Step 2: UE-side develops an actual encoder against the nominal decoder. - Alt. 2: UE-side directly develops and trains the actual encoder.

Alt 1 provides better performance compared to Alt 2 while brings more complexity. Specifically, UE utilizes the same model input/output (i.e. CSI feedback/target CSI) for training of reconstruction model and develops the generation model on top of the re-trained reconstruction model. Therefore, Alt 1 ensures better match with the trained generation model at gNB. While considering target CSI was already precluded, Alt 2, i.e. UE directly trains the generation model should be adopted.

Proposal 2: For Direction A sub-option 3a-1, UE directly trains the encoder with the parameters exchanged, i.e. Alt.2.

Direction A sub-option 4-1

For Direction A sub-option 4-1, dataset is exchanged from gNB to UE for model re-training. The dataset content is agreed as follows[3].

Agreement(RAN1#118bis) For Directions of addressing inter-vendor collaboration complexity for two-sided CSI compression, - For Direction A: - Further study the following options with potential down-selection - 4-1: dataset is {target CSI, CSI feedback}, - 3a-1: encoder parameter exchange, with {target CSI} - Note: Additional information beyond what’s listed above may need to be exchanged if necessary.

Once the dataset is shared, UE trains the generation model taking {target CSI, CSI feedback} as {model input, model output} with different training methods/algorithms provided by UE vendors. Obviously, without the standardized reference model, sub-option 4-1 achieves better training performance and a more fit generation model paired with the reconstruction model.

As discussed above, for training Alt 1 which may bring better performance, when target CSI is available at UE-side, UE utilizes the exchanged CSI feedback/target CSI for training of reconstruction model and trains the generation model on top of the re-trained reconstruction model accordingly.

Proposal 3: For Direction A sub-option 4-1, adopt Alt.1 for generation model training, i.e. UE firstly trains a nominal reconstruction model and then trains a new generation model.

CSI feedback type and format

For dataset exchange, a dataset comprises one or multiple data pairs. Each data pair consists of a target CSI and the corresponding CSI feedback. The agreement related to CSI feedback is shown in RAN1#122 as the following [4]

Agreement(RAN#122) For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, down select one of the following options: - Option 1: The exchanged CSI feedback is the latent message before quantization. - Option 2: The exchanged CSI feedback is the binary sequence at the output of quantization. - Note: Quantization codebook is exchanged in addition if not specified in the spec. - Note: Whether or not using the Quantization codebook for model training, is up to UE implementation.

This agreement lists the options of CSI feedback type, wherein the CSI feedback type may be the latent message before quantization or the binary sequence at the output of quantization.

With option 1, a UE trains an encoder within a CSI generation part at the UE side by using a data pair without considering quantization. This option may be a simple way that the UE can train the encoder directly. With option 2, a UE needs to convert CSI feedback to a latent message. Then, the UE use the latent message to train an encoder within a CSI generation part at the UE side. Moreover, this option may place an addition burden on the UE. Thus, we propose to select option 1 as CSI feedback type.

Proposal 4: For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, support to select option 1 as CSI feedback type.

Conclusion

In this contribution, we have discussed our views on Inter-vendor training collaboration and make the following proposals:

Proposal 1: For Direction A sub-option 3a-1, if the parameters exchanged from gNB are not available, UE performs CSI feedback (compression) with the specified model provided by RAN4.

Proposal 2: For Direction A sub-option 3a-1, UE directly trains the encoder with the parameters exchanged, i.e. Alt.2.

Proposal 3: For Direction A sub-option 4-1, adopt Alt.1 for generation model training, i.e. UE firstly trains a nominal reconstruction model and then trains a new generation model.

Proposal 4: For Option 4-1 under Direction A in AI/ML based CSI compression, for CSI feedback type and format, support to select option 1 as CSI feedback type.

References

RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, RAN#108.

Chairman’s notes, RAN1#120bis.

Chairman’s notes, RAN1#118bis.

Chairman’s notes, RAN1#122.