3GPP TSG RAN WG1 #122bis			R1-2507584

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	11.5

Source:	InterDigital, Inc.

Title:	Energy Efficiency in 6GR air interface

Document for:	Discussion

# Introduction

In RAN#108, the study item for 6GR has been approved [1]. As indicated in the SID, energy efficiency and energy saving for both network and device were included as a main objective. In RAN1#122bis, configurations and evaluation assumptions were mainly discussed. As a result, the following agreements were made:

Agreement Study how to reuse and update reference configurations in TR 38.864 for 6G BS. Agreement Study how/whether to reuse or update the power model in TR 38.864 for evaluating BS power consumption for 6G BS. Agreement - Study metric(s) for UE energy efficiency. - Study metric(s) for BS energy efficiency. Agreement Study reference configurations and power consumption model for 6G UE, considering but not restricted to the following: - TR 38.840 (UEPS), TR38.875 (RedCap), TR38.865 (eRedCap), and TR38.869 (LP-WUS/WUR) for reference configurations - TR 38.840 (UEPS), TR38.875 (RedCap), and TR38.869 (LP-WUS/WUR) for power consumption models Agreement - Study baseline BS setting(s) for evaluating 6G BS EE improvement/impact, considering NR features and 6G BS reference configuration(s) - Study baseline UE setting(s) for evaluating 6G UE EE improvement/impact, considering NR features and 6G UE reference configuration(s)

In this contribution, we provide our views on energy efficiency for 6GR.

# Discussions

## Joint energy saving between NW and UE

In 5G NR, the energy saving techniques at UE and NW were developed via separate and parallel paths over the course of multiple releases. One shortcoming of this approach is that the independent design of the two sets of techniques does not take into account the implications and impacts on the other side. For example, NW-side cell DTX and cell DRX operations are independently configured and (de)activated, and whereas the UE-side C-DRX, PDCCH skipping and search space set group (SSSG) switching are handled with yet another set of configurations.

Observation 1: In NR, all the energy saving techniques for UE and NW are supported based on separate configurations and individual signaling mechanisms.

Since the energy saving techniques at the UE and NW are well correlated, having to handle the features independent of each other (e.g. via independent configurations/signaling mechanisms) can only lead to more inefficiency in terms of latency and achievable energy saving gains.

Observation 2: Although the power saving techniques for UE and NW are separately supported, all such techniques are correlated. Therefore, independent configurations result in inefficient operations in terms of both latency and power saving gain.

In Table 1, preliminary evaluation results for joint power saving are provided. In the baseline scheme, the cell utilizes semi-static cell DTX scheme with a sleep duration of 40 ms and ON duration of 80 ms. In addition, UE utilizes C-DRX by monitoring PDCCH during DTX ON duration wherein, the C-DRX cycle is aligned with the cell DTX cycle. UE that needs to be served by a sleeping cell will have to wait for the cell until ON duration of the cell.

Compared to the cell DTX scheme, we additionally consider dynamic cell DTX and UE C-DRX schemes. Under these schemes, the cell can dynamically enter a sleep state if cell load falls below a particular (low) threshold and will remain in this state until arrival of a packet for the UE it serves. A cell that enters the sleep state will dynamically deactivate PDCCH monitoring for served UEs to reduce energy consumption at the UE. When a packet arrives for the UE, the cell will be activated (and wake up), following which the UEs will be activated and will resume PDCCH monitoring. Based on the assumption, the following two dynamic schemes with different timing of cell and UE activation were considered to evaluate benefits from joint activation of UE and NW.

Dynamic DTX/C-DRX scheme 1 (Separate activation): Arrival of data at the cell triggers activation and wakes up the cell. The cell then activates and wakes up UE.

Dynamic DTX/C-DRX scheme 2 (Joint activation): Arrival of data at the cell triggers simultaneous activation of cell and UE, followed by simultaneous wake-up of both cell and UE.

|  | UPT gain/loss (vs Baseline) (%) |  | NW side ESG (vs Baseline) (%) | UE side ESG (vs Baseline) (%) |
| --- | --- | --- | --- | --- |
|  | Mean | 5%-ile |  |  |
| Scheme 1 (Separate activation) | 13.23% | 54.61% | 59.73% | 21.32% |
| Scheme 2 (Joint activation) | 32.02% | 74.93% | 74.13% | 21.04% |

Table 1: Evaluation Results for Joint Activation of UE and NW (with low traffic load)

The evaluation results confirm that joint activation of dynamic cell DTX and UE C-DRX provides substantial benefits over both the baseline semi-static scheme and the separate activation scheme. In Scheme 1 (separate activation), additional latency from separate activation limits throughput improvement. By contrast, joint activation enables simultaneous activation of the cell and the UE upon data arrival, thereby minimizing activation latency. This reduction in latency directly translates into significant UPT gain compared to both baseline and separate activation. Furthermore, joint activation provides notably higher network-side energy saving gains than both baseline and separate activation, while maintaining comparable UE-side savings. These results highlight that synchronizing activation at both UE and network sides is a critical enabler for maximizing performance and energy efficiency.

Observation 3: While separate energy saving of NW and UE achieves gains in UPT and energy saving in both NW and UE, the gain is limited due to additional latency from separate activation.

Observation 4: Joint energy saving between NW and UE achieves significant gains in both UPT and NW energy saving while maintaining comparable UE side savings.

Proposal 1: Support joint energy saving which minimizes activation latency in both UE and NW.

Therefore, it is beneficial to support a joint UE-NW energy saving scheme in which NW-side and UE-side energy saving techniques are configured, activated and deactivated in a coordinated manner. Such joint operation may not imply a single configuration entity for UE and NW, but rather a UE configuration that inherently allows NW to align multiples UEs and its own operation consistently. For example, multiple UE-side energy saving configurations can be adaptively applied in accordance with the corresponding NW-side energy saving operation. A NW-side or UE-side energy saving configuration can include radio access parameters based on different levels of processing capability, energy saving and capacity/coverage performance. So that different configurations can be flexibly adapted with minimum latency (e.g. from active mode to non-active mode and vice versa) based on NW energy saving operation as well as available network resources to realize a good balance in terms of energy savings and performance.

Proposal 2: Support joint power saving in which UE energy saving configuration is aligned with network energy saving operation.

Proposal 3: Support fast adaptation of UE configuration based on NW energy saving operation and available network resources.

## NW energy saving

Targeted to minimize always-on signals, 5G NR techniques in time and frequency domains including adapting SSB/SIB1 periodicities, skipping SSB/SIB1 transmissions, using simplified SSBs and SSB-less SCell demonstrate significant network energy savings [4]. Similar principle should thus be applied to 6GR to achieve further energy savings. For example, increasing the (default) periodicity from 20ms to higher values (e.g., 160ms) and limiting always-on signals to a subset of carriers (e.g. SSB-less carriers) are preferable given low complexity and high potential for energy savings.

Proposal 4: Minimize the need to configure always-on signals (e.g., only present with long periodicities of at least 160 ms). Support configurations that do not require provisioning of always-on signals on all carriers.

On the other hand, from the UE perspective, it is important to mitigate impacts by sparser SSBs, e.g., increased UE cell search complexity, prolonged access delay and battery consumption in IDLE mode and the potentially degraded performance of mobility/RRM measurements, T/F sync and QCL source update, RLM and BFD/BFR operation in CONN mode.

To achieve a good balance between energy savings, performance and flexibility in various scenarios, we think two types of SSBs with different transmission patterns can be considered for 6GR. For example, an always-on (AO) Type A SSB can consist primarily of sync signals (e.g. PSS/SSS without PBCH) and another on-demand (OD) Type B SSB can include PBCH for cell access (e.g. PSS/SSS/PBCH or PBCH only).

In 5G, the applicability for OD-SSB is limited to only SCell in CONN mode. It is thus important in 6GR to enhance and expand on-demand framework by considering different on-demand mechanisms including NW configurable/indicated, dynamic (de)activation and UE requested. It can be further studied to apply a combination of the two types of SSB with always-on and on-demand transmission in a wide range of scenarios/deployments including, e.g. IDLE mode, and PCell and CONN modes.

Proposal 5: Support on-demand signals/channels (e.g., configurable, dynamic (de)activation, and/or UE-requested) where it is applicable.

Proposal 6: Support two types of SSBs that are transmitted as always-on signals and as on-demand signals.

In 5G NR, the energy saving gains achievable with on-demand SIB1 (OD-SIB1) in IDLE mode are rather limited due to the presence of always-on SSBs transmitted at 20ms periodicity. Furthermore, the applicability of OD-SIB1 in 5G is limited to the NES cell (SIB1-less cell). In these scenarios, there is very limited opportunity for NW to enter longer energy saving durations. In 6GR, it can be beneficial to consider mechanisms that enable aligning the OD-SIB1 transmissions with OD-SSB to provide further energy savings.

In addition, the consideration can include extension of OD-SIB1 to standalone cell deployment. In 5G NR, the UE is provisioned by another cell (e.g. Cell A) with the UL-WUS resources for requesting OD-SIB1 on the NES cell. For a stand-alone cell, UL-WUS configuration (e.g., resources) can be provided in SSB (e.g. PBCH) transmitted on a carrier within the stand-alone cell. In the latter case, we think a simplified UL-WUS configuration and an efficient SSB indication mechanism can be studied to reduce signaling overhead.

Proposal 7: Support on-demand SIB1 in standalone cell deployments

Time domain adaptation techniques were introduced in the late releases of 5G, however such techniques are supported only in limited scenarios along with the restriction of not impacting legacy UEs. For example, adaptation of SSB periodicity is applicable only in SCell in CONN mode and cell DTX/DRX applies only for UE-specific signals/channels such as PDCCH and PDSCH.

For realizing higher energy savings for 6GR Day-1, time domain adaptation techniques for common signals/channels such as SSBs and PRACH resources can be extended to scenarios in both CONN and IDLE mode without legacy UE restrictions. Such time domain adaptations can include adapting any of the parameters of the signals/channels including periodicity and start offset, for example. Additionally, from an efficiency perspective, it can be beneficial to extend the adaptation framework to UE dedicated signals/channels such as PUCCH/SR resources.

Proposal 8: Support time-domain adaptation (e.g., periodicity and/or offset, similar to cell-DTX/DRX) of common/dedicated channel/signal.

In addition to SSB and SIB1 adaptation, PRACH resources represent a significant opportunity for network energy saving. In NR, PRACH resources (occasions and preambles) are typically configured semi-statically and repeated with fixed periodicities, independent of actual traffic load or UE access demand. This leads to two inefficiencies:

- At NW side, NW must keep baseband receivers and correlators active to continuously monitor all configured PRACH opportunities, even during long idle periods with no access attempts.

- At the UE side, UEs may transmit preambles in opportunities that are provisioned but not truly available under NW low-power states, resulting in failed access attempts and wasted power.

For 6GR, RACH adaptation should therefore be introduced to align PRACH resource availability with network energy saving states (NES states) and UE activity patterns. Concretely, the following principles can be applied:

- State-dependent PRACH resources: In 6GR, it is desirable to align PRACH resources with the network’s availability states. In deep-sleep operation, for instance, most PRACH occasions could be deactivated, with only a minimal set remaining active. Such state-dependent adaptation allows the network to reduce continuous PRACH monitoring overhead while still retaining essential access capability when required.

- On-demand PRACH activation: If the UE has uplink data or mobility events but no PRACH resources are available in the current state, it may send a lightweight wake-up request (WUR) on reserved UL resources (e.g., PUCCH, PRACH mini-occasion, or UL-WUS). Upon reception, NW switches its availability state and reactivates PRACH resources.

- Flexible PRACH granularity: The number of RACH occasions, subcarriers, and repetitions can be dynamically scaled. For example, fewer PRACH opportunities are kept in micro-sleep, while full density is restored in active states.

- Integration with other NES features: PRACH adaptation should be coordinated with DRX/DTX cycles and OD-SSB/OD-SIB1 signaling so that idle UEs know when the cell is not accepting random access.

Proposal 9: Support PRACH resource adaptation considering Network Energy State, On-demand activation, flexible granularity and integration with other NES features.

In 6GR, it is important to enable fast and scalable designs at NW to dynamically turn on and off certain hardware components, e.g. according to load conditions. Such designs can translate into enablers for supporting flexible adaptations across multiple domains including time, spatial/TRP and frequency domains for achieving high energy savings while minimizing any impacts on UE performance. It is also important to efficiently indicate the available/active resources in DL and UL (e.g. CSI-RS, PUCCH, PRACH) associated with the adaptations in different domains without having to resort to separate configurations or signaling as done in 5G.

For 6GR, mechanisms for adapting the UE-side configurations and/or the assumptions regarding available/active resources based on NW indication can be useful to achieve more balanced UE-NW energy savings. In this regard, efficient mechanisms can be considered for indicating the changes to existing resources and availability of new resources due to the adaptations done at NW.

Proposal 10: Methods for efficient indication of available resources that may be adapted in one or more of: time, spatial/TRP, and frequency domains should be considered for specification support.

In NR, CSI measurement and reporting procedures are typically based on semi-static configurations, such as fixed CSI-RS to PDSCH power offsets and predefined numbers of antenna ports. While effective for backward compatibility, this approach limits energy saving potential in both the network and UEs. When the network dynamically adjusts its transmission parameters, for example, by lowering PDSCH power, switching off a subset of antenna ports, or entering a low-activity state, the CSI reports generated under static assumptions may no longer reflect the actual link conditions. This mismatch not only reduces the usefulness of CSI feedback but also leads to unnecessary UE measurement and reporting overhead.

For 6GR, it is desirable to make CSI measurement and reporting state-aware and on-demand, aligned with network energy saving operation. Several aspects are key:

- Dynamic CSI-RS to PDSCH power offset adaptation: The network should be able to indicate adjusted offsets, either directly or relative to semi-static values, so that the UE computes CSI feedback based on the actual transmit power being used. This avoids overestimation of channel quality when the network reduces PDSCH power for energy savings.

- Antenna port adaptation: When the number of antenna ports is varied dynamically to reduce network energy consumption, CSI measurements should be linked to the currently active set of ports. Efficient signaling of port subsets ensures that UEs measure and report only the relevant resources, lowering their measurement burden.

- On-demand CSI-RS activation: Instead of transmitting CSI-RS periodically regardless of need, CSI-RS resources can be configured as on-demand, triggered either by network signaling or UE request. For example, if UE detects channel degradation, the UE can request activation of specific CSI-RS sets. Additionally, the network may reduce or suspend CSI-RS transmission during stable channel conditions, resuming only when updated reports are required.

- State-dependent CSI reporting: CSI reporting configurations can be adapted to network availability states. For example, during reduced activity, only minimal or long-term CSI reports may be expected, while detailed reporting is triggered when the network returns to active state.

Proposal 11: Support CSI reporting adaptation considering network energy saving by including dynamic power offset adaptation, antenna port adaptation, on-demand CSI-RS activation and state-dependent CSI reporting.

In NR, low power receiver functionality was considered mainly for the UE side, but the network side remains constrained by the need to continuously monitor PRACH, PDCCH, and reference signals. This imposes significant standby power consumption, since NW must keep parts of its RF chain, correlators, and baseband processing active even when there is no traffic demand. The problem is amplified in massive MIMO and multi-panel deployments, where the number of always-on RF paths is large, and in capacity booster cells or hotspot deployments that are heavily loaded only during peak hours but otherwise underutilized.

For 6GR, it is highly desirable to extend low power receiver capability to the network. With NW-side low power receiver, NW can deactivate its main radio and baseband units while maintaining only a lightweight detection front-end to monitor wake-up signals from UEs. This enables the network to remain in low-activity mode for extended periods without sacrificing responsiveness. When a wake-up request is received, NW transitions to its active state, resuming full operation in time to serve UE traffic.

Proposal 12: Support low power receiver at NW side, enabling the network to detect wake-up requests without activating the full radio chain.

While NW-side low power receivers enable deeper energy savings, activation latency remains a critical challenge. Current designs often rely on sequential activation: NW powers on first, then signals the UE, leading to delays and reduced user-perceived throughput. As shown in Section 2.1, sequential activation limits the benefits. By contrast, joint activation, where both NW and UE are activated simultaneously, achieves greater network-side energy saving gain compared to separate activation.

To realize such joint activation, support of a low power transmitter at NW side is desired. This transmitter can operate without powering up the full main radio chain, sending lightweight downlink wake-up signals that are sufficient to activate the UE’s low power receiver. In this way, NW can initiate coordinated activation while still in a low-power state, reducing the need for prolonged standby operation of its full RF/baseband chain.

Proposal 13: Support low power transmitter capability at NW side to provide lightweight downlink wake-up signaling detectable by UE low power receivers.

## UE energy saving

In current NR operation, the offset between the scheduling command on PDCCH and the start of PDSCH reception is flexible, but it is not adapted to the constraints of aggressive power-saving operation. As 6GR moves toward deeper network and device energy saving, it is increasingly important to ensure that UEs in power-saving states have sufficient time to become fully operational before receiving downlink data.

A central factor in determining the appropriate offset is the UE status. When UE is in a deep power-saving state, much of its RF and baseband hardware is turned off to minimize standby consumption. Transitioning from this state requires additional time to re-activate RF chains, restart oscillators, and bring decoding units back online. In such cases, a larger minimum scheduling offset is necessary to ensure that the UE can successfully prepare for PDSCH reception. On the other hand, UE that is already in a high-performance or active state can respond quickly with minimal wake-up overhead, and therefore shorter offsets are sufficient without compromising reliability.

By supporting application of different minimum scheduling offsets that consider the UE’s power-saving state, 6GR can avoid overly conservative one-size-fits-all configurations while still protecting UEs from being scheduled too aggressively. This ensures reliable decoding, prevents unnecessary standby power consumption, and provides a flexible framework for balancing latency, reliability, and energy efficiency.

Proposal 14: Support application of different minimum PDSCH scheduling offsets considering UE status to ensure reliable decoding while minimizing unnecessary standby power consumption.

Scheduling bursty traffic in the uplink in NR is challenging if one targets for high energy efficiency with reasonable resource efficiency. Such traffic may consist of packets or packet groups that are variable in size, inter-packet arrival time and delay bounds. The network can maximize resource efficiency by configuring SR for a UE, providing a first small grant for transmission of a BSR after receiving the SR and then a second grant for transmission of data available for transmission. This comes at the expense of latency and energy efficiency since multiple UL and DL transmissions are required for transmitting even low amount of data and there is a minimum delay between DCI and corresponding UL transmission. The network can improve latency by over-provisioning the dynamic grant after SR reception or configuring one or more configured grant(s) with over-provisioned size. but both approaches degrade resource efficiency and possibly adds network complexity due to increased need of blind decoding. Uplink control information triggered by measurements such as L1 measurements for LTM or beam management also generates bursty transmissions. To meet its high energy efficiency requirements, 6GR needs to support scheduling scheme that targets more efficient support of bursty traffic from uplink data and/or control.

Proposal 15: Support energy-efficient uplink scheduling schemes for bursty transmission of data and control information.

To achieve this goal, one possible starting point is to provide a small resource to the UE for indicating a payload (i.e. TB size and/or UCI size) to the NW when data or UCI becomes available for transmission, from a set of configured payload sizes. Such resource request avoids blind decoding at the network side if configured grants are used. This also enables indicating a dynamic grant with shorter delay to PUSCH since the UE can construct the TB before receiving the indication. Figure 1 shows an example of this.

Figure 1. Resource request with TB size indication

Proposal 16: Support uplink transmission scheme where UE selects and indicates a TB size and/or UCI size to the network prior to transmission.

As elaborated above, network energy saving schemes and UE energy saving schemes are correlated. One representative example would be DTX/DRX operation in network and UE. It would be natural that the UE is not able to receive/transmit signals when it is not in on duration of cell DTX/DRX. Although the reception/transmission is prohibited by the specification, configurations for cell DTX/DRX, UE DTX/DRX and PDCCH skipping indications are fully separate. In 6GR, designing more optimized specification support for cell/UE DTX/DRX operations is preferred. For example, UE DTX/DRX configurations and PDCCH skipping indications can be done on top of cell DTX/DRX configurations, so that the UE can support more flexible indication/activation methods with minimized indication/configuration overheads.

Figure 2. UE DTX/DRX and PDCCH skipping operations 
based on cell DTX/DRX configuration.

Proposal 17: Support UE DTX/DRX operation and PDCCH skipping configurations that can efficiently incorporate and align with cell DTX/DRX operation.

Similar to other UE energy saving schemes, LP-WUS was studied and introduce in the later time of 5G-A (i.e., studied in Rel-18 and introduced in Rel-19). Due to the fact that a large population of 5G-A devices in the network does not have LP-WUS capabilities, the motivation and benefit of implementing OOK/OFDM-based low power receiver are limited. For 6GR, supporting low power receiver as a mandatory feature for all device types may not be practical although it is desirable, however, support of low power receiver in some device types (e.g., eMBB or IoT UEs) can provide significant power saving gains than other UE energy saving schemes. Having said that, support of low power receiver and its associated signaling mechanism should be considered from 6GR Day-1.

Proposal 18: Support low power receiver at UE from 6GR Day-1.

In Rel-19, the LP-WUS design incorporated OOK with overlaid OFDM, reflecting the attempt to combine two complementary approaches for low power receiver operation. The motivation was clear: OOK was attractive because of its extremely low power consumption enabled by envelope detection, while OFDM was included as it aligns with the existing OFDM-based receiver chain at UE side as well as OFDM-based transmitter at NW side, minimizing additional implementation complexity. This dual inclusion was intended to balance energy efficiency with implementation difficulty.

However, evaluations in Rel-19 also showed that OOK-based LP-WUS presents notable coverage limitations. To match the link budget of PUSCH used for Msg3, OOK reception requires a significant increase in overhead. While OOK enables very low-power detection, the additional overhead required to ensure sufficient coverage makes it less suitable for 6GR.

On the other hand, while OFDM-based low power receiver consumes relatively higher energy, it can achieve comparable coverage to Msg3 without requiring excessive overhead. Furthermore, as OFDM-based low power receiver potentially reuses existing receiver components, the additional implementation burden can be minimized for both UE and NW.

Proposal 19: Support OFDM-based low power receiver in 6GR.

The most important principle for achieving the most energy saving gain would be maintaining UE sleep state with low power receiver as much as possible. One potential enhancement would be supporting mobility and cell (re)selection by low power receiver. By supporting mobility and cell (re)selection by low power receiver, the UE can maintain connection with NW without waking up MR and achieve better power saving gain when there’s no traffic.

Another potential enhancement would be supporting low power UL transmission. One of the representative drawbacks of low-power receiver in NR is absence of UL transmitter. As the UE needs to wake up MR for other UE operations than receiving the LP-WUS. For example, the UE in deep sleep needs to wake up MR just for reporting CSI/beam information without any need of other PDSCH reception. Supporting low power transmitters would be a solution to maximize the UE energy saving gain. For transmitter architectures, low power transmitters studied in Ambient IoT can be a starting point.

Proposal 20: Support low power receiver operations for mobility and cell (re)selection.

Proposal 21: Study feasibility and benefits of supporting low power UL transmission to maximize UE energy saving gain with low power receiver.

- Low power transmitters studied in Ambient IoT can be a starting point.

# Summary

In this contribution, we discussed energy efficiency in 6GR for both network and UE. Based on the discussion, we made the following observations and proposals.

Observation 1: In NR, all the energy saving techniques for UE and NW are supported based on separate configurations and individual signaling mechanisms.

Observation 2: Although the power saving techniques for UE and NW are separately supported, all such techniques are correlated. Therefore, independent configurations result in inefficient operations in terms of both latency and power saving gain.

Observation 3: While separate energy saving of NW and UE achieves gains in UPT and energy saving in both NW and UE, the gain is limited due to additional latency from separate activation.

Observation 4: Joint energy saving between NW and UE achieves significant gains in both UPT and NW energy saving while maintaining comparable UE side savings.

Proposal 1: Support joint energy saving which minimizes activation latency in both UE and NW.

Proposal 2: Support joint power saving in which UE energy saving configuration is aligned with network energy saving operation.

Proposal 3: Support fast adaptation of UE configuration based on NW energy saving operation and available network resources.

Proposal 4: Minimize the need to configure always-on signals (e.g., only present with long periodicities of at least 160 ms). Support configurations that do not require provisioning of always-on signals on all carriers.

Proposal 5: Support on-demand signals/channels (e.g., configurable, dynamic (de)activation, and/or UE-requested) where it is applicable.

Proposal 6: Support two types of SSBs that are transmitted as always-on signals and as on-demand signals.

Proposal 7: Support on-demand SIB1 in standalone cell deployments

Proposal 8: Support time-domain adaptation (e.g., periodicity and/or offset, similar to cell-DTX/DRX) of common/dedicated channel/signal.

Proposal 9: Support PRACH resource adaptation considering Network Energy State, On-demand activation, flexible granularity and integration with other NES features.

Proposal 10: Methods for efficient indication of available resources that may be adapted in one or more of: time, spatial/TRP, and frequency domains should be considered for specification support.

Proposal 11: Support CSI reporting adaptation considering network energy saving by including dynamic power offset adaptation, antenna port adaptation, on-demand CSI-RS activation and state-dependent CSI reporting.

Proposal 12: Support low power receiver at NW side, enabling the network to detect wake-up requests without activating the full radio chain.

Proposal 13: Support low power transmitter capability at NW side to provide lightweight downlink wake-up signaling detectable by UE low power receivers.

Proposal 14: Support application of different minimum PDSCH scheduling offsets considering UE status to ensure reliable decoding while minimizing unnecessary standby power consumption.

Proposal 15: Support energy-efficient uplink scheduling schemes for bursty transmission of data and control information.

Proposal 16: Support uplink transmission scheme where UE selects and indicates a TB size and/or UCI size to the network prior to transmission.

Proposal 17: Support UE DTX/DRX operation and PDCCH skipping configurations that can efficiently incorporate and align with cell DTX/DRX operation.

Proposal 18: Support low power receiver at UE side from 6GR Day-1.

Proposal 19: Support OFDM-based low power receiver in 6GR.

Proposal 20: Support low power receiver operations for mobility and cell (re)selection.

Proposal 21: Study feasibility and benefits of supporting low power UL transmission to maximize UE energy saving gain with low power receiver.

- Low power transmitters studied in Ambient IoT can be a starting point.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator),

- TR 38.840, “Study on User Equipment (UE) power saving in NR”,

- TR 38.875, “Study on support of reduced capability NR devices”,

- TR 38.864, “Study on network energy savings for NR”,

- TR 38.869, “Study on low-power wake up signal and receiver for NR”.

# Appendix – Simulation assumptions

| Attributes | Values or assumptions |
| --- | --- |
| Scenario | Urban Macro cell |
| Carrier Frequency | 4 GHz |
| System bandwidth | 20 MHz |
| Layout | Single layer: Hex. Grid 7 site, 3 sectors per site |
| ISD | 500m |
| Duplexing | FDD |
| Numerology | 15 kHz, 14 OFDM symbol slot |
| BS antenna config. | 32 Tx/Rx RUs - (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH = dV = 0.5 λ |
| UE antenna config | 4 Tx/Rx RUs - (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1), dH = dV = 0.5 λ |
| BS Tx Power | 49 dBm |
| UE Tx Power | 23 dBm |
| User distribution and UE speed | 20% Outdoor in cars: 30 km/h, 80% Indoor in houses: 3 km/h |
| Traffic model | 36.889 Table A.1.1. Uplink: We consider a uniform packet arrival rate across all UEs (and cells) with low traffic load. |
| Cell sleeping threshold | RU threshold 10% |
| Activation/wake-up time for scheme 1 (Separate activation) | Cell activation time 1ms followed by a cell wake-up time 6ms UE activation time 1ms followed by a UE wake-up time 4ms |
| Activation/wake-up time for scheme 2 (Joint activation) | Simultaneous activation time 1ms followed by a simultaneous wake-up time 6ms for both cell and UE |