3GPP TSG RAN WG1 #122bis           	                                   R1-2507609

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item: 11.3.2

Source: MediaTek Inc.

Title: 6G frame structure and numerology

Document for: Discussion and decision

- Introduction

In the SID of Rel-20 6G study [1], the objective of frame structure and numerology are listed as below:

(2)  Physical Layer structure for 6GR,

  - …

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - …

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

In RAN1 #122 [2], the following is agreed:

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Conclusion

- Numerologies for sensing is up to sensing agenda discussion.

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

This contribution provides our views of this study (6G frame structure and numerology).

- Discussion on Numerology

2.1 SCS, CBW and FFT Size

In 5G, the maximum CBW (channel bandwidth) for a single carrier is 100 MHz for Frequency Range 1 (FR1) and 400 MHz for Frequency Range 2 (FR2), while the most commonly used SCS (subcarrier spacing) is 15kHz for FR1 FDD, 30kHz for FR1 TDD, and 120kHz for FR2, both with max RB (resource block) number being 273 and max FFT (fast Fourier transform) size being 4096. Also, the most commonly used BW for FR1 and FR2 are both 100MHz in the field.

Observation 1: In 5G, the most commonly used CBW for FR1 and FR2 are both 100MHz in the field.

Observation 2: In 5G, the most commonly used SCS (subcarrier spacing) is 15kHz for FR1 FDD, 30kHz for FR1 TDD, and 120kHz for FR2.

Proposal 1: For a given duplex mode (e.g. FDD, TDD), strive for single numerology (incl. SCS and CP) in a spectrum range in 6G as shown in Figure 1, especially for ~7GHz spectrum range.

Proposal 2: For a given duplex mode (e.g. FDD, TDD), strive for single max CBW at least for ~1GHz-10GHz, as shown in Figure 1.

Proposal 3: Max FFT size is restricted to 8192 points for 6G, as shown in Figure 1.

Figure 1. Table of suggested SCS, CBW and FFT Size for 6G

For the OFDM symbol length duration, as the basic OFDM operation procedure is not intended to be changed in 6G, the definition of OFDM symbol length in 6G can follow 5G NR design, while the extended CP case can be removed as it is rarely used in 5G.

Proposal 4: For efficient MRSS, reuse 5G symbol length, subframe length and radio frame length in 6G.

2.2 Simulation results of BLER versus SNR for different SCS

In this subsection, we provide the simulation results of BLER versus SNR for different SCS. The link level simulation (LLS) configuration is provided in Figure 2 (6 layers) and Figure 3 (8 layers). The simulation results are provided in Figure 4 to 6 (6 layers) and 7 to 9 (8 layers), and the corresponding observations are drawn below.

Figure 2. Link level evaluation assumption for 6 layers (Tx EVM is not considered)

Figure 3. Link level evaluation assumption for 8 layers (Tx EVM is not considered)

Figure 4. BLER vs. SNR for 30ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 6 layers

Figure 5. BLER vs. SNR for 100ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 6 layers

Figure 6. BLER vs. SNR for 300ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 6 layers

Figure 7. BLER vs. SNR for 30ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 8 layers

Figure 8. BLER vs. SNR for 100ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 8 layers

Figure 9. BLER vs. SNR for 300ns delay spread and 3/30/60/120km/hr UE speed in 7/15 GHz carrier frequency and 8 layers

Observation 3: For 7GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread.

Observation 4: For 15GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread.

Proposal 5: For all SCS, only normal CP is supported in 6G.

Proposal 6: For around 7GHz/15GHz spectrum range, further study the feasibility of 30KHz and 60KHz SCS, considering max 1024QAM with max 6 or 8 MIMO layers, UE speed 3/30/60/120 km/hr and 30/100/300 ns delay spread of indoor and outdoor scenarios (30kHz is slightly preferred by us here).

2.3 Support of 400 MHz contiguous bandwidth for around 7GHz

To support contiguous 400 MHz bandwidth for around 7GHz from an implementation perspective, two approaches can be considered:

- Alternative 1 (A1 in Figure 10): single 400 MHz PHY carrier.

- Alternative 2 (A2 in Figure 10): two 200 MHz PHY carriers aggregated into one logical cell (multi-carrier cell).

A1 requires much higher UE complexity and is unlikely to be deployed worldwide given the rarity of 400 MHz contiguous spectrum allocations. By contrast, A2 is more scalable and forward-compatible: it can support contiguous spectra larger than a single-carrier FFT allows (e.g., 400 MHz, 600 MHz) without changing the maximum CBW definition, and it avoids the implementation and cost penalties of an ultra-wide single carrier. Moreover, since the amount of UL traffic is usually lower than DL, a 400 MHz PHY carrier may not be necessary for UL. An asymmetric bandwidth for DL and UL is therefore more reasonable (e.g., 400 MHz for DL and 200 MHz for UL). In A1, a center frequency change may be required when switching between DL and UL, and this may require a longer guard time for the DL/UL switch.

Table 1 compares the pros and cons of the two approaches for contiguous 400 MHz. In summary:

- Scheduling flexibility and signalling/control overhead are similar for both approaches.

- A2 offers higher maximum attainable SNR because each RF path handles a smaller bandwidth than in A1. Larger per-carrier bandwidth tends to increase frequency-response ripple or gain droop, which degrades the overall SNR of the RX chain.

- A1 can achieve lower UE power consumption when a full 400 MHz single carrier is used. A2 can achieve lower power consumption in common scenarios where the bandwidth is ≤200 MHz (e.g., PDCCH-only), because only one of the RF chains needs to be active. The latter scenario is more common and has a greater impact on UE battery life.

- A1 can provide marginally better spectrum utilization due to a smaller total guard band; however, the difference is small (for example, on the order of ~0.36% as shown in Figure 10). To further improve spectrum utilization for A2, RAN4 can evaluate whether the guard band between two contiguous carriers can be reduced (for example, to ≤0.64%, as shown in Figure 11) by relaxing ACLR/SEM requirements.

Figure 10. Two approaches to support contiguous 400 MHz (guard band: 0.46% for A1, 0.82% for A2)

Figure 11. Illustration of guard band reduction for A2 (guard band: ≤0.64%)

Table 1. Comparison between A1 (Single PHY carrier with 400MHz) and A2 (MC cell with 2x200MHz)

Observation 5: A2 (multi-carrier with 2×200 MHz) yields a higher achievable SNR and lower UE power consumption in the critical PDCCH-only scenario. A1 (single 400 MHz PHY carrier) requires wider RF chains, which substantially increases UE implementation complexity and cost. Although A1 may offer marginally better spectrum utilization due to a slightly smaller total guard band (0.46% for A1 vs. 0.82% for A2), the difference is small and can be mitigated by guard-band optimization (e.g., reduced to ≤0.64% for A2).

Observation 6: A2 is a scalable and forward-compatible solution that supports contiguous spectrum >200 MHz (e.g., 400 MHz, 600 MHz) without changing the maximum CBW definition by aggregating additional 200 MHz PHY carriers. This approach eases global deployment across regions with differing contiguous allocations.

Observation 7: Compared with A1, A2 is more flexible in supporting asymmetric bandwidth for DL and UL, and it is possible to maintain the same center frequency when switching between DL and UL.

To support A2, it cannot be fully transparent to the BS (i.e., the UE cannot choose A1 or A2 without informing the BS). For example, for UL transmissions, the DC tone locations differ between A1 and A2 and should be reported to the BS. In addition, to preserve the low PAPR property, a signal cannot be transmitted across two carriers separated by a guard band (if existed) if DFT-s-OFDM is used.

Observation 8: It should not be left to UE implementation to choose A1 or A2 for supporting a contiguous 400 MHz bandwidth. A UE capability report is needed. Otherwise, performance (e.g., signal quality, PAPR) may be degraded at least for UL transmissions.

Proposal 7: RAN1 to study multi-carrier cell solution to support contiguous spectrum >200 MHz for around 7GHz as a scalable option.

Proposal 8: RAN4 to study whether the guard band between contiguous carriers can be reduced (e.g., ≤0.18% or zero) by relaxing ACLR/SEM requirements to improve the spectrum utilization.

- Discussion on Frame Structure

In NR, a slot serves two key roles:

- Logical-unit reference for resource allocation, UE capability, and other features. For example:

- PDCCH monitoring BDs and the limit on non-overlapped #CCEs are defined per slot.

- HARQ-ACK codebook construction is slot-based, where HARQ-ACK PUCCHs within a slot are multiplexed into the same codebook.

- The start of the slot serves as a reference for time-domain allocation.

- Scheduling restriction: Although a physical channel can start anywhere within a slot, it is not allowed to cross the slot boundary. This restriction limits scheduling flexibility for PxSCH/PUCCH. For coverage and latency-bounded traffic, a physical channel may need to cross the slot boundary. In NR, this limitation was partially addressed by mechanisms such as TBoMS for coverage and PUSCH Repetition Type-B for latency-bounded services.

Observation 9: In NR, a slot serves two key roles:

- Logical-unit reference for resource allocation, UE capability, and other features. For example:

- Scheduling restriction

Observation 10: After 5G R15, coverage enhancement proposals in 5G often require scheduling crossing slot boundary, for example, TB processing over multi-slot (TBoMS) and DMRS bundling over multiple slots.

Proposal 9: Study the support of PXSCH across slot boundary to enhance coverage and reduce latency.

In 5G NR, the TDD UL/DL configuration is quite flexible and can be adjusted in symbol level. But only a few of TDD UL/DL configurations are deployed in real network. It’s suggested to remove this unnecessary flexibility in 6G, e.g., something similar to LTE.

Proposal 10: Study whether symbol-level flexibility of TDD UL/DL configurations is needed in 6G, considering the identified use cases.

- Discussion on Duplex

RAN1#122 agreed the following:

Agreement

- Study and identify the lessons learned from NR duplex modes

- On 6GR duplexing study, RAN1 considers at least following duplex types

  - FD-FDD

  - Semi-static TDD

  - gNB semi-static SBFD

  - HD-FDD on UE side

  - Dynamic TDD

- Study whether to consider following duplexing types

  - gNB dynamic SBFD

  - UE SBFD

  - gNB FD

  - Note: Other duplex modes are not precluded

6G will need to operate on paired spectrum and unpaired spectrum. For paired spectrum, FD-FDD operation is expected to be catered for at both network and UE side. In addition, Half-Duplex operation at the UE side should be considered to accommodate for device complexity reduction, potentially depending on the frequency band.

Proposal 11: Target both FD-FDD and HD-FDD operation at UE side for paired bands.

For unpaired spectrum in 6G, two duplexing schemes should be studied: TDD and SBFD. In TDD operation, a semi-static indication of the TDD pattern serves as a baseline. This pattern can be aligned among base stations - whether they belong to the same operator or different operators - resulting in synchronous TDD. Alternatively, the pattern may not be aligned among base stations, leading to asynchronous TDD. Asynchronous TDD poses a risk of inter-gNB and inter-UE Cross-Link Interference (CLI), for which RAN1 should investigate effective CLI mitigation and handling procedures.

Proposal 12: Target semi-static TDD operation in unpaired bands as a baseline, with consideration for CLI mitigation for asynchronous semi-static TDD scenarios.

NR specifications support dynamic TDD from Rel-15 by defining “Flexible” symbols, where the transmission direction is not predetermined. Additionally, Slot Format Indication (SFI) was introduced to enable the network to dynamically change the TDD pattern for “Flexible” symbols. However, due to a lack of use cases and the presence of high co-channel CLI, dynamic TDD was not deployed in 5G networks. CLI in dynamic TDD is typically very high because the DL/UL transmission on specific PRBs of aggressor gNB/UE interferes with the same PRBs of the UL/DL reception at a victim gNB/UE, with no frequency isolation unlike with SBFD.

Observation 11: Due to a lack of use cases and the presence of high co-channel CLI, dynamic TDD was not deployed in 5G networks.

From the UE perspective, “Flexible” symbols introduce complexity for the UE (and specifications) due to collision handling between UL and DL scheduling. Collision handling creates uncertainty in UE processing, where the UE cannot process some channels/signals until the “last minute” because they might be cancelled by another channel. Additionally, cancelling a channel might impact the transmission of other channels (e.g., multiplexing among PUSCHs/PUCCHs across different carriers).

Observation 12: The uncertainty in the transmission direction caused by the overdesign of dynamic TDD in NR led to high implementation complexity for both UE and gNB.

If dynamic TDD to be considered for 6G, less complex forms should be considered, such as allowing the TDD pattern to change by some cell-specific or UE-specific signalling.

Proposal 13: For dynamic TDD study, consider the following:

- The targeted deployment to evaluate the impact of inter-gNB and inter-UE CLI.

- The targeted use-case (e.g., DL/UL ratio change for coverage enhancement) for dynamic TDD.

gNB-SBFD has been specified for NR in R19 and can provide significant benefits in 6G. Compared to TDD, gNB-SBFD offers advantages in terms of UL coverage, throughput, and latency. With the introduction of new “greenfield” bands in 6G, network side-SBFD can be deployed without the issue of co-channel CLI with neighbouring non-SBFD legacy gNBs in the same carrier. However, to achieve this and to ensure optimal SBFD awareness, it is important that SBFD is catered for in the initial 6G basic design. Prior to that, SBFD operation will need to be fully catered for in the 6G study phase.

However, like flexible symbols in dynamic TDD, both transmission directions are valid in SBFD symbols in R19. This led to the same UE complexity and issues as flexible TDD. One approach to address this issue is to provide UE-specific link direction (e.g., via RRC or MAC-CE) to indicate to the UE which direction (UL or DL) are valid for that UE on a given SBFD symbols.

Observation 13: SBFD improves the system performance in unpaired spectrum in terms of latency and UL coverage/throughput, and performance can be maximised in new bands with SBFD-aware equipment.

Proposal 14: Study TDD enhanced with SBFD as a fundamental 6G design component for unpaired bands.

Proposal 15: If network-side SBFD is supported in 6G, link direction should be provided to the UE (half-duplex UEs).

UE-SBFD can further enhance performance compared to network-side SBFD by eliminating the half-duplex trade-off at the UE, allowing simultaneous transport of both downlink and uplink traffic for the same UE. UE-SBFD can be opportunistically enabled in favourable conditions to avoid high self-interference, such as with narrow uplink allocation and low transmission power. Additionally, more dynamic “frequency separation” can be ensured between the uplink and downlink allocations at the UE to reduce self-interference, without impacting system spectral efficiency. Implementing SBFD at both the gNB and UE can significantly improve uplink latency performance and system capacity compared to TDD operation, which is essential for latency-sensitive applications like XR (see Figure 11).

Figure 12. UL performance gains with SBFD: Dense Urban, XR Traffic (10Mbps data-rate, 30ms PDB), FR1/4GHz.

Observation 14: UE-SBFD further improves the capacity for latency-bound services and coverage by eliminating the trade-off between DL and UL transmissions.

Proposal 16: Study UE-side SBFD on top of network-side SBFD for 6G.

- Conclusion

In this contribution, we focus on the discussions of 6G SI on frame structure and numerology, and have the following observations/proposals:

6G numerology related observations/proposals:

Observation 1: In 5G, the most commonly used CBW for FR1 and FR2 are both 100MHz in the field.

Observation 2: In 5G, the most commonly used SCS (subcarrier spacing) is 15kHz for FR1 FDD, 30kHz for FR1 TDD, and 120kHz for FR2.

Proposal 1: For a given duplex mode (e.g. FDD, TDD), strive for single numerology (incl. SCS and CP) in a spectrum range in 6G as shown in Figure 1, especially for ~7GHz spectrum range.

Proposal 2: For a given duplex mode (e.g. FDD, TDD), strive for single max CBW at least for ~1GHz-10GHz, as shown in Figure 1.

Proposal 3: Max FFT size is restricted to 8192 points for 6G, as shown in Figure 1.

Proposal 4: For efficient MRSS, reuse 5G symbol length, subframe length and radio frame length in 6G.

Observation 3: For 7GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread.

Observation 4: For 15GHz carrier frequency and 6/8 layers, 60/120KHz subcarrier spacings (SCS) provide better performance (less than 1 dB gap vs. 30KHz SCS) in a wireless channel with 30ns and 100ns delay spread while only 30KHz SCS are feasible (60/120KHz SCS are NOT feasible for 256/1024QAM with 6/8 MIMO layers) in a wireless channel with 300ns delay spread.

Proposal 5: For all SCS, only normal CP is supported in 6G.

Proposal 6: For around 7GHz/15GHz spectrum range, further study the feasibility of 30KHz and 60KHz SCS, considering max 1024QAM with max 6 or 8 MIMO layers, UE speed 3/30/60/120 km/hr and 30/100/300 ns delay spread of indoor and outdoor scenarios (30kHz is slightly preferred by us here).

Observation 5: A2 (multi-carrier with 2×200 MHz) yields a higher achievable SNR and lower UE power consumption in the critical PDCCH-only scenario. A1 (single 400 MHz PHY carrier) requires wider RF chains, which substantially increases UE implementation complexity and cost. Although A1 may offer marginally better spectrum utilization due to a slightly smaller total guard band (0.46% for A1 vs. 0.82% for A2), the difference is small and can be mitigated by guard-band optimization (e.g., reduced to ≤0.64% for A2).

Observation 6: A2 is a scalable and forward-compatible solution that supports contiguous spectrum >200 MHz (e.g., 400 MHz, 600 MHz) without changing the maximum CBW definition by aggregating additional 200 MHz PHY carriers. This approach eases global deployment across regions with differing contiguous allocations.

Observation 7: Compared with A1, A2 is more flexible in supporting asymmetric bandwidth for DL and UL, and it is possible to maintain the same center frequency when switching between DL and UL.

Observation 8: It should not be left to UE implementation to choose A1 or A2 for supporting a contiguous 400 MHz bandwidth. A UE capability report is needed. Otherwise, performance (e.g., signal quality, PAPR) may be degraded at least for UL transmissions.

Proposal 7: RAN1 to study multi-carrier cell solution to support contiguous spectrum >200 MHz for around 7GHz as a scalable option.

Proposal 8: RAN4 to study whether the guard band between contiguous carriers can be reduced (e.g., below ~0.25% or zero) by relaxing ACLR/SEM requirements to improve the spectrum utilization.

6G frame structure related observations/proposals:

Observation 9: In NR, a slot serves two key roles:

- Logical-unit reference for resource allocation, UE capability, and other features. For example:

- Scheduling restriction

Observation 10: After 5G R15, coverage enhancement proposals in 5G often require scheduling crossing slot boundary, for example, TB processing over multi-slot (TBoMS) and DMRS bundling over multiple slots.

Proposal 9: Study the support of PXSCH across slot boundary to enhance coverage and reduce latency.

Proposal 10: Study whether symbol-level flexibility of TDD UL/DL configurations is needed in 6G, considering the identified use cases.

6G duplex related observations/proposals:

Observation 11: Due to a lack of use cases and the presence of high co-channel CLI, dynamic TDD was not deployed in 5G networks.

Observation 12: The uncertainty in the transmission direction caused by the overdesign of dynamic TDD in NR led to high implementation complexity for both UE and gNB.

Observation 13: SBFD improves the system performance in unpaired spectrum in terms of latency and UL coverage/throughput, and performance can be maximised in new bands with SBFD-aware equipment.

Observation 14: UE-SBFD further improves the capacity for latency-bound services and coverage by eliminating the trade-off between DL and UL transmissions.

Proposal 11: Target both FD-FDD and HD-FDD operation at UE side for paired bands.

Proposal 12: Target semi-static TDD operation in unpaired bands as a baseline, with consideration for CLI mitigation for asynchronous semi-static TDD scenarios.

Proposal 13: For dynamic TDD study, consider the following:

- The targeted deployment to evaluate the impact of inter-gNB and inter-UE CLI.

- The targeted use-case (e.g., DL/UL ratio change for coverage enhancement) for dynamic TDD.

Proposal 14: Study TDD enhanced with SBFD as a fundamental 6G design component for unpaired bands.

Proposal 15: If network-side SBFD is supported in 6G, link direction should be provided to the UE (half-duplex UEs).

Proposal 16: Study UE-side SBFD on top of network-side SBFD for 6G.

- Reference

RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO, RAN #108

Chairman’s Notes, RAN1 #122 (Rel-20 11.3.2 Frame structure)