3GPP TSG RAN WG1 #122bis	R1-2507614

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		8.8

Source:	MediaTek Inc.

Title:	Multi-carrier enhancements for NR Phase 3

Document for:		Discussion and Decision

# Introduction

This contribution highlights a maintenance issue to resolve for MCE phase 3.

# Discussion

The RRC parameters list in [1] for the Rel-19 MC-DCI functionality for extended DCI-0_3 and 1_3 specifies support for configuring an extended TDRA table size and associated multi-PxSCH TDRA per cell configuration, both of which were deemed necessary to support for “Sub-feature 2: Multi-PxSCH within a scheduled cell”.

However, for the UE only supporting “Sub-feature 1: Scheduling of scheduled cells of different SCSs” it seems that these RRC extensions were not identified as necessary to operate the feature and that the R18 RRC parameters could be sufficient for that purpose.

Therefore, it seems ambiguous as to whether a UE supporting just Sub-feature 1 should support the R19 RRC or whether it can expect to be configured with that sub-feature using R18 RRC. Such ambiguity in the specifications would potentially lead to IODT issues in the field if the feature was commercialized, e.g. a UE could assume R18 RRC parameters would be used to configure the feature but network uses R19 RRC parameters which UE doesn’t support, or vice versa. This issue should be resolved.

Proposal: Resolve the ambiguity as to whether the R18 RRC parameters or R19 RRC parameters for MC-DCI shall be used to configure multi-SCS scheduling for a UE not supporting multi-cell multi-PxSCH scheduling. Communicate the outcome to RAN2.

# Conclusion

The following proposal is made.

Proposal: Resolve the ambiguity as to whether the R18 RRC parameters or R19 RRC parameters for MC-DCI shall be used to configure multi-SCS scheduling for a UE that does not also support multi-cell multi-PxSCH scheduling. Communicate the outcome to RAN2.

# References

[1] R1-2503243, Rel-19 higher layers parameters list Post RAN1#121, Ericsson