3GPP TSG RAN WG1#122b                                R1-2507630

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.1.1.1

Source:	Transsion Holdings

Title:	Discussion on inference related aspects of CSI compression

Document for:	Discussion and decision

- Introduction

In RAN#108[1], a new WID for Rel-20 AI/ML evolution was approved[1], with the following objectives to enhance inference related aspects of CSI spatial/frequency compression without temporal aspects:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we discuss enhancements of inference related aspects of CSI spatial/frequency compression without temporal aspects.

- 2 Discussion

- 2.1 Measurement and report configuration

Regarding inference report configuration, the following agreement was achieved in RAN1#122 meeting.

Agreement: For inference report configuration, support separate configuration of subband, Tx port and payload size from pairing ID.

Regarding the spec impacts of CSI report configuration of CSI compression, the signaling to trigger the AI/ML model CSI compression, pairing mechanism between CSI generation model at UE and CSI reconstruction model at Network, and how Network indicates the CSI feedback payload to UE should be specified. In addition, regarding the CSI report related aspects, including the CSI-RS resource configuration, CSI reporting configuration, and CSI mapping, should be specified as well.

Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered:

- Signaling to trigger the AI/ML model CSI compression

- Pairing between CSI generation model and CSI reconstruction model

- Indicate the CSI feedback payload

- CSI-RS resource configuration

- CSI reporting configuration

- 2.2 CQI determination

Regarding CQI determination, two options on CQI calculation were achieved in RAN1#112 meeting.

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Regarding Option 1a, if CQI is calculated based on target CSI with realistic channel measurement, the calculated CQI is overestimated since the CSI reconstruction error is not considered. Regarding Option 1b, if CQI is calculated based on target CSI with realistic channel measurement and potential adjustment, the CQI compensation can be achieved based on the adjustment information and the calculated CQI may be more accurate. Regarding Option 1c, if CQI is calculated based on legacy codebook, the calculated CQI may be underrated due to more channel information loss incurred by legacy codebook. Regarding Option 2a, if CQI is calculated based on CSI reconstruction output, the CQI is calculated based on the assumption that CSI reconstruction model is deployed at UE side. Hence, the model shall be transferred or delivered between two sides. Regarding Option 2b, if CQI is calculated using two stage approach, the calculated CQI may be outdated since two stage approach may result in large delay. Therefore, regarding CQI determination, we support Option 1b.

Proposal 2: Regarding CQI determination, support Option 1b:

- Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

- 2.3 Quantization methods

Regarding quantization method, the following agreements were achieved in RAN1#122 meeting.

Agreement For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment, Ql,ν is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting. - dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific. - If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν. Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: dzl,ν, Ql,ν and L are independent on subband and port configurations, - FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations. Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

Quantization at CSI generation part refers to the process of projecting floating values to binary bits, which is usually the last step in CSI generation model; on the other hand, dequantization at CSI reconstruction part refers to the reverse procedure, which is usually the first step in CSI reconstruction model.

Regarding quantization methods, scalar quantization and vector quantization were studied in Rel-18 and Rel-19. For scalar quantization, each element in the latent vector will be mapped to a new discretized element and the elements of latent vector will be quantized one by one, where each element can be quantized with a few numbers of bits, i.e., the number of bits per latent dimension. Since each element in the latent vector is quantized with a few numbers of bits, the number of bits per latent dimension should be aligned between Network and UE so that the reported bits can be dequantized in Network.

Since the bounded floating value of each element in the latent vector is mapped to a certain level, the the minimum and maximum range of quantization should be aligned between Network and UE.

Proposal 3: Regarding configuration of scalar quantization, the following can be aligned between Network and UE:

- The number of bits per latent dimension

- The minimum and/or maximum range of quantization

For vector quantization, the elements of whole latent vector or the elements of segment of latent vector will be mapped to a pre-defined codeword or vector at a time. If the length of the latent vector is large and vector quantization is based on the elements of whole latent vector, it is practically difficult to design codebook. Hence, vector quantization based on the elements of segment of latent vector is preferable. For vector quantization based on the elements of segment of latent vector, the whole latent vector can be divided into several segments by segment size and the elements of segment of latent vector can be quantized with a few numbers of bits, i.e., the number of bits per segment. Hence, the number of bits per segment and segment size should be aligned between CSI generation part at UE and CSI reconstruction part at Network to achieve a satisfying performance.

Proposal 4: Regarding configuration of vector quantization, the following can be aligned between Network and UE:

- The number of bits per segment

- Segment size

- The size of codebook

Regarding quantization codebook for inter-vendor collaboration Direction A, two alternatives were achieved in the last meeting. Regarding Alt1, the quantization codebook can be exchanged from NW-side to UE-side along with each exchanged dataset or model parameters, where flexible and  dynamic quantization codebook can be provided for quantization and dequantization between gNB and UE. Regarding Alt2, a standardized quantization codebook decided by RAN1 and the fixed quantization codebook can be used for quantization and dequantization between gNB and UE, which is would be the simplest way. Therefore, regarding quantization codebook for inter-vendor collaboration Direction A, we support Alt2.

Proposal 5: Regarding quantization codebook for inter-vendor collaboration Direction A, support Alt2:

- Alt2: Use a standardized quantization codebook decided by RAN1.

For vector quantization, the elements of segment of latent vector will be mapped to a pre-defined codeword or vector at a time and hence it is necessary to design codebook to support vector quantization. Regarding the codebook design, following two alternatives can be considered. Regarding Alt-1, a unified codebook is designed for all the segments of latent vector. By this way, a unified codebook can be shared for different segments of latent vector, where the complexity of codebook design and the elements of segment of latent vector mapping to codeword can be reduced. Regarding Alt-2, a dedicated codebook is designed for each segment of latent vector. By this way, a dedicated codebook can be applied to a dedicated segment, where the codebook may provide more accurate quantization.

Proposal 6: Regarding codebook design of vector quantization, the following alternatives can be considered:

- Alt-1: A unified codebook is designed for all the segments of latent vector

- Alt-2: A dedicated codebook is designed for each segment of latent vector

- 2.4 UCI mapping

In legacy, PMI indicators are mapped to group 0/1/2 for eType II CSI feedback. If PUSCH resource is insufficient, a portion of Part 2 CSI information can be omitted according to the group index and priority value, which is defined based on layer index, SD basis index and FD basis index.

For CSI spatial/frequency compression, since the target CSI is compressed per layer, it is straightforward that compressed target CSI can be omitted per layer. However, compressed target CSI of an entire layer is dropped, the omission granularity is coarse. Thus, in addition to CSI omission per layer, the compressed target CSI can be omitted per segment for vector quantization and per scalar of latent space for scalar quantization.

Proposal 7: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered:

- Alt-1: Layer-based priority rule

- Alt-2: Segment based priority rule for vector quantization

- Alt-3: Scalar of latent space based priority rule for scalar quantization.

In legacy, regarding UCI mapping of Part 2 CSI, group 0/1/2 is defined for eType II CSI feedback, where wideband CSI is included in group 0 and even/odd subband CSI is included in group 1/2. However, for CSI spatial/frequency compression, since the target precoding matrices corresponding to all subbands are jointly compressed into a latent space, wideband and subband information can not derived from the compressed target CSI. Thus, the compressed target CSI can not be grouped based on the wideband and subband information. RAN1 should study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

Proposal 8: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

- 3 Conclusion

In this contribution, we discuss potential enhancements of inference related aspects of CSI spatial/frequency compression without temporal aspects. Based on the discussion above, we provide the following proposals:

Proposal 1: The following potential spec impacts related to CSI report configuration of CSI compression should be considered:

- Signaling to trigger the AI/ML model CSI compression

- Pairing between CSI generation model and CSI reconstruction model

- Indicate the CSI feedback payload

- CSI-RS resource configuration

- CSI reporting configuration

Proposal 2: Regarding CQI determination, support Option 1b:

- Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment

Proposal 3: Regarding configuration of scalar quantization, the following can be aligned between Network and UE:

- The number of bits per latent dimension

- The minimum and/or maximum range of quantization

Proposal 4: Regarding configuration of vector quantization, the following can be aligned between Network and UE:

- The number of bits per segment

- Segment size

- The size of codebook

Proposal 5: Regarding quantization codebook for inter-vendor collaboration Direction A, support Alt2:

- Alt2: Use a standardized quantization codebook decided by RAN1.

Proposal 6: Regarding codebook design of vector quantization, the following alternatives can be considered:

- Alt-1: A unified codebook is designed for all the segments of latent vector

- Alt-2: A dedicated codebook is designed for each segment of latent vector

Proposal 7: Regarding CSI omission in CSI spatial/frequency compression, the following alternatives can be considered:

- Alt-1: Layer-based priority rule

- Alt-2: Segment based priority rule for vector quantization

- Alt-3: Scalar of latent space based priority rule for scalar quantization.

Proposal 8: In CSI spatial/frequency compression, study the necessity of grouping the compressed target CSI and whether the compressed target CSI can be grouped based on layer information.

- 4 Reference

[1]RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, Qualcomm