3GPP TSG RAN WG1 #122-bis	        		   			R1-2507702

# Prague, CZ, October 13th – 17th, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SR triggered SSSG switching [SRTrig_SSSGSwitch] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-xx |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Based on the RAN1’s parameter list (R1-2503243), RAN2 has incorporated required RRC parameters associated with SR-triggered SSSG (Search Space Set Group) switching in TS38.331 (CR 5396) with the following changes: - The srTriggeredSSSG-r19 parameter is removed since SR-triggered SSSG switching can be enabled/disabled based on the presence/absence of the srTriggeredSSSGId-r19 parameter. - The name of srTriggeredSSSGId-r19 is changed to sr-TriggeredSearchSpaceSwitchGroupId to align with the other existing parameters in SearchSpaceSwitchConfig. For alignment, the same changes should be reflected in TS38.213 accordingly. - Clause 10.4 of TS 38.213 specifies the procedure for SR-triggerd SSSG switching, which was introduced in Rel-19 TEI. Currently, this procedure is defined only for the PUCCH-based SR scenario. In contrast, the SR-triggered termination of PDCCH skipping, as specified in Rel-17, provides further elaboration: - Behaviors depending on SR transmission timing relative to the reception of DCI with a PDCCH skipping indication: - If SR comes before the DCI, any PDCCH skipping indication received while the SR is pending is ignored. - If SR comes after the DCI, ongoing PDCCH skipping is terminated. - Behaviors for both PUCCH-based and RACH-based SR transmissions. Theregore, to ensure consistency between the two related features (PDCCH skipping and SSSG switching), it is recommended that similar levels of detail be incorporated into the description of the SR-triggered SSSG switching procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 1: - The name of pdcch-SSSGIndexAfterSR parameter is changed to sr-TriggeredSearchSpaceSwitchGroupId - The SSSGIndexSwitchAfterSR parameter is removed For 2, Additional details on SR-triggered SSSG switching are provided: - Behaviors depending on SR transmission timing relative to the reception of DCI with an SSSG switching indication. - Behaviors for the RACH-based SR transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for SR triggered SSSG switching is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.331 CR 5396 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *** Unchanged parts are omitted ***

## 10.4	Search space set group switching and skipping of PDCCH monitoring

## *** Unchanged parts are omitted ***

When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication

-	at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3,

-	at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6

When a UE is provided a group index by sr-TriggeredSearchSpaceSwitchGroupId

-	If the UE transmits a PUCCH providing a positive scheduling request before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to start PDCCH monitoring according to search space sets with a group index different from the one provided by sr-TriggeredSearchSpaceSwitchGroupId, the UE shall monitor PDCCH according to search space sets with group index provided by sr-TriggeredSearchSpaceSwitchGroupId from the beginning of a first slot that is not earlier than Pswitch symbols after a last symbol of the PUCCH transmission, regardless of search space set group switching indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321].

-	if the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to start PDCCH monitoring according to search space sets with a group index different from the one provided by sr-TriggeredSearchSpaceSwitchGroupId, the UE shall monitor PDCCH according to search space sets with group index provided by sr-TriggeredSearchSpaceSwitchGroupId from the beginning of a first slot that is not earlier than Pswitch symbols after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group.

-	if the UE transmits a RACH due to positive SR, the UE shall monitor PDCCH according to search space sets with group index provided by sr-TriggeredSearchSpaceSwitchGroupId on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running.

Pswitch

When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field.

-	If the UE transmits a PUCCH providing a positive SR before the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE shall monitor PDCCH regardless of PDCCH skipping indication on all serving cells of the corresponding Cell Group when the SR is pending [11, TS 38.321].

-	If the UE transmits a PUCCH providing a positive SR after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, the UE resumes PDCCH monitoring starting at the beginning of a first slot that is after a last symbol of the PUCCH transmission in all serving cells of the corresponding Cell Group.

-	When the UE is provided pdcch-MonitoringResumptionAfterNack, after the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of the serving cell, if the UE transmits a PUCCH or a PUSCH providing a NACK value associated with a PDSCH reception that is scheduled by a DCI format in a PDCCH reception on the serving cell, the UE terminates PDCCH skipping, starting from the beginning of a first slot that is after a last symbol of the PUCCH or PUSCH transmission on the serving cell.

-	During the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running, the UE shall not skip PDCCH monitoring on SpCell.

-	After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a SpCell, when contention resolution is successful [11, TS 38.321], the UE resumes PDCCH monitoring on the SpCell.

-	After the UE detects a DCI format providing the PDCCH monitoring adaptation field indicating to the UE to skip PDCCH monitoring for the duration on the active DL BWP of a serving cell, when a pending SR is cancelled [11, TS 38.321], the UE resumes PDCCH monitoring in all serving cells of the corresponding Cell Group.

-	If UE transmits a RACH due to positive SR, the UE shall not skip PDCCH monitoring on any serving cell of the corresponding Cell Group during the time of ra-ResponseWindow or msgB-ResponseWindow or the duration where ra-ContentionResolutionTimer is running. If DRX is configured and the DRX group of the serving cell enters outside Active Time, the UE terminates PDCCH skipping for the serving cell.

## *** Unchanged parts are omitted ***