3GPP TSG RAN WG1 #122-bis	R1-2507733

Prague, Czech Republic, Oct. 13th – 17th, 2025

Agenda Item:	10.1.1.1

Source:		Toyota ITC

Title:		Inference Related Aspects of AI/ML for CSI Spatial/Frequency Compression         Without Temporal Aspects (“Case 0”)

Document for:	Discussion and Decision

# Introduction

In RAN#108 meeting, the Rel-20 NR_AIML_air_Ph2 WID [1] was approved. There are a few objectives for RAN 1 in the new WID. Among these RAN 1 objectives, the ones for CSI feedback enhancement, with the inference related aspects are highlighted in the excerpt below.

Provide specification support for the following aspects: …… CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring ……

At the previous RAN 1 #122 meeting, some of the topics were covered and a set of initial agreements is achieved [2]. In this contribution, the remaining open items are discussed.

# Specification of Inference Related Aspects

## CSI Type of CSI Feedback

At the previous RAN 1 #122 meeting, the starting point of the discussion on inference related aspects was the type of CSI to be reported [3]. On that topic, the following agreement was made [2].

Agreement For CSI feedback via two-sided model, support at least precoding matrix as target CSI type. - FFS whether consider channel matrix as further improvement, starting with evaluations, e.g. potential fusion with SRS measurement (SRS period/hopping, DL/UL reciprocity), frequency granularity of channel matrix, number of Rx antennas, etc.

With regards to the use of channel matrix as a further improvement for the CSI type to be reported, this statement is justified by some simulation results during the study item phase. Also, reporting the entire channel matrix as a form of CSI feedback is also mentioned by a few companies in the context of 6GR. Within the scope of NR Release 20, reporting of the channel matrix as CSI type of CSI feedback is proposed as a complement to SRS, with the goal to obtain timely high granularity channel knowledge both in downlink and uplink. This implies that at least the system is TDD and that there is UL-DL reciprocity. In addition, other channel and system conditions must be met to see gain from the use of channel matrix as CSI type feedback:

- the bandwidth over which CSI RSs are configured must be very large, e.g. 100 MHz,

- the period of the SRS hopping pattern, should be quite large, i.e. in the order of dozens of msec,

- the channel must have high frequency selectivity, i.e. the frequency selectivity must be much smaller than the subband size, for the CSI reporting per subband to be coarse

- the load in the cell, either in DL, or in UL, or in both must be high enough to justify the need for the timely and fine granularity channel matrix knowledge at the scheduler.

In addition, the introduction of channel matrix as CSI feedback, would require significant standardization effort; namely, when considering the granularity and quantization levels of the channel matrix, independently of the channel matrix reporting being done per antenna port and subbands, or, per beam.

In this case, the performance evaluation of the scheme with channel matrix as CSI feedback type must consider the signaling overhead.

Observation 1: For 5G NR Release 20, the use of channel matrix as CSI feedback type provides gains in system throughput only within a set of scenarios. The additional signaling overhead must be considered in the evaluations of the feature. The standardization effort also must be considered.

Proposal 1: For 5G NR Release 20, consider the channel matrix as CSI feedback type, if the gain in system/user throughput is observed in most of the typical 5G NR deployments. Performance evaluations should consider the extra signaling overhead.

## CSI Feedback Type: Precoding Matrix

For the CSI feedback type via two-sided model, at least precoding matrix is selected [2].

### Precoding Matrix Format

The basic format of the precoding matrix as CSI feedback was specified at the previous meeting [2].

Agreement Specify the precoding matrix feedback via two-sided model as follows: For a certain layer l and rank ν, - A precoding matrix is mapped with a latent message zl=z0l,z1l,⋯,zdzl,ν-1l, with l = 1, ..., v where l is the layer index. - The mapping is subject to [at least] a pairing ID configured by high-layer signalling. - FFS dzl,ν is common or specific for different layers or ranks - Note: the terminology “latent message” is used for discussion purposes, the detailed name can be discussed further - zl is quantized and mapped to bit sequence before reporting.

The precoding matrix format, if the latent message zl=z0l,z1l,⋯,zdzl,ν-1l for a certain layer is a vector of complex values znl, reported per subband, sb, and per layer l.

If the precoding matrix format is similar to the Release 16 CSI eType II Codebook, or a format close to it, then the reporting of the precoding matrix for a certain layer l, and subband, sb, in the form of a latent message zl is a vector of size equal to the antenna ports; hence, dzl,ν is equal to the number of antenna ports.

If the precoding matrix for CSI compression is reported in the beam-domain, i.e. in the angular and delay domain, then, the value of dzl,ν corresponds to the number of reported beams per layer.

Independently of the domain in which the latent message zl will be reported, the decision to be made is if the value of dzl,ν is common for different layers for a certain rank, or, if it is layer specific. For both cases of latent message reporting, i.e. per port reporting, or reporting in the beam-domain, there is a trade-off:

- if the dzl,ν is common for all layers, then, there might be some higher uplink signaling in cases where the number of antenna ports is significantly higher than the number of beams per layer,

- if the dzl,ν is layer-specific, then, the UE needs to indicate per layer, the value of  dzl,ν, which results into addition bits for uplink signaling.

To perform this trade-off evaluation, the following needs to be clarified first:

- precoding matrix reporting per port, or reporting in the beam-domain,

- maximum number of antenna ports,

- maximum rank

Observation 2: Decision on whether the value dzl,ν in the latent message zl is layer specific, or common to all layers depends on:

- whether the precoding matrix reporting is done per port, or in the beam-domain

- the maximum number of antenna ports

- the maximum rank.

If dzl,ν is common to all layers and the latent message reporting zl is done per antenna port, then, it is to be decided whether the value of dzl,ν is the same for all ranks, or, if dzl,ν depends on the rank.

Proposal 2: Specify

- the maximum number of antenna ports

- maximum rank

- reporting per antenna port, or reporting in the beam-domain

of the precoding matrix, prior to deciding whether dzl,ν is common, or specific for different layers, or ranks.

### Precoding Matrix Quantization

With regards to the CSI feedback (precoding matrix) quantization, a set of agreements were made at the previous RAN 1 #122 meeting [2].

Agreement For precoding matrix feedback, for a certain layer l of rank ν, the latent message zl contains dzl,ν real values - If scalar quantization (SQ) supported, it refers to the quantization of each real value independently, - The payload size for layer l and rank ν is determined by dzl,ν×Ql,ν, where Ql,ν is the number of bits per scalar. - If vector quantization (VQ) is supported, it refers to the quantization of a segment of L (L>1) consecutive real values jointly, and the segment(s) are separately quantized. - The payload size for layer l and rank ν is determined by dzl,νL×Ql,ν, where L is the length of the vector segment, Ql,ν is the number of bits per segment - Note: Combining two real-values as a complex value and performing amplitude-phase quantization is precluded. Agreement: For precoding matrix feedback, consider the following aspects for payload determination - Support only SQ, or only VQ or both - dzl,ν and / or Ql,ν and / or L are fixed values or determined from multiple values based on NW configuration or UE reporting. - dzl,νand / or Ql,ν is layer-common or layer-specific, rank-common or rank-specific. - If configured, dzl,νand / or Ql,ν and / or L above are configured separately from pairing ID, or configured by Pairing ID. - The configuration may be via other parameters which are used to derive dzl,ν, L and Ql,ν. Note: Consider limiting the total number of possible payload sizes considering the inter-vendor training challenges. Note: dzl,ν, Ql,ν and L are independent on subband and port configurations, - FFS each combination of dzl,ν, Ql,ν and L is applicable to a certain set of subband and port configurations. Agreement: For quantization codebook used for quantizing the feedback, - For inter-vendor collaboration Direction A, select one of the following: - Alt1: Exchange quantization codebook from NW-side to UE-side along with each exchanged dataset or model parameters - Alt2: Use a standardized quantization codebook decided by RAN1.

With regards to quantization, both options of scalar quantization (SQ) and vector quantization (VQ) are possible. Also, the quantization level, Ql,ν, for a certain layer, l, and rank v can be either layer specific, or rank specific, or rank common. For the VQ case, there is an additional parameter, L, which defines the segment size, where segment is the set of grouped values within the latent message zl.

With the goal of the WI in mind, the specification of the parameters dzl,ν, Ql,ν and L should result in the lowest amount of signaling overhead both in downlink and uplink. Therefore, the number of options and configurations for the latent message quantization should be minimized to the necessary and meaningful options. Taking this into account and having in mind the uncertainty on the maximum number of antenna ports and on the maximum rank, and considering that a new parameter, L, must be specified and signaled, the benefit of VQ is questionable.

Observation 3: The benefit of vector quantization is questionable with the current assumptions.

Proposal 3: Support scalar quantization for the latent message zl.

The quantization level, Ql,ν, is tightly linked with the number of beams that can be supported by the channel and that can be transmitted by the gNB. Therefore, Ql,ν, should be rank specific. For a certain channel rank, different quantization levels for different layers does not seem to provide significant gains in signaling overhead.

Proposal 4: The quantization level, Ql,ν should be rank specific and common for all layers for a certain rank.

With the above in mind, the quantization codebook should be defined in a way that some flexibility is provided, without resulting into high signaling overhead, either in downlink, or in uplink.

Therefore, exchanging quantization codebook from NW-side to UE-side along with each exchanged dataset, or model parameters of a restricted size should be supported.

Proposal 5: For inter-vendor collaboration Direction A, support the exchange of quantization codebook from NW-side to UE-side along with each exchanged dataset, or model parameters of a restricted size.

# Conclusion

In this contribution inference related topics of the AI/ML based CSI compression in frequency domain were discussed. The following observations are made:

Observation 1: For 5G NR Release 20, the use of channel matrix as CSI feedback type provides gains in system throughput only within a set of scenarios. The additional signaling overhead must be considered in the evaluations of the feature. The standardization effort also must be considered.

Observation 2: Decision on whether the value dzl,ν in the latent message zl is layer specific, or common to all layers depends on:

- whether the precoding matrix reporting is done per port, or in the beam-domain

- the maximum number of antenna ports

- maximum rank.

Observation 3: The benefit of vector quantization is questionable with the current assumptions.

Therefore, we make the following proposals:

Proposal 1: For 5G NR Release 20, consider the channel matrix as CSI feedback type, if the gain in system/user throughput is observed in most of the typical 5G NR deployments. Performance evaluations should consider the extra signaling overhead.

Proposal 2: Specify

- the maximum number of antenna ports

- maximum rank

- reporting per antenna port or in the beam-domain

of the precoding matrix, prior to deciding whether dzl,ν is common, or specific for different layers, or ranks.

Proposal 3: Support scalar quantization for the latent message zl.

Proposal 4: The quantization level, Ql,ν should be rank specific and common for all layers for a certain rank.

Proposal 5: For inter-vendor collaboration Direction A, support the exchange of quantization codebook from NW-side to UE-side along with each exchanged dataset, or model parameters of a restricted size.

# Reference

- RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements”, Qualcomm, RAN#108, May 2025.

- R1-2506702, “Final Report of 3GPP TSG RAN 1 WG 1 #122 v1.0.0”, Chairman Notes, RAN1#122, August 2025.

- R1-2506495, “FL summary #1 on AIML inference related aspects”,  Moderator (Qualcomm), RAN1#122, August 2025.