3GPP TSG RAN WG1 #122bis	             			            		             		 R1-2507746

Prague, Czech Republic, Oct 13th –17th, 2025

Agenda Item:	11.2

Source:	AT&T

Title:	Evaluation Assumptions for 6GR Air Interface

Document for:	Discussion/Decision

# Introduction

The study on 6G Radio was agreed in RAN#108 in RP-252912 [1] aiming “to develop one non-backward compatible radio access technology to meet a broad range of use cases, and requirements as defined during the RAN requirements study captured in TR38.914 [2]. The study should strive at dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, focusing on practical user experience. The study should identify principles to ensure extensibility and deliver superior performance.”

To evaluate the performance of the technology components, functionalities, use cases and services, it is important that the evaluation methodologies used be grounded in realistic assumptions that reflect the operators’ needs and expectations for 6GR features and services deployments.

In this contribution, we discuss evaluation scenarios and methodology, traffic models, and evaluation metrics that are important to be captured for evaluation of multiple technology proposals for 6GR.

# Deployment Scenarios and Antenna Modeling

The following agreement was made in RAN1#122 on the collection of evaluation assumptions for deployment scenarios and antenna modeling.

Agreement

- The deployment scenarios in TR38.914 should be considered for evaluation assumption

- The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2

  - Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda.

  - Note: Subcarrier spacing decision is up to AI 11.3.2.

To this end, the session moderator created an excel sheet to capture the evaluation assumptions of different companies post RAN1#122 in [3] following the agreements in RAN#109 [2].

Conclusion

- Template in R1-2506582 is to be used for collecting inputs from companies.

  - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

Our preferred assumptions on antenna modeling and deployment scenarios are captured in [3].

In addition to baseline assumptions on antenna modelling for various carrier frequencies (700MHz, 2GHz, 4GHz, 7GHz, 30GHz), and deployment scenarios, we emphasize the importance of considering realistic assumptions in carrying out all evaluations in 6GR air interface design. This includes for example realistic assumptions on interference modeling, receiver assumptions, as well as traffic models.

Proposal 1: For 6GR evaluations, realistic assumptions are used for interference modeling, channel estimation, and traffic modelling.

Another important aspect that needs to be considered in 6GR interface evaluations is energy efficiency (EE). It is expected that 6GR interface should be energy efficient by design starting from 6GR day 1, rather than having EE solutions as add-ons to the 6GR interface design. Moreover, EE continues to be paramount for both network and UE sides and should be considered a single objective mutually benefiting both sides in a common framework, rather than a conflicting set of objectives across both sides. With the emergence of Extra-Large (XL) MIMO, and the corresponding increase in RF chains, etc., EE becomes even more instrumental in 6GR interface, including transmit power as well as power consumed to operate the transmitter and/or receiver. In order to ensure that EE is an integral part of the 6GR design, metrics that take system throughput and EE jointly into account need to be considered for most 6GR interface evaluations.

Proposal 2: For 6GR evaluations, include Energy Efficiency metric(s) as key performance metrics from day 1.

# Traffic Models

Legacy non-full-buffer FTP traffic models commonly used in RAN1 simulations are increasingly misaligned with the contemporary service behaviors. Single-file transfers with simplistic inter-arrival/” reading time” abstractions do not generally represent today’s interactive, session based, and transport-aware applications. The baseline set of traffic models in 5G should thus be revisited for 6G evaluations.

In RAN1#122, the following agreement was made on traffic models for 6GR evaluations:

Agreement

- Study which of the following traffic models are to be used for 6G evaluations, e.g.,

  - Full buffer

  - FTP Model 1 (in TR 36.814)

  - FTP Model 2 (in TR 36.814)

  - FTP Model 3 (in TR 36.872)

  - XR Traffic models (in TR 38.838)

  - VoIP model (as in TR 36.814)

  - Instant message (as in TR 38.840)

- Study whether to introduce the following traffic models for 6G evaluations considering, e.g.,

  - FTP-3 variant with packet delay budget requirement

    - Details FFS

  - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget)

    - Details FFS

  - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc.

    - Details FFS

- Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency)

Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

We first present our views on the existing traffic models and their applicability for evaluation scenarios.

Full buffer traffic, while not realistic, remains valuable as a system stress test: it exposes peak spectral efficiency and provides a clean environment to assess PHY and MIMO features without application-layer confounders. Its stable queues and continuous scheduling are useful for calibrating link adaptation, DL CSI enhancements, MU-MIMO, and validating channel-model assumptions, and it preserves backward compatibility across releases. However, results based solely on full buffer can be misleading.

Observation 1: Full buffer traffic is non-representative of user behavior but is important for stress testing, peak spectral efficiency measurement, PHY/MAC calibration, and maintaining backward compatibility across releases.

For bursty traffic, RAN1 simulations has relied on FTP Models 1/2/3 and XR. FTP Model 1 and much of FTP Model 2 no longer capture the dynamics of modern applications, whereas FTP Model 3 better reflects per-UE burst arrivals and can be configured to mimic instant messaging. FTP Model 2 remains helpful as a proxy for web browsing through its “reading time” (user think time) and for regression against LTE/NR until a dedicated HTTP model exists.

Observation 2: Legacy FTP models vary in realism. FTP Model 1 is overly simplistic. FTP Model 3 captures per UE burstiness and can mimic instant messaging. FTP Model 2 is less representative than FTP3 but remains useful as a web browsing proxy (via think time) and for regression against LTE/NR where no dedicated HTTP model exists.

VoIP continues to be relevant for conversational services. XR traffic is also broadly useful beyond XR applications as a latency/jitter sensitive anchor with tight UL control loops, enabling evaluations of HARQ timing and K1 selection.

Observation 3: VoIP remains a relevant workload for conversational real-time services.

Observation 4: XR traffic provides a latency/jitter sensitive anchor and reflects UL heavy control loops, enabling study of HARQ timing, K1 selection, and related behaviors.

Based on these observations, we propose to reuse full buffer, FTP model 2, FTP model 3, VoIP and XR traffic for 6G evaluations, when applicable.

Proposal 3: Full Buffer, FTP Model 2, FTP Model 3, VoIP and XR traffic models are reused for 6G evaluations

Proposal 4: FTP Model 3 is considered as the main bursty traffic model for general evaluations. FTP Model 2 is only considered as a web browsing proxy and for regression until a dedicated HTTP model is available.

Proposal 5: When applicable, full buffer traffic model can be used in addition to a bursty traffic model for performance evaluations.

Note: Full buffer traffic model is not exclusively used to make observations/decisions on performance evaluations

Proposal 6: The XR traffic model is used as a baseline for latency/jitter sensitive workloads in general evaluations, including evaluation of HARQ timing and K1 selection.

We next discuss the introduction of a new traffic model for 6G to better represent emerging traffic classes such as Generative AI (GenAI) traffic. GenAI traffic is rapidly growing and occupying a considerable percentage of traffic on the network. GenAI traffic includes multimodal interactions (text, image, video). Existing traffic models previously discussed only capture partial aspects of GenAI. For instance, FTP2’s think-time assumption does not reflect service-side behaviors such as time-to-first token and token streaming rates, and XR’s strict QoS is not suitable for all GenAI components (e.g., user prompts).

Proposal 7: For 6G evaluations, introduce a new traffic model for AI/ML traffic.

Note: Thrive to minimize modeling complexity for the new traffic model.

Realistic network conditions in 5G-A going into 6G involve mixed traffic. The mixed traffic approach studied in TR 36.899 needs to be however reassessed for applicability in current and next generation networks, given the nature of the traffic in current networks, such as GenAI, updated web browsing, video streaming, VoIP, XR and other bursty data. We thus propose to introduce a new mixed traffic model for 6G evaluations.

Proposal 8: For 6G evaluations, introduce a new traffic model considering mixed traffic.

Proposal 9: Consider enhancements on the mixed traffic model in TR36.899 to include representations of traffic characteristics of AI/ML, video streaming, web browsing, VoIP and XR traffic.

# Conclusion

In this contribution, we presented our views on evaluation assumptions for 6GR air interface design features and services. We made the following observations and proposals:

Observation 1: Full buffer traffic is non-representative of user behavior but is important for stress testing, peak spectral efficiency measurement, PHY/MAC calibration, and maintaining backward compatibility across releases.

Observation 2: Legacy FTP models vary in realism. FTP Model 1 is overly simplistic. FTP Model 3 captures per UE burstiness and can mimic instant messaging. FTP Model 2 is less representative than FTP3 but remains useful as a web browsing proxy (via think time) and for regression against LTE/NR where no dedicated HTTP model exists.

Observation 3: VoIP remains a relevant workload for conversational real-time services.

Observation 4: XR traffic provides a latency/jitter sensitive anchor and reflects UL heavy control loops, enabling study of HARQ timing, K1 selection, and related behaviors.

Proposal 1: For 6GR evaluations, realistic assumptions are used for interference modeling, channel estimation, and traffic modelling.

Proposal 2: For 6GR evaluations, include Energy Efficiency metric(s) as key performance metrics from day 1.

Proposal 3: Full Buffer, FTP Model 2, FTP Model 3, VoIP and XR traffic models are reused for 6G evaluations

Proposal 4: FTP Model 3 is considered as the main bursty traffic model for general evaluations. FTP Model 2 is only considered as a web browsing proxy and for regression until a dedicated HTTP model is available.

Proposal 5: When applicable, full buffer traffic model can be used in addition to a bursty traffic model for performance evaluations.

Note: Full buffer traffic model is not exclusively used to make observations/decisions on performance evaluations

Proposal 6: The XR traffic model is used as a baseline for latency/jitter sensitive workloads in general evaluations, including evaluation of HARQ timing and K1 selection.

Proposal 7: For 6G evaluations, introduce a new traffic model for AI/ML traffic.

Note: Thrive to minimize modeling complexity for the new traffic model.

Proposal 8: For 6G evaluations, introduce a new traffic model considering mixed traffic.

Proposal 9: Consider enhancements on the mixed traffic model in TR36.899 to include representations of traffic characteristics of AI/ML, video streaming, web browsing, VoIP and XR traffic.

# References

[1] RP-251881, New SID: Study on 6G Radio, NTT Docomo, Prague Czech Republic

[2] RP-252888, pCR for 38.914 on Deployment scenarios for 6G  Deutsche Telekom (moderator)

[3] R1-2507292 post-122 email discussion on 6GR common evaluation assumptions, Huawei (moderator)

[3] 3GPP TR38.914 Study on 6G Scenarios and Requirements, Rel. 20