3GPP TSG RAN WG1 #122bis			R1-2507787

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.3.2

Source: 	ASUSTeK

Title:  	Discussion on A-IoT active device

Document for:	Discussion and Decision

- Introduction

In last RAN1 meeting [1], there are some agreements on study for active device 2b/C as described as below.

Agreement

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Agreement

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Agreement

Study necessity and feasibility of enhancements of D2R for Device 2b and for Device C, considering the following aspects or potential enhancements:

- Considering that Device 2b and Device C can adjust the LO towards a desired frequency, whether to also support baseband small FS for Device 2b and Device C

- D2R frequency resources for FDM(A) (e.g., considering D2R transmission bandwidth, guard-band size between FDMed D2R transmissions, CFO, SFO, etc.)

- How a device determines the frequency resource for D2R transmission

- FDM(A) for D2R transmissions other than Msg1 and Msg3

Agreement

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

In this contribution, we discuss some aspects for A-IoT active device 2b and C.

- Discussion

As described in SID [2] on Ambient IoT in NR outdoor for active devices, there are some differences from passive device supported in Rel-19 A-IoT.

The first difference is that the active devices have higher peak power consumption, such that the maximum distance between reader and active device can be possibly 50-500 m. In response to the larger coverage, inventory procedure for active devices in outdoor should consider great number of devices than CBRA in Rel-19 A-IoT. Thus, for reducing inventory latency, it needs to support X>2 time domain resource(s) for D2R transmission(s) for Msg1, especially SFO of active devices is assumed to be better than any SFO value considered for Device 1. Moreover, D2R power control for active devices is worth study for efficient power utilization and interference mitigation in random access procedure. Since there is no RRC for A-IoT devices, reader will need to provide some power control-related parameters via R2D transmission. For instance, R2D transmit power, targeted reader received power, and TPC command (if closed-loop power control is supported) can be included in D2R scheduling information.

Proposal 1:  For CBRA, support X>2 time domain resource(s) for D2R transmission(s) for Msg1.

Proposal 2:  For power control of active devices, D2R scheduling information includes: 
R2D transmit power, targeted reader received power, and TPC command (if closed-loop power control is supported).

The second difference is that the active devices generate D2R transmission internally by device itself, with no external carrier wave. The Rel-19 method of referring D2R frequency location from carrier wave does not work on active devices. Thus, it needs study on how to derive D2R frequency location. Moreover, for FR1 licensed spectrum in FDD, Ambient IoT in NR outdoor supports R2D in DL spectrum and D2R in UL spectrum. However, WID on Ambient IoT Phase 2 [3] will support D2T2-C for active Device(s) such that R2D in UL spectrum and D2R in UL spectrum. No matter reader type is transparent or not, it needs to consider such spectrum differences. To simplify the design, active device can adjust LO towards a reference frequency and then apply baseband small frequency shift.

Proposal 3:  Considering no carrier wave and spectrum differences, active device adjusts LO towards 
a reference frequency and then apply baseband small frequency shift.

The third difference is that the active devices support traffic type DO-A, e.g., for sensor application. As study result in TR 38.769 [4], Rel-19 A-IoT only supports D2R transmission being indicated from a R2D transmission, so it is not applicable at least for the first D2R transmission for DO-A traffic. Thus, it needs study on the main issue that how reader provides D2R resource(s) for the first D2R transmission autonomously triggered by active devices. From our aspect, periodic signaling is not preferred due to its overhead. Instead, reader can transmit paging messages which target devices with pending DO-A data. If devices have pending DO-A data, they perform CBRA with time synchronization with the paging message; otherwise, devices neglect the paging message. The occasions for transmitting the paging messages can be up to reader implementation.

Proposal 4:  For the first D2R transmission autonomously triggered by active device, 
reader implements CBRA triggering which targets devices with pending DO-A data.

- Conclusion

In this contribution, we have following proposals for A-IoT active device 2b and C:

Proposal 1:  For CBRA, support X>2 time domain resource(s) for D2R transmission(s) for Msg1.

Proposal 2:  For power control of active devices, D2R scheduling information includes: 
R2D transmit power, targeted reader received power, and TPC command (if closed-loop power control is supported).

Proposal 3:  Considering no carrier wave and spectrum differences, active device adjusts LO towards 
a reference frequency and then apply baseband small frequency shift.

Proposal 4:  For the first D2R transmission autonomously triggered by active device, 
reader implements CBRA triggering which targets devices with pending DO-A data.

- Reference

- 3GPP TSG RAN WG1 #122

- RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”

- RP-251885, “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2”

- TR 38.769 v19.0.0