3GPP TSG RAN WG1 #122bis			R1-2507791

Prague, Czech Republic, Oct 13rd – 17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance on Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Agenda Item:	8.3

Document for: 	Discussion and Decision

- Introduction

In this contribution, remaining issues for SBFD are discussed.

- SBFD TX/RX

Default valid symbol type

For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. According to the latest TS 38.331 CR, the RRC parameter is optional. Further clarification may be needed for the case when the valid symbol type configuration is not provided. Following alternatives can be considered.

- Alt 1: The valid symbol type is always provided (i.e. mandatory present) for DL semi-statically configured receptions without activation DCI if sbfd-Config2-Reception is provided, and is always provided for UL semi-statically configured transmissions without activation DCI if sbfd-Config2-Transmission is provided.

- Alt 2: To determine the default valid symbol type as “non-SBFD”.

- Alt 3: To determine the valid symbol type based on the symbol type of the first transmission/reception occasion from frame 0.

Alt 1 and Alt 2 are simpler than Alt 3, and Alt 2 can save some signalling compared with Alt 1. Therefore, Alt 2 is preferred.

Proposal 1: For configuration 1, for valid symbol type determination for semi-statically configured transmissions/receptions without activation DCI,

- if the valid symbol type configuration is not provided, default valid symbol type is determined as “non-SBFD”.

CSI reporting subband

At the RAN1#117 meeting, CSI reporting subband partially overlapping with RB outside DL subband, due to unaligned CSI reporting subband boundary and SBFD subband boundary, was discussed and following agreement was made.

Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only.

Above agreement was not captured in the latest version of specification. However, a similar issue about partial RBG due to unaligned RBG boundary and SBFD subband boundary is captured in clause 5.1.2.2.1 and clause 6.1.2.2.1 in TS 38.214. Therefore, it’s better to clarify UE behaviour for the case when CSI reporting subband overlapping with RB outside DL subband.

Proposal 2: TP#1 is adopted for CSI reporting subband overlapping with RB outside DL subband.

UL transmission for SBFD

At the RAN1#121 meeting, it was agreed that for SRS resource set with usage set to ‘nonCodebook’, if associated CSI-RS is periodic or semi-persistent, only CSI-RS in the same symbol type as SRS is used to calculate SRS precoder. While no consensus was reached on how to handle the issue when associated CSI-RS is aperiodic (as the following yellow-highlighted parts).

#### Proposal 2-9 Proposed Agreement: For each SRS resource set with usage set to 'nonCodebook', - In case the associated CSI-RS is periodic or semi-persistent, only CSI-RS in the same symbol type as that for SRS is used to calculate SRS precoder. - In case the associated CSI-RS is aperiodic, an aperiodic associated CSI-RS for an SRS-ResourceSet is located at the first available slot which is no earlier than the slot with the scheduling DCI. - A slot is defined as an available slot if the symbols of the CSI-RS in the slot are of the same symbol type as the symbol type for the SRS-ResourceSet.

To follow the principle agreed for the case when P/SP CSI-RS associated with SRS, i.e. same symbol type of CSI-RS is used for SRS precoder calculation, the associated AP CSI-RS should be in the same symbol type as the SRS resource set. However, we think the new rule to determine slot of the associated AP CSI-RS is unnecessary. Considering that available slot offset for AP SRS can be indicated by triggering DCI, gNB can send the triggering DCI in a proper slot so that the associated AP CSI-RS in the same symbol type of the SRS.

Proposal 3: For each SRS resource set with usage set to 'nonCodebook', in case the associated CSI-RS is aperiodic, UE expects the associated aperiodic CSI-RS is in the same symbol type as the SRS resource set. TP#2 is adopted.

HARQ process ID for multi-PUSCH/PDSCH

In legacy, for multi-PDSCH scheduled by single DCI, the indicated HP ID is applied for the first PDSCH not overlapping with UL symbol, and HP ID is incremented by 1 for subsequent PDSCHs not overlapping with UL symbol. For multi-PUSCH scheduled by single DCI, the indicated HP ID is applied for the first PUSCH not overlapping with DL symbol or SSB symbol, and HP ID is incremented by 1 for subsequent PUSCHs not overlapping with DL symbol or SSB symbol. For multi-PUSCH CG, HARQ process ID is calculated for the first configured grant PUSCH, and it is incremented by 1 for each CG PUSCH not overlapping with DL symbol or SSB symbol.

When a PDSCH/PUSCH occasion is across SBFD symbols and non-SBFD symbols, or when a PDSCH/PUSCH occasion is in the invalid symbol type if configuration 1 is provided, the PDSCH/PUSCH occasion is dropped. Whether HP ID is counted for such dropped PDSCH/PUSCH occasion needs to be clarified.

The main motivation of not counting HP ID for PDSCH/PUSCH occasions that can’t be received/transmitted in legacy rule is to improve HARQ process utilization efficiency, and the same motivation is still valid. Therefore, it is preferred to not count HP ID for PDSCH/PUSCH occasions across SBFD symbols and non-SBFD symbols or in the invalid symbol type.

Proposal 4: TP#3 and TP#4 are adopted.

- For multi-PDSCH scheduled by single DCI:

  - The indicated HP ID is applied to the first PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols

  - HP ID is incremented by 1 for each subsequent PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

- For multi-PUSCH scheduled by single DCI:

  - The indicated HP ID is applied to the first PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols

  - HP ID is incremented by 1 for each subsequent PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

- For multi-PUSCH CG:

  - HP ID is incremented by 1 for each subsequent configured grant PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

HARQ-ACK feedback for multi-PDSCH scheduled by single DCI

For type 1 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if timeDomainHARQ-BundlingType1 is provided, bundled HARQ-ACK is determined by binary AND operation of HARQ-ACK information bits for PDSCHs not overlapping with UL symbols. For type 2 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if nrofHARQ-BundlingGroups is provided, for a PDSCH group including at least one PDSCH not overlapping with UL symbol, bundled HARQ-ACK for the group is determined by binary AND operation of HARQ-ACK information bits for PDSCHs not overlapping with UL symbols in the group. For a PDSCH group only including PDSCHs overlapping with UL symbol, NACK is generated.

When a PDSCH occasion is across SBFD symbols and non-SBFD symbols, or when a PDSCH occasion is in the invalid symbol type if configuration 1 is provided, the PDSCH occasion is dropped. If following the legacy HARQ-ACK codebook construction rule when time domain bundling is configured, when there is a PDSCH not received due to across both SBFD symbols and non-SBFD symbols or due to invalid symbol type, NACK is determined for the multiple PDSCHs after applying binary AND operation, which results in meaningless information for reporting. A straightforward solution is to exclude PDSCH occasion across both SBFD symbols and non-SBFD symbols or in invalid symbol type before applying the binary AND operation.

Proposal 5: TP#5 is adopted.

- For type 1 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if timeDomainHARQ-BundlingType1 is provided, HARQ-ACK for the multiple PDSCHs scheduled by single DCI is determined by binary AND operation of the HARQ-ACK information bits of PDSCH receptions that do not overlap with UL symbols, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided.

Proposal 6: TP#6 is adopted.

- For type 2 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if nrofHARQ-BundlingGroups is provided,

- For a TBG including at least one PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided, UE assumes a PDSCH overlapping with UL symbol, or across SBFD symbols and non-SBFD symbols, or in invalid symbol type if configuration 1 is provided, is correctly received.

- For a TBG including only PDSCHs overlapping with UL symbol, or across SBFD symbols and non-SBFD symbols, or in invalid symbol type if configuration 1 is provided, UE generates NACK value for the TBG.

- The number of TBGs including at least one PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and in invalid symbol type if configuration 1 is provided, is used to determine PUCCH transmission power.

SPS PDSCH collision handling

In legacy, for collision handling among multiple SPS PDSCHs, SPS PDSCH overlapping with UL symbol is excluded before the collision handling procedure.

When an SPS PDSCH occasion is across SBFD symbols and non-SBFD symbols, or when an SPS PDSCH occasion is in the invalid symbol type if configuration 1 is provided, the SPS PDSCH occasion is dropped. Similarly, excluding SPS PDSCH occasions across SBFD symbols and non-SBFD symbols or in invalid symbol type before the SPS collision handling procedure is beneficial to allow other overlapping SPS PDSCHs to be received.

Proposal 7: TP#7 is adopted.

- The SPS PDSCH collision handling procedure is applied after excluding SPS PDSCH across SBFD and non-SBFD symbols, or in the invalid symbol type if configuration 1 is provided.

HARQ-ACK codebook for SPS PDSCHs

In legacy, for HARQ-ACK codebook for SPS PDSCHs, the SPS PDSCHs not received due to overlapping with UL symbol are excluded.

When an SPS PDSCH occasion is across SBFD symbols and non-SBFD symbols, or when an SPS PDSCH occasion is in the invalid symbol type if configuration 1 is provided, the SPS PDSCH occasion is dropped. Similarly, excluding SPS PDSCH occasions in invalid symbol type from the SPS HARQ-ACK codebook is beneficial for PUCCH overhead reduction.

Proposal 8: TP#8 is adopted.

- For configuration 1, for SPS HARQ-ACK codebook construction, SPS PDSCHs not received due to across SBFD and non-SBFD symbols, or in the invalid symbol type if configuration 1 is provide are not included.

TDD CA collision handling

At the RAN1#120 meeting, it was agreed that SBFD operation on one TDD carrier in multi-carrier scenario is supported. Whether/how to handle the impact on half duplex CA case should be further discussed.

Agreement Support SBFD operation on one TDD carrier in multi-carrier scenario. - Note: If the TDD carrier for SBFD operation is an Scell, support UE dedicated signaling for cell-specific configuration of time and frequency location of SBFD subbands for the Scell. - FFS whether/how to handle half-duplex CA case

Half duplex TDD CA collision handling is supported since Rel-16. For multiple cells with same SCS provided with directionalCollisionHandling-r16 = 'enabled', UE handles directional collision between the multiple cells by the rule specified in TS 38.213. For directional collision handling among the multiple cells, UE determines a reference cell first. Then UE handles the collision between reference cell and another cell among the multiple cells.

When SBFD operation is configured on a cell among the multiple cells, a semi-static DL symbol configured with SBFD subbands may be used for DL reception or UL transmission. Therefore, collision handling rule for TDD CA needs to be updated with considering possible two directions on a SBFD DL symbol. A straightforward solution is to take SBFD DL symbols as flexible symbols.

Proposal 9: TP#9 is adopted.

- For TDD CA directional collision handling with one SBFD cell, the legacy rule is reused with considering SBFD symbol as flexible symbol.

UL cancellation DCI

For UL cancellation DCI format 2_4, the time domain position for the indicated resources is obtained by excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon.

Considering SBFD operation with UL subband in DL symbols, there may be UL transmission in DL symbols. With the legacy rule, UL transmission in UL subband in SBFD symbols configured as DL can’t be cancelled by DCI 2_4. Therefore, enhancement on application of UL cancellation DCI format 2_4 should be considered. A straightforward solution is to update the determination of TCI symbols, i.e. excluding non-SBFD DL symbols and SSB symbols.

Proposal 10: TP#10 is adopted.

- For UL cancellation DCI format 2_4, the TCI symbols are determined excluding non-SBFD DL symbols and SSB symbols.

PUCCH resource in SBFD symbols

It was agreed that if separate frequency configuration for SBFD is not provided, starting PRB configured for non-SBFD symbols is used for PUCCH transmission in SBFD symbols.

One possible issue may be UE behavior if the PUCCH frequency resource configured for non-SBFD symbols overlaps with RB outside UL usable PRBs. For such case, separate discussion may be needed for P/SP PUCCH transmissions (e.g., SR PUCCH, P/SP CSI PUCCH) and PUCCH repetitions. For SR/CSI PUCCH transmission occasions, a PUCCH transmission occasion in SBFD symbols overlapping with RB outside UL usable PRBs is dropped. For PUCCH repetitions, a PUCCH repetition occasion in SBFD symbols overlapping with RBs outside UL usable PRBs can be postponed.

Proposal 11: TP#11 is adopted.

- If separate frequency configuration for SBFD is not provided for a PUCCH resource,

  - For P/SP PUCCH (e.g. SR PUCCH, P/SP CSI PUCCH), if a PUCCH in SBFD symbols overlap with RB outside UL usable PRBs, the PUCCH is dropped.

  - For PUCCH repetitions, if a PUCCH repetition in SBFD symbols overlaps with RB outside UL usable PRBs, a slot is determined as valid if the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

PUCCH repetition slot counting

At the RAN1#119 meeting, following agreement was made on TX/RX in the invalid symbol type for configuration 1. For PUCCH repetitions, a slot in which the PUCCH resource is in the invalid symbol type is not counted in the number of repetitions.

Agreement Update the following agreement made in RAN1#118 meeting: Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) with Configuration 1, - For PUSCH repetition type A with available slot counting, A-SRS with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH with neither TBoMS nor PUSCH repetition type A with available slot counting,and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

In the latest version of clause 11.1 of TS 38.213, it was clarified that for configuration 1, PUCCH repetitions are transmitted in the same symbol type as the first repetition. However, the impact on PUCCH repetition slot counting is not explicitly captured. It may cause ambiguity whether PUCCH repetition dropping or postponing is performed if the PUCCH in a slot is in the invalid symbol type.

In our understanding, it is better to clarify UE behavior on PUCCH repetition slot determination for configuration 1, which is similar to the explicit clarification for PUSCH repetition type A in Clause 6.1.2.1a of TS 38.214.

#### 6.1.2.1a Resource allocation in time domain for SBFD For a UE scheduled with PUSCH transmission occasions across SBFD symbols and non-SBFD symbols in different slots, - if the UE is not configured with [sbfd-Configuration2-Transmission], - the UE transmits only the PUSCH in a valid symbol type. <*************************************omitted***************************************> - For PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3 when AvailableSlotCounting is enabled and K>1 or TB processing over multiple slots or PUSCH repetition type A scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, RAR UL grant or fallbackRAR UL grant, - a slot containing the transmission occasion that is not in the valid symbol type is not counted in the number of N∙K slots. - In case the valid symbol type is SBFD symbol, a slot is counted in the number of N∙K slots if the symbols allocated for the transmission occasion in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <*************************************omitted***************************************>

Proposal 12: TP#12 is adopted.

SPS HARQ-ACK deferring

For PUCCH, SPS HARQ-ACK deferring was introduced in Rel-17. When a PUCCH for SPS HARQ-ACK overlaps with semi-static DL symbol or SSB symbol, UE will defer the PUCCH until no overlapping with semi-static DL symbol or SSB symbol, as shown in Fig 9. Considering SBFD operation with UL subband in semi-static DL symbols, impact on SPS HARQ-ACK deferring condition and target slot determination should be considered.

A straightforward solution is to defer SPS HARQ-ACK if the determined SPS HARQ-ACK PUCCH resource overlaps with non-SBFD DL symbol or SSB symbol, or it is across SBFD symbols and non-SBFD symbols, or if it overlaps with RB outside UL usable PRBs in SBFD symbols. The target slot can be determined as the first slot in which the determined PUCCH resource is not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols, and not overlapping with RB outside UL usable PRBs in SBFD symbols.

Fig 9: Example of SPS HARQ-ACK deferring

Proposal 13: TP#13 is adopted.

- If SPS HARQ-ACK deferring is enabled,

- SPS HARQ-ACK is deterred if the determined SPS HARQ-ACK PUCCH resource overlaps with non-SBFD DL symbol or SSB symbol, or it is across both SBFD symbols and non-SBFD symbols, or if it overlaps with RB outside UL usable PRBs in SBFD symbols.

- The target slot for deferring is the first slot in which the determined PUCCH resource is not overlapping with non-SBFD DL symbol or SSB symbol, and not overlapping with SBFD symbols and non-SBFD symbols, and not overlapping with RB outside UL usable PRBs in SBFD symbols.

- Proposed TPs

TP#1: CSI reporting subband

TS 38.214, v19.1.0 #### 5.2.1.4 Reporting configurations The UE shall calculate CSI parameters (if reported) assuming the following dependencies between CSI parameters (if reported) - LI shall be calculated conditioned on the reported CQI, PMI, RI and CRI - CQI shall be calculated conditioned on the reported PMI, RI and CRI - PMI shall be calculated conditioned on the reported RI and CRI - RI shall be calculated conditioned on the reported CRI. <**************************************omitted***************************************> For CSI reporting, a UE can be configured via higher layer signaling with one out of two possible subband sizes, where a subband is defined as contiguous PRBs and depends on the total number of PRBs in the bandwidth part according to Table 5.2.1.4-2. Table 5.2.1.4-2: Configurable subband sizes Bandwidth part (PRBs); Subband size (PRBs). 24 – 72; 4, 8. 73 – 144; 8, 16. 145 – 275; 16, 32. The reportFreqConfiguration contained in a CSI-ReportConfig indicates the frequency granularity of the CSI Report. A CSI Reporting Setting configuration defines a CSI reporting band as a subset of subbands of the bandwidth part, where the reportFreqConfiguration indicates: - the csi-ReportingBand as a contiguous or non-contiguous subset of subbands in the bandwidth part for which CSI shall be reported. - A UE is not expected to be configured with csi-ReportingBand which contains a subband where a CSI-RS resource linked to the CSI Report setting has the frequency density of each CSI-RS port per PRB in the subband less than the configured density of the CSI-RS resource. - If a CSI-IM resource is linked to the CSI Report Setting, a UE is not expected to be configured with csi-ReportingBand which contains a subband where not all PRBs in the subband have the CSI-IM REs present. - For a CSI-ReportConfig with the higher layer parameter symbolType set to ‘sbfd’, UE can be configured with a CSI subband if at least one PRB in the CSI subband is within PRBs that are both in the active DL BWP and in the DL sub-band(s). CSI shall be reported only for PRBs that are both in the active DL BWP and in the DL sub-band(s) in SBFD symbols. - wideband CQI or subband CQI reporting, as configured by the higher layer parameter cqi-FormatIndicator. When wideband CQI reporting is configured, a wideband CQI is reported for each codeword for the entire CSI reporting band. When subband CQI reporting is configured, one CQI for each codeword is reported for each subband in the CSI reporting band. - wideband PMI or subband PMI reporting as configured by the higher layer parameter pmi-FormatIndicator. When wideband PMI reporting is configured, a wideband PMI is reported for the entire CSI reporting band. When subband PMI reporting is configured, except with 2 antenna ports, a single wideband indication (i1 in Clause 5.2.2.2) is reported for the entire CSI reporting band and one subband indication (i2 in clause 5.2.2.2) is reported for each subband in the CSI reporting band. When subband PMIs are configured with 2 antenna ports, a PMI is reported for each subband in the CSI reporting band. - a UE is not expected to be configured with pmi-FormatIndicator if codebookType is set to 'typeII-r16', 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18'. A CSI Reporting Setting is said to have a wideband frequency-granularity if - reportQuantity is set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI', cqi-FormatIndicator is set to 'widebandCQI' and pmi-FormatIndicator is set to 'widebandPMI', or - reportQuantity is set to 'cri-RI-PMI-CQI', codebookType is set to 'typeII-PortSelection-r17', 'typeII-CJT-PortSelection-r18' or 'typeII-Doppler-PortSelection-r18' with M=1 and cqi-FormatIndicator is set to 'widebandCQI', or - reportQuantity is set to 'cri-RI-i1' or - reportQuantity is set to 'cri-RI-CQI' or 'cri-RI-i1-CQI' and cqi-FormatIndicator is set to 'widebandCQI', or - reportQuantity is set to 'cri-RSRP' or 'ssb-Index-RSRP' or 'cri-SINR', or 'ssb-Index-SINR' or 'cri-RSRP-Index' or 'ssb-Index-RSRP-Index' or 'cri-SINR-Index', or 'ssb-Index-SINR-Index', or - reportQuantity is set to 'tdcp', or - reportQuantity is set to 'cli-SRS-RSRP', or - reportQuantity is set to or 'cli-RSSI' otherwise, the CSI Reporting Setting is said to have a subband frequency-granularity. A CSI Reporting Setting with codebookType set to 'typeI-SinglePanel' and the corresponding CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, as described in clause 5.2.1.4.1, can be configured with wideband frequency-granularity only if csi-ReportMode is set to 'Mode1' and numberOfSingleTRP-CSI-Mode1 is set to X=0, as described in clause 5.2.1.4.2. If the UE is configured with a CSI Reporting Setting for a bandwidth part with fewer than 24 PRBs, the CSI reporting setting is expected to have a wideband frequency-granularity, and, if applicable, the higher layer parameter codebookType is set to 'typeI-SinglePanel'. The first subband size is given by and the last subband size given by if and if If a UE is configured with semi-persistent CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic or semi-persistent. If a UE is configured with aperiodic CSI reporting, the UE shall report CSI when both CSI-IM and NZP CSI-RS resources are configured as periodic, semi-persistent or aperiodic. A UE configured with DCI format 0_1, 0_2 or 0_3 does not expect to be triggered with multiple CSI reports with the same CSI-ReportConfigId. <*****************************************omitted******************************************>

TP#2: UL transmission for SBFD

TS 38.214, v19.1.0 #### 6.1.1.3 UL transmission for SBFD The UE does not expect to be configured with srs-ResourceSetId and srs-ResourceIndicator2 in rrc-ConfiguredUplinkGrant. If txConfig is set to 'codebook', the UE does not expect to be configured with precodingAndNumberOfLayers2 in rrc-ConfiguredUplinkGrant. If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. In case the resourceType of the associated CSI-RS resource is set to periodic or semi-persistent, only CSI-RS in the same symbol type as configured in symbolType for the SRS resource set is used to calculate the precoder used for the transmission of SRS. In case the resourceType of the associated CSI-RS resource is set to aperiodic, UE expects the associated aperiodic CSI-RS is in the same symbol type as configured in symbolType for the SRS resource set. For Type 1 PUSCH transmissions with a configured grant, - if the UE is not configured with sbfd-Config2-Transmission, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the SRS-ResourceSet where symbolType corresponds the same symbol type as the valid symbol type of PUSCH transmission. If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet where symbolType corresponds the same symbol type as the valid symbol type of PUSCH transmission. - if the UE is configured with sbfd-Config2-Transmission, srs-ResourceIndicator is applicable to both SRS resource sets where symbolType is set to ‘non-sbfd’ and SRS resource sets where symbolType is set to ‘sbfd’. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet where symbolType is set to ‘sbfd’. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet where symbolType is set to ‘non-sbfd’. For Type 2 PUSCH transmissions with a configured grant, or for PUSCH transmission occasions across SBFD symbols and non-SBFD symbols scheduled by DCI format 0_1, 0_2, or 0_3, - if the UE is not configured with sbfd-Config2-Transmission, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the SRS-ResourceSet where symbolType corresponds the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource is prior to the PDCCH carrying the SRI. If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet where symbolType corresponds to the same symbol type as the valid symbol type of PUSCH transmission. - if the UE is configured with sbfd-Config2-Transmission, the SRS resource indicator is applicable to both SRS resource sets where symbolType is set to ‘non-sbfd’ and SRS resource sets where symbolType is set to ‘sbfd’. For PUSCH transmissions in SBFD symbols, the indicated SRI in a slot is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in a slot is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet SRS resource sets where symbolType is set to ‘sbfd’. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet where symbolType is set to ‘non-sbfd’. If only a single SRS resource is configured in both SRS resource sets where symbolType is set to ‘non-sbfd’ and SRS resource sets where symbolType is set to ‘sbfd’, the SRI field is absent from DCI. <*************************************omitted***************************************>

TP#3: HARQ process ID for multi-PDSCH scheduled by single DCI

TS 38.214, v19.1.0 ## 5.1 UE procedure for receiving the physical downlink shared channel For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700, and when no configuration is provided the UE may assume a default number of 8 processes. A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not across both SBFD symbols and non-SBFD symbols, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in invalid symbol type if the UE is not configured with [sbfd-Configuration2-Transmission]. When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA', the PDSCH includes two PDSCH transmission occasions. For each PDSCH, if either PDSCH occasion overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, the PDSCH is not received and HARQ process ID is not increment for the PDSCH. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH if the UE is not capable of receiving FDMed unicast and multicast PDSCH per slot per carrier. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration , with N=13 for =0, N=13 for =1, N=20 for =2, N=24 for =3, N=96 for m=5, and N=192 for m=6. When receiving PDSCH scheduled with SI-RNTI, P-RNTI, MCCH-RNTI, G-RNTI for broadcast, or Multicast MCCH-RNTI, G-RNTI for multicast in RRC_INACTIVE state, the UE may assume that the DM-RS port of PDSCH is quasi co-located with the associated SS/PBCH block with respect to Doppler shift, Doppler spread, average delay, delay spread, spatial RX parameters when applicable. <*************************************omitted***************************************>

TP#4: HARQ process ID for multi-PUSCH

TS 38.214, v19.1.0 ## 6.1 UE procedure for transmitting the physical uplink shared channel PUSCH transmission(s) can be dynamically scheduled by an UL grant in a DCI, or the transmission can correspond to a configured grant Type 1 or Type 2. The configured grant Type 1 PUSCH transmission is semi-statically configured to operate upon the reception of higher layer parameter of configuredGrantConfig including rrc-ConfiguredUplinkGrant without the detection of an UL grant in a DCI. The configured grant Type 2 PUSCH transmission is semi-persistently scheduled by an UL grant in a valid activation DCI according to clause 10.2 of [6, TS 38.213] after the reception of higher layer parameter configuredGrantConfig not including rrc-ConfiguredUplinkGrant. If configuredGrantConfigToAddModList is configured, more than one configured grant configuration of configured grant Type 1 and/or configured grant Type 2 may be active at the same time on an active BWP of a serving cell. The UE can be configured with a list of up to 64 TCI-UL-State configurations within the higher layer parameter BWP-UplinkDedicated. Each TCI-UL-State configuration contains a parameter for configuring one reference signal, if applicable, for determining UL TX spatial filter for dynamic-grant and configured-grant based PUSCH and PUCCH resource in a CC, and SRS. <****************************************omitted******************************************> When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and, when applicable, indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213], and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if the UE is not configured with [sbfd-Configuration2-Transmission]. When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-States or TCI-UL-States, - a UE having a PUSCH transmission scheduled or activated by DCI format 0_0 should apply the first indicated TCI state to the PUSCH transmission, - a UE configured with a PUSCH transmission corresponding to a Type 1 configured grant is expected to be configured with the higher layer parameter applyIndicatedTCI-State indicating the first, the second or both of the indicated TCI states to be applied for the PUSCH transmission. If 'both' TCI states are indicated, the UE should apply the first indicated TCI state to the PUSCH transmission occasion(s) or the PUSCH antenna port(s) associated with the first SRS resource set for CB/NCB transmission, and the second indicated TCI state to the PUSCH transmission occasion(s) or the PUSCH antenna port(s) associated with the second SRS resource set for CB/NCB transmission; otherwise the UE should apply either the 'first' or 'second' indicated TCI state to all PUSCH transmission occasions. - If the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in different ControlResourceSets in the active DL BWP, the first and the second indicated TCI states correspond to the indicated TCI-States or TCI-UL-States specific to coresetPoolIndex value 0 and value 1, respectively, and applyIndicatedTCI-State does not indicate both of the indicated TCI states to be applied for the PUSCH transmission. For the PUSCH retransmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, the parameters in pusch-Config are applied for the PUSCH transmission except for p0-NominalWithoutGrant, p0-PUSCH-Alpha, powerControlLoopToUse, pathlossReferenceIndex described in clause 7.1 of [6, TS 38.213], mcs-Table, mcs-TableTransformPrecoder described in clause 6.1.4.1 and transformPrecoder described in clause 6.1.3. For a UE configured with two uplinks in a serving cell, PUSCH retransmission for a TB on the serving cell is not expected to be on a different uplink than the uplink used for the PUSCH initial transmission of that TB. A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1, 0_2 or 0_3 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS 38.321]. Upon detection of a DCI format 0_1 or 0_2 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. Upon detection of a DCI format 0_3 with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_3, the UE ignores all fields for the PUSCH that would carry an aperiodic CSI report according to clause 5.2.3 on the scheduled cell with the smallest serving cell index in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH on the serving cell with the smallest serving cell index as indicated by this DCI format 0_3. When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and, when applicable, indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, and not across both SBFD symbols and non-SBFD symbols, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and, when applicable, indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or across both SBFD symbols and non-SBFD symbols, or in invalid symbol type if the UE is not configured with [sbfd-Configuration2-Transmission]. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16 or outOfOrderOperationUL-r18, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1, 0_2 or 0_3 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. <****************************************omitted******************************************>

TP#5: Type 1 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI

TS 38.213, v19.1.0 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell c, an active DL BWP, and an active UL BWP, as described in clause 12, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU. If serving cell c is deactivated, the UE uses as the active DL BWP for determining the set of MA,c occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: <************************************omitted**********************************> A UE determines o0ACK, o1ACK,...,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set MA,c defines a total number Mc of occasions for PDSCH reception or SPS PDSCH release or TCI state update for serving cell c corresponding to the HARQ-ACK information bits. Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cells including, when applicable, cells in the set S0 and the set S1 Set j=0- HARQ-ACK information bit index Set NcellsDL to the number of serving cells configured by higher layers for the UE while c<NcellsDL Set m=0 – index of occasion for candidate PDSCH reception, or SPS PDSCH release, or TCI state update while m<Mc if timeDomainHARQ-BundlingType1 is provided for serving cell c and a PDSCH associated with occasion m is scheduled by a DCI format indicating a TDRA row that includes more than one SLIV entry if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and not across both SBFD symbols and non-SBFD symbols, and in the same symbol type as the first SLIV in the TDRA row if the UE is not configured with [sbfd-Configuration2-Transmission], scheduled by the DCI format on serving cell c; j=j+1; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to second transport blocks in PDSCH receptions, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and not across both SBFD symbols and non-SBFD symbols, and in the same symbol type as the first SLIV in the TDRA row if the UE is not configured with [sbfd-Configuration2-Transmission], scheduled by the DCI format on serving cell c; else ojACK= NACK; j=j+1; ojACK= NACK; end if j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c if the PDSCH is associated with the last SLIV in the TDRA row; ojACK = binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and not across both SBFD symbols and non-SBFD symbols, and in the same symbol type as the first SLIV in the TDRA row if the UE is not configured with [sbfd-Configuration2-Transmission], scheduled by the DCI format of serving cell c if the UE receives one transport block, the UE assumes ACK for the second transport block; else ojACK = NACK; end if j=j+1; else if the PDSCH is associated with the last SLIV in the TDRA row; ojACK =binary AND operation of the HARQ-ACK information bits corresponding to all transport blocks in PDSCHs, that do not overlap with an uplink symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and not across both SBFD symbols and non-SBFD symbols, and in the same symbol type as the first SLIV in the TDRA row if the UE is not configured with [sbfd-Configuration2-Transmission], scheduled by the DCI format of serving cell c else ojACK = NACK; end if j=j+1; end if else if harq-ACK-SpatialBundlingPUCCH is not provided, PDSCH-CodeBlockGroupTransmission is not provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c, ojACK= HARQ-ACK information bit corresponding to a first transport block of this cell; j=j+1; ojACK = HARQ-ACK information bit corresponding to a second transport block of this cell; j=j+1; elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c, ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell if the UE receives one transport block, the UE assumes ACK for the second transport block; j=j+1; elseif PDSCH-CodeBlockGroupTransmission is provided, and NHARQ-ACK,cCBG/TB,max CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell c, Set nCBG=0- CBG index while nCBG<NHARQ-ACK,cCBG/TB,max oj+nCBGACK = HARQ-ACK information bit corresponding to CBG nCBG of the first transport block; if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c oj+nCBG+NHARQ-ACK,cCBG/TB,maxACK = HARQ-ACK information bit corresponding to CBG nCBG of the second transport block; end if nCBG=nCBG+1; end while j=j+NTB,cDL∙NHARQ-ACK,cCBG/TB,max, where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell c; else ojACK = HARQ-ACK information bit of serving cell c; j=j+1; end if end if m=m+1; end while c=c+1; end while If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=c=0NcellsDL-1m=0Mc-1Nm,creceived+c=0NcellsDL-1m=0Mc-1Nm,creceived,CBG where - NcellsDL are all DL cells where the UE is configured to receive unicast or multicast PDSCHs - Mc is the cardinality for the union of all sets MA,c of occasions for unicast or multicast PDSCH receptions or SPS PDSCH releases for serving cell c - Nm,creceived is the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release or TCI state update in PDSCH reception occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH. - If timeDomainHARQ-BundlingType1 is provided for serving cell c and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers as received only a PDSCH associated with the last SLIV. - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH reception occasion m for serving cell c if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format that supports CBG-based PDSCH receptions and the UE reports corresponding HARQ-ACK information in the PUCCH. <*************************************omitted***************************************>

TP#6: Type 2 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI

TS 38.213, v19.1.0 #### 9.1.3.1 Type-2 HARQ-ACK codebook in physical uplink control channel If a UE is configured to monitor PDCCH for multicast DCI formats with CRC scrambled by one or more G-RNTIs for multicast or G-CS-RNTIs that the UE generates a Type-2 HARQ-ACK codebook, the UE separately applies the procedures in this clause per G-RNTI for multicast or per G-CS-RNTI using maxNrofCodeWordsScheduledByDCI in pdsch-ConfigMulticast except the procedures for SPS PDSCHs and applies the procedures in this clause using maxNrofCodeWordsScheduledByDCI provided in pdsch-Config for unicast DCI formats excluding the unicast DCI format activating SPS PDSCH receptions, and determines the Type-2 HARQ-ACK codebook by concatenating the Type-2 HARQ-ACK codebook for unicast DCI formats excluding the unicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-RNTI values, followed by the HARQ-ACK codebooks for the multicast DCI formats in ascending order of the corresponding G-CS-RNTI values excluding the multicast DCI format activating SPS PDSCH receptions, followed by the HARQ-ACK codebooks for unicast and multicast SPS PDSCH receptions. <************************************omitted*************************************> If a UE is - not provided PDSCH-CodeBlockGroupTransmission for any serving cell, and - not provided pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for any serving cell, or provided nrofHARQ-BundlingGroups with value of 1 for any serving cell provided pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 or - for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions and does not schedule more than one PDSCH reception, or - for PDSCH receptions scheduled by a DCI format on a serving cell when the UE is provided nrofHARQ-BundlingGroups with value of 1, or - for SPS PDSCH reception, or - for a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as described in clause 7.2.1, as nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c+g=0G-1VDAI,mlast,gDL-c=0Ncells,gDL-1UDAI,c,gmodTD,gNTB,max,gDL+c=0Ncells,gDL-1m=0Mg-1Nm,c,greceived+NSPS,c,g where - NcellsDL is a number of serving cells where the UE is configured to receive unicast PDSCHs - Ncells,gDL is a number of serving cells where the UE is configured to receive multicast PDSCHs for a G-RNTI g for multicast or a G-CS-RNTI g - G is a total number of G-RNTIs for multicast or G-CS-RNTIs configured to the UE - M is the number of PDCCH monitoring occasions for unicast DCI formats - Mg is the number of PDCCH monitoring occasions for multicast DCI formats with CRC scrambled by G-RNTI g for multicast or G-CS-RNTI g - TD=2NC-DAIDL where NC-DAIDL the number of bits for the counter DAI field in unicast DCI formats - TD,g=2NC-DAI,gDL where NC-DAI,gDL the number of bits for the counter DAI field in multicast DCI formats with CRC scrambled by G-RNTI g for multicast or G-CS-RNTI g - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling PDSCH reception or having associated HARQ-ACK information without scheduling PDSCH reception, that the UE detects within the M PDCCH monitoring occasions. - if Ncells,gDL=1, VDAI,mlast,gDL is the value of the counter DAI in the last multicast DCI format with G-RNTI g for multicast, or G-CS-RNTI g, scheduling PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the Mg PDCCH monitoring occasions - if NcellsDL>1 or if Ncells,gDL>1 - if the UE does not detect any DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the counter DAI in a last DCI format the UE detects in the last PDCCH monitoring occasion - if the UE detects at least one DCI format that includes a total DAI field in a last PDCCH monitoring occasion within the M or Mg, for G-RNTI g for multicast or G-CS-RNTI g, PDCCH monitoring occasions where the UE detects at least one DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c, VDAI,mlastDL or VDAI,mlast,gDL, respectively, is the value of the total DAI in the at least one DCI format that includes a total DAI field - VDAI,mlastDL=0 or VDAI,mlast,gDL=0 if the UE does not detect any DCI format scheduling PDSCH reception, or having associated HARQ-ACK information without scheduling PDSCH reception, for any serving cell c in any of the M or Mg PDCCH monitoring occasions, respectively. - UDAI,c or UDAI,c,g, for G-RNTI g for multicast or G-CS-RNTI g, is the total number of DCI formats scheduling PDSCH receptions providing transport blocks with enabled HARQ-ACK information, or having associated HARQ-ACK information without scheduling a PDSCH reception, that the UE detects within the M or Mg PDCCH monitoring occasions, respectively, for serving cell c. UDAI,c=0 or UDAI,c,g=0 if the UE does not detect any DCI format scheduling PDSCH reception providing a transport block with enabled HARQ-ACK information, or having associated HARQ-ACK information without scheduling PDSCH reception, for serving cell c in any of the M or Mg, respectively, PDCCH monitoring occasions. - NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1. - NTB,max,gDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c and harq-ACK-SpatialBundlingPUCCH is not provided for G-RNTI g for multicast or G-CS-RNTI g; otherwise, NTB,max,gDL=1. - Nm,creceived or Nm,c,greceived, for G-RNTI g for multicast or G-CS-RNTI g, is - if harq-ACK-SpatialBundlingPUCCH is not provided, the number of transport blocks the UE receives in a PDSCH, or the number of transport block groups the UE receives in PDSCHs if nrofHARQ-BundlingGroups with NHARQ-ACKTBG,max=1 is provided, scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c, or - if harq-ACK-SpatialBundlingPUCCH is provided, the number of PDSCHs, or the number of PDSCH groups if nrofHARQ-BundlingGroups with NHARQ-ACKTBG,max=1 is provided, scheduled by a DCI format that the UE detects in PDCCH monitoring occasion m for serving cell c, or - the number of DCI formats that the UE detects and have associated a HARQ-ACK information without scheduling PDSCH reception in PDCCH monitoring occasion m for serving cell c. - NSPS,c or NSPS,c,g, for G-RNTI g for multicast or G-CS-RNTI g, is the number of SPS PDSCH receptions by the UE on serving cell c for which the UE transmits corresponding HARQ-ACK information in the same PUCCH as for HARQ-ACK information corresponding to PDSCH receptions within the M or Mg PDCCH monitoring occasions, respectively. If a UE - is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and - is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - a DCI format having associated HARQ-ACK information without scheduling PDSCH reception, - TCI state update, and - TB-based PDSCH receptions on the NcellsDL,CBG serving cells and on the NcellsDL,TB serving cells, - NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions, and - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ-ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ-ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ-ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ-ACK,cCBG/TB,max<NHARQ-ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ-ACK,maxCBG/TB,max-NTB,cDL⋅NHARQ-ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c - the pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook If OACK+OSR+OCSI≤11, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling CBG-based PDSCH reception that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling CBG-based PDSCH reception for any serving cell c that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for any serving cell c in any of the M PDCCH monitoring occasions - UDAI,cCBG is the total number of DCI formats scheduling CBG-based PDSCH receptions that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cCBG=0 if the UE does not detect any DCI format scheduling CBG-based PDSCH reception for serving cell c in any of the M PDCCH monitoring occasions - Nm,creceived,CBG is the number of CBGs the UE receives in a PDSCH scheduled by a DCI format that supports CBG-based PDSCH reception that the UE detects in PDCCH monitoring occasion m for serving cell c and the UE reports corresponding HARQ-ACK information in the PUCCH If a UE is provided nrofHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions on the serving cell c where, for a maximum number of NPDSCH,cmax PDSCH receptions scheduled by a DCI format on the serving cell c, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by nrofHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the second TBs as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c. For a TBG associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, and not across both SBFD symbols and non-SBFD symbols, and in same symbol type as the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], the UE assumes that TB(s) provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], are correctly received. For a TBG associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], the UE generates a NACK value for the TBG. If a UE is provided nrofHARQ-BundlingGroups and harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over PDSCH reception groups for PDSCH receptions scheduled by a DCI format on the serving cell c where a maximum number of PDSCH reception groups, NHARQ-ACK,cTBG,max, is provided by nrofHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c, after binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of each PDSCH reception. For a PDSCH reception group associated with at least one PDSCH that does not overlap with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, and not across both SBFD symbols and non-SBFD symbols, and in same symbol type as the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], the UE assumes that TBs provided by a PDSCH that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], are correctly received. For a PDSCH reception group associated only with PDSCHs that overlap with UL symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first SLIV if the UE is not configured with [sbfd-Configuration2-Transmission], the UE generates a NACK value for the PDSCH reception group. If a UE is provided either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 and neither provided nrofHARQ-BundlingGroups nor harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport blocks for PDSCH receptions. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NPDSCH,c HARQ-ACK information bits for the first TBs in the ascending order of the starting of PDSCH receptions and, if applicable, generates NPDSCH,c HARQ-ACK information bits for the second TBs in the ascending order of the starting of PDSCH receptions. For a PDSCH reception that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first PDSCH reception of the NPDSCH,c PDSCH receptions if the UE is not configured with [sbfd-Configuration2-Transmission], the UE generates a NACK value for the first TB and, if applicable, generates a NACK value for the second TB in the PDSCH reception. If NPDSCH,c<NPDSCH,cmax, the UE generates a NACK value for the last NTB,cDL∙NPDSCH,cmax-NTB,cDL∙NPDSCH,c HARQ-ACK information bits where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c and NPDSCH,cmax is determined by the maximum number of SLIVs amongst all rows of the TDRA table configured by either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3. If a UE is provided either pdsch-TimeDomainAllocationListForMultiPDSCH or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 and harq-ACK-SpatialBundlingPUCCH and not provided nrofHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over PDSCH receptions for PDSCH receptions scheduled by a DCI format on the serving cell c. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NPDSCH,c HARQ-ACK information bits for the PDSCH receptions in the ascending order of the starting of PDSCH receptions after binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of each PDSCH reception. For a PDSCH reception that overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, or across both SBFD symbols and non-SBFD symbols, or in different symbol type from the first PDSCH reception of the NPDSCH,c PDSCH receptions if the UE is not configured with [sbfd-Configuration2-Transmission], the UE generates a NACK value for the PDSCH reception. If NPDSCH,c<NPDSCH,cmax, the UE generates a NACK value for the last NPDSCH,cmax-NPDSCH,c HARQ-ACK information bits. If a UE - is provided pdsch-TimeDomainAllocationListForMultiPDSCH and, if provided, nrofHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and - is not provided pdsch-TimeDomainAllocationListForMultiPDSCH or is provided nrofHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications - NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for - SPS PDSCH reception, - any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and - PDSCH reception scheduled by a DCI format scheduling one PDSCH - PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells, - NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and - if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and - instead of generating one or two HARQ-ACK information bits per PDSCH for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits for the PDSCH receptions scheduled by a DCI format, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided nrofHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided nrofHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1. If for a serving cell c where the UE is provided nrofHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided nrofHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax HARQ-ACK information bits for serving cell c. - The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable. - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook. If OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with nHARQ-ACK,TBG=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cTBGmodTDNHARQ-ACK,maxTBG,max+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,TBG where - if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH receptions for any serving cell c from the NcellsDL,TBG serving cells with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information that the UE detects within the M PDCCH monitoring occasions - if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,TBG serving cells that the UE detects within the M PDCCH monitoring occasions - VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,TBG serving cells in any of the M PDCCH monitoring occasions - UDAI,cTBG is the total number of DCI formats scheduling more than one PDSCH receptions with TBG-based HARQ-ACK information or with TB-based HARQ-ACK information for any serving cell c from the NcellsDL,TBG serving cells that the UE detects within the M PDCCH monitoring occasions for serving cell c. UDAI,cTBG=0 if the UE does not detect any DCI format scheduling more than one PDSCH receptions for serving cell c in any of the M PDCCH monitoring occasions - if harq-ACK-SpatialBundlingPUCCH is provided, - if nrofHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of PDSCH groups that include at least one PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated if provided, and not across both SBFD symbols and non-SBFD symbols, and in same symbol type as the first PDSCH reception if the UE is not configured with [sbfd-Configuration2-Transmission], that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if nrofHARQ-BundlingGroups is not provided, Nm,creceived,TBG is the number of PDSCHs that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if harq-ACK-SpatialBundlingPUCCH is not provided, - if nrofHARQ-BundlingGroups is provided, Nm,creceived,TBG is the number of TBGs including at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated if provided, and not across both SBFD symbols and non-SBFD symbols, and in same symbol type as the first PDSCH reception if the UE is not configured with [sbfd-Configuration2-Transmission], that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH - if nrofHARQ-BundlingGroups is not provided, Nm,creceived,TBG is the number of transport blocks in PDSCHs that the UE receives in serving cell c from the NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH. If a UE is provided by mc-DCI-SetofCellsToAddModList a number of sets of serving cells and is provided USS sets to monitor PDCCH for detection of DCI format 1_3, the UE separately applies the following procedures for determining a corresponding second Type-2 HARQ-ACK sub-codebook for scheduling cells associated with DCI format 1_3 that - schedules PDSCH receptions on more than one serving cells from a set of serving cells, and/or - does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH reception on one or more serving cells from the set of serving cells - in the following, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes from the procedures for determining a first Type-2 HARQ-ACK sub-codebook that is associated with unicast SPS PDSCH receptions or with any unicast DCI format scheduling a PDSCH reception on a single serving cell, or has associated HARQ-ACK information without scheduling a PDSCH reception as described in this clause. The UE appends the second Type-2 HARQ-ACK sub-codebook to the first Type-2 HARQ-ACK sub-codebook. Denote by NC-DAIDL the number of bits for the counter DAI field in DCI format 1_3 and set TD=2NC-DAIDL. Denote by VC-DAI,c,mDL the value of the counter DAI in a DCI format 1_3 scheduling PDSCH receptions on more than one serving cells among the more than one serving cells, in PDCCH monitoring occasion m according to Table 9.1.3-1. Denote by VT-DAI,mDL the value of the total DAI in DCI format 1_3 scheduling PDSCH receptions on more than one cells in PDCCH monitoring occasion m according to Table 9.1.3-1. The UE assumes a same value of total DAI in all DCI formats 1_3 in PDCCH monitoring occasion m that schedule more than one PDSCH receptions on more than one serving cells from a set of serving cells. The UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK, for a total number of OACK HARQ-ACK information bits in the second Type-2 HARQ-ACK sub-codebook according to the following pseudo-code. Set Ncells,setDL,max to the maximum number of serving cells in ScheduledCell-ListDCI-1-3 of a set of serving cells provided by MC-DCI-SetofCells, across the number of sets of serving cells, that can be scheduled PDSCH receptions by DCI format 1_3 Set NsetsTB,max to the maximum total number of TBs in PDSCH receptions that can be scheduled by a DCI format 1_3 over more than one serving cells in a set of serving cells across the number of sets of serving cells Set NsetsDL to the number of sets of serving cells MC-DCI-SetofCells in a PUCCH group Set NcellsDL to the number of serving cells, across NsetsDL sets of serving cells in the PUCCH group Set c to the index of serving cells, c=0,…, NcellsDL-1, a lower index corresponds to a lower RRC index of a corresponding serving cell - if the UE indicates type2-HARQ-ACK-Codebook and receives NPDSCH, cm>1 PDSCHs on a serving cell c that are scheduled by NPDCCH, cm DCI formats 1_3 in PDCCH receptions at a same PDCCH monitoring occasion m, where - each of the DCI formats 1_3 schedules more than one PDSCH receptions on respective more than one serving cells, - c is the smallest cell index among the respective more than one serving cells that are scheduled by each of the NPDCCH, cm DCI formats 1_3, and - c is same across the NPDCCH, cm DCI formats 1_3 the serving cell c is counted NPDCCH, cm times for PDCCH monitoring occasion m in increasing order of the respective earliest PDSCH reception starting time, for each of the NPDCCH, cm DCI formats 1_3, among the NPDSCH, cm PDSCH receptions Set mc to the index of a serving cell, in a set of indexes of serving cells arranged in ascending order, from the set of Ncells,setDL,max serving cells, mc=0,…, Ncells,setDL,max-1 Set m=0 – PDCCH monitoring occasion index for detection of a DCI format 1_3 scheduling PDSCH receptions on more than one serving cells from a set of serving cells: lower index corresponds to earlier PDCCH monitoring occasion Set j=0 Set Vtemp=0 Set Vtemp2=0 Set Vs=∅ Set M to the number of PDCCH monitoring occasions while m<M c=0 if harq-ACK-SpatialBundlingPUCCH is not provided, while c<NcellsDL if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and the PUCCH transmission with the HARQ-ACK information starts at or after a slot for the active UL BWP change c=c+1; else if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells if VC-DAI,c,mDL≤Vtemp j=j+1; end if Vtemp=VC-DAI,c,mDL; if VT-DAI,mDL=∅ Vtemp,2=VC-DAI,c,mDL; else Vtemp,2=VT-DAI,mDL; end if cnt=0; mc=0; while mc<Ncells,setDL,max if the UE is scheduled PDSCH reception on serving cell mc, if any, from the more than one serving cells if PDCCH monitoring occasion m is before an active DL BWP change on serving cell mc, and the active DL BWP change is not triggered in PDCCH monitoring occasion m, and the PUCCH with the HARQ-ACK information starts at or after a slot for the active DL BWP change, if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc, oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = NACK; oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+1+cntACK = NACK; cnt=cnt+2; else oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = NACK; cnt=cnt+1; end if else if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+1+cntACK = HARQ-ACK information bit corresponding to the second transport block of this cell cnt=cnt+2; else oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the transport block of this cell cnt=cnt+1; end if end if end if mc=mc+1; end while while cnt< NsetsTB,max oNsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+cntACK= NACK; cnt=cnt+1; end while Vs=Vs∪NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1, …, NsetsTB,max⋅TD⋅j+NsetsTB,max⋅VC-DAI,c,mDL-1+NsetsTB,max-1; end if c=c+1; end if end while else while c<NcellsDL if PDCCH monitoring occasion m is before an active UL BWP change on the serving cell of PUCCH transmission if the UE is provided pucch-sSCellDyn or pucch-sSCellDynDCI-1-2 or pucch-sSCellDynDCI-1-3, or an active UL BWP change on the PCell if the UE is not provided pucch-sSCellDyn and pucch-sSCellDynDCI-1-2 and pucch-sSCellDynDCI-1-3, and the PUCCH transmission with the HARQ-ACK information starts at or after a slot for the active UL BWP change c=c+1; else if there is a PDSCH reception on serving cell c that is scheduled by a DCI format scheduling more than one PDSCHs that provide respective more than one transport blocks with enabled HARQ-ACK information on more than one serving cells, where the DCI format is associated with a PDCCH reception in PDCCH monitoring occasion m and c is the smallest serving cell index among the more than one serving cells if VC-DAI,c,mDL≤Vtemp j=j+1; end if Vtemp=VC-DAI,c,mDL; if VT-DAI,mDL=∅ Vtemp,2=VC-DAI,c,mDL; else Vtemp,2=VT-DAI,mDL; end if cnt=0; mc=0; while mc<Ncells,setDL,max if the UE is scheduled PDSCH reception for transport blocks with enabled HARQ-ACK information on serving cell mc, if any, from the more than one serving cells if PDCCH monitoring occasion m is before an active DL BWP change on serving cell mc, and the active DL BWP change is not triggered in PDCCH monitoring occasion m, and the PUCCH with the HARQ-ACK information starts at or after a slot for the active DL BWP change, oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅(VC-DAI,c,mDL-1)+cntACK = NACK; else if maxNrofCodeWordsScheduledByDCI is 2 for serving cell mc if the PDSCH reception provides two transport blocks oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = binary AND operation of the HARQ-ACK information bits corresponding to the first and second transport blocks of this cell else oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK = HARQ-ACK information bit corresponding to the first transport block of this cell end if else oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK= HARQ-ACK information bit of this cell end if cnt=cnt+1; end if end if mc=mc+1; end while while cnt< Ncells,setDL,max oNcells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+cntACK= NACK; cnt=cnt+1; end while Vs=Vs∪Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+1…, Ncells,setDL,max⋅TD⋅j+Ncells,setDL,max⋅VC-DAI,c,mDL-1+Ncells,setDL,max-1; end if c=c+1; end if end while end if m=m+1; end while Vtemp=j mod4TD×4TD+Vtemp; if UE does not set Vtemp2=VT-DAIUL and TD=2 Vtemp2=Vtemp; end if j=j×TD4; if Vtemp2<Vtemp j=j+1; end if if harq-ACK-SpatialBundlingPUCCH is not provided, OACK=NsetsTB,max⋅4⋅j+Vtemp2 else OACK=Ncells,setDL,max⋅4⋅j+Vtemp2 end if oiACK=NACK for any i∈0,1,⋯,OACK-1\Vs . If - OACK+OSR+OCSI≤11 and NsetsDL>0, and - a UE is not provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for any serving cell in a PUCCH group, or is provided nrofHARQ-BundlingGroups with value of 1 for any serving cell for which the UE is provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, or - for PDSCH receptions scheduled by a DCI format 1_3 that does not schedule more than one PDSCH reception on each serving cell, or - for PDSCH receptions scheduled by a DCI format 1_3 on a serving cell when the UE is provided nrofHARQ-BundlingGroups with value of 1 for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived where - in the following - a DCI format 1_3 schedules more than one PDSCH receptions providing transport blocks with enabled HARQ-ACK information - a dormancy indication is considered as a PDSCH reception providing a single transport block with enabled HARQ-ACK information - VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions. - UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions. - NTB,maxDL,MC=NsetsTB,max if harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL,MC=Ncells,setDL,max. - Nm,sreceived is - the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided - the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided If OACK+OSR+OCSI≤11 and NsetsDL>0, for obtaining a PUCCH transmission power as described in clause 7.2.1, the UE determines nHARQ-ACK=nHARQ-ACK,0+nHARQ-ACK,1, where nHARQ-ACK,0 is the value of nHARQ-ACK for the first Type-2 HARQ-ACK sub-codebook and nHARQ-ACK,1 is the value of nHARQ-ACK for the second Type-2 HARQ-ACK sub-codebook that is determined as nHARQ-ACK=VDAI,mlastDL-s=0NsetsDL-1UDAI,smodTDNTB,maxDL,MC+s=0NsetsDL-1m=0M-1Nm,sreceived where - in the following - a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on more than one serving cells, or - a DCI format 1_3 schedules more than one PDSCH receptions, providing transport blocks with enabled HARQ-ACK information, on a single serving cell c, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1 - a dormancy indication is considered as a PDSCH reception, providing a single transport block with enabled HARQ-ACK information, corresponding to the first SLIV in the row of pdsch-TimeDomainAllocationList or pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3, if the UE is not provided nrofHARQ-BundlingGroups for a corresponding serving cell - a dormancy indication is considered as one or more PDSCH receptions providing respective one or more transport blocks with enabled HARQ-ACK information, corresponding to a TBG with smallest index or a PDSCH reception group with smallest index, if the UE is provided nrofHARQ-BundlingGroups for a corresponding serving cell - VDAI,mlastDL is the value of the total DAI field in a last DCI format 1_3 the UE detects in a last PDCCH monitoring occasion within the M PDCCH monitoring occasions where the UE detects at least one DCI format 1_3. VDAI,mlastDL=0 if the UE does not detect any DCI format 1_3 in any of the M PDCCH monitoring occasions. - UDAI,s is the total number of DCI format 1_3 that the UE detects within the M PDCCH monitoring occasions for the set s of serving cells. UDAI,s=0 if the UE does not detect any DCI format 1_3 associated with scheduling on set s of serving cells in any of the M PDCCH monitoring occasions. - NTB,maxDL,MC is - NTB,maxDL,MC=NsetsHARQ-ACK,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is provided for at least one serving cell in the PUCCH group; - NTB,maxDL,MC=NsetsTB,max if pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 is not provided for any serving cell in the PUCCH group and harq-ACK-SpatialBundlingPUCCH is not provided; - otherwise, NTB,maxDL,MC=Ncells,setDL,max. - Nm,sreceived is - the number of transport blocks, in PDSCH receptions not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is not provided - the number of more than one PDSCHs, not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, scheduled by a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is not provided - the number of transport block groups (TBGs) with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is not provided and nrofHARQ-BundlingGroups is provided - the number of PDSCH groups, with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated if provided, from/based on PDSCH receptions associated with a DCI format 1_3 that the UE detects in PDCCH monitoring occasion m for set s of serving cells, if harq-ACK-SpatialBundlingPUCCH is provided and nrofHARQ-BundlingGroups is provided. If a UE is provided pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for a serving cell in a set of serving cells provided by MC-DCI-SetofCells in a PUCCH group, the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-codes with the following modifications - the first HARQ-ACK sub-codebook is associated with - unicast SPS PDSCH receptions - any unicast DCI format scheduling a single PDSCH reception on a serving cell - any unicast DCI format having associated HARQ-ACK information without scheduling a PDSCH reception - any DCI format 1_3 scheduling more than one PDSCH receptions on a serving cell c for which the UE is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max=1 - the second HARQ-ACK sub-codebook is associated with DCI format 1_3 that - schedules PDSCH receptions on more than one serving cells from the set of serving cells, or - schedules more than one PDSCH receptions on a serving cell c, from the set of serving cells, for which the UE is not provided nrofHARQ-BundlingGroups or is provided nrofHARQ-BundlingGroups with value NHARQ-ACK,cTBG,max>1, or - does not include a SCell dormancy indication field or the SCell dormancy indication field is reserved, indicates SCell dormancy, and schedules PDSCH receptions on one or more serving cells from the set of serving cells - in the above, and for the purpose of providing HARQ-ACK information corresponding to SCell dormancy indication by the DCI format 1_3 - the UE assumes that the UE receives one PDSCH, with first SLIV in the row of pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that the PDSCH provides one transport block that the UE correctly decodes, if the UE is not provided nrofHARQ-BundlingGroups for the serving cell - the UE assumes that the UE receives one or more PDSCHs associated with a TBG with smallest index, or a PDSCH reception group with smallest index, among one or more PDSCH receptions scheduled by the DCI format 1_3, on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication, as described in Clause 10.3, and that each of the one or more PDSCHs provides one transport block that the UE correctly decodes if the UE is provided nrofHARQ-BundlingGroups for the serving cell - the UE assumes incorrect decoding for transport blocks in each of the remaining PDSCH receptions scheduled by the DCI format 1_3 on the serving cell associated with fields in DCI format 1_3 for SCell dormancy indication - instead of generating NsetsTB,max HARQ-ACK information bits when harq-ACK-SpatialBundlingPUCCH is not provided, or generating Ncells,setDL,max HARQ-ACK information bits when harq-ACK-SpatialBundlingPUCCH is provided, for the PDSCH receptions scheduled by a DCI format 1_3, the UE generates - NsetsHARQ-ACK,max HARQ-ACK information bits for the PDSCH receptions scheduled by the DCI format 1_3, where NsetsHARQ-ACK,max is the maximum number, over the number NsetsDL sets of serving cells provided by MC-DCI-SetofCells in the PUCCH group, of the sum of - NTB,cDL⋅NPDSCH,cmax if nrofHARQ-BundlingGroups is not provided for a serving cell c, or - NTB,cDL⋅NHARQ-ACK,cTBG,max if nrofHARQ-BundlingGroups is provided for the serving cell c, across serving cells of a respective set of serving cells MC-DCI-SetofCells that can be scheduled PDSCH reception by a DCI format 1_3, where - NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, NTB,cDL=1; - NPDSCH,cmax is the maximum number of SLIVs over all rows of the TDRA table provided by pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for serving cell c; and - NHARQ-ACK,cTBG,max is provided by nrofHARQ-BundlingGroups for serving cell c - If the UE is provided with pdsch-TimeDomainAllocationListForMultiPDSCH-DCI-1-3 for the serving cell mc in the set s, by applying the above procedure, the UE generates - NTB,cDL⋅NPDSCH,cmax HARQ-ACK information bits if nrofHARQ-BundlingGroups is not provided for a serving cell mc - NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits if nrofHARQ-BundlingGroups is provided for the serving cell mc - If for a set s of serving cells provided by MC-DCI-SetofCells, NsetsHARQ-ACK,s is equal to the sum of - NTB,cDL⋅NPDSCH,cmax if nrofHARQ-BundlingGroups is not provided for a serving cell c, or - NTB,cDL⋅NHARQ-ACK,cTBG,max if nrofHARQ-BundlingGroups is provided for the serving cell c, across the serving cells of the set s that are scheduled PDSCH receptions by a DCI format 1_3, and NsetsHARQ-ACK,s <NsetsHARQ-ACK,max, the UE generates NACK values for the last NsetsHARQ-ACK,max-NsetsHARQ-ACK,s HARQ-ACK information bits corresponding to the DCI format 1_3 - The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook. - The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook. Table 9.1.3-1: Value of counter DAI for NC-DAIDL=2 and of total DAI DAI MSB, LSB; VC-DAIDL or VT-DAIDL; Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, or number of PDCCH monitoring occasions associated with PDCCH for scheduling PDSCH receptions on more than one cells, denoted as Y and Y≥1. 0,0; 1; Y-1modTD+1=1. 0,1; 2; Y-1modTD+1=2. 1,0; 3; Y-1modTD+1=3. 1,1; 4; Y-1modTD+1=4. Table 9.1.3-1A: Value of counter DAI for NC-DAIDL=1 DAI; VC-DAIDL; Number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH transmission(s) associated with PDCCH or PDCCH generating a HARQ-ACK information bit without scheduling a PDSCH reception or providing TCI state update is present, denoted as Y and Y≥1. 0; 1; Y-1modTD+1=1. 1; 2; Y-1modTD+1=2. <*************************************omitted***************************************>

TP#7: SPS collision handling

TS 38.214, v19.1.0 ## 5.1 UE procedure for receiving the physical downlink shared channel For downlink, a maximum of 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell as defined in [13, TS 38.306]. The number of processes the UE may assume will at most be used for the downlink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPDSCH or nrofHARQ-ProcessesForPDSCH-v1700, and when no configuration is provided the UE may assume a default number of 8 processes. <*************************************omitted***************************************> If more than one PDSCH on a serving cell each without a corresponding PDCCH transmission are in a slot, after resolving overlapping with symbols in the slot indicated as uplink by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated, or across both SBFD symbols and non-SBFD symbols, or in invalid symbol type if the UE is not configured with [sbfd-Configuration2-Transmission], or determined as non-active periods of cell DTX, if the serving cell is activated with cell DTX, based on [10, TS 38.321], a UE receives one or more PDSCHs without corresponding PDCCH transmissions in the slot as specified below. ‒ Step 0: set j=0, where j is the number of selected PDSCH(s) for decoding. Q is the set of activated PDSCHs without corresponding PDCCH transmissions within the slot ‒ Step 1: A UE receives one PDSCH with the lowest configured sps-ConfigIndex within Q, set j=j+1. Designate the received PDSCH as survivor PDSCH. ‒ Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping (even partially) with the survivor PDSCH in step 1 are excluded from Q. ‒ Step 3: Repeat step 1 and 2 until Q is empty or j is equal to the number of unicast/multicast PDSCHs in a slot supported by the UE. A UE capable of PDSCH repetitions for broadcast channels, which assumed the DCI format 1_0 in the Type0 PDCCH CSS of searchSpaceZero transmitted with two inter-slot repetitions may assume that PDSCHs scheduled by the DCI format 1_0 have also been transmitted with inter-slot repetitions in the same slots as the Type0 PDCCH CSS, with the same RV as indicated by the DCI format 1_0. For a cell detected in cell search procedure with synchronization raster defined in Table 5.4.3.1-2 or Table 5.4.3.1-3 of [8, TS 38.101-1], the size of CORESET 0 for the cell in this clause refers to the size of punctured CORESET 0 as defined in clause 7.3.2.2 of [4, TS 38.211] if any. <*************************************omitted***************************************>

TP#8: HARQ-ACK feedback for SPS PDSCHs

TS 38.213, v19.1.0 ### 9.1.2 Type-1 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. In clauses 9.1.2, 9.1.2.1, and 9.1.2.2, if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static for only one of unicast or multicast HARQ-ACK codebook, the Type-1 HARQ-ACK codebook is generated considering only one of respective unicast or multicast configurations for PDSCH receptions or for PDCCH monitoring for detection of DCI formats. A UE does not provide a Type-1 HARQ-ACK codebook if the Type-1 HARQ-ACK codebook would include only HARQ-ACK information for transport blocks associated with HARQ processes with disabled HARQ-ACK information. <*************************************omitted***************************************> In the following pseudo-code, SPS PDSCH receptions associated with a SPS PDSCH configuration are activated by a DCI format with CRC scrambled by a CS-RNTI or by a DCI format with CRC scrambled by a G-CS-RNTI. Set NcellsDL to the number of serving cells configured to the UE Set NcSPS to the number of SPS PDSCH configurations configured to the UE for serving cell c Set NcDL to the number of DL slots for SPS PDSCH receptions on serving cell c with HARQ-ACK information multiplexed on the PUCCH Set j=0 – HARQ-ACK information bit index Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell while c<NcellsDL Set s=0 – SPS PDSCH configuration index: lower indexes correspond to lower RRC indexes of corresponding SPS configurations while s<NcSPS Set nD=0 – slot index while nD<NcDL if { a UE is configured to receive SPS PDSCHs from slot nD-NPDSCHrepeat+1 to slot nD for SPS PDSCH configuration s on serving cell c, excluding SPS PDSCHs that are not required to be received in any slot among overlapping SPS PDSCHs, if any according to [6, TS 38.214], or based on a UE capability for a number of PDSCH receptions in a slot according to [6, TS 38.214], or due to overlapping with a set of symbols - indicated as uplink by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, or across both SBFD symbols and non-SBFD symbols, or in invalid symbol type if the UE is not configured with [sbfd-Configuration2-Transmission], and/or - determined as non-active period of cell DTX [11, TS 38.321] where, for unicast SPS PDSCHs, NPDSCHrepeat is provided by pdsch-AggregationFactor-r16 in SPS-Config or, if pdsch-AggregationFactor-r16 is not included in SPS-Config, by pdsch-AggregationFactor in PDSCH-config and, for multicast SPS PDSCHs, NPDSCHrepeat is provided by repetitionNumber if contained in an entry indicated by the time domain resource assignment field in the DCI format scheduling the PDSCH repetition, or provided by pdsch-AggregationFactor-r16 if included in SPS-Config or, otherwise, NPDSCHrepeat=1, and HARQ-ACK information for the SPS PDSCH is associated with the PUCCH } ojACK = HARQ-ACK information bit for this SPS PDSCH reception j=j+1; end if nD=nD+1; end while s=s+1; end while c=c+1; end while <*************************************omitted***************************************>

TP#9: TDD CA collision handling

TS 38.213, v19.1.0 ## 11.1 Slot configuration A slot format includes downlink symbols, uplink symbols, and flexible symbols. The following are applicable for each serving cell. If a UE is provided tdd-UL-DL-ConfigurationCommon, the UE sets the slot format per slot over a number of slots as indicated by tdd-UL-DL-ConfigurationCommon. <*************************************omitted***************************************> If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, where the symbol is configured by higher layers as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, - uplink, if the symbol is flexible or is indicated as SBFD by tdd-UL-DL-ConfigurationCommon and the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible or is indicated as SBFD by tdd-UL-DL-ConfigurationCommon and the UE is configured by higher layers to receive PDCCH, PDSCH or CSI-RS on the symbol. And if another cell among the cells configured with directionalCollisionHandling-r16 operates in the same frequency band as the reference cell, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated to indicate a symbol as downlink, and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol or SBFD symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, if the reference cell and another cell among the cells configured with directionalCollisionHandling-r16 operate in different frequency bands, the UE - assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layer configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink and when applicable, indicated as non-SBFD or uplink on another cell and as uplink or downlink and when applicable, indicated as non-SBFD for the reference cell, respectively, - transmits a signal/channel scheduled by a DCI format on a symbol of another cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell, and when applicable, indicated as non-SBFD - is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols or SBFD symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on another cell, and regardless of whether the reference cell and another cell operate in same or different frequency bands, the UE - does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell - does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol or SBFD symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell - does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and when applicable, indicated as non-SBFD, or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell - does not transmit a SRS that is configured by higher layers on a set of symbols on another cell if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and when applicable, indicated as non-SBFD, or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell - does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell - assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, and when applicable, indicated as non-SBFD, on another cell to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively. After the UE applies the procedures described above for directional collision handling within the set of cells that have been configured with directionalCollisionHandling-r16, the UE does not expect any directional collision among the serving cells that the UE is not capable of simultaneous transmission and reception. <*************************************omitted***************************************>

TP#10: UL cancellation DCI

TS 38.213, v19.1.0 ## 11.2A Cancellation indication If a UE is provided UplinkCancellation, the UE is provided, in one or more serving cells, search space sets for monitoring the first PDCCH candidate with a CCE aggregation level of LCI CCEs of each search space set for detection of a DCI format 2_4 [5, TS 38.212] with a CI-RNTI provided by ci-RNTI as described in clause 10.1. UplinkCancellation additionally provides to the UE - a set of serving cells, by ci-ConfigurationPerServingCell, that includes a set of serving cell indexes and a corresponding set of locations for fields in DCI format 2_4 by positionInDCI - a number of fields in DCI format 2_4, by positionInDCI-forSUL, for each serving cell for a SUL carrier, if the serving cell is configured with a SUL carrier - an information payload size for DCI format 2_4 by dci-PayloadSize-ForCI - an indication for time-frequency resources by timeFrequencyRegion For a serving cell having an associated field in a DCI format 2_4, for the field denote by - NCI a number of bits provided by ci-PayloadSize - BCI a number of PRBs provided by frequencyRegionforCI in timeFrequencyRegion - TCI a number of symbols, excluding symbols for reception of SS/PBCH blocks and DL symbols indicated by tdd-UL-DL-ConfigurationCommon, and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, from a number of symbols that - is provided by timeDurationforCI in timeFrequencyRegion, if the PDCCH monitoring periodicity for the search space set with the DCI format 2_4 is one slot and there are more than one PDCCH monitoring occasions in a slot, or - is equal to the PDCCH monitoring periodicity, otherwise. - GCI a number of partitions for the TCI symbols provided by timeGranularityforCI in timeFrequencyRegion GCI sets of bits from the MSB of the NCI bits have a one-to-one mapping with GCI groups of symbols where each of the first GCI-TCI+TCIGCI⋅GCI groups includes TCIGCI symbols and each of the remaining TCI-TCIGCI⋅GCI groups includes TCIGCI symbols. A UE determines a symbol duration with respect to a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. For a group of symbols, NBI=NCIGCI bits from MSB of each set of bits have a one-to-one mapping with NBI groups of PRBs where each of the first NBI-BCI+BCINBI⋅NBI groups includes BCINBI PRBs and each of the remaining BCI-BCINBI⋅NBI groups includes BCINBI PRBs. A UE determines a first PRB index as NRFRstart=Ocarrier+RBstart and a number of contiguous RBs as BCI=LRB from frequencyRegionforCI that indicates an offset RBstart and a length LRB as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB or FrequencyInfoUL that indicates Ocarrier for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection. An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of the PDCCH reception where the UE detects the DCI format 2_4. A UE that detects a DCI format 2_4 for a serving cell cancels a PUSCH transmission or an actual repetition of a PUSCH transmission [6, TS 38.214] if the PUSCH transmission is with repetition Type B, as determined in clauses 9 and 9.2.5 or in clause 6.1 of [6, TS 38.214], or an SRS transmission on the serving cell if, respectively, - the transmission is PUSCH with priority 0, if the UE is provided uplinkCancellationPriority, - a group of symbols, from the TCI symbols, has at least one bit value of '1' in the corresponding set of NBI bits in the DCI format 2_4 and includes a symbol of the (repetition of the) PUSCH transmission or of the SRS transmission, and - a group of PRBs, from the BCI PRBs, has a corresponding bit value of '1' in the set of bits corresponding to the group of symbols in the DCI format 2_4 and includes a PRB of the (repetition of the) PUSCH transmission or of the SRS transmission, where - the cancellation of the (repetition of the) PUSCH transmission includes all symbols from the earliest symbol of the (repetition of the) PUSCH transmission that is in a group of symbols having corresponding bit values of '1' in the DCI format 2_4; - the cancellation of the SRS transmission includes only symbols that are in one or more groups of symbols having corresponding bit values of '1' in the DCI format 2_4. If, based on an indication by a DCI format 2_4, a UE cancels a PUSCH transmission or an SRS transmission, the UE does not expect to be scheduled by a second DCI format to transmit a PUSCH or an SRS over symbols that include symbols of the cancelled PUSCH transmission or SRS transmission, where the last symbol of the PDCCH reception providing the second DCI format is no earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. <*************************************omitted***************************************>

TP#11: PUCCH resource in SBFD symbols

TS 38.213, v19.1.0 ### 9.2.1 PUCCH Resource Sets If a UE does not have dedicated PUCCH resource configuration, provided by PUCCH-ResourceSet in PUCCH-Config, a PUCCH resource set is provided by pucch-ResourceCommon through an index to a row of Table 9.2.1-1 for transmission of HARQ-ACK information on PUCCH in an initial UL BWP of NBWPsize PRBs. For operation in FR2-2, nrofPRBs provided in PUCCH-ConfigCommon can also provide a number of NRB RBs for the PUCCH resource set; otherwise NRB=1. The PUCCH resource set includes sixteen resources, each corresponding to a PUCCH format, a first symbol, a duration, a PRB offset RBBWPoffset, and a cyclic shift index set for a PUCCH transmission. The UE transmits a PUCCH using frequency hopping if not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon; otherwise, the UE transmits a PUCCH without frequency hopping. An orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 in Table 9.2.1-1 except when index 3, 7, or 11 is indicated by pucch-ResourceCommon and useInterlacePUCCH-PUSCH in BWP-UplinkCommon is provided. The UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant as described in clause 8.3. If a UE is not provided any of pdsch-HARQ-ACK-Codebook, pdsch-HARQ-ACK-Codebook-r16, or pdsch-HARQ-ACK-OneShotFeedback, the UE generates at most one HARQ-ACK information bit. If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤15, as rPUCCH=2⋅nCCE,0NCCE+2⋅∆PRI, where NCCE is a number of CCEs in a CORESET of a PDCCH reception with the DCI format, as described in clause 10.1, nCCE,0 is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. When the PDCCH reception by a UE includes first and second PDCCH candidates from respective first and second search space sets, as described in clause 10.1, the CORESET and nCCE,0 are associated with the search space set having the smaller index. If - the first search space set has larger index than the second search space set and includes the first PDCCH candidate and a third PDCCH candidate that have same first CCE index and CCE aggregation levels 8 and 16, or 16 and 8, respectively, - the second search space set includes the second PDCCH candidate that has same index and same CCE aggregation level as the first PDCCH candidate, and a fourth PDCCH candidate that has same index and same CCE aggregation level as the third PDCCH candidate, - the CORESET associated with the first search space set has cce-REG-MappingType = 'nonInterleaved' and has duration of one symbol, and - the second PDCCH candidate has different first CCE index than the fourth PDCCH candidate the UE determines nCCE,0 from the PDCCH candidate with CCE aggregation level 16 among the second PDCCH candidate and the fourth PDCCH candidate. If the PUCCH transmission is associated with a PRACH transmission in first PRACH occasions, as described in clause 8, NBWPsize is the number of RBs in the active UL BWP and RBUL_SBstart=0; otherwise, NBWPsize is the number of RBs in both the active UL BWP and in the UL sub-band and RBUL_SBstart is the index of the first RB from the RBs that are in the active UL BWP and in the UL sub-band as described in clause 11.1. If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBUL_SBstart+RBBWPoffset⋅NRB+rPUCCHNCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBUL_SBstart+NBWPsize-RBBWPoffset∙NRB-1+rPUCCHNCS∙NRB, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBUL_SBstart+NBWPsize-RBBWPoffset∙NRB-1+rPUCCH-8NCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBUL_SBstart+RBBWPoffset∙NRB+rPUCCH-8NCS∙NRB - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as If a UE is provided a PUCCH resource by pucch-ResourceCommon and is provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines for the PUCCH resource an interlace index m as m=m0+rPUCCHNCSmodM where M is a number of interlaces [4, TS 38.211] and m0=RBBWPoffset is an interlace index offset and RBBWPoffset is as given in Table 9.2.1-1 - the UE determines an initial cyclic shift index in a set of initial cyclic shift indexes as rPUCCHmodNCS, where NCS is the total number of initial cyclic shifts indexes in the set of initial cyclic shift indexes in Table 9.2.1-1 - if pucch-ResourceCommon indicates - index 0: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH≥10 - index 1 or 2: the first symbol is 9 for a PUCCH resource with PUCCH format 0 if rPUCCH=15 - index 3, 7, or 11: an orthogonal cover code with index 1 is used for a PUCCH resource with PUCCH format 1 if rPUCCH≥10; otherwise, an orthogonal cover code with index 0 is used for a PUCCH resource with PUCCH format 1 - the UE does not expect pucch-ResourceCommon to indicate index 15 Table 9.2.1-1: PUCCH resource sets before dedicated PUCCH resource configuration Index; PUCCH format; First symbol; Number of symbols; PRB offset RBBWPoffset; Set of initial CS indexes. 0; 0; 12; 2; 0; {0, 3}. 1; 0; 12; 2; 0; {0, 4, 8}. 2; 0; 12; 2; 3; {0, 4, 8}. 3; 1; 10; 4; 0; {0, 6}. 4; 1; 10; 4; 0; {0, 3, 6, 9}. 5; 1; 10; 4; 2; {0, 3, 6, 9}. 6; 1; 10; 4; 4; {0, 3, 6, 9}. 7; 1; 4; 10; 0; {0, 6}. 8; 1; 4; 10; 0; {0, 3, 6, 9}. 9; 1; 4; 10; 2; {0, 3, 6, 9}. 10; 1; 4; 10; 4; {0, 3, 6, 9}. 11; 1; 0; 14; 0; {0, 6}. 12; 1; 0; 14; 0; {0, 3, 6, 9}. 13; 1; 0; 14; 2; {0, 3, 6, 9}. 14; 1; 0; 14; 4; {0, 3, 6, 9}. 15; 1; 0; 14; NBWPsize4; {0, 3, 6, 9}. If a UE has dedicated PUCCH resource configuration, the UE is provided by higher layers with one or more PUCCH resources. A PUCCH resource includes the following parameters: - a PUCCH resource index provided by pucch-ResourceId - an index of the first PRB prior to frequency hopping or for no frequency hopping by startingPRB, or by startingPRB-SBFD if the PUCCH transmission is over SBFD symbols as described in clause 11.1 and startingPRB-SBFD is provided, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - an index of the first PRB after frequency hopping by secondHopPRB, or by secondHopPRB-SBFD if the PUCCH transmission is over SBFD symbols as described in clause 11.1 and secondHopPRB-SBFD is provided, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - an indication for intra-slot frequency hopping by intraSlotFrequencyHopping, if a UE is not provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - an index of a first interlace by interlace0, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - if provided, an index of a second interlace by interlace1, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - an index of an RB set by rb-SetIndex, if a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated - an indication for applying one or both of TCI-State or TCI-UL-State by applyIndicatedTCI-State, if provided - a configuration for a PUCCH format provided by format For operation with shared spectrum channel access, the UE does not expect that two hops of a PUCCH transmission are in different RB sets. For a PUCCH transmission in SBFD symbols as described in clause 11.1, if the PUCCH transmission overlaps with PRB outsidethe PRBs that are both in the active UL BWP and in the UL sub-band, the PUCCH transmission is not transmitted. The UE expects that useInterlacePUCCH-PUSCH in BWP-UplinkCommon and useInterlacePUCCH-PUSCH in BWP-UplinkDedicated are provided either in all UL BWPs or in none of the UL BWPs for a serving cell. If a UE is provided useInterlacePUCCH-PUSCH in BWP-UplinkDedicated, the UE determines available RBs for PUCCH transmissions within the active UL BWP as the intersection of RBs corresponding to an interlace index provided by interlace0 and, if provided, interlace1, and RBs of an RB set provided by rb-SetIndex. The intersection results in Minterlace,0PUCCH RBs in the first interlace and the UE expects that Minterlace,0PUCCH is either 10 or 11. If interlace1 is provided, the intersection results in Minterlace,1PUCCH RBs in the second interlace and the UE expects that Minterlace,1PUCCH is either 10 or 11. <*************************************omitted***************************************> ### 9.2.6 PUCCH repetition procedure A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfMsg4HARQ-ACK-Repetitions. If numberOfMsg4HARQ-ACK-Repetitions provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The UE transmits any PUCCH with repetitions over NPUCCHrepeat slots before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1. In the remaining of this clause, a UE without dedicated PUCCH resource configuration determines a value of a parameter, if applicable, according to Table 9.2.1-1 and/or as specified above in this clause for a PUCCH transmission with repetitions from the UE. <***************************************omitted***************************************> For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or non-SBFD flexible symbols, or consecutive SBFD symbols as described in clause 11.1 if PUCCH PRBs within both the active UL BWP and the UL sub-band, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols For paired spectrum or supplementary uplink band, the UE determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting. <***************************************omitted***************************************>

TP#12: PUCCH repetition slot counting

### 9.2.6 PUCCH repetition procedure A UE that does not have dedicated PUCCH resource configuration and indicates a capability to transmit with repetitions a PUCCH with HARQ-ACK information [11, TS 38.321], determines a number of NPUCCHrepeat slots for repetitions of a PUCCH transmission with HARQ-ACK information based on an indication by numberOfMsg4HARQ-ACK-Repetitions. If numberOfMsg4HARQ-ACK-Repetitions provides more than one values, the DAI field in a DCI format 1_0 with CRC scrambled by a TC-RNTI scheduling a PDSCH reception that includes a UE contention resolution identity indicates NPUCCHrepeat from the more than one values. The UE transmits any PUCCH with repetitions over NPUCCHrepeat slots before dedicated PUCCH resource configuration is provided. The UE transmits each repetition of the PUCCH using frequency hopping as described in Clause 9.2.1. In the remaining of this clause, a UE without dedicated PUCCH resource configuration determines a value of a parameter, if applicable, according to Table 9.2.1-1 and/or as specified above in this clause for a PUCCH transmission with repetitions from the UE. <***************************************omitted***************************************> For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or non-SBFD flexible symbols, or consecutive SBFD symbols as described in clause 11.1 if PUCCH frequency resource not overlapping with PRB outsidethe PRBs that are both in the active UL BWP and in the UL sub-band, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols If the UE is not configured with [sbfd-Configuration2-Transmission], UE transmits PUCCH repetition only in a valid symbol type. A slot containing the transmission occasion that is not in the valid symbol type is not counted in the number of NPUCCHrepeat slots. For HARQ-ACK PUCCH repetitions, the valid symbol type is the symbol type of the first PUCCH repetition occasion. For a PUCCH transmission with SP-CSI reports, or for a PUCCH with SR, [LRR, UEIRI], or P-CSI, the valid symbol type is indicated by the respective configurations [12, TS 38.331]. For paired spectrum or supplementary uplink band, the UE determines the NPUCCHrepeat slots for a PUCCH transmission as the NPUCCHrepeat consecutive slots starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting. <***************************************omitted***************************************>

TP#13: SPS HARQ-ACK deferring

TS 38.213, v19.1.0 #### 9.2.5.4 UE procedure for deferring HARQ-ACK for SPS PDSCH If a UE is provided sps-HARQ-Deferral and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs in a first slot, if any, the UE determines a PUCCH resource for a PUCCH transmission with first HARQ-ACK information bits for SPS PDSCH receptions that the UE would report for a first time, and the PUCCH resource - is provided by SPS-PUCCH-AN-List as described in clause 9.2.1, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided - is not cancelled by an overlapping PUCCH or PUSCH transmission of larger priority index - overlaps with a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, or indicated for a SS/PBCH block by ssb-PositionsInBurst, or belonging to a CORESET associated with a Type0-PDCCH CSS set, or overlapping with RB outside UL usable PRBs in SBFD symbols the UE - determines an earliest second slot and, after performing the procedures in clauses 9.2.1 and 9.2.3 to determine a PUCCH with third HARQ-ACK information bits including second HARQ-ACK information bits and then performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs, if any, a PUSCH or a PUCCH in the earliest second slot to multiplex the third HARQ-ACK information bits that include second HARQ-ACK information bits from the first HARQ-ACK information bits , where the second HARQ-ACK information bits correspond to SPS PDSCH configurations with sps-HARQ-Deferral values that are larger than or equal to a time difference, with reference to slots for PUCCH transmissions on the primary cell, between the second slot and the slot of the SPS PDSCH reception, if any - if the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 HARQ-ACK codebook in a slot as described in clause 9.1.4, the UE stops the procedure to determine the earliest second slot in the slot - if the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines the earliest second slot and a corresponding cell based on the periodic cell switching pattern as described in clause 9.A - if the UE multiplexes the second HARQ-ACK information in a PUSCH, or in a PUCCH using a resource that is not from SPS-PUCCH-AN-List, or from n1PUCCH-AN if SPS-PUCCH-AN-List is not provided, the UE stops the procedure to determine the earliest second slot in the slot - if the UE multiplexes the second HARQ-ACK information in a first PUCCH using a resource provided by SPS-PUCCH-AN-List, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided, of smaller priority index and the UE drops the first PUCCH transmission due to an overlapping with a second PUSCH or PUCCH transmission of larger priority index, the UE stops the procedure to determine the earliest second slot in the slot - if the UE multiplexes the second HARQ-ACK information in a first PUCCH using a resource provided by SPS-PUCCH-AN-List, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided, and the PUCCH transmission is not dropped due to an overlapping with a PUSCH or PUCCH transmission of larger priority and does not have any symbol that overlaps with a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated and, when applicable, are indicated as non-SBFD by tdd-UL-DL-ConfigurationCommon, or indicated for a SS/PBCH block by ssb-PositionsInBurst, or belonging to a CORESET associated with a Type0-PDCCH CSS set, or overlapping with RB outside UL usable PRBs in SBFD symbols, the UE stops the procedure to determine the earliest second slot in the slot - the second HARQ-ACK information bits, generated as described in clause 9.1.2, are appended to fourth HARQ-ACK information bits the UE generates as described in clauses 9.1.2, 9.1.2.1, 9.1.3.1, or 9.1.5 - if the UE would receive a PDSCH providing a TB for a same HARQ process as a HARQ-ACK information bit from the second HARQ-ACK information bits prior to transmitting the PUCCH or the PUSCH, the UE does not include the HARQ-ACK information bit in the second HARQ-ACK information bits. The UE does not expect to be provided both sps-HARQ-Deferral and nrofSlots or pucch-RepetitionNrofSlots for any PUCCH resource of same priority. OACK is the number of the third HARQ-ACK information bits. If OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH transmission in the second slot, as described in clause 7.2.1, as nHARQ-ACK= nHARQ-ACK,0+nHARQ-ACK,1 where - nHARQ-ACK,0 is the number of HARQ-ACK information bits, if any, that the UE determines as described in clause 9.1.2.1, 9.1.3.1 or 9.1.5 for the fourth HARQ-ACK information bits, - nHARQ-ACK,1 is determined as described in clause 9.1.2.1 or 9.1.3.1 for the second HARQ-ACK information bits. <*************************************omitted***************************************>

- Conclusion

In this contribution, we discussed enhancements for SBFD TX/RX/measurement procedures. We have the following observations and proposals:

Proposal 1: For configuration 1, for valid symbol type determination for semi-statically configured transmissions/receptions without activation DCI,

- if the valid symbol type configuration is not provided, default valid symbol type is determined as “non-SBFD”.

Proposal 2: For CSI report based on CSI-RS on SBFD symbols, a CSI reporting subband fully overlapping with PRB outside DL usable PRBs is not expected.

Proposal 3: TP#1 is adopted for CSI reporting subband overlapping with RB outside DL subband.

Proposal 4: For each SRS resource set with usage set to 'nonCodebook', in case the associated CSI-RS is aperiodic, UE expects the associated aperiodic CSI-RS is in the same symbol type as the SRS resource set. TP#2 is adopted.

Proposal 5: TP#3 and TP#4 are adopted.

- For multi-PDSCH scheduled by single DCI:

  - The indicated HP ID is applied to the first PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols

  - HP ID is incremented by 1 for each subsequent PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

- For multi-PUSCH scheduled by single DCI:

  - The indicated HP ID is applied to the first PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols

  - HP ID is incremented by 1 for each subsequent PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

- For multi-PUSCH CG:

  - HP ID is incremented by 1 for each subsequent configured grant PUSCH not overlapping with non-SBFD DL symbol or SSB symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided

Proposal 6: TP#5 is adopted.

- For type 1 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if timeDomainHARQ-BundlingType1 is provided, HARQ-ACK for the multiple PDSCHs scheduled by single DCI is determined by binary AND operation of the HARQ-ACK information bits of PDSCH receptions that do not overlap with UL symbols, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided.

Proposal 7: TP#6 is adopted.

- For type 2 HARQ-ACK feedback for multi-PDSCH scheduled by single DCI, if nrofHARQ-BundlingGroups is provided,

- For a TBG including at least one PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and not in invalid symbol type if configuration 1 is provided, UE assumes a PDSCH overlapping with UL symbol, or across SBFD symbols and non-SBFD symbols, or in invalid symbol type if configuration 1 is provided, is correctly received.

- For a TBG including only PDSCHs overlapping with UL symbol, or across SBFD symbols and non-SBFD symbols, or in invalid symbol type if configuration 1 is provided, UE generates NACK value for the TBG.

- The number of TBGs including at least one PDSCH not overlapping with UL symbol, and not across both SBFD symbols and non-SBFD symbols, and in invalid symbol type if configuration 1 is provided, is used to determine PUCCH transmission power.

Proposal 8: TP#7 is adopted.

- The SPS PDSCH collision handling procedure is applied after excluding SPS PDSCH across SBFD and non-SBFD symbols, or in the invalid symbol type if configuration 1 is provided.

Proposal 9: TP#8 is adopted.

- For configuration 1, for SPS HARQ-ACK codebook construction, SPS PDSCHs not received due to across SBFD and non-SBFD symbols, or in the invalid symbol type if configuration 1 is provide are not included.

Proposal 10: TP#9 is adopted.

- For TDD CA directional collision handling with one SBFD cell, the legacy rule is reused with considering SBFD symbol as flexible symbol.

Proposal 11: TP#10 is adopted.

- For UL cancellation DCI format 2_4, the TCI symbols are determined excluding non-SBFD DL symbols and SSB symbols.

Proposal 12: TP#11 is adopted.

- If separate frequency configuration for SBFD is not provided for a PUCCH resource,

  - For P/SP PUCCH (e.g. SR PUCCH, P/SP CSI PUCCH), if a PUCCH in SBFD symbols overlap with RB outside UL usable PRBs, the PUCCH is dropped.

  - For PUCCH repetitions, if a PUCCH repetition in SBFD symbols overlaps with RB outside UL usable PRBs, a slot is determined as valid if the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Proposal 13: TP#12 is adopted.

Proposal 14: TP#13 is adopted.

- If SPS HARQ-ACK deferring is enabled,

- SPS HARQ-ACK is deterred if the determined SPS HARQ-ACK PUCCH resource overlaps with non-SBFD DL symbol or SSB symbol, or it is across both SBFD symbols and non-SBFD symbols, or if it overlaps with RB outside UL usable PRBs in SBFD symbols.

- The target slot for deferring is the first slot in which the determined PUCCH resource is not overlapping with non-SBFD DL symbol or SSB symbol, and not overlapping with SBFD symbols and non-SBFD symbols, and not overlapping with RB outside UL usable PRBs in SBFD symbols.

References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- TR 38.858, Study on Evolution of NR Duplex Operation

- R1-2401117, Discussion on SBFD random access operation, February 2024, NTT DOCOMO

- R1- 2401118, Discussion on CLI handling for sub-band full duplex (SBFD), February 2024, NTT DOCOMO

- 3GPP, RAN1#116 meeting, Chairman’s notes

- 3GPP, RAN1#117 meeting, Chairman’s notes

- 3GPP, RAN1#118 meeting, Chairman’s notes

- 3GPP, RAN1#118bis meeting, Chairman’s notes

- 3GPP, RAN1#119 meeting, Chairman’s notes

- 3GPP, RAN1#120 meeting, Chairman’s notes

- 3GPP, RAN1#120bis meeting, Chairman’s notes

- 3GPP, RAN1#121 meeting, Chairman’s notes

- TS 38.213, Physical layer procedures for control (Release 19), v19.0.0

- TS 38.214, Physical layer procedures for data (Release 19), v19.0.0