3GPP TSG RAN WG1 Meeting #123	    	R1-2508447

Dallas, USA, Nov 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on evaluation results

Agenda item:	10.3.1

Document for:	Discussion & Decision

- Background

During RAN#108 meeting, a new study item on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices was approved for Rel-20 [1], which shall study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications.

According to one of the objectives in the study item, it will focus on necessary evaluation assumptions of deployment scenarios for coverage and coexistence evaluations, and defining link budget calculation for coverage.

A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

The evaluation methodology, link budget template and parameter assumptions of link-level simulations for Ambient IoT in NR outdoor deployment scenarios have been achieved. This contribution provides the observations based on our evaluation results.

- Link performance (LLS)

  - Link level assumptions

For both R2D and D2R link, Budget-Alt2 is used for device 2b/C, the required SNRs/SINRs for receiver sensitivity calculation are acquired by LLS results. For coverage evaluation purpose, the following parameters listed in table 2.1-1 are assumed in the link-level simulation to obtain the required SNR/SINR for coverage evaluation.

Table 2.1-1: Coverage evaluation assumptions for link-level simulation for Rel-20 study

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | 900MHz |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 with update reported by companies |
| [0d] | Channel model |  | TDL-C |
| [0e] | Delay spread |  | an RMS delay spread of 300 ns |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 |
| [0h2] |  | Number of TXRUs | 2 |
| [0m] | Reference data rate |  | 1 kbps (M) 0.1 kbps for message size of 20 bits or 96 bits (O) Exact data rate for R2D: 3.6kbps, 1.79kbps Exact data rate for D2R: 0.72kbps, 0.104kbps, 0.077kbps |
| [0n] | Message size |  | 20 bits |
| [0p] | BLER target |  | 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm SFO after clock sync/calibration: For Device 2b: 10^3 (M) ppm, 10^2 (O) ppm For Device C: 10 ppm Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. Initial CFO: For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm CFO after clock sync/calibration: For Device 2b: 100 (M) ppm, 200 (O) ppm, 50 (O) ppm For Device C: 10 ppm Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| [0r] | Device 2b/C |  | ZIF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180kHz |
| [1b] | ED bandwidth |  | 180kHz |
| [1c] | BB LPF |  | [5]-order Butterworth/RC filter with cutoff frequency at half of ED bandwidth. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 M=1 |
| [1f] | Line code |  | Manchester, or no line code |
| [1g] | FEC |  | 1/3 CC |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Sampling average |
| [1k] | Repetition |  | Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition |
| D2R specific parameters |  |  |  |
| [2a1] | Transmission bandwidth |  | 15kHz, 1SB |
| [2a2] | Chip rate |  | 7kbps |
| [2a3] | Receiver bandwidth |  | 15kHz |
| [2d] | Modulation |  | BPSK |
| [2e] | Line code |  | No line coding |
| [2g] | FEC |  | 1/3 CC |
| [2h] | ADC bit width |  | Companies to report, e.g., 11-bit |
| [2j] | D2R receiver |  | coherent receiver |
| [2k] | Repetition |  | 1, 10, 16 |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

  - LLS results

The link-level simulation results for R2D and D2R of different cases and detailed simulation assumptions are summarized in the section 2.2.1 and section 2.2.2 separately.

    - R2D

For R2D link, the preamble preceding each PRDCH to indicate the start and chip duration of R2D is not needed for Rel-20 Device 2b/C, the time synchronization can be maintained by device 2b/C acquiring periodic synchronization signal. Therefore, 20bit PRDCH payload with 6bit CRC attachment is considered in the link-level simulation. And we initially evaluate the cases that assuming OOK-4 modulation with M=1, 1/3 CC and no repetition, both Manchester coding and no line code are considered in the R2D Link-level simulation are considered. The required SNRs corresponding to 10% BLER target and some detailed assumptions of each case are listed in the Table 2.2.1-1.

Table 2.2.1-1. R2D LLS results

| LLS case | Message size (bit) | Data rate (kbps) | Line code | M value | Repetition number | BLER | Required SNR |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R2D-Case1 | 20 | 3.6 | No | 1 | 1 | 10% | 4.95 |
| R2D-Case2 | 20 | 1.79 | Manchester | 1 | 1 | 10% | 2.36 |

Figure 2.2.1-1: R2D BLER-SNR for 20bit payload size

Observation 1: For R2D, assuming 20bits message size with 6bit CRC attachment, OOK-4 modulation, M=1, 1/3 CC and no repetition,

- When considering Manchester coding, 10% BLER is achieved at 2.36dB SNR.

- When line code is not considered, 10% BLER is achieved at 4.95dB SNR.

    - D2R

For D2R link, the block structures of {Preamble = 64/512 chips, payload=20bit, midamble of 4 chips are inserted every 6 data chips, CRC=6bit} are considered in the link-level simulation. And based on the common parameters assumed in Table 2.1-1 with BPSK modulation, no line code, 1/3 CC, the required SNRs of different simulation cases when BLER target is set to 10% are listed in the Table 2.2.2-1, where different repetition numbers are considered.

For device 2b and device C, it is recommended to consider similar device architectures that generate D2R transmission internally, from our perspective, identical local oscillator for generating carrier frequency with limited accuracy can be used for device 2b and device C to satisfy low power consumption. And similar low sampling frequency and CFO after calibration are assumed in our link-level simulation.

Table 2.2.2-1. D2R LLS results

| LLS case | Block structure | Chip rate (kcps) | Data rate (kbps) | Repetition number | BLER | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| D2R-Case1 | {Preamble = 64 chips, payload=20bit, midamble of 4 chips are inserted every 6 data chips, CRC=6bit} | 7 | 0.72 | 1 | 10% | -4.7 |
| D2R-Case2 | {Preamble = 512 chips, payload=20bit, midamble of 4 chips are inserted every 6 data chips, CRC=6bit} | 7 | 0.104 | 10 | 10% | -14.39 |
| D2R-Case3 | {Preamble = 512 chips, payload=20bit, midamble of 4 chips are inserted every 6 data chips, CRC=6bit} | 7 | 0.077 | 16 | 10% | -16.1 |

Observation 2: For D2R, assuming 20bits message size with 6bit CRC attachment, BPSK, no line code, 1/3 CC, the link performance can be improved by increasing repetition number,

- 10% BLER is achieved at -4.7dB SNR for preamble=64chips with no repetition.

- 10% BLER is achieved at -14.39dB SNR for preamble=512chips with 10 repetitions, and -16.1dB for preamble=512chips with 16 repetitions.

- Link budget evaluation

  - Assumptions for coverage evaluation

The deployment scenarios for coverage and coexistence evaluation have been identified in the objectives of the new SID, it will focus on the Deployment scenario 4 with topology 1 (D4T1) with no external carrier wave. And it is assumed that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. The spectrum deployment for R2D and D2R, FR1 licensed spectrum in FDD in-band to NR and in standalone bands are considered, with R2D in DL spectrum and D2R in UL spectrum.

To evaluate the coverage performance of Ambient IoT in outdoor scenarios for active devices, the link budget calculation is performed. The complete link budget template and assumptions for each item, calculation formula are reached during previous meetings. Based on the agreed link budget template table, our detailed assumptions for link budget calculation are listed in the following Table 3.1-1.

Table 3.1-1 Link budget template

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz | 900MHz |
| [0D] | Pathloss model | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | Refer to LLS table [0h1] and [0h2] | 1 |
| [1E] | Total Tx Power (dBm) | For standalone: 43 dBm For in-band: 38 dBm over R2D transmission BW | Device 2b: -10 dBm Device C: {0, 5} dBm |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | 180kHz | 15kHz |
| [1G] | Tx antenna gain (dBi) | 17dBi | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | 3dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | 180kHz | 15kHz |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | Device 2b: 15 dB Device C: 13 dB | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2E1] | Receiver interference density (dBm/Hz) | -169.3 dBm/Hz | -165.7 dBm/Hz |
| [2F] | Effective Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For UMa NLOS: 6dB For RMa NLOS: 8dB | For UMa NLOS: 6dB For RMa NLOS: 8dB |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [3E] | Penetration margin (dB) | {0, 20} dB | {0, 20} dB |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

  - Evaluation results of UMa scenario

Based on the detailed assumptions listed in the Table 3.1-1 and link-level simulation results illustrated in Section 2.2, the link budget calculation is performed by considering UMa NLOS pathloss model in this section. The MPL and distance of R2D and D2R link are summarized respectively in the following tables. Both the cases assuming 0dB penetration loss and 20dB penetration loss are considered.

    - R2D maximum distance

For standalone scenarios

Table 3.2.1-1. UMa-20bit-10%BLER-BS Tx power 43dBm-0dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 144.1 | 149.1 | 3108 |
|  | R2D-Case2 | 1.79 | 146.7 | 151.7 | 3620 |
| Device C | R2D-Case1 | 3.6 | 145.9 | 150.9 | 3453 |
|  | R2D-Case2 | 1.79 | 148.5 | 153.5 | 4022 |

Table 3.2.1-2. UMa-20bit-10%BLER-BS Tx power 43dBm-20dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 144.1 | 129.1 | 956 |
|  | R2D-Case2 | 1.79 | 146.7 | 131.7 | 1114 |
| Device C | R2D-Case1 | 3.6 | 145.9 | 130.9 | 1063 |
|  | R2D-Case2 | 1.79 | 148.5 | 133.5 | 1238 |

For in-band scenarios

Table 3.2.1-3. UMa-20bit-10%BLER-BS Tx power 38dBm-0dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 139.1 | 144.1 | 2315 |
|  | R2D-Case2 | 1.79 | 141.7 | 146.7 | 2696 |
| Device C | R2D-Case1 | 3.6 | 140.9 | 145.9 | 2572 |
|  | R2D-Case2 | 1.79 | 143.5 | 148.5 | 2996 |

Table 3.2.1-4. UMa-20bit-10%BLER-BS Tx power 38dBm-20dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 139.1 | 124.1 | 712 |
|  | R2D-Case2 | 1.79 | 141.7 | 126.7 | 830 |
| Device C | R2D-Case1 | 3.6 | 140.9 | 125.9 | 792 |
|  | R2D-Case2 | 1.79 | 143.5 | 128.5 | 922 |

Observation 3: For device 2b in UMa NLOS scenarios and standalone band deployment (BS Tx power=43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 3108m without penetration loss, and is about 956m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 3620m without penetration loss, and is about 1114m with 20dB penetration loss.

Observation 4: For device 2b in UMa NLOS scenarios and in-band deployment (BS Tx power=38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 2315m without penetration loss, and is about 712m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 2696m without penetration loss, and is about 830m with 20dB penetration loss.

Observation 5: For device C in UMa NLOS scenarios and standalone band deployment (BS Tx power=43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 3453m without penetration loss, and is about 1063m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 4022m without penetration loss, and is about 1238m with 20dB penetration loss.

Observation 6: For device C in UMa NLOS scenarios and in-band deployment (BS Tx power=38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 2572m without penetration loss, and is about 792m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 2996m without penetration loss, and is about 922m with 20dB penetration loss.

    - D2R maximum distance

Table 3.2.2-1. UMa-20bit-10%BLER-0dB penetration loss

| Device type | Tx power (dBm) | LLS case | Data rate (kbps) | D2R MCL (dB) | D2R MPL (dB) | D2R distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Device 2b | -10 | D2R-Case1 | 0.72 | 117 | 122 | 628 |
|  |  | D2R-Case2 | 0.104 | 126.7 | 131.7 | 1112 |
|  |  | D2R-Case3 | 0.077 | 128.4 | 133.4 | 1230 |
| Device C | 0 | D2R-Case1 | 0.72 | 127 | 132 | 1132 |
|  |  | D2R-Case2 | 0.104 | 136.7 | 141.7 | 2004 |
|  |  | D2R-Case3 | 0.077 | 138.4 | 143.4 | 2216 |
|  | 5 | D2R-Case1 | 0.72 | 132 | 137 | 1520 |
|  |  | D2R-Case2 | 0.104 | 141.7 | 146.7 | 2690 |
|  |  | D2R-Case3 | 0.077 | 143.4 | 148.4 | 2976 |

Table 3.2.2-2. UMa-20bit-10%BLER-20dB penetration loss

| Device type | Tx power (dBm) | LLS case | Data rate (kbps) | D2R MCL (dB) | D2R MPL (dB) | D2R distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Device 2b | -10 | D2R-Case1 | 0.72 | 117 | 102 | 193 |
|  |  | D2R-Case2 | 0.104 | 126.7 | 111.7 | 342 |
|  |  | D2R-Case3 | 0.077 | 128.4 | 113.4 | 378 |
| Device C | 0 | D2R-Case1 | 0.72 | 127 | 112 | 348 |
|  |  | D2R-Case2 | 0.104 | 136.7 | 121.7 | 617 |
|  |  | D2R-Case3 | 0.077 | 138.4 | 123.4 | 682 |
|  | 5 | D2R-Case1 | 0.72 | 132 | 117 | 468 |
|  |  | D2R-Case2 | 0.104 | 141.7 | 126.7 | 828 |
|  |  | D2R-Case3 | 0.077 | 143.4 | 128.4 | 916 |

Observation 7: It is observed that the D2R maximum distance can be bottleneck between R2D and D2R link in some cases, especially for device 2b due to the limited device transmit power.

Observation 8: For device 2b with -10dBm Tx power in UMa NLOS scenarios, when the data rate is low, the coverage distance is about 628~1230m without penetration loss, and is about 193~378m with 20dB penetration loss.

Observation 9: For device C in UMa NLOS scenarios, when the data rate is approximately 1kbps or 0.1kbps,

- When Tx power of device C is assumed as 0dBm, the coverage distance is about 1132~2216m without penetration loss, and is about 348~682m with 20dB penetration loss.

- When Tx power of device C is assumed as 5dBm, the coverage distance is about 1520~2976m without penetration loss, and is about 468~916m with 20dB penetration loss.

Observation 10: D2R coverage distance can be increased with larger repetition numbers.

  - Evaluation results of RMa scenario

In this section, the link budget calculation results are summarized by considering RMa NLOS pathloss model.

    - R2D maximum distance

For standalone

Table 3.3.1-1. RMa-20bit-10%BLER-BS Tx power 43dBm-0dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 144.1 | 147.1 | 5464 |
|  | R2D-Case2 | 1.79 | 146.7 | 149.7 | 6376 |
| Device C | R2D-Case1 | 3.6 | 145.9 | 148.9 | 6078 |
|  | R2D-Case2 | 1.79 | 148.5 | 151.5 | 7093 |

Table 3.3.1-2. RMa-20bit-10%BLER-BS Tx power 43dBm-20dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 144.1 | 127.1 | 1659 |
|  | R2D-Case2 | 1.79 | 146.7 | 129.7 | 1936 |
| Device C | R2D-Case1 | 3.6 | 145.9 | 128.9 | 1845 |
|  | R2D-Case2 | 1.79 | 148.5 | 131.5 | 2153 |

For in-band

Table 3.3.1-3. RMa-20bit-10%BLER-BS Tx power 38dBm-0dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 139.1 | 142.1 | 4056 |
|  | R2D-Case2 | 1.79 | 141.7 | 144.7 | 4733 |
| Device C | R2D-Case1 | 3.6 | 140.9 | 143.9 | 4512 |
|  | R2D-Case2 | 1.79 | 143.5 | 146.5 | 5265 |

Table 3.3.1-4. RMa-20bit-10%BLER-BS Tx power 38dBm-20dB penetration loss

| Device type | LLS case | Data rate (kbps) | R2D MCL (dB) | R2D MPL (dB) | R2D distance (m) |
| --- | --- | --- | --- | --- | --- |
| Device 2b | R2D-Case1 | 3.6 | 139.1 | 122.1 | 1231 |
|  | R2D-Case2 | 1.79 | 141.7 | 124.7 | 1437 |
| Device C | R2D-Case1 | 3.6 | 140.9 | 123.9 | 1370 |
|  | R2D-Case2 | 1.79 | 143.5 | 126.5 | 1598 |

Observation 11: For device 2b in RMa NLOS scenarios and standalone band deployment (BS Tx power 43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 5464m without penetration loss, and is about 1659m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 6376m without penetration loss, and is about 1936m with 20dB penetration loss.

Observation 12: For device 2b in RMa NLOS scenarios and in-band deployment (BS Tx power 38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 4056m without penetration loss, and is about 1231m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 4733m without penetration loss, and is about 1437m with 20dB penetration loss.

Observation 13: For device C in RMa NLOS scenarios and standalone band deployment (BS Tx power 43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 6078m without penetration loss, and is about 1845m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 7093m without penetration loss, and is about 2153m with 20dB penetration loss.

Observation 14: For device C in RMa NLOS scenarios and in-band deployment (BS Tx power 38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 4512m without penetration loss, and is about 1370m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 5265m without penetration loss, and is about 1598m with 20dB penetration loss.

    - D2R maximum distance

Table 3.3.2-1. RMa-20bit-10%BLER-0dB penetration loss

| Device type | Tx power (dBm) | LLS case | Data rate (kbps) | D2R MCL (dB) | D2R MPL (dB) | D2R distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Device 2b | -10 | D2R-Case1 | 0.72 | 117 | 120 | 1084 |
|  |  | D2R-Case2 | 0.104 | 126.7 | 129.7 | 1931 |
|  |  | D2R-Case3 | 0.077 | 128.4 | 131.4 | 2139 |
| Device C | 0 | D2R-Case1 | 0.72 | 127 | 130 | 1967 |
|  |  | D2R-Case2 | 0.104 | 136.7 | 139.7 | 3505 |
|  |  | D2R-Case3 | 0.077 | 138.4 | 141.4 | 3881 |
|  | 5 | D2R-Case1 | 0.72 | 132 | 135 | 2650 |
|  |  | D2R-Case2 | 0.104 | 141.7 | 144.7 | 4722 |
|  |  | D2R-Case3 | 0.077 | 143.4 | 146.4 | 5229 |

Table 3.3.2-2. RMa-20bit-10%BLER-20dB penetration loss

| Device type | Tx power (dBm) | LLS case | Data rate (kbps) | D2R MCL (dB) | D2R MPL (dB) | D2R distance (m) |
| --- | --- | --- | --- | --- | --- | --- |
| Device 2b | -10 | D2R-Case1 | 0.72 | 117 | 100 | 329 |
|  |  | D2R-Case2 | 0.104 | 126.7 | 109.7 | 586 |
|  |  | D2R-Case3 | 0.077 | 128.4 | 111.4 | 649 |
| Device C | 0 | D2R-Case1 | 0.72 | 127 | 110 | 597 |
|  |  | D2R-Case2 | 0.104 | 136.7 | 119.7 | 1064 |
|  |  | D2R-Case3 | 0.077 | 138.4 | 121.4 | 1178 |
|  | 5 | D2R-Case1 | 0.72 | 132 | 115 | 805 |
|  |  | D2R-Case2 | 0.104 | 141.7 | 124.7 | 1434 |
|  |  | D2R-Case3 | 0.077 | 143.4 | 126.4 | 1587 |

Observation 15: For device 2b with -10dBm Tx power in RMa NLOS scenarios, when the data rate is approximately 1kbps or 0.1kbps, the coverage distance is about 1084~2139m without penetration loss, and is about 329~649m with 20dB penetration loss.

Observation 16: For device C in RMa NLOS scenarios, when the data rate is approximately 1kbps and 0.1kbps,

- When Tx power is assumes as 0dBm, the coverage distance is about 1967~3881m without penetration loss, and is about 597~1178m with 20dB penetration loss.

- When Tx power is assumes as 5dBm, the coverage distance is about 2650~5229m without penetration loss, and is about 805~1587m with 20dB penetration loss.

  - Coverage target

In SID, the objective is that RAN1 design should support the following scenario, it is clearly mentioned that the A-IoT BS reader are deployed on the same site as existing NR macro BS.

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2.

For evaluation purpose, the following layout assumption was agreed. With 3 sectors per site placed on a hexagonal grid, the cell range can be calculated as 2*(ISD/3). Considering ISD=750m for UMa scenarios and ISD=1732 for RMa scenarios, the cell range is about 2*750/3=500m for UMa scenarios and 2*71732/3=1155m for RMa scenarios.

Agreement For Rel-20 study, the following layout assumption is used for evaluation purpose: - Hexagonal grid, 19 macro sites, 3 sectors per site - Note: the same layout assumption is used for UMi

Proposal 1: The maximum achievable coverage should be at least 500m for UMa and at least 1155m for RMa.

Based on our observations in Section 3.2 and Section 3.3, the following cases can achieve the coverage of 500m for UMa scenarios and 1155m for RMa scenarios.

- For UMa scenario with 0dB penetration loss and 20bits payload,

  - For R2D, more than 500m coverage can be satisfied by the cases of BS Tx power=38dBm/43dBm for device 2b/C

  - For D2R, more than 500m coverage can be satisfied by device 2b (-10dBm Tx power) and device C (0/5dBm Tx power)

- For UMa scenario with 20dB penetration loss and 20bits payload,

  - For R2D, more than 500m coverage can be satisfied by the cases of BS Tx power=38dBm/43dBm for device 2b/C

  - For D2R, more than 500m coverage can be satisfied by device C (0dBm Tx power) with 10 or 16 repetitions, and device C (5dBm Tx power)

- For RMa scenario with 0dB penetration loss and 20bits payload,

  - For R2D, more than 1155m coverage can be satisfied by the cases of BS Tx power=38dBm/43dBm for device 2b/C

  - For D2R, more than 1155m coverage can be satisfied by device 2b (-10dBm Tx power) with 10 or 16 repetitions and device C (0/5dBm Tx power)

- For RMa scenario with 20dB penetration loss and 20bits payload,

  - For R2D, more than 1155m coverage can be satisfied by the cases of BS Tx power=38dBm/43dBm for device 2b/C

  - For D2R, more than 1155m coverage can be satisfied by device C (0dBm Tx power) with 16 repetitions, and device C (5dBm Tx power) with 10 or 16 repetitions

Observation 17: For R2D in UMa scenarios with 0dB or 20dB penetration loss, assuming BS Tx power=38dBm/43dBm, more than 500m coverage distance can be satisfied for device 2b/C.

Observation 18: For D2R in UMa scenarios,

- When assuming 0dB penetration loss, both device 2b and device C can achieve more than 500m coverage

- When assuming 20dB penetration loss, device C (5dBm Tx power) can achieve more than 500m coverage, and device C (0dBm Tx power) with repetitions can achieve more than 500m coverage

Observation 19: For R2D in RMa scenarios with 0dB or 20dB penetration loss, more than 1155m coverage distance of device 2b/C can be achieved in the cases assuming BS Tx power=38dBm/43dBm.

Observation 20: For D2R in RMa scenarios,

- When assuming 0dB penetration loss, device 2b with repetitions and device C (0/5dBm Tx power) can achieve more than 1155m coverage

- When assuming 20dB penetration loss, device C with repetitions can achieve more than 1155m coverage

Besides, as mentioned in the objectives of the SID, it is assumed that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. According to the study on 5G NR coverage target in TR 38.913, the MCL is the limit value of the coupling loss at which the service can be delivered, and therefore defines the coverage of the service. For a basic MBB service, the target on MCL is about 140dB~143dB [5]. Also according to the coverage evaluation in TR38.830 [6], the target defined by MCL=139.2dB is applicable to Rural scenarios, and used for drawing observation bottleneck identification.

Therefore, we think that under co-site deployment, the coverage of A-IoT outdoor scenarios needs to achieve approximately 140dB MCL.

Proposal 2: For the study of outdoor scenarios of Ambient IoT, the coverage target about 140dB MCL needs to be considered at least for co-site deployment.

- Energy harvesting

Considering the energy harvesting issue, regarding that device 2b with hundreds µW peak power consumption and device C with 1 mW ~10 mW peak power consumption, it would be challenging to be driven by RF energy harvesting. RF energy harvesting for hundreds µW peak power consumption or even several mW peak power consumption takes a considerable amount of time to store the energy and it would be challenge for a capacitor. Second, with higher activation threshold, the coverage distance for device 2b and device C would be very limited. From our understanding, other than the RF energy harvesting, energy can be provided through the harvesting of light, motion, heat, or any other power source that could be seen suitable.

Agreement For Rel-20 study, RAN1 assumes companies to report energy harvesting/storage assumptions to obtain the reported data rates for the following aspects: - Energy source - Energy storage capacity

Proposal 3: For energy harvesting of device 2b and device C, various power sources provided by external environment other than RF energy harvesting can be considered.

- Energy source including solar, light, motion, heat, etc.

- Conclusions

In this contribution, we share our observations on the coverage evaluation results of Ambient IoT. The observations and proposals are summarized as follows.

Observation 1: For R2D, assuming 20bits message size with 6bit CRC attachment, OOK-4 modulation, M=1, 1/3 CC and no repetition,

- When considering Manchester coding, 10% BLER is achieved at 2.36dB SNR.

- When line code is not considered, 10% BLER is achieved at 4.95dB SNR.

Observation 2: For D2R, assuming 20bits message size with 6bit CRC attachment, BPSK, no line code, 1/3 CC, the link performance can be improved by increasing repetition number,

- 10% BLER is achieved at -4.7dB SNR for preamble=64chips with no repetition.

- 10% BLER is achieved at -14.39dB SNR for preamble=512chips with 10 repetitions, and -16.1dB for preamble=512chips with 16 repetitions.

Observation 3: For device 2b in UMa NLOS scenarios and standalone band deployment (BS Tx power=43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 3108m without penetration loss, and is about 956m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 3620m without penetration loss, and is about 1114m with 20dB penetration loss.

Observation 4: For device 2b in UMa NLOS scenarios and in-band deployment (BS Tx power=38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 2315m without penetration loss, and is about 712m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 2696m without penetration loss, and is about 830m with 20dB penetration loss.

Observation 5: For device C in UMa NLOS scenarios and standalone band deployment (BS Tx power=43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 3453m without penetration loss, and is about 1063m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 4022m without penetration loss, and is about 1238m with 20dB penetration loss.

Observation 6: For device C in UMa NLOS scenarios and in-band deployment (BS Tx power=38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 2572m without penetration loss, and is about 792m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 2996m without penetration loss, and is about 922m with 20dB penetration loss.

Observation 7: It is observed that the D2R maximum distance can be bottleneck between R2D and D2R link in some cases, especially for device 2b due to the limited device transmit power.

Observation 8: For device 2b with -10dBm Tx power in UMa NLOS scenarios, when the data rate is low, the coverage distance is about 628~1230m without penetration loss, and is about 193~378m with 20dB penetration loss.

Observation 9: For device C in UMa NLOS scenarios, when the data rate is approximately 1kbps or 0.1kbps,

- When Tx power of device C is assumed as 0dBm, the coverage distance is about 1132~2216m without penetration loss, and is about 348~682m with 20dB penetration loss.

- When Tx power of device C is assumed as 5dBm, the coverage distance is about 1520~2976m without penetration loss, and is about 468~916m with 20dB penetration loss.

Observation 10: D2R coverage distance can be increased with larger repetition numbers.

Observation 11: For device 2b in RMa NLOS scenarios and standalone band deployment (BS Tx power 43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 5464m without penetration loss, and is about 1659m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 6376m without penetration loss, and is about 1936m with 20dB penetration loss.

Observation 12: For device 2b in RMa NLOS scenarios and in-band deployment (BS Tx power 38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 4056m without penetration loss, and is about 1231m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 4733m without penetration loss, and is about 1437m with 20dB penetration loss.

Observation 13: For device C in RMa NLOS scenarios and standalone band deployment (BS Tx power 43dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 6078m without penetration loss, and is about 1845m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 7093m without penetration loss, and is about 2153m with 20dB penetration loss.

Observation 14: For device C in RMa NLOS scenarios and in-band deployment (BS Tx power 38dBm),

- When data rate in LLS is assumed as 3.6kbps, the coverage is about 4512m without penetration loss, and is about 1370m with 20dB penetration loss

- When data rate in LLS is assumed as 1.79kbps, the coverage is about 5265m without penetration loss, and is about 1598m with 20dB penetration loss.

Observation 15: For device 2b with -10dBm Tx power in RMa NLOS scenarios, when the data rate is approximately 1kbps or 0.1kbps, the coverage distance is about 1084~2139m without penetration loss, and is about 329~649m with 20dB penetration loss.

Observation 16: For device C in RMa NLOS scenarios, when the data rate is approximately 1kbps and 0.1kbps,

- When Tx power is assumes as 0dBm, the coverage distance is about 1967~3881m without penetration loss, and is about 597~1178m with 20dB penetration loss.

- When Tx power is assumes as 5dBm, the coverage distance is about 2650~5229m without penetration loss, and is about 805~1587m with 20dB penetration loss.

Observation 17: For R2D in UMa scenarios with 0dB or 20dB penetration loss, assuming BS Tx power=38dBm/43dBm, more than 500m coverage distance can be satisfied for device 2b/C.

Observation 18: For D2R in UMa scenarios,

- When assuming 0dB penetration loss, both device 2b and device C can achieve more than 500m coverage

- When assuming 20dB penetration loss, device C (5dBm Tx power) can achieve more than 500m coverage, and device C (0dBm Tx power) with repetitions can achieve more than 500m coverage

Observation 19: For R2D in RMa scenarios with 0dB or 20dB penetration loss, more than 1155m coverage distance of device 2b/C can be achieved in the cases assuming BS Tx power=38dBm/43dBm.

Observation 20: For D2R in RMa scenarios,

- When assuming 0dB penetration loss, device 2b with repetitions and device C (0/5dBm Tx power) can achieve more than 1155m coverage

- When assuming 20dB penetration loss, device C with repetitions can achieve more than 1155m coverage

Proposal 1: The maximum achievable coverage should be at least 500m for UMa and at least 1155m for RMa.

Proposal 2: For the study of outdoor scenarios of Ambient IoT, the coverage target about 140dB MCL needs to be considered at least for co-site deployment.

Proposal 3: For energy harvesting of device 2b and device C, various power sources provided by external environment other than RF energy harvesting can be considered.

- Energy source including solar, light, motion, heat, etc.

- References

- RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, 3GPP TSG RAN Meeting #108.

- RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, 3GPP TSG RAN Meeting #109.

- Chairman's Notes RAN1#122, Aug 25th – 29th, 2025.

- Chairman's Notes RAN1#122bis, Oct 13th – 17th, 2025.

- 3GPP TR 38.913, Study on scenarios and requirements for next generation access technologies.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.