3GPP TSG RAN WG1 #123		              R1-2508510

DALLAS, USA, NOV 17th – 21th, 2025

Agenda Item:     10.2.2

Source:	QUECTEL

Title:	Discussion on enhancing DL CSI acquisition

Document for:	Discussion and decision

Introduction

At RAN #108, the Rel-20 WID on NR MIMO Phase 6 (RP‑251856) [1]targeted improvements in DL CSI acquisition. Building on agreements and open items discussed in RAN1 #122/#122‑bis, this contribution consolidates prior views and proposes a unified, state-agnostic framework for early SRS/CSI/CSI‑RS triggering that covers UE transitions (IDLE/INACTIVE→CONNECTED), SCell activation, and SCell dormancy exit. It further introduces a compact trigger‑state model, precise relative‑offset timelines, a dedicated SIB for configuration, and adaptive CSI‑RS density selection for large‑port deployments.

In RAN#108 meeting, new WID RP‑251856 on NR MIMO is shown the following objectives :

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

While Rel‑17 fast SCell activation has reduced the synchronization bottleneck, CSI availability after state transitions and SCell re-enablement remains inconsistent across procedures. Key issues include:

-Disjoint triggering paths across scenarios (MSG4 vs. MAC CE vs. DCI) lead to redundant specifications and corner cases.

-Ambiguity on the interpretation of MSG4 (MAC-level vs. RRC-level) impacts latency budgeting.

- Inefficient capability handling for INACTIVE UEs when valid context already exists.

- Fixed CSI‑RS density per port count lacks adaptability to mobility, CQI variance, and energy constraints.

.

Discussion

Early-CSI /SRS/CSI/CSI-RS Trigger Procedure

During RAN1 #122 meeting[2]  a general procedure of early SRS-AS/CSI-RS/CSI triggering for the case of transition from IDLE to CONNECTED mode was agreed.

Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3 - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately

In the latest RAN1 #122bis meeting [3], further clarification for Steps 1 and 2 supporting early aperiodic SRS has been reached as follows:

Agreement For early triggering of SRS-AS when UE transition from IDLE/INACTIVE to CONNECTED mode, support Option-2 as follows: - In Step-1, SIBx provides one or multiple SRS configurations based on one or multiple ‘xTyR’ UE capability assumptions. - At least one SRS configuration(s) resource sets for SRS-AS in an SRS configuration is provided for a same ‘xTyR’ - FFS: Information to be provided in SRS configuration(s) - FFS: Whether Configuration information could be SRS configuration or the CSI report configuration for PMI-free report. - FFS: Whether SRS configuration can be associated with CSI report configuration for PMI-free report. - In Step-2, UE reports through MSG3 which/whether the SRS configuration(s) provided in SIBx is/are supported. - FFS: How to report which/whether the SRS configuration(s) provided in SIBx is/are supported.

According to the discussions at RAN1 #122bis, the detailed method for configuring SRS-AS resources via SIBx and relating it to UE capability reporting is still for further study. In the legacy aperiodic SRS-AS triggering mechanism, the UE reports its xTyR capability corresponding to supported SRS resource set(s) for antenna switching, the network configures UE-specific SRS resource set(s) accordingly, and the transmission is triggered by the SRS request field in DCI associated with the configured aperiodic SRS resource.

For early triggering of SRS-AS, considering that UEs may support different SRS-AS capabilities for antenna switching, it is desirable that the network can associate multiple SRS resource configurations with one or more UE capability combinations. In line with the agreements that more than one SRS configuration for SRS-AS may be provided for the same xTyR, the UE may be configured, via an RRC parameter (e.g. an SRS-AS configuration list), with an index to one or more SRS configurations. Each SRS configuration in this list includes explicit indication of the antenna switching information (xTyR) and can be selected at runtime by the network (e.g. via an Early-CSI TriggerState or other trigger signaling) without requiring additional UE capability reporting.

Proposal 1: Allow configuration of an SRS-AS configuration list per UE, where each entry is compatible with one or more reported xTyR capabilities and is referenced by index in early-trigger signaling, enabling flexible early SRS-AS selection without new capability signaling.

Regarding periodic or semi-persistent SRS-AS/CSI-RS for early CSI transmission, based on the agreements from RAN1 #122/#122bis, at least aperiodic SRS-AS/CSI-RS/CSI triggered via MSG4 of 4-step RACH (for IDLE/INACTIVE) and via legacy SCell activation MAC CE / switching DCI (for SCell procedures) shall be supported for early-CSI. The use of periodic/semi-persistent SRS-AS/CSI-RS specifically for early-CSI purposes is left for further study. Since the same 4-step RACH procedure is used for UEs in IDLE and INACTIVE, and the same legacy signaling mechanisms are reused for SCell activation and dormancy exit, this contribution focuses on a unified four-phase aperiodic-based early-CSI procedure, which avoids duplicated definitions, reduces latency, and enables common parameterization across RRC states and SCell procedures.

PUSCH allocation for early triggered CSI report

In the RAN1 #122bis meeting [3], MSG4-triggered aperiodic CSI reporting has been reached as follows

Agreement: On PUSCH allocation for MSG4-triggered aperiodic CSI reporting associated with aperiodic CSI-RS for CSI, when UE transition from IDLE/INACTIVE to CONNECTED mode, down select one from the following alternatives to determine PUSCH for carrying the aperiodic CSI report by RAN1#123: - Alt-1: The PUSCH is scheduled by MAC CE in MSG4 along with the aperiodic CSI report triggering. - Alt-2: The PUSCH is scheduled by a legacy DCI after MSG4 - Alt-3: The PUSCH is configured by SIBx providing resource/reporting configuration for early triggering of aperiodic CSI reporting. Note: The timeline of PUSCH for aperiodic CSI reporting is separately discussed.

With Alt-1, the UL resources for MSG4-triggered aperiodic CSI reporting are scheduled in the same MSG4 that carries the trigger with lower latency and deterministic timing, so the UE can deterministically perform trigger PUSCH CSI report without waiting for an additional DCI. This minimizes time-to-CSI and is better aligned with the early-CSI objective than Alt-2, which inherently adds at least one extra scheduling step and delay and reduced signaling overhead and complexity. Bundling trigger and PUSCH allocation in MSG4 avoids an additional PDCCH transmission and related blind decoding for a separate legacy DCI. This simplifies UE behavior in the RACH / initial access phase and reduces control channel overhead compared to Alt-2.Unified and robust procedure for early CSI.
Alt-1 provides a single, self-contained control message for early-CSI, making the procedure less sensitive to race conditions (e.g., MSG4 vs. subsequent DCI arrival, CORESET changes, search space reconfiguration). The UE does not need to track extra candidate DCIs under partially configured search spaces, which improves robustness, especially at cell edge or under heavy load, and thus better alignment with MSG4 semantics at connection setup. MSG4 is already the first downlink transmission after MSG3 that finalizes access and can carry MAC CE; using it as the primary vehicle for both trigger and UL allocation follows existing design principles and avoids introducing a new special case behavior for an additional DCI (Alt-2). Alt-3 remains a safe optional fallback. Pre-configured PUSCH via SIBx (Alt-3) is suitable as an optional complement for implementations preferring semi-static resources, but lacks the per-UE, per-situation flexibility of Alt-1. Therefore, Alt-1 is preferred as the primary dynamic mechanism.

Proposal 2: For PUSCH allocation of MSG4-triggered aperiodic CSI reporting, support Alt-1 (PUSCH scheduled by MAC CE in MSG4 together with the trigger) as the primary solution

PDSCH allocation for early triggered CSI report

In the RAN1 #122bis meeting [3], using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI has been reached as follows

Agreement: For UE transition from IDLE/INACTIVE to CONNECTED mode, using MAC-CE in the PDSCH for MSG4 to trigger early aperiodic SRS-AS/CSI-RS/CSI - Note: How to determine the PUSCH for aperiodic CSI reporting is separately discussed.

For the unified four-phase early-CSI procedure (configuration → capability → trigger → execution), the role of the early triggered aperiodic CSI report is to enable more accurate and faster PDSCH scheduling for the initial and reconfigured downlink transmissions, without introducing new PDSCH formats or additional allocation mechanisms.

In the IDLE/INACTIVE 4-step RACH case, MSG4 may carry the Early-CSI-TriggerStateIndex, which selects the relevant CSI-ReportConfig and (if applicable) SRS-AS configuration and their timing offsets. The UE performs the configured TRS/CSI-RS measurements and transmits the corresponding aperiodic CSI report. The gNB then uses this early CSI to schedule subsequent PDSCH(s) for user data on the serving cell with improved beam / port / MCS selection. Similarly, for SCell activation and SCell dormancy exit, the enhanced SCell MAC CE or switching DCI can indicate an Early-CSI-TriggerStateIndex; once the associated early CSI report is obtained, the gNB uses it to schedule PDSCH on the (re-)activated SCell.

- PDSCH for user data after early-CSI follows the legacy DCI-based scheduling, reusing existing CORESET/search space and PDSCH allocation rules; and

- the only requirement is that, when the gNB intends to rely on the early CSI, the relevant PDSCH should be scheduled not earlier than the earliest CSI report completion time implied by the configured offsets; otherwise, the gNB may schedule PDSCH using baseline (non-early) assumptions.

Proposal 3: Use the legacy DCI-based PDSCH allocation and let early triggered aperiodic CSI reports simply provide improved input for beam/port/MCS selection, without introducing new PDSCH allocation schemes for early-CSI-specific operation.

Relative-Offset Timing Profile

Use a single relative‑offset profile across scenarios to guarantee tracking before CSI measurement and to allow reciprocity‑aided estimation.

Constraints: kCSI > kCSI‑RS > kTRS and kSRS ∈ [kTRS − Δ1, kCSI + Δ2].

Recommended nominal offsets (illustrative): kTRS = +2, kCSI‑RS = +4, kCSI = +6 slots; kSRS configurable within [−2, +8].

Proposal 4: Adopt relative‑offset timing profile across all early‑CSI triggers.

SIB‑CSI Config (New SIBx) for Early-CSI Resources

Define a new SIB carrying: (i) CSI‑RS/CSI‑IM resource sets, (ii) SRS resource sets with AS capability tags (xTyR), (iii) CSI report configurations, and (iv) Early‑CSI‑TriggerState list and associations. SIB1 announces schedule and change version.

Proposal 5: Introduce SIB‑CSIConfig to avoid SIB1 size pressure and to keep early‑CSI configuration modular.

Reporting Quantities and Modes

Supported report quantities for early CSI are limited to:

PMI‑based: cri‑RI‑PMI‑CQI (wideband Type‑I SP codebook)

PMI‑free:  cri‑RI‑CQI (wideband)

Proposal 6: Constrain to the above two report quantities for simplicity and fast ramp‑up.

Procedures by Scenario

The further considerations on early SRS/CSI-RS/CSI triggering under the following three scenarios based on the agreements reached in RAN1#122 meeting.

- Scenario-1: UE transitioning from IDLE/INACTIVE to CONNECTED mode.

- Scenario-2: UE transitioning from SCell deactivation to activation.

- Scenario-3: UE transitioning from SCell dormancy state to non-dormancy state.

Scenario-1: UE transitioning from IDLE/INACTIVE to CONNECTED mode

Applies to FR1 UEs using aperiodic early-CSI during RRC connection via 4-step RACH. MSG4 is treated as the first DL transmission after MSG3 capable of triggering Early-CSI. UE sends RRC ResumeRequest, gNB retrieves context including early-CSI capability; gNB may trigger Early-CSI in first DL after MSG3 without a new capability report; UE reuses stored parameters unless SIB-CSIConfig version changed (then switch at boundary). UE could fallback to Setup: If resume fails, fall back to 4 including capability in MSG3, treat earliest DL with MAC CE/DCI as Early-CSI trigger.

When serving gNB changed since last CONNECTED, UE prefers SI-based configuration until RRC-dedicated overrides. If TRS cannot be confirmed before kCSI-RS, defer CSI within T_ECSI; otherwise release trigger. Apply configured SRS power and common TA for cross-slot SRS.

UE has read SIB1 and SIB-CSIConfig; Detailed timeline:

(1) Read SIB-CSIConfig;

(2) MSG3 capability (unless skipped);

(3) MSG4 with state i and offsets;

(4) Execute TRS→CSI-RS→CSI and SRS;

(5) Switch to UE-dedicated overrides when available with safe boundary.

According to the timeline, UE may report capability via MSG3,gNB selects Early-CSI-TriggerStateIndex i bound to {CSI report config, SRS-AS set} and optional offsets {kTRS,kCSI-RS,kCSI,kSRS}. gNB could schedule MSG4 so offsets satisfy kCSI > kCSI-RS > kTRS.that is include TriggerStateIndex in MAC CE on MSG4 PDSCH ,optionally mirror in DCI for robustness.

Therefore, UE selects a state compatible with UE capability, if mismatch, re-issue with compatible state or switch to PMI-free, ensure TRS precedes CSI-RS measurement when CSI-RS for tracking is used. UE would resolve state i via SIB-CSIConfig, apply TRS at n+kTRS, CSI-RS at n+kCSI-RS, CSI report at n+kCSI (PUSCH),and send SRS as per kSRS. If TRS quality is insufficient, defer CSI to next occasion within T_ECSI, upon expiry, drop trigger and optionally report cause.  Considering to collision handling priority: TRS > CSI-RS > SRS unless UL collision rules dictate otherwise.

Proposal 7： Use the first DL transmission after MSG3 (MSG4 for IDLE and the corresponding DL for INACTIVE resume) as the unified Early-CSI trigger for IDLE/INACTIVE→CONNECTED, carrying an Early-CSI-TriggerStateIndex (3 bits) and optional relative offsets {kTRS, kCSI-RS, kCSI, kSRS}.

- Proposal 8: If the triggered Early-CSI-TriggerState is not compatible with the UE’s reported capability or configured SIB-CSIConfig, the gNB re-issues a compatible Early-CSI-TriggerStateIndex or falls back to PMI-free (cri-RI-CQI) reporting, and the UE ignores unsupported triggers.

Scenario-2: UE transitioning from SCell deactivation to activation

In Scenario-2,for early SRS/CSI/CSI-RS triggering during SCell activation, the following two agreements were achievedin RAN1 #122bits meeting as below.

Agreement: On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

For Early-CSI TriggerState. Alt-2 allows the SCell activation command (e.g. enhanced SCell MAC CE) to explicitly indicate which SRS resource set(s) and CSI-ReportConfig(s) are used (e.g. via an Early-CSI-TriggerStateIndex). This provides a direct and transparent mapping from activation → TriggerState → SRS-AS/CSI configuration, instead of relying on implicit interpretation of RRC configuration. It is better  for per-UE / per-SCell flexibility in system. With Alt-2, the gNB can select different early-CSI configurations per UE, per SCell, and per activation event without reconfiguring RRC each time. This is important when multiple SRS-AS patterns, CSI-ReportConfigs, and UE capability combinations (xTyR) must be supported concurrently.

Alt-1 requires rules to infer, from RRC configuration alone, which resources are implicitly triggered upon SCell activation. This becomes ambiguous when multiple configurations exist, SIB/RRC versions change, or capabilities differ. Alt-2 keeps the trigger decision in the activation command itself, which simplifies normative text and improves interoperability. It will be robust for early-CSI operation. Early aperiodic SRS-AS/CSI is timing-sensitive. An explicit indication in the activation command avoids relying on slow or implementation-specific mapping logic and ensures that the UE and gNB have a consistent view of which configuration is active at the moment of SCell (re-)activation.

Based on the above, we recommend Alt-2 (Explicit mechanism) as the primary solution.

Proposal 9： For early aperiodic SRS-AS transmission and early aperiodic CSI reporting on an activated SCell, Alt-2 (Explicit mechanism) is preferred..

Scenario-3: UE transitioning from SCell dormancy state to non-dormancy state.

When an SCell exits dormancy, the gNB may deliver the Early-CSI-TriggerStateIndex via legacy switching DCI (carrying the TriggerStateIndex or referencing a preconfigured entry) or, when co-scheduled, via the Enhanced SCell MAC CE; for wake-up latency minimization the gNB should use compact offsets (e.g., kTRS ≤ +2, kCSI-RS ≤ +4, kCSI ≤ +6). The UE shall treat the earliest valid trigger (last-wins if multiple arrive closely; sequence/timestamp FFS) as authoritative, resolve the TriggerState from SIB-CSIConfig, and execute TRS→CSI-RS→CSI with the configured offsets; SRS-AS handling follows Scenario-2. If TRS acquisition on the re-activated SCell fails within T_ECSI, the UE may keep that SCell dormant and, where configured, notify the gNB using a lightweight mechanism (FFS); the gNB may temporarily increase CSI-RS/TRS density via DDS or defer the SCell exit for that UE. For multi-SCell dormancy exit, the scheduler should stagger TriggerStates and kCSI across UEs/SCells to limit PUSCH bursts while preserving timely CSI availability; cross-carrier reporting is allowed as configured. If a configuration mismatch occurs (e.g., TriggerStateIndex not present in the current SIB-CSIConfig version), the UE shall ignore the trigger and revert to legacy dormancy-exit behavior until a consistent configuration is provided. Mode adaptation is gNB-controlled: for high mobility or uncertain tracking the gNB may first select PMI-free (cri-RI-CQI) and later upgrade to PMI-based (cri-RI-PMI-CQI) once tracking stabilizes (with hysteresis/hold-down timers to avoid flapping, FFS)._

Proposal 10: For multi-SCell exits, stagger TriggerStates and spread kCSI to cap concurrent PUSCH reports.

Proposal 11: On configuration mismatch (TriggerStateIndex vs current SIB-CSIConfig), ignore the trigger and revert to legacy exit until consistency is restored.

Exceptional Conditions and Recovery

gNB may batch Early-CSI triggers across UEs to align CSI-RS and reduce overhead; TriggerStateIndex enables per-UE granularity with shared RS. And for high-mobility or uncertain tracking, gNB may first use PMI-free (cri-RI-CQI) then upgrade to PMI-based after stabilization. There are some exceptional conditions, for example:

Missing RS and guard handling — If a mandatory RS required by the active Early-CSI TriggerState (e.g., TRS) is not detected with sufficient quality within the configured measurement window, the UE shall postpone any dependent operation (CSI-RS measurement and/or CSI reporting and/or SRS transmission when reciprocity is assumed) to the next valid occasion that still lies within T_ECSI. The UE shall maintain internal counters for consecutive RS-miss events and, upon T_ECSI expiry, shall drop the trigger instance, reset the counters for that instance, and optionally expose a cause code (FFS) for network diagnostics. For IDLE/INACTIVE→CONNECTED, the UE then proceeds with connection establishment; for SCell procedures, the UE preserves PCell behavior and does not block other SCells.

Configuration mismatch — If the UE cannot resolve the received early-CSI-TriggerStateIndex to a valid entry in SIB-CSIConfig (e.g., state out of range, version mismatch, or SIB change indicated), the UE shall ignore the trigger and revert to the legacy behavior for the ongoing procedure (e.g., periodic or non-early CSI, Rel-15/17 baseline). The gNB is expected to re-issue a supported state or re-schedule the trigger after broadcasting the updated SIB-CSIConfig version. Implementations may include an RRC-level event or MAC CE indication (FFS) to facilitate rapid recovery.

Message race and supersession — When multiple triggers pertaining to the same UE and cell are received closely in time (e.g., a DCI-based trigger followed by a MAC-CE trigger), the UE shall apply a last-wins policy using a monotonic sequence number or timestamp (FFS) bound to the trigger. Any previously scheduled but not-yet-executed measurement/report occasions for superseded triggers shall be canceled. The UE shall ensure atomic activation of the new state to avoid mixed RS/offset application.

TRS acquisition failure in SCell flows — If the UE fails TRS acquisition on a target SCell within T_ECSI, it shall keep that SCell in dormant/deactivated condition and, where configured, notify the gNB using a lightweight mechanism (e.g., cause bit in an existing MAC CE or a compact RRC event; FFS exact vehicle). Repeated failures may trigger a gNB policy to increase CSI-RS/TRS density temporarily or to defer SCell activation for the UE.

Collision policy — In the presence of time-domain overlaps, the UE shall apply the priority order TRS > CSI-RS > SRS unless UL collision rules/regulatory constraints impose otherwise. CSI reporting on PUSCH preempts best-effort UL data; the gNB may opportunistically borrow UL resources but should not jeopardize the earliest CSI availability required for closed-loop operation.

Security and integrity — The UE shall ignore Early-CSI triggers if cell integrity checks (e.g., PCI/frequency consistency with SIB-announced values, PLMN/TAC constraints) fail or if the trigger references resources not authorized by the current SIB-CSIConfig version. The UE may attempt reacquisition of SI and, upon success, accept subsequent triggers.

Multi-UE Scheduling and Prioritization

The gNB scheduler may batch Early-CSI triggers for multiple UEs so that their CSI-RS occasions align in time/frequency, thereby amortizing RS overhead and simplifying beam-sweeping. Per-UE differentiation is retained via distinct early-CSI-TriggerStateIndex values that bind each UE to its own CSI report configuration and SRS-AS resource set. When batching, the scheduler should consider UL bottlenecks for PUSCH-based reports and stagger kCSI offsets across UEs where needed.

Mobility aware report mode selection — For high-mobility UEs or when tracking uncertainty is high (e.g., large Doppler estimate, high CQI variance), the gNB should initially prefer PMI-free (cri-RI-CQI) reporting to accelerate link adaptation without waiting for stable precoder estimates. Once TRS/CSI-RS tracking stabilizes over a configurable window (e.g., N consecutive occasions with bounded frequency offset and CQI variance; thresholds), the gNB may upgrade to PMI-based (cri-RI-PMI-CQI). Hysteresis and hold-down timers should be used to avoid mode flapping. If conditions degrade, the scheduler may downgrade to PMI-free and optionally increase CSI-RS density via DDS until stability is restored.

Proposal 12: Support batching of Early-CSI triggers across UEs and mobility-aware PMI-free/PMI-based mode adaptation as informative guidance for scheduler design.

Conclusion

In this contribution, we provided our views on the details of the remaining issues on CSI enhancements. The following proposals are made:

Proposal 1: Allow configuration of an SRS-AS configuration list per UE, where each entry is compatible with one or more reported xTyR capabilities and is referenced by index in early-trigger signaling, enabling flexible early SRS-AS selection without new capability signaling.

Proposal 2: For PUSCH allocation of MSG4-triggered aperiodic CSI reporting, support Alt-1 (PUSCH scheduled by MAC CE in MSG4 together with the trigger) as the primary solution

Proposal 3: Use the legacy DCI-based PDSCH allocation and let early triggered aperiodic CSI reports simply provide improved input for beam/port/MCS selection, without introducing new PDSCH allocation schemes for early-CSI-specific operation.

Proposal 4: Adopt relative-offset timing profile across all early-CSI triggers.

Proposal 5: Introduce SIB-CSIConfig to avoid SIB1 size pressure and to keep early-CSI configuration modular.

Proposal 6: Constrain to the above two report quantities for simplicity and fast ramp-up.

Proposal 7： Use the first DL transmission after MSG3 (MSG4 for IDLE and the corresponding DL for INACTIVE resume) as the unified Early-CSI trigger for IDLE/INACTIVE→CONNECTED, carrying an Early-CSI-TriggerStateIndex (3 bits) and optional relative offsets {kTRS, kCSI-RS, kCSI, kSRS}.

Proposal 8: If the triggered Early-CSI-TriggerState is not compatible with the UE’s reported capability or configured SIB-CSIConfig, the gNB re-issues a compatible Early-CSI-TriggerStateIndex or falls back to PMI-free (cri-RI-CQI) reporting, and the UE ignores unsupported triggers.Scenario-2: SCell deactivation to activation

Proposal 9： For early aperiodic SRS-AS transmission and early aperiodic CSI reporting on an activated SCell, Alt-2 (Explicit mechanism) is preferred..

Proposal 10: For multi-SCell exits, stagger TriggerStates and spread kCSI to cap concurrent PUSCH reports.

Proposal 11: On configuration mismatch (TriggerStateIndex vs current SIB-CSIConfig), ignore the trigger and revert to legacy exit until consistency is restored.

Proposal 12: Support  batch Early-CSI triggers for multiple UEs so that their CSI-RS occasions align in time/frequency

References

[1] RP-251856, New WID: NR MIMO Phase 6

[2] Draft_Minutes_report_RAN1#122

[3] Draft_Minutes_report_RAN1#122bits