3GPP TSG-RAN WG1 Meeting #123	R1-2508511

Dallas, USA, November 17th – 21st 2025

Agenda Item:	10.3.1

Source:	Ericsson

Title:	Discussion on Rel-20 A-IoT evaluation assumptions and results

Document for:	Discussion, Decision

# 1	Introduction

Ambient IoT (A-IoT) is a new 3GPP IoT technology designed for deployment in 3GPP systems. It relies on ultra-low complexity devices with ultra-low power consumption, targeting very low-end IoT applications. In Release 19 (Rel-19), standardization work was completed for the simplest form of A-IoT, referred to as Device 1, focusing on indoor use cases and deployment scenarios. The specifications related to physical layer aspects are captured in ‎[1]. Definitions and decisions from preceding study items are documented in ‎[2] and ‎[3].

The Rel-20 study item description (SID) ‎[4] addresses the study of RAN aspects for A-IoT in outdoor use cases and so-called active A-IoT devices. In this contribution, we have presented our link level simulation and link budget evaluation results together with our observations.

The SID states the following regarding SI objective:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage

# 2	Link budget evaluations

## 2.1	General evaluation methodology

Rel-20 SI is focusing on active A-IoT devices, so called as device 2b and C, in outdoor scenarios. The descriptions of these devices are as follows.

• Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. • Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device.

The SID, in the text below, emphasizes that Rel-20 A-IoT technology should be clearly differentiated from existing 3GPP LPWA IoT technologies (e.g., NB-IoT, even with reduced peak transmit power) as well as from 6G LPWA. The coverage target for Rel-20 A-IoT outdoor devices should reflect this differentiation, considering the SID guidelines and the specified distance range of 50 to 500 meters. Any reconsiderations or enhancements beyond the values specified in the SID should be discussed at the RAN plenary level.

While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m

- For Rel-20 A-IoT outdoor devices, the coverage target should reflect the differentiation from 6G LPWA, consider the SID guidelines and the mentioned distance range of 50 to 500 m.

Any reconsiderations or enhancements beyond the values for maximum distance between device and reader specified in the SID should be discussed at the RAN plenary level.

Harvesting/storage assumptions

In RAN1 #122, it was agreed that companies should report their harvesting/storage assumptions to obtain the reported data rates, as reflected in the following agreement.

Agreement For Rel-20 study, RAN1 assumes companies to report energy harvesting/storage assumptions to obtain the reported data rates for the following aspects: - Energy source - Energy storage capacity

The transmission duration for a given data rate R (in kbps) and a payload size of K bits can be calculated as K/R ms. The corresponding values for the agreed data rates and payload size are summarized in Table 1.

Table 1: Transmission time for a given data rate and payload size in ms.

|  | 20 bits | 96 bits | 400 bits |
| --- | --- | --- | --- |
| 1 kbps | 20 ms | 96 ms | 400 ms |
| 7 kbps | 2.8 ms | 13.7 ms | 57.1 ms |
| 50 kbps | 0.4 ms | 1.96 ms | 8 ms |

Assuming capacitor value of C as energy storage, and considering the A-IoT device power consumption P (calculated based on the agreed P_tx+6 dB) and voltage V, the capacitor discharge time is calculated as:

T=CV22P

The discharge time is calculated for three capacitor values (16 μF, 24 μF, and 100 μF) using the agreed transmission power for Device 2b and Device C. The results are summarized in Table 2.

Table 2: Discharge time for different capacitor sizes

|  | Device 2b |  | Device C |  |  |
| --- | --- | --- | --- | --- | --- |
|  | -20 dBm+6 dB [40 µW] | -10 dBm+6 dB [400 µW] | -3 dBm+6 dB [2 mW] | 0 dBm+6 dB [4 mW] | 5dBm+6 dB [12.6 mW] |
| 16 μF | 450 ms | 45 ms | 9 ms | 4.5 ms | 1.4 ms |
| 24 μF | 675 ms | 67.5 ms | 13.5 ms | 6.7 ms | 2.1 ms |
| 100 μF | 2812.5 ms | 281.2 ms | 56.3 ms | 28.1 ms | 8.9 ms |

When transmitting at lower data rates or with larger payload sizes, the transmission time increases. To support this, especially for Device C, which has higher transmission power, larger capacitor sizes are required. For example, comparing the Table 1 and Table 2, for Device C, transmitting at lower data rates (1 and 7 kbps) with a payload size of 400 bits is not feasible even with a 100 µF capacitor.

- For Device C, even a capacitor size of 100 µF may not be sufficient for transmitting large payload sizes at lower data rates.

Note that the values in Table 1 only show the transmission duration for sending a single block. While from a coverage perspective it may be sufficient to consider transmission of just one block, in practice, selecting the capacitor or battery size requires accounting for the overall process completion time (Sustainable operation time), such as the inventory completion time.

# 3	Link-level simulations

R2D and D2R common parameters

Our assumptions for link-level simulation (LLS), common to both R2D and D2R links, are listed in Table 3.

Table 3: D2R and R2D common parameters

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 900 MHz, 15 KHz SCS |
| Number of Tx/Rx for BS | 2 |
| Number of Tx/Rx for Device: | 1 |
| SFO after calibration | Device 2b:1000 ppm Device C: 10 ppm |
| CFO after calibration | Device 2b:100 ppm Device C: 10 ppm |
| Data rate: | 7 kbps |
| Channel Model: | TDL-C, RMS delay spread of 300ns, device velocity 3 km/h |

## 3.1	R2D

The LLS assumptions configured for R2D are listed in the table below:

Table 4: R2D parameter setting

| Parameters | Assumptions |
| --- | --- |
| R2D bandwidth | 180 KHz |
| M | {1,2,6} |
| R2D Message size | {20,96,400} bits |
| Adjacent carrier interference | No |
| Architecture | ZIF |
| ED bandwidth | With considering CFO |
| Line coding | Manchester |
| ADC bits | 4 |
| Sampling rate | 1.92 Msps |
| FEC | {No, 1/3 TBCC} |

The general simulation results for devices 2b and C for different message lengths and M, conducted without FEC, the table shows the SNR required to achieve 10% and 1% BLER.

Table 5: R2D BLER performance for different devices, and M values (without FEC)

| Device Type | Msg size(bits) | M | BW (PRBs) | FEC | 10%BLER | 1%BLER |
| --- | --- | --- | --- | --- | --- | --- |
| 2b | 20 | 1 | 1 | No | 10.03 | 17.24 |
| 2b | 20 | 2 | 1 | No | 9.4 | 16.4 |
| 2b | 20 | 6 | 1 | No | 11.88 | 21.33 |
| 2b | 96 | 1 | 1 | No | 11.55 | 20.46 |
| 2b | 96 | 2 | 1 | No | 12.3 | 24 |
| 2b | 96 | 6 | 1 | No | 20.82 | NaN |
| 2b | 400 | 1 | 1 | No | NaN | NaN |
| 2b | 400 | 2 | 1 | No | NaN | NaN |
| 2b | 400 | 6 | 1 | No | NaN | NaN |
| C | 20 | 1 | 1 | No | 8.69 | 15.92 |
| C | 20 | 2 | 1 | No | 8.14 | 15.36 |
| C | 20 | 6 | 1 | No | 11.55 | 21.2 |
| C | 96 | 1 | 1 | No | 10.22 | 19.13 |
| C | 96 | 2 | 1 | No | 10.44 | 19.88 |
| C | 96 | 6 | 1 | No | 18.93 | NaN |
| C | 400 | 1 | 1 | No | 13.64 | NaN |
| C | 400 | 2 | 1 | No | 11.78 | NaN |
| C | 400 | 6 | 1 | No | 24.39 | NaN |

- For no-FEC case on R2D,

- Best performance is achieved at M=2 for device 2b and C, considering the trade-off between OOK symbol duration and overall transmission time.

- Device 2b cannot decode a 400-bit message correctly under the considered SFO value.

- For device 2b, M=6 requires approximately [1, 3.3] dB higher SNR at 10% BLER than M=2 for message bits of [20, 96], respectively.

- For device C, M=6 requires approximately [2, 3.5, 5] dB higher SNR at 10% BLER than M=2 for message bits of [20, 96, 400], respectively.

We have also considered R2D message is implemented with 1/3 TBCC encoding, where the generator is same with TS 36.212 5.3. The performance table summarizes the SNR required to achieve 10% and 1% BLER for device 2b and C.

Table 6 BLER performance for different devices (with FEC)

| Device Type | Msg size(bits) | M | BW (PRBs) | FEC | 10%BLER | 1%BLER |
| --- | --- | --- | --- | --- | --- | --- |
| 2b | 20 | 1 | 1 | TBCC (1/3) | 5.52 | 11.84 |
| 2b | 20 | 2 | 1 | TBCC (1/3) | 4.42 | 10.67 |
| 2b | 20 | 6 | 1 | TBCC (1/3) | 5.82 | 13.84 |
| 2b | 96 | 1 | 1 | TBCC (1/3) | NaN | NaN |
| 2b | 96 | 2 | 1 | TBCC (1/3) | NaN | NaN |
| 2b | 96 | 6 | 1 | TBCC (1/3) | NaN | NaN |
| 2b | 400 | 1 | 1 | TBCC (1/3) | NaN | NaN |
| 2b | 400 | 2 | 1 | TBCC (1/3) | NaN | NaN |
| 2b | 400 | 6 | 1 | TBCC (1/3) | NaN | NaN |
| C | 20 | 1 | 1 | TBCC (1/3) | 4.23 | 11.48 |
| C | 20 | 2 | 1 | TBCC (1/3) | 3.39 | 9.54 |
| C | 20 | 6 | 1 | TBCC (1/3) | 4.87 | 13.17 |
| C | 96 | 1 | 1 | TBCC (1/3) | 6.57 | 13.52 |
| C | 96 | 2 | 1 | TBCC (1/3) | 4.82 | 13.17 |
| C | 96 | 6 | 1 | TBCC (1/3) | 7.75 | 16.29 |
| C | 400 | 1 | 1 | TBCC (1/3) | NaN | NaN |
| C | 400 | 2 | 1 | TBCC (1/3) | 9.54 | NaN |
| C | 400 | 6 | 1 | TBCC (1/3) | 10.39 | 36.22 |

- With 1/3 TBCC FEC on R2D,

- For device 2b, block decoding is only possible when the message size is no greater than 20 bits and the performance improvement is around 4.5~6 dB @10% BLER.

- For device C, the performance improvement caused by FEC increases as M increases and the message size becomes larger, but fails to reach 10% BLER at M=1 for 400 message bits.

## 3.2	D2R

3.2.1     Parameter definition

Chip rate

Figure 1 Chip-interval illustration for a fixed information rate of 1 kbps (Fixed Fs )

In our simulation the sampling rate Fs=nfft×subcarreir_spacing is a fixed value and the information (uncoded) data rate is R. With Manchester line coding, each information bit maps to two chips; without Manchester, each bit maps to one chip.

If a channel code of rate RFEC is applied, the coded bit rate is

Rcoded=RRFEC

Hence the chip rate is

Rc=Nchipbit ×Rcoded

where Nchipbit=2 for Manchester and Nchipbit=1 without Manchester.

For RFEC=1 (no FEC),  Rc=NchipbitR

For RFEC=1/3 (FEC),  Rc=3NchipbitR

Thus, to maintain the same information rate, the FEC case requires three times the chip rate (and three times the noise bandwidth) of the no-FEC case when the line coding is the same. With fixed Fs, the oversampling factor is OS=FsRc.

At the receiver, the D2R receiver bandwidth is used to filter out the D2R signals for calculating noise and interference (if any) power. Assume a matched filter followed by down sampling is used to match the transmitter’s modulation. The receiver bandwidth should be 2Rc.

Receiver bandwidth

At the receiver we define the noise-equivalent bandwidth Brx​ used to compute the in-band noise/interference power. The receiver employs a filter matched to the transmit chip pulse, followed by down-sampling to the chip rate. For rectangular-pulse OOK/Manchester, the effective (one-sided baseband) bandwidth is approximately the chip rate:

Brx≈Rc

For two-sided passband, 
Brx≈2Rc

Eb/N0 vs SNR

We report results on both EbN0 (per information bit) and SNR (defined over each scheme’s receiver noise bandwidth Brx).

For Manchester OOK with coding rate Rc, the two metrics relate as

SNR=PN0Brx=RBrx∙EbN0

As a result, we have

SNRdB=EbN0SNR+10log10RRc=EbN0SNR+10log10RFECNchipbit

Therefore,

- Manchester + No FEC: SNRdB=EbN0SNR-3.01 dB

- Manchester + FEC: SNRdB=EbN0SNR-7.78 dB

- No Manchester + No FEC: SNRdB=EbN0SNR dB

- No Manchester + FEC: SNRdB=EbN0SNR-4.77 dB

Interpretation:

- EbN0 curves show pure coding gain (bandwidth-agnostic) of the coherent/non-coherent receiver

- SNR curves include the bandwidth cost of coding (e.g., FEC triples the bandwidth cost), answering whether coding still improves performance at a given front-end SNR.

The impact of CFO

The carrier frequency is 900 MHz, for device 2b the CFO after sync/calibration is 100 ppm. As a result, the CFO is around

CFO≈900×106×100×10-6=90kHz

For the modulated OOK signal with Manchester encoding at a data rate 1bps, the modulated signal occupies around 4kHz bandwidth. With the above CFO, the whole 4kHz band is now centered at ±90kHz (residual IF) instead of DC. We made a table for the two devices

Table 7 CFO and the corresponding signal BW for device 2b and device C

| Device type | Data rate | FEC [N0, 1/3] | Manchster encoding | CFO (ppm) | CFO (Hz) | Signal BW |
| --- | --- | --- | --- | --- | --- | --- |
| 2b | 1 Kbps | No | Yes | 100 | 90 kHz | 4 kHz |
| 2b | 1 Kbps | Yes | Yes | 100 | 90 kHz | 12 kHz |
| 2b | 7 Kbps | No | Yes | 100 | 90 kHz | 28 kHz |
| 2b | 7 Kbps | Yes | Yes | 100 | 90 kHz | 84 kHz |
| C | 1 Kbps | No | Yes | 10 | 9 kHz | 4 kHz |
| C | 1 Kbps | Yes | Yes | 10 | 9 kHz | 12 kHz |
| C | 7 Kbps | No | Yes | 10 | 9 kHz | 28 kHz |
| C | 7 Kbps | Yes | Yes | 10 | 9 kHz | 84 kHz |

- What happened if we do LPF then sample

  - Typical baseband LPF (cutting off frequency is about the same as the signal bandwidth): it rejects everything at ±90kHz. The desired signal will be heavily attenuated.

  - Wider LPF (cutoff frequency >90kHz): it passes the 90kHz band, but the sampled data still has 90kHz offset which will then be mixed out to do coherent/noncoherent detection.

- Practical fixes

  - Complex down-conversion (ZIF/low IF): mixed with e-j2πfCFOt, then apply LPF and sampling

Figure 2 complex down-conversion receiver

  - Envelope detection for OOK signals: rectify the signal first, then apply LPF/MF to a few kHz. CFO mostly vanishes in the envelope, so we could sample at low rate without caring about the 90kHz tone. However, the performance would be related to the sampling rate we used as higher sampling rates incur more noise after rectifying the signals.

Figure 3 Envelope detection receiver

From the table above, device C at higher data rates (e.g., 7 kbps) has a higher bandwidth compared with CFO. Baseband LPF would work in this case.  At lower data rates, however, the CFO is typically much larger than the signal bandwidth, so a baseband LPF approach can suppress or misplace the desired signal. For OOK, a practical rule of thumb is to use envelope (energy) detection, which is relatively insensitive to CFO. Accordingly, in the following performance evaluations for devices 2b and C, we adopt an envelope-detection receiver.

Noncoherent receiver vs. coherent receiver

- Noncoherent receiver uses envelope + integrate-and-dump, which is phase-free.

- Coherent receiver tracks carrier phase/frequency and timing. It uses pilot/preamble for channel estimation and then apply MF/ZF/MMSE equalization.

3.2.2     LLS results

In this section, we will present

- Plots

  - BLER vs. SNR

  - BLER vs. EbN0

- Tables (values read from the curves)

  - Required EbN0 at BLER target, plus coding gain in dB

  - Required SNR at the same BLER targets for both schemes, showing the net system-level gain/penalty once bandwidth is accounted for.

The LLS assumptions configured for D2R are as below:

Table 8 D2R parameter setting

| Parameters | Assumptions |
| --- | --- |
| D2R Message size | {20,96,400} bits |
| Receiver filter | Envelope LPF receiver |
| Line coding | Manchester |
| Sampling rate | 240 ksps |
| FEC | {No, 1/3 TBCC} |
| Transmission bandwidth | 15kHz |

The simulation results are generated with the noncoherent receiver which has a structure in Figure 3.

Message size: 20 bits

Figure 4 BLER plot for message size 20 bits in TDL_C channel (R=1kbps)

Table 9 BLER performance comparison for Device-2b and Device-C at 1 kbps data rate (msg=20bits)

| Device type | Data rate | Receiver BW | FEC | Rep. | BS Rx | Msg size | 10% BLER (SNR) | 1% BLER (SNR) | 10% BLER （EbN0） | 1% BLER (EbN0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2b | 1 Kbps | 4kHz | No | No | 2 | 20 | 23.8dB | 30dB | 26.8dB | 33.0dB |
| 2b | 1 Kbps | 12kHz | Yes | No | 2 | 20 | 13.8dB | 19.4dB | 21.5dB | 27.2dB |
| C | 1 Kbps | 4kHz | No | No | 2 | 20 | 16.2dB | 22.1dB | 19.2dB | 25.1dB |
| C | 1 Kbps | 12kHz | Yes | No | 2 | 20 | 9.2dB | 14.7dB | 17.0dB | 22.5dB |

Figure 5 BLER plot for message size 20 bits in TDL_C channel (R=7kbps)

Table 10 BLER performance comparison for Device 2b and Device C at 7 kbps data rate (msg=20bits)

| Device type | Data rate | Receiver BW | FEC | Rep. | BS Rx | Msg size | 10% BLER (SNR) | 1% BLER (SNR) | 10% BLER （EbN0） | 1% BLER (EbN0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2b | 7 Kbps | 28kHz | No | No | 2 | 20 | 18.0dB | 24.0dB | 21.0dB | 27.0dB |
| 2b | 7 Kbps | 84kHz | Yes | No | 2 | 20 | 10.7dB | 16.6dB | 18.5dB | 24.4dB |
| C | 7 Kbps | 28kHz | No | No | 2 | 20 | 13.1dB | 19.1dB | 16.1dB | 22.2dB |
| C | 7 Kbps | 84kHz | Yes | No | 2 | 20 | 6.3dB | 12.3dB | 14.1dB | 20.1dB |

Message size: 96 bits

Figure 6 BLER plot for message size 96 bits in TDL_C channel (R=1kbps)

Table 11 BLER performance comparison for Device 2b and Device C at 1 kbps data rate (msg=96bits)

| Device type | Data rate | Receiver BW | FEC | Rep. | BS Rx | Msg size | 10% BLER (SNR) | 1% BLER (SNR) | 10% BLER （EbN0） | 1% BLER (EbN0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2b | 1 Kbps | 4kHz | No | No | 2 | 96 | 28.7 | 35.1 | 31.7 | 38.1 |
| 2b | 1 Kbps | 12kHz | Yes | No | 2 | 96 | 28.1 | NAN | 35.8dB | NAN |
| C | 1 Kbps | 4kHz | No | No | 2 | 96 | 18.3dB | 24.6dB | 21.3dB | 27.6dB |
| C | 1 Kbps | 12kHz | Yes | No | 2 | 96 | 8.6dB | 12.6dB | 16.3dB | 20.4dB |

Figure 7 BLER plot for message size 96 bits in TDL_C channel (R=7kbps)

Table 12 BLER performance comparison for Device 2b and Device C at 7 kbps data rate (msg=96bits)

| Device type | Data rate | Receiver BW | FEC | Rep. | BS Rx | Msg size | 10% BLER (SNR) | 1% BLER (SNR) | 10% BLER （EbN0） | 1% BLER (EbN0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2b | 7 Kbps | 28kHz | No | No | 2 | 96 | 20.9dB | 26.7dB | 23.9dB | 29.7dB |
| 2b | 7 Kbps | 84kHz | Yes | No | 2 | 96 | 17.3dB | 24.1dB | 25.1dB | 31.9dB |
| C | 7 Kbps | 28kHz | No | No | 2 | 96 | 14.1dB | 19.9dB | 17.1dB | 22.9dB |
| C | 7 Kbps | 84kHz | Yes | No | 2 | 96 | 6.9dB | 12.6dB | 14.7dB | 20.4dB |

Message size: 400 bits

Figure 8 BLER plot for message size 400 bits in TDL_C channel (R=7kbps)

Table 13 BLER performance comparison for Device 2b and Device C at 7 kbps data rate (msg=400bits)

| Device type | Data rate | Receiver BW | FEC | Rep. | BS Rx | Msg size | 10% BLER (SNR) | 1% BLER (SNR) | 10% BLER （EbN0） | 1% BLER (EbN0) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2b | 7 Kbps | 28kHz | No | No | 2 | 400 | NaN | NaN | NaN | NaN |
| 2b | 7 Kbps | 84kHz | Yes | No | 2 | 400 | NaN | NaN | NaN | NaN |
| C | 7 Kbps | 28kHz | No | No | 2 | 400 | 15.7dB | 22.1dB | 18.7dB | 25.1dB |
| C | 7 Kbps | 84kHz | Yes | No | 2 | 400 | 7.0dB | 12.1dB | 14.8dB | 19.9dB |

The following observations have been made based on the D2R LLS results:

- CFO impact: At a 900 MHz carrier, CFOs of 10 ppm and 100 ppm correspond to roughly 9 kHz and 90 kHz offsets. For a 1 kbps link (very narrowband), these offsets far exceed the signal bandwidth, so a conventional baseband LPF would filter out the desired signal unless you first remove the offset (down-convert) or use envelope/energy detection.

- Higher data rate helps: raising the data rate from 1 to 7kbps, the required Eb/N0 is reduced by about 4-6 dB

- Device C >> Device 2b: Device C typically needs ~5-10dB less Eb/N0 compared with Device 2b. Device 2b could not hit the BLER target for 400 bit message size while Device C could.

- FEC impact: For Device C, FEC gives a steady ~2–3 dB gain and keeps performance nearly flat vs message length. For Device 2b, FEC helps strongly at 1 kbps, but at 7 kbps & 96 bits it shows no gain.

# 4	Link-budget evaluation results and observations

In this section, we have presented our link budget evaluation results considering the following assumptions:

Table 14: Link budget evaluation assumptions

| Path loss model | UMa/RMa |
| --- | --- |
| M value for R2D | 2 |
| Data rate | 7 kbps |
| Payload size | 20 bits |

For the R2D link, as in Rel-19, FEC was not considered, while for the D2R link, FEC was applied with a rate of R = 1/3. The required SNR for these assumptions is summarized in the following table.

Table 15: Required SNR [dB]

| Device | Link | SNR |
| --- | --- | --- |
| Device 2b | R2D | 10.03 dB |
|  | D2R | 10.7 dB |
| Device C | R2D | 8.69 dB |
|  | D2R | 6.3 dB |

4.1. Device 2b

Standalone

For Device 2b and standalone deployment scenario, we have considered Rel-19 assumption NO FEC for the R2D link and FEC in D2R, and obtained the 3D distance summarized in Table 4 and Table 5, for penetration rates of 0 and 20dB, respectively.

Table 16: Coverage distance (m), Device 2b, without penetration loss, OOK for D2R, standalone

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -20 | 2184.8 m | 85.8 m | 3826.7 m | 144.7 m |
| -10 | 2184.8 m | 154.7 m | 3826.7 m | 262.7 m |

Table 17: Coverage distance (m), Device 2b, with penetration loss, OOK for D2R, standalone

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -20 | 672.4 m | 26.4 m | 1161.1 m | 43.9 m |
| -10 | 672.4 m | 47.6 m | 1161.1 m | 79.7 m |

In band

We have also considered the in-band deployment scenario with BS Tx power of 38 dBm, our results are summarized in Table 7 and Table 8, respectively for penetration rates of 0 and 20dB.

Table 18: Coverage distance (m), Device 2b, without penetration loss, OOK for D2R, in band

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -20 | 1627.3 m | 85.8 m | 2840.5 m | 144.7 m |
| -10 | 1627.3 m | 154.7 m | 2840.5 m | 262.7 m |

Table 19: Coverage distance (m), Device 2b, with penetration loss, OOK for D2R, in band

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -20 | 500.8 m | 26.4 m | 862.3 m | 43.9 m |
| -10 | 500.8 m | 47.6 m | 862.3 m | 79.7 m |

4.2. Device C

For Device C, we have considered Rel-19 assumption NO FEC for the R2D link and FEC in D2R. Our results are summarized in Table 6 and Table 7, for penetration rates of 0 and 20dB, respectively.

Table 20: Coverage distance (m), Device C, without penetration loss, OOK for D2R, standalone

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -3 | 2627.1 m | 302.8 m | 4611.2 m | 518.3 m |
| 0 | 2627.1 m | 361.3 m | 4611.2 m | 619.8 m |
| 5 | 2627.1 m | 485.2 m | 4611.2 m | 835.0 m |

Table 21: Coverage distance (m), Device C, with penetration loss, OOK for D2R, standalone

|  | UMa |  | RMA |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -3 | 808.5 m | 93.2 m | 1399.8 m | 157.3 m |
| 0 | 808.5 m | 111.2 m | 1399.8 m | 188.1 m |
| 5 | 808.5 m | 149.3 m | 1399.8 m | 253.5 m |

In band

We have also considered the in-band deployment scenario with BS Tx power of 38 dBm, our results are summarized in Table 7 and Table 8, respectively for penetration rates of 0 and 20dB.

Table 22: Coverage distance (m), Device C, without penetration loss, OOK for D2R, in band

|  | UMa |  | RMa |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -3 | 1956.7 m | 302.8 m | 3422.8 m | 518.3 m |
| 0 | 1956.7 m | 361.3 m | 3422.8 m | 619.8 m |
| 5 | 1956.7 m | 485.2 m | 3422.8 m | 835.0 m |

Table 23: Coverage distance (m), Device C, with penetration loss, OOK for D2R, in band

|  | UMa |  | RMA |  |
| --- | --- | --- | --- | --- |
| Device Tx power [dBm] | R2D | D2R | R2D | D2R |
| -3 | 602.2 m | 93.2 m | 1039.1 m | 157.3 m |
| 0 | 602.2 m | 111.2 m | 1039.1 m | 188.1 m |
| 5 | 602.2 m | 149.3 m | 1039.1 m | 253.5 m |

For both Device 2b and Device C, there is a significant difference between the R2D and D2R coverage. In all evaluated scenarios, the D2R link acts as the main bottleneck. The R2D link, on the other hand, consistently achieves coverage beyond 500 m. Since the overall coverage is limited by the D2R link, adding FEC or other coverage enhancement techniques to the R2D link does not improve the total coverage. Therefore, any coverage enhancement efforts should focus on the D2R link.

- For both Device 2b and Device C, there is a clear difference between R2D and D2R coverage.

- In all considered scenarios, the D2R link is the main bottleneck, while the R2D link has coverage above 500 m.

- Since the D2R link limits overall coverage, adding FEC or other improvements to R2D does not increase coverage. Coverage enhancement should focus on the D2R link.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	For Rel-20 A-IoT outdoor devices, the coverage target should reflect the differentiation from 6G LPWA, consider the SID guidelines and the mentioned distance range of 50 to 500 m.

Observation 2	For Device C, even a capacitor size of 100 µF may not be sufficient for transmitting large payload sizes at lower data rates.

Observation 3	For no-FEC case on R2D,

    - Best performance is achieved at M=2 for device 2b and C, considering the trade-off between OOK symbol duration and overall transmission time.

    - Device 2b cannot decode a 400-bit message correctly under the considered SFO value.

    - For device 2b, M=6 requires approximately [1, 3.3] dB higher SNR at 10% BLER than M=2 for message bits of [20, 96], respectively.

    - For device C, M=6 requires approximately [2, 3.5, 5] dB higher SNR at 10% BLER than M=2 for message bits of [20, 96, 400], respectively.

Observation 4	With 1/3 TBCC FEC on R2D,

    - For device 2b, block decoding is only possible when the message size is no greater than 20 bits and the performance improvement is around 4.5~6 dB @10% BLER.

    - For device C, the performance improvement caused by FEC increases as M increases and the message size becomes larger, but fails to reach 10% BLER at M=1 for 400 message bits.

Observation 5	For both Device 2b and Device C, there is a clear difference between R2D and D2R coverage.

Observation 6	In all considered scenarios, the D2R link is the main bottleneck, while the R2D link has coverage above 500 m.

Observation 7	Since the D2R link limits overall coverage, adding FEC or other improvements to R2D does not increase coverage. Coverage enhancement should focus on the D2R link.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Any reconsiderations or enhancements beyond the values for maximum distance between device and reader specified in the SID should be discussed at the RAN plenary level.

# References

TS 38.291, “Technical Specification Group Radio Access Network; Ambient IoT Physical layer”

TR 38.769 V19.0.0, “Study on Solutions for Ambient IoT (Internet of Things)”, 3GPP.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, 3GPP.

RP-251840, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, June 2025.

11-22/1562-04-0amp, IEEE 802.11 Working Group, “Draft Technical Report on Support of AMP-IoT Devices in WLAN,” IEEE, Nov. 2022.

TR 38.869 V18.0.0, “Study on low-power Wake-up Signal and Receiver for NR," Release 18, Jan. 2024.