3GPP TSG-RAN WG1 Meeting #123	R1-2508512

Dallas, USA, November 17th – 21st 2025

Agenda Item:	10.3.2.1

Source:	Ericsson

Title:	R2D signals, channels, waveform, and procedures

Document for:	Discussion, Decision

# Introduction

The Rel-20 study item “Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” [1] is to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technologies e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT.

The SID [1] states the following regarding SI objective:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. Define link budget calculation for coverage

In this contribution, we present our views on the necessary and feasible changes to the Rel-19 air interface for Device 2b/C, including changes to the R2D waveform and modulation, line coding, FEC, CRC and repetitions, bandwidth, timing and synchronization signals, L1 control/scheduling, and multiplexing.

# R2D: Physical channels and signals generation

## R2D waveform and modulation

RAN1#122 made the following agreement regarding R2D waveform and modulation:

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24

The selection of the proper modulation order M for OOK-4 requires consideration of the following criteria:

- Bandwidth: The transmission bandwidth limits the maximum value of M. For example, for 1 PRB bandwidth with a dual-sideband (DSB) signal, M can be a divisor of 12, but smaller than 6. Similarly, for 2 PRBs, M can be a divisor of 24, but smaller than 12.

- Impact of CP insertion: With increasing modulation order M, the relative size of the CP to the ON/OFF chip duration of OOK increases. Consequently, there is a risk of the CP being mistakenly treated as valid data, leading to degraded detection performance.

- Combining gain of energy detection: Higher modulation order M degrades OOK detection performance due to reduced ON chip duration and total energy, resulting in worse coverage for A-IoT transmission.

- Robustness against time and frequency offsets: For higher modulation orders (M), timing and frequency errors have a more pronounced impact

Table 1 shows the peak data rate and ON/OFF duration for different M values.

Table 1: ON/OFF duration and data rate for different M and 15 kHz SCS

|  | M=1 | M=2 | M=6 | M=12 | M=24 | M=32 |
| --- | --- | --- | --- | --- | --- | --- |
| ON/OFF duration | 66.67 µs | 33.33 µs | 11.11 µs | 5.56 µs | 2.77 µs | 2.08 µs |
| Peak Data rate (without Manchester encoding) | 14 kbps | 28 kbps | 84 kbps | 168 kbps | 336kbps | 448 kbps |
| Peak Data rate (with Manchester encoding) | 7 kbps | 14 kbps | 42 kbps | 84 kbps | 168 kbps | 224 kbps |

To down-select the proper M values, we provide extensive simulation results in Figure 1 and Figure 2. The simulation parameters and assumptions are summarized in Table 2.

For Device 2b and Device C, a 10% BLER cannot be achieved with M = 24 or 32.

Support M = 1, 2, 6, and 12 for Device 2b and Device C.

Table 2: R2D simulation parameters

| Parameters | Assumptions |  |
| --- | --- | --- |
| Msg Info | 96 bits |  |
| Device | 2b | c |
|  | CFO: 100 ppm SFO : 1000 ppm | CFO: 10 ppm SFO: 10 ppm |
| M | [1,2,6,12,24,32] |  |
| Number of PRBs | 6 |  |
| Sampling rate | 1.92 Msamples/scond |  |
| Device architecture | ZIF |  |

Figure 1: BLER vs. SNR for different values of M for device 2b.

Figure 2: BLER vs. SNR for different values of M for device C.

## R2D coding, FEC, CRC and repetitions

### R2D Line coding

R2D transmission supports only the Manchester line code. Manchester encoding doubles the bandwidth/time usage compared to schemes without line coding; however, OOK modulation without Manchester encoding suffers from a few drawbacks:

- Synchronization issues: The absence of an embedded clock signal makes synchronization challenging. Additionally, long sequences of consecutive 0s or 1s can cause DC drift, leading to synchronization loss and decoding errors.

- Threshold sensitivity: To decode OOK without Manchester encoding, the receiver must set a precise threshold to distinguish between 0 and 1, which can be challenging in noisy environments or with fluctuating signal strength.

- Noise susceptibility: The lack of inherent error detection mechanisms makes OOK without Manchester encoding more susceptible to noise and interference.

OOK modulation without Manchester encoding has several drawbacks, such as synchronization issues, threshold sensitivity, and susceptibility to noise.

Given these drawbacks of OOK without Manchester encoding, we propose the following.

Manchester line coding also needs to be supported for Device 2b/C in the R2D link.

### R2D Scrambling

FL made the following proposal on the necessity and feasibility of scrambling for PRDCH in RAN1#122b.

[H] Proposal 4.2a-r1 For PRDCH, study necessity and feasibility of scrambling for Device 2b and for Device C for the following purposes. - Interference randomization - To avoid continuous 1s or 0s (and replace Manchester encoding for better spectral efficiency)

Scrambling is a bit-level operation used to randomize the transmitted bit patterns so that the signal appears statistically uniform. This prevents long runs of identical bits, which can degrade clock recovery and synchronization at the receiver. By randomizing the sequence, scrambling also ensures a more even power spectral distribution, reducing narrowband interference and avoiding spectral peaks that may occur with structured data patterns. Although scrambling does not add redundancy or perform error correction, it enhances the effectiveness of other physical-layer techniques (such as FEC) by providing a well-distributed bitstream that is more robust to channel impairments. In addition, it reduces inter-cell interference.

The scrambling and descrambling operations require only a small Linear Feedback Shift Register (LFSR) and one XOR operation per bit, resulting in negligible computational and hardware complexity at both the transmitter and receiver.

R2D scrambling is necessary to reduce inter-cell interference and ensure statistical uniformity, preventing long runs of identical bits and improving clock recovery, synchronization, and spectral efficiency if no Manchester coding option is supported

Scrambling and descrambling are feasible, requiring only a small LFSR and one XOR per bit, leading to negligible hardware complexity at both transmitter and receiver.

R2D scrambling for PRDCH is necessary and feasible.

### R2D FEC

RAN1#122 made the following agreement regarding R2D FEC:

Agreement Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects: - Coding schemes: convolutional code (e.g TBCC), RM code - Note: focus on coding schemes already specified in LTE or NR - FEC code rates: at least 1/2, 1/3 - Interleaver or LTE bit collection scheme for TBCC

RAN1#122b made the following agreement regarding R2D FEC:

Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, ¼ Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, ¼ Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility. Agreement Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied

Channel coding can potentially provide a performance gain over uncoded transmissions by extending coverage but requires additional complexity. The trade-offs between error correction, bandwidth utilization, and computational complexity should be considered. Given A-IoT design constraints, it is important to prioritize low-complexity encoding and decoding in the device. The convolutional code with generators (133,171,165)8 implies:

  - Memory m=6 (highest polynomial degree D6); trellis states 2m =64

    - 1388=10110112  G0(D)=D6+D4+D3+D+1

    - 1718=11110012  G1(D)=D6+ D5+D4+D3+1

    - 1658=11101012  G2(D)=D6+ D5+D4+D2+1

Figure 3 and Figure 4 compare the BLER performance versus Eb/N₀ for Devices 2b and C, respectively, with and without TBCC FEC.

Figure 3: BLER vs Eb/N0 for different M values for device 2b

Figure 4: BLER vs Eb/N0 for different M values for device C

The coverage analysis in our companion contribution [8] shows that Device C can reach the RAN design target of 500 meters without FEC.

Device C can reach the RAN design target of 500 meters without FEC.

FEC is not necessary for Device C to reach the RAN design target.

Similar to Rel-19 A-IoT, R2D transmissions can be assumed to not use FEC as baseline.

### R2D repetitions

RAN1#122 made the following agreement regarding R2D repetitions:

Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied)

RAN1#122b made the following agreement regarding R2D repetitions:

Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

TR 38.769 defines repetition as follows:

- Block-level repetitions: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times

- Bit-level Type-2 repetitions: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times

- Chip-level repetitions (in case of M=1 with FEC if applied): Each chip after line coding (if used) or after square wave modulation (if used) is repeated Rchip time.

As discussed during the Rel-19 A-IoT SI, chip-level repetition is almost equivalent to long chip transmission (i.e., smaller modulation index M) and therefore does not need to be supported. Block-level repetition requires a large volatile memory to store all received repetitions of a block, which could be challenging for Device 2b and C.

Chip-level repetition is equivalent to long chip transmission, i.e., smaller modulation index M

Block-level repetition requires a large volatile memory to store all received repetitions of a block.

The coverage analysis in our companion contribution [8] shows that Device C can reach the RAN design target of 500 meters without repetition.

Repetition is not necessary for Device C to reach the RAN design target.

Similar to Rel-19 A-IoT, R2D transmissions can be assumed to not use repetitions as baseline.

### R2D CRC

Both PRDCH and PDRCH support transmission using the generator polynomials specified in TS 38.212:

- CRC-6 polynomial: x6+x5+1

- CRC-16 polynomial: x16+x15+x2+1 (which is the same as RFID CRC)

A threshold of 24 bits is used to select between CRC-6 and CRC-16. A similar CRC polynomial and threshold can be considered for Device 2b/C in Rel-20.

For Device 2b/C in Rel-20, a similar CRC polynomial and threshold as in Rel-19 can be considered.

## R2D bandwidth

The channel bandwidths and maximum transmission bandwidths supported in Rel-19 for A-IoT are the ones depicted in Figure 5.

Figure 5: Channel bandwidths and maximum transmission bandwidths currently supported for Rel-19

During RAN1#122-bis discussions, it was mentioned that the Frequency Division Multiplexing (FDM) technique for Rel-20 A-IoT will require a much larger transmission bandwidth. At some point, a 5 MHz channel bandwidth which consists of 25 PRBs was mentioned as an example. If a larger transmission bandwidth were deemed needed for A-IoT outdoor, then to minimize specification impacts the smallest channel bandwidths available in NR can be considered which are 3 MHz CBW and 5 MHz CBW as depicted below:

Figure 6: Smallest channel bandwidths and maximum transmission bandwidths supported in 5GNR

One aspect that needs to be understood is that the concept of transmission bandwidth and channel bandwidth go together, especially in an outdoor scenario because the latter one gives the RF framework and both DL and UL are subject to the same RF framework. This is important because RAN1 and RAN4 need to align terminologies, which is essential for discussing the location of the potential A-IoT channel bandwidths to be deployed within NR band n8, and eventually the sync-raster design. For example, if the A-IoT channel bandwidth (CBW) were to consist of 5 MHz, then up to 7 CBWs of 5 MHz each can be deployed into the 35 MHz composing NR band n8:

Figure 7: Total number of A-IoT channel bandwidths of 5MHz that can be deployed across NR band n8.

The concept of transmission bandwidth and channel bandwidth go together, especially in an outdoor scenario because the latter one gives the RF framework and both DL and UL are subject to the same RF framework. This is essential for discussing the location of the potential A-IoT channel bandwidths to be deployed within NR band n8 (see Figure above), and eventually the sync-raster design.

In Rel-20 A-IoT, if FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW and 5 MHz CBW can be considered. The maximum transmission bandwidth is 15-PRBs for the 3 MHz CBW, whereas the maximum transmission bandwidth is 25 PRBs for the 5 MHz CBW.

In Rel-20 A-IoT, if FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW and 5 MHz CBW can be considered.

## Sync signals

Before discussing the sync signal(s), it is important to get an overall picture of the very first time the sync-signal is used and the subsequent usage of it. The figure below depicts our current understanding of the A-IoT outdoor initial access procedure, which shows that the sync-signal is the very first signal used for establishing communication between a given device and a specific reader at a given designated frequency.

Figure 10: Rel-20 A-IoT outdoor initial access procedure

In Rel-20 A-IoT, both the device and the reader (base station) will be outdoors, even in a licensed spectrum, the device will be exposed to a variety of radio frequency (RF) signals from multiple sources, therefore the receiver must filter those signals to communicate with a specific reader at a designated frequency (sync-raster point), where the device will detect and decode a sync-signal. In the subsections below we provide our views on design considerations for the sync-signal and the system information broadcast for Rel-20 A-IoT outdoor.

#### Receiver architecture considerations

For Rel-20 A-IoT, the device is active and two receiver architectures can be considered: the IF-ED receiver and the ZIF receiver. As illustrated in TR 38.769, these architectures are shown in Figure 8 and Figure 9.

In the IF-ED architecture, the envelope detector eliminates the phase component of the received signal while preserving the energy-related information for subsequent baseband processing. As a result, using an IF-ED receiver for the cell search processing in baseband differs from that of a conventional OFDM receiver. For example, since correlation with predefined sequences requires phase information, then existing signals such as the SSB cannot be reused by a device based on an IF-ED receiver architecture.

For devices with IF-ED architecture (which eliminates the phase component of the received signal), existing synchronization signal blocks like SSB cannot be reused for reader/cell search.

Figure 11: Architecture of IF-ED receiver

Figure 12: Architecture of ZIF receiver

In the ZIF architecture, the envelope detector is implemented in the baseband digital domain, enabling the reuse of an existing SSB block for cell search. However, this approach introduces significant complexity. For example, in the case of existing an SSB block, the architecture would need to comply with the existing synchronization raster rules, and the analog-domain low-pass filter (LPF) would need to adapt to the SSB bandwidth. On the other hand, baseband processing in the digital domain needs to support both OFDM-based and OOK-based signal processing, further increasing system complexity.

For devices with a ZIF architecture, reusing an existing SSB block for reader/cell search introduces significant complexity and requires the baseband processing in the digital domain to support both OFDM-based and OOK-based signal processing.

Thus, to reduce device complexity, a new synchronization signal can be introduced for reader/cell search in Rel-20 which should be agnostic to the device type.

Introduce new synchronization signal(s) for reader/cell search (e.g., OOK waveform), such that it can be detected agnostically to the receiver architectures considered for Rel-20 A-IoT outdoor devices.

#### Synchronization design

Rel-20 A-IoT outdoor can operate in NR bands (e.g., NR band n8 is mentioned in the SID), thus one question is whether we can expect a simultaneous operation of A-IoT outdoor and NR co-existing on the same spectrum within a given NR band, or if in such spectrum within a given NR band will be dedicated to either A-IoT or NR (i.e., one RAT or the other but not both simultaneously). This is depicted in the figure below, which in our current understanding could impose different design constraints on e.g., the sync-signal design and other A-IoT signals and channels.

| - Spectrum exclusively used by NR | - Spectrum exclusively used by A-IoT | - Spectrum simultaneously used by both NR and A-IoT |
| --- | --- | --- |

Figure 12: Potential spectrum utilization in an NR band: A) Spectrum exclusively used by NR, or B) Spectrum exclusively used by A-IoT, or C) Spectrum simultaneously used by NR and A-IoT

There might be design constraints to consider e.g., on the sync-signal, depending on whether in an NR band i) The spectrum is intended to be exclusively used by A-IoT or ii) The spectrum is intended to be simultaneously used by both NR and A-IoT.

RAN1 to discuss whether in an NR band, i) The spectrum is intended to be exclusively used by A-IoT or ii) The spectrum is intended to be simultaneously used by both NR and A-IoT. If “i)” then the sync-signal (and other PHY-layer aspects) may be subject to design constraints (e.g., aiming at collision avoidance at least to some extent) as to co-exist with NR.

While above gets clarified, the synchronization signal design used for reader/cell search is discussed in the subsection below without accounting for any specific constraint derived from a simultaneous operation/co-existence of Rel-20 A-IoT outdoor with NR.

##### Synchronization signal used for cell search: SS1 design

The synchronization signal for A-IoT outdoor (a.k.a. SS1) employed for reader/cell search shall be transmitted periodically according to a set of predefined candidate values. Assume the worst-case duration between SS1 and PRDCH is approximately equal to the SS1 periodicity. The table below illustrates the timing drift per period caused by sampling frequency offset (SFO) for different SS1 periodicities. Dense SS1 transmission can accelerate reader/cell search and improve timing synchronization at the cost of increased resource consumption (and energy consumption at the device side) and signaling overhead, whereas sparse SS1 transmission may reduce resource usage but require extra preamble overhead for reliable PRDCH demodulation.

Table 3: The timing drift per period caused by sampling frequency offset (SFO) for different SS1 periodicities

| Periodicity(ms) | SFO (ppm) | Time drift (us) |
| --- | --- | --- |
| 80 | 100 | 8 |
| 160 | 100 | 16 |
| 320 | 100 | 32 |
| 80 | 10 | 0.8 |
| 160 | 10 | 1.6 |
| 320 | 10 | 3.2 |

The synchronization signal used for reader/cell search (a.k.a. SS1) shall be transmitted periodically, with a periodicity of 160 ms. FFS other values (if any).

The number of sequences should be small to reduce the payload and complexity on the device. From a cell deployment perspective, the sequence design could allow for 3 or 4 sequences per set, with the exact number determined based on the deployment of cells to ensure proper differentiation between neighboring cells. Each cell is allocated one sequence from this set to enable identification and differentiation of neighboring cells. To minimize inter-cell interference, the sequences should exhibit good autocorrelation and cross-correlation properties. The detailed sequence design is FFS and can refer to the LP-SS defined in the Rel-19 WUR study.

The number of SS1 sequences to enable identification and differentiation of neighboring readers/cells should be small, for example up to 3 or 4 sequences per set with good auto-correlation and cross-correlation properties.

Since SS1 is used for reader/cell search and regular timing synchronization, the binary sequence pattern for the OOK waveform should provide robust detection performance and high timing accuracy. Pseudo-random sequences, such as m-sequences or Gold sequences, can be considered for generating the synchronization signal waveform. To enhance detection and timing performance, the sequence should avoid long runs of consecutive ON or OFF symbols and increase the number of ON/OFF transitions (i.e., the sequence should have good auto-correlation properties).

Pseudo-random binary sequences, such as m-sequences or Gold sequences, could be considered for use as the OOK sequence pattern for the synchronization signal.

##### CFO calibration

According to the Rel-20 A-IoT device LLS assumptions, the initial carrier frequency offset (CFO) is 103~104 ppm for device 2b and 50 ppm for device C. After CFO calibration, the residual CFO is 100 ppm for Device 2b and 10 ppm for Device C.

When the carrier frequency is 900 MHz, the initial frequency offset of device 2b can be as large as 9 MHz. Such a large CFO may place the received signal outside the signal pass filter bandwidth if the filter design does not account for this offset, causing the device to miss the synchronization signals. To avoid this issue, one possible way is to make the low-pass filter (LPF) bandwidth larger than the sum of the initial frequency offset and the signal bandwidth during the cell search. As a result, the bandwidth would grow with carrier frequency increases.

Figure 12: Example of a low-pass filter accounting for the initial CFO.

Observation: During reader/cell search, the signal pass filter bandwidth should take the initial CFO into account to avoid miss detection, and adjacent carrier interference grows as the carrier frequency increases (This also implies that the A-IoT sync-raster points must be sufficiently far apart from each other as to take the initial CFO into account).

To further reduce interference after passing a wideband filter that accounts for CFO, the device will likely need to perform a finer frequency search to detect the synchronization signal, which in turn further reduces the initial CFO.

After detecting the synchronization signal, the device needs to perform CFO calibration. From TR 38.769, the CFO drift is only 0.1ppm/s or 1ppm/s which is very small. Therefore, once calibrated, the device does not require further CFO adjustment for at least several seconds.

Observation: According to TR 38.769 the CFO drift is slow only 0.1ppm/s or 1ppm/s, Therefore, the CFO calibration can be performed infrequently (no adjustment required for at least several seconds) due to the minimal CFO drift.

CFO calibration using an unmodulated sinusoid single tone

One feasible approach for CFO calibration is to use a single-carrier waveform and execute the calibration in the RF domain. This method is relatively simple to implement, but it requires transmitting an additional periodic signal for calibration and is sensitive to adjacent-channel interference as well as channel fading.

Observation: For CFO calibration, single-carrier (a.k.a. unmodulated sinusoid single tone) method is simple but needs additional periodic signal.

CFO calibration using the synchronization signal

Another approach is to reuse the synchronization signal to estimate the frequency offset and apply compensation in the baseband domain. After detecting the synchronization signal, the device can estimate the frequency offset by analyzing the signal power across different frequency offsets, in order to satisfy the calibrated CFO requirements of 100 ppm for device 2b and 10 ppm for device c. For example, the reader transmits the synchronization signal using a centered bandwidth sweeping method, where each centered bandwidth covers a portion of the overall synchronization signal bandwidth as shown in Figure 3, e.g., if the synchronization signal spans 12 subcarriers, then three sweeping candidates (i.e., subsets of subcarriers) can be pre-configured, each subset occupying6 subcarriers from the total 12 (Note: the subcarriers in the sets may overlap).

Figure 12: Example of using the sync-signal for CFO calibration

The device detects the synchronization signal power at different strengths on each of the subsets and based on it performs CFO compensation. For example, if the strongest synchronization signal power was detected at the uppermost subset (green box in the above figure) then the device applies a frequency correction or shift of - 6xfsc Hz, where fsc is the frequency width of each subcarrier e.g., 15 000 Hz. Similarly, if the strongest synchronization signal power was detected at the leftmost subset (orange box in the above figure) then the device applies a frequency correction or shift of + 6xfsc Hz, and finally if the strongest synchronization signal power was detected at the middle subset (blue box in the above figure) then no frequency correction is performed.

Observation: Perform CFO estimation by using synchronization signal is feasible within certain residual frequency error range.

Observation: Using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm), whereas the CFO calibration of device C may require using single-tone (a.k.a. unmodulated sinusoid single tone) to meet a residual CFO of 10 ppm.

Proposal: If Rel-20 A-IoT outdoor were to support only device 2b then the sync-signal can be used for CFO calibration, whereas if only device C or both devices 2b and C were to be supported then a single-tone (a.k.a. unmodulated sinusoid single tone) signal can be supported for CFO calibration.

## R2D control information

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted Agreement For the functionality of chip duration determination of the data payload in the PRDCH, study the following options: - Option 1: same as the chip duration of the corresponding L1 R2D control information - Option 2: indicated by the corresponding L1 R2D control information Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

In this section, we focus on R2D control information associated with R2D scheduling. The R2D control information relevant for D2R scheduling is discussed in our companion contribution for D2R design.

In Rel-19, the R2D scheduling parameters include the M value (or chip duration) and the R2D TBS information (excluding CRC length). The M value of the PRDCH is provided by the CAP part of R-TAS, whereas R2D TBS information is provided in higher-layer signaling (at least for messages with variable size). Furthermore, the end of PRDCH can be indicated via an R2D postamble.

In Rel-20, the R2D scheduling parameters may potentially also include additional parameters (i.e., in addition to M values and the R2D TBS information), depending on the outcome of the coverage discussions in RAN1. The additional parameters include, e.g., MCS-like information, such as number of repetitions and FEC code rate, as well as frequency domain information to support R2D FDM for different devices. Furthermore, if the additional parameters described above are introduced, all control signaling (including M values and TBS) can be carried in the R2D control information (instead of using CAP and postamble for M values and TBS). In other words, we do not think that CAP and postamble are needed if L1 R2D control information is introduced.

Also, the R2D control information should be carried in R2D L1 control information (and not higher-layer control information). This makes it easier for the device to decode the control information and proceed with decoding the data payload in PRDCH. With regards to the necessary information to be included in the L1 R2D control, it’s worth considering it on a case-by-case basis as follows:

Paging PRDCH

The IDs of one or more device may be multiplexed in the same paging PRDCH. Since the number of devices that are paged is not known beforehand, the TBS of the paging PRDCH can vary. Additional information such as M value, and if relevant, the number of repetitions and the code rate can cater to the coverage-edge scenarios (a similar approach is also taken in NR where the MCS value corresponds to cell-edge scenarios). That is, these values can be fixed and correspond to those that provide the best coverage.

Furthermore, depending on if there would be flexibility to place the paging PRDCH anywhere within the A-IoT carrier, the frequency domain information would be needed. However, this aspect needs further discussion in RAN1. Note that R2D FDM for paging PRDCH is not needed.

In summary, TBS could be the only control information that needs to be indicated for paging PRDCH. For TBS indication, either L1 R2D control information or postamble can be considered. If additional flexibility is needed (e.g., the M value), they can be considered but are strictly not necessary.

Broadcast PRDCH

For broadcast PRDCH, depending on the need for potential A-IoT enhancements in future releases, the TBS may or may not vary. That is, if there is a need to increase the contents of broadcast information, TBS need to be indicated, otherwise, not. If TBS needs to be indicated for broadcast PRDCH, either L1 R2D control information or postamble can be considered, similar to paging PRDCH. As also discussed for paging PRDCH, for additional flexibility, information such as the M value can be considered but it is strictly not necessary.

Other PRDCH (Msg2, Msg4. etc.)

Other PRDCH transmissions, such as Msg2 and Ms4, would benefit from additional flexibility compared to paging and broadcast PRDCH. Therefore, the TBS, the M value, and the number of repetitions and the code rate (if considered), can be included in the L1 control information. Additionally, since R2D FDM could also be supported, the frequency domain information may need to be included as well.

For paging PRDCH, the necessary information to be indicated includes at least TBS. The TBS can be indicated via L1 R2D control information or postamble.

      - Our preference is to use postamble than L1 R2D control information.

      - The M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

      - FFS whether the frequency domain information would be needed so as to place paging PRDCH anywhere within the A-IoT carrier.

For broadcast PRDCH, RAN1 discusses the need for variable TBS, e.g., considering potential needs to increase the contents of broadcast information in a future release. Depending on this need, the TBS may or may not need to be indicated.

      - If needed, the TBS can be indicated via L1 R2D control information or postamble where our preference is to use postamble than L1 R2D control information

      - The M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

For other PRDCH such as Msg2 and Msg4, the necessary information to be included in the L1 R2D control include at least TBS, the M value, the frequency domain information, and if considered, the number of repetitions and the code rate.

Regarding chip duration (M value) determination of the data payload in the PRDCH, for paging and broadcast PRDCH we do not see a strong need for flexible M values, as discussed above. Therefore, the chip duration can be fixed. On the other hand, for Msg2 and Msg4 PRDCH, which could benefit from additional flexibility, the M value can be indicated in the corresponding L1 R2D control information (i.e., Option 2).

For the functionality of chip duration determination of the data payload in the PRDCH, the chip duration is fixed for paging and broadcast PRDCH and indicated by the corresponding L1 R2D control information for other PRDCH such as Msg2 and Msg4.

Regarding chip duration (M value) determination of the L1 R2D control information, we think it should be fixed chip duration (i.e., Option 1). The other options such as blind detection of the chip duration from predefined set (Option 2), use of CAP (Option 3), use of a set of different binary sequences (Option 4) would make the decoding of the L1 R2D control information more complicated (and consequently, more time consuming) for the device. Note that L1 R2D control information should be received with high reliability (e.g., 1% BLER) compared to the data payload in the PRDCH (e.g., 10% BLER). It is unclear whether Options 2, 3, and 4 can provide such high reliability and require additional work, including simulations, to design and validate the solutions in RAN1. The use of broadcast information to indicate the chip duration (Option 5) is also a simple solution but does not work for paging and broadcast PRDCH. We do not see a need for additional flexibility for chip duration of the L1 control information for Msg2 and Msg4 PRDCH alone.

Therefore, our preference is to have a fixed chip duration (M value) for the L1 R2D control information. The fixed M value can be the smallest M value corresponding to the best coverage. We think that TBS can also be fixed to make it easier for the device to decode L1 control information. Note that, even in NR, DCI has a fixed (small) MCS to provide high reliability and good coverage.

For the functionality of chip duration determination of the L1 R2D control information, choose Option 1 (Fixed chip duration; no use of CAP for chip duration determination) for all L1 R2D control information.

Regarding the time location of L1 R2D control information, i.e., whether L1 R2D control information and data payload of PRDCH are contiguous in time (Option 1) or not (Option 2), our understanding is that it should be Option 2, i.e., there should be a short gap between the L1 R2D control information and the data payload of PRDCH if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH simultaneously. This gives required time for the device to decode the L1 R2D control information and do the CRC check associated with the control information. This prevents devices having to buffer for long periods of time which could negatively impact the power consumption at the device. This is particularly important considering that there may not be a postamble indicating the end of PRDCH in Rel-20. On the other hand, i.e., if the device is capable of decoding and buffering simultaneously, no time gap may be needed.

This also points to the direction that L1 R2D control information is included in another PRDCH than the PRDCH that carries that data payload.

For time location of L1 R2D control information, L1 R2D control information and data payload of PRDCH are not contiguous in time if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH simultaneously. Otherwise, it can be contiguous.

## R2D multiplexing

In RAN1 #122b meeting, the following was agreed on R2D FDM.

Agreement Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects: - complexity of device to support FDM - how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

During the meeting, there were discussions on the impact of bandwidth on R2D FDM, but no agreement reached due to divergent views on R2D bandwidth. Based on our view on R2D bandwidth in Section 2.3, we illustrate our understanding of R2D FDM from bandwidth perspective in Figure 14. Taking 5MHz channel bandwidth as an example, the maximum transmission bandwidth is 25 PRBs and guard bands on both sides of the channel are needed for co-existing with adjacent channels which could be other NR transmissions or Ambient IoT transmissions by another operator. Within the 25 PRBs, multiple transmissions can be allocated by the reader with transmission bandwidth 1, 2, 3 etc. Guard bands are also needed between transmission bandwidths due to CFO on the device side. From the device perspective, it will be configured with a center frequency and a corresponding bandwidth for the R2D reception which will cover the transmission bandwidth and guard band. Therefore, the design of R2D FDM depends on both transmission bandwidth and channel bandwidth.

Guard bands are also needed between transmission bandwidths to compensate the CFO on the device side.

Consider channel bandwidth, transmission bandwidth and guard band for R2D FDM from the same reader for Device 2b and for Device C.

Figure 14 Illustration of R2D FDM with 5MHz bandwidth

# Conclusion

In the previous sections we made the following observations:

Observation 1	For Device 2b and Device C, a 10% BLER cannot be achieved with M = 24 or 32.

Observation 2	OOK modulation without Manchester encoding has several drawbacks, such as synchronization issues, threshold sensitivity, and susceptibility to noise.

Observation 3	R2D scrambling is necessary to reduce inter-cell interference and ensure statistical uniformity, preventing long runs of identical bits and improving clock recovery, synchronization, and spectral efficiency if no Manchester coding option is supported

Observation 4	Scrambling and descrambling are feasible, requiring only a small LFSR and one XOR per bit, leading to negligible hardware complexity at both transmitter and receiver.

Observation 5	Device C can reach the RAN design target of 500 meters without FEC.

Observation 6	FEC is not necessary for Device C to reach the RAN design target.

Observation 7	Chip-level repetition is equivalent to long chip transmission, i.e., smaller modulation index M

Observation 8	Block-level repetition requires a large volatile memory to store all received repetitions of a block.

Observation 9	Repetition is not necessary for Device C to reach the RAN design target.

Observation 10	The concept of transmission bandwidth and channel bandwidth go together, especially in an outdoor scenario because the latter one gives the RF framework and both DL and UL are subject to the same RF framework. This is essential for discussing the location of the potential A-IoT channel bandwidths to be deployed within NR band n8 (see Figure above), and eventually the sync-raster design.

Observation 11	In Rel-20 A-IoT, if FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW and 5 MHz CBW can be considered. The maximum transmission bandwidth is 15-PRBs for the 3 MHz CBW, whereas the maximum transmission bandwidth is 25 PRBs for the 5 MHz CBW.

Observation 12	For devices with IF-ED architecture (which eliminates the phase component of the received signal), existing synchronization signal blocks like SSB cannot be reused for reader/cell search.

Observation 13	For devices with a ZIF architecture, reusing an existing SSB block for reader/cell search introduces significant complexity and requires the baseband processing in the digital domain to support both OFDM-based and OOK-based signal processing.

Observation 14	There might be design constraints to consider e.g., on the sync-signal, depending on whether in an NR band i) The spectrum is intended to be exclusively used by A-IoT or ii) The spectrum is intended to be simultaneously used by both NR and A-IoT.

Observation 15	Guard bands are also needed between transmission bandwidths to compensate the CFO on the device side.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Support M = 1, 2, 6, and 12 for Device 2b and Device C.

Proposal 2	Manchester line coding also needs to be supported for Device 2b/C in the R2D link.

Proposal 3	R2D scrambling for PRDCH is necessary and feasible.

Proposal 4	Similar to Rel-19 A-IoT, R2D transmissions can be assumed to not use FEC as baseline.

Proposal 5	Similar to Rel-19 A-IoT, R2D transmissions can be assumed to not use repetitions as baseline.

Proposal 6	For Device 2b/C in Rel-20, a similar CRC polynomial and threshold as in Rel-19 can be considered.

Proposal 7	In Rel-20 A-IoT, if FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW and 5 MHz CBW can be considered.

Proposal 8	Introduce new synchronization signal(s) for reader/cell search (e.g., OOK waveform), such that it can be detected agnostically to the receiver architectures considered for Rel-20 A-IoT outdoor devices.

Proposal 9	RAN1 to discuss whether in an NR band, i) The spectrum is intended to be exclusively used by A-IoT or ii) The spectrum is intended to be simultaneously used by both NR and A-IoT. If “i)” then the sync-signal (and other PHY-layer aspects) may be subject to design constraints (e.g., aiming at collision avoidance at least to some extent) as to co-exist with NR.

Proposal 10	The synchronization signal used for reader/cell search (a.k.a. SS1) shall be transmitted periodically, with a periodicity of 160 ms. FFS other values (if any).

Proposal 11	The number of SS1 sequences to enable identification and differentiation of neighboring readers/cells should be small, for example up to 3 or 4 sequences per set with good auto-correlation and cross-correlation properties.

Proposal 12	Pseudo-random binary sequences, such as m-sequences or Gold sequences, could be considered for use as the OOK sequence pattern for the synchronization signal.

Proposal 13	For paging PRDCH, the necessary information to be indicated includes at least TBS. The TBS can be indicated via L1 R2D control information or postamble.

	Our preference is to use postamble than L1 R2D control information.

	The M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

	FFS whether the frequency domain information would be needed so as to place paging PRDCH anywhere within the A-IoT carrier.

Proposal 14	For broadcast PRDCH, RAN1 discusses the need for variable TBS, e.g., considering potential needs to increase the contents of broadcast information in a future release. Depending on this need, the TBS may or may not need to be indicated.

	If needed, the TBS can be indicated via L1 R2D control information or postamble where our preference is to use postamble than L1 R2D control information

	The M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

Proposal 15	For other PRDCH such as Msg2 and Msg4, the necessary information to be included in the L1 R2D control include at least TBS, the M value, the frequency domain information, and if considered, the number of repetitions and the code rate.

Proposal 16	For the functionality of chip duration determination of the data payload in the PRDCH, the chip duration is fixed for paging and broadcast PRDCH and indicated by the corresponding L1 R2D control information for other PRDCH such as Msg2 and Msg4.

Proposal 17	For the functionality of chip duration determination of the L1 R2D control information, choose Option 1 (Fixed chip duration; no use of CAP for chip duration determination) for all L1 R2D control information.

Proposal 18	For time location of L1 R2D control information, L1 R2D control information and data payload of PRDCH are not contiguous in time if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH simultaneously. Otherwise, it can be contiguous.

Proposal 19	Consider channel bandwidth, transmission bandwidth and guard band for R2D FDM from the same reader for Device 2b and for Device C.

# References

RP-251840  “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, June 2025.

TS 38.291, “Technical Specification Group Radio Access Network; Ambient IoT Physical layer”

TR 38.769 V19.0.0, “Study on Solutions for Ambient IoT (Internet of Things)”, 3GPP.

TR 38.848 V18.0.0, “Study on Ambient IoT (Internet of Things) in RAN”, 3GPP

TR 38.869 V18.0.0, “Study on low-power Wake-up Signal and Receiver for NR (Release 18)”, 3GPP, January 2024.

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), RAN#106, December 2024.

TS 22.369, “Service requirements for Ambient power-enabled IoT”, 3GPP, June 2024.

R1-2508511, “Discussion on Rel-20 A-IoT evaluation assumptions and results”, RAN1#123, November, 2025.