3GPP TSG RAN WG1 #123	R1-2508543

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	10.3.2.1

Source:	NEC

Title:	Study on R2D signals, channels, waveform and procedures

Document for:	Discussion and Decision

1	Introduction

In RAN#109 meeting, a SID (RP-252964) [1] on Ambient IoT(A-IoT) was updated with RAN1 details below.

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 …… This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

In this paper, we discuss and give our views on R2D signals, channels, waveform and procedures of A-IoT.

2	Discussion

2.1	R2D waveform and modulation

During the RAN1#122bis [2], the following agreements have been reached w.r.t the chip duration determination for both data payload in the PRDCH and the newly introduced L1 R2D control information:

Agreement For the functionality of chip duration determination of the data payload in the PRDCH, study the following options: - Option 1: same as the chip duration of the corresponding L1 R2D control information - Option 2: indicated by the corresponding L1 R2D control information Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration

The first issue needs to be addressed is whether L1 R2D control information and data payload in the PRDCH can have different and separate chip duration length. From our perspective, the answer should be yes since the target reliability requirement for control information may be different and higher than data payload which may cause different chip duration for them. Further, based on the RAN1 outcome until now, it is obvious the coding and repetition scheme for data payload can be dynamically adjusted based on the indication from L1 control information. Therefore, the chip duration should be able to match such changes, e.g., to adjust the different reliability and data rate requirement. In summary, the following proposal can be made:

Proposal 1: The chip duration of the data payload in the PRDCH is indicated by the corresponding L1 R2D control information, i.e., Option 2 should be adopted.

Then, for how to determine the chip duration for L1 R2D control information itself, among the five options, we firstly think Option 2 should be precluded due to more complexity and power consumption will be brought according to the blind detection operation; for option 3, as discussed in section 4, in Rel-20 A-IoT, the R2D reception operation is more like sample-based other than the edge detection in Rel-19, so we also think this option should be removed. The down-selection can be made among Option 1,4,5 by further comparing the pros and cons of each option during WI phase.

Proposal 2: For chip duration determination of the L1 R2D control information, study among the following options (Option 2 and 3 are precluded).

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 4: Use a set of different binary sequences with fixed or variable length to indicate the chip duration.

- Option 5: Use broadcast information (if any) to indicate the chip duration.

2.2 R2D FEC, bit collection and repetitions

FEC scheme for R2D

During the RAN1#122bis [2], the following agreements have been reached w.r.t the necessity and feasibility of FEC for R2D and repetition:

Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, 1/4

Since Rel-20 A-IoT is targeting for a more extended coverage in outdoor scenario, compared to Rel-19, FEC for R2D for Device 2b and for Device C should be studied to achieve this requirement. For the coding schemes, in our view, the TBCC coding scheme with 1/3 code rate specified in Rel-19 for D2R FEC can be a good starting point. Regarding RM code, we think it should be precluded since it is only feasible for the packet with small payload size and the maximum packet size may be 1000bits in A-IoT, then two types of FEC need to be supported but it is too complex for A-IoT device; for the tailed convolutional code, it is not preferred as well due to the fragmentary bits and the essential aspect is the device especially for device C is surely have the capability to perform decoding for TBCC, then introducing such a coding scheme is redundant.

Regarding the code rate, in our view, 1/3 should be baseline since it is the mother code rate for LTE TBCC, 1/2 can also be supported because it can be based on a simple puncture operation. For 1/4, because it may introduce additional burden on the memory and further complexity on decoding, we propose to further support it only if the bottleneck still has been identified for the R2D coverage in case of 1/3 code rate and maximum number of repetitions are applied.

Furthermore, according to the specific deployment as we discussed during Rel-19 for D2R and the design for device 2b/C for outdoor scenario should be reused for indoor scenario without so strength requirement on coverage, R2D transmission with no FEC should also be studied for R2D during Rel-20.

Proposal 3: For Device 2b and Device C, TBCC with {1/2, 1/3} code rate can be the starting point for R2D FEC.

- R2D transmission without FEC can also be considered;

- Further study the necessity to introduce other code rate (i.e., 1/4).

Repetition scheme for R2D

To enhance the device’s ability to support larger coverage, repetition is another efficient way other than FEC. During the last meeting, three types of repetition schemes have been agreed to be studied for R2D. From our point of view, block-level repetition should be prioritized for the same reason, regarding why it is the only repetition scheme supported by Rel-19. In more details, Bit-level repetitions and Chip-level repetitions were excluded from the discussion in Rel-19 D2R because they could not achieve good time diversity and were somehow equivalent to using a smaller M value. Therefore, RAN1 does not need to make a duplicated study on these two types of repetition schemes. Regarding the candidate number of block-level repetitions, {1, 2} can be a starting point and RAN1 can study larger number, e.g., 4, to meet the extended coverage requirements.

Proposal 4: For Device 2b and Device C, Block-level repetition with candidate repetition number {1,2} can be the starting point for R2D repetition.

- De-prioritize the study of Bit-level repetition and Chip-level repetition during Rel-20;

- RAN1 further study the larger number, e.g., 4 for R2D Block-level repetition.

There is another agreement related to whether to swap the order of repetition and FEC based on the current TS38.291 where repetition is done first then FEC:

Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

If the Rel-19 order is reused, the device is required to do the decoding and the reverse operation of bit collection and interleave, based on the larger number of bits after repetition, which will result in greater complexity for the device and an increased burden on the memory. So, we prefer for R2D, the order should be channel coding (including interleaver if supported) first and then block-level repetition (if supported).

Proposal 5: For R2D, FEC (including bit collection and/or interleave) should be done before the block-level repetition, in which the order is swapped compared to Rel-19.

Bit collection and interleave scheme for R2D

Agreement Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied

By LTE bit collection, the coded bits of the same information bits is evenly distributed among all symbols. In other words, bit collection can achieve the benefit of time diversity, and sub-block interleaver may not offer additional benefit. Furthermore, a device type may have limited memory size, and the decoding process may need device to record lots of received signals. Hence, if memory size limitation exists by evaluation, Alt 3 could be a direction for study.

Proposal 6: For interleaving for PRDCH with FEC, both Alt 2 and Alt 3 can be further evaluated, and Alt 3 can be adopted if memory size limitation exists in device(s).

FEC, interleaving and repetition scheme for L1 R2D control information

For R2D, the channel coding (including potential bit collection and/or interleave) and repetition for the data payload of PRDCH should be determined by the device before it perform decoding. Thus, L1 R2D control information with a separate CRC is introduced for this motivation in last meeting.

Regarding the coding and repetition scheme for L1 R2D control information, we did not find it is necessary to adopt any other scheme other than TBCC and Block-level repetition that would result in different from data payload of PRDCH. However, we think that due to different requirements, the exact code rate and/or the repetition number L1 R2D control information can be different from data payload of PRDCH. Furthermore, since L1 R2D control information may be used for indicating parameters for data payload of PRDCH, the L1 R2D control need a separate interleaving.

Proposal 7: TBCC and Block-level repetition are applied to L1 R2D control information.

- The exact code rate and/or repetition number of L1 R2D control information can be different from the data payload of PRDCH.

- A separate interleaving is needed for L1 R2D control.

Moreover, how to determine the channel coding (including potential bit collection and/or interleave) and repetition for L1 R2D control information itself also needs further study in RAN1. From our perspective, the mechanism on how to determine the chip duration of L1 R2D control information can be a good reference, which can refer to the Proposal 2 in section 2.1. To achieve a unified design for different R2D aspects, for L1 R2D control information, we propose to adopt a same option for both coding and repetition scheme determination and chip duration determination (among the options after the proposed down-selection in Proposal 2).

Figure 1 Illustration for the L1 R2D control information channel coding and repetition determination

Proposal 8: For channel coding (including potential bit collection and/or interleave) and repetition scheme determination of the L1 R2D control information, make down-selection among the following options.

- Option 1: Fixed and pre-defined scheme.

- Option 2: Use a set of different binary sequences with fixed or variable length for indication.

- Option 3: Use broadcast information (if any) or very early-stage transmission for indication.

Furthermore, if repetition is applied to both L1 R2D control information and the data payload of the corresponding PRDCH, the transmission order of one R2D is to complete the transmission of all the repetition of control information first, i.e., {L1 control Rep#0, L1 control Rep#1…L1 control Rep#(N1-1), Data payload Rep#0, Data payload Rep#1…Data payload Rep#N2-1}, assuming N1 and N2 are the repetition number of L1 R2D control information and data payload of the corresponding PRDCH, respectively. The purpose is to ensure that the L1 R2D control information can be correctly decoded before the data payload decoding based on the control information indication.

Figure 2 Illustration of R2D transmission with the repetition number for both L1 R2D control and Data payload equal to 2

Proposal 9: If repetition is applied to both L1 R2D control information and the data payload of the corresponding PRDCH, the transmission order of one R2D is {L1 control Rep#0, L1 control Rep#1…L1 control Rep#(N1-1), Data payload Rep#0, Data payload Rep#1…Data payload Rep#(N2-1)}.

- N1 and N2 are the repetition number of L1 R2D control information and data payload of the corresponding PRDCH, respectively.

2.3 R2D signals

2.3.1 R2D SIP, CAP and midamble

During the RAN1#122bis [2] meeting, the following agreements have been reached related to the R2D SIP, CAP and midamble:

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP: - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence). - Option 2: Reuse the Rel-19 SIP. Agreement For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options: - Option 1: Reuse CAP and/or Manchester encoding - Option 2: sequence-based SIP with or without R2D midamble(s) - Option 3: using the CFO calibration signal Agreement For the study of R2D binary sequence-based SIP, study the following aspects: - Sequence type - Option 1: m-sequence - Option 2: Golay sequence - Length of sequence - Number of sequence(s) - FFS: whether to apply Manchester coding to the sequence - Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

First, the most essential issue in this section is whether Rel-19 SIP can be reused in Rel-20. From our perspective, it may be no longer feasible since Rel-20 A-IoT is target for outdoor scenario different from Rel-19, which make it is hard for the device still to perform edge detection for decoding. Therefore, a binary sequence-based SIP (e.g., M-sequence or Golay sequence) should be preferred to make the device perform, e.g., sampling-based detection other than Rel-19.

Regarding the design of the sequence, in our view, the Rel-19 design for D2R can be a starting point where m sequence with the length of 7 or 31 is adopted. Regarding the number of sequences, we think only single sequence with a single length should be supported due to the device may not have the prior information on when the R2D transmission will occur, and it should perform long-term detection based on the sequence of SIP to identify the starting of R2D transmission. If more than one sequences are defined, more complexity and power consumption on the device side may appear. Moreover, the R2D transmission is usually target for multiple devices as a common transmission, higher overhead of SIP is expected, other than 2-symbol SIP adopted in Rel-19.

Proposal 10: For Rel-20 R2D SIP design, the Rel-19 SIP based on edge detection is NOT pursued.

Proposal 11: For Rel-20 R2D SIP design, a single binary sequence with single length should be defined.

Regarding whether CAP is still required for Rel-20 A-IoT, as aforementioned in section 2.1, the Rel-19 CAP cannot be reused for chip duration determination and CAP is only needed in one of the three options. Therefore, from our perspective, CAP is required to be designed only when RAN1 has agreed to use different binary sequences to indicate the R2D chip duration and/or coding and repetition schemes for L1 R2D control information; otherwise, it is unnecessary to define CAP for Rel-20 R2D. On this basis, the potential CAP in Rel-20 can only be sequence-based with multiple candidate sequence or length unlike SIP, due to it should be detected after the successful detection of SIP, i.e., long-term detection on CAP is not needed, then we think the complexity and power consumption seems to be acceptable to the device.

Figure 3 Illustration of R2D transmission structure (CAP can be omitted according to the discussion)

Proposal 12: For Rel-20 R2D design, binary sequence-based CAP should be defined only when RAN1 has agreed to use different binary sequences to indicate the R2D chip duration and/or coding and repetition schemes for L1 R2D control information, i.e., Option 4 in P2 and/or Option 2 in P8 are finally agreed.

- Otherwise, it is unnecessary to define CAP for Rel-20 R2D.

As mentioned above, in Rel-20, device cannot keep the timing and SFO tracking based on the raising/falling edge due to the detection operation based on edge detection is NOT feasible for the outdoor scenario any more. Therefore, midamble can be considered for R2D to make the device can keep timing and SFO tracking, especially for the long R2D transmission. Then further issues would be whether midamble have same or different sequence or length as preamble, how to insert the midamble considering there are L1 R2D control information and data payload, etc.

Proposal 13: For Rel-20 R2D design, binary sequence-based midamble can be considered for timing and SFO tracking.

- FFS: Whether midamble have same or different sequence or length as preamble;

- FFS: How to insert the midamble considering there are L1 R2D control information and corresponding data payload.

2.3.2 R2D sync signal

To support the DO-A traffic type and as mentioned in [3], a new R2D sync signal was introduced at least for initial time and frequency synchronization and the potential initial cell search.

Agreement Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking. - Alt1: periodic synchronization signal - Alt2: aperiodic synchronization signal

The first aspect is the sequence design of the newly introduced sync signal, since it is mainly used by the device to perform correlation operation, the sync signal surely should be a binary-sequence based signal. On the other hand, to the same end as the design of SIP in section 2.3.1, it is better to have only single candidate sequence with single length to avoid the complexity and power consumption brought by long-term monitoring based on multiple sequences. A further issue is whether the sequence of the newly introduced R2D sync signal and R2D SIP is same or not, considering the device may not have the prior information about whether the next upcoming R2D transmission is the R2D sync signal or PRDCH (may including L1 R2D control information) with a SIP in the beginning.

Proposal 14: The newly introduced R2D sync signal should be binary sequence-based

- Only single candidate sequence with single length should be defined for the signal;

- RAN1 may need further discuss whether the sequence of the newly introduced R2D sync signal and R2D SIP is same or not.

Regarding whether the sync signal is periodic or aperiodic, from reader perspective, we think this totally depends on the reader’s implementation. For example, reader can strictly follow a periodicity to perform the sync signal transmission to meet the requirements of e.g., periodic type DO-A traffic. The reader can also, based on certain prior information (e.g., the wake-up time duration of most devices), concentrate the transmission of sync signal over the corresponding period. The reader can also transmit the sync signal in an on-demand manner, in response to a specific event-trigger, such as a wildfire.

Figure 4 Examples for R2D sync signal transmission manner

On the other hand, no matter how the reader transmits the sync signal, the essential point is from device's perspective, it doesn't need to precisely know the transmission time of the sync signal from reader, because this would require the device to continuously and accurately perform time counting during long-term time period, which would be very difficult implement for the device due to the additional power consumption.

Proposal 15: The R2D sync signal transmission can be periodic or aperiodic, which is based on reader’s implementation.

- It is difficult and unnecessary for the device to know the exact transmission time of the R2D sync signal transmission.

2.3.3 CFO calibration signal

For CFO calibration signal, there is an agreement on the signal design and the relation to other R2D transmission.

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Since the CFO calibration is mainly applied to the LO in RF level, which cannot be based on the baseband clock, for the signal design, Option c is not feasible to this end; regarding the necessity to introduce unmodulated multiple sinusoid single tones, we do not find strong motivation to do this because more complexity and difficulty may be brought to the device for detecting and calibrating based on the signal. On the above basis, an unmodulated sinusoid single tone signal is more feasible for CFO calibration purpose. Furthermore, to mitigate the ambiguity and effectively eliminate the CFO as much as possible, the exact frequency of CFO calibration signal may need to be explicitly indicated from reader and then determined by the device.

Proposal 16: Only option a should be considered for CFO calibration signal design, i.e., the signal is unmodulated sinusoid single tone.

- The exact frequency of CFO calibration signal may need to be explicitly indicated from reader and determined by device.

Regarding the time location(s) of CFO calibration signal and the relation to other R2D transmission, in our view, the key issue is RAN1 should firstly make a conclusion to decide whether the CFO calibration signal is only used for D2R transmission or for both D2R transmission and R2D reception. After having a clear answer to this question, RAN1 can make more easier further down-selection among the five options because some of them may be naturally precluded. Thus, we would like to provide the following proposal:

Proposal 17: For the time location(s) of CFO calibration signal RAN1 should firstly study and make a conclusion to decide whether the CFO calibration signal is:

- Alt.1: Only used for D2R transmission;

- Alt.2: Used for both D2R transmission and R2D reception;

- The potential down-selection among the five options should be done after and based on the study outcome.

Another dimension of discussion about time location of CFO calibration signal is whether CFO calibration signal is transmitted separately or jointly with R2D transmission. Different transmission schemes of CFO calibration signal have different options of time location(s) for CFO calibration signal.

With the scheme of CFO calibration signal transmitted jointly with R2D transmission, at least option 1 and option 2 of the agreement are of most related. Going to specific relation for the two options, when CFO calibration signal is located before L1 R2D control info and data payload of PRDCH, both L1 R2D control info and data payload of PRDCH will be decoded after CFO calibration thus with more accurate performance. In this case, CFO calibration is more on facilitating R2D reception. While when CFO calibration signal is located after L1 R2D control info and data payload of PRDCH, then CFO calibration would be more of facilitating following D2R transmission(s). With both cases envisioned in actual transmission instances, location of CFO calibration signal before L1 R2D control information and after data payload of PRDCH are preferable for option 1 and option 2 respectively.

Proposal 18: For option 1, location before L1 R2D control information is preferred.

Proposal 19: For option 2, location after data payload of PRDCH is preferred.

With the scheme of CFO calibration signal transmitted separately, option 3, option 4 and option 5 would be more related. As it is still under discussion on synchronization signal and broadcast information, option 4 and option 5 can be further discussed after support of synchronization signal and broadcast information achieved. For option 3 where CFO calibration signal is related with PDRCH, relatively larger complexity on overhead of SIP as well as indication and detection of separate CFO calibration signal is envisioned.

Observation 1: For option 3, relatively larger complexity on overhead of SIP as well as indication and detection of CFO calibration signal is envisioned.

In summary, to satisfy different and various scenarios/conditions for actual transmission, multiple options of time location(s) for CFO calibration signal may be needed. If this is the case, reader indicates flexibly which time location to be applied also seems necessary.

Proposal 20: Option index indication is necessary if multiple options of time location(s) are supported.

2.4 L1 R2D control information

Different from Rel-19 A-IoT where channel coding and repetition can be applied only to D2R, for R2D, these schemes are also introduced in Rel-20, as mentioned in section 2.2. In order to indicate such parameters before the data payload of PRDCH and make sure the device have prior information on how to decode the payload, L1 R2D control information is agreed to be studied in the RAN1#122bis meeting, the related agreements are shown in below:

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted Agreement For the functionality of indicating the end of the PRDCH, study the following option: - Option 1: Use the L1 R2D control information to indicate the TBS. Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel.

For how to determine the chip duration and coding/repetition scheme, duplicated discussion will not be put in this section because these two aspects have already been captured in section 2.1 and 2.2, respectively.

Regarding the necessary information or field to be included in the L1 R2D control information, according to the above agreements, R2D TBS definitely should be included for indicating the end of PRDCH; more, as aforementioned, according to section 2.1 and 2.2, Bit duration for data payload of PRDCH, Channel Coding indicator (may also including Interleave/Bit Collection indicator) and Block Repetition number field should also be included; Another field may need to be involved is Interval Bits for R2D midamble to indicated how to insert the R2D midamble for PRDCH, if it is supported as in section 2.3.1, P13.

Proposal 21: At least the following fields should be included in L1 R2D control information:

- R2D TBS;

- Bit duration for data payload of PRDCH;

- Channel Coding indicator (may also including Interleave/Bit Collection indicator);

- Block Repetition number;

- Interval Bits for R2D midamble (if supported);

- FFS: other fields or necessary information to be included.

Considering the time location and frequency resource for L1 R2D control information, at first, we think there is no need to reserve some gap b/w L1 R2D control information and the corresponding data payload of PRDCH, because we believe for device 2b and C during the decoding of L1 R2D control information the device can parallelly store the data payload in buffer, such a simple parallel operation can be achieved by the device and only for a short period the device needs to do these two operations simultaneously; Secondly, we do not think the L1 R2D control information and the data payload of PRDCH can have different frequency resource which is more like a cross-carrier scheduling as in NR, because this may cause additional retuning time for device in-between the reception L1 R2D control information and the data payload of PRDCH, on the other hand, additional overhead in L1 R2D control information is also introduced. Based on the above analysis, we propose:

Proposal 22: For the L1 R2D control information preceding and scheduling the data payload of PRDCH:

- L1 R2D control information and data payload of PRDCH are contiguous in time, i.e., Option 1 should be adopted;

- The frequency resource of L1 R2D control information is always same to the data payload of PRDCH.

Another aspect is whether a specific control channel should be defined for L1 R2D control information. We think this is NOT necessary because the coding/repetition scheme for the control information is like the data payload, and aslo no specific reference RS would be dedicatedly introduced for the control information. Therefore, no additional channel should be defined. As we have proposed contiguous L1 R2D control and data, they can be included a single PRDCH transmission, obviously.

Proposal 23: L1 R2D control information and the corresponding data payload are included in a same and single PRDCH.

- Do NOT pursue another R2D channel dedicated for L1 R2D control information.

- 2.5 R2D FDM

During the RAN1#122bis meeting, the following agreement has been reached for R2D FDM design.

Agreement Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects: - complexity of device to support FDM - how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Thanks to the IF/ZIF receiver architecture what we proposed to be prioritized in Rel-20, the active device is capable of distinguishing and correctly obtaining each of the FDMed transmission, which can enable the multiple R2D transmission to be carried out in a parallel manner. So, from our perspective, support R2D FDM does not introduce obvious complexity to the device for detection and decoding.

Observation 2: Support R2D FDM does not introduce obvious complexity to the device due to the proper IF/ZIF receiver architecture for device 2b and C in Rel-20.

Then a further issue is which message(s) to support R2D FDM. In Rel-19, during an inventory procedure, the A-IoT paging message, Access trigger message, Msg2, Msg4 (NACK) and the potential subsequent R2D transmission can only be transmitted one by one in time domain, i.e., in TDM manner and no parallel operation can be achieved due to only TDMed R2D transmission can be supported by the RF-ED receiver.

Figure 5 Illustration of the inventory/random access procedure in Rel-19

In Rel-20, the coverage target (and therefore also the capacity target) for the outdoor scenario shall be larger than Rel-19 in which indoor scenario is focused, and then more efficient inventory/random access procedure are expected to keep the completion time acceptable. Since now multiple R2D transmission to be carried out in can be in a FDMed and parallel manner. As shown in below figure, Access triggers message and Msg2 from different access occasions can be transmitted simultaneously to improve the inventory efficiency and reduce completion time. From our perspective, for the same reader case, R2D FDM can be realized by the following R2D transmissions from a same reader: Access Trigger message, Random ID Response message (Msg2 in CBRA), NACK Feedback message.Note that any combination of the above R2D message to perform FDM is not precluded; However, for different reader case, it is firstly needed to be clarified that whether the FDMed R2D sync signal from multiple readers should be supported, considering this may related to other WGs discussion, as mentioned in [3].

Proposal 24: FDM is supported at least for the following message(s).

- From a same reader,

  - Access Trigger message, Random ID Response message (Msg2 in CBRA), NACK Feedback message, any combination of them to perform FDM is not precluded;

- From different readers,

  - FFS: whether the R2D sync signals from different readers can be FDMed.

Regarding the transmission BW, unlike the minimum number of PRBs for PRDCH defined in Rel-19, the exact number of PRBs should be defined for PRDCH in Rel-20 since the device needs to filter out the R2D transmission on the narrow band (e.g., 1 or 2 PRBs). Furthermore, since there may still be residual CFO even after the calibration by the device, there should be some guard-band in-between each of the two adjacent R2D transmission BWs. Considering how to define the channel BW for R2D, we think it should belong RAN4’s discussion scope, not RAN1.

Proposal 25: The exact transmission BW size of R2D transmission should be studied in RAN1 to support R2D FDM.

- The transmission BW should be X PRBs (X is an integer number, e.g., X=1, 2);

- There should be some guard-band in-between each of the two adjacent R2D transmission BWs;

- How to define the channel bandwidth should be discussed in RAN4.

Regarding how a device determines the frequency resource for receiving FDMed R2D transmissions, we think the following options can be considered:

- Option 1: A set of candidates R2D frequency resources/channels are configured by a very first R2D transmission (e.g., broadcast information associated with periodic synchronization signal, A-IoT paging message, etc), and then the exact frequency resource/channel for a R2D transmission can be indicated by another previous R2D transmission.

- Option 2: A set of candidates R2D frequency resources/channels are configured by a very first R2D transmission (e.g., broadcast information associated with periodic synchronization signal, A-IoT paging message, etc), and then the exact frequency resource/channel for a R2D transmission is associated with the previous corresponding D2R transmission T/F resource.

- Option 3: A set of candidates R2D frequency resources/channels are configured by a very first R2D transmission (e.g., broadcast information associated with periodic synchronization signal, A-IoT paging message, etc), and then the exact frequency resource/channel for a R2D transmission can be associated to the specific coverage level which is determined by the device’s RSRP measurement to the R2D transmission (e.g., R2D sync signal in section 2.3.2).

- Note that the combination(s) of above Options are not precluded.

Figure 6 Illustration of the three options of resource determination for R2D FDM

Proposal 26: On how a device determines the frequency resource for receiving one of the FDMed R2D transmissions, there are three options can be considered:

- Option 1: The exact frequency resource/channel for a R2D transmission is indicated by another previous R2D transmission;

- Option 2: The exact frequency resource/channel for a R2D transmission is associated with the previous corresponding D2R transmission T/F resource;

- Option 3: The exact frequency resource/channel for a R2D transmission is associated with a specific coverage level which is determined by the device’s RSRP measurement;

- The exact frequency resource/channel belongs to a set of candidates R2D frequency resources/channels configured by a very first R2D transmission;

- The combination(s) of above options are not precluded.

3	Conclusion

In this contribution, we give our views on air interface for R2D aspects. We observe and propose that:

Observation 1: For option 3, relatively larger complexity on overhead of SIP as well as indication and detection of CFO calibration signal is envisioned.

Observation 2: Support R2D FDM does not introduce obvious complexity to the device due to the proper IF/ZIF receiver architecture for device 2b and C in Rel-20.

Proposal 1: The chip duration of the data payload in the PRDCH is indicated by the corresponding L1 R2D control information, i.e., Option 2 should be adopted.

Proposal 2: For chip duration determination of the L1 R2D control information, study among the following options (Option 2 and 3 are precluded).

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 4: Use a set of different binary sequences with fixed or variable length to indicate the chip duration.

- Option 5: Use broadcast information (if any) to indicate the chip duration.

Proposal 3: For Device 2b and Device C, TBCC with {1/2, 1/3} code rate can be the starting point for R2D FEC.

- R2D transmission without FEC can also be considered;

- Further study the necessity to introduce other code rate (i.e., 1/4).

Proposal 4: For Device 2b and Device C, Block-level repetition with candidate repetition number {1,2} can be the starting point for R2D repetition.

- De-prioritize the study of Bit-level repetition and Chip-level repetition during Rel-20;

- RAN1 further study the larger number, e.g., 4 for R2D Block-level repetition.

Proposal 5: For R2D, FEC (including bit collection and/or interleave) should be done before the block-level repetition, in which the order is swapped compared to Rel-19.

Proposal 6: For interleaving for PRDCH with FEC, both Alt 2 and Alt 3 can be further evaluated, and Alt 3 can be adopted if memory size limitation exists in device(s).

Proposal 7: TBCC and Block-level repetition are applied to L1 R2D control information.

- The exact code rate and/or repetition number of L1 R2D control information can be different from the data payload of PRDCH.

- A separate interleaving is needed for L1 R2D control.

Proposal 8: For channel coding (including potential bit collection and/or interleave) and repetition scheme determination of the L1 R2D control information, make down-selection among the following options.

- Option 1: Fixed and pre-defined scheme.

- Option 2: Use a set of different binary sequences with fixed or variable length for indication.

- Option 3: Use broadcast information (if any) or very early-stage transmission for indication.

Proposal 9: If repetition is applied to both L1 R2D control information and the data payload of the corresponding PRDCH, the transmission order of one R2D is {L1 control Rep#0, L1 control Rep#1…L1 control Rep#(N1-1), Data payload Rep#0, Data payload Rep#1…Data payload Rep#(N2-1)}.

- N1 and N2 are the repetition number of L1 R2D control information and data payload of the corresponding PRDCH, respectively.

Proposal 10: For Rel-20 R2D SIP design, the Rel-19 SIP based on edge detection is NOT pursued.

Proposal 11: For Rel-20 R2D SIP design, a single binary sequence with single length should be defined.

Proposal 12: For Rel-20 R2D design, binary sequence-based CAP should be defined only when RAN1 has agreed to use different binary sequences to indicate the R2D chip duration and/or coding and repetition schemes for L1 R2D control information, i.e., Option 4 in P2 and/or Option 2 in P8 are finally agreed.

- Otherwise, it is unnecessary to define CAP for Rel-20 R2D.

Proposal 13: For Rel-20 R2D design, binary sequence-based midamble can be considered for timing and SFO tracking.

- FFS: Whether midamble have same or different sequence or length as preamble;

- FFS: How to insert the midamble considering there are L1 R2D control information and corresponding data payload.

Proposal 14: The newly introduced R2D sync signal should be binary sequence-based

- Only single candidate sequence with single length should be defined for the signal;

- RAN1 may need further discuss whether the sequence of the newly introduced R2D sync signal and R2D SIP is same or not.

Proposal 15: The R2D sync signal transmission can be periodic or aperiodic, which is based on reader’s implementation.

- It is difficult and unnecessary for the device to know the exact transmission time of the R2D sync signal transmission.

Proposal 16: Only option a should be considered for CFO calibration signal design, i.e., the signal is unmodulated sinusoid single tone.

- The exact frequency of CFO calibration signal may need to be explicitly indicated from reader and determined by device.

Proposal 17: For the time location(s) of CFO calibration signal RAN1 should firstly study and make a conclusion to decide whether the CFO calibration signal is:

- Alt.1: Only used for D2R transmission;

- Alt.2: Used for both D2R transmission and R2D reception;

- The potential down-selection among the five options should be done after and based on the study outcome.

Proposal 18: For option 1, location before L1 R2D control information is preferred.

Proposal 19: For option 2, location after data payload of PRDCH is preferred.

Proposal 20: Option index indication is necessary if multiple options of time location(s) are supported.

Proposal 21: At least the following fields should be included in L1 R2D control information:

- R2D TBS;

- Bit duration for data payload of PRDCH;

- Channel Coding indicator (may also including Interleave/Bit Collection indicator);

- Block Repetition number;

- Interval Bits for R2D midamble (if supported);

- FFS: other fields or necessary information to be included.

Proposal 22: For the L1 R2D control information preceding and scheduling the data payload of PRDCH:

- L1 R2D control information and data payload of PRDCH are contiguous in time, i.e., Option 1 should be adopted;

- The frequency resource of L1 R2D control information is always same to the data payload of PRDCH.

Proposal 23: L1 R2D control information and the corresponding data payload are included in a same and single PRDCH.

- Do NOT pursue another R2D channel dedicated for L1 R2D control information.

Proposal 24: FDM is supported at least for the following message(s).

- From a same reader,

  - Access Trigger message, Random ID Response message (Msg2 in CBRA), NACK Feedback message, any combination of them to perform FDM is not precluded;

- From different readers,

  - FFS: whether the R2D sync signals from different readers can be FDMed.

Proposal 25: The exact transmission BW size of R2D transmission should be studied in RAN1 to support R2D FDM.

- The transmission BW should be X PRBs (X is an integer number, e.g., X=1, 2);

- There should be some guard-band in-between each of the two adjacent R2D transmission BWs;

- How to define the channel bandwidth should be discussed in RAN4.

Proposal 26: On how a device determines the frequency resource for receiving one of the FDMed R2D transmissions, there are three options can be considered:

- Option 1: The exact frequency resource/channel for a R2D transmission is indicated by another previous R2D transmission;

- Option 2: The exact frequency resource/channel for a R2D transmission is associated with the previous corresponding D2R transmission T/F resource;

- Option 3: The exact frequency resource/channel for a R2D transmission is associated with a specific coverage level which is determined by the device’s RSRP measurement;

- The exact frequency resource/channel belongs to a set of candidates R2D frequency resources/channels configured by a very first R2D transmission;

- The combination(s) of above options are not precluded.

References

[1]	RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, 3GPP TSG RAN Meeting #109, September 15-18, 2025.

[2]	Draft_Minutes_report_RAN1#122bis_v104

[3] 	R1-2508545, Study on other procedures for device 2b and C, NEC, RAN1#123, Dallas, USA, Nov 17th – 21st, 2025.