3GPP TSG RAN WG1 #123			R1-2509268

Dallas, USA, Nov 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the inter-vendor training collaborations for two-sided AI/ML models

Agenda Item:	10.1.2

Document for: 	Discussion and Decision

- Introduction

At the RAN Plenary #108 meeting, the New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements [1] was approved. The working scope for the inter-vendor training collaboration for two-sided models is as follows,

Inter-vendor training collaboration for two-sided AI/ML models

- Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback

  - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback

- Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback

Interoperability and RRM requirement [RAN4]:

- To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability

  - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration

- Necessary RRM requirement for specified LCM procedures.

  - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

This contribution discusses the necessary specification support for the inter-vendor training collaborations.

- On the checkpoint for Option 1 and Option 3a-1

As agreed in WID[1], a checkpoint in RAN #110 has been set to check the progress of RAN4 regarding inter-vendor collaboration Option 1 and Option 3a-1.  At the RAN4 #116bis meeting, the following was agreed upon related to the RAN4 responses to the RAN checkpoint [4].

Definitions: Fully specified the model is defined as a specified structure with parameter in [TS]. Reference model means UE/NW is not mandated to implement it. Test decoder means TE vendor needs to implement it for testing purpose Direct C in RAN1: fully specified reference encoder and decoder Option 3 in RAN4: fully specified test decoder and the reference encoder is under discussion (it is agreed to have a fully specified reference encoder) Direction A 3a-1 in RAN1: specify the reference encoder structure. Presumably, NW will send the parameters to the UE. Relation between RAN1 and RAN4: • The test decoder of option 3 is the same as the reference decoder of direction C - Test decoder of option 3 will be specified by RAN4 - If RAN1 develops the reference decoder for direction C which is different from RAN4’s, further discussion across RAN4 and RAN1 is needed. • The reference encoder is the same for both option 3 and direction C • The option 3 reference encoder structure is the same as the encoder structure for direction A option 3a-1. - Reference encoder of option 3 will be specified in RAN4. How to respond to the RAN checkpoint • Option 3 is feasible for RAN4 to complete. RAN4 will take into account the conclusions in RAN1 on scalability in order to create a scalable model. - RAN4 agrees that the test decoder of option 3 is the same as the reference decoder of direction C and the reference encoder of option 3 is the same as the reference encoder of direction C. If so, RAN4 believes that direction C should be feasible to complete too. • Direction A 3a-1 is feasible for RAN4 to complete from reference encoder perspective. • FFS on detailed steps to develop the model • RAN4 will send LS on RAN4’s agreement regarding the RAN checkpoint.

Based on the agreements, RAN4 has confirmed the feasibility of both Option 1 and Option 3a-1 from the RAN4 perspective. According to the work plan in [4], RAN4 will define the test decoder, which is the same as the reference decoder, and the reference encoder. The corresponding discussions for Option 1 and Option 3a-1 can be started from RAN1 #124, after confirming the RAN4 progress in RAN #110.

Proposal 1

- RAN1 starts the discussions for inter-vendor collaboration Option 1 and Option 3a-1 from RAN1 #124 after checking the RAN4 confirmation in RAN #110.

- Specification support for dataset exchange

Performance target

At the RAN1 #122bis meeting, the agreements on the performance target for dataset exchange are the following [3],

Agreement: - For Option 4-1 under Direction A in AI/ML based CSI compression, both average SGCS and average NMSE as performance target in the exchanged dataset are supported. Both average SGCS and average NMSE are provided. - NW side can optionally send NMSE and/or SGCS values at X-percentiles.

For the agreed two performance target types, the SGCS is a type of end-to-end performance target. The UE that supports it should also train the CSI reconstruction part to test the end-to-end performance. However, the SGCS exchanged to the UE is obtained based on a model used in the NW side, which is unknown to the UE. The UE has no information about the conditions under which the exchanged SGCS is obtained. The SGCS cannot assist UE with the model training.

Meanwhile, the UE can use an arbitrary decoder to test whether the end-to-end performance target is satisfied, which differs from the one used by NW and is unknown to NW. After the UE verifies the performance target with a UE-decided decoder and reports that the model is applicable, NW cannot trust the reporting at all. Although performance monitoring can identify performance issues with the UE-side model, it occurs after the model has been activated for a while, and damage to the NW performance has been introduced.

Observation 1

- If the SGCS type of performance target is exchanged without any reference to the decoder,

  - UE has no idea of the conditions under which the SGCS is calculated. Therefore, it does not assist with the model training.

  - NW has no idea of the conditions under which the UE verifies the model performance. Therefore, NW cannot trust the UE applicability report.

To solve this issue, a reference decoder for the exchanged SGCS is necessary. Based on the latest RAN4 progress [4], the test decoder to be specified by RAN4 is a proper reference decoder that does not disclose any proprietary information of NW or UE vendors. Therefore, we have the following proposal,

Proposal 2

- UE/UE-side assumes that the test decoder specified by RAN4 is used for calculating the SGCS type of performance target in the exchanged dataset.

  - Note: Refer to R4-2514614 for the definition of the test decoder and its relation to the reference decoder.

- Specification support for the pairing ID

At the RAN1 #122 meeting, the following was agreed upon about the pairing ID [2].

Agreement: For Option 4-1 under Direction A in AI/ML based CSI compression, support exchange of single pairing ID along with dataset exchange. - One pairing ID is assigned to one dataset. - The dataset can have different number of Tx port, number of subbands, and CSI payload size configurations (including different quantization codebooks if needed). - From RAN1 perspective, the uniqueness of the pairing ID needs further studied. - FFS: the impact on pairing ID(s) when additional data samples are added to an exchanged dataset, if supported.

One of the remaining issues is the assignment of the pairing ID. This issue applies not only to Option 4-1 but also to all three inter-vendor options. The assignment of the pairing should be designed taking all three options into account, and a unified framework for model pairing with Direction A (Option 3a-1 and 4-1) and Direction C is preferred.

For Direction C (Option 1), because the reference model for Direction C will be specified, a unique ID can be allocated for each specified model. These IDs are naturally globally unique and are assigned in the specifications. To accommodate these IDs into a unified pairing ID framework, a range of pairing ID space can be reserved for Direction C, as demonstrated in the example shown in Table 1.

Table  1: An example of pairing ID allocation for Directions A and C.

| ID | Assignment |
| --- | --- |
| 0 ~ X | Used/Reserved for the specified reference model. |
| X+1 ~ Y | Notified to UE during dataset or model parameter exchange. |

For Direction A, the pairing ID is assigned by NW and notified to the UE. The IDs other than the ones reserved for the reference model can be used by Direction A. The proposal for this scheme is as follows,

Proposal 3

- Support the following pairing ID assignment for Direction C and Direction A, respectively.

  - For Direction C (Option 1), the ID is specified for each specified reference model.

  - For Direction A, the ID, which is different from the ones reserved for Direction C, is assigned by NW.

    - Note: The details of ID assignment are up to RAN2.

A key issue about the pairing ID is the uniqueness of the ID, i.e., within what scope the UE should treat the two IDs as the same one if their values are the same. Since NW is deployed and operated by MNOs, who master the information about the deployment environments and network device characteristics, MNO can assign the pairing ID and ensure that the UE can use the same model for a pairing ID when the UE is in the same MNO’s network.

During the discussions, proposals have been made that the pairing ID should be globally unique, which would enable model reusing among MNOs. If the pairing ID is unique, it requires the offline engineer among MNOs to coordinate the assignment of pairing IDs, which significantly increases the complexity of inter-vendor collaborations. And the model reusing among MNOs is a conner case since the models are usually optimized according to the deployment of the networks. Therefore, at least the PLMN uniqueness of the pairing should be supported, and it is for further study to determine whether to support global uniqueness.

The management of the pairing ID is also an issue in practical networks. After a pairing ID is notified with model parameters or datasets, the UE/UE-side will develop the model that can be paired with NW. These models will be stored in the UE or UE-side servers. In practical deployment, NW can upgrade its AI/ML implementations based on the evolution of AI/ML technologies or the collection of more data in the deployment scenarios. After that, NW can notify new or existing UEs with a new pairing ID and corresponding model parameters or datasets. After all UEs upgrade their models, the old one and its pairing ID can be abandoned. However, although the old model and ID will never be used again, the UE side will store them since it does not have the information about whether this model will be used again, and the ID is also occupied to prevent future usage. To resolve this issue, a mechanism is necessary to release the pairing ID, along with the corresponding models and datasets stored by the UE/UE-side. For example, a validation duration can be introduced, and RAN1 can further study the corresponding UE behaviors.

From the RAN1 perspective, the uniqueness of the pairing ID and the corresponding management are only working assumptions if the potential number of pairing IDs or the update frequency of the pairing ID is considered during the design. The uniqueness of the pairing ID and the ID assignment/release/expiry is up to RAN2 or other WGs. To facilitate the RAN1 discussions, we propose the following conclusion in RAN1.

Proposal 4: For RAN1 to conclude

- RAN1 assumes the following for the normative work about the pairing ID (e.g., for discussing the assignment frequency of the pairing ID, total number of the pairing ID, etc)

  - The pairing ID is at least per PLMN unique. The PLMN uniqueness or global uniqueness can be notified to the UE in case both are supported.

  - The pairing ID can be released or expire by a mechanism specified for higher layers.

  - Note: The details of pairing ID assignment and release/expiry are up to other WGs.

- Conclusion

In this contribution, we have the following proposals,

Observation 1

- If the SGCS type of performance target is exchanged without any reference to the decoder,

  - UE has no idea of the conditions under which the SGCS is calculated. Therefore, it does not assist with the model training.

  - NW has no idea of the conditions under which the UE verifies the model performance. Therefore, NW cannot trust the UE applicability report.

Proposal 1

- RAN1 starts the discussions for inter-vendor collaboration Option 1 and Option 3a-1 from RAN1 #124 after checking the RAN4 confirmation in RAN #110.

Proposal 2

- UE/UE-side assumes that the test decoder specified by RAN4 is used for calculating the SGCS type of performance target in the exchanged dataset.

  - Note: Refer to R4-2514614 for the definition of the test decoder and its relation to the reference decoder.

Proposal 3

- Support the following pairing ID assignment for Direction C and Direction A, respectively.

  - For Direction C (Option 1), the ID is specified for each specified reference model.

  - For Direction A, the ID, which is different from the ones reserved for Direction C, is assigned by NW.

    - Note: The details of ID assignment are up to RAN2.

Proposal 4: For RAN1 to conclude

- RAN1 assumes the following for the normative work about the pairing ID (e.g., for discussing the assignment frequency of the pairing ID, total number of the pairing ID, etc)

  - The pairing ID is at least per PLMN unique. The PLMN uniqueness or global uniqueness can be notified to the UE in case both are supported.

  - The pairing ID can be released or expire by a mechanism specified for higher layers.

  - Note: The details of pairing ID assignment and release/expiry are up to other WGs.

References

- Moderator (Qualcomm), RP-251870, “New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements,” Jun. 2025.

- RAN1 chair (CMCC), “Draft Report of 3GPP TSG RAN WG1 #122”, Aug. 2025.

- RAN1 chair (CMCC), “Draft Report of 3GPP TSG RAN WG1 #122bis”, Oct. 2025.

- Ericsson, R4-2514614, “WF on R20 NR_AIML_air_Ph2”, Oct. 2025.