3GPP TSG RAN WG1 #123	R1-2509325

Dallas, US, 17th – 21th November 2025

Agenda item:       10.2.2

Source:	IIT Kanpur

Title:	Discussion on enhancing DL CSI acquisition

Document for:     Discussion and Decision

# Introduction

In RAN#108, enhancements on DL CSI acquisition were approved as part of the Rel-20 NR MIMO phase 6 WID [1]. The objectives for enhancing DL CSI acquisition are stated as follows:

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

In this document, we discussed the issues that were kept for further studies in the RAN1#122 meeting.

# Discussion

The following agreements were reached during the RAN1#122 meeting.

Agreement: CSI-RS frequency-domain density ρ = 1/4 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports - K=4 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=2 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 64 CSI-RS ports - K=4 32-port NZP CSI-RS resources in a CSI-RS resource set aggregating 128 CSI-RS ports FFS: K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports Agreement: CSI-RS frequency-domain density ρ = 1/3 and 1/6 can be configured to the K NZP CSI-RS resources at least for the following cases: - K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports FFS: K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports

Based on the discussion and the agreements reached in RAN1#122, in this contribution, we address the following issues.

- K=3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/4.

- K=2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/3 and 1/6.

fig 1: Possible configuration of 16 port resources with desnity 1/4 to aggregate 128 ports

Time

16 port NZP CSI-RS

16 port NZP CSI-RS

16 port NZP CSI-RS

ZP CSI-RS

Three 16-port NZP CSI-RS resources are required in a CSI-RS resource set to aggregate 48 CSI-RS ports. The few possible configurations with density 1/4 are as follows:

Time

16 port NZP CSI-RS

16 port NZP CSI-RS

16 port NZP CSI-RS

ZP CSI-RS

fig 2: Possible configuration of 16 port resources with density 1/4 to aggregate 128 ports.

We observed that all possible configurations of 16-port NZP CSI-RS resources used to aggregate 48 ports with a density of 1/4 result in resource wastage under legacy rate-matching patterns as shown in fig 1 & 2. However, certain configurations lead to less wastage than others. Therefore, if we want to enable K = 3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with a density of 1/4, we cannot enable all possible configurations; instead, we should choose the configuration that minimizes resource wastage.

Proposal 1: Support the configuration that leads to less wastage of resources for K = 3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/4.

Two 24-port NZP CSI-RS resources in a CSI-RS resource set are needed to aggregate 48 CSI-RS ports. The few possible configuration with density 1/3 are as follows:

Time

24 port NZP CSI-RS

24 port NZP CSI-RS

ZP CSI-RS

fig 3: Possible configuration of 24 port resources with density 1/3 to aggregate 128 ports.

Like the above case there is wastage of resources if we enable K=2 24-port NZP CSI-RS resources in a CSI-RS resource set under legacy rate matching patterns as shown in fig 3.

Proposal 2: Support the configuration that leads to less wastage of resources for K = 2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/3.

Proposal 3: Support the configuration that leads to less wastage of resources for K = 2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/6.

To support all possible combination without any restriction we need PRB indication parameter from higher layer like the case of density = 1/2 in legacy. W

Proposal 4: Provide the offset value for each resource from the higher layer.

- Conclusion

In this section we provide the summary of our proposals

Proposal 1: Support the configuration that leads to less wastage of resources for K = 3 16-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/4.

Proposal 2: Support the configuration that leads to less wastage of resources for K = 2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/3.

Proposal 3: Support the configuration that leads to less wastage of resources for K = 2 24-port NZP CSI-RS resources in a CSI-RS resource set aggregating 48 CSI-RS ports with density 1/6.

Proposal 4: Provide the offset value for each resource from the higher layer.

# 4. References

[1] 3GPP RAN#108, New WID: NR MIMO Phase 6 [RP-251856].