3GPP TSG RAN WG1 #124		R1-2600167

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source:	OPPO

Title:	Maintenance for Rel-19 IoT NTN

Agenda Item:	8.7.2

Document for:	Discussion and Decision

Introduction

In this contribution, we continue to discuss the remaining issues on IoT NTN OCC.

IoT_NTN_Ph3

NPUSCH slot mapping for TB transmission

For NPUSCH format 1 in R19 IoT-NTN OCC, slot-level OCC is supported for single-tone 15kHz SCS and symbol-level OCC is supported for 3.75kHz SCS. Furthermore, RAN1 made agreements that the granularity of OCC spreading for data is one slot for 15kHz SCS and one symbol for 3.75kHz SCS respectively as follows:

Agreement in RAN1#119 For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot. - Note: whether RU length is extended or not after applying OCC is a separate discussion. Agreement in RAN1#120bis For the 3.75kHz SCS symbol-based OCC scheme, the granularity of spreading for data is one symbol.

Accordingly, the physical resource mapping procedure captures the above agreement into clause 10.1.3.6 in TS36.211 as follows:

10.1.3.6 Mapping to physical resources in TS36.211 For a UE communicating over NTN in FDD operation, if the higher layer parameter npusch-OCC-Enabled is configured, OCC is indicated as enabled in DCI Format N0 as described in [3], and MrepNPUSCH≥2, - For Δf=15 kHz, after mapping to a slot, the slot shall be repeated NidenticalNPUSCH-1 additional times and pairs of slots shall be multiplied by q(m) before continuing the mapping of z(∙) to the following slot, the process repeats for all the slots in the NPUSCH format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3]. - For Δf=3.75 kHz, after mapping to a data symbol, the data symbol shall be repeated NidenticalNPUSCH-1 additional times and pairs of data symbols shall be multiplied by q(m) while skipping the DMRS symbol and guard period within the slot, before continuing the mapping of z(∙) to the following data symbol, the process repeats for all the data symbols in the NPUSCH Format 1 transmission before OCC is applied, where for NscRU=1, MOCC=2, NidenticalNPUSCH=MOCC, and q(m) equals +1+1 or +1-1 as indicated by the DCI Format N0 as described in TS36.212 [3].

As illustrated in Fig 1, for symbol-level OCC for 3.75kHz SCS, each data symbol will be repeated 1 additional time before mapping the next data symbol. In this case, the portion of NPUSCH associated with a TB mapped to slot#0 before OCC spreading is actually transmitted in NB-IoT UL slot#0-slot#1 after OCC spreading.

Similarly, for slot-level OCC for 15kHz SCS, each slot is repeated 1 additional time before mapping the next slot. Also, the portion of NPUSCH associated with a TB mapped to slot#0 before OCC spreading is actually transmitted in NB-IoT UL slot#0-slot#1 after OCC spreading. That is, the slot index determination for TB transmission after OCC spreading is the same for symbol-level OCC and for slot-level OCC.

Fig 1. Resource mapping for symbol-level OCC and for slot-level OCC

Observation 1: The slot index determination for TB transmission after OCC spreading is the same for symbol-level OCC and for slot-level OCC.

However, we note that the NPUSCH slot mapping procedure for TB transmission in clause 16.5.1.2 in TS36.213 does not capture the above agreement correctly. The related description is copied below for convenience.

16.5.1.2 Modulation order, redundancy version and transport block size determination in TS36.213 The NPUSCH associated with a TB is transmitted in N NB-IoT UL slots associated with the TB, ni , i=0,1,…,N-1. For the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots associated with the TB ni ,, the redundancy version associated with the TB is determined by, , where - if - if the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled is indicated in corresponding DCI - L=2 - otherwise - L=1 - otherwise - . Portion of NPUSCH codeword with associated with a TB as defined in clause 6.3.2 in [4] mapped to slot of allocated resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni i=jB+LbL+l, l=0,1,…,L-1 for and i=jB+2Lb2L+2l+mod(bL,2),l=0,1,...L-1 for

As highlighted (the yellow part) in the above text, only L=2 is introduced in R19 IoT-NTN to support both symbol-level OCC for 3.75kHz SCS and slot-level OCC for 15kHz SCS, and the slot mapping procedure for TB transmission (the cyan part) keeps unchanged compared to the legacy procedure.

Substitute b=0, j=0 and L=2 into the above slot mapping procedure for TB transmission:

- for 3.75kHz SCS, portion of NPUSCH codeword with rvidx(0) associated with a TB mapped to slot 0 of allocated NRU resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni i=2b2+l=0,1, which is consistent with the resource mapping procedure in TS36.211.

- for 15kHz SCS, portion of NPUSCH codeword with rvidx(0) associated with a TB mapped to slot 0 of allocated NRU resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni i=4b4+2l+mod(bL,2)=0,2, which is contradictory with the resource mapping procedure in TS36.211. Thus, when OCC is enabled for 15kHz SCS, some adaptation on the slot mapping procedure in TS36.213 should be considered to align with the resource mapping procedure in TS36.211.

Observation 2: For 15kHz SCS, when OCC is enabled, the slot mapping procedure for TB transmission in TS36.211 is not aligned with the resource mapping procedure in TS36.213.

Proposal 1: Adopt TP#1 for TS36.213 v19.2.0 to correct the slot mapping procedure for TB transmission for slot-level OCC for 15kHz SCS.

- Reason for change The slot mapping procedure for TB transmission for 15kHz SCS in clause 16.5.1.2 in TS36.211 is not aligned with the resource mapping procedure in clause 10.1.3.6 in TS36.213. - Summary of change Adapt the slot mapping procedure for TB transmission for 15kHz SCS in clause 16.5.1.2 in TS36.211 to support OCC spreading in the unit of one slot. - Consequences if not approved The slot mapping procedure for TB transmission for 15kHz SCS in clause 16.5.1.2 in TS36.211 does not support OCC spreading in the unit of one slot. -------------------- start of TP#1 for 36.213 -------------------- 16.5.1.2 Modulation order, redundancy version and transport block size determination *** Unchanged parts are omitted *** The NPUSCH associated with a TB is transmitted in N NB-IoT UL slots associated with the TB, ni , i=0,1,…,N-1. For the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots associated with the TB ni ,, the redundancy version associated with the TB is determined by, , where - if - if the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled is indicated in corresponding DCI - L=2 - otherwise - L=1 - otherwise - . If the UE is configured with higher layer parameter npusch-OCC-Enabled and NRep>1 and OCC enabled is indicated in corresponding DCI, portion of NPUSCH codeword with associated with a TB as defined in clause 6.3.2 in [4] mapped to slot of allocated resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni i=jB+LbL+l, l=0,1,…,L-1 for and for . Otherwise, Portion of NPUSCH codeword with associated with a TB as defined in clause 6.3.2 in [4] mapped to slot of allocated resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni i=jB+LbL+l, l=0,1,…,L-1 for and i=jB+2Lb2L+2l+mod(bL,2),l=0,1,...L-1 for *** Unchanged parts are omitted *** -------------------- end of TP#1 ---------------------------------

Resource reservation

In the last meeting, RAN1 has excessively discussed the interaction between OCC and uplink reserved resources, but no consensus was achieved and further analysis is necessary. According to the reply LS on NR NB-IoT in-band operation [1], RAN2 confirmed that it is feasible to deploy IoT NTN within an NR NTN carrier, so OCC functionality should be supported when uplink reserved resources are configured for NB-IoT co-existence with NR.

Proposal 2: OCC functionality should be supported for the scenario of IoT NTN coexistence with NR NTN.

According to the current specification, when the NPUSCH transmission overlaps with the fully reserved uplink subframe or the reserved uplink symbols, the postponement or puncture on NPUSCH should be performed to avoid the collision. The details are specified in clause 10.1.3.6 of TS36.211 as follows:

TS36.211 10.1.3.6 Mapping to physical resources If higher layer parameter resourceReservationConfigUL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space with the Resource reservation field in the DCI set to 1 including NPUSCH format 1 transmission without a corresponding NPDCCH, or in case of NPUSCH format 2 transmission associated with C-RNTI using UE-specific NPDCCH search space, - In a subframe for Δf=15 kHz or a slot for Δf=3.75 kHz that is overlapping with any fully reserved uplink subframe as defined in clause 16.5 in [4], - for Δf=15 kHz, the NPUSCH transmission is postponed until the next NB-IoT uplink subframe that is not fully reserved. - for Δf=3.75 kHz, the NPUSCH transmission in the slot is postponed until the next slot spanning over two contiguous uplink subframes not overlapping with any uplink subframe that is fully reserved. - In a subframe for Δf=15 kHz or a slot for Δf=3.75 kHz that is not overlapping with any fully reserved uplink subframe, any SC-FDMA symbols overlapping with reserved symbols shall be counted in the NPUSCH mapping but not used for transmission of the NPUSCH.

For Δf=3.75 kHz, RAN1 agreed that the symbol-level OCC scheme is supported for data symbols and the granularity of OCC spreading is an SC-FDMA symbol. Regarding DMRS pattern, RAN1 agreed to support TDM DMRS over 4 slots for UE multiplexing, where the multiplexed UE only transmits DMRS over the first 2 slots or the last 2 slots within the 4 slots. Furthermore, the following agreement was made in RAN1#121 meeting to ensure the alignment of the TDM DMRS structure for OCC multiplexed UEs:

Agreement For the TDM DMRS positions that are activated within the TDM DMRS pattern: For an NPUSCH format 1 with 3.75kHz SCS allocated to start in slot m, the NPUSCH is postponed to start in the next slot, whose index satisfies (SFN * 5 + ns) mod 4 = 0. The UE transmits TDM DMRS in the nth slot after the start of its NPUSCH transmission according to: Criterion; DMRS activity. OCC sequence index 0 [1 1]; OCC sequence index 1 [1 -1]. n mod 4 = 0; ON; OFF. n mod 4 = 1; ON; OFF. n mod 4 = 2; OFF; ON. n mod 4 = 3; OFF; ON.

As illustrated in Fig 2, the NPUSCH transmission with 3.75kHz SCS OCC starts in slot#n, which satisfies (5*nf+ns) mod 4=0. If slot#n+2 and slot#n+3 overlap with the fully reserved subframes, the NPUSCH transmission in slot#n+2 and slot#n+3 will be postponed to slot#n+4 and slot#n+5, which collides with the next TDM DMRS group and the TDM DMRS structure determined by the absolute slot index will be broken.

Fig 2. 3.75kHz OCC group overlaps with fully reserved uplink subframes

The issue of the broken TDM DMRS group is more serious for 20ms resource reservation periodicity (typical SSB period in NR). As depicted in Fig 3, 5 continuously fully reserved uplink subframes are configured with 20ms periodicity. In this case, the TDM DMRS group will be constantly broken and even the OCC group cannot be guaranteed to be transmitted in the continuous slots. Therefore, RAN1 should consider an effective solution to avoid the broken TDM DMRS group due to overlapping with the reserved resource for 3.75kHz OCC transmission.

Fig 3. 3.75kHz OCC group overlaps with fully reserved uplink subframes with 10ms periodicity

Observation 3: For Δf=3.75 kHz, when NPUSCH transmission with symbol-level OCC overlaps with the fully reserved uplink subframe, the postponement with the unit of two contiguous uplink subframes may break the TDM DMRS structure determined by the absolute slot index and even the OCC group.

Proposal 3: RAN1 should consider an effective solution to avoid the broken TDM DMRS group due to the reserved resource for 3.75kHz OCC transmission.

To avoid the occurrence of the broken TDM DMRS group due to the misalignment between the reserved resource and TDM DMRS structure, some companies propose to force the network to configure the reserved subframes with multiples of 8ms. In our understanding, this configuration restriction will result in a large amount of resource waste. As illustrated in Fig 4, if the original reserved subframes are subframe#0-subframe#4 with 20ms periodicity, the reserved subframes will be expanded to 32 subframes with 40ms periodicity to comply with the multiples of 8ms, where 22 subframes per 40 subframes are wasted. Moreover, the reserved subframes are semi-static configuration, so the resource waste will be constant during the resource reservation configuration.

Fig 4. Reserved subframes are multiples of 8ms

Observation 4: The configuration restriction on the resource reservation of multiples of 8ms will result in a large amount of resource waste (over 50%), and resource waste will be constant during the resource reservation configuration.

Alternatively, one possible solution is to perform the NPUSCH postponement based on the TDM DMRS group. As illustrated in Fig 5, when slot#n+2 and slot#n+3 within the TDM DMRS group overlaps with the fully reserved subframes, the NPUSCH transmission in the whole slot sets of the TDM DMRS group is postponed to the next TDM DMRS group..

Proposal 4: If a TDM DMRS group consisting of 4 slots overlaps with any fully reserved uplink subframe, the NPUSCH transmission in the whole slot sets of the TDM DMRS group is postponed until the next 4 slots starting with the first slot satisfying 5nf+ns mod 4=0 and not overlapping with any fully reserved uplink subframe.

Fig 5. TDM DMRS group based postponement

Conclusion

In this contribution, we present our view and analysis on the remaining issues for R19 IoT NTN. The following observations and proposals are made:

Observation 1: The slot index determination for TB transmission after OCC spreading is the same for symbol-level OCC and for slot-level OCC.

Observation 2: For 15kHz SCS, when OCC is enabled, the slot mapping procedure for TB transmission in TS36.211 is not aligned with the resource mapping procedure in TS36.213.

Observation 3: For Δf=3.75 kHz, when NPUSCH transmission with symbol-level OCC overlaps with the fully reserved uplink subframe, the postponement with the unit of two contiguous uplink subframes may break the TDM DMRS structure determined by the absolute slot index and even the OCC group.

Observation 4: The configuration restriction on the resource reservation of multiples of 8ms will result in a large amount of resource waste (over 50%), and resource waste will be constant during the resource reservation configuration.

Proposal 1: Adopt TP#1 for TS36.213 v19.2.0 to correct the slot mapping procedure for TB transmission for slot-level OCC for 15kHz SCS.

Proposal 2: OCC functionality should be supported for the scenario of IoT NTN coexistence with NR NTN.

Proposal 3: RAN1 should consider an effective solution to avoid the broken TDM DMRS group due to the reserved resource for 3.75kHz OCC transmission.

Proposal 4: If a TDM DMRS group consisting of 4 slots overlaps with any fully reserved uplink subframe, the NPUSCH transmission in the whole slot sets of the TDM DMRS group is postponed until the next 4 slots starting with the first slot satisfying 5nf+ns mod 4=0 and not overlapping with any fully reserved uplink subframe.

Reference

[1]   R2-2503064, “Reply LS on NR NB-IoT in-band operation”, RAN2, RAN2#129bis, April 7-11, 2025