3GPP TSG RAN WG1 #123			R1-2508469

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.6.1

Source:	Moderator (Thales)

Title:	FL Summary #4 - NR-NTN GNSS resilience

Document for:	Discussion, Decision

## Introduction

This Feature Lead Summary (FLS) document aims to collect and align company views on the Release-20 study concerning GNSS-resilient NR-NTN operation (RP-251933). It provides a summary of the contributions submitted under agenda item 10.6.1 at TSG-RAN WG1 #123, along with key issues identified. The objective of this FLS is to facilitate consensus building and offer recommendations for prioritizing discussions, including considerations for the potential postponement of certain topics.

A total of 43 contributions have been submitted for discussion and decision at the current meeting. The list of company Tdocs can be found in the reference section of this FLS. A compilation of company proposals regarding NR-NTN resilience to GNSS unavailability and degradation is available in document R1-2509476.

SI Objectives are [1]:

The study will be carried assuming that - The GNSS information in UE with GNSS capability may be 1/ temporarily unavailable or 2/ available but with GNSS position accuracy degradation or 3/ available but with increased GNSS measurement period for power saving purpose - This study will initially focus on 1/ (where GNSS information is temporarily unavailable) - Note: This study does not include any enhancement on positioning for the above scenario Study GNSS resilient NR-NTN operation [RAN1, RAN4] - Assess impact on initial access and connected mode procedures for NR-NTN, including potential implications if any on RAN4 RRM specifications - Aim to minimize physical layer procedures impact - Avoid physical layer channel/signal changes - Backward compatible issues with legacy NTN capable UEs will be assessed and should be prevented - Applicable to NGSO and GSO constellations, NTN operating in below and in above 10 GHz frequency bands, transparent and regenerative satellites - Check in RAN#112 (June, 2026) to decide whether to proceed with normative work, extend the study item, or postpone normative work to Release 21

# Topic#1 Evaluation assumptions and methodology

This section aims to further discuss the evaluation assumptions/parameters and the evaluation methodology.

## Background

On topic#1, RAN1#122 made the following agreements:

Agreement

For the study on GNSS resilient operation, scenarios should be considered where:

- Scenario 1: The UE cannot rely on its GNSS for timing and frequency compensation on the service link.

- Scenario 2: There is a previously acquired GNSS based position

  - UE has not received GNSS information for time duration T from the last acquired GNSS position and hence, the GNSS accuracy is degraded

    - FFS: value of T

    - FFS: GNSS based UE location validity duration

    - FFS: GNSS accuracy

- Note: Two scenarios above belong to description as in SID.

Agreement

For the study on GNSS resilient operation, the following scenarios are considered for evaluation purposes:

| Satellite orbit | GSO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude | 35786 km | 1200 km | 600 km |
| Satellite scenario parameters | At least Table 6.1.1.1-1 in TR 38.821, Parameters in Table 6.1.1.1-2 in TR 38.821could be considered. |  |  |
| Beam size (note 3) | At least values captured in Table 6.1.1.1-1/2 are considered, |  |  |
| Minimum elevation angle | 30° (LEO), 12.5° (GSO) |  |  |
| Frequency ranges/bands | FR1 NTN (2GHz/Ku: 14 GHz) and FR2 NTN (30 GHz) |  |  |
| UE type | Handheld (L/S band), VSAT (Ku/Ka) |  |  |
| UE speed (note 1) | L/S band: 3 km/h, 120km/h, 1500 km/h, Ku/Ka: 3 km/h , 120 km/h, 1500 km/h, (note 2) |  |  |
| Beam/cell type (note 4) | (Quasi)-Earth-fixed beams/cells and Earth-moving |  |  |
| Note 1: UE altitude is considered Note 2: 10 km altitude in case of aircraft scenario. 1500 km/h is for aircraft scenario. Note 3: Same values can be reused for other elevation angle than nadir. Beam size of the edge beam can be reported by companies. Note 4: Footprint of the beam could be: case 1: within the orbital plane, case 2: at 90° with respect to orbital plane. Companies to report the elevation angle of the beam. Note 5: Other parameters can be reported by companies. |  |  |  |

Note: The cyan highlighted parameters are prioritized for evaluation purposes.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| CMCC | Proposal 1: For the study on GNSS resilient operation, the detailed satellite parameters of Ku-band for evaluation purpose need to be defined. Proposal 2: It is suggested to prioritize the LEO based deployment in S-band for the study on GNSS resilient operation. Proposal 3: For alternatives representing two calculation methodologies, Alt1 with some modifications is selected, such as revising the subscripts of some variables in the formulas. The version in the FL recommendation in R1-2508178 can be used as a reference. Proposal 4: The minimum elevation should be calculated at farthest w.r.t satellite point of the uncertainty area. Proposal 5: If considering the limitation of simultaneously active beams in different NR NTN deployment scenarios, the feasibility of network broadcasting the configuration of reference signals or the reference location need to be studied. |
| CMCC | Proposal 7: It should be further discussed whether a unified solution should be strived to solve both low speed mobility (3km/h, 120km/h) and high speed mobility (1500km/h) of the air traffic. Proposal 9: For evaluating positioning performance of GNSS resilient NR-NTN operation, the evaluation metrics and assumptions of RAT dependent positioning methods study in Rel-18 NR NTN can be considered as a start point. |
| ST Engineering iDirect | Proposal 7: Discuss evaluation scenarios and methodology for connected mode. Proposal 8: Add an evaluation framework for time and frequency tracking in connected mode, considering the propagation delay, with a round-trip time of 4 ms for LEO-600 when the satellite is at nadir and up to 12.6 ms when the satellite is at an elevation angle of 30⁰. |
| CATT | - Confirm Alt-1 working assumption with the additional of cross orbit calculation procedure. d=RE+hue2sin2a+hs-hue2+2hs-hueRE+hue-RE+huesina φ=RcRE β=arcsin⁡RE+hue*sinφ2d ∆fd=2*Vsat+Vue*sinβc*fc - Based on existing evaluation results, it is observed that some extreame cases will cause large gap between the requirement and existing PRACH capability. We suggest differentiating the priority of use cases for Rel20 and 6G since the study window of 5G-A and 6G is overlapped. |
| Xiaomi | Proposal 1. Given the linkage to RAN4 specification on timing adjustment accuracy and positioning margin, it’s suggested RAN1 needs to involve RAN4 in the location uncertainty value X determination. |
| OPPO | The following formulas are used to calculate the differential one-way Doppler for across orbit case: dcenter=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα β=Rcdcenter fdref=vssinβ+vuecosαcfc ∆fd=2fdfar Illustration of satellite across orbit |
| Ericsson | Proposal 2 Use calculation methodology Alt 2 as baseline when a single calculation methodology is selected. |
| ZTE | Proposal 4: For evaluation of differential delay and Doppler, the minimum elevation is at farthest w.r.t satellite point of the uncertainty area. Proposal 5: UE altitude uncertainty should be assumed as zero considering that altitude is generally measured by barometric pressure and not relevant to GNSS error. |
| ETRI | Proposal 2: RAN1 to consider the following aspects from UE perspectives for GNSS performance degradation modelling: - Detection mechanism to clarify GNSS status (and corresponding detection probability/latency on GNSS status) - Time-To-Recover - Max position drift / drift variation - FFS, other aspects Proposal 3: RAN1 to consider the following aspects from system perspectives for GNSS performance degradation modelling: - (Timing) pattern of GNSS-unavailability occurrence Maximum duration of GNSS-unavailable state (i.e., max outage duration) • FFS, other aspects. Proposal 1: RAN1 to select Alt 1 for future evaluations: - The typos in Alt1 from the Agreement need to be corrected (the version from final FLS is correct) as follows: - ∆t=dfar-dnearc and ∆fd=fdfar-fdnear - fdnear=vssinβ1-vuecosγcfc - φ1=φ-2RcRE+hue (for non-nadir beam) and φ=RcRE+hue (for nadir beam) - It is preferred to capture Alt 1 for the NTN channel model TR Proposal 5: RAN1 to consider the beam footprints projected onto the Earth’s surface for evaluation of NR-NTN GNSS resilience. • The values of X and T should be determined by considering the radius of edge beam, rather than by the radius of Nadir beam. Proposal 9: RAN1 to consider the Scenario 1 as baseline - Scenario 2 can be optionally considered during this SI |
| Honor | Proposal 1: RAN1 to define the GNSS accuracy and evaluate the threshold T at which the degraded GNSS accuracy exceeds network’s acceptable limits under different scenarios. Proposal 2: The GNSS based UE location validity duration can be equal to T. The extended validity duration can be determined based on the network indication. |
| Lenovo | Observation 1: Both methodologies for the calculation of the differential one-way delay and the differential one-way Doppler/frequency offset provide similar results for differential one-way delays but differ slightly in differential one-way Doppler/frequency offset. |
| TOYOTA | Proposal 3: In Case b where the location uncertainty area is a circle of radius X km, values up to the half of the beam diameter should be considered. Based on the agreement in RAN WG1 #122, beam diameters in Tables 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821 should be considered. Proposal 4: For evaluation of Case b where the location uncertainty area is a circle of radius X km, at least, values up to the half of the Nadir beam diameter stated in Tables 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821 should be considered. Values up to 45 km should be considered for LEO600 (S/2GHz). Values up to 95 km and 45 km for S/2GHz and Ka/30GHz, respectively should be considered for LEO600. Values up to 225 km and 140 km for S/2GHz and Ka/30GHz, respectively should be considered for GEO. Proposal 5: For evaluation of the largest delay and Doppler, for both Case a and Case b, the uncertainty area is defined by a single set of values where the highest value is obtained from the respective beam/cell diameter (e.g., Tables 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821). The observations are reported and collected separately for Case a and Case b. Proposal 6: For Case a evaluations, the uncertainty area should be defined taking the lower elevation/non-Nadir beam diameters into account. A realistic scenario in which the lower elevation/non-Nadir beam diameter is several times larger than the Nadir beam diameter should be considered. |
| NTT DOCOMO | Proposal 1: For Scenario 2: - T is no smaller than the GNSS based UE location validity duration - FFS: upper bound of the value of T |
| Johns Hopkins University | Proposal 2: Large beam sizes can be advantageously used to provide coverage in sparsely populated regions using only a single beam. Therefore, RAN1 to study techniques for providing initial access in scenarios with large beam sizes, e.g., for locations within the 6-dB beamwidth contour. Proposal 3: RAN1 to incorporate the Earth’s motion into the equations that calculate the time variations of the Doppler frequency and the delays. |
| CSCN | Proposal 1: The worst case of GNSS resilient operation, i.e., Scenario 1, should be prioritized. Solutions designed for Scenario 1 can be partially or fully adopted to address the timing and frequency synchronization of Scenario 2. Proposal 7: It may be worth considering whether there is an issue with the one-way Doppler equation in Alt-2. |
| CEWiT | Proposal 1: The following methodology (Alt 2 from last meeting) can be used to evaluate one-way differential delay and one-way differential Doppler: For the calculation of one-way differential delay and one-way differential Doppler, the following geometry model is considered. For the variables in the equations, refer to Figure-X in the agreements section. Considering satellite location at s=0,0,RE+h One-way delay between a point u and satellite: done-wayu=¦u-s¦2c One-way Doppler between u and satellite (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speed vue: dopplerone-wayu=vcfcu-s,x>done-wayu+vuecfcs-u,k>done-wayu Let R be the location uncertainty area. Differential one-way delay = maxu∈Rdone-wayu-minu∈Rdone-wayu Differential one-way doppler = maxu∈Rdopplerone-wayu-minu∈Rdopplerone-wayu For evaluation purpose, R can be a circle on surface of earth (height can be considered optionally), placed such that the furthest point in the uncertainty area can view the satellite at an elevation angle e and is located at an azimuth angle ϕ from the orbital plane (x-y plane). Note: Assume satellite and UE moves parallel to the x-y plane. ⋅,⋅> is the vector inner product, ∨.∨2 is the vector magnitude. |

## Summary of companies’ contributions

Regarding calculation methodology Alt 1 and Alt 2 for differential delay and Doppler evaluation:

Most companies recognize Alt 1 as the traditional, baseline approach for calculating differential delay and Doppler in NTN systems. Companies such as CMCC, CATT, ETRI, ZTE, Toyota, Honor, Lenovo, MediaTek, and OPPO generally support Alt 1, with several stressing the need for corrections to typos (in last meeting agreement), updates to formula variables, or inclusion of additional scenarios such as cross-orbit configurations. ETRI and CATT, for instance, specifically recommend using Alt 1 for future evaluations, provided necessary corrections are implemented, with ETRI emphasizing its relevance for modeling and uncertainty definition based on edge beam radius. Many companies such as Ericsson, Lenovo and MediaTek note that Alt 1 and Alt 2 typically yield practically identical results for differential delay and that minor differences in Doppler calculations are only significant in special or edge-case scenarios. MediaTek explicitly states that both methodologies are fundamentally equivalent for delay evaluation, and practical NTN configurations reflect this equivalence.

On the other hand, Alt 2 (which is a vector-based, more rigorous approach) has been advocated by Ericsson and CEWiT. Ericsson prefers Alt 2 as the unified baseline when a single method is chosen, highlighting its greater flexibility, its ability to model important phenomena like Earth rotation, varying user mobility, and complex edge-beam cases that Alt 1 may not handle well. CEWiT provides a robust set of vector-based equations aligned with Alt 2, emphasizing suitability for thorough evaluation. CSCN also lends support to Alt 2, particularly for worst-case scenario (Scenario 1) analyses, although it flags the need to carefully review the one-way Doppler equation for accuracy.

Several companies contribute nuanced perspectives. For example, OPPO presents explicit geometric formulas that extend Alt 1 to cross-orbit cases, while iDirect (though not expressing a direct Alt 1/Alt 2 preference) highlights the need for real-world parameter consideration such as round-trip delays and propagation effects. Xiaomi and Johns Hopkins University also do not specify a methodological preference but emphasize the importance of cross-group alignment on assumptions (Xiaomi) and the incorporation of Earth’s motion and large beam sizes in calculations (Johns Hopkins University). NTT DOCOMO focuses its input on the appropriate setting of timing boundaries essential for the evaluation, rather than on specifying a preferred calculation alternative.

The emerging consensus from these collective views is that Alt 1, particularly when corrected and carefully applied, remains a standard and acceptable choice for most fundamental evaluations. Many companies, however, see the value in Alt 2; for example, when modeling Earth rotation, edge beams, or high-mobility use cases. Multiple contributors agree that for most practical deployments, both approaches produce similar results for delay and only minor differences for Doppler, provided that all practical geometric and mobility factors are properly considered.

In short:

| Method | Advantages | Critique/limitation | Companies supporting |
| --- | --- | --- | --- |
| Alt 1 | Simpler, standard geometric approach | Needs corrections; lacks Earth rotation, edge-beam flexibility, typos present | CMCC, CATT, ETRI, ZTE |
| Alt 2 | Rigorous, vector-based, flexible | Potential for complexity; needs review of Doppler equation | Ericsson, CEWiT |
| Both (Equivalence) | Delay results fundamentally equivalent; Doppler differences minor in practical scenarios | -- | MediaTek, Lenovo (comparisons), CSCN (flagging issue) |

## Initial proposal

### Proposal 1-1

Based on the above discussion the following proposal is made:

Proposal 1-1-v0

Confirm Alt-1 working assumption as follows.

For non-nadir beam dfar=RE+hue2sin2α+hs-hue2+2hs-hueRE+hue-RE+huesinα. φ1=φ-2RcRE. dnear=RE+hue2+RE+hs2-2RE+hueRE+hscosφ1. φ=sin-1dfarRE+hscosα. β=π2-α+φ. β1=sin-1RE+huednearsinφ1. γ=cos-1RE+hsdnearsinφ1. fdfar=vssinβ+vuecosαcfc. fdnear=vssinβ1+vuecosγcfc. ∆t=dfar-dnearc. ∆fd=fdfar-fdnear For nadir beam dnear=hs-hue φ=RcRE dfar=RE+hue2+RE+hs2-2RE+hueRE+hscosφ β=sin-1RE+huedfarsinφ α=π2-β+φ fdfar=vssinβ+vuecosαcfc ∆t=dfar-dnearc ∆fd=2fdfar.

Companies are encouraged to comment on Proposal 1-1-v0

| Company | Comments |
| --- | --- |
| CMCC2 | We are fine with the proposal. The version from last meeting’s FL summary can be used. |
| OPPO | We are generally fine with this proposal, but the current Alt1 version can be only used to calculate the one-way differential doppler for the non-nadir beam within the orbit plane (i.e., case 1 in the collected results), which should be clarified in this proposal. For example, we can say “for non-nadir beam within the orbit plane: ” fdfar=vssinβ+vuecosαcfc. fdnear=vssinβ1+vuecosγcfc. ∆fd=fdfar-fdnear. In addition, we note that the non-nadir beam at 90° with respect to orbital plane is also considered in the evaluation (i.e., case 2 in the collected results), so the corresponding calculation methodology on the one-way differential doppler should also be discussed as a component of Alt 1. For sake of discussion, the following formula can be taken into account: “for non-nadir beam at 90° with respect to orbital plane” β2=Rcdfar. fdfar=vssinβ+vuecosαcfc. ∆fd=2fdfar. Fig. Illustration of non-nadir beam at 90° with respect to orbital plane |
| Spreadtrum | Fine with the proposal |
| Samsung | Alt. 1 is not agreed as working assumption. It is unclear to confirm working assumption. Though it is preferable to consider fdnear=vssinβ1-vuecosγcfc as we believe that both edge UEs move the opposite direction (i.e., heading to the center of beam). If majority is fine with it, we can accept. |
| JHUAPL | There is an underlying assumption that the Earth’s rotational motion is neglected. To be accurate, this assumption should be stated explicitly. |
| Tejas Networks | Support the proposal |
| Huawei, HiSilicon | Fine with the proposal |
| Toyota ITC | Generally fine with the proposal. Agree with OPPO that formulation should include for non-nadir beam at 90° with respect to orbital plane. Note that this scenario is included in the spread sheet |
| Ericsson | There are issues with Alt 1 that seem not straightforward to solve. If they can be solved, we are fine to go with Alt 1, otherwise we prefer that Alt 2 is included in the TR. - Alt 1 does not apply to beams outside the orbital plane. Can equations for an edge beam at 90 degrees relative to the orbital plane be added? - It needs to be clarified how to model satellite velocity vs Earth rotation. The satellite velocity is >3000 m/s for a GEO satellite (in ECI), but still this should not induce to any Doppler shift relative to the UE, since the UE on the Earth surface "moves along" with the satellite. For GEO (0° inclination), it would be most correct to set vs=0. For LEO, the Earth rotation velocity could be subtracted from the UE velocity but how this is done depends on the satellite inclination angle, UE position and movement direction, etc. - Consider if vue should have opposite sign in fdnear (for the non-nadir beam case) to take into account the max differential Doppler contribution of UE velocity. |
| LGE | OK. |
| Panasonic | Support. |
| Nokia | From technical point of view, our understanding is that modifications has happened to the Alt1 equations, so it is not a confirmation of the working assumption, but a confirmation of working assumption with modifications. If this is to happen, we need to have the changes highlighted. Alternatively, the working assumption is not confirmed – and a new agreement is made. |
| ZTE | Fine with the proposal. The formula φ=sin-1dfarRE+hscosα for non-nadir beam can be move before the formula φ1=φ-2RcRE, since φ is needed for calculation of φ1. |
| Apple | Support |
| ETRI | We are fine with the proposal. |
| Futurewei | No strong preference for Alt-1 or Alt-2. Recommend capturing both because the two alternatives are complementary. Further, both may need revisions. For example, the definition of UE elevation angle α should be equal to 90° with respect to the agreed evaluation assumptions for nadir beams. However, in Alt-1, the elevation angle depends on φ=RcRE and β. Suggest using a different symbol for α. |
| Qualcomm | The azimuth angle is missing in the formulation. |

### Proposal 1-2

While both Alt 1 and Alt 2 are widely accepted, Alt 2 is increasingly favored for evaluations due to its flexibility, accuracy, and ability to handle complex modeling needs. No company objects to Alt 2, and most recommend that whatever methodology is selected, it should be well-defined, corrected, and grounded in practical deployment scenarios to ensure consistency and reliability in evaluation outcomes.

From Moderator’s perspective, both alternatives could be captured in the TR.

Proposal 1-2-v0:

Alt2 could be also captured in the TR 38.742 (e.g. in the annex).

Companies are encouraged to comment on Proposal 1-2-v0

| Company | Comments |
| --- | --- |
| OPPO | Support. Serval companies spot an error on the one-way doppler calculation, and the correction may be needed, e.g., dopplerone-wayu=vcfcu-s,x>¦u-s¦2done-wayu+vuecfcs-u,k>¦u-s¦2done-wayu |
| CSCN | Support, and one correction would be needed for the equation of one-way Doppler. dopplerone-wayu=vc2fcu-s,x>done-wayu+vuec2fcs-u,k>done-wayu |
| CEWiT | Support |
| Samsung | No strong view. However, according to the last meeting agreement, we should select one of them. |
| ST Engineering iDirect | In general, we agree that the two alternatives can be captured in the TR. This only makes sense if simulation results for both alternatives are presented. Suggestion to have a dedicated section “Evaluation methodology” in TR 38.742, e.g., in clause 5.4. |
| JHUAPL | Again, to be accurate, the underlying assumption that the Earth’s rotational motion is neglected should be stated. |
| Tejas Networks | Support . Alt2 is more generic form for calculation of differential delay and doppler and can be captured in the TR with necessary corrections |
| Huawei, HiSilicon | As illustrated in our contribution, Alt2 should be corrected for the red part： One-way doppler between u and satellite: (assuming satellite is moving along x=1,0,0 with speed v and UE moving along k=cosα,sinα,0 with speedvue: dopplerone-wayu=vc2fcu-s,x>done-wayu+vc2fcu-s,k>done-wayu Also fine with OPPO’s version. |
| Toyota ITC | Support (with the equation correction). |
| Ericsson | See response to Proposal 1-1-v0. |
| Panasonic | Support. |
| Nokia | We find that Alt2 definitely should be included to the TR. For us the bigger question is whether we should also include Alt1. Currently we have a preference for Alt2 only, since it would simplify the descriptions in the TR such that the text does not need to refer to which of the models is being used. |
| ZTE | OK with the proposal if Alt2 is correctly captured, e.g., the formula for Doppler calculation should be divided by distance instead of delay as commented by other companies. |
| NTT DOCOMO | We are fine with the proposal. Alt 2 should be included into the TR. |
| Futurewei | OK with the proposal |
| Qualcomm | Agree with proposed change from Huawei. |

# Topic#2 Evaluation of differential delay and Doppler

## Background

RAN1#122bis made the following agreement:

Agreement:

For the evaluation of GNSS resilient NR-NTN operation, consider the following assumptions:

- For scenario 1 and 2

  - The following time/frequency differences should be evaluated:

    - The differential one-way delay (in µs) as calculated between the largest UE-Sat delay and smallest UE sat delay within the uncertainty area.

      - For UL performance evaluation 2 times differential one-way delay should be considered.

    - The differential one-way Doppler/frequency offset (in ppm) is calculated between the largest UE-sat Doppler/frequency offset and smallest UE-sat Doppler/frequency offset within the uncertainty area.

      - For UL performance evaluation, scaled one-way differential Doppler/frequency offset to determine UL differential Doppler/frequency offset should be considered. Scaling factor should be reported.

## Companies’ proposals

### Proposal 2-1

The above summary shows substantial consensus on the physics: differential delay and Doppler scale up with satellite altitude, beam size, frequency, user velocity. The maximum differential delay is observed at lower elevation (30° for LEO or 12.2° for GSO). The maximum differential Doppler is observed at 90°elevation angle.

For a more consolidated evaluation in TR 38.742, it is proposed to use a spreadsheet to collect companies’ results on differential round trip delay and differential round Doppler.

FL Recommendation:

Companies are encouraged to fill in/complete/check the companion spreadsheet The companion spreadsheet [*] is used to collect companies’ results on differential round trip delay and differential round Doppler. - [*] Evaluation of differential delay and Doppler v000_Moderator

Companies are encouraged to comment on FL Recommendation:

| Company | Comments |
| --- | --- |
| OPPO | Support. And one question for clarification, the spreadsheet is used to collect calculation results on one-way differential delay and one-way round doppler, instead of differential round trip delay and differential round Doppler? |
| Spreadtrum | Clarification in needed that the companion spreadsheet is used for one-way differential delay and one-way differential Doppler, not for differential round trip delay and round trip Doppler. |
| Tejas Networks | Support but need clarification as spreadsheet is used to collect one-way differential delay and one-way differential doppler. |
| Toyota ITC | As detailed in our contribution, spread sheet proposed values do not reflect the realistic scenario. - Spread sheet only captures Nadir beam diameters. Non-nadir beam can be much larger than Nadir pointing beam - Proposed values of X km is too small for some scenario in practice - Proposed format of the spreadsheet unnecessarily overloads the simulation efforts Below, respective observations are copied: Observation 4: In Case b where the location uncertainty area is a circle of radius X km, the highest value of X km is the radius of a circular beam or cell. Observation 5: Currently proposed X = 1, 5, 10, 25 km values for Case b evaluations do not reflect the practical uncertainty areas of S/2GHz, Ka/30GHz carrier frequencies and LEO600/LEO1200/GEO orbits. Observation 6: For evaluation of the largest differential delay and Doppler, values for uncertainty area can be defined as: • For Case a: A circular uncertainty area where the diameter of the area is obtained from the respective Tables such as 6.1.1.1-1 and 6.1.1.1-2 in TR 38.821, and • For Case b: A discrete set of values of X km where the highest value of X km is the radius of the corresponding uncertainty area considered for Case a. |
| Ericsson | Support. Our values seem to be correctly copied to the spreadsheet. |
| LGE | Supportive for the direction. As other companies have mentioned, it should be clarified that the spreadsheet is about one-way delay/Doppler. We have added our results to the spreadsheet. |
| Nokia | We would like to have more clarity on the purpose of this. Is the intention to have “calibration” or “check of whether companies can implement equations”? |
| Futurewei | Is alignment to be done if evaluation results from different companies deviate from each other? |

## Scenario 1

| Companies | Proposals and observations |
| --- | --- |
| FUTUREWEI | Observation 1: Maximum differential one-way Doppler shift ∆fDS occurs at the largest elevation angle (i.e., 90°) and the largest radius (at X=rbeam) with LEO-600 being the highest among all the scenarios. Observation 2: Maximum differential one-way delay ∆T occurs at the lowest elevation angle (i.e., 12.5° for GSO, and 30° for LEO-600 and LEO-1200) and the highest radius (at X=rbeam) with GSO being the largest among all the scenarios. |
| Eutelsat | Error: Reference source not found Error: Reference source not found Error: Reference source not found Error: Reference source not found |
| vivo | Error: Reference source not found Error: Reference source not found Error: Reference source not found |
| Thales | Observation 4: For Case a, where the location uncertainty is defined by the area served by the cell or beam, and considering performance within the orbital plane: - The observed one-way differential delay increases with both the beam diameter and decreasing elevation angle. - At low elevation angles (12.5° for GEO and 30° for LEO), differential delays are considerably higher than at high elevation angles (90°). This is expected given the longer slant path and greater spatial diversity at low elevation angles. For case b, where the location uncertainty is defined by the area of X km radius, and considering performance within the orbital plane: - The results demonstrate that one-way differential delay increases significantly with the expansion of the uncertainty area (X) and reduction of elevation angle, regardless of orbit or frequency band. For instance, at a low elevation angle of 12.5°, the delay can reach up to 16.37 µs for a 5 km radius and 32.64 µs for a 10 km radius, and up to 81.44 µs for a 25 km radius, across all orbit types and both S-band (2 GHz) and Ka-band (30 GHz). - In contrast, at a high elevation angle of 90°, the maximum delay drops sharply, remaining below 0.08 µs for 5 km, 0.3 µs for 10 km, and below 2 µs even for the largest uncertainty radius. Observation 5: For Case a, where the location uncertainty is defined by the area served by the cell or beam, and considering performance within the orbital plane: - Doppler values increase substantially as the elevation angle increases; from 12.5° up to 90°. - At 12.5° elevation, the differential Doppler for LEO600 ranges from 0.073 ppm (Set1, S-band, 50 km beam) to 0.138 ppm (Set2, S-band, 90 km beam), whereas for GEO it is always below 0.03 ppm. - At 30° elevation, LEO600 values increase to 0.382 ppm (Set1) and 0.724 ppm (Set2), with GEO values remaining well below 0.1 ppm. - At 90° elevation, LEO600 Doppler jumps up to 2.094 ppm (Set1) and as high as 3.737 ppm (Set2), while GEO remains at 0.072 to 0.129 ppm (S-band). - At 12.5° or 30°, the observed Doppler is at its minimum for all orbits, while at 90°, Doppler values reach their highest levels. - For LEO600 at 90° elevation: The highest ppm values are reached (up to 3.737 ppm in Set2). - Larger beam diameters tend to result in higher maximum differential Doppler, particularly evident in LEO cases. - For GEO: Across all cases, the maximum ppm values remain below 0.13 ppm, indicating a relatively stable frequency environment. For case b, where the location uncertainty is defined by the area of X km radius, and considering performance within the orbital plane: - The one-way differential Doppler increases notably with larger uncertainty areas and higher elevation angles, especially for LEO orbits in both S-band and Ka-band. For example, at a 90° elevation and a 25 km uncertainty radius, the differential Doppler reaches up to 1.051 ppm for LEO600. At the same elevation, for smaller uncertainty radii such as 5 km and 10 km, the differential Doppler for LEO600 is 0.211 ppm and 0.421 ppm, respectively. In contrast, the values remain negligible for GEO orbits across all conditions. Proposal 3: Document the observations above in TR 38.742, together with the results compiled in the accompanying spreadsheet, ‘Evaluation of Differential Delay and Doppler v1.00_Thales’. |
| Huawei | Observation 2: In FR1-NTN, when UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by a beam (scenario 1 + case a), - the differential one-way delay and one-way Doppler can be as large as 142us and 2.09ppm under LEO600, 256us and 1.81ppm under LEO1200, 812us and 0.0008ppm under GEO. Observation 3: In FR2-NTN, when UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by a beam (scenario 1 + case a), - the differential one-way delay and one-way Doppler can be as large as 57us and 0.84ppm under LEO600, 115us and 0.81ppm under LEO1200, 356us and 0.0003ppm under GEO. Observation 4: When UE cannot rely on its GNSS for timing and frequency compensation on the service link and the horizontal location uncertainty area is the area served by a beam (scenario 1 + case a), - the differential delay in edge beam is much larger than that in nadir beam; - the differential doppler value in nadir beam is larger than that in edge beam across the orbit plane and then larger than that in edge beam along the orbit plane; - the larger the location uncertainty area is, the larger the differential delay and doppler values are. |
| CATT | - Based on the results of table 1 for scenario 1, for GSO, the beam size can reach up to 250km, resulting in a differential one-way delay of over 800us. - Based on the results of table 1 for scenario 1, for LEO-600, due to the extremely fast movement speed of the satellite, a differential frequency offset of more than 4.5 kHz in S/C band and 27 kHz in Ka band will be obtained. |
| Xiaomi | Proposal 2: Adopt the Differential delay and Doppler values in table 2 [R1-2508681] for further evaluation performance analysis including the performance evaluation of PRACH. |
| Tejas Network | Observation 1: The maximum one-way differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.26 ms,0.535 ms and 1.5 ms respectively for Set 2 scenarios in S band and the maximum one-way residual frequency offset are 7.58 khz , 7.96 khz and 0.00185 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively. Observation 2: The maximum one-way differential delay for LEO 600km,LEO 1200km and GEO scenarios are 0.0575 ms,0.115 ms and 0.357 ms respectively for Set 2 scenarios in Ka band and the maximum one-way residual frequency offset are 26.74 khz , 27.13 khz and 0.848 khz for LEO 600 km ,LEO 1200 km and GEO orbits respectively. Observation 3: The maximum one-way differential delay for LEO 600km,LEO 1200km and GEO scenarios are 1.18 ms,2.24 ms and 3.46 ms respectively for Set 2 scenarios in S band considering the beam size increases with reducing elevation angle |
| Samsung | Observation 1: For LEO-600 2GHz, maximum differential delay is 71.8 μs if the reference point is applied. Otherwise, 143 μs. Observation 2: For LEO-600 2GHz, maximum differential Doppler is 2107 Hz, if the reference point is applied. Otherwise, 4181 Hz. Observation 3: For LEO-1200 2GHz, maximum differential delay is 129.2 μs if the reference point is applied. Otherwise, 256.9 μs. Observation 4: For LEO-1200 2GHz, maximum differential Doppler is 1821 Hz if the reference point is applied. Otherwise, 3612 Hz. Observation 5: For GSO 2GHz, maximum differential delay is 405.8 μs if the reference point is applied. Otherwise, 810.9 μs. Observation 6: For GSO 2GHz, maximum differential Doppler is 134 Hz, which happens for aircraft travelling at 1500 km/h if the reference point is applied. Otherwise, 5426 Hz. Observation 7: For GSO 14GHz, maximum differential delay is 178.8 μs if the reference point is applied. Otherwise, 357.8 μs. Observation 8: For GSO 14GHz, maximum differential Doppler is 413 Hz, which happens for aircraft travelling at 1500 km/h if the reference point is applied. Otherwise, 3797 Hz. |
| ZTE | Proposal 6: The largest differential delay and largest differential Doppler among all potential elevation angles should be used for PRACH performance evaluation considering the satellite mobility and semi-static configuration of PRACH format. Observation 3: The one-way differential delay and one-way differential Doppler for beams within orbital plane are evaluated as follows: see R1-2508854 |
| ETRI | Observation 2: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between UEs at the farthest and closest points from the satellite for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows: see R1-2508970 Observation 3: When the location uncertainty area is the area served by the beam, the maximum differential position (Δp), delay (Δt), and the differential Doppler values (Δfd) between the farthest UE and the reference location for Set-1 in Table 6.1.1.1-1 TR 38.821 are as follows: see R1-2508970 Proposal 7: RAN1 to consider the following values of maximum differential delay (Δt), - Δt = 487 μs for Set-1 LEO 600km - Δt = 800 μs for Set-1 LEO 1200km Proposal 8: RAN1 to consider the following values of maximum differential Doppler (Δfd), - Δfd = 1.51 ppm for Set-1 LEO 600km - Δfd = 1.41 ppm for Set-1 LEO 1200km |
| Lenovo | Observation 2: For all orbits, the differential one-way delay increases as the uncertainty area widens, reflecting the larger difference in slant path between the nearest and farthest edge users of the beam. Observation 3: At 60° elevation, the same uncertainty area produces noticeably smaller delays than at 30°, because the line-of-sight is more vertical and the path-length difference across the footprint shrinks. Observation 4: For LEO satellites, Doppler differences reach thousands of hertz even over modest footprint radius, particularly at higher elevations where the satellite’s velocity component toward/away from the user is largest. Observation 5: The delay values at higher UE speeds closely match those seen at lower UE speeds, as delay spread depends almost entirely on satellite altitude, beam radius, and elevation angle, not on UE motion. |
| MediaTek | Observation 1: The maximum differential delay and maximum Doppler in ppm are agnostic to the frequency band (i.e. it depends on the beam diameter assumption and satellite orbit height). Observation 2: The maximum differential delay for Case a Case 1 are for - Set 1: 163 us, 293 us, and 815 us for LEO-600, LEO-1200 and GEO respectively - Set 2: 293 us, 620 us, and 1634 us for LEO-600, LEO-1200 and GEO respectively Observation 3: The differential delay for Case b Case 1 with 2.5 km uncertainty area are 16 us for LEO-600, LEO-1200 and GEO. Observation 4: The maximum differential Doppler for Case a Case 1 are for - Set 1: 2.09 ppm, 1.87 ppm, and 0.07 ppm for LEO-600, LEO-1200 and GEO respectively - Set 2: 3.77 ppm, 3.94 ppm, and 0.13 ppm for LEO-600, LEO-1200 and GEO respectively Observation 5: The differential Doppler for Case b Case 1 with uncertainty area with X=5 km at nadir are 0.21 ppm, 0.104 ppm and 0.0014 ppm for LEO-600, LEO-1200 and GEO respectively. Proposal 1: RAN1 to capture observations on differential delay and differential Doppler shift. |
| China Telecom | Observation 1: Calculations indicate that the differential one-way delay decreases with increasing elevation angle in both the along-orbit and cross-orbit planes. Observation 2: Calculations show that the differential one-way Doppler in the along-orbit plane increases as the elevation angle rises. |
| Qualcomm | Proposal 2: Capture Table 1 in the TR (see R1-2509228). |
| NTT DOCOMO | Observation 1: - Regarding the one-way differential delay considering uncertain UE location, - In GNSS resilience scenario 1, the maximum one-way differential delay is 288us for LEO-600, 519us for LEO-1200, 1625us for GSO; Observation 2: - Regarding the one-way differential doppler considering uncertain UE location, - For FR1-NTN, - In GNSS resilience scenario 1, the maximum one-way differential doppler is 8.4kHz for LEO-600, 7.6kHz for LEO-1200, 0.28kHz for GSO; - For FR2-NTN, - In GNSS resilience scenario 1, the maximum one-way differential doppler is 126.2kHz for LEO-600, 113.6kHz for LEO-1200, 4.2kHz for GSO; |
| Panasonic | Observation 1: Due to symmetry, the observed differential delays are identical for case 1 and case 2, i.e, along-track beam and cross-track beam. Observation 2: The lower the elevation angle, the larger the differential one-way delay. Observation 3: The worst case differential Doppler occurs at elevation angle 90° when the signs of Doppler shifts at opposite cell edges are different. Observation 4: For elevation angles less than 90° the differential Doppler shift for cross-track beams (case 2) is larger than for along-track beams (case 1), but less than the worst case differential Doppler. Observation 5: For case b, smaller differential delay and differential Doppler than for case a are observed because of the smaller uncertainty area. |
| Johns Hopkins University | Observation 2: For the same antenna beamwidth size, the highest differential Doppler frequency offset occurs for nadir-pointing beams, and the highest differential one-way delay occurs for off-nadir-pointing beams. |
| LGE | Observation #5: Differential delay increases while differential Doppler decreases as the elevation angle increases. Observation #6: Differential delay and Doppler are almost proportional to the radius of the uncertainty region. |
| CEWiT | Observation 1: For Reference-Point based TA computation in LEO at 600 km with a beam radius of 25 km, the maximum timing error is 82.08 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 1.040 ppm, observed at an elevation angle of 90°. Observation 2: For Reference-Point based TA computation in LEO at 1200 km with a beam radius of 45 km, the maximum timing error is 147.75 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.936 ppm, observed at an elevation angle of 90°. Observation 3: For Reference-Point based TA computation in GSO at 35786 km with a beam radius of 125 km, the maximum timing error is 410.35 μs, occurring at an elevation angle of 10°, while the maximum frequency error is 0.087 ppm, observed at an elevation angle of 90°. |

## Scenario 2

| Companies | Proposals and observations |
| --- | --- |
| vivo | Error: Reference source not found Error: Reference source not found Error: Reference source not found Error: Reference source not found Error: Reference source not found |
| Huawei | Observation 7: In scenario 2 + case b, when the elevation angle of a previously acquired GNSS based position is 30°, the maximum tolerable location uncertainty X and the time duration T is constrained by the tolerance to timing error of the preamble formats. Observation 8: In scenario 2 + case b, when the elevation angle of a previously acquired GNSS based position is 90°, the maximum tolerable location uncertainty X and the time duration T is constrained by the tolerance to frequency error of the preamble formats. Observation 9: Increasing GNSS measurement period can significantly reduce UE power consumption which is important to prolong the battery life of portable device. |
| CATT | - The evaluation results of Scenario 2 based on a limited uncertain area are shown in Table 2, which can be taken as one starting point for existing PRACH format usage. - When the radius of the uncertain area is 10km in scenario 2, the maximum differential one-way delay will be more than 65us, and the maximum differential one-way frequency offset will be 1.8 kHz in S/C band and 27 kHz in Ka band. |
| Tejas Network | Observation 4 : In the worst-case scenario, the maximum differential delay and differential doppler observed for farther beams in Scenario 2 is equivalent to that in Scenario 1, provided the GNSS displacement error reaches the beam diameter. Proposal 2 : The worst-case differential delay and residual Doppler across all elevation angles and beam configurations in Scenario 2 are equivalent to those in Scenario 1, assuming the maximum GNSS displacement error spans the beam diameter. Consequently, any mitigation or compensation strategy developed for Scenario 1 is directly applicable to Scenario 2 as well. |
| NTT DOCOMO | Observation 1: - Regarding the one-way differential delay considering uncertain UE location, - In GNSS resilience scenario 2, if uncertainty range is assumed as 2km, the maximum one-way differential delay is 11.5us for LEO-600/1200, 13us for GSO; if uncertainty range is assumed as 5km, the maximum one-way differential delay is 28.9us for LEO-600/1200, 32.5us for GSO. Observation 2: - Regarding the one-way differential doppler considering uncertain UE location, - For FR1-NTN, - In GNSS resilience scenario 2, if uncertainty range is assumed as 2km, the maximum one-way differential doppler is 0.34kHz for LEO-600, 0.17kHz for LEO-1200, 0.002kHz for GSO; if uncertainty range is assumed as 5km, the maximum one-way differential doppler is 0.84kHz for LEO-600, 0.42kHz for LEO-1200, 0.006kHz for GSO. - For FR2-NTN, - In GNSS resilience scenario 2, if uncertainty range is assumed as 2km, the maximum one-way differential doppler is 5kHz for LEO-600, 2.5kHz for LEO-1200, 0.03kHz for GSO; if uncertainty range is assumed as 5km, the maximum one-way differential doppler is 12.7kHz for LEO-600, 6.3kHz for LEO-1200, 0.08kHz for GSO. |

## Initial proposal

### Proposal 2-2

This is a preliminary observation based the results reported from different companies:

Proposal 2-2-v0:

Observation

Based on evaluation results collected from multiple sources, the following ranges have been observed for differential delay and Doppler:

| Orbit | Differential Delay (one-way, typical max) | Differential Doppler (one-way, typical max) |
| --- | --- | --- |
| LEO-600 | 143–255 µs | 2.1 –3.76 ppm |
| LEO-1200 | 257–535 µs | 1.8–3.94 ppm |
| GEO/GSO | 810–1461 µs | 0.02–0.08 ppm |

Companies are encouraged to comment on Proposal 2-2-v0

| Company | Comments |
| --- | --- |
| OPPO | We prefer to focus on proposal 2-3 only to determine the differential delay and doppler case by case. |
| Spreadtrum | It seems that proposal 2-2 can be included in proposal 2-3. We prefer to focus on proposal 2-3 which is more detailed. |
| CEWiT | Support |
| Samsung | Because the value range is so wide, it is better to make observations separately for each scenario and assumption. In this format, only the maximum value ends up remaining, so the range information does not seem to have any real meaning. |
| ST Engineering iDirect | We agree to include a summary of the differential delay and Doppler values that need to be considered, in addition to the tables in Proposal 2-3 and 2-4. It is suggested to explicitly mention the (worst case) scenarios for which these numbers apply (lowest elevation for delay, nadir for Doppler, with parameter set 1 for the lower and set 2 for the upper value). |
| JHUAPL | At lower elevation angles the beam size will be elongated resulting in higher differential delay values. |
| Tejas Networks | The scenarios and beam diameters that are considered needs to be specified for clarity and the elevation angle considered for obtaining the ranges of delay and doppler can also be specified. Range can be specified in a table format similar to proposal 2-3 which is case by case and can be done after converging on the average values specified by different companies |
| Huawei, HiSilicon | Proposal 2-2 can be concluded from proposal 2-3. We should discuss proposal 2-3 first. Moreover, the range in proposal here contains both set1 and set2, we should not mix the values of set1 and set2, as there will be only one deployment for one satellite |
| TCL | We shaer the same opinion with the most companies. We can discuss proposal 2-3 first. |
| Toyota ITC | Agree with other companies that we should capture observations for different scenarios. Also, the values of case b are not realistic. Given the large number of cases (see for example FL spread sheet), observations should be captured. |
| Ericsson | Support with a few comments: - Clarify that this is for Case a (uncertainty area is the beam footprint). - The Doppler values for GEO/GSO need to be reviewed. Are the non-zero Doppler due to UE velocity, satellite inclination angle (i.e., GSO), or lack of Earh rotation? What UE speed/inclination angle do the values correspond to? |
| LGE | This proposal seems not necessary because proposals 2-3 and 2-4 cover more detailed observations. Moreover, it is difficult to understand the meaning of the above observation since the results are not separated for each case (e.g., case a, case b with X = {1, 5, 10, 25} km). |
| Panasonic | Intuitively these ranges seem to make sense, but how are they derived? What does “typical max” mean? Why is typical max Doppler of LEO1200 larger than LEO600? |
| Lenovo | Support |
| Nokia | It would be nice to have a separation of the results such that we can see which part of the results is coming from the propagation conditions, and which part of the results is coming from the “company variability”. After all, “company variability” should be close to zero, as we are talking about physics and modelling of the propagation of signals, which again is evaluated against Alt1 and Alt2 (which basically consists of a set of equations). |
| ZTE | The results are only for case a. The evaluation results for case b should also be listed in observation if such observation will be drawn. |
| ETRI | We prefer to discuss proposal 2-3 first. |
| Futurewei | Recommend to present and discuss the above table after Proposal 2-3 and Proposal 2-4 are agreed. |

### Proposal 2-3

Proposal 2-3-v0:

Capture in the TR 38.742 the following tables of differential one-way delay reflecting averaged results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1:  within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.020 |  |  | 357.371 |
|  | 30 | 143.215 | 257.658 |  | 57.620 | 115.067 |  |
|  | 90 | 3.132 | 5.573 | 8.122 | 0.438 | 1.024 | 1.426 |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1512.776 |  |  | 909.097 |
|  | 30 | 256.722 | 537.644 |  | 143.301 | 257.586 |  |
|  | 90 | 10.153 | 24.720 | 25.881 | 3.221 | 5.489 | 10.049 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 809.862 |  |  | 357.237 |
|  | 30 | 143.037 | 257.788 |  | 57.652 | 114.992 |  |
|  | 90 | 1.933 | 3.228 |  | 0.303 | 0.674 |  |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1461.267 |  |  | 906.427 |
|  | 30 | 255.222 | 535.176 |  | 143.037 | 257.038 |  |
|  | 90 | 6.100 | 14.927 |  | 1.933 | 3.228 |  |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.355 | 32.468 | 64.998 | 162.803 |
|  | 30 | 5.619 | 28.787 | 57.579 | 143.324 | 5.707 | 28.806 | 57.639 | 143.630 |  |  |  |  |
|  | 90 | 0.007 | 0.087 | 0.347 | 2.171 | 0.002 | 0.042 | 0.189 | 1.179 | 0.001 | 0.009 | 0.035 | 0.220 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.511 | 32.534 | 65.094 | 162.760 |
|  | 30 | 5.770 | 28.804 | 57.505 | 142.963 | 5.770 | 28.816 | 57.587 | 143.284 |  |  |  |  |
|  | 90 | 0.003 | 0.076 | 0.304 | 1.900 | 0.002 | 0.041 | 0.165 | 1.031 | 0.000 | 0.008 | 0.031 | 0.193 |

Companies are encouraged to comment on Proposal 2-3-v0

| Company | Comments |
| --- | --- |
| CMCC2 | We are fine with the proposal. |
| OPPO | We note that most calculation results are well aligned among companies, except the one-way differential delay within the nadir beam. Take LEO600 set 1 as an example, 8 companies provide the value of 1.9us but 4 companies provide the value of 7-7.6us for the one-way differential delay at the elevation angle of 90°. We can firstly align our results before capture in the TR. |
| Spreadtrum | For the non-nadir beam (30° for LEO and 12.5° for GEO), we think most companies are aligned. For the nadir beam, the evaluation results seem to have a little divergence. |
| CEWiT | Support |
| Samsung | It should be discussed later after collecting all inputs from other companies. |
| JHUAPL | At lower elevation angles the beam size will be elongated. This will result in higher differential delay values. |
| Tejas Networks | We are fine with the proposal and table structure. We note that for non-nadir beams most of the companies are aligned in the differential delay numbers. For nadir beams there are some results which are deviating more from the numbers which majority of the companies are proposing . We should converge on the Nadir beam numbers. |
| Huawei, HiSilicon | Should not average all the values, theree is still a big difference between different companies’ values. We recommend to average similar values and provide several candidate average values for further study. |
| Toyota ITC | Given that we evaluate the largest differential Doppler/delay, separating case a and b does not bring any value and overly complicate the results/presentation. Propose to prioritize the discussion on Proposal 2-1 (spreadsheet). Please see the detailed comment in Proposal 2-1. |
| Ericsson | We think we first should try to align values between companies in the spreadsheet and then come back to agreeing on values for the TR. |
| LGE | We are fine with the direction. The values need to be updated after the results are collected. |
| Panasonic | We are also fine with the non-nadir results. The nadir results need double-checking. (Please change “diamter” to “diameter”.) |
| Lenovo | Support |
| Nokia | As stated in the previous comment, there should be no need for “average” here. Hopefully companies have applied the equations for Alt1 and Alt2 correctly. |
| ZTE | Fine with the intention, but some values are not aligned. For example, the delays should be same between case-1 and case-2. |
| ETRI | We are fine. |
| Futurewei | Some values for elevation angle = 90° are missing in the table (Case a: the location uncertainty area is the area served by the cell or beam ¦ case 2: at 90° with respect to orbital plane) for GEO. Further clarifications are needed for the differential one-way delay calculation of elevation angle = 90° for case 1: within the orbital plane. |
| Qualcomm | Why are the values of differential delay different for along and cross-track? Propose to revisit later. |

### Proposal 2-4

Proposal 2-4-v0:

Capture in the TR 38.742 the following tables of differential one-way Doppler reflecting averaged results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1:  within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.007 |  |  | 0.003 |
|  | 30 | 0.380 | 0.399 |  | 0.139 | 0.157 |  |
|  | 90 | 2.101 | 1.832 | 0.029 | 0.841 | 0.731 | 0.013 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.014 |  |  | 0.008 |
|  | 30 | 0.712 | 0.887 |  | 0.379 | 0.395 |  |
|  | 90 | 3.769 | 3.821 | 0.066 | 2.100 | 1.817 | 0.041 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.044 |  |  | 0.023 |
|  | 30 | 1.225 | 1.134 |  | 0.506 | 0.518 |  |
|  | 90 | 2.096 | 1.207 | 0.047 | 0.838 | 0.800 | 0.021 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.074 |  |  | 0.049 |
|  | 30 | 2.206 | 2.400 |  | 1.227 | 1.134 |  |
|  | 90 | 3.769 | 2.546 | 0.086 | 2.097 | 1.801 | 0.053 |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.000 | 0.000 | 0.001 | 0.001 |
|  | 30 | 0.014 | 0.069 | 0.140 | 0.359 | 0.008 | 0.040 | 0.081 | 0.204 |  |  |  |  |
|  | 90 | 0.079 | 0.400 | 0.757 | 1.998 | 0.038 | 0.192 | 0.385 | 0.962 | 0.000 | 0.001 | 0.003 | 0.007 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.001 | 0.002 | 0.003 | 0.007 |
|  | 30 | 0.047 | 0.234 | 0.471 | 1.185 | 0.025 | 0.122 | 0.244 | 0.610 |  |  |  |  |
|  | 90 | 0.074 | 0.417 | 0.834 | 2.092 | 0.035 | 0.200 | 0.400 | 1.004 |  |  |  |  |

Companies are encouraged to comment on Proposal 2-4-v0

| Company | Comments |
| --- | --- |
| CMCC2 | We are fine with the proposal. |
| OPPO | The one-way differential doppler of case 2 for LEO1200@2GHz seems incorrect, where the average value is 1.81ppm for LEO1200 set1 and 3.8ppm for LEO1200 set2. |
| CEWiT | Support |
| Samsung | It should be discussed later after collecting all inputs from other companies. |
| JHUAPL | This is fine. |
| Tejas Networks | We are fine with the proposal |
| Huawei, HiSilicon | Same comment as previous proposal. |
| Toyota ITC | Given that we evaluate the largest differential Doppler/delay, separating case a and b does not bring any value and overly complicate the results/presentation. Propose to prioritize the discussion on Proposal 2-1 (spreadsheet). Please see the detailed comment in Proposal 2-1. |
| Ericsson | Same comment as to proposal 2-3. |
| LGE | We are fine with the direction. The values need to be updated after the results are collected. |
| Panasonic | Support |
| Nokia | As stated in the previous comment, there should be no need for “average” here. Hopefully companies have applied the equations for Alt1 and Alt2 correctly. |
| ZTE | Fine with the intention, but some values are not aligned. For example, the dopplers at 90 degree elevation should be same between case-1 and case-2. |
| ETRI | We are fine, but it’s better to clarify the UE speeds. |
| Futurewei | Carrier frequencies are missing in the last two tables. |
| Qualcomm | Same comment as above. |

# Topic#3 Timing and Doppler tolerances for PRACH

## Background

Previous meeting agreement:

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

## Companies’ proposals

| Companies | Proposals and observations |
| --- | --- |
| Eutelsat | Error: Reference source not found Error: Reference source not found Error: Reference source not found Error: Reference source not found Error: Reference source not found |
| CMCC | Proposal 6: For PRACH performance evaluation, the scaling factor can be assumed as 2 in the criteria for tolerable frequency shift of existing PRACH preamble formats. |
| Huawei | Proposal 1: For the analytical evaluation of existing PRACH preamble formats, the timing error Te_NTN can be used. |
| CATT | Observation 5: Preamble Format 0/1/2/3 with long sequence (839) has a higher tolerance for timing errors. Among them, Format 1 has the strongest tolerance for timing errors, reaching up to 682.49us for unrestricted set. Proposal 3: The timing error limit of UE specificed in TS 38.133 can be directly used for the PRACH performance evaluation. Proposal 4: It is reasonable to set the value of the scaling factor to 2 for Doppler shift. |
| Tejas Network | Proposal 1 : Aligned to the frequency of DL signal, the UL frequency misalignment at the BS side is double the residual Doppler shift. The scaling factor for one-way differential Doppler/frequency offset for UL performance evaluation is 2 . Observation 5 : The maximum supported two-way differential delay for PRACH is 684.38 us for long format 1 but only with a maximum doppler support of 3.125 khz with restricted type B. For long formats, maximum supported doppler is 12.5 khz with restricted type B for long format 3. Observation 6 : For short formats , the doppler support is typically half the SCS and limiting differential two-way delay is up to 66.56 us (format C2,15 khz SCS) |
| Ericsson | Observation 10 The agreed formula for the limit on differential roundtrip delay should be modified to consider the case that no cyclic shifts are configured for unrestricted sets. |
| ZTE | Proposal 7: The additional timing error is assumed as Te = 29*64*Tc for S band and Te = 13*64*Tc for Ka band. Proposal 8: The scaling factor 2 should be considered for the scaled one-way differential Doppler/frequency offset. Proposal 9: The largest differential RTT and largest scaled differential Doppler among all elevation angles should be considered for PRACH tolerance evaluation. |
| MediaTek | Observation 6: For GNSS position error due to GNSS not available or not accurate, there are two interpretations for the term “Te” in the RACH tolerance formula agreed in the RAN1#122bis: - Interpretation 1 for Te: The GNSS position error is included in the Te. in the left side of the RACH tolerance formula. The GNSS error may be much larger than the RAN4 assumption in Rel-17 (i.e. GNSS position error >> 50 m). - Interpretation 2 for Te: The GNSS position error is included in the differential RTT in the right side of the RACH tolerance formula. The timing error Te, is as defined in RAN4 in TS 38.133 Section 7.1C. The preamble format should allow for a timing error which is the sum of the differential delay due to the beam size and the differential delay due to the GNSS position error range. Proposal 2: The GNSS position error is included in the Te. in the left side of the RACH tolerance formula for differential delay. |
| Qualcomm | Observation 2: For PRACH performance evaluation, the following should be considered as baseline for additional timing errors: - Timing error: Te should be set according to RAN4 requirements in [3, TS 38.133, Table 7.1.2-1] (and not according to Te_NTN in [3, TS 38.133, Table 7.1C.2-1]). Observation 3: The frequency error tolerance of a PRACH format should be: (scaling factor*(Differential Doppler) +2fe×fc) ≥ 1+γΔfRA, where the scaling factor =2. The LHS in the above inequality is the differential received uplink frequency at the gNB within the location uncertainty area. Observation 4: The condition for timing error tolerance of a PRACH format agreed in RAN1-122bis does not consider the fact that NCS can be 0 under certain PRACH configuration. In that case, maxNCSLRA should be replaced by 1. Proposal 3: Revise the agreement from RAN1#122bis as follows. For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria: - PRACH tolerance is exceeded if - Differential RTT+2Te≥ min (NCP, Sequence duration× Scaling Factor), where; - NCP is cyclic prefix duration - Scaling factor =1 if NCS can be set to zero, otherwise Scaling factor =maxNCSLRA according to Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211 - LRA is the ZC sequence length - Te is an additional timing error. - Companies to report how Te is derived. As baseline, Te is to be set according TS 38.133, Table 7.1.2-1. - Differential RTT is 2 time one-way delay. - Note: the delay spread is neglected - GP can be considered and reported for the case of consecutive RO. - Or (Differential Round Trip Doppler +2 fcfe) ≥ 1+γΔfRA - For restricted set A, γ=2 For restricted set B, γ=4. For non-restricted set, γ=0 - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively. - Othe values of γ can be used and reported with justification - fe=0.1 ppm - fc is the uplink carrier frequency Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset. The same scaling factor as per the previous agreement is to be reported Note: the above is applicable to single PRACH transmission. |
| LGE | Observation #1: For NR PRACH preamble Format 1 with non-restricted set, TO/FO tolerance are up to 399.52 μs and 0.43 ppm. Observation #2: For NR PRACH preamble Format 1 with restricted set Type A, TO/FO tolerance are up to 225.98 μs and 1.68 ppm. Observation #3: For NR PRACH preamble Format 1 with restricted set Type B, TO/FO tolerance are up to 130.63 μs and 2.93 ppm. Observation #4: For NR PRACH preamble Format C2, TO/FO tolerance are up to 4.14 μs and 4 ppm. |
| CEWiT | Observation 4: For successful UL transmission (e.g. PUSCH) in NTN, the maximum allowable timing error is 0.944μs. This indicates that NTN scenarios cannot be accommodated by reference-location-based TA determination. |

## Summary of companies’ contributions

Timing Error (Te):

Eutelsat and Thales provide detailed clarifications on the definitions of Te, noting differences in interpretation between RAN1 and RAN4, specifically, that Te_NTN as set by RAN4 includes round trip error Tp, whereas RAN1 subtracts it from the cyclic prefix duration. Eutelsat also observes that the Te_NTN values from RAN4 generally permit a nonzero guard time (except for 120 kHz UL SCS) and that typical downlink synchronization and oscillator errors are minor compared to UE location uncertainty without GNSS. In the case of PRACH Format 3, the guard period is even less than the CP, and ultimately, Eutelsat finds that 2×Te is negligible compared to the CP or guard period. Eutelsat requests that any liaison statements between RAN1 and RAN4 always clarify the specific Te definition and calculation used. Huawei proposes use of the Te_NTN value from RAN4 for all analytical PRACH performance evaluations. ZTE recommends using specific fixed additional timing error values for S band and Ka band. MediaTek discusses ambiguity in the handling of GNSS position error within the Te term, whether it should be treated as part of the additional timing error on the left side of the RACH tolerance formula or folded into the differential RTT term on the right. MediaTek suggests including GNSS uncertainty directly into Te when GNSS is unavailable or inaccurate, as real-world GNSS errors may far exceed RAN4's default assumptions. Qualcomm proposes a clarified criteria for PRACH timing and Doppler tolerance, specifying that Te should strictly follow the values in TS 38.133 and not other definitions.

| Company | Observations / Proposals regarding Timing Error (Te) |
| --- | --- |
| Eutelsat | - Te_NTN (RAN4) ≠ Te (RAN1 FLS); Te_NTN includes Tp, RAN1 Te subtracts it. |
|  | - RAN4’s Te_NTN permits nonzero guard time except 120kHz UL SCS. |
|  | - 2×Te negligible vs CP/Guard period. |
|  | - Requests clarity on Te definitions in liaisons. |
| Huawei | - For analytical evaluation, use Te_NTN as timing error metric. |
|  | - Proposes using UE timing error limit from TS 38.133 for evaluation. |
| ZTE | - Assumes Te = 29×64×Tc for S band, 13×64×Tc for Ka band in evaluations. |
| Thales | Te=Td+Ta+Tf+Tm, where - Td: UE downlink synchronization error, calculated as: 1/(2*BW) where BW is for the PBCH DMRS - Tr : Timing Advance Command (TAC) resolution error - Ta :TA Timing accuracy adjustment error - Tf : Accumulated timing drift - Tm : Additional impairments e.g. Channel/multipath delay spread. Thereby, a value of T_e=305 ns for SCS of SSB of 15 kHz T_e=49 ns for SCS of SSB of 120 kHz. Clearly, this T_e is very small compared to the limiting differential round trip delays |
| MediaTek | - Te may include GNSS error if unavailable/significantly inaccurate. |
|  | - Two Te interpretations: (1) GNSS error inside Te (preferred), (2) GNSS error added to RTT. |
|  | - Proposes including GNSS error in Te (left side of formula). |
| Qualcomm | - Te should be as per TS 38.133, Table 7.1.2-1 (not Te_NTN). |
|  | - Detailed formula for evaluating PRACH tolerance using additional timing error Te. |

Scaling factors for Doppler/Frequency Offset:

CMCC and CATT recommend using a scaling factor of 2 when considering tolerable Doppler shifts in PRACH evaluations, with CATT emphasizing that some preamble formats (especially Format 1 with the long sequence) have strong timing error tolerance. CATT further suggests directly adopting UE timing errors as specified in TS 38.133 for PRACH evaluation. Tejas Networks details specific Doppler and delay performance limits per format and SCS, proposing a scaling factor of 2 for one-way differential Doppler in UL and highlighting the close link between supported Doppler, SCS, and delay across PRACH formats.

Qualcomm breaks down the analytical evaluation, prescribing the scaling factor (2 by default), how to treat cyclic shifts, and how to select the relevant tolerances for different PRACH configurations. For Doppler, Qualcomm aligns on using the scaling factor in the calculation and provides detailed criteria for various PRACH set types, also noting oscillator tolerances and the impact on detection performance. Ericsson observed that the scaling factor must account for both DL and UL Doppler effects in the total frequency offset at the gNB.

| Company | Observations / Proposals on Scaling Factor for Doppler/Frequency Offset |
| --- | --- |
| Eutelsat | - Frequency errors due to 0.1 ppm UE oscillator are small in some formats/SCS. (No explicit scaling factor stated.) |
| CMCC | - Proposes scaling factor of 2 for tolerable frequency shift in PRACH performance evaluation. |
| Huawei | (No explicit statement on scaling factor for Doppler/frequency in inputs.) |
| CATT | - Proposes scaling factor of 2 is reasonable for Doppler shift in evaluation. |
| Tejas | - Proposes scaling factor for one-way differential Doppler/frequency offset for UL performance evaluation is 2. |
|  | - UL freq. misalignment at BS is double Doppler. |
| Ericsson | - If Differential Doppler is the one-way relative shift, the scaling factor should be the sum of DL and UL Doppler (scaling factor = f_(c,dl) + f_(c,ul)). If it is the one-way absolute DL Doppler shift, the scaling factor should be 1 plus the ratio of UL to DL Doppler (scaling factor = 1 + f_(c,ul)/f_(c,dl)). |
| ZTE | - Scaling factor of 2 should be considered for scaled one-way differential Doppler/frequency offset. |
|  | - Largest RTT and Doppler among elevation angles should be used. |
| MediaTek | (No explicit statement on scaling factor for Doppler/frequency in provided summary.) |
| Qualcomm | - Proposes analytical formula where scaling factor = 2 in Doppler/frequency offset evaluation. |
|  | - (scaling factor) × (Differential Doppler) + 2fₑf_c ≥ (1+γ)Δf_RA. |
| LGE | No explicit proposal on scaling factor; provides format-specific TO/FO tolerances. |
| CEWiT | No input on scaling factor for Doppler/frequency offset. |

## Initial proposal

### Proposal 3-1

From moderator perspective, the transmission error Te considered for PRACH transmission may be calculated as follows:

Te=Td+Ta+Tf+Tm, where

- Td:  UE downlink synchronization error, calculated as: 1/(2*BW) where BW is for the PBCH DMRS

- Tr : Timing Advance Command (TAC) resolution error

- Ta :TA Timing accuracy adjustment error

- Tf : Accumulated timing drift

- Tm : Additional impairments e.g. Channel/multipath delay spread.

| Td(ns) | 1/(2*BW) where BW is for the PBCH DMRS | 138,89 | 138,89 | 17,36 |
| --- | --- | --- | --- | --- |
| Tr (ns) | 16∙(64∙T_c)⁄(2*2^μ) | 260,42 | 130,21 | 32,55 |
| Ta (ns) |  | 130,21 | 130,21 | 16,28 |
| Tf (ns) | 0.1 /1,000,000 * 160 ms | 16,00 | 16,00 | 16,00 |
| Tm (ns) | Channel/multipath delay spread | 20,00 | 20,00 | 0,00 |

Thereby, a value of Te=305ns for SCS of SSB of 15 kHz Te=49ns  for SCS of SSB of 120 kHz. Clearly, this Te is very small compared to the limiting differential round trip delays.

As a possible way forward, the moderator proposes to consider the Differential roundtrip delay/Doppler limits provided in [R1-2508843, Ericsson] as working assumptions that could be confirmed during this meeting.

Proposal 3-1-v0:

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 398,74 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 99,10 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9,38 | 69 | 0,39 | 66,67 | 8,59 |
| A1 | 30 | 139 | 4,69 | 69 | 0,33 | 33,33 | 4,04 |
| A1 | 60 | 139 | 2,34 | 69 | 0,11 | 16,67 | 2,12 |
| A1 | 120 | 139 | 1,17 | 69 | 0,11 | 8,33 | 0,94 |
| A2 | 15 | 139 | 18,75 | 69 | 0,39 | 66,67 | 17,97 |
| A2 | 30 | 139 | 9,38 | 69 | 0,33 | 33,33 | 8,72 |
| A2 | 60 | 139 | 4,69 | 69 | 0,11 | 16,67 | 4,46 |
| A2 | 120 | 139 | 2,34 | 69 | 0,11 | 8,33 | 2,12 |
| A3 | 15 | 139 | 28,13 | 69 | 0,39 | 66,67 | 27,34 |
| A3 | 30 | 139 | 14,06 | 69 | 0,33 | 33,33 | 13,41 |
| A3 | 60 | 139 | 7,03 | 69 | 0,11 | 16,67 | 6,80 |
| A3 | 120 | 139 | 3,52 | 69 | 0,11 | 8,33 | 3,29 |
| B1 | 15 | 139 | 7,03 | 69 | 0,39 | 66,67 | 6,25 |
| B1 | 30 | 139 | 3,52 | 69 | 0,33 | 33,33 | 2,86 |
| B1 | 60 | 139 | 1,76 | 69 | 0,11 | 16,67 | 1,53 |
| B1 | 120 | 139 | 0,88 | 69 | 0,11 | 8,33 | 0,65 |
| B2 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B2 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B2 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B2 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| B3 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B3 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B3 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B3 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| C0 | 15 | 139 | 16,41 | 69 | 0,39 | 66,67 | 15,63 |
| C0 | 30 | 139 | 8,20 | 69 | 0,33 | 33,33 | 7,55 |
| C0 | 60 | 139 | 4,10 | 69 | 0,11 | 16,67 | 3,87 |
| C0 | 120 | 139 | 2,05 | 69 | 0,11 | 8,33 | 1,82 |
| C2 | 15 | 139 | 66,67 | 69 | 0,39 | 66,67 | 32,31 |
| C2 | 30 | 139 | 33,33 | 69 | 0,33 | 33,33 | 15,90 |
| C2 | 60 | 139 | 16,67 | 69 | 0,11 | 16,67 | 8,05 |
| C2 | 120 | 139 | 8,33 | 69 | 0,11 | 8,33 | 3,91 |

Note: Te in TS 38.133 clause 7.1 for TN is considered in the PRACH evaluation.

Companies are encouraged to comment on Proposal 3-1-v0

| Company | Comments |
| --- | --- |
| CMCC2 | Regarding the additional timing error Te, from our perspective, the timing error requirements for NTN are derived by taking the GNSS positioning error, error of UE determining the satellite location and extra margin into consideration, resulting in larger values compared with the timing error limits for TN. since the case that GNSS information is temporarily unavailable is studied in Rel-20 GNSS resilience operation, at least the relaxation of GNSS positioning error requirements will not be considered for determining Te in the criteria. Therefore, the timing error limit for TN in Table 7.1.2-1 of TS38.133 can be used for calculating the tolerable timing error. We are generally fine with the working assumption tables except the values for PRACH format C0, the CP length are respectively 40.36µs, 20.18µs, 10.09µs, and 5.05µs for SCS=15kHz, 30kHz, 60kHz, 120kHz, leading to the differential roundtrip delay limits of 32.31µs, 15,90µs, 8,05µs, 3,91µs. |
| Samsung | Okay |
| Tejas Networks | Support. |
| Huawei, HiSilicon | For unrestricted set, we should use NCS=0 to maximize the round-trip delay limit, i.e. to be comparable to the length of CP. Note that the equation for PRACH tolerance is not applicable for NCS=0. Seems the CP length of B3 and C0 is not correct in the table. |
| Toyota ITC | Propose to collect PRACH tolerance evaluation results first (e.g., a separate spreadsheet). |
| Ericsson | As the source of the tables above, we are of course fine with this WA. The Te values used are the ones from TS 38.133, Table 7.1.2-1 for the respective SCS. The calculations follow the agreed formula. In our contribution, we make the following observation (which has also been observed by other companies): Observation 10 The agreed formula for the limit on differential roundtrip delay should be modified to consider the case that no cyclic shifts are configured for unrestricted sets. If the equation is corrected, it would be min (Ncp, Sequence duration) -2Te ≤ Differential RTT for unrestricted sets. The following rows in the tables would be impacted by this: 1; unrestricted; 839; 684,38; 419; 0,39; 800; 398,74683,59. 3; unrestricted; 839; 103,13; 419; 0,39; 200; 99,10102,34. C2; 60; 139; 16,67; 69; 0,11; 16,67; 8,0516,44. |
| LGE | Since TO/FO tolerance for the PRACH format has a clearly agreed-upon formula, we think that a separate table is not necessary. The parameters left as FFS points in the previous agreement should be discussed. For example, how to calculate Te and/or scaling factor should be discussed first. |
| Panasonic | Fine for us. |
| Nokia | OK as a starting point, but we would like to highlight/emphasize that the “Differential roundtrip delay limit [µs]” is an absolute limit, and may be smaller in reality (RAN4 requirements), gNB implementation, etc. |
| Sony | Agree with Nokia. |
| ZTE | For unrestricted set, the max Ncs should be L (i.e., sequence length) considering that the cyclic shift can be configured not used. Hence, the tolerable differential RTT should be determined based on Ncp instead of L/2. |
| Apple | Ok with this WA |
| Futurewei | For unrestricted long PRACH preamble formats, maxNcs=419is not used in the calculation because the cyclic shift is zero. Therefore, SequenceDuration×maxNcsL=SequenceDuration. Suggest adding a new column to the table for the following: min(Ncp,SequenceDuration×maxNcsL) |
| Qualcomm | For unrestricted set, max timing resolution can be achieved with N_cs = 0. |

### Proposal 3-2

Proposal 3-2-v0:

Working assumption

Differential one-way Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0,1 | 2 | 2 | 2,0 | 0,31 |
| 0,1,2 | type A | 1250 | 1 | 0,1 | 2 | 2 | 2,0 | 0,62 |
| 0,1,2 | type B | 1250 | 3 | 0,1 | 2 | 2 | 2,0 | 1,25 |
| 3 | unrestricted | 5000 | 0 | 0,1 | 2 | 2 | 2,0 | 1,25 |
| 3 | type A | 5000 | 1 | 0,1 | 2 | 2 | 2,0 | 2,50 |
| 3 | type B | 5000 | 3 | 0,1 | 2 | 2 | 2,0 | 5,00 |

Differential one-way Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0,1 | 2 | 2 | 2,0 | 3,75 |
|  | 30 | 30000 | 0 | 0,1 | 2 | 2 | 2,0 | 7,50 |
|  | 60 | 60000 | 0 | 0,1 | 30 | 20 | 2,5 | 0,80 |
|  | 120 | 120000 | 0 | 0,1 | 30 | 20 | 2,5 | 1,60 |

Companies are encouraged to comment on Proposal 3-2-v0

| Company | Comments |
| --- | --- |
| OPPO | We are fine with using the scaling factor of scaling factor = 1 + f_(c,ul)/f_(c,dl). However, we should be careful when comparing the differential one-way doppler and PRACH tolerance because the one-way differential doppler in the spreadsheet is calculated based on the f_(c,ul) = 30GHz instead of f_(c,dl)=20GHz. |
| Samsung | It is unclear why γ is used 1 and 3. In last RAN1 meeting, we agreed “For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0”. |
| Tejas Networks | Same view as Samsung “For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0” was agreed in RAN1#122_bis and other values can only be provided with proper justification |
| Huawei, HiSilicon | OK for S band. The scaling factor for Ka band is not clear, should be clarified. |
| Toyota ITC | Propose to collect PRACH tolerance evaluation results first (e.g., a separate spreadsheet). |
| Ericsson | The proposal is based on Ericsson's tables, but unfortunately we made a minor calculation error (missing conversion from GHz to Hz for fc,UL). The corrected tables should look like shown below. Regarding the setting of γ=1 and γ=3 for restricted sets A and B, this is to align with the note in the agreement from RAN1#122b "Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively". Preamble; Set; ΔfRA [Hz]; γ; fe [ppm]; fc,UL [GHz]; fc,DL [GHz]; scaling factor; Differential one-way Doppler limit [ppm]. 0,1,2; unrestricted; 1250; 0; 0,1; 2; 2; 2,0; 0,310,21. 0,1,2; type A; 1250; 1; 0,1; 2; 2; 2,0; 0,620,53. 0,1,2; type B; 1250; 3; 0,1; 2; 2; 2,0; 1,251,15. 3; unrestricted; 5000; 0; 0,1; 2; 2; 2,0; 1,251,15. 3; type A; 5000; 1; 0,1; 2; 2; 2,0; 2,502,40. 3; type B; 5000; 3; 0,1; 2; 2; 2,0; 5,004,90. Preamble format; SCS; ΔfRA [Hz]; γ; fe [ppm]; fc,UL [GHz]; fc,DL [GHz]; scaling factor; Differential one-way Doppler limit [ppm]. all short formats; 15; 15000; 0; 0,1; 2; 2; 2,0; 3,753,65. 30; 30000; 0; 0,1; 2; 2; 2,0; 7,507,40. 60; 60000; 0; 0,1; 30; 20; 2,5; 0,800,72. 120; 120000; 0; 0,1; 30; 20; 2,5; 1,601,52. |
| LGE | Since TO/FO tolerance for the PRACH format has a clearly agreed-upon formula, we think that a separate table is not necessary. The parameters left as FFS points in the previous agreement should be discussed. For example, how to calculate Te and/or scaling factor should be discussed first. |
| Panasonic | Support. The scaling factor here is 1+f_UL/f_DL, which implies that UE bases its UL frequency on the received DL frequency. The base station then has to deal with the combined Doppler from DL and UL. During the discussion at RAN1-122bis some companies suggested that UE can compensate for the DL Doppler, which would mean that it could compensate for UL Doppler as well, and there wouldn’t be a problem with Doppler at all. We have our doubts that this works. |
| Nokia | OK as a starting point, but we would like to highlight/emphasize that the “Differential one-way Doppler limit [ppm]” is an absolute limit, and may be smaller in reality (RAN4 requirements), gNB implementation, etc. To give an example, we have in our contribution R1-2509007 given examples of the PRACH detection performance for a few selected PRACH preamble formats, where performance degradation of up to 5 dB has been seen. So again and as also stated earlier,the “limit” is where things will definitely break, If not earlier – and not where things will still work. |
| Sony | Agreement needs to be made on the values of γ |
| ZTE | The calculated values seem not correct. For example, the tolerable one-way Doppler of restricted type B of 0,1,2 is (1250*5/2e9*1e6 - 0.1*2)/2 = 1.4625 ppm, which is larger than the shown value. The other results also need to be corrected. |
| Apple | Share the views with HW, for Ka/ku band, the γ need to be clarified. |
| Futurewei | The scaling factor = 2.5 needs further clarification because the differential one-way Doppler is computed using the uplink carrier frequency = 30 GHz and the downlink frequency = 20 GHz. |
| Qualcomm | Why is the scaling factor 2.5 for 30 GHz? |

# Topic#4 PRACH performance evaluation

## Background

RAN1#122bis made the following agreement.

Agreement:

For PRACH performance evaluation for existing PRACH preamble formats based on analytical characterization, adopt the following criteria:

- PRACH tolerance is exceeded if

  - min (Ncp, Sequence duration×maxNcsL) -2Te ≤ Differential RTT, where;

    - Ncp is cyclic prefix duration

    - max Ncs is given by Tables 6.3.3.1-5, 6.3.3.1-6 and 6.3.3.1-7 of TS 38.211

    - L is the ZC sequence length

    - Te is an additional timing error.

      - Companies to report how Te is derived.

    - Differential RTT is 2 time one-way delay.

    - Note: the delay spread is neglected

    - GP can be considered and reported for the case of consecutive RO.

  - Or  (scaling factor*(Differential Doppler) +2* fe×fc) ≥ 1+γΔfRA

    - For restricted set A, γ=2. For restricted set B, γ=4. For non-restricted set, γ=0

      - Note: In practice, the detection performance degrades at frequency offsets exceeding 1 SCS and 2 SCS with restricted set A and B, respectively.

      - Othe values of γ can be used and reported with justification

    - fe=0.1 ppm

    - fc is the uplink carrier frequency

    - Note: The scaling factor*Differential Doppler is equal to the UL differential Doppler/frequency offset.

    - The same scaling factor as per the previous agreement is to be reported

Note: the above is applicable to single PRACH transmission.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| FUTUREWEI | Proposal 1: Capture the PRACH format evaluation results in R1-2508333 (in Tables 21 and 22) for FR1 and FR2 in TR 38.742. |
| Spreadtrum | Observation 1 For set-1 S-band LEO-600, long PRACH format 1 can cover maximum differential RTT in most situations but can’t cover maximum differential round RTD. Observation 2 For set-1 S-band LEO-1200, long PRACH format 1 can’t cover maximum differential RTT and maximum differential round RTD in most situation. Observation 3 For set-1 and set 2 S-band GEO, long PRACH format 1 can’t cover maximum differential RTT but can cover maximum differential RTD. Observation 4 For set-2 S-band LEO-600 and LEO-1200, long PRACH format 1 can’t cover maximum differential RTT and maximum differential RTD in most situations. Observation 5 For set-1 S-band LEO-600, LEO-1200 and GEO, short PRACH format can cover maximum differential RTD in most situations. Observation 6 For set-1 S-band LEO-600, LEO-1200 and GEO, short PRACH format can’t cover maximum differential RTT but can cover maximum differential RTD in most situations. Observation 7 For set-2 S-band LEO-600 and LEO-1200, short PRACH format can’t cover maximum differential RTT and maximum differential RTD in most situations. |
| vivo | Observation 12: In FR 2, the existing PRACH formats cannot cope with the maximum timing error and frequency error with a cell-specific common timing and frequency pre-compensation, based on current RAN4 requirements. |
| CMCC | Observation 2: Without GNSS facilitating the timing and frequency pre-compensation, under the beam footprint size-level positioning error, larger CP and subcarrier spacing are required at satellite or gNB to receive preamble correctly. Observation 3: Under current assumption of beam footprint size, the current CP design in NR cannot support the LEO-1200 and GEO in S-band. Observation 4: Current short format preamble cannot be used in the LEO-600, LEO-1200 and GEO in Ka-band, if the positioning uncertainty area is in a beam footprint size level. Proposal 8: The tolerable positioning error under current PRACH format can be studied to determine the radius of the location uncertainty area. |
| Thales | Observation 6: PRACH Format 1 reliably meets delay tolerances for narrow-uncertainty area (≤10 km), regardless of orbit or elevation, supporting flexible and robust random access. For moderately larger beams (around 25 km at low elevation), compliance is possible with permissive ZCZ settings, but may be violated under stricter conditions. Wide-beam scenarios consistently exceed PRACH Format 1 delay limits, indicating a fundamental barrier to its use in these cases. Observation 7: The analysis shows that PRACH compliance with round-trip delay tolerances is reliably achieved only for satellite scenarios with reduced uncertainty area size (≤10 km diameter) across all orbits and elevation angles. Wide-beam deployments (such as in satellite parameter sets 1 and 2), in GEO and LEO, routinely exceed PRACH delay limits for long formats, making them impractical. To enable GNSS-resilient NR NTN operation, solutions should be considered that ensure PRACH compliance with round-trip delay delay differences observed in common deployment scenarios. Observation 8: In the non-restricted set (γ=0), all long PRACH formats, except for GEO in S band and Format 3 (potentially usable for GEO in Ka band), exceed Doppler tolerance limits, indicating limited applicability. For restricted set A (γ=2), PRACH formats 0, 1, and 2 remain within tolerance for GEO in both S and Ka bands when the composite Doppler effect (2× Differential Doppler + 2× fe×fc) is less than 3.75 kHz. PRACH format 3 can tolerate higher Doppler values (up to 15 kHz), making it suitable for all LEO satellites and GEO in Ka band. Similarly, with more stringent restrictions in set B (γ=4), formats 0, 1, and 2 are viable for GEO in both bands if the Doppler effect is below 6.25 kHz, while format 3 supports even higher limits (up to 25 kHz), enabling deployment for all LEO satellites and GEO in Ka band. This demonstrates that restricted sets enhance PRACH tolerance to Doppler, broadening the range of supported satellite and frequency band scenarios, especially for format 3. Observation 9: In the absence of a GNSS fix, short PRACH preambles are resilient to differential frequency offsets/Doppler. Observation 10: In the absence of a GNSS fix, the maximal differential delay encountered is beyond the detection capability supported by current NR PRACH short formats. Therefore, potential solutions should be discussed to address this issue. |
| Huawei | Proposal 3: For UL performance evaluation, two times differential one-way Doppler/frequency offset should be used. Observation 5: For the edge beam in scenario 1, none of existing preamble formats can work in FR1-NTN and FR2-NTN. Although the long preamble format can tolerate the large differential delay for some cases in FR1-NTN, the differential Doppler exceeds the tolerance. Short preamble formats can tolerate the large differential Doppler but cannot handle the differential delay in FR1-NTN and FR2-NTN. Observation 6: For the nadir beam in scenario 1, preamble format 3 and all short formats can work in FR1-NTN, and most short preamble formats can work in FR2-NTN. |
| CATT | Observation 6 The applicability of the existing PRACH formats in scenario 1 is shown in Table 4 [R1-2508594]. Under the conditions of Scenario 1, none of the existing PRACH formats can tolerate both timing and frequency offset errors simultaneously. Observation 7 The applicability of the existing PRACH formats in scenario 2 is shown in Table 5. In the Ka band, no other format can simultaneously tolerate timing and frequency offset errors except for format C2 for LEO-600. Observation 8 In the S/C band, when the radius of the uncertain region is 5km in scenario 2, format 0(setA, setB)/1(setA, setB)/2(setA, setB)/3(non, setA)/ C2 can simultaneously tolerate timing and frequency offset errors. When the radius of the uncertain region increases to 10km, only format 1(setB)/2(setB) can simultaneously tolerate timing and frequency offset errors. |
| Xiaomi | Observation 1. NR defined PRACH formats cannot cover the differential delay and Doppler requirements defined in Table 2 [R1-2508681]. |
| Tejas Network | Observation 7: For S band GEO scenarios and LEO 1200km Set 2 scenario , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats. Observation 8: For S band LEO 600km(Set 1 and Set 2 ) and LEO 1200km (Set 1) scenarios , the maximum differential delay scenarios can only be handled by the PRACH long format 1 but only with a doppler support of maximum 3.125 khz which is insufficient for nadir scenarios. Observation 9: For S band LEO scenarios , the maximum residual doppler for cases can be supported using short formats(suitable SCS) and Long Format 3 with restricted Sets . But the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams. Observation 10: For Ka band LEO 600 km, LEO 1200 scenarios , the maximum differential delay scenarios can be handled by the PRACH long format 1 for Set 1 satellite configuration. The maximum residual doppler is supported by short formats C0/C2 with SCS 60khz or higher but the short formats have very less limiting differential two-way delay value hence may not be feasible for beams other than nadir beams. Observation 11 : The maximum two-way differential delay values supported by short formats is 66.6 microseconds (C2) which can support only a GNSS uncertainty area upto 5-6 km approximately even with good resilience to doppler shifts Observation 12: For S band scenarios with elongating beam diameters , maximum differential delays clearly exceed the PRACH limiting differential two-way delay for all PRACH formats. Observation 13: Although several PRACH formats can support delay and doppler separately for some of the cases but they do not support large values of delay and doppler simultaneously which is the case for many NTN scenarios. Consequently, the detection capabilities of the existing PRACH formats are inadequate for NTN scenarios that operate without GNSS assistance. Proposal 3: Enhancements are required in the existing PRACH and NR Initial Access procedures to support the large residual dopplers and differential delays in various NTN scenarios without GNSS availability. |
| Samsung | Observation 9: The existing PRACH format 1 with Restricted Set A can cover all scenarios except for GSO @2GHz if the reference point is applied. Observation 10: The existing PRACH format 1 with Restricted Set B can cover all scenarios except for GSO @2&14GHz if the reference point is applied. Observation 11: The existing PRACH format 1 with Restricted Set B can cover GSO @14GHz if the reference point is applied. Proposal 1: RAN1 may need to consider PRACH enhancement only for GSO @2GHz due to insufficient delay tolerance requirement. For other scenarios, RAN1 does not need to consider PRACH enhancement when the reference location is used, as the current PRACH format can accommodate the delay and Doppler tolerances of GNSS-resilient UEs. |
| Ericsson | Observation 11 The magnitude of maximum residual Doppler shift, even the current values obtained under optimistic assumptions, seems challenging to be handled by the existing PRACH. The large timing uncertainty compounds the challenge even further. Observation 12 The CP duration with long PRACH formats is about 2 to 14 times shorter than the residual delay value for LEO. The maximum CP duration in short PRACH formats is about 20 times shorter than the maximum residual delay for LEO. This implies that the PRACH receiver will need to apply multiple timing hypotheses within the PRACH receive window to detect the preamble, which will increase the receiver complexity, especially for short formats with a small CP duration. Observation 13 Without GNSS-based UL pre-compensation at initial access, the differential delay and Doppler shift values at the PRACH receiver will be much larger than that in legacy PRACH transmission with GNSS-based UL pre-compensation. Observation 18 Solution 1, Solution 2 and Solution 3 are not mutually exclusive and should all be considered to improve robustness of PRACH preamble, reduce time/frequency uncertainty, and detect preamble despite large residual delay. |
| ZTE | Observation 4: No long PRACH format can handle the differential delay and Doppler in LEO-600 S-band, LEO-1200 S-band and GSO S-band in case a, where the location uncertainty is the area served by beam. Observation 5: For case b, long PRACH format can handle the differential delay and Doppler in LEO-600 S-band and LEO-1200 S-band with X=10km, and in GSO S-band with X=25km. Observation 6: No short PRACH format can handle the differential delay and Doppler in any evaluated scenario in case a, where the location uncertainty is the area served by beam. Observation 7: For case b S-band, short PRACH format can handle the differential delay and Doppler in LEO-600, LEO-1200 and GSO with X=5km. For case b Ka-band, short PRACH format can handle the differential delay and Doppler in LEO-600, LEO-1200 and GSO with X=1km Observation 8: For case a (the location uncertainty area is the area served by the cell or beam), existing PRACH format cannot handle the differential delay and Doppler for all evaluated cases. Observation 9: For case b (the location uncertainty area is a circle of radius X km), existing PRACH format can handle the differential delay and Doppler for moderate X values, e.g., X=10km for S-band and X=1km for Ka-band. |
| Amazon | Observation 1: When the radius of uncertainty area is 25 km, the existing PRACH formats are not able to simultaneously tolerate the expected range of differential delay and differential Doppler, even in low-speed scenarios. Observation 2: When the radius of uncertainty area is 10 km, the existing PRACH formats can support low-speed scenarios, but fail to support high-speed scenarios. Observation 3: Considering the expected range of differential delay, unrestricted sets with Ncs= 0 are most appropriate for GNSS-resilient operation. |
| Nokia | Observation 1: For cases where UE location uncertainty is defined within a cell area for cell diameter at 50 km and satellite at LEO-600, existing 5G NR random access preambles will not be able to meet requirements for maximum differential Doppler and maximum differential RTT. Observation 2: PRACH detection performance will suffer a substantial loss of up to 4 dB, even for cases where the Doppler offset is less than half the SCS. |
| Apple | Observation 1: For scenario 1 with Case A, existing PRACH formats can’t tolerate both differential RTT and differential round trip Doppler at all evaluated elevation angles. Observation 2: For scenario 2 with Case B, supported uncertain area is different for different PRACH format with or without restricted sets. |
| TOYOTA ITC | Observation 1: For some scenarios, the maximum one-way differential delay and Doppler are beyond the current PRACH tolerance limits. |
| MediaTek | Observation 7: The RACH tolerance to difference delay for long RACH preambles can allow GNSS position error range of 1650 m with RACH preamble format 3 with SCS = 5 kHz; or 6580m with RACH preamble formats 0,1, 2 with SCS = 1.25 kHz. Observation 8: The RACH tolerance to difference delay for short RACH preambles can allow GNSS position error range of up to 3310 m with RACH preamble format A3, B4, C0, C2 with SCS = 15 kHz. Observation 9: The RACH tolerance to difference Doppler shift can allow beam diameters of up to 90 km @ 2 GHz (S Band) at LEO-600 km with long preamble format 3 with SCS = 5 kHz. Observation 10: The RACH tolerance to difference Doppler shift can allow beam diameters of up to 190 km @ 14 GHz (Ku band); and 90 km @ 30 GHz (S band) at LEO-1200 km with short preamble with SCS = 15 kHz. Larger beam diameter up to 280 km can also be supported in GEO. Observation 11: The RACH tolerance to differential delay is a more significant factor for GNSS position error compared to differential Doppler shift. Proposal 3: RAN1 to capture observations on RACH tolerance to differential delay and differential Doppler |
| China Telecom | Observation 3: In most cases, the errors of one-way delay or Doppler induced by location uncertainty fail to meet the timing and frequency synchronization requirements of NTN. |
| Qualcomm | Observation 5: Current NR PRACH formats (long formats including restricted sets A/B and short formats) is not designed to operate under the time-frequency errors observed in GNSS-resilient NTN. Observation 6: Short PRACH formats cannot resolve any timing error larger than the symbol duration for 15 KHz SCS (i.e. 66.66 μs) and hence is incompatible with Scenario 1 and Scenario 2 with any reasonable size of location uncertainty area. Proposal 4: Capture the evaluation results in Table 2-2 in the TR (see R1-2509228). Observation 9: Based on link level simulation, long PRACH formats in FR2 NTN offer enough link margin even with the addition of phase noise. Proposal 7: RAN1 to study the impact of phase noise on the detection of the existing PRACH formats in FR2 NTN. |
| NTT DOCOMO | Observation 3: - Comparing the tolerable timing and doppler with the differential RTT and doppler, it can be observed that - In scenario 1, the differential delay exceeds the tolerance of all existing PRACH formats. - In scenario 2, the differential delay exceeds the tolerance of some PRACH formats (3, A1, B1, B2, B3, C0, C2). - In scenario 1 and 2, the differential doppler of NGSO exceeds the tolerance of long PRACH formats (0, 1, 2, 3). - In these cases, enhancements to increase PRACH tolerance and reducing time/frequency uncertainty are needed. |
| Panasonic | Observation 6: The following are observed for case a (uncertainty area is the area served by the cell or beam). - No existing PRACH formats satisfy the criteria. - The unrestricted set of PRACH format 1 satisfies the time domain criteria of case a for LEO600/1200 Set1-parameter, but the restricted sets A and B do not because the max N_CS term starts to dominate. Even with - For Ka-band, all PRACH formats satisfy the frequency domain criteria but does not satisfy time domain criteria due to the short CP length of short format. - For GEO all PRACH formats satisfy the frequency domain criteria but fail the time domain criteria. - Little dependence on the UE speed. Observation 7: The following are observed for case b (uncertainty area is a circle of radius X km). - For S-band, PRACH format 1 with restricted set B satisfies the criteria for X up to 10 km for both LEO600/1200 and GEO. - For Ka-band, all PRACH formats satisfy the frequency domain criteria for at least up to X=25km. X to satisfy the time domain criteria depends on PRACH formats. Format C2 satisfies the criteria for X up to around 600 m. |
| Google | Observation 1: Considering the agreed UE speed for evaluation is up to 1500 km/hr, the existing PRACH restrict sets may not be feasible. |
| Johns Hopkins University | Proposal 1: RAN1 to study supporting initial access to low-elevation angle scenarios where the differential delay is significantly higher than in high-elevation angle scenarios. |
| LGE | Observation #7: In FR1 NR-NTN, regarding TO tolerance, NR PRACH format (e.g., Format 1) is feasible except in GSO scenarios with elevation angle ≤70°. Observation #8: In FR1 NR-NTN, regarding FO tolerance, NR PRACH format (e.g., Format 1) is feasible when scaling factor = 1, but not feasible in LEO scenarios with scaling factor = 2 and elevation angle ≥70°. Observation #9: In FR2 NR-NTN, NR PRACH format (e.g., Format C2) is not feasible for TO tolerance at any elevation except the nadir beam, but is feasible in all cases for FO tolerance. |
| CSCN | Observation 5: Due to the large differential time and frequency offset, availability of CS multiplexing would be limited. Proposal 9: Necessary optimizations of the preamble reception and RAR windows should be introduced. |
| CEWiT | Observation 5: For Reference-Point based TA Computation, detection of short PRACH formats may not be successful in NTN scenarios. Except for long PRACH format 1, other long PRACH formats (0,2, and 3) are unlikely to be detected reliably, as their timing error limits are too low to accommodate the larger delays typical of NTN. |

## Summary of companies’ contributions

This is a summary of companies' inputs regarding PRACH performance evaluation:

| Company | Summary of inputs/proposals on PRACH Performance Evaluation |
| --- | --- |
| FUTUREWEI | For both FR1 and FR2, the ability of existing PRACH formats to meet timing and frequency tolerances strongly depends on the beam size and scenario; while some formats meet the requirements for smaller uncertainty areas, their tolerances are exceeded at specific beam diameters. |
| Spreadtrum | Existing long PRACH formats often cannot cover maximum differential RTT/RTD, especially for LEO/GEO and wider beams; short formats better for RTD but not RTT in most cases. |
| vivo | Existing PRACH formats in FR2 can't handle maximum timing/frequency errors with common cell-based pre-compensation per current RAN4 requirements. |
| CMCC | Larger CP and SCS needed without GNSS; current NR CP design doesn't support S-band LEO-1200/GEO; short formats unsuitable for Ka-band and wide beams. |
| Thales | PRACH Format 1 meets delay tolerances for ≤10km uncertainty; moderate beams possible with flexible ZCZ; wide beams exceed limits. Restricted sets enhance Doppler tolerance. |
| Huawei | None of the existing formats work for edge beams in FR1/FR2; short formats work for Doppler in nadir beams but not for delay. |
| CATT | No PRACH format can tolerate timing/doppler errors simultaneously for scenario 1; only certain formats (esp. C2 for LEO-600/Ka) work for specific conditions in scenario 2. |
| Xiaomi | NR-defined PRACH formats cannot cover needed differential delay and Doppler simultaneously in evaluated NTN scenarios. |
| Tejas Network | Several PRACH formats support delay/doppler separately but not simultaneously for many NTN scenarios; enhancements needed for GNSS-resilient operation. |
| Samsung | PRACH Format 1 with restricted sets covers most scenarios if reference point used, except GSO @2GHz (suggest enhancement only for this case). |
| Ericsson | Existing PRACH struggles with large residual Doppler and timing uncertainty; receiver complexity increases, esp. for short formats; multiple solutions should be considered. |
| ZTE | Existing PRACH formats fail for large uncertainty areas; for moderate uncertainty (e.g., X=10km S-band, X=1km Ka-band), some formats are viable. |
| Amazon | For 25km uncertainty, PRACH formats cannot tolerate required delay/doppler, even at low speed; for 10km, low-speed supported but not high-speed. |
| Nokia | 5G NR PRACH not able to meet max differential Doppler or RTT for large cell/uncertainty; detection suffers significant performance loss (up to 4 dB). |
| Apple | Existing PRACH formats can't meet differential RTT/Doppler needs at all elevation angles for wider uncertainty areas; performance varies by scenario and restricted sets. |
| TOYOTA ITC | Maximum differential delay and Doppler exceed current PRACH tolerance limits in some scenarios. |
| MediaTek | RACH tolerance for differential delay and Doppler varies by format/SCS; long preambles allow for larger GNSS errors and beam diameters than short formats; differential delay is critical. |
| China Telecom | Delay/Doppler errors due to location uncertainty fail to meet NTN synchronization requirements in most cases. |
| Qualcomm | Current PRACH formats (long/short/restricted) not robust against time/frequency errors in GNSS-resilient NTN; requests capturing evaluation results and phase noise impact study. |
| NTT DOCOMO | In many use cases, differential delay/doppler exceed all PRACH format tolerances; enhancements are needed to increase PRACH tolerance in NTN. |
| Panasonic | For uncertainty over entire beam, no PRACH formats satisfy requirements; for smaller area, only specific formats/sets are viable. |
| Google | Existing PRACH restricted sets may not be feasible for UE speed up to 1500 km/hr. |
| Johns Hopkins Univ. | Study needed to support initial access for low elevation angles with increased differential delay. |
| LGE | Format 1 feasible for TO tolerance in FR1 except GSO at low elevation; FO tolerance not feasible in certain LEO/elevation cases; in FR2, only FO tolerance feasible for some beams. |
| CSCN | CS multiplexing limited by large timing/frequency offset |
| CEWiT | Only long PRACH format 1 reliable for reference-point TA; other formats fail to accommodate typical NTN delays. |

Key consensus:

Existing PRACH formats often cannot handle both large differential delay and Doppler simultaneously for wide beams, or high UE speeds.

Smaller uncertainty areas (beam diameters ≤10 km) and careful set/format selection (esp. restricted sets) can make some formats viable.

## Initial proposal

### Proposal 4-1

Based on the evaluation results of differential delay and Doppler, as well as the calculated timing and Doppler tolerances and limits for PRACH from multiple sources, we can make the following observations for now.

More detailed assessments of PRACH performance across different PRACH formats, scenarios, satellite orbits, beam configurations, and relevant parameters; which could be included in the TR, will be discussed at a later stage (hopefully in this meeting), once evaluation results and the agreed timing and Doppler limits for the different PRACH preamble formats ((Proposals 2-x/ Proposals 3-x)) are finalized.

Proposal 4-1-v0

Based on the evaluation results of differential delay and Doppler, as well as the calculated timing and Doppler tolerances for PRACH from multiple sources: - The existing PRACH formats (both long and short) are inadequate for many NTN scenarios under the study on NR-NTN GNSS resilience. - Differential delay (due to large beams, low elevation, and location uncertainty) and differential Doppler (due to high motion and frequency bands) frequently exceed the detection capabilities of existing PRACH. - Short PRACH formats are especially problematic due to tiny cyclic prefixes (CP) and limited tolerable timing and frequency errors. - Enhancements are required to support the large differential delays/Dopplers in various NTN scenarios without GNSS availability

Companies are to comment on Proposal 4-1-v0

| Companies | Comments |
| --- | --- |
| CMCC1 | This is a general observation over all the scenarios and use cases. Though we share the same understanding as FL that most cases, the current PRACH format cannot work if the uncertainty area is very large or as large as cell radius. But in your understanding, the current PRACH format can work in some specific cases with additional limitation on the uncertainty area. For example, if the uncertainty area is limited to 10km radius, PRACH format 1 can still work in LEO-600km. |
| OPPO | We prefer to firstly make detailed observations on the RPACH performance evaluation across different RPACH formats and satellite scenarios. Then, we can decide whether to discuss this high-level proposal. |
| Spreadtrum | We share the same view with OPPO. |
| CSCN | The PRACH performance evaluation results should be determined first, based on which we could start this discussion. |
| CEWiT | It is preferable to first evaluate the performance of PRACH. Since the long PRACH Format 1 already supports many scenarios—for example, LEO at 600 km—enhancements are primarily needed for other cases, especially those involving the short formats. |
| Samsung | We believe it is crucial to first establish observations on the actual error margins (or performance limitations) of the existing PRACH formats. This is necessary to properly evaluate whether future proposals offer genuine improvement and can sufficiently compensate for the significant delay/Doppler errors (e.g., those arising in GNSS-resilient scenarios). A simple statement, such as 'the existing PRACH formats are inadequate,' provides no useful information for this analysis. |
| Tejas Networks | Support the proposal |
| Huawei, HiSilicon | Should clarify that this proposal is only for Scenario 1 and the enhancements should within the SID scope. |
| Toyota ITC | Agree with Oppo and Samsung. We should complete the evaluation and make such claim more meaningful manner. |
| Toyota ITC | Agree with Oppo and Samsung. We should complete the evaluation and make such claim more meaningful manner. |
| Ericsson | Support in principle but we have one comment on the statement that short PRACH formats are "especially problematic due to tiny cyclic prefixes (CP) and limited tolerable timing and frequency errors". The part about frequency errors may not be entirely correct. The SCS with short preambles is larger than with long preambles. So, in that sense their tolerable frequency errors are larger than long preambles even without restricted sets. |
| LGE | Before discussing PRACH enhancements, we need to have consensus observations in RAN1. The observations should be organized by condition (e.g., LEO600/LEO1200/GSO, case a/case b with X = {1, 5, 10, 25} km, FR1/FR2), and timing offset (TO) and frequency offset (FO) perspectives should be distinguished. For example, the followings can be observed: - For case a, many existing PRACH format are not tolerant to TO, whereas some formats (e.g., Format 3 or short formats) exhibit tolerance to FO - For case b, where X ≤ 10 km, some existing PRACH formats are tolerant to both TO and FO Furthermore, the target of PRACH improvements must be clear. For example, the discussion should be clear about whether to improve TO tolerance or FO tolerance. For example, if TO tolerance can be improved in FR1, PRACH format 3 supports sufficient FO tolerance. Moreover, since changing the physical structure of PRACH format is out of WI scope, solutions that reduce the uncertainty area should be pursued to mitigate differential delay and Doppler issues. |
| Panasonic | We agree with the general direction of our FL. But we should consolidate PRACH performance results first, then discuss next steps. |
| Nokia | Agree |
| Sony | Agree with OPPO, Samsung, Toyota, Panasonic. |
| ZTE | Not support. At least for case b, the existing PRACH format can support many cases, e.g., X>=10km for S-band. It is preferred to have observations case by case, showing which scenarios can work and which cannot. Moreover,as existing PRACH format can support may NTN scenarios, e.g., for case b X<=10km and even for case a when elevation angle is high. The enhancements may not be required. It is preferred to draw an observation only for evaluation results of PRACH. The enhancements can be discussed separately. |
| Apple | Agree with OPPO, solid data is helpful for further discussion. |
| ETRI | We have the same view with OPPO, Samsung, and Toyota ITC. |
| Futurewei | From the first sentence of the proposal, the intention is to capture the outcome of the evaluation results. However, the last bullet deal with enhancements, which are not in line with the intention of the proposal. Suggest the following revision: Based on the evaluation results of differential delay and Doppler, as well as the calculated timing and Doppler tolerances for PRACH from multiple sources: - The existing PRACH formats (both long and short) exceed the calculated timing and Doppler tolerances.are inadequate for many NTN scenarios under the study on NR-NTN GNSS resilience. - Differential delay (due to large beams, low elevation, and location uncertainty) and differential Doppler (due to high motion and frequency bands) frequently exceed the detection capabilities of existing PRACH. - Short PRACH formats are especially problematic due to tiny cyclic prefixes (CP) and limited tolerable timing and frequency errors. - Enhancements are required to support the large differential delays/Dopplers in various NTN scenarios without GNSS availability |

# Topic#5 Pre-compensation of Common TA / Doppler on the service link

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Proposal 1 Cell-level or beam-level common TA and common Doppler of service link can be used for UL time/frequency compensation when UE cannot rely on its GNSS for timing and frequency compensation on the service link. |
| CMCC | Proposal 14: For the GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link, - The UE performs timing and frequency compensation based on the reference location provided by network - Service link time/frequency pre-compensation based on last acquired GNSS position. |
| Thales | Proposal 1: In NR NTN, when GNSS information is unavailable (i.e., the UE cannot determine its position accurately using GNSS), the UE should be provided with assistance information to partially pre-compensate for round trip delay and Doppler effects on the service link: - For (quasi) Earth-fixed cells, a reference location for the serving cell may be provided. - FFS: Whether the reference location is provided per cell or per beam. - For Earth-moving cells, a common Timing Advance (TA) and common Doppler value for the service link may be indicated. Proposal 6: Following approach to support the initial TA acquisition for UL transmission can be studied in NTN with degraded or unavailable GNSS fix: - UE pre-compensates the common RTT on the service link, relative to referenceLocation - Reference location of the serving cell provided via NTN (quasi)-Earth fixed cell - movingReferenceLocation - Reference location of the serving cell of an NTN Earth-moving cell at a time reference. Time reference of this field is indicated by epochTime in ntn-Config of the serving cell. - Additional UE-specific TA indicated in RAR. FFS: The need and support of negative TAC values in RAR. Proposal 8: For UL frequency compensation in case of degraded or unavailable GNSS position fix: - Both open loop and closed loop UL frequency control can be studied - Compensation of common frequency offset at UE side - Estimation of common frequency offset and pre-compensation at UE side is conducted based on Reference location of the serving cell and satellite ephemeris. - Closed-loop method for UL frequency compensation, e.g., the gNB will signal a frequency adjustment command to UE |
| CATT | - Introducing common TA drift of beam center of service link is necessary to reduce the timing errors due to satellite moving, thereby avoiding frequent timing adjustments. |
| OPPO | Proposal 3: For UL frequency synchronization in connected mode, enhancements on compensation of residual UE-specific frequency offset are not needed. |
| Tejas Network | Proposal 4 :The beam-specific reference point used for common Timing Advance (TA) calculation in Scenario 1 should be defined by the network such that it represents the location within the beam contour closest to the satellite Proposal 5: The UE should pre-compensate the common doppler with respect to the beam centre in scenario 1 which is the reference point signalled by the network to the UE so that the maximum differential residual doppler within the uncertainty area reduces to half the value. |
| Ericsson | Proposal 3 Study UL pre-compensation for initial access based on a reference location, broadcast by gNB, used by the UE as its own location at preamble transmission. Proposal 6 Study pre-compensation for RRC_CONNECTED state based on a reference location, broadcast by gNB, used by the UE as its own location for UL transmission. |
| Sharp | Observation: A maximum estimation error can be included with a reference distance to reduce the uncertainty, but not sufficient to guarantee the PRACH detection with required tolerance. |
| Honor | Proposal 7: For scenario 1, and for scenario 2 when the latest GNSS based UE location is invalid during RRC connected mode, support UE side time pre-compensation based on reference location. |
| InterDigital | Proposal 2: Support time frequency compensation for UL transmissions at the UE against a reference location as a baseline approach for GNSS resilient NTN operation. |
| Lenovo | Proposal 2: RAN1 to study the feasibility of providing time and frequency pre-compensation values or reference point through SIB for uplink synchronization during initial access. Drift rate or derivation of drift rate of time frequency pre-compensation values can be considered for signalling overhead reduction. Proposal 3: RAN1 to study the iterative RACH procedure with updated TA and frequency compensation values after each of the failed RACH attempts. Proposal 4: Support the use of historic data (e.g., last acquired GNSS position) during initial access to compensate for time and frequency errors in uplink for shorter GNSS outages. |
| Apple | Proposal 1: To support initial access of a UE without valid GNSS information, network indicates the reference location for UE’s time and frequency pre-compensation for PRACH transmission. |
| Ofinno | Proposal 7: The network broadcasts a reference TA for the UEs in Scenario #1 or in Scenario # 2, to adjust their uplink timing for transmitting PRACH. Observation 12: A single reference TA value may not be suitable for a large NTN cell supporting multiple beams. Proposal 8: The network broadcasts one reference TA per beam or one reference TA per group of beams in an NTN cell with multiple beams. |
| Qualcomm | Proposal 1: RAN1 to study how to perform uplink time and frequency adjustments for GNSS-resilient UEs in Scenarios 1 and 2 - As a baseline for evaluations, RAN1 can assume that the UE is able to compensate for the nominal timing and frequency offset. - The nominal time and frequency adjustment can be based on a reference location inside the uncertainty area. - For evaluation purposes, the reference location can be set as follows: - Scenario 1: Reference location can be the center of the beam. - Scenario 2: Reference location can be the center of the uncertainty area. |
| LGE | Proposal #3: In Rel-20 NR-NTN, RAN1 studies cell- or beam-specific reference information for TO/FO pre-compensation, including the following aspects: - Coupling with existing common TA - Integration with UE-specific TO/FO adjustment Proposal #4: In Rel-20 NR-NTN, RAN1 studies cell- or beam-specific reference information for TO/FO pre-compensation corresponding to multiple reference points. |
| CSCN | Proposal 2: For Scenario 1, the gNB may broadcast assistance information (e.g., reference location) for the UE to calculate and compensate the common TA, while the differential offset could be estimated and corrected through PRACH. Proposal 3: Beam-specific reference location configuration could be considered to limit the maximum differential delay within the beam footprint. Proposal 4: The reference location should be appropriately selected. Proposal 6: For Scenario 1, the reference location based scheme could be used, where the gNB broadcasts beam-specific reference location to enable the autonomously common FO calculation and compensation at UE side. |
| CEWiT | Proposal 2: The satellite should compute reference TA using its location and the geometry of the cell/beam. The reference TA is TA between the satellite and reference point on the ground (RP2). Proposal 3: The satellite should transmit the reference TA in SIB1 for the UE’s preamble transmission. |

## Summary of companies’ contributions

In GNSS-resilient NR-NTN scenarios, companies widely propose that uplink timing and frequency compensation should rely on network-provided assistance; such as cell or beam-level reference locations, common TA/Doppler values, or drift rates, when the UE cannot use its own GNSS information.

- Network-provided reference for uplink compensation:
Networks broadcast or indicate a reference location/point (per cell, beam, or group of beams) for UE uplink time/frequency pre-compensation, often set as the beam center or uncertainty area center.
(Spreadtrum, CMCC, Thales, Ericsson, Apple, Lenovo, Ofinno, Qualcomm, LGE, CEWiT, TEJAS Network, CSCN, InterDigital, Honor, CATT)

- Common TA/Doppler values:
Use of cell- or beam-level common TA and Doppler/frequency offset values to enable UE pre-compensation, potentially signaled via SIB or other broadcast methods.
(Spreadtrum, Thales, Tejas Network, CEWiT, CSCN, Ofinno, CMCC, Lenovo, LGE, InterDigital, CATT)

## Initial proposal

### Proposal 5-1

Several companies broadly support network-assisted uplink time and frequency compensation using reference-based values/Common TA and common Doppler to ensure GNSS-resilient NR-NTN operation, along with the need for further study and enhancement of compensation and signaling mechanisms to optimize synchronization and overhead.

Proposal 5-1-v0

For the study of GNSS resilient operation, it is assumed that the UE pre-compensates the common RTT and common Doppler on the service link, based on assistance information indicated by the Network. This assistance information could be: - A reference location e.g. in case of (quasi)-Earth fixed cell - A Common TA/Common Doppler relative to service link e.g. in case of moving cell

Companies are encouraged to comment on Proposal 5-1-v0

| Company | Comments |
| --- | --- |
| CMCC1 | Generally fine with the 1st bullet. For the 2nd bullet of common TA and Doppler, it should be further clarified how it can work in the case of moving cell. Since the satellite and the serving area on the earth is always moving, the common TA and Doppler should be updated accordingly. The frequent updates of the common TA and Doppler are needed in the case of moving cell. It should be clarified how to gNB to deliver the common TA and Doppler to the UE and how to solve the overhead issue. |
| OPPO | Support. |
| Spreadtrum | Support |
| CSCN | Support the 1st bullet, to reduce the residual delay and frequency offset, beam-specific reference locations should be considered. Regarding the 2nd bullet, detailed clarifications are needed about how to determine the common TA/Doppler value. |
| CEWiT | Support |
| Samsung | Fine |
| ST Engineering iDirect | Support |
| Tejas Networks | Support the proposal. Need clarity on the exact location of the reference location for different scenarios |
| Huawei, HiSilicon | A reference location can also be applied for moving cell and UE can propagate the reference location with ephemeris, RAN2 already support moving RP for moving cell. We prefer to delete the e.g. part. |
| TCL | Ok with the proposal. We slight prefer the first bullet. For moving cell, UE can combine the RP information and satellite ephemeris to make the common compensation. But UE can not compensate the service link if only the common value is available. |
| Toyota ITC | This is one proposed solution. Therefore, we should discuss what assistance information is needed for different solutions first. |
| Ericsson | We support the main paragraph and the first bullet. We think a common solution should be selected (using a reference location) for both Earth-fixed and Earth-moving cells. |
| LGE | First, it is not desirable to consider separate solutions for the (quasi)-Earth fixed cell case and the moving cell case in terms of complexity and specification works. Our understanding of the second sub-bullet is to introduce a formula for TO/FO pre-compensation to the service link, similar to the common TA formula for conventional feeder links. We also believe that more than one reference information should be provided to reduce the level of uncertainty. Therefore, we propose the following modification: For the study of GNSS resilient operation, it is assumed that the UE pre-compensates the common RTT and common Doppler on the service link, based on assistance information indicated by the Network. This assistance information could be: - One or more common TO/FO pre-compensation formula for the service link |
| Panasonic | Support |
| Lenovo | Support |
| Nokia | Proposal is having a special proposal number. Just to be aware of this. We support the direction of this proposal, but think it may be too early to focus only on these two alternatives. For instance, as also highlighted in our contribution R1-2509007, there may also be a period of time where the UE’s previous location was known, but has been considered “out of GNSS coverage”. After all, the study item clearly uses the word temporarily unavailable, implying that the UE geo-location used to be available. Hence, we would like to amend the above such that it also takes into account the following: - UE may also use its last known position (for a restricted period of time) There may be more than one “reference location” within a cell – and in this connection it is important that we separate the terminology a bit from what is used for conditional handover as defined in RAN2. |
| NEC | We support the first bullet and remove the e.g. part. |
| Sony | We support sentiment and first bullet, but think it is unnecessary to use a different soltution for Earth-moving beams. Agree with companies suggesting consolidated solution for both scenarios. |
| ZTE | Firstly, the common TA/common Doppler based pre-compensation can work for both earth moving and earth fixed scenarios. The legacy common TA for feeder link can work for a fixed base station, which is similar to earth fixed cell case. Hence, it is not preferred to restrict the scenario for the solution, i.e., removing “e.g. in case of moving cell” for second subbullet. Moreover, the precompensation based on assistance information may not be necessary for all scenarios. For example, when UE can use previously acquired GNSS position or position obtained from RAT-positioning for pre-compensation, the assistance information may not be needed. Such cases should also be considered. |
| Apple | Support |
| ETRI | We share the same view with Toyota ITC. |
| Ofinno | Support the proposal. |
| Google | Support |
| Qualcomm | Support |

# Topic#6 Impact on connected mode

## Background

RAN1#122bis made the following agreement:

Agreement:

RAN1 to assess the impacts on connected mode during NR NTN GNSS-resilient operation, including at least:

- Impact on signalling overhead.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Observation 8 For set-1 S-band LEO-600 and GEO, current TAC in RAR can cover the maximum differential RTT in most situations when SCS is 15kHz. Observation 9 Signaling overhead is increased significantly since NW need to send TAC MAC CE frequently to help UE maintain synchronization if GNSS information is unavailable for LEO-600km and 1200km. |
| CMCC | Observation 7: As the elevation angle changes from 30o to 90o, the gNB needs to send approximately 300 TA update commands to the UE. This high rate imposes a significant signaling overhead and degrades system performance. Proposal 11: It is proposed to study the enhancements to reduce the transmission of TA commands when GNSS is not available. |
| Thales | Observation 11: Simulation results show that signalling overhead increases when the radius of the uncertainty area or satellite altitude grows, and is further amplified in scenarios using higher subcarrier spacings (such as 120 kHz in the Ka/Ku band), which require more frequent timing advance corrections. This effect is especially pronounced in larger uncertainty areas, where the minimal period between corrections decreases, leading to a higher rate of signalling events. Overall, tighter timing requirements and wider uncertainty areas both contribute to greater signalling overhead in NR NTN operations. Observation 12: In the absence of a GNSS fix in NR NTN, the signalling overhead becomes significant, as shown by the increased rate of TA commands required to maintain connectivity. For instance, based on simulation results, in the S-band with a 25 km uncertainty radius and 15 kHz SCS, the minimal period between corrections is 0.180 seconds, which translates to approximately 5.6 TA commands per second. In the Ka/Ku band under the same uncertainty and with 120 kHz SCS, the period drops to just 0.077 seconds, resulting in about 13 TA commands per second. These examples highlight that, in GNSS-resilient operation, without a solution to mitigate higher signalling overhead, NR NTN systems experience a high rate of TA signalling, substantially increasing protocol overhead and posing a challenge for efficient system operation. Observation 13: Simulation results clearly show that in the absence of a GNSS fix, NR NTN systems require significantly more frequent frequency adjustment commands (FAC), resulting in increased signalling overhead. For instance, in the S-band with a 15 kHz SCS at LEO600, increasing the uncertainty radius from 25 km (corresponding to approximately 2.19 FACs per second) to 50 km (about 4.39 FACs per second) nearly doubles the number of FACs required. A similar trend is observed at higher altitudes and larger uncertainty areas, such as at LEO1200 with a 90 km radius, where the FAC interval is 0.569 seconds, resulting in roughly 1.76 FACs per second. These findings highlight the need for effective mitigation strategies, as frequent FAC signalling can have a considerable impact on overall system efficiency in GNSS-resilient NTN scenarios. |
| ST Engineering iDirect | Observation 6: Using existing TA command updates and a potential FA command update, an update interval of 500 ms is insufficient to meet the stringent timing and frequency accuracy requirements. Observation 7: The existing TA command cannot correct the timing offset for LEO-600 deployment scenarios to meet the timing accuracy requirement of the CP-OFDM waveform, considering numerology 3 with 120 kHz SCS and for Ka band frequencies. Observation 8: With higher-order loops, the timing and frequency errors are within the accuracy requirements, even when the timing and frequency errors are only signaled once per second Observation 9: To limit the overhead of the time and frequency error signaling, the error may be sent when a pre-defined threshold is exceeded. Observation 10: The proposed time and frequency tracking does not use the two-line element (TLE) data from the satellite. Observation 11: Because the satellite position is not required in time-frequency tracking in connected mode, the SIB-19 periodicity can be enlarged, lowering the SIB-19 overhead. Observation 12: By using an error threshold for signaling timing and frequency errors, the overhead can be limited. The amount of signaling depends on the used threshold and is lower for the frequency error than for the timing error. Observation 13: The error threshold should be chosen smaller than the required timing or frequency accuracy. |
| Huawei | Observation 13: The existing close loop TA adjustment can alleviate the timing error for UL transmission other than PRACH in GNSS resilient scenarios. |
| Ericsson | Observation 15 When UL pre-compensation is based on a reference location, to keep the TA within timing accuracy limits specified by RAN4, TA commands may need to be sent more frequently than 2-3 times per second. |
| MediaTek | Observation 13: The TA commands may need to be sent more frequently than 2-3 times per second to meet the timing accuracy requirements for UE pre-compensation. |
| Qualcomm | Observation 10: UE with location uncertainty cannot meet the Doppler pre-compensation assumption; the network must provide frequency adjustments to keep UE within a certain limit from the UL carrier frequency (e.g. within ±0.1 ppm) at gNB reception. Observation 11: For a 600 km LEO, a static UE experiences RTT drift due to satellite motion. The gNB may need to issue up to ~20 TA commands per second to maintain a timing alignment within the CP for 15kHz SCS. Proposal 9: RAN1 to study the impact of timing drift on closed loop TA, and potential enhancements to reduce issuance of TA commands for UEs in connected mode. Proposal 10: For the evaluation of frequency and timing adjustment methods, RAN1 will consider evaluation methodology: - Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ - Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth) - Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method - Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms - Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively Using the steps mentioned above, RAN1 to evaluate timing and frequency adjustment mechanisms using the following KPI: - Number of TA and FA commands issued within T. |
| CSCN | Observation 2: When UE has no available GNSS information for a long time, TA adjustments via TAC in the connected mode face critical challenges of excessive signaling overhead and outdated TAC. |

## Summary of companies’ contributions

Companies’ inputs regarding the impact on connected mode, focusing on timing and frequency synchronization and associated signaling overhead:

Spreadtrum: TAC capability: For set-1 S-band LEO-600 and GEO, the current TAC in the RAR generally covers the RTT when using a subcarrier spacing (SCS) of 15 kHz. Signaling Overhead: In scenarios where GNSS is unavailable (LEO-600km and 1200km), the network must frequently transmit TAC MAC Control Elements to keep UEs synchronized, leading to significant signaling overhead.

CMCC: Frequent updates: As the satellite elevation angle shifts from 30° to 90°, the gNB may need to send as many as 300 TA update commands to each UE, severely increasing signaling and degrading system performance. CMCC suggests studying ways to reduce the required frequency of TA commands when GNSS is not available.

Thales: Simulation Results (TA Overhead): Signaling overhead grows with larger uncertainty areas and higher satellite altitudes. This burden further intensifies with bigger SCS (e.g., 120 kHz in Ka/Ku band). Quantitative Insights: In the S-band (25 km uncertainty, 15 kHz SCS), about 5.6 TA commands per second are needed; in Ka/Ku (same uncertainty, 120 kHz SCS), up to 13 TA commands per second may be required. Without mitigation, protocol overhead becomes unmanageably high. Frequency Adjustment Commands (FAC): FAC overhead also escalates as uncertainty radius and altitude grow; for example, with a 15 kHz SCS in LEO-600, increasing uncertainty radius from 25 km to 50 km roughly doubles FACs needed (e.g., from ~2.2 to ~4.4 per second).

iDirect: TA/FA Update Interval Limits: An update interval of 500 ms for TA and FA is insufficient to meet strict accuracy demands. Current TA Command Insufficiency: The existing TA command cannot correct timing offsets for LEO-600 (120 kHz SCS/Ka band), failing to meet CP-OFDM timing needs.

Huawei: Closed-Loop TA: Existing closed-loop TA adjustment can alleviate timing errors for uplink transmission (other than PRACH) in GNSS resilient NTN, helping maintain link quality.

Ericsson: High update frequency needed: When uplink pre-compensation uses a reference location, TA commands may need to be transmitted more than 2-3 times per second to maintain required accuracy set by RAN4.

MediaTek: Frequent command requirement: To satisfy uplink timing pre-compensation, especially without GNSS, more than 2-3 TA commands per second may be necessary.

Qualcomm: Doppler pre-compensation limitation: With uncertain UE location, the assumption of effective Doppler pre-compensation breaks down; the network must regularly provide frequency adjustment to keep error within tight bounds (e.g., ±0.1 ppm). TA Command Rate: For LEO-600 and 15 kHz SCS, up to 20 TA commands per second may be required for a static UE due to rapid RTT drift as the satellite moves—a substantial signaling rate. Evaluation Proposal: Qualcomm proposes RAN1 examine the impact of timing drift on closed-loop TA and evaluate the number of required TA/Frequency Adjustment commands using a systematic simulation-based methodology.

CSCN: Without GNSS, the network faces critical problems as TA adjustments (via TAC) quickly become outdated and require frequent updates, further escalating signaling burden.

## Initial proposal

### Proposal 6-1

To be added at second round.

# Topic#7 Potential solutions for PRACH

## Background

RAN1#122bis made the following observation:

Observation:

For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#122bis:

- Solution 1: To improve robustness:

  - Solutions based on modified random access procedures, using existing PRACH formats:

    - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

    - Solution 1B: New PRACH restricted sets.

    - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in RAR

    - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command)

    - Solution 1E: Selection of a set of existing PRACH root

    - Solution 1F: Using TA margin and RO masking.

    - Other solutions are not precluded

- Solution 2: Solutions based on reducing time-frequency uncertainty

  - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

  - Solution 2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

  - Solution 2C: Solutions based on broadcasting DL timestamp(s)

  - Solution 2D: UE side time/frequency pre-compensation based on reference location.

  - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

  - Solution 2F: UE side time/frequency pre-compensation based on reference signal.

  - Other solutions are not precluded

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

- Other Solutions are not precluded

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Eutelsat | Error: Reference source not found |
| Spreadtrum | ## Proposal 1 Cell-level or beam-level common TA and common Doppler of service link can be used for UL time/frequency compensation when UE cannot rely on its GNSS for timing and frequency compensation on the service link. ## Proposal 2 Support solution 1D to improve robustness. ## Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) |
| vivo | Error: Reference source not found Error: Reference source not found |
| CMCC | Proposal 13: Enhanced PRACH formats and/or preamble sequences, including update of PRACH resource configuration, will not be specified in the WI phase in the Rel-20 GNSS resilient NR-NTN operation. Proposal 14: For the GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link, - The UE performs timing and frequency compensation based on the reference location provided by network - Service link time/frequency pre-compensation based on last acquired GNSS position. |
| Thales | Proposal 1: In NR NTN, when GNSS information is unavailable (i.e., the UE cannot determine its position accurately using GNSS), the UE should be provided with assistance information to partially pre-compensate for round trip delay and Doppler effects on the service link: - For (quasi) Earth-fixed cells, a reference location for the serving cell may be provided. - FFS: Whether the reference location is provided per cell or per beam. - For Earth-moving cells, a common Timing Advance (TA) and common Doppler value for the service link may be indicated. Proposal 5: RAN1 to identify and evaluate technical solutions to address excessive differential delays which are beyond the detection capability supported by current NR PRACH short formats. |
| ST Engineering iDirect | Proposal 9: Select during RAN1#123 which potential solutions we will consider in the remainder of the study. Proposal 10: For large beam sizes, multiple reference locations can be considered. The UE will transmit multiple PRACH using pre-compensation based on the different locations. Proposal 11: Signaling enhancements for Msg-2 in combination with the solution from Section 4.3, i.e., using implementation-based techniques, should be further studied. Observation 21: A wide detector might not work optimal when there are a lot of UEs in the network. Proposal 13: Study implementation-based techniques to enable initial access under GNSS-resilient operation. |
| Huawei | Proposal 6: UE can acquire coarse location information from multiple satellite DL-TDOA and make TA/FO pre-compensation for PRACH transmission. Proposal 11: The applied PRACH maybe different in different cases, e. g. PRACH restricted sets or unrestricted sets. |
| NEC | Proposal 1: Adopt Solution 2C, solutions based on broadcasting DL time stamp(s), to provide time advance and Doppler shift estimation under GNSS-resilient mode. |
| CATT | - Further study the feasibility of solution 2A with single/multi-satellite DL-TDOA based on current specifications to address GNSS resilence. |
| Xiaomi | Proposal 3. For Doppler proof preamble signal design, optimized roots can be chosen fitting the normalized Doppler frequency value up to a certain threshold k under given constraints on Ncs. Proposal 4. Multi-stage frequency compensation can be applied for under large frequency error case. |
| OPPO | Proposal 1: For GNSS resilience operation, RAN1 does not pursue PRACH enhancement in Rel-20. |
| Fraunhofer | Proposal 1: RAN1 to consider the center of the satellite’s uncertainty area to be used as a reference point, which is used by an NTN UE to pre-compensate UL time and frequency in case of a resilient GNSS operation, which helps minimizing the residual differential delay/doppler with respect to the reference point. |
| Tejas Network | Proposal 6: The proposed Dual Format PRACH uses the existing formats as that of the legacy NR PRACH and can be configured such that two formats are separated in either time or frequency with minimal procedural changes to the existing Initial Access setup. Proposal 7: The RACH Occasion (RO) should be defined to accommodate the maximum differential delay relative to the beam-specific reference point. Although the actual signal duration for each PRACH transmission is approximately 1 ms, the overall RO duration is determined by the maximum differential delay. Proposal 9: Robust performance for preamble detection can be achieved by the proposed Dual Format PRACH in the worst-case scenarios of differential delay and residual doppler values in NTN Scenarios where GNSS is unavailable. |
| Samsung | Proposal 2: For GNSS-resilient operation, UE-side time/frequency pre-compensation based on a network-provided reference location (Solution 2D) should be studied as the baseline mechanism for initial access, as it addresses the worst-case scenario without assumptions on prior UE state. Proposal 3: The feasibility of solutions based on multi-satellite measurements (e.g., Solution 2A, 2F) or DL timestamps (Solution 2C) must be clarified, especially regarding their applicability, complexity, and latency for a UE in RRC_IDLE mode, before they are considered for further study. Proposal 4: Study of Solution 1A/2B should focus on the procedural aspect of multiple PRACH transmissions applying different, pre-defined pre-compensation hypotheses. Enhancements requiring new PRACH formats or roots (part of Solution 1A) should be deprioritized as they are not aligned with the SI objective of avoiding physical layer signal changes. Proposal 5: RAN1 should clearly distinguish between solutions addressing the initial acquisition (i.e., enabling a coarse PRACH detection despite large uncertainty) and solutions addressing residual error correction (i.e., fine-tuning after detection, such as Solution 1C/1D). Proposal 6: RAN1 to conduct a comprehensive study of all potential solutions. This study, while acknowledging potential difficulties in establishing a single common evaluation environment, is encouraged to include rigorous qualitative analysis (e.g., specification impact, practical effectiveness, UE complexity) and quantitative analysis (where applicable). The study should aim to identify a single, fundamental solution |
| Ericsson | Proposal 4 Study conjugated preambles and other PRACH enhancements that do not require physical layer channel/signal changes to enable initial access in the absence of GNSS information at the NTN UE. Proposal 10 Prioritize Solution 1A, 1D, 2D and 3. Proposal 11 PRACH preamble reception with large max differential delay can be handled by implementation-based techniques using a long enough PRACH processing window and multiple timing hypotheses. Proposal 12 A suitable PRACH scheme for LEO when GNSS is unavailable should be composed of two preamble transmissions: the first transmission uses a ZC sequence of an existing NR preamble format, and the second transmission uses a ZC sequence that is the complex conjugate of the first ZC sequence. Note that the complex conjugate of a ZC sequence is a ZC sequence with a root different from the original ZC sequence. Thus, the second transmission is also an existing NR preamble format. The existing Rel-18 PRACH repetition scheme can be used as baseline. |
| ZTE | Proposal 10: The solution 2E can be considered for initial access. The study can focus on the scenarios where GNSS error is moderate, e.g., X<= 10km for S band or X<=1km for Ka band. Proposal 11: The requirement for initial access can be relaxed to support GNSS resilience initial access, e.g., allowing access of UE without valid GNSS position. |
| Amazon | Proposal 1: scheduling constraints, use of appropriate PRACH Configuration Indices and enhancements to the preamble detection algorithm at the gNB should be used to overcome the CP limitation. Proposal 2: Support transmission of multiple preambles by the UE in different PRACH Occasions. Proposal 3: Support on-demand transmission of a second preamble by the UE based on feedback from the gNB, e.g., via Random Access Response. Proposal 4: For transmitting multiple preambles by the UE, the relation between the root indices should be studied with the objective of achieving maximum robustness to peak detection inaccuracies. |
| Sharp | Proposal 1: Multiple PRACH transmissions with different timing/frequency estimations should be studied with higher priority as proposed in Solution 1A and Solution 2B. Proposal 2: For long preamble formats, the UE may transmit multiple preambles with different TA pre-compensations under a power ramping counter if the maximum timing error is beyond the tolerance. - The step size is determined based on the preamble detection tolerance - Consider gNB preamble detection enhancements including extended detection region and partial preamble detection Proposal 3: NTN UE can define extended RO region in consecutive RO configurations for short preamble transmission - gNB may relax the reception restrictions and allow cross RO preamble detection. Proposal 4: Solution 1D should be supported with negative TA value and extended TA value range in RAR for GNSS resilience operation. |
| TCL | Proposal 8: In Rel-20 NTN, TA drift information can be considered to indicate. Proposal 9: Based on the complexity of implementation and impact on the specification, we prefer solution 2D and solution 1D as a high priority. |
| Honor | Proposal 3: Support solution 2E i.e., service link time/frequency pre-compensation based on the last acquired GNSS position if it is valid. Proposal 4: For scenario 1, and for scenario 2 when the latest GNSS based UE location is invalid during initial access, support solution 2D, namely UE side time/frequency pre-compensation based on reference location. Proposal 5: Support solution 1D, i.e., signalling enhancements for Msg2 including enhanced TAC and frequency corrections in RAR. |
| Nokia | Proposal 1: Further study solution 1A for initial access. Proposal 2: UE performs transmission of multiple PRACH with different roots for initial access only if a timer from the last acquired GNSS location is expired. Observation 4: Introduction of new PRACH restricted set(s) is not a viable solution for the scenario of LEO-600 with 25km cell radius. Proposal 3: Down-prioritize Solution 1B. Proposal 4: Down-prioritize Solution 1C. Proposal 5: Solution 1D is a complementary aspect to study of solution 1A and may be studied further. Proposal 6: Signalling enhancements for Msg2 RAR to be designed to be multiplexed with legacy UEs. Proposal 7: Down-prioritize Solution 1E until clarification on the intention and mechanism behind the proposal is clarified. Proposal 8: Down-prioritize Solution 1F until further clarifications are available on the meaning of TA margin and how it would solve the ambiguity of time and frequency offset contribution to the PRACH correlation peak. Observation 5: Solution 2A would be dependent on specific network deployments. Proposal 9: Down-prioritize Solution 2A. Proposal 10: Down-prioritize Solution 2B. Proposal 11: Down-prioritize Solution 2C. Proposal 12: Further study Solution 2D. Proposal 13: Further study Solution 2E. Proposal 14: Down-prioritize Solution 2F until clarification on the intention and mechanism behind the proposal is clarified. Proposal 15: Down-prioritize solution 3 (and any related implementation based approaches that forces additional gNB processing overhead). Proposal 16: Introduce Solution 3B: Consider UE autonomous barring of a cell in case the UE cannot guarantee initial access transmissions that falls within the time and frequency requirements due to lack of accurate GNSS information. Proposal 17: Introduce Solution 3C1: Study whether it would be feasible to enforce beam footprints that are sufficiently small, which will ultimately allow a UE to access a cell without impacting the PRACH detection performance. Proposal 18: Introduce Solution 3C2: Study whether it would be feasible to enforce sufficiently dense satellite deployments which will allow a UE to access a cell without impacting the PRACH detection performance. |
| InterDigital | Proposal 1: Do not study the solutions modifying RACH signals/procedure in GNSS resilient NTN SI. Proposal 3: Support the refinement of time frequency compensation adjustments at the UE side based upon DL measurement of RSs. |
| Lenovo | Proposal 1: Support the use of system time reference for internal clock offset and delay estimation in case of UE’s clock previously not synchronized with the network. |
| Sony | Observation 2 - It is unlikely that any one solution class will provide a complete solution; a combination of solution classes will be required. Observation 3 - Different solutions are applicable to different scenarios, so a single powerful solution is not necessary. |
| Ofinno | Proposal 1: Study the solution based on multiple PRACH transmissions as one of the candidate solutions to enhance timing/frequency adjustment during initial access. Proposal 2: Introduce separate PRACH resource sets: - a first set (Set 1) for UEs whose GNSS is available, - a second set (Set 2) for UEs in Scenario # 1 (i.e., which cannot rely on GNSS), and a third set (Set 3) for UEs in Scenario # 2 (i.e., which didn’t receive GNSS since time duration T). Proposal 6: Study the feasibility of avoiding or reducing unnecessary PRACH attempts solution based on multiple PRACH transmission attempts. |
| TOYOTA | Proposal 2: For uplink initial access, multiple reference locations is adopted as a candidate technique for GNSS resilient operation. Proposal 7: The listed solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty (based on inputs/discussions in RAN1#122bis), should be clarified with respect to the scope of the GNSS resilience study scope and objectives. Proposal 8: To accommodate the GNSS resilient operation, RAN1 to study a unified framework for uplink time/frequency synchronization which encompasses initial access and connected state procedures, configurations and parameters. Proposal 9: The proposed 3-step unified framework for uplink time and frequency synchronization to be adopted as the uplink synchronization framework for GNSS resilience operation. The three-steps are: • Step 1: Time/frequency compensation prior to PRACH transmission(s) • Step 2: Time/frequency compensation during PRACH transmission(s) • Step 3: Time/frequency compensation after PRACH transmission(s) Proposal 10: Listed solutions for initial access (Observation: RAN WG1 #122-bis) are considered under Step 1 and 2 of the proposed 3-step unified uplink synchronization frameworks. |
| China Telecom | Proposal 3: Support to study timing and frequency synchronization enhancing mechanism in NTN GNSS resilience. |
| Qualcomm | Proposal 5: RAN1 to study the following methods for enhancements in the random access procedure using the existing PRACH formats. - Option 1: New PRACH restricted sets: - Option 1-A: New PRACH restricted sets based on maximum differential RTT-RTD within the location uncertainty area. - Option 1-B: New PRACH restricted sets based on joint distribution of differential RTT-RTD within the location uncertainty area. - Option 2: Solutions based on multiple PRACH / Msg-3 transmissions: - Option 2-A: Transmission of multiple PRACH with the same root sequence and receiving multiple responses. - Option 2-B: Transmission of a single PRACH followed by reception of multiple Msg-3 grants in the RAR with different time–frequency corrections. - Option 2-C: Transmission of two PRACH using a pre-determined root pair and receiving a single response. |
| NTT DoCoMo | Proposal 3: - For Rel-20 NTN GNSS resilience, regarding initial access, study the following: - Initial TA acquisition - Solution 2D: UE side time/frequency pre-compensation based on reference location. (for scenario 1) - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position (for scenario 2) - Reduce the timing/frequency error caused by UE uncertain location - Solution 1D: Signalling enhancements for Msg2 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command) (for scenario 1 and 2) - Solution 1A/2B: multiple PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) (for scenario 1) |
| Panasonic | Proposal 1: Necessity of PRACH related enhancement should be clarified. No PRACH related enhancement is needed for Scenario 2 with a certain condition, e.g. low mobility UEs. Proposal 2: The enhancement targets, i.e. which cases e.g. long/short formats, delay/Doppler, and what degree to be improved should be identified from the analytical evaluation results. Proposal 3: The following should be further studied as the baseline schemes. - For scenario 1, UE pre-compensation based on reference location (Solution 2D) - For scenario 2, UE pre-compensation based on past acquired UE location (Solution 2E) - For both scenarios 1 and 2, T/F command in Msg.2 (Solution 1D) as an enhancement for PUSCH transmissions after PRACH. Proposal 4: Candidate solutions for delay tolerance, Doppler tolerance and/or estimation ability of timing offset and frequency offset should be studied after identifying the required improvements. |
| Google | Proposal 1: For GNSS resilience RACH enhancement, study the optimized PRACH set including preambles generated from individual root sequences (i.e., no cyclic shift). The optimization can be defining a new restrict set, or giving flexibility for BS to configure a feasible PRACH set. Proposal 2: To compensate the PRACH detection performance degradation due to limited preamble set selection and residual frequency shift, PRACH repetition mechanism introduced in release 18 should be considered as the baseline. Proposal 3: Reuse existing reference location indications (e.g., referenceLocation, movingReferenceLocation) for UE to determine uncertainty area. |
| Johns Hopkins University | Proposal 4: RAN1 to study the use of downlink broadcast timestamps to estimate Doppler frequency. Proposal 5: RAN1 to study the use of downlink broadcast timestamps to estimate the one-way delay. |
| LGE | Proposal #2: In Rel-20 NR-NTN, RAN1 studies solutions for GNSS-resilient operation that reuse the existing PRACH structure without introducing significant changes to physical channels or signals, in line with SI constraints. Proposal #4: In Rel-20 NR-NTN, RAN1 studies cell- or beam-specific reference information for TO/FO pre-compensation corresponding to multiple reference points. Proposal #5: In Rel-20 NR-NTN, RAN1 studies the feasibility of using downlink Doppler measurement for uplink FO pre-compensation. Proposal #6: In Rel-20 NR-NTN, RAN1 studies the separation of ROs according to TO/FO pre-compensation accuracy |

## Summary of companies’ contributions

Here is a recap of the preferences and support expressed by companies for each major category of initial access solutions to increase PRACH tolerance for GNSS-resilient NR-NTN, based on the RAN1#122bis solutions set:

1. Robustness via modified random access procedures (Solution 1 series)

- Strong support for multiple PRACH attempts and signaling enhancements:

  - Sharp, Amazon, Nokia, Ofinno, Tejas Network, Ericsson, Google, Thales, Panasonic support Solution 1A (multiple PRACH transmissions/hypotheses).

  - Nokia, Qualcomm, Panasonic, TCL, Spreadtrum, Honor: Support Solution 1D (signaling enhancements for Msg2, e.g., TA/frequency commands in RAR).

  - Nokia, CMCC, Amazon, Google: Prefer solutions that enhance PRACH robustness but avoid new format/physical layer signal changes.

  - Ofinno, Amazon, Sharp, Ericsson, Google: Endorse relation between preamble root selection for improved robustness.

  - vivo, ZTE, Nokia, OPPO, CATT: Caution regarding potential spec impact, overhead, and complexity for solutions involving new PRACH formats, restricted sets, or major changes to Msg1/Msg3.

  - Qualcomm: Proposes new PRACH restricted sets (1B) and joint approaches for robustness but flags practical limits for uncertainty areas.

2. Solutions reducing time/frequency uncertainty (Solution 2 series)

- High preference for UE-side time/frequency pre-compensation, reference location, and last GNSS position:

  - Samsung, TCL, Honor, NTT DoCoMo, Panasonic, Ericsson, Thales, Spreadtrum, OPPO, LGE: Strongly prefer Solution 2D (pre-compensation by UE based on reference location), viewed as baseline and highly effective for large uncertainties.

  - NTT DoCoMo, Honor, Panasonic, ZTE: Support Solution 2E (pre-compensation by UE based on last acquired GNSS position) if last GNSS is valid.

  - NEC, Johns Hopkins, LGE, Google: Support Solution 2C (broadcasting DL timestamp(s)) for estimation, can be combined with other solutions for robustness.

  - Huawei, Qualcomm, CATT, TCL, Amazon: Support studying single/multi-satellite DL-TDOA-based solutions (2A), but note dependency on network deployment and practical constraints.

3. Multi-reference location approaches

- Preference for multiple reference point solutions:

  - Toyota, Spreadtrum, LGE, Fraunhofer, Tejas Network, Sony: Endorse providing reference locations at cell/beam level or multiple reference points within beams to reduce differential delay/Doppler and support pre-compensation.

4. Implementation-Based techniques (Solution 3 series)

- Limited support, mostly as complementary approaches:

  - ST Engineering iDirect, Ericsson, Thales, Ofinno: Suggest implementation-based solutions (e.g., extended PRACH processing windows, multiple timing hypotheses, large search windows) primarily as supplementary, with caveats on increased gNB complexity and limited overall efficacy.

  - Nokia, Ofinno, Sony: Advise against focusing only on Solution 3 due to resource requirements and complexity.

5. Avoidance/minimization of Physical Layer/SI changes

- General preference for solutions requiring minimal physical layer modification:

  - CMCC, OPPO, TCL, InterDigital, Nokia, LGE, Huawei, ZTE: Prefer approaches that reuse existing NR PRACH formats and avoid physical channel or signal changes.

  - ZTE, InterDigital: Explicitly exclude solutions that change Msg1/Msg3 or physical channel from study scope.

  - Google, Ofinno, Nokia: Favor optimization and configuration of existing preamble sets, referencing Rel-18 PRACH repetition as a baseline.

Other Observations and Special Considerations

- Eutelsat: Stresses prioritizing solutions that work best at low elevation angles.

- Spreadtrum, Thales, Fraunhofer: Recognize need for common TA/Doppler compensation and residual error adjustment mechanisms.

- vivo, ZTE, TCL, LGE, Lenovo, MediaTek: Note the impact of resource overhead, preamble capacity, UE power consumption, latency, and specification complexity when evaluating and combining candidate solutions.

- Sony: Asserts that multiple solution classes, tailored to scenario specifics, are required rather than seeking a universal solution.

- Toyota: Proposes and details a unified three-step framework for uplink time/frequency synchronization, advocating a structured study and deployment of compensation techniques at all stages of initial access and connected state.

## Initial proposal

### Proposal 7-1

It is essential to gain a comprehensive understanding of companies’ preferences regarding the potential solutions discussed so far. Given the variety of solutions and technical trade-offs involved, we propose to collect, as a first step, companies’ prioritized preferences and rationale for each solution, with attention to performance, resource overhead, specification impact, and implementation complexity. The aim is that, by the end of the meeting, we can reach agreement on the potential solutions to be considered for further study.

Proposal 7-1-v0

Companies are invited to provide within the following table their prioritized preferences and rationale for the potential solutions to GNSS-resilient NR-NTN initial access, in order to establish a clear view of companies support and guide the decision on the most appropriate enhancements going forward.

Companies are encouraged to share views on Proposal 7-1-v0

| Company | Comments |
| --- | --- |
| OPPO | Without valid GNSS information, solution 2A, 2D, 2E should be prioritized to reduce the time-frequency uncertainty to a tolerable value range. In addition, based on the evaluation results, the value range of differential delay and doppler is very large, so solution 1D should be considered to guarantee the performance of the subsequent uplink transmission. Finally, if the above solutions can not combat the large differential delay and doppler within the uncertainty area, solution 3 should also be taken into account. |
| Spreadtrum | We prefer solution 2D combining with solution 1D. Solution 2D is the baseline. With solution 2D, UE can pre-compensation for most of the delay and Doppler of the service link. The residual delay and Doppler after pre-compensation can be solved by solution 1D, e.g., network correct the residual delay/Doppler through msg2. The details of enhancements for Msg2 can be further discussed. Additionally, solution 3 can also be considered which can avoid physical layer channel/signal changes. |
| CSCN | Solution 2D, 1D, and 3 should be prioritized. For solution 2D, it should be adopted as the baseline solution, and beam-specific reference location configuration could be considered. Regarding solution 2A, the multi-satellite DL-TDOA positioning highly depends on network deployment, while the accuracy of single-satellite positioning decreases significantly. Therefore, further study is expected to evaluate the feasibility of this solution under general network deployment assumptions. |
| CEWiT | The following solutions can be considered for further study: under solution 1 => 1A and 1D and under solution 2=> 2A, 2B, 2D and 2E |
| ST Engineering iDirect | We support the aim to reach agreement on potential solutions by the end of this meeting. Prioritization - Solution 2B: Multiple PRACH attempts (e.g., multi-reference locations). For small cells, we can use a single reference location (i.e., solution 2D). - Solution 1D: Signaling enhancements - Implementation-based techniques as complimentary approach As for the final conclusions of the study, we believe that we should aim for a single solution, rather than the “toolbox-based approach” that was promoted by some companies in RAN1#122b. In line with the 6G vision of a lean standard, we should not have too many final solutions targeted at different deployment scenarios. |
| JHUAPL | Support solutions 2A and 2C. Also, 2C could be used in both single and multiple satellite scenarios. |
| Tejas Networks | For GNSS resilient operation for NTN , prioritize on the solution 1A to improve robustness based on PRACH enhancements using existing formats . Prioritize on solution 2D for reducing time-frequency uncertainty and can be considered as a baseline to all solutions The solution 3 should also be prioritized along with the above solutions to tackle the large differential delays in GNSS resilient NR-NTN operation On solution 1D, this is not a solution to improve the robustness of PRACH to time and frequency offsets. This aspect basically comes along with other solutions as an enhancement to msg2 after the msg1 successfully received. Solutions to prioritize : 1A,1D,2D and 3 |
| Huawei, HiSilicon | We support to prioritize the following solutions Solution 2A Multiple satellite DL-DTOA is sufficient for UE to make pre-compensation for PRACH transmission considering the tolerance of existing PRACH formats Solution 1D to further support signalling enhancements after Msg2 during initial access To ensure the UL synchronization of Msg4-ACK and Msg5. Solution 2D Solution 2E |
| Interdigital | We think UE performing T-F pre-compensation using reference location (2D) and then doing the updates based upon RSs (2F) are the interesting approaches to explore further. The UEs may further refine T-F compensation based upon Msg2 feedback from the network. In some cases, reference location can be the last acquired GNSS location (2E). Solution 2D: UE side time/frequency pre-compensation based on reference location. Solution 2F: UE side time/frequency pre-compensation based on reference signal. |
| TCL | We prefer solution 1D and solution 2D as a high priority. Also, the solution 2A about DL-TDOA for satellites positioning can also be considered for study. |
| Toyota ITC | We should note that without proper alignment on results and observations (Different doppler/delay, PRACH tolerance), discussing solutions is bit too early. Moreover, some solutions are not clear whether they are within the scope of SID and their detail implementation, see some details in our contribution. We should encourage companies to provide details about proposed solutions. Agree with ST Engineering iDirect. As shown in our contribution evaluations, solutions based on reference locations (Solution 2B: Multiple PRACH attempts (e.g., multi-reference locations) or single reference location (i.e., solution 2D) are needed. |
| Ericsson | Prioritize Solution 1A, 1D, 2D and 3. A brief rationale is given below: - A combination of solution 1A and 3 is needed to handle the large differential delay and Doppler observed in many NTN scenarios. - Solution 1D is needed to correct the residual delay and Doppler before UE transmits Msg3 PUSCH - Solution 2D is needed to reduce the absolute delay/Doppler and reduce the difference between UEs with/without GNSS |
| LGE | We believe that solutions that reduce the size of the uncertainty area are the most practical solution for GNSS resilient operation. From this perspective, we prefer the following initial access solution: First, the network provides TO/FO pre-compensation assistance information (e.g., multiple common TO/FO pre-compensation formula for the service link), and the UE performs multiple PRACH transmission attempts corresponding to the multiple TO/FO pre-compensation information (e.g., similar to solution 2D and 2B). The assistance information may be determined based on the discussion in proposal 5-1. We are also open to study solution 2E and solution 2F (rather than solution 2C). |
| Panasonic | Solutions 2D and 2E should be baseline solutions for scenario 1 and 2, respectively, because these are straightforward approaches and have small spec impact. Solutions 1D is not related to PRACH tolerance but msg3 PUSCH tolerance. For the other solutions, since different solutions have different enhancement targets (i.e. what can be solved), what to be solve (e.g. timing tolerance, Doppler tolerance, time and frequency estimation) should be clarified first, and then we can discuss the priorities of the solutions. |
| Lenovo | We prefer to prioritize 1D, 2B, 2C, and 2E solutions. |
| Nokia | We would see the solutions above as a set of tool in a toolbox. We can see multiple candidates having their justification at different times (as they are targeting solving different things): For instance from Robustness point of view, we are supportive of 1A and 1D (for the signaling). It is to be noted that 1D cannot stand by itself, as there would be no “detection assistance”. From the “reducing uncertainty”, we are supporting that further studies happen for 2D and 2E (we do not know why this was left out of the above summary). Additionlly, as we proposed in our contribution one simple solution to a UE not being able to pre-compensate to a needed level it so simply apply self-barring to avoid doing harm to the network operation – we find this a simple and effective solution. Sometimes we simply need to give up and accept that we cannot have service all the time (and at any cost). |
| NEC | We support prioritizing the following solutions: Solution 2C: Provide both timing and frequency pre-compensation to a UE under single- or multiple-satellite directly. Solution 1D Solution 2D |
| Sony | We prefer to prioritize the following: “Estimating Pre-Compensation” set (1A, 2B, 2C, 2D) with additional signaling based on PRACH measurements (1C, 1D, 2F) |
| ZTE | We prefer to prioritize reduction of uncertainty considering that the SID aims to avoid changes of physical signal/channel and minimizes impact on procedure. Solution 2E can be prioritized, which reduces the uncertainty area and leverages legacy implementation as much as possible. We can be open to study solution 2D and 1D if justified necessary. |
| Apple | Our preference is 1D, 2D, 2E. |
| Ofinno | We need solutions for both robustness and for reducing time-frequency uncertainty. - Solutions 1: We support Solution 1B. But we are also fine with solutions 1A, 1C, and 1D. - Solutions 2: We support is 2B. We are also ok with 2D-F (which are related to each other). 2A is not feasible, e.g., as a standalone solution, at least due to satellite visibility and RRC connection requirements. |
| NTT DOCOMO | Solution 2D (for scenario 1) and Solution 2E (for scenario 2) should be prioritized, and these solutions should be used as a baseline for each scenario. Solution 2D/2E is straightforward and has a smaller spec impact compared to other solutions, making it a reasonable baseline solution. However, combining other solutions is also acceptable if necessary. |
| Google | To avoid lengthy discussion, we suggest to evaluate existing PRACH enhancement features first, then discuss other solutions, if needed. |
| Amazon | We prefer Solution 1A, however, the “conjugated preambles” solution is not ideal. The relation between the root indices should be carefully studied with the objective of achieving maximum robustness to peak detection inaccuracies at the receiver. |

# Topic#8 Potential solutions for connected mode

## Background

## Summary of companies’ contributions

This is the summary of various companies’ proposals, observations, and inputs on the evaluation of differential delay and Doppler, drawn from the collated inputs above:

General findings across companies:

- Differential delay is maximized at low elevation angles and with larger uncertainty areas/beam diameters; it is highest for GEO/GSO satellites due to longer slant paths.

- Differential Doppler is maximized at high elevation angles (nadir) and with larger beam diameters/uncertainty areas; it is highest for LEO satellites due to their high velocity.

- Both effects scale with beam size and location uncertainty area (radius).

- Worst case values vary with orbit (GSO>LEO for delay; LEO>GSO for Doppler) and frequency band (higher bands lead to larger Doppler frequency shifts).

- Many companies note that Doppler and delay values are relatively independent of frequency bands when analyzed as ppm (Doppler) or µs (delay), pending same geometric assumptions.

Key ranges:

| Orbit | Differential Delay (One-way, typical max) | Differential Doppler (typical max) |
| --- | --- | --- |
| LEO-600 | 142–487 µs (up to 1.2 ms in large beams) | 2–4.2 kHz (up to 3.7 ppm / >25kHz) |
| LEO-1200 | 256–800 µs | 1.8–3.9 kHz (up to 1.87–3.94 ppm) |
| GEO/GSO | 814–1625 µs (up to 1.6 ms) | 0.072–0.14 kHz (up to 0.13 ppm) |

Values depend strongly on beam size/uncertainty area and frequency band for Doppler (in Hz), but not in ppm.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| Spreadtrum | Proposal 1 Cell-level or beam-level common TA and common Doppler of service link can be used for UL time/frequency compensation when UE cannot rely on its GNSS for timing and frequency compensation on the service link. Proposal 3 Closed-loop adjustment mechanism for frequency offset can be considered both for initial access procedure and RRC connected mode. |
| CMCC | Proposal 14: For the GNSS resilient NR-NTN operation, the following methods can be studied when the UE cannot rely on its GNSS for timing and frequency compensation on the service link, - The UE performs timing and frequency compensation based on the reference location provided by network - Service link time/frequency pre-compensation based on last acquired GNSS position. |
| Thales | Proposal 1: In NR NTN, when GNSS information is unavailable (i.e., the UE cannot determine its position accurately using GNSS), the UE should be provided with assistance information to partially pre-compensate for round trip delay and Doppler effects on the service link: - For (quasi) Earth-fixed cells, a reference location for the serving cell may be provided. - FFS: Whether the reference location is provided per cell or per beam. - For Earth-moving cells, a common Timing Advance (TA) and common Doppler value for the service link may be indicated. Proposal 6: Following approach to support the initial TA acquisition for UL transmission can be studied in NTN with degraded or unavailable GNSS fix: - UE pre-compensates the common RTT on the service link, relative to referenceLocation - Reference location of the serving cell provided via NTN (quasi)-Earth fixed cell - movingReferenceLocation - Reference location of the serving cell of an NTN Earth-moving cell at a time reference. Time reference of this field is indicated by epochTime in ntn-Config of the serving cell. - Additional UE-specific TA indicated in RAR. FFS: The need and support of negative TAC values in RAR. Proposal 7: For GNSS resilient physical layer operation, study the following options: - Option 1: NTA, adjcommon should include the network-controlled common TA (as in Rel-17) and CommonTARefLoc which is the common RTD between the referenceLocation and the satellite. - Option 2: The differential TA is under network control (closed loop) via RAR and MAC CE TAC, that is included within NTA. - Option 3: The differential TA is under UE control (open loop), that is included within NTA,offset. FFS: How UE determines the timing drift. Proposal 8: For UL frequency compensation in case of degraded or unavailable GNSS position fix: - Both open loop and closed loop UL frequency control can be studied - Compensation of common frequency offset at UE side - Estimation of common frequency offset and pre-compensation at UE side is conducted based on Reference location of the serving cell and satellite ephemeris. - Closed-loop method for UL frequency compensation, e.g., the gNB will signal a frequency adjustment command to UE |
| ST Engineering iDirect | Observation 1: The current specifications define a 6-bit TA MAC CE and 12-bit absolute TA MAC CE to signal timing corrections from the gNB to the UE for closed-loop timing control. Proposal 1: Introduce a new MAC CE to signal estimated timing errors, instead of the timing advance correction that is defined in the existing TA MAC CE. Proposal 2: The new MAC CE contains a signed 10-bit value TE that represents the estimated time error TErr (in seconds), calculated as follows - TErr = NTE * Tc, with NTE the time error in Tc units - NTE = TE * 64/2µ, with TE the MAC CE payload and numerology µ Proposal 3: Send the MAC CE with the estimated timing error to the UE when a pre-defined error threshold is exceeded. Observation 2: The error threshold used to transmit the timing error should be smaller than the timing accuracy requirement of half the CP length. Proposal 4: Add a MAC CE to signal estimated frequency errors from the gNB to the UE. Proposal 5: The new MAC CE contains a signed 10-bit value FE that represents the estimated frequency error FErr (in Hz), calculated as follows - FErr = FE * FSCS, with FE the MAC CE payload and FSCS the SCS for the used numerology Proposal 6: Send the MAC CE with the estimated frequency error to the UE when a pre-defined error threshold is exceeded. The error threshold used to transmit the frequency error should be smaller than the frequency accuracy requirement of 0.1 ppm. Observation 3: The MAC CE with estimated frequency error and the MAC CE with the estimated timing error can be combined in a single entry. Observation 6: Using existing TA command updates and a potential FA command update, an update interval of 500 ms is insufficient to meet the stringent timing and frequency accuracy requirements. Observation 7: The existing TA command cannot correct the timing offset for LEO-600 deployment scenarios to meet the timing accuracy requirement of the CP-OFDM waveform, considering numerology 3 with 120 kHz SCS and for Ka band frequencies. Observation 8: With higher-order loops, the timing and frequency errors are within the accuracy requirements, even when the timing and frequency errors are only signaled once per second. Observation 9: To limit the overhead of the time and frequency error signaling, the error may be sent when a pre-defined threshold is exceeded. Observation 10: The proposed time and frequency tracking does not use the two-line element (TLE) data from the satellite. Observation 11: Because the satellite position is not required in time-frequency tracking in connected mode, the SIB-19 periodicity can be enlarged, lowering the SIB-19 overhead. Observation 12: By using an error threshold for signaling timing and frequency errors, the overhead can be limited. The amount of signaling depends on the used threshold and is lower for the frequency error than for the timing error. Observation 13: The error threshold should be chosen smaller than the required timing or frequency accuracy. Observation 14: As the proposed closed-loop solution for time and frequency only uses two newly added MAC CEs, there is limited impact on existing 5G NR NTN specifications. Observation 15: The proposed closed-loop solution with new MAC CEs to signal timing and frequency errors does not require any knowledge of the UE or gNB/satellite position, and it works for any orbit (LEO, MEO, GEO). Observation 16: The presented solution can be combined with existing pre-compensation methods and will address UE mobility and TLE errors. |
| Huawei | Proposal 12: The closed-loop frequency adjustment should be introduced in RRC CONNECTED. Proposal 13: UE should report the availability and validity of GNSS position information in GNSS resilient operation. |
| CATT | - The maximum adjustment range of initial TAC is 2ms, which may meet the requirement for GNSS unavailable temporarily case. - A frequency adjustment control (FAC) mechanism similar to the timing adjustment in the RACH response should be introduced. - Introducing common TA drift of beam center of service link is necessary to reduce the timing errors due to satellite moving, thereby avoiding frequent timing adjustments. Proposal 13: Extended CP can be supported to tolerate larger timing errors, thereby reducing the frequency of close-loop TA command. |
| Xiaomi | Observation 3. The closed loop frequency adjustment during initial access and frequency maintaining in connected mode can be performed either at gNB side or UE side. In case the value for frequency adjustment is controllable, the gNB side frequency error mitigation does not lead to much performance degradation while simplifies UE behavior and specification impact. |
| OPPO | Proposal 2: For UL time synchronization in connected mode, reuse legacy close-loop TA update procedure, and study the mechanism how to make gNB be aware of temporarily unavailable of GNSS capability at UE side. Proposal 3: For UL frequency synchronization in connected mode, enhancements on compensation of residual UE-specific frequency offset are not needed. |
| Fraunhofer | Proposal 2: RAN1 to study enhancements to timing correction closed loop mechanism to support an NTN operation in which GNSS signal is temporarily unavailable or performance degraded. |
| Tejas Network | Proposal 12: RAN1 to study enhancements to MAC CE TA signalling capability and investigate the inclusion of TA drift-related parameters (e.g., drift rate and drift variation rate) within the TA command. Proposal 13: It is recommended that RAN1 study the need for frequency adjustment commands, including the potential inclusion of frequency drift parameters (e.g., drift rate and drift variation rate). |
| Ericsson | Proposal 7 Study signalling enhancements (MAC CE) aiming at correcting time/frequency offsets in RRC_CONNECTED state, including: • Enhanced TA command (e.g., TA drift rate and TA drift variation rate) • Frequency adjustment command • Position adjustment command (indicating UE-specific reference location to the UE) Proposal 8 Study the GNSS operations specified for IoT-NTN in Rel-18 to see if any solutions can be adopted for GNSS-resilient operation of NR-NTN. Proposal 9 Study conjugated preambles and other methods to enable PDCCH order PRACH in the absence of GNSS information at the NTN UE. |
| ZTE | Proposal 13: To improve GNSS resilience in connected mode, indication of frequency command from network to UE can be supported. Proposal 14: To reduce overhead of TA command, indication of TA drift rate from network to UE can be supported. |
| ETRI | Proposal 11: RAN1 to define a baseline scheme for GNSS resilient operation among the following implementation-/non-3GPP-based solutions: - Option 0: NTN UL transmission is not allowed when GNSS is unavailable (Rel-19 UE behaviour) - Option 1: Dual-(multi-)GNSSs - Option 2: Dual-(multi-)frequencies - Option 3: OS-NMA - Option 4: UL time/frequency compensation via DL synchronization and signals - Option 5: LPP-based approach (note: no enhancements on LPP itself) - FFS, other options and/or a combination above |
| TCL | Proposal 3: The enhancement of the closed-loop time control can be studied, for example, TAC in RAR can indicate a negative value. Proposal 4: Frequency closed-loop adjustment can be considered for GNSS-resilient NR-NTN. Proposal 6: The enhancement of GNSS operation for IoT-NTN in Rel-18 can be considered whether it can be adopted for NR-NTN. |
| Honor | Proposal 8: RAN1 to study the enhancement in closed-loop timing adjustment on the basis of reference location during RRC connected mode. Proposal 9: RAN1 to study the following enhancements for frequency compensation during RRC connected mode: - For open-loop frequency control, the UE compensates the frequency based on the measurement of the downlink reference signal. - For close-loop frequency control, the NW indicates frequency corrections to the UE. Proposal 10: RAN1 to investigate closed-loop indication methods for drift in TA and frequency of the service link during RRC connected mode. |
| Nokia | Proposal 19: In case of temporary unavailable GNSS information during RRC_CONNECTED mode, a UE uses the latest measured GNSS location for calculation of NTA,adjUE and FO pre-compensation values for UL transmissions at least until instructed otherwise by the network. Proposal 20: RAN1 to investigate various options for implementing closed loop commands for UE maintaining UL TA and FO synchronization. Such options may consider at least (a) geo-location based tracking commands or (b) direct TA/FO commands. Proposal 21: RAN1 to study mechanisms for group common TAC in a single PDSCH for reducing signalling overhead. |
| InterDigital | Proposal 5: Support closed loop time-frequency compensation for GNSS resilient NTN operation. |
| IMU | Proposal 1: Study frequency closed-loop adjustment alongside TAC during/after RA for residual FO removal. Proposal 1: Allow an implicit indication of the UE pre-compensation basis. |
| Lenovo | Proposal 8: RAN1 to study the use of historic data (e.g., last acquired GNSS location) in connected state to predict the continuous change in propagation delay and Doppler errors over a shorter GNSS outages duration. Proposal 10: RAN1 to check the feasibility of configuring frequent timing and frequency correction commands by the network, when a UE experience a GNSS outages in connected state. |
| Apple | Proposal 4: For a UE in RRC connected mode without valid GNSS inforamtion, considering TA drift rate and drift vairant indication in TA command to reduce the signaling overhead. Proposal 5: For a UE in RRC connected mode without valid GNSS inforamtion, support the frequency offset indication from network. Proposal 6: For a UE in RRC connected mode, consider both “legacy open and closed loop TA mode” and “enhanced TA mode”, as well as node synchronization between UE and network. |
| TOYOTA | Proposal 8: To accommodate the GNSS resilient operation, RAN1 to study a unified framework for uplink time/frequency synchronization which encompasses initial access and connected state procedures, configurations and parameters. |
| MediaTek | Proposal 5: Study closed-loop frequency adjustment during initial access and RRC connected mode. Proposal 6: Study enhanced MAC CE TAC with TA drift. |
| China Telecom | Proposal 3: Support to study timing and frequency synchronization enhancing mechanism in NTN GNSS resilience. |
| Qualcomm | Proposal 8: RAN1 to study procedures to provide closed-loop frequency adjustments in connected mode. |
| NTT DOCOMO | Proposal 4: For Rel-20 NTN GNSS resilience, regarding Connected mode, support at least the following: - Enhancement on UL time synchronization - Study enhancements on TA indication, e.g., NW estimation/indication of UE specific TA, increased value range for TA command, etc. - Enhancement on UL frequency synchronization - Study frequency adjustment mechanism, e.g., NW-assisted frequency compensation. |
| google | Proposal 4: To reduce the TA command signalling overhead for GNSS resilience UE, study feasible indication range and step size of TA command. Proposal 5: For GNSS resilience UE, to perform Doppler shift compensation in RRC CONNECTED mode, study feasible measurement enhancements based on existing reference signals (e.g., CSI-RS, PT-RS, or NCD-SSB). |
| Johns Hopkins University | Proposal 4: RAN1 to study the use of downlink broadcast timestamps to estimate Doppler frequency. Proposal 5: RAN1 to study the use of downlink broadcast timestamps to estimate the one-way delay. |
| LGE | Proposal #9: In Rel-20 NR-NTN, RAN1 studies mechanisms to measure and compensate for UE-specific residual TO/FO errors in connected mode, including: - Uplink-based estimation methods - Downlink-based estimation methods Proposal #10: In Rel-20 NR-NTN, RAN1 studies whether/how to integrate UE-specific TO/FO correction control in connected mode. Proposal #11: In Rel-20 NR-NTN, RAN1 studies procedures for uplink synchronization validity and recovery in connected mode. |
| CSCN | Proposal 8: The closed-loop frequency synchronization mechanism should be introduced when using the reference location indication based common FO compensation scheme. |

## Summary of companies’ contributions

Most companies support a combination of network-assisted reference compensation, frequent (closed-loop) signaling, and intelligent fallback to historical or downlink-measured data for robust connected mode operation in GNSS-challenged scenarios. There is significant focus on signaling efficiency (e.g., drift indication, group commands), adaptive open/closed-loop methods including both timing and frequency closed loop control.

The proposed approaches:

Closed-loop & open-loop adjustment mechanisms

- Frequent/periodic closed-loop updates (network instructs UE via TA and frequency adjustment commands).

- Open-loop methods where network provides parameters for the UE to independently estimate necessary pre-compensation.

Signaling enhancements (MAC CE, TA, Frequency Command, Overhead Reduction)

- Improved MAC CE signaling (TA drift rate, frequency adjustment, negative TAC, etc.)

- Methods to reduce signaling overhead, such as group-common commands or indication of drift parameters.

Integrating Downlink/measurement-based methods

- Leverage downlink signals (reference signals, broadcast timestamps) for uplink TO/FO estimation and pre-compensation.

GNSS position & historical data use

- UE uses last acquired GNSS location or historic pre-compensation data during transient outages; network may signal when UE must switch to another method.

Adopting/adapting IoT-NTN solutions

- Study whether aspects of GNSS-resilient operation from IoT-NTN (as specified in previous releases) can be reused or adapted for NR-NTN.

## Initial proposal

### Proposal 8-1

Proposal 8-1-v0

For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for connected mode: - Enhanced TA command (e.g., TA drift rate and TA drift variation rate) - Frequency adjustment command in MAC CE i.e. closed-loop frequency control in connected mode Overhead reduction should be considered

Companies are encouraged to comment on Proposal 8-1-v0

| Company | Comments |
| --- | --- |
| CMCC1 | Fine with the proposal. But the details needs more discussion. For the enhanced TA command, is it the only way by using TA drift rate and TA drift variation rate ? In our thinking, the reference location can also be used for the calculation of TA and also for the tracking of TA. The variation of TA or the propagation delay between satellites and reference location can also be used to update the TA at UE sided. For the frequency adjustment command in MAC CE, we also see some proposal and candidate solutions not only focused on MAC CE based solutions. The updates to the proposal are as below. For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for connected mode: - Enhanced TA command (e.g., TA drift rate and TA drift variation rate) - Frequency adjustment command in MAC CE i.e. closed-loop frequency control in connected mode Overhead reduction should be considered |
| OPPO | We should firstly evaluate the potential issue on the signalling overhead in the connected mode before making the conclusion. |
| Spreadtrum | We agree with main sentence and have two comments on sub-bullets. For the first sub-bullet, “(e.g., TA drift rate and TA drift variation rate)” should be removed. Indication of TA drift related parameters is just one solution for enhancement of TA command and other solutions are not precluded. For the second sub-bullet, in our view, in GNSS resilient scenario, network need to indicate frequency adjustment command in connected mode, but whether use MAC CE or other signalings need more discussion. |
| CSCN | Support. The signalling overhead and potential solutions in connected mode should be discussed. |
| CEWiT | Support |
| ST Engineering iDirect | We have corrected the company proposal in 8.2, as our observations related to initial access were wrongly copied to this section. In R1-2508481, we present simulation results of enhanced closed loops when the timing and frequency error is signaled, rather than the correction or drift rate. As shown in our contributions, the signaling is greatly reduced when using higher-order tracking loops at the UE. The timing and frequency error can be combined in a single MAC CE. Delay error (left) and frequency error (right) for a LEO-600 pass, considering a timing and frequency error signaling with an update interval of 1 s. |
| JHUAPL | Support |
| Tejas Networks | Support the proposal |
| Huawei, HiSilicon | Too early to discuss about the detailed signalling. |
| Interdigital | We generally support the proposal from the FL. We think that UEs may refine their time frequency compensation based upon measurements over RSs. This can be used for overhead reduction for closed loop time frequency commands as well. |
| TCL | Support |
| Toyota ITC | We support connected state solutions. However, we prefer to focus on proper evaluation on connected state to identify how much corrections are needed in time and frequency (with results alignment among companies). |
| Ericsson | Add "…consider at least the following enhancements…". |
| LGE | We need to agree on the necessity for a solution and the issues first. For example, since TA drift corresponds to changes in relative distance, it physically contains Doppler information. Therefore, if TA drift is introduced, the necessity of additional FA command is unclear. It needs further discussion and clarification. |
| Panasonic | At this stage, we prefer more generic agreement like the following. For the study on GNSS resilient NR-NTN operation, RAN1 to consider a mechanism to maintain the time and frequency synchronization during RRC connected mode. Overhead reduction should be considered. |
| Lenovo | Support in general. We agree with Ericson to add “at least”. |
| Nokia | Support in principle – but it may be premature to reach a conclusion/agreement already now. One aspect to highlight here is that when mentioning “overhead reduction to be considered”, this is having multiple aspects: - UE power consumption (especially for non-reference position based approaches, we need both UL signals for the gNB to estimate and correction signals to be sent to the UE). - Network overhead. Unfortunately, given that we are covering a large geographical area, the compensation need to be performed potentially for a larger amount of UEs, meaning that we may risk having the entire network capacity spent on “tracking devices” and no capacity left for “data transmissions”. We would prefer removing the e.g. from the parenthesis from the enhanced TA command (e.g. TA drift rate and TA drift variation rate) such that focus is on the general principle, or alternatively start building a more extensive list of alternatives such that we do not only have one example. |
| Sony | Support, agree with CMCC modifications. |
| ZTE | Fine with the proposal. |
| Apple | Support |
| Ofinno | Support the proposal |
| Google | Support in general. But for frequency adjustment, we suggest to leave it open in the current stage. For the study on GNSS resilient NR-NTN operation, RAN1 to consider the following enhancements for connected mode: - Enhanced TA command (e.g., TA drift rate and TA drift variation rate) - Frequency adjustment command in MAC CE i.e. closed-loop frequency control in connected mode Overhead reduction should be considered |

# Topic#9 RAT dependent positioning method

Positioning based method is another solution discussed in companies contributions. This method is used to acquire a coarse UE location.

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| CMCC | Proposal 9: For evaluating positioning performance of GNSS resilient NR-NTN operation, the evaluation metrics and assumptions of RAT dependent positioning methods study in Rel-18 NR NTN can be considered as a start point. Observation 5: Single satellite with 4 measurement occasions and 3 RSTDs can achieve 4.23km and 2.02km positioning accuracy (or positioning uncertainties) in 24s and 36s. Observation 6: Multiple satellites with at least two measurement occasions can achieve 4.34km and 2.23 km positioning accuracy in 4s and 8s. And with larger time intervales, the positioning errors would be even lower to 1km and less than 1km. Proposal 10: It is proposed to study the RAT-dependent positioning to improve the GNSS resilience. - DL-TDOA positioning method via PRS measurement with single satellite can be prioritized for the evaluation of GNSS resilient NR-NTN operation. |
| ST Engineering iDirect | Observation 17: Solutions based on DL-TDOA will not work for GEO and MEO deployment scenarios. Observation 18: UE positioning via DL-TDOA using signaling from a single LEO satellite is not feasible, because of the long acquisition time. Observation 19: UE positioning via DL-TDOA using signaling from multiple LEO satellites has the following implementation challenges - Satellites need to be synchronized - Poses constraints on the satellite constellation - Even for a large constellation, the number of visible satellites can be limited due to building and vegetation blockage - For higher frequencies, directive antennas are used, which limits the number of visible satellites Observation 20: In connected mode, time and frequency tracking using closed loops is less complex than position-based solutions using DL-TDOA. Proposal 12: Do not consider DL-TDOA for the Rel-20 5G-A study on GNSS-resilient NR NTN operation, but consider PNT in 6G. |
| Huawei | Proposal 6: UE can acquire coarse location information from multiple satellite DL-TDOA and make TA/FO pre-compensation for PRACH transmission. Observation 15: In LEO-600 and LEO-1200, UE can see at least 2 satellites in most of serving time. The larger constellation size, the more satellites UE can see. The higher latitude UE locates, the more satellite UE can see. Observation 16: For LEO-1200 constellation (263 satellites in total), UE can achieve with 90% probability the location uncertainty of 1.29km by measuring DL-TDOA from 3 satellites and 0.15km by measuring DL-TDOA from 4 satellites. Observation 17: Under LEO600 constellation with 800 satellites, if a UE can observe 4 satellites, the horizontal location uncertainty of 53m for 90% of UEs can be achieved; Under LEO600 constellation with 1400 satellites, if a UE can observe 5 satellites, an accuracy of 33m for 90% of UEs can be achieved. Observation 18: When employing the DL-TDOA to acquire coarse UE location, the differential Doppler and delay can be significantly reduced compared to the values in scenario 1 + case a, which can be supported by legacy preamble formats when certain location accuracy is met. |
| CATT | - The probability not less three visible satellites exceeds 62%, which can meet the requreiment of 2D positioning. - Considering the UE clock drift, under conditions of a 16-second measurement interval and 16 measurement instances, 90% of UEs achieve a positioning accuracy of 9.38 km with an associated latency of 240 seconds. - Eliminating clock drift can improve the 90th percentile positioning accuracy from 76.11 km to 2.78 km with a significant enhancement for the case of measurement gap 16s and 4RSTD. - For two visible satellites positioning, the 90th percentile positioning accuracy reaches 7.06 km when taking into account UE clock drift. - For two visible satellites positioning, the 90th percentile positioning accuracy improves to 2.69 km if without UE clock drift. - In the three-satellite positioning scenario, 90% of UEs achieve a positioning accuracy of 77.32 meters. - When NTN UE does not rely on PRS measurement window of current specification, the restriction of differential propagation delay of multiple satellites is avoided by UE autonously detection. - From the perspective of system complexity, a UE-based DL-TDOA positioning solution is preferred to help the position information acquisition. - It is recommended to implement clock drift compensation or calibration mechanisms to improve single-satellite positioning accuracy. - Allow UE to autonomously detect multi-satellite PRS without PRS measurement window restriction to execute positioning operation. - For multi-satellite positioning, the mutual interference among PRS from different satellites can be considered to mitigate. - For multi-satellite positioning, the network should support multi-beam overlapping coverage within designated service areas. |
| ESA | Observation 5: With a 20° elevation mask, a UE always sees ≥3 satellites globally, and ≥4 satellites at mid- and high latitudes, ensuring strong positioning availability. Note, the serving satellite (for communication services) is assumed to always be above 30 degrees elevation angles as per TR 38.821 scenario. Observation 6: With a 30° elevation mask, global 3D availability with ≥4 satellites cannot be always guaranteed; only mid- and high latitudes achieve reliable coverage. Observation 7: The barometric pressure sensor method, already specified in LPP (TS 37.355), enables 3D positioning even when only three satellites are visible. Observation 8 - When considering minimum synchronization error, NTN positioning can fully substitute GNSS for initial NTN access, since horizontal positioning error is below 50m at any user latitude (for user masking angle of 20°) and below 100m at any user latitude (for user masking angle of 30°). - When considering 50ns synchronization error, NTN positioning can serve as enabler for relaxed GNSS resilient NTN access solutions, since horizontal positioning errors approximately below 1km are found for all cases. Observation 9: Coarse localization based on multi-satellite DL-TDOA, complemented by barometric pressure sensor, could shrink location uncertainty area in Case b (typically, better than few hundreds of meters according to Table 6 and Table 7) and reduces the probability of Case a, though not eliminating it as availability for 3 or more satellites cannot be guaranteed everywhere in the world if minimum user elevation angle is 30 degrees also in reception of DL-PRS (according to Table 4). Observation 10: In the single-satellite case, accumulating measurements over the entire pass yields the best theoretical accuracy; however, this is not practical, as the satellite would have completed its pass before the UE reaches maximum accuracy. Moreover, UE clock drift, not yet included in the preliminary results, would further degrade performance over such long durations. Observation 11: To limit UE clock drift effects, 1-minute measurement sessions have been analysed. The resulting positioning accuracy shows a strong dependence on latitude, with poorer results at lower latitudes due to limited geometric variation between the UE and the satellite. Further investigation is needed to assess whether acceptable accuracy can be achieved at certain latitudes, possibly in combination with enhancements to PRACH procedures. Integration periods longer than 1 minute (but shorter than a full pass) may improve performance, though VSAT clock drift will increasingly contribute to residual errors. Proposal 1: For DL-TDOA, evaluate positioning performance based on the Release 18 Network Verified Location methodology [5] – extended to the multi-satellite case. Proposal 2: As part of a toolbox approach (i.e., multiple solutions not just one) for GNSS-resilient NTN operation, it is proposed to enable coarse localization based on DL-TDOA to support TA/FO pre-compensation for uplink transmission. Note, this method is intended as a complementary aid, not a replacement to other relevant methods discussed as part of this study item. Proposal 3: Assess feasibility and implementation aspects of DL-TDOA positioning for GNSS resilient operations. Proposal 4: Capture the performance assessment for DL-TDOA in the study TR. |
| OPPO | Observation 2: Using existing NR positioning, the positioning accuracy error is within 173 m at 95% CDF point. This ensures the timing offset is within CP length and the frequency error is within 0.1 ppm at 2GHz carrier frequency. Proposal 4: For GNSS resilience operation, RAN1 considers UE using existing NR positioning to obtain UE location in Rel-20. |
| Airbus | Observation 1: Coarse UE localization with RAT-dependent positioning can be performed in NR-NTN without any specification enhancement and used in combination with other solutions for GNSS resilient NR-NTN operation. Observation 2: For the example LEO constellation with minimum user elevation of 20°, the visibility of at least 3 satellites is ensured at any user latitude. Observation 3: For the example LEO constellation with minimum user elevation of 30°, the visibility of at least 3 satellites significantly depends on the user latitude. Observation 4: A minimum of 3 satellites is sufficient to perform horizontal DL-TDOA positioning, e.g. by using a barometric pressure sensor. Observation 5: For the example LEO constellation with minimum user elevation of 20°, the horizontal DL-TDOA positioning accuracy at 95% with at least 3 satellites is ensured to be below 200 m at any user latitude. Observation 6: For the example LEO constellation with minimum user elevation of 30°, the horizontal DL-TDOA positioning accuracy at 95% with at least 3 satellites is ensured to be below 750 m at any user latitude. Observation 7: RAT-dependent positioning can significantly reduce the UE location uncertainty, resulting in a significant reduction of the time-frequency uncertainty for NR-NTN initial access. Proposal 1: RAT-dependent positioning is proposed to complement solutions for initial access in GNSS resilient NR-NTN operation, by significantly reducing the UE location uncertainty. Proposal 2: Multi-satellite DL-TDOA should be prioritized as RAT-dependent positioning method for coarse UE localization and reduction of UE location uncertainty. |
| Amazon | Observation 4: In addition to PRACH robustness improvements that enable quick and reliable connection establishment to the network under large time-frequency uncertainties, position knowledge acquisition by the UEs is still necessary for GNSS resilient operations. Proposal 5: To investigate the adequacy and requirements that UE position acquisition methods based on, e.g., DL-TDOA and two-way ranging (TWR) impose on the transmission of reference signals such as SSB, PRS, PRACH and UL-RS, particularly in UE mobility scenarios and considering realistic LO models. |
| TCL | Observation 1: RAT-dependent positioning can be considered for NTN positioning. Observation 2: Considering the positioning accuracy, Time-based positioning methods can be considered for UE-based positioning to access the network. Observation 3: If single-satellite positioning is adopted for UE-based positioning, the position accuracy and access latency should be considered. Proposal 1: For multiple-satellite positioning, each satellite can be considered as a TRP. Proposal 2: For multiple-satellite positioning, the number of satellites for positioning should be decided. |
| InterDigital | Observation 7: If the UEs are able to acquire and maintain their location through NR based signals, the legacy procedures, e.g., time and frequency compensation, timing adjustments, etc., may be applied similar to the legacy NR NTN operation. Observation 8: The UEs need to acquire the configuration of positioning reference signals from multiple satellites to determine UE location based upon NR signals. Observation 9: The UEs being able to receive and estimate the signals from multiple satellites for location estimation will result in significant processing complexity at the UE side. Observation 10: NR NTN satellites transmitting RSs to enable UE location determination will result in significant overhead and will lead to poor spectral efficiency for the system. Observation 11: UE location estimation based upon NTN satellites transmitting RSs will require dense satellite constellations designed so that the UEs in different locations may receive and measure signals from multiple satellites. Proposal 8: Do not further study RAT based positioning for GNSS resilient NTN operation. |
| IMU | Observation 1: Single-satellite positioning can be considered for GNSS-resilient NR-NTN using time-separated measurements within a pass. Observation 2: The required inter-epoch interval is geometry-dependent (line-of-sight rotation during the pass), not a fixed constant. Observation 3: If single-satellite positioning is adopted for UE-based positioning, position accuracy vs. access latency must be jointly considered. |
| Ofinno | Observation 8: The LPP specification (TS 37.355) supports signaling and procedures related to the UE-based DL-TDOA for determining the UE position. Observation 9: For determining positioning based on DL-TDOA, the UE needs to send a request to LMF via LPP to receive DL-TDOA assistance data from the LMF. Observation 10: Current specifications (e.g., TS 38.133) specify only UE requirements related to the RSTD measurements and don’t specify UE requirements related to UE-based DL-TDOA. Proposal 5: Study the feasibility of UE-based DL-TDOA for determining the location position during initial access procedure across all the relevant working groups (RAN1, RAN2, and RAN4). Observation 11: The solution based on multiple PRACH transmission attempts is likely to increase initial access delay. |
| ViaSat | Observation 1: Always-ON synchronization signals can be used by a UE with no GNSS location information to derive a coarse location estimate by making multiple measurements with sufficient time difference even with a single LEO satellite in its view. Observation 2: There are many ways to estimate UE location and derive a Timing Advance estimate based on synchronization signals. The specific algorithm(s) can be left for UE implementation. Proposal 1: It is proposed to study UE coarse location estimation based on always-ON synchronization signals (PSS/SSS) from a single satellite and use the Timing Advance estimate for improving uncertainty in the RACH procedure. Proposal 2: Study performance improvement for location estimation based on measurements from multiple satellites. Proposal 3: Study impact of coarse timing advance estimate from synchronization signals on RACH procedure enhancement methods. |
| Qualcomm | Observation 12: Enabling DL-only UE-based positioning solutions to reduce time-frequency uncertainty using measurements from multiple satellites depends on satellite constellation characteristics. Observation 13: Enabling DL-only UE-based positioning solutions to reduce time-frequency uncertainty using measurements from single satellites is not feasible because: - The UE requires RSTD measurements over long time window (e.g. >24 seconds). This will introduce significant latency before random access to NTN cell for GNSS-resilient UEs. - Even with larger time windows, whether the UE will be able to reduce the time-frequency uncertainty to the range that can be handled by the existing PRACH formats depends on UE local oscillator errors. |
| Google | Observation 2: Uncertainty area reduction can be achieved by UE implementation, for example, observing Doppler shift and TDOA variation by measuring existing signals, or determining a narrowed uncertain area according to the last acquired GNSS and accelerometer sensor. |
| CEWiT | Proposal 4: The UE should be able to estimate the location using SSB and SIB19 when GNSS temporarily unavailable. UE should estimate TDoA using SSB from multiple satellites to estimate its location. UE specific timing advance estimated using UE location and the ephemeris information from the SIB19. The TDoA estimation at the UE is implementation specific, and it will not use positioning framework of NR. Proposal 5: The UE should employ multilateration techniques to determine its location based on measurements from multiple satellites. The determined location is used to calculate the corresponding TA. |

## Background

## Summary of companies’ contributions

Several companies propose that multi-satellite, RAT-dependent positioning (primarily through DL-TDOA) can provide reliable coarse location estimates, supporting GNSS-resilient NR-NTN operations. Proposals emphasize methods for coarse location and TA/FO compensation, while some highlight complexities (visibility, latency in case of single satellite in view) and recommend limiting the scope for certain deployment scenarios. This a recap of companies inputs regarding DL-only positioning method for GNSS-resilient NR-NTN operations:

| Company | Key proposals | Key observations |
| --- | --- | --- |
| CMCC | Study RAT-dependent positioning (DL-TDOA, PRS); prioritize single satellite DL-TDOA evaluation. | More measurements/satellites improve accuracy; multi-satellite scenarios can achieve <1km accuracy with larger intervals. |
| ST Engineering iDirect | Exclude DL-TDOA for Rel-20, GEO/MEO; consider PNT for 6G. | DL-TDOA faces synchronization, visibility, antenna constraints; complex vs. time/frequency tracking; single LEO not feasible. |
| Huawei | Use multi-satellite DL-TDOA for coarse location and TA/FO compensation. | Large constellations, especially at higher latitudes, enable high multi-satellite visibility; sub-km to meter-level accuracy possible. |
| CATT | Prefer UE-based DL-TDOA; add clock drift compensation; autonomous PRS detection; mitigate PRS interference. | Three-satellite cases yield high accuracy; clock drift impacts results; multi-beam coverage recommended. |
| ESA | Evaluate DL-TDOA per Rel-18; enable coarse localization as complementary aid; feasibility assessment. | 3+ satellites generally visible; barometric sensor aids vertical positioning; <50-100m accuracy possible with minimum sync error. |
| OPPO | Use existing NR positioning for GNSS-resilient NR NTN. | NR positioning provides <173m error (95% CDF); sufficient timing/frequency accuracy for RACH. |
| Airbus | Use RAT positioning to reduce location uncertainty and facilitate access; prioritize multi-satellite DL-TDOA. | At least 3 satellites visible at lower elevation; sub-200m accuracy globally with 3 satellites. |
| Amazon | Investigate DL-TDOA/TWR for UE position acquisition and requirements for reference signal transmission. | Position knowledge necessary in addition to PRACH improvement, especially under large time-freq uncertainty and mobility. |
| TCL | Multi-satellite RAT-based positioning; determine satellite count based on needs. | Time-based multi-satellite positioning improves accuracy; trade-offs for single-satellite methods. |
| InterDigital | Recommend not to further study RAT-based positioning for resilience. | High UE/system complexity and overhead; dense constellations needed for effectiveness. |
| IMU | Single-satellite positioning with time-separated measurements; consider accuracy vs. access latency trade-off. | Inter-epoch interval is geometry-dependent, not fixed; access latency vs. accuracy is a key consideration. |
| Ofinno | Feasibility study of UE-based DL-TDOA for initial access across groups. | Existing LPP supports DL-TDOA signaling; multi-PRACH attempts may increase access delay. |
| ViaSat | Study always-ON sync signals for coarse location; analyze impacts on RACH; measure via single/multiple satellites. | Multiple measurements with single LEO can provide coarse location; various algorithms possible. |
| Qualcomm | DL-only multi-satellite measurements needed; single satellite DL-TDOA is impractical due to latency. | Multi-satellite positioning depends on constellation; single satellite requires long measurement windows and high latency. |
| Google | Reduce uncertainty via UE Doppler/TDOA measurement and sensor data. | Uncertainty area reduction possible via UE implementation and sensor fusion (Doppler, GNSS, accelerometer). |
| CEWiT | UE estimates position via SSB/SIB19 and multilateration; TA derived from position and ephemeris info. | UE can estimate location using SSB from multiple satellites outside NR positioning framework; multilateration recommended. |

## Initial proposal

### Proposal 9-1

Proposal 9-1-v0

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA. - This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications. - Evaluation assumptions and parameters should be defined during this meeting.

Companies are to comment on Proposal 9-1-v0

| Companies | Comments |
| --- | --- |
| CMCC1 | Support the proposal in general. |
| OPPO | Support. |
| CSCN | Fine with the proposal. |
| CEWiT | Support |
| ST Engineering iDirect | We believe that this proposal should be part of Proposal 7-1. Single satellite DL-TDOA is impractical due to latency. The implementation of multi-satellite DL-TDOA in Ka band frequencies is impractical due to synchronization, visibility, antenna constraints. Therefore, we propose to de-prioritize RAT-dependent positioning methods in this study. |
| Tejas Networks | This is a specific solution within the Solution list under reducing time/frequency ambiguity specifically solution 2A . First we should decide upon the potential solutions which should be prioritized and further studied and then study the feasibility , performance and potential impacts on existing specifications for all potential solutions as per proposal 7-1 rather than focusing on a single solution at this point of time |
| Huawei, HiSilicon | UE-based positioning is related to service, we prefer the following wording for the main bullet. For GNSS-resilient NTN operation, study DL-only UE-based positioning method, coarse location acquisition using DL-TDOA. |
| Eutelsat | Coarse location methods should be considered. Propoents of PNT should note that typically a UE/ VSAT UE will not be illuminated by multiple SAN. Considerations include diverting Tx power to positioning signals and PFD limits (with multiple SAN illuminating the same area). |
| TCL | We think the numbers of satallites and the latency for single satellite positioning should also be considered. In theory, the more satellites are used for positioning and the higher accuracy can be achieved. The measurement gap under the single satellite is huge according to the reult in Rel-18. |
| Toyota ITC | Agree with ST Engineering iDirect. Should clarify this solution and can be discussed under Proposal 7-1. |
| Ericsson | Fine to study. Our view is that DL-TDOA-based positioning will not work in many deployment scenarios (e.g. since large constellations are required), and therefore a more robust solution will anyway be needed in addition. We should avoid specifying multiple solutions to the same problem. |
| Panasonic | Support. |
| Airbus | Support. The DL-TDOA UE-based positioning method needs to be studied to address its support for GNSS-resilient NTN operation, in terms of feasibility and achievable performance. Initial contributions show the feasibility to use multi-satellite DL-TDoA without specification impact and as a complementary solution, by providing coarse UE location in case of GNSS unavailability. The evaluation assumptions and parameters from those initial contributions could be considered as baseline. |
| Nokia | We would be OK to study this further, but we are not confident that we will get results that would be applicable for all deployment scenarios. In our preference this should be performed with lower priority. |
| Sony | Fine for companies to study further, but agree with assessment of ST Engineering and Nokia that this approach may be limited. Open to simplified ideas based on DL signal reception. |
| ZTE | OK to study if the intention is just to evaluate the accuracy. |
| ESA | Support. We agree to define the evaluation assumptions. |
| Ofinno | This is part of 7-1 under 2A. We agree with ST Engineering. |
| Google | Support |

# Topic#10 GNSS Status reporting and control

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| vivo | Error: Reference source not found Proposal 14: Study the GNSS status report to help NW decide whether to transmit TA command and FO command, considering signalling overhead reduction. |
| CMCC | Proposal 12: The assistance information on GNSS based UE location validity duration can be reported by UE. |
| Huawei | Proposal 13: UE should report the availability and validity of GNSS position information in GNSS resilient operation. |
| Fraunhofer | Proposal 9: RAN 1 to study mechanisms to assist an NTN UE evaluating the reliability of an available GNSS signal. |
| Tejas Network | Proposal 8: RAN1 should study Network assisted or UE assisted methods to detect the GNSS unavailability in a UE and the corresponding signalling aspects |
| Ericsson | Proposal 1 Study network-assisted and UE-assisted methods to detect a loss of GNSS information in a cell or UE. |
| ZTE | Proposal 12: Network supporting relaxed requirement need to indicate that UE without valid GNSS position is allowed to access. Proposal 15: Status report for GNSS should be supported when UE without valid GNSS position is allowed to transmit in connected mode. Proposal 16: gNB need to indicate UE the extension time for UL transmission when UE without valid GNSS position is allowed to transmit in connected mode. Proposal 17: gNB triggered GNSS update should be supported when UE without valid GNSS position is allowed to transmit in connected mode. |
| TCL | Proposal 5: The remaining validity duration report for GNSS should be supported for both random access or connected mode. Proposal 7: The state of GNSS can be reported implicitly via the PRACH resources, such as RO or preamble during random access. |
| Honor | Proposal 11: RAN1 to study signalling solutions to enable the network to be aware of the UE’s GNSS status, e.g., the UE may need to report its GNSS status when the latest GNSS based UE location accuracy has degraded or become invalid. |
| InterDigital | Proposal 6: Support UE reporting of its GNSS availability state to the network. Proposal 7: Support network indication for the UEs to validate their GNSS availability and/or trigger GNSS resilient NTN operation. |
| Lenovo | Proposal 9: RAN1 to study the mechanism for indication of GNSS connectivity loss in connected mode. |
| Ofinno | Proposal 9: The GNSS reception can indicate information such as: whether the UE is currently in scenario #1 or in scenario #2, time when the UE last acquired GNSS, time when the UE moved to scenario #1 or scenario #2, etc. ## Proposal 10: The UE indicates its GNSS reception status to the network. |
| China Telecom | Proposal 1: Support reporting UE GNSS status to the network, and discuss how and when to trigger GNSS status report from UE side, if supported. |
| Qualcomm | Proposal 11: RAN1 to study signaling/procedures for handling the loss/gain of GNSS-based location during connected mode. |
| NTT DOCOMO | Proposal 5: Study how to determine whether the GNSS-resilient operation or GNSS-based operation is applied, e.g., - Based on NW indication - Based on UE measurement, and - E.g., request or report to NW with or without confirmation response - E.g., UE decision with conditions |
| LGE | Proposal #12: In Rel-20 NR-NTN, RAN1 studies whether/how to determine GNSS positioning accuracy degradation. Proposal #13: In Rel-20 NR-NTN, RAN1 studies potential impacts from switching between GNSS-based operation and GNSS-resilient operation. |

## Summary of companies’ contributions

Here is a recap of companies' proposals and observations on GNSS status reporting, mode transition, and related signaling:

| Company | Proposals/Observations |
| --- | --- |
| Vivo | Study mechanisms for GNSS mode transition; study GNSS status reporting to optimize TA/FO commands and reduce overhead. |
| CMCC | UE can report validity duration of GNSS-based location assistance information. |
| Huawei | UE should report GNSS position information availability and validity in GNSS-resilient operation. |
| Fraunhofer | Network may provide assistance info indicating GNSS signal reliability (e.g., spoofing); study mechanisms to assess GNSS reliability. |
| Tejas Network | Study network- or UE-assisted methods to detect GNSS unavailability and related signaling aspects. |
| Ericsson | GNSS loss may impact cells or individual UEs; study network- and UE-assisted detection methods for GNSS loss. |
| ZTE | Support network indication for UEs without valid GNSS; enable status report, UL transmission extension, triggered GNSS update. |
| TCL | Support reporting of GNSS validity duration in both random access and connected mode; implicit GNSS state reporting via PRACH. |
| Honor | Study signaling to make network aware of UE’s GNSS status, especially when GNSS-based location degrades or becomes invalid. |
| InterDigital | Support UE reporting of GNSS availability; network can indicate need for validation or trigger resilient NTN operation. |
| Lenovo | Study mechanism for GNSS connectivity loss indication in connected mode. |
| Ofinno | Network awareness of UE GNSS reception status is beneficial; UE to indicate current scenario, last GNSS acquisition, status. |
| China Telecom | Support UE reporting GNSS status; study triggering conditions and timing for GNSS status reports. |
| Qualcomm | Study signaling/procedures to handle GNSS location loss/gain during connected mode. |
| NTT DOCOMO | Study determination of GNSS-based or GNSS-resilient operation (by network indication or UE measurement/report); include request/response process. |
| LGE | Study indicators/techniques for GNSS positioning accuracy degradation and impacts of operation mode switching. |

## Initial proposal

### Proposal 10-1

Based on the above discussion, the following initial proposal is made:

Proposal 10-1-v0

RAN1 to study network/UE methods for detecting GNSS loss, including reporting of GNSS status and mechanisms for transitioning operational modes e.g., from a GNSS based mode to GNSS resilient mode.

Companies are encouraged to comment on Proposal 10-1-v0

| Companies | Comments |
| --- | --- |
| CMCC1 | In general fine with the proposal. The GNSS module is within or at UE side, NW have no knowledge whether or not UE can acquire the GNSS position. The mechanism that UE reporting the GNSS issue should be supported. The different status of UE working with GNSS and without GNSS should be discussed. |
| OPPO | Support. |
| Spreadtrum | Support |
| CSCN | Support |
| CEWiT | Support |
| ST Engineering iDirect | The proposal should focus on the reporting, rather than the detection of GNSS loss. |
| Tejas Networks | Support |
| Huawei, HiSilicon | We don’t need to study the mechanisms of network/UE methods for detecting GNSS loss as it is out of 3GPP scope, for the proposal we prefer to the following wording: RAN1 to study network/UE methods for detecting GNSS loss, including reporting of GNSS status and mechanisms for transitioning operational modes e.g., from a GNSS based mode to GNSS resilient mode. |
| Interdigital | We support the proposal from the FL. |
| Eutelsat | Support |
| TCL | We think the report of GNSS state or change of the GNSS state should be considered for both connected mode and RA procedure. |
| Toyota ITC | Support in general. Mechanisms for transitioning operational modes may be required. |
| Ericsson | Support |
| LGE | We are supportive to study but the proposal should be more general as follows: RAN1 to study network/UE methods for detecting GNSS loss, including reporting of GNSS status and mechanisms for transitioning operational modes e.g., from a GNSS based mode to GNSS resilient mode. |
| Panasonic | Support. Although “mode” is described in the proposal as e.g., whether to define a “mode” for GNSS based operation and GNSS resilient operation needs further discussion. |
| Lenovo | We agree with Huawei comments and would like to additionally add “in connected state”, since such reporting mechanism would only be feasible in connected state. |
| Nokia | OK to study. As far as we see the situation, the most fundamental aspect to resolve is to make both UE and network aware of “degraded performance due to lack of proper GNSS based geo-location”, such that network knows how to handle the entire situation. UE should preferably not start doing any actions autonomously. |
| NEC | General fine. |
| Sony | Support. |
| ZTE | As the GNSS module is equipped on GNSS, UE will know the status of GNSS status instead of network. Hence, the GNSS status should be determined by UE instead of network. |
| Apple | Support |
| ETRI | Support |
| Ofinno | We support the intent of the proposal. But it is too soon to limit the contents of what to report. We propose the following: AN1 to study network/UE methods for detecting GNSS loss, including reporting of GNSS status and mechanisms for transitioning operational modes e.g., from a GNSS based mode to GNSS resilient mode. |
| Google | Support |
| Futurewei | Support |

# Topic#11 Coexistence with Legacy UEs

## Background

## Companies’ proposals

| Companies | Proposals |
| --- | --- |
| OPPO | Proposal 5: Separate PRACH configurations including PRACH formats and PRACH occasions for GNSS capable and GNSS unavailable UEs. |
| Tejas Network | Observation 16 :The NTN Cell can contain both GNSS available UEs and GNSS unavailable UEs and separate PRACH formats and separate PRACH occasions may be configured. |
| Ericsson | Observation 2 An NTN cell may need to simultaneously support UEs with and without temporary loss of GNSS information. Observation 14 Separate PRACH resources may be needed to support initial access for UEs with and without GNSS information in a cell. |
| Honor | Proposal 6: RAN1 to study coexistence strategies between GNSS-dependent and GNSS-unavailable UEs, including configuring separate PRACH resources during initial access for UEs with and without valid GNSS-based location. |
| Lenovo | Proposal 5: RAN1 to study the necessary enhancements to RACH resources for UEs with and without GNSS to avoid unnecessary delays for UEs with GNSS. |
| Apple | Proposal 3: Different RACH resources are used for UEs without valid GNSS information and for UEs with valid GNSS information. |
| Ofinno | Proposal 2: Introduce separate PRACH resource sets: - a first set (Set 1) for UEs whose GNSS is available, - a second set (Set 2) for UEs in Scenario # 1 (i.e., which cannot rely on GNSS), and - a third set (Set 3) for UEs in Scenario # 2 (i.e., which didn’t receive GNSS since time duration T). |

## Summary of companies’ contributions

Several companies have highlighted the need to differentiate PRACH/RACH resources for UEs based on their GNSS capability. Proposals from OPPO, Honor, Lenovo, Apple, and Ofinno, supported by observations from Ericsson and Tejas Network, consistently recommend separate PRACH formats, occasions, or resource sets for GNSS-capable and GNSS-unavailable UEs within NTN cells. This approach is intended to ensure optimized and efficient initial access for all UEs, preventing unnecessary delays for GNSS-capable devices while accommodating those without valid GNSS information.

## Initial proposal

### Proposal 11-1

Proposal 11-1-v0

Study coexistence strategies between legacy (GNSS-dependent) and GNSS-unavailable UEs, including the separation of PRACH resources.

Companies are encouraged to comment on Proposal 11-1-v0

| Companies | Comments |
| --- | --- |
| CMCC1 | Support the proposal. It is possible that legacy UE with GNSS and the UEs of GNSS resilience work in the same cell. The study of coexistence between legacy UE and GNSS unavailable UEs should be considered. |
| OPPO | Support |
| Spreadtrum | Support |
| CSCN | Support |
| CEWiT | Support |
| ST Engineering iDirect | Support, with the suggestion to rephrase as: “Study coexistence strategies between legacy UEs (having an accurate position based on GNSS) and UEs without an accurate GNSS position, including the separation of PRACH resources.” |
| Tejas Networks | Support |
| Huawei, HiSilicon | Support |
| Interdigital | We support the proposal from the FL. |
| Eutelsat | Support |
| TCL | Support |
| Toyota ITC | Support in general. Note that separation of PRACH resources is only one solution and proposal can be updated to reflect it. |
| Ericsson | Support |
| LGE | We are supportive to study but the proposal should be more general as follows: Study coexistence strategies between legacy (GNSS-dependent) and GNSS-unavailable UEs, including the separation of PRACH resources. |
| Panasonic | Support. |
| Lenovo | Support |
| Nokia | Coexistence and efficient coexistence with UEs that are able to behave well (i.e., perform proper pre-compensation) is essential. We should never have any UEs that are able to destroy performance (or efficiency) for the well-behaved UEs. Whether separation of PRACH resources is really needed should be further discussed and proved, as we don’t fully agree with it. |
| NEC | Support |
| Sony | Support. |
| ZTE | Generally fine. But it is preferred to change “GNSS-unavailable” to “GNSS-temporarily-unavailable” to be aligned with description in SID. |
| Apple | Support |
| Ofinno | We support it in principle. We agree with ST Engineering. We should cover UEs in both Scenario 1 and Scenario 2. |

# Topic#12 TR 38.742 skeleton

## Initial proposal

### Proposal 12-1

Proposal 12-1-v0

Endorse the skeleton for TR 38.742 proposed in R1-2508479 [4].

Companies are encouraged to comment on the skeleton for TR 38.742 proposed in R1-2508479 [4].

| Companies | Comments |
| --- | --- |
| ST Engineering iDirect | Suggest to split Section 7 in two parts: - Evaluation of solutions for GNSS-resilient NR-NTN operation during initial access - Evaluation of solutions for GNSS-resilient NR-NTN operation in RRC connected As there might be multiple solutions, we propose to have a (sub)section per solution, and discuss evaluation and spec impact per solution. Proposal to add a section dedicated to evaluation methodology. |
| Toyota ITC | Support ST Engineering iDirect. Suggest splitting the solutions and evaluations to two sections. - Evaluation of solutions for GNSS-resilient NR-NTN operation during initial access - Evaluation of solutions for GNSS-resilient NR-NTN operation in RRC connected |
| Panasonic | Support. |
| Nokia | OK |
| Sony | Support. |
| ZTE | We have some comments on the skeleton. - Regarding clause 4, it is enough to directly reference to the SID. No need to introduce clauses for specific description of backgrounds and motivations. - To align with the SID, “GNSS unavailability” can be changed to “GNSS temporarily unavailable”, e.g., in clause 6 - Regarding clause 7.3, which RAN WGs are included need to be clarified. - As we are still studying, it is not clear what open issues and future work are. Moreover, whether there will be normative work depends on the RAN plenary discussion at check point. Hence, it’s too early to have clauses 8.2 and 8.3 at current stage. |
| Futurewei | Recommend to add a new subsection for evaluation results. |

# Proposals for Monday offline

- Proposal 1-2

Proposal 1-2-v0:

For the calculation of one-way differential delay and one-way differential Doppler, adopt Alt2 with the following correction on the one-way Doppler:

dopplerone-wayu=vc2fcu-s,x>done-wayu+vc2fcu-s,k>done-wayu

Note: Earth’s rotational motion is neglected for LEO.

- Proposal 3-3

Proposal 3-3-v0:

The inequation for the limit on differential roundtrip delay, agreed at RAN#122bis is not applicable for unrestricted set.

For unrestricted set PRACH tolerance is exceeded if

min (Ncp, Sequence duration) -2Te ≤ Differential RTT.

Proposal 3-4-v0:

For PRACH performance evaluation, Te used for the limit on differential roundtrip delay determination

is to be set according to TS 38.133, Table 7.1.2-1.

Note: the impact of Te is marginale.

Proposal 3-5-v0:

For UL performance evaluation, scaled one-way differential Doppler/frequency offset  (in kHz) to determine UL differential Doppler/frequency offset is calculated as follows:

scalingfactor=1+fc,ulfc,dl.

fc,ul and fc,dl  are the uplink and downlink carrier frequency respectively.

Note:  the differential doppler is based on the DL.

- Proposal 5-1

Proposal 5-1-v0

For the study of GNSS resilient operation, if the UE cannot rely on its GNSS for timing and frequency compensation on the service link, it is assumed that the UE pre-compensates the common RTT and common Doppler on the service link, based on assistance information indicated by the Network. This assistance information could be:

- A reference location e.g. in case of (quasi)-Earth fixed cell

- A Common TA/Common Doppler relative to service link e.g. in case of moving cell

- Proposal 9-1

Proposal 9-1-v0

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA.

- This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications.

- Evaluation assumptions and parameters should be defined during this meeting.

# Proposals for Tuesday offline

Proposal 2-3

Proposal 2-3-v1:

Working assumption

Capture in the TR 38.742 the following tables of differential one-way delay reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.339 |  |  | 357.420 |
|  | 30 | 143.308 | 257.815 |  | 57.610 | 115.088 |  |
|  | 90 | 1.908 | 3.343 | 4.815 | 0.304 | 0.661 | 0.931 |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.859 |  |  | 906.662 |
|  | 30 | 255.312 | 535.290 |  | 143.065 | 257.081 |  |
|  | 90 | 6.131 | 14.885 | 15.548 | 1.909 | 3.307 | 6.038 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.174 |  |  | 357.432 |
|  | 30 | 142.847 | 257.067 |  | 57.543 | 114.991 |  |
|  | 90 | 1.900 | 3.342 |  | 0.304 | 0.660 |  |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.827 |  |  | 906.657 |
|  | 30 | 255.268 | 535.228 |  | 143.047 | 257.054 |  |
|  | 90 | 6.150 | 14.907 |  | 1.900 | 3.274 |  |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.515 | 32.554 | 65.106 | 162.779 |
|  | 30 | 5.776 | 28.839 | 57.580 | 143.205 | 5.776 | 28.852 | 57.646 | 143.603 |  |  |  |  |
|  | 90 | 0.003 | 0.076 | 0.302 | 1.774 | 0.002 | 0.035 | 0.157 | 0.967 | 0.001 | 0.009 | 0.032 | 0.188 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.511 | 32.534 | 65.094 | 162.760 |
|  | 30 | 5.771 | 28.812 | 57.505 | 142.963 | 5.770 | 28.816 | 57.587 | 143.284 |  |  |  |  |
|  | 90 | 0.003 | 0.076 | 0.304 | 1.900 | 0.002 | 0.041 | 0.165 | 1.031 | 0.000 | 0.008 | 0.031 | 0.193 |

Note: these values do not take into account UE altitude uncertainty.

Note: For Set1/2 parameters the beam size is assumed to be fixed within the satellite coverage.

Proposal 2-4

Proposal 2-4-v1:

Working assumption

Capture in the TR 38.742 the following tables of differential one-way Doppler reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.011 |  |  | 0.005 |
|  | 30 | 0.371 | 0.390 |  | 0.146 | 0.168 |  |
|  | 90 | 2.112 | 1.843 | 0.046 | 0.848 | 0.767 | 0.020 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.023 |  |  | 0.012 |
|  | 30 | 0.754 | 0.921 |  | 0.410 | 0.425 |  |
|  | 90 | 3.790 | 3.861 | 0.102 | 2.118 | 1.843 | 0.052 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.041 |  |  | 0.024 |
|  | 30 | 1.212 | 1.125 |  | 0.497 | 0.514 |  |
|  | 90 | 2.100 | 1.805 | 0.044 | 0.855 | 0.821 | 0.024 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diamter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.070 |  |  | 0.054 |
|  | 30 | 2.191 | 2.386 |  | 1.224 | 1.136 |  |
|  | 90 | 3.776 | 3.799 | 0.079 | 2.138 | 1.849 | 0.062 |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.000 | 0.000 | 0.001 | 0.002 |
|  | 30 | 0.015 | 0.075 | 0.150 | 0.387 | 0.009 | 0.043 | 0.086 | 0.219 |  |  |  |  |
|  | 90 | 0.085 | 0.407 | 0.861 | 2.034 | 0.041 | 0.208 | 0.415 | 1.038 | 0.000 | 0.002 | 0.004 | 0.009 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.001 | 0.002 | 0.003 | 0.007 |
|  | 30 | 0.047 | 0.235 | 0.472 | 1.189 | 0.024 | 0.121 | 0.243 | 0.610 |  |  |  |  |
|  | 90 | 0.084 | 0.421 | 0.841 | 2.102 | 0.040 | 0.202 | 0.404 | 1.010 |  |  |  |  |

Note: the above results do not take into account the Earth rotation

Note: these values are for static/slow moving UEs.

Note: these values do not take into account UE altitude uncertainty.

Note: For Set1/2 parameters the beam size is assumed to be fixed within the satellite coverage.

Proposal 3-1

Proposal 3-1-v1:

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 398,74683,59 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 99,10102,34 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9,38 | 69 | 0,39 | 66,67 | 8,59 |
| A1 | 30 | 139 | 4,69 | 69 | 0,33 | 33,33 | 4,04 |
| A1 | 60 | 139 | 2,34 | 69 | 0,11 | 16,67 | 2,12 |
| A1 | 120 | 139 | 1,17 | 69 | 0,11 | 8,33 | 0,94 |
| A2 | 15 | 139 | 18,75 | 69 | 0,39 | 66,67 | 17,97 |
| A2 | 30 | 139 | 9,38 | 69 | 0,33 | 33,33 | 8,72 |
| A2 | 60 | 139 | 4,69 | 69 | 0,11 | 16,67 | 4,46 |
| A2 | 120 | 139 | 2,34 | 69 | 0,11 | 8,33 | 2,12 |
| A3 | 15 | 139 | 28,13 | 69 | 0,39 | 66,67 | 27,34 |
| A3 | 30 | 139 | 14,06 | 69 | 0,33 | 33,33 | 13,41 |
| A3 | 60 | 139 | 7,03 | 69 | 0,11 | 16,67 | 6,80 |
| A3 | 120 | 139 | 3,52 | 69 | 0,11 | 8,33 | 3,29 |
| B1 | 15 | 139 | 7,03 | 69 | 0,39 | 66,67 | 6,25 |
| B1 | 30 | 139 | 3,52 | 69 | 0,33 | 33,33 | 2,86 |
| B1 | 60 | 139 | 1,76 | 69 | 0,11 | 16,67 | 1,53 |
| B1 | 120 | 139 | 0,88 | 69 | 0,11 | 8,33 | 0,65 |
| B2 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B2 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B2 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B2 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| B3 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B3 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B3 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B3 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| C0 | 15 | 139 | 16,41 | 69 | 0,39 | 66,67 | 15,63 |
| C0 | 30 | 139 | 8,20 | 69 | 0,33 | 33,33 | 7,55 |
| C0 | 60 | 139 | 4,10 | 69 | 0,11 | 16,67 | 3,87 |
| C0 | 120 | 139 | 2,05 | 69 | 0,11 | 8,33 | 1,82 |
| C2 | 15 | 139 | 66,67 | 69 | 0,39 | 66,67 | 32,31 |
| C2 | 30 | 139 | 33,33 | 69 | 0,33 | 33,33 | 15,90 |
| C2 | 60 | 139 | 16,67 | 69 | 0,11 | 16,67 | 8,05 |
| C2 | 120 | 139 | 8,33 | 69 | 0,11 | 8,33 | 3,91 |

Proposal 3-2

Proposal 3-2-v0:

Working assumption

Differential one-way Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0,1 | 2 | 2 | 2,0 | 0,310,21 |
| 0,1,2 | type A | 1250 | 1 | 0,1 | 2 | 2 | 2,0 | 0,620,53 |
| 0,1,2 | type B | 1250 | 3 | 0,1 | 2 | 2 | 2,0 | 1,251,15 |
| 3 | unrestricted | 5000 | 0 | 0,1 | 2 | 2 | 2,0 | 1,251,15 |
| 3 | type A | 5000 | 1 | 0,1 | 2 | 2 | 2,0 | 2,502,40 |
| 3 | type B | 5000 | 3 | 0,1 | 2 | 2 | 2,0 | 5,004,90 |

Differential one-way Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0,1 | 2 | 2 | 2,0 | 3,65 |
|  | 30 | 30000 | 0 | 0,1 | 2 | 2 | 2,0 | 7,40 |
|  | 60 | 60000 | 0 | 0,1 | 30 | 20 | 2,5 | 0,72 |
|  | 120 | 120000 | 0 | 0,1 | 30 | 20 | 2,5 | 1,52 |

Proposal 5-1

Proposal 5-1-v0

For the study of GNSS resilient operation, if the UE cannot rely on its GNSS for timing and frequency compensation on the service link, it is assumed that the UE pre-compensates RTT and Doppler on the service link, based on assistance information indicated by the Network.

Proposal

For the study of GNSS resilient operation, for evaluation purposes, it is assumed that the UE pre-compensates the RTT and the Doppler on the service link for UL transmission, based on assistance information indicated by the Network.

Proposal 9-1

Proposal 9-1-v0

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA.

- This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications.

- Evaluation assumptions and parameters should be defined during this meeting.

# Proposals for Tuesday online

### Proposal 2-3

Proposal 2-3-v1:

Working assumption

Capture in the TR 38.742 the following tables of differential one-way delay reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.3 |  |  | 357.4 |
|  | 30 | 143.3 | 257.8 |  | 57.6 | 115.1 |  |
|  | 90 | 1.9 | 3.3 | 4.8 | 0.3 | 0.7 | 0.9 |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.9 |  |  | 906.6 |
|  | 30 | 255.3 | 535.3 |  | 143.1 | 257.1 |  |
|  | 90 | 6.1 | 14.9 | 15.5 | 1.9 | 3.3 | 6.0 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 810.2 |  |  | 357.4 |
|  | 30 | 142.9 | 257.7 |  | 57.5 | 114.9 |  |
|  | 90 | 1.9 | 3.3 |  | 0.3 | 0.7 |  |

  - Set2- parameters

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 1451.8 |  |  | 906.6 |
|  | 30 | 255.3 | 535.2 |  | 143.0 | 257.0 |  |
|  | 90 | 6.1 | 14.9 |  | 1.9 | 3.3 |  |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.515 | 32.554 | 65.106 | 162.779 |
|  | 30 | 5.8 | 28.8 | 57.6 | 143.2 | 5.8 | 28.8 | 57.646 | 143.603 |  |  |  |  |
|  | 90 | 0.0 | 0.1 | 0.3 | 1.8 | 0.0 | 0.0 | 0.1 | 0.9 | 0.0 | 0.0 | 0.0 | 0.2 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way delay (in µs) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 6.5 | 32.5 | 65.1 | 162.8 |
|  | 30 | 5.8 | 28.8 | 57.5 | 142.9 | 5.8 | 28.8 | 57.6 | 143.3 |  |  |  |  |
|  | 90 | 0.003 | 0.1 | 0.3 | 1.9 | 0.0 | 0.0 | 0.2 | 1.0 | 0.0 | 0.0 | 0.0 | 0.2 |

Note: these values do not take into account UE altitude uncertainty.

Note: For Set1/2 parameters the beam size is assumed to be fixed within the satellite coverage.

### Proposal 2-4

Proposal 2-4-v1:

Working assumption

Capture in the TR 38.742 the following tables of differential one-way Doppler reflecting results obtained from multiple sources:

- Case a: the location uncertainty area is the area served by the cell or beam | case 1: within the orbital plane |

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.011 |  |  | 0.005 |
|  | 30 | 0.371 | 0.390 |  | 0.146 | 0.168 |  |
|  | 90 | 2.112 | 1.843 | 0.046 | 0.848 | 0.767 | 0.020 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.023 |  |  | 0.012 |
|  | 30 | 0.754 | 0.921 |  | 0.410 | 0.425 |  |
|  | 90 | 3.790 | 3.861 | 0.102 | 2.118 | 1.843 | 0.052 |

- Case a: the location uncertainty area is the area served by the cell or beam | case 2: at 90° with respect to orbital plane

  - Set1- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 50 | 90 | 250 | 20 | 40 | 110 |
| Elevation angle (°) | 12.5 |  |  | 0.041 |  |  | 0.024 |
|  | 30 | 1.212 | 1.125 |  | 0.497 | 0.514 |  |
|  | 90 | 2.100 | 1.805 | 0.044 | 0.855 | 0.821 | 0.024 |

  - Set2- parameters

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO1200 | GEO | LEO600 | LEO1200 | GEO |
|  | Band/Carrier Frequency | S/2GHz | S/2GHz | S/2GHz | Ka/30GHz | Ka/30GHz | Ka/30GHz |
|  | Beam diameter (km) | 90 | 190 | 450 | 50 | 90 | 280 |
| Elevation angle (°) | 12.5 |  |  | 0.070 |  |  | 0.054 |
|  | 30 | 2.191 | 2.386 |  | 1.224 | 1.136 |  |
|  | 90 | 3.776 | 3.799 | 0.079 | 2.138 | 1.849 | 0.062 |

- Case b: the location uncertainty area is a circle of radius X km | case 1:  within the orbital plane.

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.000 | 0.000 | 0.001 | 0.002 |
|  | 30 | 0.015 | 0.075 | 0.150 | 0.387 | 0.009 | 0.043 | 0.086 | 0.219 |  |  |  |  |
|  | 90 | 0.085 | 0.407 | 0.861 | 2.034 | 0.041 | 0.208 | 0.415 | 1.038 | 0.000 | 0.002 | 0.004 | 0.009 |

- Case b: the location uncertainty area is a circle of radius X km |  case 2: at 90° with respect to orbital plane

|  |  | Differential one-way Doppler (ppm) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orbit | LEO600 | LEO600 | LEO600 | LEO600 | LEO1200 | LEO1200 | LEO1200 | LEO1200 | GEO | GEO | GEO | GEO |
|  | X (km) | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 | 1 | 5 | 10 | 25 |
| Elevation angle (°) | 12.5 |  |  |  |  |  |  |  |  | 0.001 | 0.002 | 0.003 | 0.007 |
|  | 30 | 0.047 | 0.235 | 0.472 | 1.189 | 0.024 | 0.121 | 0.243 | 0.610 |  |  |  |  |
|  | 90 | 0.084 | 0.421 | 0.841 | 2.102 | 0.040 | 0.202 | 0.404 | 1.010 |  |  |  |  |

Note: the above results do not take into account the Earth rotation

Note: these values are for static/slow moving UEs.

Note: these values do not take into account UE altitude uncertainty.

Note: For Set1/2 parameters the beam size is assumed to be fixed within the satellite coverage.

### Proposal 3-1

Proposal 3-1-v1:

Working assumption

Differential roundtrip delay limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | Unrestricted | 839 | 103,13 | 419 | 0,39 | 800 | 102,34 |
| 0 | type A | 839 | 103,13 | 237 | 0,39 | 800 | 102,34 |
| 0 | type B | 839 | 103,13 | 137 | 0,39 | 800 | 102,34 |
| 1 | unrestricted | 839 | 684,38 | 419 | 0,39 | 800 | 683,59 |
| 1 | type A | 839 | 684,38 | 237 | 0,39 | 800 | 225,20 |
| 1 | type B | 839 | 684,38 | 137 | 0,39 | 800 | 129,85 |
| 2 | unrestricted | 839 | 152,60 | 419 | 0,39 | 800 | 151,82 |
| 2 | type A | 839 | 152,60 | 237 | 0,39 | 800 | 151,82 |
| 2 | type B | 839 | 152,60 | 137 | 0,39 | 800 | 129,85 |
| 3 | unrestricted | 839 | 103,13 | 419 | 0,39 | 200 | 102,34 |
| 3 | type A | 839 | 103,13 | 237 | 0,39 | 200 | 55,71 |
| 3 | type B | 839 | 103,13 | 137 | 0,39 | 200 | 31,88 |

Differential roundtrip delay limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | L | Ncp [µs] | MaxNcs | Te [µs] | Sequence duration [µs] | Differential roundtrip delay limit [µs] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A1 | 15 | 139 | 9,38 | 69 | 0,39 | 66,67 | 8,59 |
| A1 | 30 | 139 | 4,69 | 69 | 0,33 | 33,33 | 4,04 |
| A1 | 60 | 139 | 2,34 | 69 | 0,11 | 16,67 | 2,12 |
| A1 | 120 | 139 | 1,17 | 69 | 0,11 | 8,33 | 0,94 |
| A2 | 15 | 139 | 18,75 | 69 | 0,39 | 66,67 | 17,97 |
| A2 | 30 | 139 | 9,38 | 69 | 0,33 | 33,33 | 8,72 |
| A2 | 60 | 139 | 4,69 | 69 | 0,11 | 16,67 | 4,46 |
| A2 | 120 | 139 | 2,34 | 69 | 0,11 | 8,33 | 2,12 |
| A3 | 15 | 139 | 28,13 | 69 | 0,39 | 66,67 | 27,34 |
| A3 | 30 | 139 | 14,06 | 69 | 0,33 | 33,33 | 13,41 |
| A3 | 60 | 139 | 7,03 | 69 | 0,11 | 16,67 | 6,80 |
| A3 | 120 | 139 | 3,52 | 69 | 0,11 | 8,33 | 3,29 |
| B1 | 15 | 139 | 7,03 | 69 | 0,39 | 66,67 | 6,25 |
| B1 | 30 | 139 | 3,52 | 69 | 0,33 | 33,33 | 2,86 |
| B1 | 60 | 139 | 1,76 | 69 | 0,11 | 16,67 | 1,53 |
| B1 | 120 | 139 | 0,88 | 69 | 0,11 | 8,33 | 0,65 |
| B2 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B2 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B2 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B2 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| B3 | 15 | 139 | 11,72 | 69 | 0,39 | 66,67 | 10,94 |
| B3 | 30 | 139 | 5,86 | 69 | 0,33 | 33,33 | 5,21 |
| B3 | 60 | 139 | 2,93 | 69 | 0,11 | 16,67 | 2,70 |
| B3 | 120 | 139 | 1,46 | 69 | 0,11 | 8,33 | 1,24 |
| C0 | 15 | 139 | 16,41 | 69 | 0,39 | 66,67 | 15,63 |
| C0 | 30 | 139 | 8,20 | 69 | 0,33 | 33,33 | 7,55 |
| C0 | 60 | 139 | 4,10 | 69 | 0,11 | 16,67 | 3,87 |
| C0 | 120 | 139 | 2,05 | 69 | 0,11 | 8,33 | 1,82 |
| C2 | 15 | 139 | 66,67 | 69 | 0,39 | 66,67 | 32,31 |
| C2 | 30 | 139 | 33,33 | 69 | 0,33 | 33,33 | 15,90 |
| C2 | 60 | 139 | 16,67 | 69 | 0,11 | 16,67 | 8,05 |
| C2 | 120 | 139 | 8,33 | 69 | 0,11 | 8,33 | 3,91 |

### Proposal 3-2

Proposal 3-2-v0:

Working assumption

Differential one-way Doppler limits for long PRACH preamble formats considered for the study are given in the following table:

| Preamble | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0,1 | 2 | 2 | 2,0 | 0,21 |
| 0,1,2 | type A | 1250 | 1 | 0,1 | 2 | 2 | 2,0 | 0,53 |
| 0,1,2 | type B | 1250 | 3 | 0,1 | 2 | 2 | 2,0 | 1,15 |
| 3 | unrestricted | 5000 | 0 | 0,1 | 2 | 2 | 2,0 | 1,15 |
| 3 | type A | 5000 | 1 | 0,1 | 2 | 2 | 2,0 | 2,40 |
| 3 | type B | 5000 | 3 | 0,1 | 2 | 2 | 2,0 | 4,90 |

Differential one-way Doppler limits for short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0,1 | 2 | 2 | 2,0 | 3,65 |
|  | 30 | 30000 | 0 | 0,1 | 2 | 2 | 2,0 | 7,40 |
|  | 60 | 60000 | 0 | 0,1 | 30 | 20 | 2,5 | 0,72 |
|  | 120 | 120000 | 0 | 0,1 | 30 | 20 | 2,5 | 1,52 |

### Proposal 5-1

Proposal 5-1-v0

For the study of GNSS resilient operation, if the UE cannot rely on its GNSS for timing and frequency compensation on the service link, it is assumed that the UE pre-compensates RTT and Doppler on the service link, based on assistance information indicated by the Network.

Proposal

For the study of GNSS resilient operation, for evaluation purposes, it is assumed that the UE pre-compensates the RTT and the Doppler on the service link for UL transmission, based on assistance information indicated by the Network.

### Proposal 9-1

Proposal 9-1-v0

For GNSS-resilient NTN operation, study DL-only UE-based positioning method, using DL-TDOA.

- This should include assessing the feasibility, evaluating the achievable UE position accuracy, and identifying any potential impacts on existing specifications.

- Evaluation assumptions and parameters should be defined during this meeting.

# 3rd round proposals

### Proposal 4

Proposal 4-2-v0

Companies are encouraged to comment on the template for PRACH performance evaluation v000_Moderator

| Companies | Comments |
| --- | --- |
| Ericsson | In general it looks good. Three comments: - No need to have the 90° elevation case for both Case a (1) and Case a (2) (and similarly for Case b) since they are the same beam. We suggest to remove the 90° elevation case from Case a (2) and Case b (2). - There is a typo: "unrestrected"  "unrestricted" in several places - Format B4 is missing |
| Tejas Networks | Support the template |
| CEWiT | Fine with the template |
| Eutelsat | Good template – agree with Ericsson some minor improvements can be made before finalising. |
| Qualcomm | - For the case a(1) and case a(2), we should define that the “UA size (km)” is actually modelled by a circle of a given diameter. We suggest something as follows: UA size diameter (km) - For case b (1) and (2), we should add a column “Band/freq” similar to case a. - Change “TA” to “TO” (timing offset) to be consistent with the row “FO” - In the “Comment” sheet, maybe we can report both the gap and margin? (with e.g. margin being a negative number and gap being a positive number). This way, both gap and margin follow the same equation, with gap being a positive number and margin a negative number Gap; The gap should be reported when PRACH tolerance is exceeded (in µs for TA and kHz for FO). Margin; Negative number. The margin with respect to the PRACH tolerance (in µs for TA and kHz for FO). |

### Proposal 7-2

Proposal 7--v0

For the evaluation of GNSS resilient NR-NTN operation, consider the following solutions for initial access procedure at least to increase PRACH tolerance and/or reducing time/frequency uncertainty:

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 1B: New PRACH restricted sets.

- Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission.

- Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking.

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 2B: Multiple random access PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

- Solution 2F: UE side time/frequency pre-compensation based on reference signal.

- Solution 2G: UE-side time/frequency pre-compensation using TA/Doppler compensation formula.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded

Proponents should justify that the solutions are within SID scope.

As part of the evaluation of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity

- the signalling overhead

Companies are encouraged to comment Proposal 7-2-v0 .

| Companies | Comments |
| --- | --- |
| LGE | The problem the solution is trying to solve should be clearly stated. The current proposal does not justify the need for the solution, as the purpose for which the solution is intended has been deleted or omitted. In this sense, we think that this should be an updated observation from the previous meeting, and we would like to add some solutions as follows: For the study on GNSS resilient NR-NTN operation, the following solutions for initial access to increase PRACH tolerance and reducing time/frequency uncertainty are listed based on inputs/discussions in RAN1#123: - To improve robustness: - Solutions based on modified random access procedures, using existing PRACH formats: - Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats - Solution 1B: New PRACH restricted sets. - Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA. - Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command). - Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission. - Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking. - Solution 1G: Adaptation of PRACH configuration. - Other solutions are not precluded. - To reduce time-frequency uncertainty - Solution 2A: Single/multi-satellite DL-TDOA based on current specifications. - Solution 2B: Multiple random access PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area) - Solution 2C: Solutions based on broadcasting DL timestamp(s). - Solution 2D: UE side time/frequency pre-compensation based on reference location. - Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position - Solution 2F: UE side time/frequency pre-compensation based on reference signal. - Solution 2G: UE-side time/frequency pre-compensation using TA/Doppler compensation formula. - Other solutions are not precluded - Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay. - Other Solutions are not precluded - Proponents should justify that the solutions are within SID scope. |
| Ericsson | We support the proposal. Some editorial proposals: - Remove the sentence "- Solutions based on modified random access procedures, using existing PRACH formats:" since it does not apply to all sub-bullets below it (e.g., 2A, 2B, …). - Put all bullets on the same level. - Keep only one instance of "other solutions are not precluded" |
| Tejas Networks | We support the proposal with editorial changes as suggested by Ericsson |
| CEWiT | Fine with the proposal |
| Eutelsat | Generally fine. It is unclear how proponents would justify their idea being in scope (comparison with SID wording?). |
| Qualcomm | Based on some offline discussion, we think the following bullet would be more acceptable to companies. Instead of: - Proponents should justify that the solutions are within SID scope. We can add something like As part of the evaluation of the solutions above, specification impact (including how the solutions remain within the scope of the SID) should be assessed. |

### Proposal 7-3

Proposal 7-3-v0

For the evaluation of DL-only UE-based positioning method, using DL-TDOA, consider the following evaluation assumptions and parameters:

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600 km, 1200 km, GEO |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821. |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1-NTN: 2GHz, S-band. FR2-NTN: Ku/Ka |
| BW | To be reported by companies. 10 MHz for FR1 and 50 MHz for FR2 |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | Case 1 : At least 1 Satellites Case 2 : At least 3 Satellites |
| UE type | As RAN1#122 agreement |
| Positioning signals | DL-PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | To be reported. |
| Time window for measurement collection | To be reported. |
| UE speed | As RAN1#122 agreement |
| UE minimum elevation angle | As RAN1#122 agreement |
| Ue clock drift/error | To be reported |
| Performance metrics | UE 2D as baseline. 3D can be reported by companies |

Companies are encouraged to comment Proposal 7-3-v0 .

| Companies | Comments |
| --- | --- |
| Ericsson | - GEO should also be considered. - UE clock drift should be modelled. |
| Tejas Networks | Support |
| Eutelsat | Agree with Ericsson regarding clock drift. GEO should be considered but LEO needs to be the main focus. |
| Qualcomm | For the multi-satellite case, the UE elevation angle is not well defined since different satellites will have different elevation angles. We can replace this value with “To be reported by companies” |

### Proposal 7-4

Proposal 7-4-v0

For PRACH performance evaluation, consider the following evaluation assumptions and parameters:

| Parameter | Value |
| --- | --- |
| Carrier Frequency (fc | 2 GHz/30GHz |
| PRACH Format | To be reported |
| PRACH Configuration | To be reported |
| Phase Noise Model | Model-1, UE in (TR 38.803, Section 6.1.11.2 ) |
| Timing Error | Uniform random variable in (0, CP duration) |
| Frequency Error | Uniform random variable in (-0.5ΔfRA,0.5ΔfRA) |
| Channel Model | NTN TDL-C |
| Delay Spread | 10 ns |
| Doppler Spread | 5 Hz |
| Detection Threshold | Set to achieve 0.1% False alarm rate |
| Timing Error Tolerance | bereported |
| Frequency Error Tolerance | 0.1 ppm |

Companies are encouraged to comment Proposal 7-4-v0 .

| Companies | Comments |
| --- | --- |
| Ericsson | - We already have an agreement from RAN1#122 for some of these parameters, e.g., carrier frequency and channel model. - Our understanding is that these assumptions will be used when evaluating the impact of differential delay/Doppler (for existing PRACH or PRACH enhancements). In this case, the timing error and frequency error should be set to actual values observed in NTN according to previous observations. - Clarify what "Timing error tolerance" and "Frequency error tolerance" mean. Are they the tolerances of the UE transmitter, i.e., Timing error tolerance equals Te? |
| Tejas Networks | The Parameters for PRACH LLS evaluation was agreed in RAN1#122. We understand that these parameters will used to evaluate the performance of existing PRACH or enhanced PRACH under the defined differential delay and doppler values in the agreed working assumptions . In that case the timing error and frequency error to be considered will be the differential delay and doppler calculated for the particular scenario |
| Eutelsat | Support. There may need to be some clarifications (as noted by Ericsson). |
| Qualcomm | We would like to note that a previous table for PRACH evaluations was agreed as follows, we could add additional values as proposed: - For the “Phase noise model”, we suggest to make it applicable only to Ku/Ka band. For timing and frequency error, we can make “Based on UL differential timing / frequency error”. |

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods, RAN1 will consider the following evaluation methodology:

- Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ

- Step 2: Propagate the satellite during the simulation time T (assuming a circular trajectory around the center of Earth)

- Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method

- Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms

- Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively

- Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

| Scenario | Radius of uncertainty area (X) | Frequency band | SSB SCS | SCS of UL signals | Max Diff RTD at 30° | Minimal Period of TA corrections [s] |  | Max Period of TA corrections [s] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Tp | Tp_relax | Tp | Tp_relax |
| LEO600 | 25km | S | 15 kHz | 15 kHz |  |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 25km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO600 | 50km | S | 15 kHz | 15 kHz | 290.7μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 50km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO1200 | 45km | S | 15 kHz | 15 kHz | 260.7μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 45km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO1200 | 90km | S | 15 kHz | 15 kHz | 523.2μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 90km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |

### Proposal 12-1

Proposal 12-1-v0

Endorse the skeleton for TR 38.742 proposed in R1-2508479 [4].

Companies are encouraged to comment on the skeleton for TR 38.742 proposed in R1-2508479 [4].

| Companies | Comments |
| --- | --- |
| ST Engineering iDirect | Suggest to split Section 7 in two parts: - Evaluation of solutions for GNSS-resilient NR-NTN operation during initial access - Evaluation of solutions for GNSS-resilient NR-NTN operation in RRC connected As there might be multiple solutions, we propose to have a (sub)section per solution, and discuss evaluation and spec impact per solution. Proposal to add a section dedicated to evaluation methodology. |
| Toyota ITC | Support ST Engineering iDirect. Suggest splitting the solutions and evaluations to two sections. - Evaluation of solutions for GNSS-resilient NR-NTN operation during initial access - Evaluation of solutions for GNSS-resilient NR-NTN operation in RRC connected |
| Panasonic | Support. |
| Nokia | OK |
| Sony | Support. |
| ZTE | We have some comments on the skeleton. - Regarding clause 4, it is enough to directly reference to the SID. No need to introduce clauses for specific description of backgrounds and motivations. - To align with the SID, “GNSS unavailability” can be changed to “GNSS temporarily unavailable”, e.g., in clause 6 - Regarding clause 7.3, which RAN WGs are included need to be clarified. - As we are still studying, it is not clear what open issues and future work are. Moreover, whether there will be normative work depends on the RAN plenary discussion at check point. Hence, it’s too early to have clauses 8.2 and 8.3 at current stage. |
| Futurewei | Recommend to add a new subsection for evaluation results. |

# Proposals for Thursday online

### Proposal 7-2

Proposal 7-2-v0

For the evaluation of GNSS resilient NR-NTN operation, consider the following solutions for initial access procedure at least to increase PRACH tolerance and/or reducing time/frequency uncertainty:

- Solution 1A: Multiple PRACH transmissions (e.g. with different roots or different formats or with different time/frequency pre-compensation using multiple reference locations within the uncertainty area) using existing PRACH formats

- Solution 1B: New PRACH restricted sets.

- Solution 1C: Single PRACH transmission with multiple TAC/FAC commands in single/multiple RAR MAC PDU scheduling multiple msg3 with different TA/FA.

- Solution 1D: Signalling enhancements for Msg2/Msg4 (e.g. enhanced TA command, frequency adjustment command, reference point adjustment command).

- Solution 1E: configuration/selection of a set of existing PRACH root for a Single PRACH transmission.

- Solution 1F: PRACH transmission using TA margin indicated by Network and RO masking.

- Solution 1G: Adaptation of PRACH configuration.

- Solution 2A: Single/multi-satellite DL-TDOA based on current specifications.

- Solution 2B: Multiple random access PRACH attempts based on different time/frequency pre-compensation hypotheses (e.g. based on multiple reference points within the uncertainty area)

- Solution 2C: Solutions based on broadcasting DL timestamp(s).

- Solution 2D: UE side time/frequency pre-compensation based on reference location or TA/Doppler compensation information provided by gNB.

- Solution 2E: service link time/frequency pre-compensation based on last acquired GNSS position

- Solution 2F: UE side time/frequency pre-compensation based on reference signal.

- Solution 3: Implementation-based techniques e.g. using a long enough PRACH processing window and multiple timing hypotheses for PRACH preamble reception with large max differential delay.

Other Solutions are not precluded

As part of the evaluation of the solutions above, at least the following aspects should be assessed:

- specification impact when relevant (including whether the solutions remain within the scope of the SID)

- performance,

- applicability to different scenarios,

- complexity,

- the signalling overhead.

### Proposal 7-3

Proposal 7-3-v0

For the evaluation of DL-only UE-based positioning method, using DL-TDOA, consider the following evaluation assumptions and parameters:

| Parameter | Description/Value |
| --- | --- |
| Satellite Orbit | 600 km, 1200 km, GEO |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821. |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1-NTN: 2GHz, S-band. FR2-NTN: Ku/Ka |
| BW | To be reported by companies. 10 MHz for FR1 and 50 MHz for FR2 |
| Subcarrier spacing | 15 kHz for FR1-NTN, 120 kHz for FR2-NTN |
| Number of satellites in view | Case 1 : At least 1 Satellites Case 2 : At least 3 Satellites |
| UE type | As per RAN1#122 agreement |
| Positioning signals | DL-PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| Number of symbols used per occasion | 6 |
| Number of DL-PRS occasions | To be reported. |
| Time window for measurement collection | To be reported. |
| UE speed | As per RAN1#122 agreement |
| UE minimum elevation angle | As per RAN1#122 agreement |
| Ue clock drift/error | To be reported |
| Performance metrics | UE 2D as baseline. 3D can be reported by companies |

### Proposal 12-1

Proposal

Revise the skeleton for TR 38.742 proposed in R1-2508479 as follows:

# 4 Background and motivation ## 4.1 Review of previous NR-NTN releases ## 4.2 Needs for GNSS resilience NR NTN operation ## 4.3 Use cases and deployment scenarios # 5 Assumptions and study parameters ## 5.1 GNSS availability scenarios ## 5.2 System assumptions ## 5.3 Evaluation scenarios and parameters # 6 Evaluation of GNSS unavailability impacts ## 6.1 Impact on synchronization in initial access ## 6.2 Impact on synchronization in connected mode # 7 Evaluation of solutions for GNSS resilient NR-NTN ## 7.1 Potential solutions for initial access ## 7.1.1 Evaluation of solutions ## 7.1.2 Impact on specifications and RAN WGs ## 7.2 Potential solutions for RRC connected mode ## 7.2.2 Evaluation of solutions ## 7.2.3 Impact on specifications and RAN WGs

### Proposal 7-4

Proposal 1-3-v0

For the evaluation of frequency and timing adjustment methods in connected mode, RAN1 will consider the following evaluation methodology:

- Step 1: Drop the UE at a location where the satellite is visible at an elevation angle θ=30°

- Step 2: Propagate the satellite during the simulation time TSim=visibilité duration (assuming a circular trajectory around the center of Earth)

- Step 3: The UE applies an initial timing and frequency offset (randomly distributed within Te and fe, which are the residual timing and frequency errors after successful completion of random access) and updates the timing and frequency adjustments at time t based on a correction method

- Step 4: The network evaluates the timing and frequency error after every TUL ms, assuming an uplink transmission every TUL ms

- Step 5: A timing and/or frequency correction is issued if the timing and frequency error exceeds tolerance limits Ttol and Ftol, respectively

- Using the steps mentioned above, RAN1 to evaluate timing and frequency control using the following metric: Number of TA and FA commands issued within T.

| Scenario | Radius of uncertainty area (X) | Frequency band | SSB SCS | SCS of UL signals | Max Diff RTD at 30° | Minimal Period of TA corrections [s] |  | Max Period of TA corrections [s] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Tp | Tp_relax | Tp | Tp_relax |
| LEO600 | 25km | S | 15 kHz | 15 kHz |  |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 25km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO600 | 50km | S | 15 kHz | 15 kHz | 290.7μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 50km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO1200 | 45km | S | 15 kHz | 15 kHz | 260.7μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 45km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |
| LEO1200 | 90km | S | 15 kHz | 15 kHz | 523.2μs |  |  |  |  |
|  |  |  | 15 kHz | 30 kHz |  |  |  |  |  |
|  | 90km | Ka/Ku | 120 kHz | 120 kHz |  |  |  |  |  |

# Compilation of company proposals

The compilation of proposals from companies on NR-NTN GNSS resilience is available in document R1-2509476. This document gathers contributions from various companies, presenting their recommendations and technical solutions to enhance NR-NTN performance in scenarios where GNSS signals are unavailable or degraded.

# Conclusion

# References

- R1-2508333	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

- R1-2508360	PRACH with GNSS unavailable	Eutelsat Group

- R1-2508385	Discussion on NR NTN GNSS resilience	Spreadtrum, UNISOC

- R1-2508429	Discussion on NR-NTN GNSS resilience	vivo

- R1-2508452	Discussion on NR-NTN GNSS resilience	CMCC

- R1-2508465	Considerations on NR-NTN Resilience to GNSS Unavailability and Degradation	THALES

- R1-2508481	Considerations for GNSS-resilient NR-NTN operation 	ST Engineering iDirect

- R1-2508489	GNSS resilience for NR-NTN	Huawei, HiSilicon

- R1-2508555	Discussion on NR-NTN GNSS resilience	NEC

- R1-2508594	Revision of R1-2508594 Discussion on NR-NTN GNSS resilience	CATT

- R1-2508626	Considerations for GNSS-resilient NR-NTN operation	ESA

- R1-2508681	Discussion on NR-NTN GNSS resilience	Xiaomi

- R1-2508724	Discussion on NR-NTN GNSS resilience	OPPO

- R1-2508754	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

- R1-2508760	Discussion on GNSS Resilient NR-NTN Operation	Tejas Network Limited

- R1-2508799	Discussion on NR-NTN GNSS resilience	Samsung

- R1-2508843	On NR-NTN GNSS resilience	Ericsson

- R1-2508854	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

- R1-2508872	Discussion on NR-NTN GNSS resilience	Airbus

- R1-2508875	Discussion on NR NTN GNSS Resilience	Amazon Web Services

- R1-2508889	PRACH transmission and detection in NR NTN GNSS resilient operation	Sharp

- R1-2508970	Discussion on NR-NTN GNSS resilient operations	ETRI

- R1-2508984	Discussion on the GNSS resilient NR-NTN operation	TCL

- R1-2508992	Discussion on NR-NTN GNSS resilience	HONOR

- R1-2509007	Discussion on GNSS resilient operation for NR over NTN	Nokia

- R1-2509016	NR-NTN GNSS resilience	InterDigital, Inc.

- R1-2509017	Discussion on NR-NTN GNSS resilience	IMU

- R1-2509018	Discussion on GNSS resilient NR-NTN operation	Lenovo

- R1-2509071	On NR-NTN GNSS resilience approaches	Sony

- R1-2509107	Discussion on NR-NTN GNSS resilience	Apple

- R1-2509131	Discussion on NR-NTN GNSS resilience	Ofinno

- R1-2509137	Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

- R1-2509163	GNSS resilient operations in NR NTN	MediaTek Inc.

- R1-2509178	Discussion on NR-NTN GNSS resilient operation using synchronization signals	ViaSat Satellite Holdings Ltd, Inmarsat

- R1-2509185	Discussion on NR-NTN GNSS resilience	China Telecom

- R1-2509228	NR-NTN GNSS resilience	Qualcomm Incorporated

- R1-2509277	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

- R1-2509296	Discussion on GNSS resilient NR-NTN	Panasonic

- R1-2509298	Discussion on NR-NTN GNSS resilience	Google Korea LLC

- R1-2509301	Discussion of NR-NTN GNSS Resilience	Johns Hopkins University APL

- R1-2509304	Discussion on GNSS resilient NR-NTN operation	LG Electronics

- R1-2509336	Discussion on GNSS resilience for NR-NTN	CSCN

- R1-2509347	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

- R1-2508479 	TR 38.742 skeleton for study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation