3GPP TSG RAN WG1 #124  	                                                       R1-2600487

Gothenburg, SE, Feb 09th – 13th, 2026

Source:               Moderator (vivo)

Title:           FL Summary #1 on R2D Aspects for Device 2b/C in R20 AIoT

Agenda Item:  9.3.2.1

Document for: Discussion and Decision

- Introduction

This is the feature lead (FL) summary for agenda item (AI) 9.3.2.1, covering R2D Aspects and procedures, etc.

RAN1 agreements reached in previous meetings related to this agenda item are documented in Annex B.

(Read me) FL Notes

- The company names with [xxx] in the TP proposal for TR capturing, will be replaced by TDoc numbers in final TR, as what we do in Release 19.

- For some issues, if previous agreements did not ask to further study. FL will consider these issues as low priority in this meeting.

- [blue text] is notes for easier reading, and will be removed in the TR.

- The proposals with prefix of [H], [M], [L], means high priority, medium priority and low priority, from FL point of view.

- The text proposal is provided based on the approved TP in R1-2509718.

- The proposals with prefix of [TP], means this proposal is intended for TP capturing to TR 38.769.

  - [TP][N]: means this is a New TP, to be added in TR, there are 10 proposals.

  - [TP][U]: means this is an Update TP for paragrah already in TR, there are 25 proposals.

  - [H]: Means high priority proposal for potential down selection, there are 5 proposals.

- For easy reading, Company names are added next to the reference number in the update TP, e.g., [R1-10321-123-1] in TR is changed to [R1-10321-123-1, FUTUREWEI] in the update TP, TR editor can remove company name and only keep reference number lator.

- If companies want to add information in the Update TP or New TP provided by FL, please make sure the newly added information already provided in your Tdoc.

- Proposals for online/offline discussion

  - Proposals for Tuesday morning Online

2.1.1 [TP][N] Proposal 3.2-2 Necessity of R2D Block level repetition

FL notes

- Necessity of R2D Block level repetition has no outcome in previous meetings.

- NEW TP is provided to add company views in TR38.769.

- TP is Updated based on comments from Qualcomm and Samsung in FL Summary.

FL Proposal 3.2-2-v1

Regarding Necessity of R2D Block level repetition, Capture following in TR38.769.

[Positive views]

Sources [Futurewei], [Huawei], [CMCC], [ZTE], [OPPO], [vivo], [NEC], [Panasonic], [Apple], [Qualcomm], [InterDigital], [TCL], [Quectel], [Sequans], and [IITK] report that it is necessary or beneficial to support repetition to achieve required target coverage for outdoor scenario.

- Source [Huawei] provides evaluation results shows that, it is necessary to support R2D enhancements including block repetitions to ensure R2D transmissions with high reliability, facilitate early termination, and usability for outdoor scenarios considering co-site deployment.

  - With M = 1, 1/3 TBCC, sub-block interleaver and LTE bit collection, it can achieve 334.7m - 431.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which shows significant gain for larger TBS transmissions, but still requires further enhancement for outdoor co-site deployment.

  - With M = 1, 1/3 TBCC and LTE bit collection & sub-block interleaver, block repetition number 2, it can achieve 439.0m - 589.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which can approximately satisfy the outdoor co-site deployment requirement.

- Source [vivo] provides evaluation results shows that R2D Block level repetition is necessary to achieve desirable coverage

  - For UMa scenario, to achieve 500-meter coverage, repetition is necessary for the following cases:

    - The coverage is 513.36m with 3 repetitions, while coverage is 373.46m without repetition, for {96bit payload size, 43dBm Tx power}

    - The coverage is 616.24m with 3 repetitions, while coverage is 276.53m without repetition, for {400bit payload size, 43dBm Tx power}

  - For UMa scenario, to achieve 334-meter coverage, repetition is necessary for the following cases:

    - The coverage is 382.37m with 3 repetitions, while coverage is 278.17m without repetition, for {96bit payload size, 38dBm Tx power}

    - The coverage is 458.99m with 3 repetitions, while coverage is 205.97m without repetition, for {400bit payload size, 38dBm Tx power}

    - The coverage is 341.87m with 3 repetitions, while coverage is 153.41m without repetition, for {400bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 1154-meter coverage, repetition is necessary for the following cases:

    - The coverage is 1205.07m with 6 repetitions, while coverage is 472.75m without repetition, for {400bit payload size, 43dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, repetition is necessary for the following cases:

    - The coverage is 934.3m with 3 repetitions, while coverage is 472.75m without repetition, for {400bit payload size, 43dBm Tx power}

    - The coverage is 595.03m with 3 repetitions, while coverage is 475.57m without repetition, for {96bit payload size, 38dBm Tx power}

    - The coverage is 693.53m with 3 repetitions, while coverage is 350.92m without repetition, for {400bit payload size, 38dBm Tx power}

    - The coverage is 514.81m with 3 repetitions, while coverage is 260.49m without repetition, for {400bit payload size, 33dBm Tx power}

- Source [ZTE] provide evaluation results shows that 2 times repetition provides 2-3 dB performance gain @BLER = 0.1 compared with baseline. 3 times repetition provides 2-4 dB performance gain @BLER = 0.1 compared with baseline.

  - The candidate values of the repetition number can be [1, 2, 4, 8] if FEC is supported for PRDCH, and [1, 2, 4, 6, 8, 12, 16, 32] is recommended if FEC is not supported for PRDCH.

- Source [CMCC] provide evaluation results shows that, with 1/3 CC but without repetition, coverage of Device 2b degrades significantly when a practical 20dB penetration loss is considered, resulting in coverage distances of 712m (UMa) and 1231m (RMa), which do not provide a sufficient reliability margin.

- Source [Futuerwei] states that, for the R2D link, block repetition will complement convolutional coding.

- Source [OPPO] states that Block-level repetition(s) offers a straightforward mechanism to improve coverage without introducing substantial processing overhead, and it should be considered for R2D transmission at least for small TBS.

- Source [Panasonic] states Block level repetition would be applied for data of any size although more benefit for small TBS.

- Source [Apple] states that Block-level repetition provides additional coverage enhancement beyond FEC alone through time diversity and energy accumulation at the receiver.

- Source [Qualcomm] states that, it is necessary to use block-level repetitions to improve the interference robustness.

- Source [TCL] states that, R2D Block level repetition can be considered together with FEC encoding to further increase the performance.

- Source [Sequans] states that block level repetitions offers gains via non-coherent combining at the reader and provides time/frequency diversitywhile keeping device complexity and occupancy low, and low (e.g., <=4) number of repetitions can be supported.

- Source [IITK] States that Block-level repetition provides the highest diversity gain for R2D by enabling combining of multiple repetitions, and remains feasible for Device 2b/C despite increased memory and latency requirements.

[Neutral views]

Source [Nokia] states that, it may be beneficial for extending R2D coverage, but the necessity depends also on how R2D coverage compares with D2R coverage.

[Negative views]

Source [Spreadtrum],[Ericsson] and [Xiaomi] state that R2D Block level repetition is not necessary, and the distance target can be fulfilled without repetition.

Source [Samsung] states that, the necessity of R2D repetition depends on whether evaluation results can satisfy the coverage requirements.

[End of TP]

2.1.2 [TP][N] Proposal 3.3-1 Necessity of R2D FEC

FL notes

- Necessity of R2D FEC has no outcome in previous meetings.

- NEW TP is provided to add company views in TR38.769.

- TP is Updated based on comments from Qualcomm and Xiaomi in FL Summary.

FL Proposal 3.3-1-v1

Regarding Necessity of R2D FEC, Capture following in TR38.769.

[Positive views]

Sources [FUTUREWEI], [Huawei], [CATT], [CMCC], [ZTE], [vivo], [NEC], [Apple], [InterDigital], [Qualcomm], [TCL], [Sequans], [Panasonic], [NTT DOCOMO] and [Quectel] report that it is feasible and necessary to support R2D FEC to achieve required target coverage for outdoor scenario.

- Source [Huawei] provides evaluation results shows that, it is necessary to support R2D enhancements including FEC to ensure R2D transmissions with high reliability, facilitate early termination, and usability for outdoor scenarios considering co-site deployment.

  - With M = 1 and no FEC, interleaving or repetitions, it can only achieve 124.4m - 267.6m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which is not sufficient for outdoor scenarios with co-site deployment.

  - Introducing FEC, specifically TBCC, provide ~4 dB performance gain for Rel-20 R2D transmissions, which is necessary for outdoor scenarios and beneficial for coverage enhancement.

- Source [vivo] provides evaluation results shows that R2D FEC is necessary to achieve desirable coverage, compared with cases without FEC and without R2D repetition.

  - For UMa scenario, to achieve 500-meter coverage, FEC is necessary for the following cases:

    - The coverage is 591.34m with 1/3 FEC, while coverage is 373.46m without FEC, for {96bit payload size, 43dBm Tx power}.

    - The coverage is 705.67m with 1/3 FEC, while coverage is 276.53m without FEC, for {400bit payload size, 43dBm Tx power}.

    - The coverage is 525.6m with 1/3 FEC, while coverage is 205.97m without FEC, for {400bit payload size, 38dBm Tx power}.

  - For UMa scenario, to achieve 334-meter coverage, FEC is necessary for the following cases:

    - The coverage is 440.45m with 1/3 FEC, while coverage is 278.17m without FEC, for {96bit payload size, 38dBm Tx power}

    - The coverage is 391.49m with 1/3 FEC, while coverage is 153.41m without FEC, for {400bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, FEC is necessary for the following cases:

    - The coverage is 984.03m with 1/3 FEC, while coverage is 472.75m without FEC, for {400bit payload size, 43dBm Tx power}.

    - The coverage is 652.24m with 1/3 FEC, while coverage is 475.57m without FEC, for {96bit payload size, 38dBm Tx power}.

    - The coverage is 730.44m with 1/3 FEC, while coverage is 350.92m without FEC, for {400bit payload size, 38dBm Tx power}.

    - The coverage is 542.2m with 1/3 FEC, while coverage is 260.49m without FEC, for {400bit payload size, 33dBm Tx power}.

- Source [vivo] further provides evaluation results shows that R2D FEC is necessary to achieve desirable coverage, compared with cases with R2D repetition and without FEC, under the same data rate.

  - For UMa scenario, to achieve 500-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 560.8m with 2 repetitions and 1/3 FEC, while coverage is 489.73m with 6 repetitions only for {96bit payload size, 38dBm Tx power}

    - The coverage is 538.14m with 2 repetitions and 1/3FEC while coverage is 445.67m with 6 repetitions only for {400bit payload size, 33dBm Tx power}

  - For UMa scenario, to achieve 334-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 347.97m with 2 repetitions and 1/2 FEC while coverage is 307.47m with 4 repetitions only for {96bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 1154-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 1219.52m with 2 repetitions and 1/2FEC while coverage is 1063.3m with 4 repetitions only for {400bit payload size, 43dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 561.94m with 2 repetitions and 1/2FEC while coverage is 487.05m with 4 repetitions only for {96bit payload size, 33dBm Tx power}

- Source [CATT] provides evaluation results shows that, without LTE interleaving, TBCC codes with coding rate of 1/2 have more than 5dB coding gain over the Manchester code in TDL-C channel with 300ns delay spread, for OOK-4 with M=2, 6, 12 at BLER=10^-1, under the same data rate.

- Source [Futurewei] states that, R2D FEC coding gains from convolutional codes far exceed the gains from repetition,and it is necessary to support any channel coding on the R2D link as errors due to long distance propagation, fading, and low SNR.

- Source [CMCC] states that, to ensure the coverage targets are met under practical conditions and to support larger payloads, the adoption of FEC and repetition becomes beneficial and necessary.

- Source [Docomo] states that, FEC offers additional coding gain and generally outperforms repetition at equivalent data rates, and FEC should be prioritized compared to repetition.

- Source [Apple] states that, FEC based on LTE TBCC is essential for achieving outdoor coverage targets for device 2b/C given the transmission power constraints.

- Source [Qualcomm] states that, it is necessary to use FEC, interleaver to improve the interference robustness.

- Source [OPPO] states that FEC should only be considered if repetition proves insufficient.

[Neutral views]

Source [Nokia] states that, it may be beneficial for extending R2D coverage, but the necessity depends also on how R2D coverage compares with D2R coverage.

[Negative views]

Source [Spreadtrum], [Ericsson], [Xiaomi] and [IITK] state that R2D FEC is not necessary, and the distance target can be fulfilled without FEC.

Source [Samsung] states that, R2D FEC requires decoding at the device side, which may significantly increase device complexity and energy consumption, while it provides limited benefit considering R2D is not bottleneck.

[End of TP]

2.1.3 [TP][N] Proposal 4.1-3 Time location of the CFO calibration signal

FL notes

- Time location of the CFO calibration signal has no outcome in previous meetings.

- NEW TP is provided to add company views in TR38.769.

- TP is Updated based on coments from ETRI, Futurewei (Editorial), Docomo and CATT

FL proposal 4.1-3-v1

For the time location of the CFO calibration signal, capture following in TR38.769

Option 1: In relation to the L1 R2D control information

- For CFO calibration signal preceding L1 R2D control information

[Positive observations]

  - Source [CATT] and [LGE] states that R2D CFO calibration signal should be located before the L1 R2D control information to enable frequency offset correction before reception of control and data, enhancing decoding performance.

  - Source [LGE] states that option 1 provides benefits for potential R2D FDM operation, as the device can perform R2D processing with reduced residual CFO.

  - Source [Panasonic] states that, if the PRDCH / L1 R2D control information is device specific, option 1 could be easier as other devices are not required to determine the TBS comparing with option 2.

  - Source [Docomo] states that CFO calibration signal transmitted before L1 R2D control can be applied for CFO calibration for L1 R2D control (if needed), PRDCH (if needed), aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

[Negative observations]

  - Source [Huawei] states that, it is not necessary to transmit an CFO calibration signal prior to an R2D transmission after the periodic block since the device has already calibrated its LO using the CFO calibration signal transmitted along with the periodic synchronization signal and the MIB-like broadcast information, and can take advantage of the residual CFO and the minimal frequency drift.

  - Source [vivo] states that, the start of CFO calibration signal cannot be reliably determined due to time drift if it is located before L1 control information and before SIP.

  - Sourve [vivo] states that, CFO calibration signal immediately preceding L1 control could cause interruption to subsequent control or data payload reception due to calibration/RF switching delay.

  - Source [TCL] states that option 1 may require the device's RF/baseband to have a high-speed frequency offset estimation loop that can be adjusted within a few symbol times, which would be a challenge for AIoT devices.

  - Source [vivo] states that, the CFO calibration accuracy is unstable if PRDCH with L1 control transmission is aperiodic.

Option 2: In relation to the data payload of the PRDCH

- For CFO calibration signal preceding data payload of PRDCH

[Positive observations]

  - Source [LGE] states that transmitting the CFO calibration signal before PRDCH reception allows the device to update its CFO estimate immediately prior to the R2D transmission, and provides benefits for potential R2D FDM operation, as the device can perform R2D processing with reduced residual CFO, when CFO calibration signal is transmitted before the data payload of the PRDCH.

  - Source [Docomo] states that CFO calibration signal transmitted after PRDCH can be applied for CFO calibration for aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

[Negative observations]

  - Source [CATT] states that R2D CFO calibration signal will not be able to apply to the detection of L1 R2D control information prior to time location of CFO calibration signal.

  - Source [vivo] states that CFO calibration signal immediately before R2D data payload would cause interruption to the payload reception.

- For CFO calibration signal after data payload of PRDCH at the end of a R2D transmission

[Positive observations]

  - Source [Samsung] states that the CFO calibration signal be placed right after the R2D data payload. This allows devices not requiring calibration to ignore the signal and enter sleep earlier, while avoiding interference with ongoing payload decoding. Besides, the impact of padding can be further studied.

  - Source [Huawei], [Samsung] and [ETRI] state that an aperiodic CFO calibration signal transmitted after the PRDCH (e.g., paging message), with its presence indicated by L1 control.

  - Source [TCL] states that placing the calibration signal after receiving the PRDCH has minor impact on timeliness and does not delay acquisition of control info.

  - Source [vivo] observes that interruption of R2D transmission can be avoided if CFO calibration signal is transmitted at the end of the R2D block (after the data payload).

  - Source [Qualcomm] supports aperiodic LO calibration using CFO calibration signal transmitted together with the PRDCH.

  - Source [Sequans] and [ETRI] state that it is efficient to include CFO calibration signal together with PRDCH transmission.

  - Source [Lenovo], [NEC], [Spreadtrum] and [Xiaomi] states that it is beneficial to place the CFO calibration signal after the PRDCH to improve performance of PDRCH.

[Negative observations]

  - Source [vivo] states that the availability of CFO calibration signal depends on traffic if not transmitted along with broadcast information.

Option 3: In relation to the PDRCH (D2R transmission),

- the following observations are made assuming the CFO calibration signal transmitted prior to the PDRCH (D2R transmission)

[Positive observations]

  - Source [ZTE], [LGE] and [Docomo] state that the periodic CFO calibration signal is transmitted before PDRCH/DO-A transmission, which is beneficial for ensuring frequency accuracy for D2R transmissions.

  - Source [Apple] notes that independent transmission of the calibration signal (Option 3) can be considered as a viable but it requires additional device wakeup.

[Negative observations]

  - Sources [CATT] states that R2D CFO calibration signal is transmitted before the PDRCH, and it cannot be used for the calibration of the device clock for LO for carrier frequency for the reception of PRDCH.

  - Source [vivo] states that, standalone CFO calibration signal in relation to PDRCH (not transmitted with R2D) causes resource fragmentation, and leads to high overhead of transmitted in relation to all PDRCH transmissions.

  - Source [vivo] and [TCL] states that the start of CFO calibration signal cannot be reliably determined due to accumulated time drift after the previous synchronization.

  - Source [ZTE] states that device power consumption may increase in this option, since the device needs to wake up twice to receive common signals.

  - Source [Sequans] states that having a separate CFO calibration signal transmission will be inefficient for ultra-low-power AIoT devices due to extra monitoring and wake/sleep overhead.

  - Sources [Huawei], [Ericsson], [vivo] and [CMCC] find that this location offers no remarkable difference or gain for D2R reception performance due to slow CFO drift. Source [Huawei] further states that periodic CFO calibration signal with a predefined period will always help to improve the D2R performance by maintaining the residual CFO level with very small additional frequency drift.

Option 4: In relation to the synchronization signal

- For CFO calibration signal transmitted preceding the synchronization signal

[Negative observations]

  - Source [vivo] states that CFO calibration signal preceding synchronization signal would cause interruption to synchronization signal reception.

- For CFO calibration signal transmitted after the synchronization signal in a R2D block

[Positive observations]

  - Source [vivo] states that CFO calibration signal can be transmitted immediately after the periodic synchronization signal (if not with broadcast). This ensure a stable accuracy due to periodic transmission.

  - Source [vivo], [ZTE] and [ETRI] state that this option minimizes resource overhead, and achieves higher resource efficiency.

  - Source [ZTE] state that placing common signals within the same time period is beneficial for device power saving.

  - Source [Huawei] states that periodic CFO calibration signals can be transmitted immediately after a periodic synchronization signal and MIB-like part.

  - Source [Apple] recommends that the CFO calibration signal be transmitted at the end of the periodic synchronization signal as a baseline, efficiently utilizing device wake-up periods.

  - Source [Qualcomm], [LGE], [OPPO] and [TCL]states that CFO calibration signal as part of the periodic synchronization signal. Source [TCL] states that periodic synchronization signals are very suitable for also performing frequency offset calibration at the same time.

  - Source [CMCC] states that AIoT device can first search the periodic synchronization signal for time synchronization and frequency acquisition due to it is robust to CFO, and then receive the CFO calibration signal.

  - Source [interDigital] states that periodic R2D synchronization signal contains at least one sequence for timing synchronization and one sequence for CFO calibration, which would reduce monitoring complexity.

  - Source [NEC] and [ETRI] state that the CFO calibration is transmitted immediately after periodic synchronization signal is beneficial to D2R performance.

  - Source [Docomo] states that CFO calibration signal is transmitted after synchronization signal and can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

[Negative observations]

  - Source [ZTE] states that the periodicity of CFO calibration signal along with periodic synchronization signal may not suitable for D2R.

Option 5: In relation to broadcast information

- For CFO calibration signal transmitted preceding broadcast information

[Positive observations]

  - Source [CATT] and [Docomo] state that R2D CFO calibration signal be received before broadcast information for better detection/decoding of PRDCH signals.

  - Source [ZTE] states that periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information, is beneficial for reliable reception of broadcast information, and also beneficial for power saving. And there is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information.

[Negative observations]

  - Source [ZTE] states that CFO calibration signal is scheduled after the synchronization signal and before the broadcast information limits the flexibility for transmission of CFO calibration signal.

  - Source [ZTE] states that the periodicity of CFO calibration signal along with broadcast information may not suitable for D2R.

  - Source [Huawei] states that it is not necessary to transmit an CFO calibration signal prior to an MIB-like transmission of the periodic block since the device may need additional delay to calibrate its LO frequency and it will potentially introduce time gap between the CFO calibration signal, which will increase the resource overhead of the system and the implementation complexity of the device.

- For CFO calibration signal transmitted after broadcast information at the end of the R2D block

[Positive observations]

  - Source [CMCC] states that the synchronization signal, CFO calibration signal, and MIB-like broadcast information can be transmitted as a block, with CFO calibration signal immediately after the synchronization signal or the MIB part, AIoT device can first search the periodic synchronization signal for time synchronization and frequency acquisition due to it is robust to CFO, and then receive the CFO calibration signal.

  - Source [Huawei] states that periodic CFO calibration signals can be transmitted immediately after a periodic synchronization signal and MIB-like part, this enable the device calibrated its LO on receiving the CFO calibration signal initially along with the synchronization signal.

  - Source [vivo] supports CFO calibration signal transmitted at the end of the R2D block with broadcast information (MIB-like or SIB1-like), ensuring stable calibration accuracy, and it provides common reference accessible to all devices, and resource overhead is minimized.

  - Source [NEC] and [Docomo] state that the CFO calibration is transmitted along and immediately after broadcast information is beneficial to D2R performance.

[End of TP]

2.1.4 [TP][N] Proposal 4.1-4 Other details of CFO calibration signal

FL Notes

- This is a new TP for other details of CFO calibration signal.

- Updated based on reply from Huawei.

- Views from Qualcomm is related to whether to support periodic CFO calibration signal or Aperiodic CFO calibration signal, and I add Qualcomm’s view to Proposal 4.1-2. Hence, not reflected in this TP.

- Views from CATT has already been captured in TR since last meeting, and can be found in proposal 4.1-2. Hence, not reflected in this TP.

FL proposal 4.1-4-v1

Regarding other details of CFO calibration signal, capture following in TR 38.769

[Periodicty]

For Periodicty of CFO calibration signal

Source [CMCC] states that, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

Source [ZTE] states that, the periodicity of CFO calibration signal should be same as or multiple of the periodicity of DO-A resources.

Source [DOCOMO] states that, considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient.

Source [Xiaomi] states that for the purpose of power saving at reader side, a large periodicity can be considered for CFO calibration signal transmission.

Source [Sequans] states that, support periodic transmission with sparse periodicity for CFO calibration signal.

[Frequency location]

For Frequency location of the CFO calibration signal

Source [NEC] state that exact frequency of CFO calibration signal may need to be explicitly indicated from reader and then determined by the device.

Source [vivo] state that in predefined frequency location within the transmission BW of the associated R2D transmission, e.g., center frequency of the periodic sync signal (AIoT-SSB) or one subcarrier adjacent to the center frequency of AIoT-SSB.

Source [Samsung] states that, it is preferred that frequency positions of the CFO calibration signal start with same frequency domain resource as of R2D transmission.

Source [Lenovo] suggest to study the following frequency-allocation alternatives for the CFO calibration signal, Alt 1. Transmit on the initial frequency used to provide initial access to A-IoT devices, and Alt 2. Transmit on the frequency that provides resources for D2R data payload transmission.

[CFO calibration signal presence/absence]

Regarding CFO calibration signal presence/absence

sources [Huawei], [Xiaomi] and [Samsung] state that the presence of CFO calibration signal can be indicated e.g., in L1 control signal, for flexibility and overhead reduction. Source [Huawei] additionally state that this indication is used in the case of aperiodic CFO calibration signals.

[End of TP]

2.1.5 [TP][N] Proposal 4.4-1 Whether synchronization signal transmission is strictly periodic

FL notes

- There is an FFS in last meeting on whether synchronization signal transmission is strictly periodic at reader

- The TP is provided to reflect company views on strictly or non-strictly periodic.

- Updated based on comments from Qualcomm, Panasonic, Lenovo, Docomo, and Huawei.

FL Proposal 4.4-1-v1

Regarding whether synchronization signal transmission is strictly periodic at reader, capture following in TR 38.769

Option 1: Synchronization signal transmission is strictly periodic at reader

[Positive observations]

- Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition since it cannot know when to expect the signals.

- Source [Huawei] state that if the synchronization signal is periodic with respect to the reader, it will be transmitted with a pre-defined period and the device would be able to detect it periodically, enabling it to perform the timing synchronization every time in order to access to the network and maintain the timing tracking to obtain the accurate timeline. The synchronization signal is periodic in order to ensure that the device can perform time and frequency synchronization periodically in order to be able to access the network in a timely manner.

- Source [Spreadtrum] states that for the reader, there is no additional cost/complexity for transmitting the sync signal strictly periodic, and for the device, it is beneficial for monitoring and energy-saving operations.

- Source [Ericsson] states that the sync-signal should be strictly periodic to handle new devices attempting access at any time, which is a non-deterministic event.

- Source [CMCC] states that strictly periodic transmission is necessary to avoid ambiguity in determining system timing, which is critical for RACH resources and DRX configurations. The overhead is low from the network perspective.

- Source [Sony] and [Docomo] states that periodic transmission should be considered as baseline for frequency acquisition and timing synchronization/tracking.

- Source [vivo] states that strictly periodic occasions help avoid blind searching within a transmission window, which would increase device power consumption.

- Source [Lenovo] states that R20 AIoT systems require precise synchronization, with periodic synchronization signals playing a critical role.

[Negative observations]

- Source [Futurewei] states that it imposes unnecessary scheduling constraints for the reader and devices may not benefit from it.

- Source [Samsung] states that strict periodicity could lead to overlaps with other signals and introduce blocking/collisions based on current resource determination methods.

Option 2: Synchronization signal transmission is not strictly periodic at reader (e.g., window-based or flexible)

[Positive observations]

- Source [Samsung] states that prefer up to reader determination of resource position of periodic synchronization signal, which ensure at least one synchronization signal being transmitted within the time interval corresponding to synchronization periodicity. It is necessary to support reader side flexibility to adjust the position of periodic synchronization signal. If other signals which can be utilized for synchronization functionality are transmitted before the expected time position of periodic synchronization signal, it is feasible to cancel/postpone the transmission of periodic synchronization signal with no noticeable performance loss.

- Source [CATT] and [Panasonic] states that R2D synchronization signal transmission should not be strictly periodic at reader to avoid collisions.

[Negative observations]

- Source [Huawei] states that window-based transmission impacts the device's ability to perform frequency acquisition due to timing uncertainty.

- Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition due to timing uncertainty since it cannot know when to expect the signals.

- Source [vivo] states that a flexible time position within a window requires blind searching, increasing power consumption, and requires additional indication of the actual location.

- Source [vivo] states that the flexibility to omit transmissions (while keeping occasions strictly periodic) is consistent with NR specification logic where BS is not mandated to transmit SSB periodically.

- Source [CMCC] states that if the R2D synchronization signal is not strictly periodic, it will lead to ambiguity for a device to determine the timing information of the system. Without timing information, Rel-20 A-IoT system is difficult to configure RACH resources to support DO-A traffic, and DRX-like configuration for device power saving, etc.

- Source [vivo] states that if transmission periodicity for periodic sync signal is long enough, e.g., longer than several hundred ms, there is no clear motivation to have flexibility for BS to adjust the transmission occasion within a window or omit for periodic sync signal.

Source [NEC] and [Qualcomm] state that, RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D sync signal. It is unnecessary to further discuss whether R2D sync signal transmission is strictly periodic at reader in RAN1, it can be totally up to reader’s implementation.

[End of TP]

2.1.6 [TP][N] Proposal 4.4-2 periodicity of periodic synchronization signal

FL notes:

- This TP is proposed last meeting, and it was commented by some companies that this belongs WI details and TP not needed.

- There are still 5 companies discussed the periodicity of periodic synchronization, the Text proposal is provided again.

- Updated based on comment from CATT in FL summary.

FL Proposal 4.4-2-v1

Capture following in TR 38.769

Periodicity of R2D periodic synchronization signal is studied, with following observations

- Source [vivo] states that it is beneficial to consider a long periodicity for periodic sync signal block (AIoT-SSB) to avoid collision between R2D/D2R transmissions and AIoT-SSB. The periodicity can be in between [320ms, 1.28s], and short periodicity may lead to collision with other R2D transmissions.

- Source [CMCC] states that frequency acquisition and reader identification latency, the timing drift due to SFO within the periodicity of the synchronization signal and the overhead of synchronization signal should be considered when determine the periodicity of the R2D periodic synchronization signal. And periodicity can be hundreds of ms, e.g., 160ms, as a starting point.

- Source [Ericsson] states that dense periodic synchronization signal can accelerate reader/cell search and improve timing synchronization at the cost of increased resource consumption (and energy consumption at the device side) and signaling overhead, whereas sparse transmission may reduce resource usage but require extra preamble overhead for reliable PRDCH demodulation. The periodicity can be 160ms, and other value can be studied.

- Source [Sequans] states that, periodicity in the order of 100s of ms can be considered, to support calibration for DO-A traffic case (i.e., device initiates D2R transmission after fast fix from such periodic R2D) without introducing large latency penalty.

- Source [ZTE] states that 40ms can be a starting point , and the Periodic R2D synchronization signal can be transmitted at a specific frame boundary.

- Source [Docomo] states that for time synchronization/tracking, additional synchronization signal without corresponding broadcast information can be beneficial for reducing accumulated time error due to SFO while avoiding additional overhead of broadcast information

- Source [CATT] states that the transmission period of the periodic synchronization signals should be longer than 160 ms.

[End of TP]

2.1.7 [TP][N] Proposal 4.4-4 whether to apply manchester coding to periodic synchronization signal

FL notes

- New TP on whether Manchester coding is applied to the binary sequence

- Updated based on comment from Qualcomm in FL summary.

FL Proposal 4.4-4-v1：

Capture the following in TR38.769

Regarding whether to apply manchester coding to periodic synchronization signal

[Negative to apply manchester coding]

- Source [Huawei] states that, applying Manchester coding to the sequence-based synchronization signal would result in a marginal performance loss with similar residual timing error performance, while increasing the complexity of the device if it is expected to remove the Manchester encoding before performing correlation with the original sequence.

- Source [Qualcomm] states that, applying Manchester coding to synchronization signal could lead to an increased false alarm rate due to its resemblance to the PRDCH.

[FFS to apply manchester coding]

- Source [Ericsson] states that, before applying Manchester coding to the binary sequence, it needs to be verified/evaluated whether it outperforms or not the performance of a binary sequence designed with good auto-correlation and cross-correlation properties.

[Positive to apply manchester coding]

- Source [OPPO] states that, applying Manchester coding to the binary sequence of synchronization signal should be considered, and the application of Manchester coding to the binary sequence ensures continuous clock embedding, which mitigates timing drift, reduces DC offset issues, and facilitates accurate signal detection with lower complexity.

- Source [CATT] states that, in order to prevent continuous 1s or 0s during the transmission of R2D synchronization signal and improving clock recovery, Manchester coding should be applied to the binary sequence of R2D periodic synchronization signal.

- Source [vivo] provide evaluation results show that for 31-bit m-sequence with Manchester coding, 31-bit m- sequence without Manchester coding, and 63-bit without Manchester coding, these three cases have comparable durations (~15 OFDM symbols), and shows similar MDR performance.Besides, timing performance for sequence with Manchester coding is slightly better compared with longer sequence w/o Manchester coding (same overall duration), due to narrower main lobe of auto-correlation can be achieved with Manchester coding.

- Source [Qualcomm] states that, there is no major impact on miss detection and timing performance if Manchester coding to synchronization signal, while this approach helps prevent consecutive 1s or 0s, it could also lead to an increased false alarm rate due to its resemblance to the PRDCH.

[End of TP]

2.1.8 [TP][N] Proposal 4.5-1 Time and frequency resources for broadcast information

FL notes

- This is a new TP, for time frequency resource for broadcast information, including, time location e.g., in relation to periodic sync signal, whether broadcast is strictly periodic, frequency location, etc.

- This TP is updated based on comments from Qualcomm, Samsung, Lenovo and ZTE.

FL Proposal 4.5-1-v1

Capture following in TR38.769 regarding time and frequency resources for broadcast information:

Time and frequency resources for broadcast information is studied, and the following observations are provided.

For time location of broadcast information

[immediately after periodic synchronization signal]

Source [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [vivo], [Apple], [CMCC], [ETRI], [Samsung], [Lenovo], [ZTE] and [Docomo] state that broadcast information can be transmitted immediately after periodic synchronization signal.

- Source [Huawei], [Spreadtrum] and [vivo] states that this ensures the device can obtain system information (e.g., reader ID) as part of the reader identification/differentiation procedure. Source [Huawei] and [vivo] state that this applies to MIB-like broadcast information, and [CMCC] state that this can be one potential option.

- Source [Xiaomi] states that the broadcast information can be transmitted at the end of the periodic synchronization signal with fixed time domain resources to simplify device implementation.

- Source [vivo] state that it is beneficial to support periodic R2D sync signal block, aka., AIoT-SSB, which can be composed of periodic sync sequence, PRDCH for MIB-like broadcast information, and CFO calibration signal, and these parts are continuous in time.

- Source [ZTE] states that periodic CFO calibration signal and broadcast information is placed after different synchronization signals.

[Separated or have Gap from periodic synchronization signal]

Source [Ericsson], [Samsung], [CMCC], [ZTE] and [Docomo] states that, the broadcast information can be transmitted not immediately after periodic synchronization signal.

- Source [Ericsson] states that broadcast information can be transmitted with a different periodicity from the periodic sync signal and with pre-defined offset to the sync signal when transmitted. Source [CMCC] also state that, MIB-like part is transmitted separately with the synchronization signal

- Source [Samsung] state that time resource can be derived by periodic synchronization signal with a fixed gap.

- Source [Docomo] state that time resource of the broadcast information can be immediately after the R2D signal or with an L1 R2D control in between.

- Source [ZTE] state that periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information. There is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information.

[For SIB1-like broadcast information]

Source [Huawei], [CMCC] and [vivo] further provide views on time domain resource for SIB1-like broadcast information

- Source [Huawei] state that, for time resources to receive the SIB-like broadcast information, the start time resource information can either be indicated in the MIB-like broadcast information, or be fixed in relation to the periodic synchronization signal/MIB-like broadcast transmission.

- Source [CMCC] state that, for the SIB1-like broadcast information, the time resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information

- Source [vivo] state that, it is beneficial to support periodic SIB1-like transmitted separately from MIB-like broadcast information. The periodicity of SIB1-like broadcast information can be equal to or larger than that for AIoT-SSB.

For whether the broadcast information transmission is strictly periodic

Source [vivo] states that the transmission occasion for broadcast information is strictly periodic.

Source [Docomo] states that periodic transmission should be considered as baseline, and aperiodic transmission can provide flexibility for reader while it may lead to additional workload, e.g., on how much time tolerance can be allowed for broadcast information transmission. Source [Docomo] further state that the periodicity of broadcast information, up to a few hundreds of ms can be feasible depending on energy storage assumption.

Source [Samsung] states that it is preferred not to limit the broadcast information being strictly periodic at reader side.

Source [Qualcomm] states that RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D broadcast information. It is unnecessary to further discuss whether R2D broadcast information is strictly periodic at reader in RAN1.

For frequency location of broadcast information

[Same frequency as the periodic synchronization signal]

Source [FUTUREWEI], [Huawei], [Spreadtrum], [Ericsson] [Xiaomi], [vivo], [OPPO], [NTT DOCOMO] [Qualcomm], [Samsung] and [Apple] state that the same frequency resource is used for broadcast information as the periodic synchronization signal.

- Source [Huawei] states that the device has only acquired the frequency synchronization of the sync raster on which it detected the periodic synchronization signal, and same frequency resource as periodic synchronization signal applies to MIB-like broadcast information.

- Source [OPPO] states that a device can assume the same center frequency for receiving both broadcast information and the periodic sync signal, as long as IF/ZIF receiver requirements are met.

- Souce [vivo] state that M value should be ≤6 for periodic sync signal and PRDCH for broadcast to ensure applicability to deployment in 1 RB Tx BW.

[Potential different frequency as the periodic synchronization signal]

Source [Samsung] states that, frequency resource for broadcast information can be same as or derived by periodic synchronization signal, and potential FDM possibility and whether transmissions occur in narrowband or wideband should be clarifyed before determining the frequency resource.

Source [Samsung] suggest to study whether system supports multiple frequency positions for sync signal (FDM among sync signals), and whether system supports multiple frequency positions for broadcast information (FDM among broadcast information), e.g. to support a raster-based search like sync/MIB acquisition.

Source [Huawei] state that, for frequency resources to receive the SIB-like broadcast information, the frequency resource information can either be indicated in the MIB-like broadcast information, or be fixed in relation to the periodic synchronization signal/MIB-like broadcast transmission.

Source [Lenovo] states that, upon synchronizing via signals broadcast on initial frequency, devices acquire system information that enumerates available frequencies for further reception/transmission.

[End of TP]

2.1.9 [TP][N] Proposal 4.5-2 Transmission parameter resources for broadcast information

FL notes

- This is a new TP for parameters, e.g., M value, FEC, repetition, payload size for broadcast information, and whether these paraleters are fixed or have some flexibility to change.

- Update based on comments from Qualcomm, Lenovo, Samsung, and Huawei(Editorial).

FL Proposal 4.5-2-v1

Capture following in TR38.769 regarding transmission parameter for broadcast information.

Transmission parameter for broadcast information is studied, and the following observations are provided.

For transmission parameters for broadcast information,

[Fixed and predefined parameters]

Source [Futurewei], [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [ZTE], [vivo], [Apple], [ETRI], [Lenovo], [Samsung] and [Panasonic] state that L1 R2D control information is not needed for broadcast information transmission, and parameters (e.g., M value, payload size, repetition, FEC related parameters) can be fixed and predefined. Hence, L1 control information is not needed.

- Source [Huawei], [vivo] and [ETRI] state that fixed and predefined parameter applies for MIB-like broadcast information for the case broadcast information is splitted into MIB and SIB.

- Source [Spreadtrum] and [Xiaomi] state that fixed and predefined parameter simplifies device implementation.

- Source [Ericsson] also state that the M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

- Source [Ericsson] state that the need for variable TBS can be discussed, e.g., considering potential needs to increase the contents of broadcast information in a future release. If needed, the TBS can be indicated via L1 R2D control information or postamble. Otherwise, the TBS can be fixed and predefined.

For parameters for SIB1-like broadcast information when broadcast information is split into MIB-like broadcast information and SIB1-like broadcast information

- Source [Huawei] state that the L1 R2D control information for the PRDCH carrying the SIB-like broadcast information is used to indicate information such as the TBS, FEC code rate and repetition number to save the indication overhead of the MIB-like broadcast information.

- Source [vivo] state that L1 R2D control information is needed for SIB1-like broadcast information, and the parameters (including parameters of L1 control information of SIB1-like broadcast information, the start time offset and periodicity) can be predefined and/or indicated by MIB-like broadcast information.

- Source [ETRI] state that if broadcast information is split into MIB and SIB, the transmission parameters of SIB may be fixed and predefined or indicated via MIB.

[Potential flexibility for transmission parameters]

Source [Qualcomm] state that for system information broadcast messages, L1 control is useful to indicate the transmission parameters, such as TBS and relevant fields for subsequent system information (SIB) within the same PRDCH. Device can perform SIB monitoring with a predefined periodicity and detect SIB scheduled by the L1 control in the same PRDCH, which is more straightforward and potentially more efficient than using a separate MIB.

Source [NTT DOCOMO] state that for each parameter of the PRDCH with broadcast information, such as M value, TBS, etc., how much flexibility should be ensured can be further studied. Opt.1) fixed value is applied, Opt.2) indicated by R2D L1 control with/without restriction on applicable values.

Source [ZTE] state that both broadcast information with or without L1 control can be studied. The code method, code rate, chip duration, and repetition can be a fixed and L1 control is not needed. The flexibility can be improved if L1 control is available for broadcast information, the chip duration of L1 control part in broadcast information, the chip duration of L1 control part is a fixed value.

[End of TP]

2.1.10 [TP][U] Proposal 3.3-4 Order of channel coding and block level repetition

FL Notes

- Update TP based on company inputs

- Remove the Figure caption in the TR

FL Proposal 3.3-4-v1

Update TR 38.769 for the order of channel coding and block level repetition

6A.1.x.y PRDCH overall [Omit Unchanged part] Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-14, ZTE], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-19, Apple] and, [R1-10321-123-24, Qualcomm], [Panasonic] , [IIT] and [Interdigital] report that channel coding before block-level repetition should be supported, as shown in Figure 6A.1.x.y-1, with following justifications: - Sources [R1-10321-123-3, vivo], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT] and [R1-10321-123-24, Qualcomm] state that less memory is required by the channel coding before block-level repetition, compared with channel coding after block-level repetition. - Source [R1-10321-123-6, Huawei] states that the order of the channel coding first, followed by block-level repetition should be supported because of the diversity gain achieved due to the increased distance of a given parity bit in the output stream when compared to the reverse order. - Source [R1-10321-123-9, CATT] states that block-level repetition before FEC scheme requires Rblock times more interleaver memory as well as higher implementation complexity and processing delay for both devices and reader. - Source [Panasonic] and [IIT] state, block-level repetition after channel coding enables time diversity and the soft-combining gain. Figure 6A.1.x.y-1: PRDCH generation – block order 1 Sources [R1-10321-123-12, OPPO] and [R1-10321-123-16, LGE] state that channel coding after block-level repetition should be supported, as shown in Figure 6A-1.x.y-2. Figure 6A.1.x.y-2: PRDCH generation – block order 2

2.1.11 [H] Proposal 3.3-5 Down-selection Order of channel coding and block level repetition

For issues raised by OPPO,

- 11 companies support channel coding before block level repetition, while 2 companies prefer the other order, based on TP.

- This proposal is trying to downselect and go with majority.

- In FL Summary OPPO also would like to clarify the order of CRC , bit-collection and block level repetition.

- It seems block level repetition is still after CRC attachement, if FEC is before block level repetition.

- After offline check with OPPO, OPPO suggest at least to clarify the order of bit collection in relation to block level repetition, a note is added to clarify that bit-collection is also included in the interleaving, thus bit-collection is performed together with channal coding, and it is also performed before block level repetition.

FL Proposal 3.3-5-v2:

For R2D, channel coding (with interleaving, if supported) is before block-level repetition (if supported).

Note: interleaving includes potential sub-block interleaving (if supported) and bit collection.

2.1.12 [H] Proposal 4.4-5 Down-selection Overlaid sequence on periodic sync signal

FL Notes

- It was agreed to study overlaid sequence for periodic sync signal.

- In this meeting, no source support overlaid sequence, and several sources express their concerns. Hence, suggest to exclude overlaid sequence on periodic sync signal.

FL Proposal 4.4-5-v1

Overlaid sequence on R2D periodic synchronization signal is not consider for R20 AIoT.

2.1.13 [TP][U] Proposal 4.4-1 Signal design for CFO calibration signal

FL Notes

- TP is agreed last meeting, and it is further updated based on company inputs this meeting

- Further Updated based on comments from Qualcomm in FL Summary.

FL Proposal 4.1-1-v1

Update TR 38.769 for signal design for CFO calibration

For the signal design, the following options are studied: Option A: Unmodulated single tone sinusoid. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-8, NEC], [R1-10321-123-10, CTC], [R1-10321-123-11, Xiaomi], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-18, Lenovo], [R1-10321-123-19, Apple], [R1-10321-123-21, MediaTek], [R1-10321-123-22, Sharp], [R1-10321-123-23, Quectel], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-26, TCL], [R1-10321-123-27, IITK], [R1-10321-123-28, CEWiT], [R1-10321-123-29, Sequans], [R1-10321-123-20, InterDigital], and [R1-10321-123-30, Sony], [Nokia] and [Panasonic] state that Option A is feasible, and can be considered for CFO calibration signal. The following design principles for this option are described according to sources: - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei] and [R1-10321-123-25, NTT DOCOMO], report with references, and state that, the CFO calibration is based on injection lock mechanism, and the working principle of injection lock is to leverage the phase interaction between an external single-tone signal and the internal oscillation signal of an oscillator to lead the oscillator’s frequency lock to that of the unmodulated sinusoid single-tone signal. - Source [R1-10321-123-5, CMCC] report with simulation result, and states that, CFO calibration using unmodulated sinusoid is performed at intermediate frequency, where the reader transmits the CFO calibration signal with a pre-defined duration, the device mixes the CFO calibration signal with its internally generated wave, can then count the number of periodicities of the residual frequency within the duration to estimate and calibrate the CFO. The following sources provide reasons to support this option: - Source [R1-10321-123-5, CMCC] report that with a duration of 1 OFDM symbol, the CFO after calibration can be reduced to less than 10 ppm. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-7, Ericsson], [R1-10321-123-10, CTC], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-24, Qualcomm], [R1-10321-123-18], and [R1-10321-123-27, IITK], [Xiaomi], [Nokia], [Apple], [Sharp], [Panasonic], [TCL], [Sequans] and [Quetel] state that unmodulated sinusoid single tone can achieve CFO calibration with low complexity. Source [ZTE] states that it allows CFO correction to be implemented using hardware circuits (e.g., AFC loop), making it well-suited for A-IoT devices. And Source [Sequans] states that the constant-amplitude single-tone allows simple phase-based CFO estimation without correlation or symbol detection. - Source [R1-10321-123-3, vivo] states that single tone signal is already supported in NB-IoT, (e.g., NPRACH), there is no co-existence issue. - Source [Qualcomm] provide evaluation results and show that, By using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=20dB and 1% error at SNR=30dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB. - Source [Qualcomm] provide evaluation results and show that Device 2b requires a higher CNR than Device C because of its larger SFO/CFO before/after calibration, e.g., by using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=8dB and 1% error at SNR=18dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB. - Source [Quectel] states that, the envelope of unmodulated signal for CFO calibration is constant, which provides higher calibration precision than modulated signal. The following sources provide reasons to not support this option: - Source [R1-10321-123-9, CATT] states that it will create the inter-channel interference to neighboring NR sub-bands. - Source [R1-10321-123-24, Qualcomm] and [CATT] states that this method may be vulnerable to deep fading, reducing reliability. - Source [CATT] states that, injection lock mechanism will consume huge energy of the device, which is unacceptable even for active devices. Option B: Unmodulated multiple single tone sinusoids. - Sources [R1-10321-123-16, LGE ], [R1-10321-123-24, Qualcomm], and [R1-10321-123-26, TCL] and [Panasonic] report that Option B can be considered for CFO calibration The following reasons were provided to support this option: - Source [R1-10321-123-16, LGE ] states that, Option B provides frequency diversity gain while maintaining low complexity; considered as optional feature. It is preferable that the tones are sufficiently separated in frequency, e.g., by at least the Rx filter bandwidth. - Source [R1-10321-123-24, Qualcomm] states that, using multiple sinusoidal tones can improve robustness against fading by providing frequency diversity. - Source [R1-10321-123-26, TCL] states that, although Option B is slightly more complex than a single tone, it is still a very simple waveform. The following reasons were provided to not support this option: - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-2, Spreadtrum], [R1-10321-123-30, Sony], [R1-10321-123-26, TCL], and [R1-10321-123-23, Quectel]. [Spreadtrum], [ZTE], [NEC] and [Quectel] state that Option B increases in detection complexity, and it may also complicate frequency resource allocation. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-6, Huawei] raised feasibility concerns on Option B - Source [R1-10321-123-3, vivo] states that even with increased complexity, it is not clear how does it work, since frequency of each tone cannot be accurately determined, due to only frequency range of each can be determined if captured by Rx filter, and the target frequency for LO calibration cannot be accurately determined. - Sources [R1-10321-123-6, Huawei], and [R1-10321-123-8, NEC], [Xiaomi] and [Quectel] state that multiple frequency components would occur when mixed with the local RF frequency signal and it is difficult to lock the LO frequency. - Source [R1-10321-123-29, Sequans] state that multiple unmodulated tones increase PAPR and require FFT-based tone separation - Source [ZTE] states that the benefit of using multiple sinusoid single tones for CFO calibration is unclear compared with single tone. - Source [vivo] states that Option b (unmodulated multiple sinusoid single tones), based on counting of number of cycles of envelop of frequency components of ¦f1 – f2¦, is not feasible for achieving 10 ppm accuracy. It requires either an extremely long duration or an excessive transmission bandwidth, both of which are impractical for R20 AIoT. - Source [CATT] states that, for Options b sinusoidal signals for CFO calibration signals transmitted on NR DL spectrum, it will create the inter-channel interference to neighbouring NR sub-bands. Option C: Multiple modulated tones, where the CFO calibration signal may be the same as or different from the synchronization signal. The following three different design principles for this option are described according to sources: - Source [R1-10321-123-7, Ericsson] state that the unmodulated multiple tones are transmitted in different frequency locations in different time resources The reader transmits the synchronization signal using a centered bandwidth sweeping method, where each centered bandwidth covers a portion of the overall synchronization signal bandwidth The device detects the synchronization signal power at different strengths on each of the subsets and based on the subset with the strongest synchronization signal power, the device applies a frequency correction - Sources [R1-10321-123-9, CATT] and [R1-10321-123-21, MediaTek] report that, CFO calibration is done in baseband domain, and CFO Calibration is OOK sequence (not R19 SIP or CAP). The CFO pre-compensation method is given as follows: Step 1: The received signals with pre-compensated CFO ±Δf is added in the received R2D signals to limited the frequency offset within ±1/2 Δf. Step 2: The correlation processing of the CFO calibration signal with the received signals, the precompensated received signals with additional Δf, and the pre-compensated received signals with additional -Δf. Step 3: The peak of the correlation values from the three correlation outputs would be selected as the received signals for the R2D signal processing. - Source [R1-10321-123-25, NTT DOCOMO] states that, the CFO calibration can be achieved by measuring chip length, and that one possible implementation is devices calibrate CFO based on SFO calibration by counting the number of samples of 0s and 1s and comparing with the defined duration of a pattern. The following reasons were provided to support this option: - Source [R1-10321-123-7, Ericsson] state that using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm) - Source [R1-10321-123-9, CATT] state that the OFDM waveform-based CFO calibration signals would be orthogonal to neighboring NR channels - Sources [R1-10321-123-9, CATT] and [R1-10321-123-21, MediaTek] state that R2D synchronization signal can be used for CFO calibration signal - Source [R1-10321-123-25, NTT DOCOMO] state that may be feasible if same clock (LO) is used for both carrier frequency generation and sampling. - Source [CATT] states that, modulated multiple tones, can be used for CFO calibration based on pre-compensation, R2D synchronization signal and M-sequence can be used for CFO calibration signal. - With the CFO pre-compensation method, if R2D CFO calibration signal adopts M-sequence with M value equals to 4 and its length equals to 7 bits, it will occupy only two OFDM symbols, the 90% CDF residual CFO after calibration at the device side is no more than 5ppm for Device C and 20ppm for Device 2b. - The pre-compensated CFO received signals should be used for CFO calibration to reduce the degradation of received signal detection and decoding performance due to frequency errors for Device 2b and Device C. The following reasons were provided to not support this option: - Source [R1-10321-123-7, Ericsson] state that the CFO calibration of device C may require using unmodulated sinusoid single tone to meet a residual CFO of 10 ppm. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC] and [R1-10321-123-28, CEWiT] state that the CFO hypothesis at AIoT receiver cannot be constructed in baseband due to phase information for received signal is not kept in envelop detector, and no I/Q path in receiver baseband. - Source [R1-10321-123-14, ZTE] and [vivo] report that, in multi-tone LP-SS discussion, the LP-WUR doesn’t have the capability to acquire good frequency calibration, similarly this method is not reliable. And source [vivo] further states that, according to agreements for LP-SS OOK sequence in LP-WUS WI (RAN1#118 meeting), the design metric only include time synchronization accuracy and RRM measurement accuracy, no design consideration and metric for frequency error calibration in LP-SS design. - Sources [R1-10321-123-6, Huawei] and [R1-10321-123-8, NEC] state that the LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure and the calibration signal should be utilized for the RF-end clock. - Source [R1-10321-123-3, vivo] state that using ON chip duration to calibration Clock it not feasible, since according to RAN4 requirements on pulse width, Table 6.4.2-1 in section 6.4.2 of Ts 389.194, the pulse of a chip, the allowed chip length can be <=1.3 Tc when the nominal chip length is Tc, in this case, it is not feasible to use chip length/pulse width for CFO calibration. - Source [R1-10321-123-18, Lenovo] states that pulse width distortion analysis is possible to be used for A-IoT device due to simplicity, but timer resolution at device is required to be much less than the difference between estimated pulse width and the transmitted pulse width. - Source [vivo] states that it is not feasible to use option c, modulated multiple tones, i.e., OOK sequence for R2D CFO calibration, as that in D2R receiver. - Each CFO hypothesis or CFO compensation should be constructed by multiplying a phase rotation sequence to local sequence, and each phase rotation sequence corresponds to a CFO hypothesis. However, only envelop/amplitude of the Rx signal can be obtained in IF-ED detector, and there is no I/Q path in receiver, phase information which reflects frequency offset is lost in receiver. - Estimation or compensation using R2D OOK sequence is still not feasible, since the R2D waveform generation is not specified, which means the sample level amplitude (with ripples) and phase at transmitter is unknown to receiver when constructing local sequence, because OOK signal generation totally up to Reader implementation. - It is not feasible to use RSRP-like metric to determine CFO at receiver. To achieve 10ppm calibration accuracy, several filter covering different frequency point should be used which is too complex for AIoT devices. Even if using so many IF filter, the received power captured into each filter would be very close. The right hypothesis cannot be reliably determined, the accuracy of CFO calibration cannot be guaranteed. - Source [NEC] states that, since the CFO calibration is mainly applied to the LO in RF level, which cannot be based on the baseband clock, for the signal design, Option c is not feasible.

2.1.14 [H] Proposal 4.4-1a Down Selection Signal design for CFO calibration signal

FL Notes

- Clear majority companies support option a, unmodulated sinusoid single tone as CFO calibration signal, and the principle of CFO calibration is clear, with references and evaluation results provided by proponents.

- Although there are few proponents for option B. The principle on how option b works is still not provided by any of the proponents, until this very last meeting, this option is less likely to be supported even we postpone the decision in WI.

- The signal design and principle of CFO calibration are diverse from supporters of option c. Clear majority companies does not support option c, and technical concerns are not fully addressed for any of the 3 working principles. This option is less likely to be supported even we postpone the decision in WI.

- It seems CATT still have concern on option a and prefer option c. I think we can first exclude option b, and we can further down-select between option a and option C in WI phase.

Hence, the proposal is updated as follows

FL Proposal 4.1-1a-v2

For CFO calibration signal, following option is not considered

- Option b:unmodulated multiple sinusoid single tones

2.1.15 [TP][U] Proposal 5.2-2 Frequency location of L1 R2D control information

FL Notes

- Updated based on company inputs

- No comments in FL summary, seems stable

FL Proposal 5.2-2-v1

Update TR 38.768 on frequency location of L1 R2D control information

Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-8, NEC], [R1-10321-123-14, ZTE], [R1-10321-123-19, Apple], and [R1-10321-123-24, Qualcomm], [Xiaomi], [LGE], [interDigital] and [CATT] report that the same frequency resource for R2D preamble, L1 control and data part should be used. - Source [R1-10321-123-1, FUTUREWEI] states that different frequency resource will cause additional latency as the transmission carrying control information must be first received followed by receiving the transmission carrying the payload. And unclear of the necessity of this option if FDM is not supported - Sources [R1-10321-123-3, vivo], and [R1-10321-123-6, Huawei], [Xiaomi] and [CATT] state that frequency retuning can be avoided if L1 control information and the corresponding data payload are located with same frequency locations. Otherwise, it will lead to additional adjustment delay and power consumption. - Sources [R1-10321-123-2, Spreadtrum] and [R1-10321-123-3, vivo] state that the motivation of transmitting L1 control and data payload in different frequency is not clear. Source [interDigital] state that little benefit is expected in a PRDCH center frequency different from the center frequency of the L1 R2D control channel that configured it. Source [R1-10321-123-12, OPPO] reports that frequency resource for R2D preamble, L1 control and data part can be different such as to reduce the load in the frequency resource used for L1 control.

2.1.16 [H] Proposal 5.2-3 Down Selection of Frequency location of L1 R2D control information

FL Notes

- 13 support the same frequency resourc while 1 company have different view, and FL suggest the same frequency resource for R2D preamble, L1 control and data part should be used in first version of the proposal

FL Proposal 5.2-3-v1 R-TAS, L1 R2D control information and data payload of PRDCH are transmitted at the same frequency resource.

- OPPO and Panasonic raise the question that whether both center frequency and bandwidth are same for these parts, and OPPO prefer to have flexibility for different BW for different parts to keep the flexibility for different BW for these parts.

- OPPO suggest to change the proposal from device perspective, to ensure more flexibility at reader side.

- The proposal is updated as follows to address above concern, as a small step forward. Whether the BW for L1 control and data payload is same or not can be discussed in WI phase.

FL Proposal 5.2-3-v2

AIoT device assumes the same centre frequency for receiving R-TAS, L1 R2D control information and corresponding data payload of PRDCH.

2.1.17 [TP][U] Proposal 5.3-2 Chip duration determination of L1 R2D control information

FL notes

- TP on chip duration determination of L1 R2D control is agreed last meeting, and further update based on company input this meeting.

- Update based on comments from Qualcomm and Docomo in FL summary.

FL Proposal 5.3-2-v1

Update TR 38.769 for Chip duration determination of L1 R2D control information.

6A.1.x.y.2 Chip duration of L1 R2D control information The following options are identified for the functionality of the chip duration determination of the L1 R2D control information: Option 1-control: A fixed chip duration is used, with no use of CAP. Option 2-control: A set of pre-defined chip durations, blindly detected by a device with no use of CAP. Option 3-control: A CAP is used, where multiple alternatives are studied: Option 3-1-control: Using the pattern of CAP defined in TS 38.291 [228]. Option 3-2-control: Enhancing CAP by the following options: Option 3-2a-control: Using the pattern of CAP defined in TS 38.291 [228] with repetitions, Option 3-2b-control: Using a pattern that associates the chip duration of the clock acquisition part to different chip durations of the L1 R2D control information. Option 4-control: A set of binary sequences with fixed or variable length is used, where multiple alternatives are studied: Option 4-1-control: Use of the binary sequence-based SIP, with no use of CAP. Option 4-2-control: Use of binary sequences for the CAP. Option 5-control: Broadcast information is used to indicate the chip duration. For Option 1-control, the following points were reported as advantages: - Sources [R1-10321-123-7, Ericsson], [R1-10321-123-9, CATT], [R1-10321-123-16, LGE ] and [R1-10321-123-10, CTC] states that fixed chip duration for L1 R2D control information lowers the complexity and power consumption of chip duration detection. - Source [R1-10321-123-9, CATT] states that fixed chip duration for L1 R2D control information minimize the ambiguity of the chip duration detection, and thus enhance the success rate of L1 R2D control information reception. - Source [R1-10321-123-7, Ericsson] states that using fixed chip duration (using the smallest M value for optimal coverage) can achieve higher reliability for L1 R2D control information. - Source [ZTE] state that the chip duration of L1 control part for broadcast information, is a fixed value. For Option 1-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] and [R1-10321-123-13, Samsung] states that to achieve the largest coverage, L1 control would need to use the lowest M, which increases overhead for each R2D transmission. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-13], [R1-10321-123-14], [R1-10321-123-6, Huawei], [R1-10321-123-3, vivo], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-5, CMCC], [Sony] and [Xiaomi] states that, if chip duration for L1 control information is fixed, the flexibility or transmission efficiency is restricted. For Option 2-control, the following points were reported as advantages: - Sources [R1-10321-123-5, CMCC] and [R1-10321-123-17, Panasonic] state that blind detection within configured set provide better flexibility for chip length for L1 R2D control information. - Source [R1-10321-123-5, CMCC] states that blind detection is viable if the number of predefined chip duration candidates is restricted. Additionally, [R1-10321-123-5, CMCC] states that Option 2 can be combined with device-specific configuration signaling to enhance flexibility without excessive complexity. For Option 2-control, the following points were reported as disadvantages: - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-13, Samsung], [R1-10321-123-3, vivo], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-7, Ericsson], and [R1-10321-123-14, ZTE], [NEC], [Sony] and [Xiaomi] state that Option 2 increases detection complexity. - Source [R1-10321-123-24, Qualcomm] states that it may be impractical due to constraints on power consumption and memory needed for buffering the incoming R2D L1 control information samples. - Sources [R1-10321-123-6, Huawei] and [R1-10321-123-16, LGE ] state that option 2 would require additional time gap between the L1 control information and the data payload of the PRDCH to allow for decoding delay. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-7, Ericsson] state that reliability is doubtful due to blind detection. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway to achieve flexibilities in coverage and resource overhead, no need to leave chip duration for blind detection. For Option 3-control, the following points were reported as advantages: - For Alt.3-1 Rel-19 CAP pattern, sources [R1-10321-123-13, Samsung] and [R1-10321-123-29, Sequans] state that Rel-19 CAP pattern to determine chip length of L1 R2D control part offer a balance between detection complexity and performance. - For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, sources [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple] and [R1-10321-123-24, Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Source [R1-10321-123-19, Apple] further states that this option maintains consistency with Release-19 design principles. Source [R1-10321-123-24, Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility. Source [Qualcomm] also provides simulation results that the CAP with 2bits and fixed length (fixed M=2 chip duration) can be detected with 10% false detection at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control). - Source [Docomo] state that options 3 have an advantage as flexible chip duration of L1 R2D control. For Option 3-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] states option 3 requires blind detection; and MD/FA rates and CAP overhead needs to be considered. - Source [R1-10321-123-14 ZTE] states option 3 results in additional overhead due to transmission of CAP. - Sources [R1-10321-123-5, CMCC], [R1-10321-123-3, vivo], and [R1-10321-123-6, Huawei] and [NEC] state that Option 3 is based on edge detection to determine the chip duration by using CAP, so the performance is limited or unfeasible due to the use of edge detection to detect the CAP pattern. Sources [R1-10321-123-14, ZTE] and [R1-10321-123-7, Ericsson] also report the performance of this option should be further evaluated. - Source [vivo] provide evaluation result show that it is not feasible to use R19 CAP (Alt 3-1) and R19 CAP with repetitions (Alt 3-2a) to determine chip length for L1 control part. The BLER performance is not even close (more than 10dB performance gap) to meet coverage of 500 meters at 10% BLER in UMa scenario. - Source [R1-10321-123-7, Ericsson] states that option 3 would make the decoding of the L1 R2D control information more complicated for the device. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection using CAP. - Source [Sony] state that using a CAP for chip length determination requires a two-part preamble design which is not necessary with a binary sequence based SIP design. For Option 4-control, the following points were reported as advantages: For Alt 4-1, using binary sequences for SIP. No use of CAP for chip duration determination - Sources [R1-10321-123-4, Nokia], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-16, LGE ], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-17, Panasonic] and [Xiaomi] state that this method can provide flexibility for R2D chip duration. - Source [R1-10321-123-6, Huawei] state that a restricted set of binary sequences of differing lengths are mapped to different chip durations and coverage distances, which can be detected with limited blind detection complexity. Parallel processing for the blind detection of different sequence lengths avoids and processing delay. Besides, R-TAS design is simplified, which consist only sequence-based SIP.Alt.4-2: using binary sequences for CAP. - Sources [R1-10321-123-4, Nokia] and [R1-10321-123-17, Panasonic] report that flexibility on chip length of L1 R2D control can be achieved. Source [Nokia] state that sequence-based SIP can be effectively used to indicate chip duration of a PRDCH for L1 control with limited indication flexibility to keep low device complexity, and only limited flexibility is needed, e.g., up to 2 sequence is sufficient. - Source [Sony] state that single-part preamble where a sequence-based signal can not only provide start and synchronization functionality, but it can also be used to carry information with regard to chip length of the subsequent control and data payload, has an advantage over the other approaches in terms of low-complexity and power consumption. For Option 4-control, the following points were reported as disadvantages: - Sources [R1-10321-123-3, vivo], [R1-10321-123-7, Ericsson], and [R1-10321-123-24, Qualcomm] and [Docomo] states that option 4 increases detection complexity. And [R1-10321-123-7] states that option 4 would make the decoding of the L1 R2D control information more complicated for the device. - Sources [R1-10321-123-14, ZTE] and [R1-10321-123-7, Ericsson] report that the performance of this option should be further evaluated. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection; source [R1-10321-123-3] further reports that it leader more challenges on preamble design, requiring larger number of sequences to indicate four M values in addition to reader/cell ID differentiation; For Option 5-control, the following points were reported as advantages: - Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE] and [R1-10321-123-24, Qualcomm] state that better flexibility can be achieved without blind detection, and sources [R1-10321-123-3, vivo] and [R1-10321-123-14, ZTE] further state that the chip duration for L1 can be indicated together with other parameters for control information, e.g., repetition and FEC related parameters. Source [R1-10321-123-5, CMCC] state that this method can be used for L1 R2D control information common for devices, for example, the L1 R2D control information for scheduling paging message. - Source [R1-10321-123-12, OPPO] state that the chip length should be pre-known by device to minimize complexity and power consumption at AIoT device. For Option 5-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] states that it requires system info acquisition and memorization. - Source [R1-10321-123-6, Huawei] and [R1-10321-123-13, Samsung] states that only to indicate chip duration in broadcast information limits flexible time resource allocation from system perspective. - Source [R1-10321-123-7, Ericsson] states that this option is a simple solution but not applicable for paging and broadcast PRDCH. - Source [R1-10321-123-25, NTT DOCOMO] states that state that option 5 has less flexibility on chip duration of L1 R2D control compared to option 2/3/4. - Source [Xiaomi] state that, as a common indication method, option 5 is an efficient way for chip duration indication of the L1 R2D control information but loses some flexibility for the L1 R2D control information transmission towards different Devices.

2.1.18 [H] Proposal 5.3-3 Down Selection Chip duration determination of L1 R2D control information

FL Notes

- Considering nothing is written specifically for Option 3-2a in TR and in the update TP, and more negative comments are made for option 2 then others options, suggest to exclude these two options.

- There is no objections for this proposal in FL summary, seems stable.

FL Proposal 5.3-3-v1

Following options are NOT considered for Chip duration determination for L1 R2D control information.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Alt.3-2a: R19 CAP pattern with repetition

2.1.19 [TP][U] Proposal 3.1-1 On Addition of M=1

FL Notes

- Views from Xiaomi, Samsung and Docomo are added in the update TP.

- Further add Samsung to main sentence of M=1 not necessary.

FL Proposal 3.1-1-v1

Update TR38.769, on addition of M=1, as follows

#### 6A.1.x.1 Mchipsymb=1 Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-4, Nokia], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-11, Xiaomi], [R1-10321-123-9, CATT], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-17, Panasonic], [R1-10321-123-19, Apple], [R1-10321-123-26, TCL], [R1-10321-123-27, IITK], and [R1-10321-123-29, Sequans] states that addition of Mchipsymb=1 is beneficial to improve R2D coverage. - Source [R1-10321-123-6, Huawei] shows that, Mchipsymb = 1 provides ~3 dB performance gain compared with Mchipsymb = 2 using correlation detection with TBS 20 bits and 96 bits, and ~2.5dB performance gain with TBS 400 bits, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to Clause 4.3.2. And Source [Xiaomi] states that longer chip length can enable better coverage performance than M=2 and the performance gain is ~3 dB. - Source [R1-10321-123-17, Panasonic] shows that, Mchipsymb = 1 shows performance gain, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to Clause 4.3.2. - Source [R1-10321-123-9, CATT] states that, for Mchipsymb=1, the larger chip duration could reduce the DS impact and improve the detection performance for Device 2b/C in outdoor deployment scenario. Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE], [R1-10321-123-12, OPPO], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO] and [Samsung] states that addition of Mchipsymb=1 is not necessary. - Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE], [R1-10321-123-12, OPPO], [R1-10321-123-25, NTT DOCOMO] and [Samsung] states that, OOK with M=2 using together with repetition achieve the same data rates as Mchipsymb=1. - Source [R1-10321-123-14, ZTE] further observed that OOK with M=2 using together with repetition can achieve better time-domain diversity gain and power boosting gain than OOK with Mchipsymb=1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, OFDM symbol with zero power is not counted. - Source [R1-10321-123-14, ZTE] states that, OOK4 with Mchipsymb =2 and Mchipsymb=4 have better detection performance than OOK4 with Mchipsymb =1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, OFDM symbol with zero power is not counted. - Source [R1-10321-123-24, Qualcomm] shows that, Mchipsymb=1 cannot provide better performance than Mchipsymb=2, if the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, and OFDM symbol with zero power is not counted. Source [R1-10321-123-24, Qualcomm] also shows that Mchipsymb=1 provide less than 2dB performance gain than Mchipsymb=2, if the CINR/CNR, for OOK4 signal across all OFDM symbols, is calculated according to Clause 4.3.2. - Source [Docomo] states that, M=1 does not offer performance improvements relative to M=2, unless power boosting during the ON duration are allowed

2.1.20 [TP][U] Proposal 3.1-2 On Removal of M=24

FL notes

- Views from Xiaomi, vivo, TCL and CATT in Tdoc this meeting, are added to the update TP.

- Update as suggest by Nokia, “it is not necessary to keep Mchipsymb = 24 for R2D” is changed to “Mchipsymb = 24 for R2D can be removed” as suggested by Nokia in FL Summary.

FL Proposal 3.1-2-v1

Update TR38.769, on removal of M=24, as follows

#### 6A.1.x.2 Mchipsymb=24 Sources [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-17, Panasonic], [R1-10321-123-11, Xiaomi] and [R1-10321-123-3, vivo] state that it is not necessary to keep Mchipsymb = 24 for R2D can be removed. - Sources [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson] and [R1-10321-123-17, Panasonic] state and provide evaluation results showing that Mchipsymb = 24 cannot work well for outdoor scenarios as its chip length is too short to resist large multipath delay and channel fading. - Source [R1-10321-123-6, Huawei] shows that Mchipsymb = 24 cannot work for outdoor scenarios with BLER close to 1 under different SNRs - Source [R1-10321-123-7, Ericsson] shows that 10% BLER cannot be achieved with Mchipsymb = 24 (even with repetitions) - Source [R1-10321-123-17, Panasonic] shows that, when Mchipsymb = 24 (and Mchipsymb>12), both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB or larger (even without SFO/CFO) - Source [R1-10321-123-3, vivo] states and provides evaluation results show that, R2D transmission with Mchipsymb = 24 cannot achieve 1% BLER even with 2 block-level repetitions, and under similar data rates, the performance of Mchipsymb = 24 (with 2 repetitions) is worse than that of M=12 (without repetition). Source [Xiaomi] states and provides evaluation results show that, there is an error floor at BLER of 10^-1 for M=24. - Source [vivo] states that CP handling method 2 is not needed for M=24 values in R20 due to better clock accuracy. Sources [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-12, OPPO], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-24, Qualcomm], [R1-10321-123-9, CATT], [TCL] and [R1-10321-123-4, Nokia] state that it is necessary to keep Mchipsymb = 24 for R2D. - Source [R1-10321-123-25, NTT DOCOMO] states that Mchipsymb = 24 can be supported for a device which is close to reader and beneficial for device power saving to reduce PRDCH reception duration with higher peak data rate - Source [R1-10321-123-1, FUTUREWEI] provides evaluation results show that Mchipsymb = 24 can still achieve 10% BLER with a reasonable SNR, and Source [CATT] provides evaluation results show that the coverage requirement of 500m at 10% BLER. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-12, OPPO], [CATT] and [R1-10321-123-13, Samsung] state that Mchipsymb = 24 is needed to achieve data rates similar as R19. - Source [R1-10321-123-4, Nokia] states that, Mchipsymb = 24 should not be removed, since it is still useful for indoor scenario, and the air interface for Device 2b/C studied for outdoor scenarios will be reused in indoor scenarios.

2.1.21 [TP][U] Proposal 3.1-3 On addition of M=32

FL Notes

- Views from Xiaomi, vivo, TCL and CATT in Tdoc this meeting, are added to the update TP.

- No comments in FL summary, seems stable.

FL Proposal 3.1-3-v1

Update TR38.769, on addition of M=32, as follows

6A.1.x.3 Mchipsymb=32 [omit unchanged part] Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-12, OPPO], [R1-10321-123-9, CATT], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-17, Panasonic], [R1-10321-123-25, NTT DOCOMO] and [R1-10321-123-7, Ericsson] state that it is not necessary to add Mchipsymb for R2D. - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-9, CATT], [R1-10321-123-14, ZTE], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-17, Panasonic], and [R1-10321-123-12, OPPO] state concern on feasibility of Mchipsymb=32 due to too short chip duration. - Source [R1-10321-123-6, Huawei] provide evaluation results show that, Mchipsymb=32 cannot work for outdoor scenarios with BLER close to 1 under different SNRs, as its chip length is too short to resist large multi-path delay and channel fading, so addition of Mchipsymb=32 is not supported. - Source [R1-10321-123-9, CATT] provide evaluation results show that, for Device 2b, Mchipsymb=32 cannot satisfy the 500m coverage requirement at 10% BLER, and for Device C, though it can reach 10% BLER, it has an error floor above 1% BLER. - Source [R1-10321-123-3, vivo] provide evaluation results show that R2D transmission with Mchipsymb=32 cannot achieve 1% BLER even with 3 block-level repetitions, and under similar data rates, the performance of M=32 (with 3 repetitions) is worse than that of Mchipsymb=12 (without repetition). Source [vivo] further states that M=32 can not be distinguished from M=24 at AIoT device. According to RAN4 requirements, when the nominal chip length is Tc, the allowed chip length can be 0.7*Tc to 1.3*Tc, which makes AIoT device difficult to differentiate M=32 from M=24. - Source [R1-10321-123-17, Panasonic] provide evaluation results show that, when Mchipsymb =32, both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB and larger due to smaller chip duration. - Source [R1-10321-123-7, Ericsson] provide evaluation results show that, for Device 2b and Device C, a 10% BLER cannot be achieved with Mchipsymb=32. Source [Xiaomi] provides evaluation results show that, there is an error floor at BLER of 10^-1 for M = 32. Source [IITK] provides evaluation results show that that BLER for M = 32 is close to 1 in outdoor scenarios across SNRs and failing to meet the 10% BLER target even with FEC and block-level repetitions - Sources [R1-10321-123-12, OPPO] and [R1-10321-123-13, Samsung] state that, higher data rate with Mchipsymb=32 lacks motivation. - Source [R1-10321-123-25, NTT DOCOMO] states that, it would increase device complexity as devices need to blindly detect and differentiate Mchipsymb=24 and Mchipsymb=32 with certain FDR.

2.1.22 [TP][U] Proposal 3.2-1 On Feasibility of R2D block level repetition

FL Notes

- Views from Samsung, Ericsson, and OPPO are added to the updated TP.

- Further updated based on ZTE comments in FL Summary.

FL Proposal 3.2-1-v1

Update TR 38.679 on feasibility of R2D block level repetition.

6A.1.x.y Repetition Block-level repetition as defined in Clause 6.1.0.1 is considered for the study; other types of repetition are not considered. It is reported by sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-12, OPPO], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], [R1-10321-123-24, Qualcomm], [R1-10321-123-28, CEWiT], [R1-10321-123-29, Sequans], [R1-10321-123-17, Panasonic], [R1-10321-123-23, Quectel], [R1-10321-123-25, NTT DOCOMO], [Samsung], [ZTE] and [R1-10321-123-27, IITK] that R2D repetition is feasible and can be considered. Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-9, CATT], [R1-10321-123-15, Transsion], [R1-10321-123-19, Apple], [Samsung], [ZTE] and [R1-10321-123-27, IITK] report that repetition schemes including block-level repetition are feasible. Source [Ericsson] and [OPPO] state that, block-level repetition requires a large volatile memory to store received repetitions of a block, Source [OPPO] further states that block-level repetition should be feasible for small TBS. whether block-level repetition(s) can be applied to large TBS e.g., thousands of bits, should be further studied.

2.1.23 [TP][U] Proposal 3.3-2 On R2D Coding Scheme

FL Notes

- This is update TP. Add/update views for Nokia, ZTE, Xiaomi, Futurewei, Sequans.

- Further updated based on comments from Samsung

FL Proposal 3.3-2-v1

Update TR 38.679 on R2D coding scheme.

6A.1.x.y.1 Coding schemes FEC for R2D is studied, with code rates of 1/2, 1/3 and 1/4 and the following candidate coding schemes: Coding scheme 1: LTE tail biting convolutional code Coding scheme 2: Tailed convolutional codes with the same polynomials as LTE tail biting convolutional codes Coding scheme 3: Reed-Muller codes For comparison between Coding scheme 1 and Coding scheme 2: - Source [R1-10321-123-1, FUTUREWEI] states, for the same number of information bits N, a tailed convolutional code encodes L-1 additional bits (tail bits) to ensure the final state returns to 0. With tail bits, a typical decoder can operate on N+L-1 bits. With TBCC, no additional bits are needed but a typical decoder operates on at least 2N bits. For moderate and larger number of information bits, tailed convolutional codes and TBCC have the same performance. For smaller number of information bits, with additional decoding iterations, the performance of TBCC can approach the performance of tailed convolutional codes. - Source [R1-10321-123-3, vivo] states, compared with LTE TBCC, tailed convolutional code can reduce decoding complexity while increase the overhead. - Source [R1-10321-123-6, Huawei] states tailed convolutional codes with the same polynomial as LTE TBCC has a performance similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits. - Source [R1-10321-123-26, TCL] states, the decoding for Tailed convolutional code maybe simpler, but the tail bits need to be sent, and the actual effective code rate slightly decreases. - Source [NOKIA] states, TBCC is prioritized over tailed CC, provided that the decoding complexity is acceptable for device 2b/C. - Source [ZTE] states, tailed CC results in a code rate loss, making the code rate less than 1/n, where n is the number of polynomials, TBCC encoding does not require additional complexity or registers compared with tailed CC. And, TBCC encoding has larger code rate compared with Zero-tail CC. - Source [Samsung] states, tailed convolutional codes suffer from reduced spectral efficiency due to zero padding. For comparison between Coding scheme 1 and Coding scheme 3: - Source [R1-10321-123-3, vivo] states RM code suffers performance degradation and larger complexity at least for larger transport block size, without power reduction benefit - Source [R1-10321-123-5, CMCC] states, RM code is deprioritized due to the restricted operational range - Source [R1-10321-123-6, Huawei] states RM codes have a 4-5 times higher power consumption than convolutional codes due to the complicated fast Hadamard transform calculation for the FEC decoder, and has a 3-11 bit TBS restriction in the NR baseline design. - Source [R1-10321-123-26, TCL] states the combinations of code length and code rate for RM code are limited, which is less flexible than convolutional codes. And the evaluation for the performance of the RM code is needed under low signal-to-noise ratio conditions. - Source [R1-10321-123-24, Qualcomm] states RM codes performs worse than TBCC. RM with k=8 or 10, n=32 has lower decoding complexity, memory cost, hardware area cost and lower power consumption than Viterbi decoding for TBCC with 1/2 or 1/3 coding rate. - Source [R1-10321-123-25, NTT DOCOMO] states decoding complexity of RM code is expected to be lower than Viterbi decoding for TBCC. RM is not supported for payload size larger than 11. - Source [ZTE] states the RM codes used for PUCCH in 5G can not satisfy the requirement of A-IoT. - Source [Xiaomi] states, TBCC is corresponding to high decoding complexity at device side than RM. - Source [Samsung] states that RM codes provide limited coding gain for long data lengths and incur exponentially increasing decoding complexity as the input length increases. If FFC is supported for R2D: - Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], [R1-10321-123-26, TCL], [R1-10321-123-29, Sequans] [R1-10321-123-12, OPPO], [R1-10321-123-17, Panasonic], [R1-10321-123-24, Qualcomm], [NOKIA], [ZTE] [Samsung] and [Sequans] report LTE TBCC (Coding scheme 1) should be supported. Source [FUTUREWEI] report LTE TBCC (Coding scheme 1) can be supported when the number of information bits is small. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-15, Transsion] report tailed CC with same polynomial as LTE TBCC (Coding scheme 2) can be supported. Source [FUTUREWEI] report tailed CC with same polynomial as LTE TBCC (Coding scheme 2) can be supported when the number of information bits is large. - Source [R1-10321-123-24, Qualcomm] reports Reed-Muller coding (Coding scheme 3) can be supported. Source [R1-10321-123-9, CATT] reports Reed-Muller coding can be supported for L1 control, if L1 control is no larger than 11 bits, otherwise, TBCC is supported.

  - Proposals for xxx

To be added

- R2D Channel design

  - M value for R2D

Spreadtrum, Ericsson, CATT, Nokia, CMCC, Xiaomi, vivo, TCL, Sequans, IIT Kanpur, provide their views on waveform and M value for R2D.

- Backgroud 1

Regarding M value for R2D, following agreements are made in previous meetings,

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24

- Backgroud 2

There are text proposals reflecting company views already captured into TR 38.769 last meeting,

- See section 6A.1.x R2D waveform, modulation and bandwidth

Issue 3.1-1 on addition of M=1

For M =1, following views are provided by companies.

- CATT states that M = 1 should be considered for R2D in outdoor for coverage enhancement.

- TCL states that M = 1 can be considered as potential M values for R20 AIoT.

- Xiaomi states that, for M = 1, longer chip length can enable better coverage performance than M=2 and the performance gain is ~3 dB.

- Docomo states that, M=1 does not offer performance improvements relative to M=2, unless power boosting during the ON duration are allowed. Consequently, the introduction of M=1 is not regarded as a promising approach for enhancing coverage.

- Samsung states that, OOK with M=2 using together with repetition achieve the same data rates as Mchipsymb=1.

FL Note:

- View from TCL and CATT has already been captured in TR. Views from Xiaomi, Samsung and Docomo are added in the update TP.

[TP][U] FL Proposal 3.1-1

FL Proposal 3.1-1-v1

Update TR38.769, on addition of M=1, as follows

#### 6A.1.x.1 Mchipsymb=1 Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-4, Nokia], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-11, Xiaomi], [R1-10321-123-9, CATT], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-17, Panasonic], [R1-10321-123-19, Apple], [R1-10321-123-26, TCL], [R1-10321-123-27, IITK], and [R1-10321-123-29, Sequans] states that addition of Mchipsymb=1 is beneficial to improve R2D coverage. - Source [R1-10321-123-6, Huawei] shows that, Mchipsymb = 1 provides ~3 dB performance gain compared with Mchipsymb = 2 using correlation detection with TBS 20 bits and 96 bits, and ~2.5dB performance gain with TBS 400 bits, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to Clause 4.3.2. And Source [Xiaomi] states that longer chip length can enable better coverage performance than M=2 and the performance gain is ~3 dB. - Source [R1-10321-123-17, Panasonic] shows that, Mchipsymb = 1 shows performance gain, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to Clause 4.3.2. - Source [R1-10321-123-9, CATT] states that, for Mchipsymb=1, the larger chip duration could reduce the DS impact and improve the detection performance for Device 2b/C in outdoor deployment scenario. Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE], [R1-10321-123-12, OPPO], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO] and [Samsung] states that addition of Mchipsymb=1 is not necessary. - Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE], [R1-10321-123-12, OPPO], [R1-10321-123-25, NTT DOCOMO] and [Samsung] states that, OOK with M=2 using together with repetition achieve the same data rates as Mchipsymb=1. - Source [R1-10321-123-14, ZTE] further observed that OOK with M=2 using together with repetition can achieve better time-domain diversity gain and power boosting gain than OOK with Mchipsymb=1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, OFDM symbol with zero power is not counted. - Source [R1-10321-123-14, ZTE] states that, OOK4 with Mchipsymb =2 and Mchipsymb=4 have better detection performance than OOK4 with Mchipsymb =1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, OFDM symbol with zero power is not counted. - Source [R1-10321-123-24, Qualcomm] shows that, Mchipsymb=1 cannot provide better performance than Mchipsymb=2, if the CINR/CNR for OFDM symbol with non-zero power is calculated according to Clause 4.3.2, and OFDM symbol with zero power is not counted. Source [R1-10321-123-24, Qualcomm] also shows that Mchipsymb=1 provide less than 2dB performance gain than Mchipsymb=2, if the CINR/CNR, for OOK4 signal across all OFDM symbols, is calculated according to Clause 4.3.2. - Source [Docomo] states that, M=1 does not offer performance improvements relative to M=2, unless power boosting during the ON duration are allowed

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| DOCOMO | Y |  |
| Samsung | Editorial | We suggest also to update the main bullet: Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE], [R1-10321-123-12, OPPO], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], [Samsung] states that addition of Mchipsymb=1 is not necessary. |
| CATT | Y |  |
| FL |  | Further update based on comment from Samsung, based on FL Summary reply. |

Issue 3.1-2 On removal of M=24

For M = 24, following views are provided by companies.

- Xiaomi states that, for M = 24, there is an error floor at BLER of 10^-1.

- Nokia states that removal of M=24 is not necessary.

- TCL states that M = 24 can be considered as potential M values for R20 AIoT.

- Ericsson states that, for Device 2b and Device C, a 10% BLER cannot be achieved with M = 24.

- CATT states that M=24 can satisfy the coverage requirement of 500m at 10% BLER, and it should be maintained for R2D outdoor to reach the similar data rate as Rel-19 and could satisfy the coverage requirement at 10% BLER.

- vivo states that CP handling method 2 is not needed for M=24 values in R20 due to better clock accuracy.

FL Note:

- CATT, Xiaomi, vivo and TCL provide additional information compared with current TR. Other views has already been captured in TR.

[TP][U] FL Proposal 3.1-2

FL Proposal 3.1-2-v1

Update TR38.769, on removal of M=24, as follows

#### 6A.1.x.2 Mchipsymb=24 Sources [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-17, Panasonic], [R1-10321-123-11, Xiaomi] and [R1-10321-123-3, vivo] state that it is not necessary to keep Mchipsymb = 24 for R2D can be removed. - Sources [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson] and [R1-10321-123-17, Panasonic] state and provide evaluation results showing that Mchipsymb = 24 cannot work well for outdoor scenarios as its chip length is too short to resist large multipath delay and channel fading. - Source [R1-10321-123-6, Huawei] shows that Mchipsymb = 24 cannot work for outdoor scenarios with BLER close to 1 under different SNRs - Source [R1-10321-123-7, Ericsson] shows that 10% BLER cannot be achieved with Mchipsymb = 24 (even with repetitions) - Source [R1-10321-123-17, Panasonic] shows that, when Mchipsymb = 24 (and Mchipsymb>12), both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB or larger (even without SFO/CFO) - Source [R1-10321-123-3, vivo] states and provides evaluation results show that, R2D transmission with Mchipsymb = 24 cannot achieve 1% BLER even with 2 block-level repetitions, and under similar data rates, the performance of Mchipsymb = 24 (with 2 repetitions) is worse than that of M=12 (without repetition). Source [Xiaomi] states and provides evaluation results show that, there is an error floor at BLER of 10^-1 for M=24. - Source [vivo] states that CP handling method 2 is not needed for M=24 values in R20 due to better clock accuracy. Sources [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-12, OPPO], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-24, Qualcomm], [R1-10321-123-9, CATT], [TCL] and [R1-10321-123-4, Nokia] state that it is necessary to keep Mchipsymb = 24 for R2D. - Source [R1-10321-123-25, NTT DOCOMO] states that Mchipsymb = 24 can be supported for a device which is close to reader and beneficial for device power saving to reduce PRDCH reception duration with higher peak data rate - Source [R1-10321-123-1, FUTUREWEI] provides evaluation results show that Mchipsymb = 24 can still achieve 10% BLER with a reasonable SNR, and Source [CATT] provides evaluation results show that the coverage requirement of 500m at 10% BLER. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-12, OPPO], [CATT] and [R1-10321-123-13, Samsung] state that Mchipsymb = 24 is needed to achieve data rates similar as R19. - Source [R1-10321-123-4, Nokia] states that, Mchipsymb = 24 should not be removed, since it is still useful for indoor scenario, and the air interface for Device 2b/C studied for outdoor scenarios will be reused in indoor scenarios.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Nokia |  | A general comment on how the issue is framed: The issue to study is not whether it is necessary to keep M=24; the issue to study is whether it is necessary to remove M=24. See the wording in the original agreement: Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: ⚫ Addition of M=1 ⚫ Addition of M=32 ⚫ Removal of M=24 |
| CATT | Y |  |
| FL |  | “it is not necessary to keep Mchipsymb = 24 for R2D” is changed to “Mchipsymb = 24 for R2D can be removed” as suggested by Nokia in FL Summary. |

Issue 3.1-3 On addition of M=32

For M= 32, following views are provided by companies.

- Spreadtrum states that M=32 should be further considered.

- Ericsson states that, for Device 2b and Device C, a 10% BLER cannot be achieved with M = 32.

- Xiaomi state that, for M = 32, there is an error floor at BLER of 10^-1

- IITK states that BLER for M = 32 is close to 1 in outdoor scenarios across SNRs and failing to meet the 10% BLER target even with FEC and block-level repetitions, and M=32 should not be supported.

- Xiaomi states that higher peak data rate and shorter transmission duration can be achieved due to shorter chip length is applied.

- vivo states that M=24 and M=32 may not be distinguishable at AIoT device. According to RAN4 requirements, when the nominal chip length is Tc, the allowed chip length can be 0.7*Tc to 1.3*Tc, which makes AIoT device difficult to differentiate M=32 from M=24, since the nominal chip length of M=24 is only 1.33 times of chip length of M=32.

- CATT states that, and there is an error floor above 1% BLER for M=32, and M =32 would not considered for R2D in outdoor, since it cannot satisfy the coverage requirement, e.g. 500m.

FL Note:

- vivo, Xiaomi and IIK provide additional information compared with current TR. Other views has already been captured in TR.

[TP][U] FL Proposal 3.1-3

FL Proposal 3.1-3-v1

Update TR38.769, on addition of M=32, as follows

6A.1.x.3 Mchipsymb=32 [omit unchanged part] Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-12, OPPO], [R1-10321-123-9, CATT], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-17, Panasonic], [R1-10321-123-25, NTT DOCOMO] and [R1-10321-123-7, Ericsson] state that it is not necessary to add Mchipsymb for R2D. - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-9, CATT], [R1-10321-123-14, ZTE], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-17, Panasonic], and [R1-10321-123-12, OPPO] state concern on feasibility of Mchipsymb=32 due to too short chip duration. - Source [R1-10321-123-6, Huawei] provide evaluation results show that, Mchipsymb=32 cannot work for outdoor scenarios with BLER close to 1 under different SNRs, as its chip length is too short to resist large multi-path delay and channel fading, so addition of Mchipsymb=32 is not supported. - Source [R1-10321-123-9, CATT] provide evaluation results show that, for Device 2b, Mchipsymb=32 cannot satisfy the 500m coverage requirement at 10% BLER, and for Device C, though it can reach 10% BLER, it has an error floor above 1% BLER. - Source [R1-10321-123-3, vivo] provide evaluation results show that R2D transmission with Mchipsymb=32 cannot achieve 1% BLER even with 3 block-level repetitions, and under similar data rates, the performance of M=32 (with 3 repetitions) is worse than that of Mchipsymb=12 (without repetition). Source [vivo] further states that M=32 can not be distinguished from M=24 at AIoT device. According to RAN4 requirements, when the nominal chip length is Tc, the allowed chip length can be 0.7*Tc to 1.3*Tc, which makes AIoT device difficult to differentiate M=32 from M=24. - Source [R1-10321-123-17, Panasonic] provide evaluation results show that, when Mchipsymb =32, both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB and larger due to smaller chip duration. - Source [R1-10321-123-7, Ericsson] provide evaluation results show that, for Device 2b and Device C, a 10% BLER cannot be achieved with Mchipsymb=32. Source [Xiaomi] provides evaluation results show that, there is an error floor at BLER of 10^-1 for M = 32. Source [IITK] provides evaluation results show that that BLER for M = 32 is close to 1 in outdoor scenarios across SNRs and failing to meet the 10% BLER target even with FEC and block-level repetitions - Sources [R1-10321-123-12, OPPO] and [R1-10321-123-13, Samsung] state that, higher data rate with Mchipsymb=32 lacks motivation. - Source [R1-10321-123-25, NTT DOCOMO] states that, it would increase device complexity as devices need to blindly detect and differentiate Mchipsymb=24 and Mchipsymb=32 with certain FDR.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| CATT | Y |  |

Issue 3.1-4 Other views on M value set

Besides, following views are provided on M values for R20 R2D

- Nokia suggest that RAN1 to capture observations regarding addition of M=1 and M=32 for TR without a conclusion or recommendation for WI.

- vivo If both M=1 and M=24 are supported together with M value of {2, 6, 12}, the value set can be configured by broadcast information

  - For outdoor scenarios with large coverage requirements, M value set of {1, 2, 6, 12} can be configured;

  - while for indoor scenario (including D2T2) or outdoor scenario with moderate coverage requirements, M value set of {2, 6, 12, 24} can be configured.

- CMCC states that for R20 R2D, SSB modulation could be considered to reduce bandwidth occupancy and support larger M value for one RB. For R2D, SSB modulation shows optimization potential in achieving the 10% BLER target with M=8 using a minimal bandwidth of only 1 PRB.

FL notes

- For M value to be supported in R20, FL do not plan to provide further proposals in this SI, which can be decided in recommendation stage, RAN planery or WI phase.

- Regarding single sideband(SSB) for R20 R2D and potential update, FL think it is difficult to collect sufficient views in this SI, given there is no previous agreements ask the group to study this issue.

If companies have further views/suggestions on M value selection. Please provide you views in the table below.

| Company | Comments |
| --- | --- |
| LGE | We also have a similar view with Nokia. The M value selection can be done in WI. |
| ZTE, Sanechips | Agree to capture the views of each company and selected in RAN plenary or WI phase. |
| FL | Company views are already reflected in the three Text proposals for TR capturing, it seems that is all we can do in SI phase. |

  - R2D Repetition

Futurewei, Huawei, Spreadtrum, OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, NEC, Panasonic, Samsung, Apple, Docomo, Qualcomm, TCL, Sequans, IITK provided views on repetition for R2D.

- Background

In previous meeting, the following agreements are made on R2D repetitions.

Agreement Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types: - Block-level repetitions - Bit-level Type-2 repetitions - Chip-level repetitions (in case of M=1 with FEC if applied) Agreement For R2D Repetition, block-level repetition is considered for R20 AIoT. - bit-level and chip level repetition are not considered for further study. Agreement Regarding R2D Repetition, Capture following observations in TR 38.769 Regarding R2D repetition, it is reported by sources [Futurewei], [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC], [OPPO], [Transsion], [LGE], [Apple], [Qualcomm], [CEWiT], [Sequans], [Panasonic], [Quectel], [NTT DOCOMO] and [IIT Kanpur] state that R2D repetition is feasible and can be considered. Sources [vivo], [Huawei], [CMCC], [OPPO], [CATT], [Transsion], [Apple], and [IIT Kanpur] report that repetition schemes including block-level repetition are feasible.

Based on agreements in previous meeting,

- Block level repetition is the only R2D repetition scheme on table.

- Company views on feasibility of Block level repetition has been captured in TR, see section 6A.1.x.y Repetition.

Issue 3.2-1 Feasibility of R2D block level Repetition

Based on contributions in this meeting, following views are provided on feasibility of R2D block level repetitions.

- Ericsson states that, Block-level repetition requires a large volatile memory to store all received repetitions of a block.

- Samsung states that, repetition schemes including block-level repetition are feasible.

- OPPO states that, the device needs to store and combine the repetitions, these operations should be feasible for small TBS, but whether block-level repetition(s) can be applied to all TBS, e.g., thousands of bits, should be further studied

FL notes

- FL suggest update TR38.769 as follows.

[TP][U] FL Proposal 3.2-1

FL Proposal 3.2-1-v1

Update TR 38.679 on feasibility of R2D block level repetition.

6A.1.x.y Repetition Block-level repetition as defined in Clause 6.1.0.1 is considered for the study; other types of repetition are not considered. It is reported by sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-12, OPPO], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], [R1-10321-123-24, Qualcomm], [R1-10321-123-28, CEWiT], [R1-10321-123-29, Sequans], [R1-10321-123-17, Panasonic], [R1-10321-123-23, Quectel], [R1-10321-123-25, NTT DOCOMO], [Samsung], [ZTE] and [R1-10321-123-27, IITK] that R2D repetition is feasible and can be considered. Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-9, CATT], [R1-10321-123-15, Transsion], [R1-10321-123-19, Apple], [Samsung], [ZTE] and [R1-10321-123-27, IITK] report that repetition schemes including block-level repetition are feasible. Source [Ericsson] and [OPPO] state that, block-level repetition requires a large volatile memory to store received repetitions of a block, Source [OPPO] further states that block-level repetition should be feasible for small TBS. whether block-level repetition(s) can be applied to large TBS e.g., thousands of bits, should be further studied.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| ZTE, Sanechips |  | The views of ZTE is omitted. We also discussed repetition(block level repetition) in our tDoc, and the following proposal was provided in our tDoc R1-2600451. Update the TR as following. Regarding R2D Repetition, Capture following observations in TR 38.769 Regarding R2D repetition, it is reported by sources [Futurewei], [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC], [OPPO], [Transsion], [LGE], [Apple], [Qualcomm], [CEWiT], [Sequans], [Panasonic], [Quectel], [NTT DOCOMO], [ZTE], and [IIT Kanpur] state that R2D repetition is feasible and can be considered. Sources [vivo], [Huawei], [CMCC], [OPPO], [CATT], [Transsion], [Apple], [ZTE], and [IIT Kanpur] report that repetition schemes including block-level repetition are feasible.. Therefore, please add [ZTE] in TR 38.679 on feasibility of R2D block level repetition. |
| CATT | Y |  |
| FL |  | Updated to add ZTE view based on FL Summary reply. |

Issue 3.2-2 Necessity of R2D block level Repetition

Based on contributions in this meeting, company views on necessity of R2D block level repetition are summarized as follows

| Position | Companies |
| --- | --- |
| Necessary | Futurewei, Huawei, OPPO, CMCC, ZTE, vivo, NEC, Panasonic, Apple, InterDigital, Qualcomm, Quectel, TCL, Sequans, IIT Kanpur |
| Not Necessary | Spreadtrum, Ericsson, Xiaomi |
| Netrual | Nokia(depending coverage comparison between R2D and D2R), Samsung |

Detailed company views can be found in FL proposal 3.2-2.

FL notes

- FL provide new TP to add company views in TR38.769.

[TP][N] FL Proposal 3.2-2

FL Proposal 3.2-2-v1

Regarding Necessity of R2D Block level repetition, Capture following in TR38.769.

[Positive views]

Sources [Futurewei], [Huawei], [CMCC], [ZTE], [OPPO], [vivo], [NEC], [Panasonic], [Apple], [Qualcomm], [InterDigital], [TCL], [Quectel], [Sequans], and [IITK] report that it is necessary or beneficial to support repetition to achieve required target coverage for outdoor scenario.

- Source [Huawei] provides evaluation results shows that, it is necessary to support R2D enhancements including block repetitions to ensure R2D transmissions with high reliability, facilitate early termination, and usability for outdoor scenarios considering co-site deployment.

  - With M = 1, 1/3 TBCC, sub-block interleaver and LTE bit collection, it can achieve 334.7m - 431.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which shows significant gain for larger TBS transmissions, but still requires further enhancement for outdoor co-site deployment.

  - With M = 1, 1/3 TBCC and LTE bit collection & sub-block interleaver, block repetition number 2, it can achieve 439.0m - 589.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which can approximately satisfy the outdoor co-site deployment requirement.

- Source [vivo] provides evaluation results shows that R2D Block level repetition is necessary to achieve desirable coverage

  - For UMa scenario, to achieve 500-meter coverage, repetition is necessary for the following cases:

    - The coverage is 513.36m with 3 repetitions, while coverage is 373.46m without repetition, for {96bit payload size, 43dBm Tx power}

    - The coverage is 616.24m with 3 repetitions, while coverage is 276.53m without repetition, for {400bit payload size, 43dBm Tx power}

  - For UMa scenario, to achieve 334-meter coverage, repetition is necessary for the following cases:

    - The coverage is 382.37m with 3 repetitions, while coverage is 278.17m without repetition, for {96bit payload size, 38dBm Tx power}

    - The coverage is 458.99m with 3 repetitions, while coverage is 205.97m without repetition, for {400bit payload size, 38dBm Tx power}

    - The coverage is 341.87m with 3 repetitions, while coverage is 153.41m without repetition, for {400bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 1154-meter coverage, repetition is necessary for the following cases:

    - The coverage is 1205.07m with 6 repetitions, while coverage is 472.75m without repetition, for {400bit payload size, 43dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, repetition is necessary for the following cases:

    - The coverage is 934.3m with 3 repetitions, while coverage is 472.75m without repetition, for {400bit payload size, 43dBm Tx power}

    - The coverage is 595.03m with 3 repetitions, while coverage is 475.57m without repetition, for {96bit payload size, 38dBm Tx power}

    - The coverage is 693.53m with 3 repetitions, while coverage is 350.92m without repetition, for {400bit payload size, 38dBm Tx power}

    - The coverage is 514.81m with 3 repetitions, while coverage is 260.49m without repetition, for {400bit payload size, 33dBm Tx power}

- Source [ZTE] provide evaluation results shows that 2 times repetition provides 2-3 dB performance gain @BLER = 0.1 compared with baseline. 3 times repetition provides 2-4 dB performance gain @BLER = 0.1 compared with baseline.

  - The candidate values of the repetition number can be [1, 2, 4, 8] if FEC is supported for PRDCH, and [1, 2, 4, 6, 8, 12, 16, 32] is recommended if FEC is not supported for PRDCH.

- Source [CMCC] provide evaluation results shows that, with 1/3 CC but without repetition, coverage of Device 2b degrades significantly when a practical 20dB penetration loss is considered, resulting in coverage distances of 712m (UMa) and 1231m (RMa), which do not provide a sufficient reliability margin.

- Source [Futuerwei] states that, for the R2D link, block repetition will complement convolutional coding.

- Source [OPPO] states that Block-level repetition(s) offers a straightforward mechanism to improve coverage without introducing substantial processing overhead, and it should be considered for R2D transmission at least for small TBS.

- Source [Panasonic] states Block level repetition would be applied for data of any size although more benefit for small TBS.

- Source [Apple] states that Block-level repetition provides additional coverage enhancement beyond FEC alone through time diversity and energy accumulation at the receiver.

- Source [Qualcomm] states that, it is necessary to use block-level repetitions to improve the interference robustness.

- Source [TCL] states that, R2D Block level repetition can be considered together with FEC encoding to further increase the performance.

- Source [Sequans] states that block level repetitions offers gains via non-coherent combining at the reader and provides time/frequency diversitywhile keeping device complexity and occupancy low, and low (e.g., <=4) number of repetitions can be supported.

- Source [IITK] States that Block-level repetition provides the highest diversity gain for R2D by enabling combining of multiple repetitions, and remains feasible for Device 2b/C despite increased memory and latency requirements.

[Neutral views]

Source [Nokia] states that, it may be beneficial for extending R2D coverage, but the necessity depends also on how R2D coverage compares with D2R coverage.

[Negative views]

Source [Spreadtrum],[Ericsson] and [Xiaomi] state that R2D Block level repetition is not necessary, and the distance target can be fulfilled without repetition.

Source [Samsung] states that, the necessity of R2D repetition depends on whether evaluation results can satisfy the coverage requirements.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Minor change considering individual company’s observation may not be same. Also, it seems not clear what is the required target coverage for outdoor scenario. [Positive views] Sources [Futurewei], [Huawei], [CMCC], [ZTE], [OPPO], [vivo], [NEC], [Panasonic], [Apple], [Qualcomm], [InterDigital], [TCL], [Quectel], [Sequans], and [IITK] report that it is necessary or beneficial to support repetition to achieve required target coverage for outdoor scenario. |
| Panasonic | Y |  |
| Samsung |  | We also proposed neutral view that necessity of R2D repetition depends on target performance requirement. Therefore, we suggest the following update: [Neutral views] Source [Nokia] states that, it may be beneficial for extending R2D coverage, but the necessity depends also on how R2D coverage compares with D2R coverage. Source [Samsung] states that, the necessity of R2D repetition depends on whether evaluation results can satisfy the coverage requirements. |
| Huawei, HiSilicon | Y |  |
| CATT | Y | Block repetition provides repetition gain and time diversity gain, which is necessary for coverage performance of outdoor scenarios. |
| FL |  | Update based on reply from Qualcomm and Samsung. |

  - R2D Channel coding

In previous meeting, the following agreements are made on R2D FEC.

Agreement Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver - Combination of alternatives can be studied Agreement For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects: - Coding schemes - LTE TBCC - Tailed convolutional code with the same polynomial as LTE TBCC - RM code - FEC code rates: 1/2, 1/3, 1/4 Agreement For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

Issue 3.3-1 Necessity of FEC

FUTUREWEI, Huawei, CATT, CMCC, Nokia, ZTE, vivo, NEC, Apple, InterDigital, Qualcomm, TCL, Sequans, Panasonic, NTT DOCOMO, Quectel, Spreadtrum, Xiaomi, Ericsson, Samsung and IITK provided views on necessity of R2D FEC in the contributions for this meeting.

Company views on necessity of R2D FEC are summarized as follows

| Position | Companies |
| --- | --- |
| Necessary | FUTUREWEI, Huawei, CATT, CMCC, Nokia, ZTE, vivo, NEC, Apple, InterDigital, Qualcomm, TCL, Sequans, Panasonic, NTT DOCOMO, Quectel, OPPO (if repetition is not sufficient) |
| Not Necessary | Spreadtrum, Ericsson, Samsung, IIT Kanpur |
| Netrual | Nokia(depending coverage comparison between R2D and D2R) |

Detailed company views can be found in FL proposal 3.3-1.

FL notes

- We have no TP and agreements for Necessity of R2D FEC last meeting.

- FL suggest the following proposal for TR capturing.

[TP][N] FL Proposal 3.3-1

FL Proposal 3.3-1-v1

Regarding Necessity of R2D FEC, Capture following in TR38.769.

[Positive views]

Sources [FUTUREWEI], [Huawei], [CATT], [CMCC], [ZTE], [vivo], [NEC], [Apple], [InterDigital], [Qualcomm], [TCL], [Sequans], [Panasonic], [NTT DOCOMO] and [Quectel] report that it is feasible and necessary to support R2D FEC to achieve required target coverage for outdoor scenario.

- Source [Huawei] provides evaluation results shows that, it is necessary to support R2D enhancements including FEC to ensure R2D transmissions with high reliability, facilitate early termination, and usability for outdoor scenarios considering co-site deployment.

  - With M = 1 and no FEC, interleaving or repetitions, it can only achieve 124.4m - 267.6m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which is not sufficient for outdoor scenarios with co-site deployment.

  - Introducing FEC, specifically TBCC, provide ~4 dB performance gain for Rel-20 R2D transmissions, which is necessary for outdoor scenarios and beneficial for coverage enhancement.

- Source [vivo] provides evaluation results shows that R2D FEC is necessary to achieve desirable coverage, compared with cases without FEC and without R2D repetition.

  - For UMa scenario, to achieve 500-meter coverage, FEC is necessary for the following cases:

    - The coverage is 591.34m with 1/3 FEC, while coverage is 373.46m without FEC, for {96bit payload size, 43dBm Tx power}.

    - The coverage is 705.67m with 1/3 FEC, while coverage is 276.53m without FEC, for {400bit payload size, 43dBm Tx power}.

    - The coverage is 525.6m with 1/3 FEC, while coverage is 205.97m without FEC, for {400bit payload size, 38dBm Tx power}.

  - For UMa scenario, to achieve 334-meter coverage, FEC is necessary for the following cases:

    - The coverage is 440.45m with 1/3 FEC, while coverage is 278.17m without FEC, for {96bit payload size, 38dBm Tx power}

    - The coverage is 391.49m with 1/3 FEC, while coverage is 153.41m without FEC, for {400bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, FEC is necessary for the following cases:

    - The coverage is 984.03m with 1/3 FEC, while coverage is 472.75m without FEC, for {400bit payload size, 43dBm Tx power}.

    - The coverage is 652.24m with 1/3 FEC, while coverage is 475.57m without FEC, for {96bit payload size, 38dBm Tx power}.

    - The coverage is 730.44m with 1/3 FEC, while coverage is 350.92m without FEC, for {400bit payload size, 38dBm Tx power}.

    - The coverage is 542.2m with 1/3 FEC, while coverage is 260.49m without FEC, for {400bit payload size, 33dBm Tx power}.

- Source [vivo] further provides evaluation results shows that R2D FEC is necessary to achieve desirable coverage, compared with cases with R2D repetition and without FEC, under the same data rate.

  - For UMa scenario, to achieve 500-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 560.8m with 2 repetitions and 1/3 FEC, while coverage is 489.73m with 6 repetitions only for {96bit payload size, 38dBm Tx power}

    - The coverage is 538.14m with 2 repetitions and 1/3FEC while coverage is 445.67m with 6 repetitions only for {400bit payload size, 33dBm Tx power}

  - For UMa scenario, to achieve 334-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 347.97m with 2 repetitions and 1/2 FEC while coverage is 307.47m with 4 repetitions only for {96bit payload size, 33dBm Tx power}

  - For RMa scenario, to achieve 1154-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 1219.52m with 2 repetitions and 1/2FEC while coverage is 1063.3m with 4 repetitions only for {400bit payload size, 43dBm Tx power}

  - For RMa scenario, to achieve 500-meter coverage, both repetition and FEC are necessary for the following cases:

    - The coverage is 561.94m with 2 repetitions and 1/2FEC while coverage is 487.05m with 4 repetitions only for {96bit payload size, 33dBm Tx power}

- Source [CATT] provides evaluation results shows that, without LTE interleaving, TBCC codes with coding rate of 1/2 have more than 5dB coding gain over the Manchester code in TDL-C channel with 300ns delay spread, for OOK-4 with M=2, 6, 12 at BLER=10^-1, under the same data rate.

- Source [Futurewei] states that, R2D FEC coding gains from convolutional codes far exceed the gains from repetition,and it is necessary to support any channel coding on the R2D link as errors due to long distance propagation, fading, and low SNR.

- Source [CMCC] states that, to ensure the coverage targets are met under practical conditions and to support larger payloads, the adoption of FEC and repetition becomes beneficial and necessary.

- Source [Docomo] states that, FEC offers additional coding gain and generally outperforms repetition at equivalent data rates, and FEC should be prioritized compared to repetition.

- Source [Apple] states that, FEC based on LTE TBCC is essential for achieving outdoor coverage targets for device 2b/C given the transmission power constraints.

- Source [Qualcomm] states that, it is necessary to use FEC, interleaver to improve the interference robustness.

- Source [OPPO] states that FEC should only be considered if repetition proves insufficient.

[Neutral views]

Source [Nokia] states that, it may be beneficial for extending R2D coverage, but the necessity depends also on how R2D coverage compares with D2R coverage.

[Negative views]

Source [Spreadtrum], [Ericsson], [Xiaomi] and [IITK] state that R2D FEC is not necessary, and the distance target can be fulfilled without FEC.

Source [Samsung] states that, R2D FEC requires decoding at the device side, which may significantly increase device complexity and energy consumption, while it provides limited benefit considering R2D is not bottleneck.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Minor change considering individual company’s observation may not be same. Also, it seems not clear what is the required target coverage for outdoor scenario. [Positive views] Sources [FUTUREWEI], [Huawei], [CATT], [CMCC], [ZTE], [vivo], [NEC], [Apple], [InterDigital], [Qualcomm], [TCL], [Sequans], [Panasonic], [NTT DOCOMO] and [Quectel] report that it is feasible and necessary to support R2D FEC to achieve required target coverage for outdoor scenario. |
| Panasonic | Y |  |
| Xiaomi |  | Thanks for FL’s great effort on the summary, and please add Xiaomi’s negative view as below. [Negative views] Source [Spreadtrum],[Ericsson], [Xiaomi] and [IITK] state that R2D FEC is not necessary, and the distance target can be fulfilled without FEC. |
| DOCOMO | Y |  |
| Huawei, HiSilicon | Y |  |
| CATT | Y | FEC is essential for combating fading in outdoor scenarios; otherwise, robust transmission cannot be achieved. |
| FL |  | Update based on reply from Qualcomm and Xiaomi |

Issue 3.3-2 FEC schemes

Regarding FEC scheme, TR 38.769 has captured company views provided in last meeting, see section “6A.1.x.y.1 Coding schemes”. Companies are continuing discussions on the comparison of different FEC schemes. Companies view are summarized as below:

- Futurewei support TBCC when the number of information bits is small and tailed convolutional codes when the number of information bits is large.

- Nokia does not support RM code, and propose to prioritize TBCC over tailed CC, provided that the decoding complexity is acceptable for device 2b/C.

- ZTE does not support the RM codes because 5G RM code cannot satisfy the requirement of A-IoT. Regarding TBCC or tailed CC, TBCC can be reused for simplicity.

- Samsung supports TBCC, if FEC is to be supported for R2D, because TBCC is well-established and validated scheme, while tailed CC suffers from reduced spectral efficiency, and RM incur exponentially increasing decoding complexity with the increased input length.

- Sequans supports TBCC as baseline scheme, to minimize standard effort and allow smooth evaluation from Rel-19 AIOT and LTE IoT.

FL notes

- FL suggest update TR38.769 as follows.

[TP][U] FL Proposal 3.3-2

FL Proposal 3.3-2-v1

Update TR 38.679 on R2D coding scheme.

6A.1.x.y.1 Coding schemes FEC for R2D is studied, with code rates of 1/2, 1/3 and 1/4 and the following candidate coding schemes: Coding scheme 1: LTE tail biting convolutional code Coding scheme 2: Tailed convolutional codes with the same polynomials as LTE tail biting convolutional codes Coding scheme 3: Reed-Muller codes For comparison between Coding scheme 1 and Coding scheme 2: - Source [R1-10321-123-1, FUTUREWEI] states, for the same number of information bits N, a tailed convolutional code encodes L-1 additional bits (tail bits) to ensure the final state returns to 0. With tail bits, a typical decoder can operate on N+L-1 bits. With TBCC, no additional bits are needed but a typical decoder operates on at least 2N bits. For moderate and larger number of information bits, tailed convolutional codes and TBCC have the same performance. For smaller number of information bits, with additional decoding iterations, the performance of TBCC can approach the performance of tailed convolutional codes. - Source [R1-10321-123-3, vivo] states, compared with LTE TBCC, tailed convolutional code can reduce decoding complexity while increase the overhead. - Source [R1-10321-123-6, Huawei] states tailed convolutional codes with the same polynomial as LTE TBCC has a performance similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits. - Source [R1-10321-123-26, TCL] states, the decoding for Tailed convolutional code maybe simpler, but the tail bits need to be sent, and the actual effective code rate slightly decreases. - Source [NOKIA] states, TBCC is prioritized over tailed CC, provided that the decoding complexity is acceptable for device 2b/C. - Source [ZTE] states, tailed CC results in a code rate loss, making the code rate less than 1/n, where n is the number of polynomials, TBCC encoding does not require additional complexity or registers compared with tailed CC. And, TBCC encoding has larger code rate compared with Zero-tail CC. - Source [Samsung] states, tailed convolutional codes suffer from reduced spectral efficiency due to zero padding. For comparison between Coding scheme 1 and Coding scheme 3: - Source [R1-10321-123-3, vivo] states RM code suffers performance degradation and larger complexity at least for larger transport block size, without power reduction benefit - Source [R1-10321-123-5, CMCC] states, RM code is deprioritized due to the restricted operational range - Source [R1-10321-123-6, Huawei] states RM codes have a 4-5 times higher power consumption than convolutional codes due to the complicated fast Hadamard transform calculation for the FEC decoder, and has a 3-11 bit TBS restriction in the NR baseline design. - Source [R1-10321-123-26, TCL] states the combinations of code length and code rate for RM code are limited, which is less flexible than convolutional codes. And the evaluation for the performance of the RM code is needed under low signal-to-noise ratio conditions. - Source [R1-10321-123-24, Qualcomm] states RM codes performs worse than TBCC. RM with k=8 or 10, n=32 has lower decoding complexity, memory cost, hardware area cost and lower power consumption than Viterbi decoding for TBCC with 1/2 or 1/3 coding rate. - Source [R1-10321-123-25, NTT DOCOMO] states decoding complexity of RM code is expected to be lower than Viterbi decoding for TBCC. RM is not supported for payload size larger than 11. - Source [ZTE] states the RM codes used for PUCCH in 5G can not satisfy the requirement of A-IoT. - Source [Xiaomi] states, TBCC is corresponding to high decoding complexity at device side than RM. - Source [Samsung] states that RM codes provide limited coding gain for long data lengths and incur exponentially increasing decoding complexity as the input length increases. If FFC is supported for R2D: - Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], [R1-10321-123-26, TCL], [R1-10321-123-29, Sequans] [R1-10321-123-12, OPPO], [R1-10321-123-17, Panasonic], [R1-10321-123-24, Qualcomm], [NOKIA], [ZTE] [Samsung] and [Sequans] report LTE TBCC (Coding scheme 1) should be supported. Source [FUTUREWEI] report LTE TBCC (Coding scheme 1) can be supported when the number of information bits is small. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-15, Transsion] report tailed CC with same polynomial as LTE TBCC (Coding scheme 2) can be supported. Source [FUTUREWEI] report tailed CC with same polynomial as LTE TBCC (Coding scheme 2) can be supported when the number of information bits is large. - Source [R1-10321-123-24, Qualcomm] reports Reed-Muller coding (Coding scheme 3) can be supported. Source [R1-10321-123-9, CATT] reports Reed-Muller coding can be supported for L1 control, if L1 control is no larger than 11 bits, otherwise, TBCC is supported.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y |  |
| Samsung |  | We proposed the performance comparison between Coding scheme 1 and scheme 3 and suggest the following update: For comparison between Coding scheme 1 and Coding scheme 3: … - Source [ZTE] states the RM codes used for PUCCH in 5G can not satisfy the requirement of A-IoT. - Source [xiaomi] states, TBCC is corresponding to high decoding complexity at device side than RM. - Source [Samsung] states, tailed convolutional codes suffer from reduced spectral efficiency due to zero padding, while RM codes provide limited coding gain for long data lengths and incur exponentially increasing decoding complexity as the input length increases. |
| CATT | Y | TBCC should be supported for R2D data. For R2D control, the FEC scheme should be determined after the size of R2D control is finalized. |
| FL |  | Update based on reply from Samsung, and split the observation to two paragraghs for tailed convolution codes and RM codes, respectively. |

Issue#3.3-3 Interleaver and bit collection schemes

In previous meetings, companies discussed 3 alternatives for interleaving:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

Companies are continuing discussions on the feasibility and necessity of interleaving, as well as a comparison of the three alternatives. Companies views are summarized as below:

Feasibility of interleaving

- vivo states that, LTE bit-collection w/ or w/o sub-block interleaving is feasible, and it can be implemented with low power consumption (~10 uW), low latency (a few micro seconds), and limited complexity (area of hundreds μm2). Considering LTE-bit collection is also memory address operation as that for interleaver, LTE bit collection is also feasible

Necessity of interleaving

- Huawei states that, it is necessary to support the sub-block interleaver and bit collection scheme to overcome the significant performance loss for large TBS transmissions due to the lack of time diversity gain

Regarding 3 alternatives for interleaving,

- CATT supports Alt 1, based on observations that, Alt 1 significantly outperforms Alt 2 and Alt 3, especially for smaller M values and larger information bit lengths, and Alt 3 exhibits the worst interleaving performance due to limited time-domain diversity capability.

- Apple supports Alt 2, because LTE bit collection scheme without sub-block interleaver provides adequate performance while minimizing memory requirements for power-constrained devices.

- NEC supports further study on Alt 2 and Alt3, and Alt 3 can be adopted if memory size limitation exists in device(s).

- Sequans thinks LTE bit collection scheme Alt 2 is a good starting place as it can provide some time diversity gain, and suggest to further evaluate scheme to reduce memory Alt 3 as baseline in WI.

FL notes

- Some of company views above has already captured in TR. Only detla information is added in the TP below.

- FL suggest update TR38.769 as follows.

[TP][U] FL Proposal 3.3-3

FL Proposal 3.3-3-v1

Update TR 38.769 for interleaving and LTE bit collection for R2D

Interleaving for PRDCH with FEC and the use of the LTE bit collection scheme with the following candidate methods are studied: Alt 1: Both the LTE sub-block interleaver and the LTE bit collection scheme are used. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-6, Huawei], [R1-10321-123-12, OPPO], [R1-10321-123-9, CATT], [R1-10321-123-16, LGE ] and [R1-10321-123-17, Panasonic] report LTE sub-block interleaver and LTE bit collection scheme can be supported, if FEC is supported. - Source [R1-10321-123-1, FUTUREWEI] states that, reusing both LTE sub-block interleaver and LTE bit collection scheme enables a device to eliminate the repetition block and associated memory as the bit collection scheme can use the memory for the sub-block interleaver for repeated bits. Another benefit is the various coding rates are achievable without modifying the encoder specification. - Source [R1-10321-123-6, Huawei] states that, reusing the LTE sub-block interleaver and LTE bit collection scheme should be supported due to its performance gain especially for larger TBS transmissions, providing 1dB and 2.8 dB performance gain compared with Alt 2 for BLER 10% and BLER 1% respectively, for a TBS of 400 bits. - Source [R1-10321-123-16, LGE ] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort - Source [R1-10321-123-17, Panasonic] states that, LTE sub-block interleaver could be beneficial for R2D to reduce the burst error caused by fading, especially for the longer message size with smaller values of M - Source [R1-10321-123-24, Qualcomm] states that, Alt 1 and Alt 2 can achieve similar BLER performance, and both require buffering full-length coded bits. - Source [CATT] states that, R2D interleaving performance is correlated with both M (for OOK-4) and K (information bit length) values; the smaller the M and the larger the K, the better the interleaving performance. Alt 1 significantly outperforms Alt 2 and Alt 3, especially for smaller M values and larger information bit lengths. - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei] and [R1-10321-123-25, NTT DOCOMO] state it is feasible to support LTE subblock interleaving and LTE bit collection. - Source [R1-10321-123-3, vivo] report with references to show that the power consumption for LTE de-interleaving for turbo decoding is tens of uW, even with multiple parallel (de)interleaving operation, and each operation with max number of bits of 6144. It can be expected that the power consumption for TBCC with LTE bit collection and with or without LTE sub-block interleaving can be even simpler. [vivo] also report that, LTE bit-collection w/ or w/o sub-block interleaving can be implemented with low power consumption (~10 uW), low latency (a few micro seconds), and limited complexity (area of hundreds μm2). Hence, both LTE TBCC with bit collection with or without LTE sub-block interleaving are feasible. - Source [R1-10321-123-6, Huawei] and [R1-10321-123-25, NTT DOCOMO] report that the additional power consumption can be supported by active devices due to higher peak power consumption, allowing them to support larger memory. Source [R1-10321-123-6, Huawei] further report that larger memory would enable these devices to support the use of a LTE de-interleaver, and can follow the same hardware as used for the LTE sub-block interleaver with LTE bit collection scheme in D2R. Alt 2: Only the LTE bit collection scheme is used. - Sources [R1-10321-123-3, vivo], [R1-10321-123-8, NEC], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple] and [R1-10321-123-24, Qualcomm] report LTE bit collection scheme without LTE sub-block interleaver can be supported, if FEC is supported. - Source [R1-10321-123-3, vivo] states that, Alt 1 and Alt 2 presents similar performance; thus Alt 2 is sufficient. Source [vivo] report that, LTE bit-collection w/o sub-block interleaving can be implemented with low power consumption (~10 uW), low latency (a few micro seconds), and limited complexity (area of hundreds μm2). Hence, LTE TBCC without LTE sub-block interleaving is feasible. - Source [R1-10321-123-14, ZTE] states that, since bit collection does not require interleaving, the complexity of Alt 2 is lower than interleaving. FEC with bit collection and FEC with sub-block interleaver has similar performance compared with FEC with bit collection. - Source [R1-10321-123-16, LGE ] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort - Source [R1-10321-123-19, Apple] states that, Alt 2 provides a good balance between performance and complexity for device 2b/C, reusing existing 3GPP designs while minimizing memory requirements. - Source [R1-10321-123-24, Qualcomm] states that, for small number of coded bits, Alt2 can be used to improve interleaving gain Alt 3: Only an updated bit collection scheme based on the LTE bit collection scheme is used. - Source [R1-10321-123-24, Qualcomm] and [R1-10321-123-8, NEC] report that, LTE bit collection scheme with segment can be supported, if FEC is supported. - Source [R1-10321-123-8, NEC] states that, Alt 3 can be adopted if memory size limitation exists in devices - Source [R1-10321-123-24, Qualcomm] states that, Alt3 with two segmentations requires half of the buffering size versus Alt2 at price of 2.5dB performance loss, but still outperforms the no-FEC approach using three block-level repetitions. For large number of coded bits, Alt3 can be applied to reduce buffering requirements - Source [R1-10321-123-3, vivo], and [R1-10321-123-6, Huawei] and [CATT] state that, Alt 3 degrades performance due to lower time diversity gain compared with Alt 1/2, and [R1-10321-123-3, vivo] observes that, Alt 3 with two segments for 1000 bits TBS also suffers loss compared with block-level repetition. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-6, Huawei] state that, since memory/power consumption by Alt 1 and Alt 2 is feasible for AIoT device, it is unnecessary to further reduce memory/power consumption by Alt 3. - Source [R1-10321-123-16, LGE ] states that, Alt 3 is potentially advantageous in terms of memory reduction for A-IoT devices, but is considered less favourable due to the extensive discussion required to evaluate the performance of various bit collection schemes Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO] and [R1-10321-123-14, ZTE] state that it is beneficial to support interleaving to improve coverage by obtaining time diversity gain. Source [Huawei] states that it is necessary to support the sub-block interleaver and bit collection scheme to overcome the significant performance loss for large TBS transmissions due to the lack of time diversity gain. With M = 1 and only 1/3 TBCC, it can achieve 160.3m - 319.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss. While with M = 1, 1/3 TBCC, sub-block interleaver and LTE bit collection, it can achieve 334.7m -431.3m for the target BLER 1% with the case of TX power 33dBm and 20 dB penetration loss, which shows significant gain for larger TBS transmissions.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| CATT | Y | We support Alt 1. |

Issue#3.3-4 Order of channel coding (with interleaver if supported) and block-level repetition

Companies are continuing discussions on the order of channel coding and block-level repetition, if block-level repetition and channel coding with interleaving is supported. Companies view are summarized as below

- [vivo][CATT][NEC][IIT][Panasonic][TCL] support block level repetition after channel coding, with following reasons:

  - Block-level repetition before FEC scheme requires larger buffer and complexity [CATT][vivo][NEC]

  - Block-level repetition after channel coding is different from higher layer repetition, e.g., larger overhead for preamble and L1 control for higher level repetition [vivo]

  - Block-level repetition after channel coding enables both the time diversity and the soft-combining gain [IIT][Panasonic][TCL]

- [OPPO] propose to support block-level repetition before channel coding, with following reasons:

- If the block-level repetition was performed after channel coding it is fundamentally the same as higher layer repetition

FL notes

- Based on vivo’s explanation on difference between block level repetition and higher layer repetition, OPPO’s argument to not support block-level repetition after channel coding does not hold.

- Considering smaller buffer size of block-level repetition after channel coding, and majority support for block-level repetition after channel coding as captured in TR in last meeting (8 companies, [vivo], [Huawei], [NEC], [CATT], [ZTE], [FUTUREWEI], [Apple] and [Qualcomm] as copied below for easy check) and newly 3 supported companies [Panasonic] [IIT]and [TCL].

- FL feel the figure is not deem needed in the TR, to reduce work load, FL suggest to delete the figure caption in the TR.

- Considering 11 companies support channel coding before block-level repetition, while 2 companies perfer the other way. FL suggests following proposal.

[TP][U] FL Proposal 3.3-4

FL Proposal 3.3-4-v1

Update TR 38.769 for the order of channel coding and block level repetition

6A.1.x.y PRDCH overall [Omit Unchanged part] Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-14, ZTE], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-19, Apple] and, [R1-10321-123-24, Qualcomm], [Panasonic] , [IIT] and [Interdigital] report that channel coding before block-level repetition should be supported, as shown in Figure 6A.1.x.y-1, with following justifications: - Sources [R1-10321-123-3, vivo], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT] and [R1-10321-123-24, Qualcomm] state that less memory is required by the channel coding before block-level repetition, compared with channel coding after block-level repetition. - Source [R1-10321-123-6, Huawei] states that the order of the channel coding first, followed by block-level repetition should be supported because of the diversity gain achieved due to the increased distance of a given parity bit in the output stream when compared to the reverse order. - Source [R1-10321-123-9, CATT] states that block-level repetition before FEC scheme requires Rblock times more interleaver memory as well as higher implementation complexity and processing delay for both devices and reader. - Source [Panasonic] and [IIT] state, block-level repetition after channel coding enables time diversity and the soft-combining gain. Figure 6A.1.x.y-1: PRDCH generation – block order 1 Sources [R1-10321-123-12, OPPO] and [R1-10321-123-16, LGE] state that channel coding after block-level repetition should be supported, as shown in Figure 6A-1.x.y-2. Figure 6A.1.x.y-2: PRDCH generation – block order 2

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic | Y |  |
| Futurewei |  | Editorial question: are the figures going to be removed? If yes, then some changes are needed report that channel coding before block-level repetition should be supported, as shown in Figure 6A.1.x.y-1, with following justifications: Sources [R1-10321-123-12, OPPO] and [R1-10321-123-16, LGE] state that channel coding after block-level repetition should be supported, as shown in Figure 6A-1.x.y-2. |
| CATT | Y |  |
| FL |  | Delect “as shown in Figure xxx”, as suggested by Futurewei. |

[H] FL Proposal 3.3-5

FL Proposal 3.3-5-v1:

For R2D, channel coding (with interleaving, if supported) is before block-level repetition (if supported).

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm | Y |  |
| [OPPO] | N | Firstly, the proposal should focus on R2D data payload only for now, it has not been agreed yet to apply repetition for L1 R2D control. We agree with the intention of the proposal, i.e., to improve the reliability of R2D data but at the meantime not to increase FEC decoding complexity. But according to TR38.769 block-level repetition is performed after CRC attachment (as below), thus the proposal is contradictory with the definition. In the sense the proposal actually introduces a new repetition scheme. Note that repetition after FEC may result in several variants, including repetition after FEC but before bit collection, repetition after bit collection, repetition after line coding, and repetition after modulation. We are also wondering whether all these variants are to be studied? From out perspective, it is clearer to say the repetition is applied after modulation as below. For R2D, the data payload after modulation can be can be repeated within a PRDCH channel coding (with interleaving, if supported) is before block-level repetition (if supported). Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times |
| Spreadtrum | Y |  |
| Panasonic | Y |  |
| LGE | Y | Okay with the proposal. |
| DOCOMO | Y |  |
| Huawei, HiSilicon | Y | We support this proposal. Having the channel coding before the block-level repetition enhances the diversity gain achieved due to the increased distance of a given parity bit in the output stream when compared to the reverse order. In response to OPPO’s comment, we do not see any confusion based on the definition of block level repetition, since it does not state that it is applied immediately after CRC attachment. In our view, the reader would attach the CRC, perform FEC, followed by interleaving and bit collection, after which block level repetition is applied. |
| ZTE, Sanechips | Y |  |
| CATT | Y | When interleaving schemes are supported for R2D with FEC, block-level repetition should be performed after FEC and interleaving. |
| NEC | Y |  |
| FL |  | For issues raised by OPPO, - It seems block level repetition is still after CRC attachement, if FEC is Introduced. - For order between bit collection and block level repetition, a note is added to clarify that bit-collection is also included in the interleaving, thus bit-collection is performed together with channal coding, and it is also performed before block level repetition. FL Proposal 3.3-5-v2: For R2D, channel coding (with interleaving, if supported) is before block-level repetition (if supported). - Note: interleaving includes potential sub-block interleaving (if supported) and bit collection. |

Issue# 3.3-6 FEC coding rate

In previous meeting, companies discussed candidate FEC code rate, including 1/2, 1/3, 1/4, and it was bit controversial whether 1/2 and 1/4 to be supported. Companies are continuing discussions on the candidate code rates. Companies view are summarized as below:

- Whether support 1/2 code rate in addition to 1/3

  - [vivo] [NEC] [Apple] support 1/2 code rate to improve spectrum efficiency

  - [CATT] supports selection between 1/2 and 1/3 based on the R2D coverage target

  - [Samsung] does not support 1/2 code rate because of worse coverage performance than 1/3 in terms of coding gain.

  - [TCL] does not support 1/2 code rate because it may not be able to meet the extreme coverage requirements.

- Whether support 1/4 code rate in addition to 1/3

  - [vivo][CATT][Samsung] do not support 1/4 code rate, with following reasons:

    - its higher decoding complexity and negligible coding gain [vivo][CATT]

    - additional standard effort to define a new polynomial [Samsung]

    - unclear additional benefit than repetition [Samsung]

  - [NEC] suggests to further study the necessity to introduce 1/4 code rate according to the evaluation results for coverage performance in AI 9.3.1

FL notes:

- FL suggest update TR38.769 as follows.

[TP][U] FL Proposal 3.3-6

FL Proposal 3.3-6-v1

Update TR 38.769 for R2D FEC code rate:

For Code rate R=1/3: - Sources [R1-10321-123-3, vivo], [R1-10321-123-4, Nokia], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-16, LGE ], [R1-10321-123-17, Panasonic], [R1-10321-123-19, Apple], [R1-10321-123-14, ZTE], [R1-10321-123-24, Qualcomm] and [R1-10321-123-26, TCL] state R=1/3 can be supported. - Source [R1-10321-123-5, CMCC] states that, 1/3 code rate is supported when interleaving is not applied. - Source [R1-10321-123-6, Huawei] states that, 1/3 code rate is necessary and feasible, and should be supported due to its performance gain and an increased power consumption for active devices. - Source [R1-10321-123-16, LGE ] states that, it is beneficial to reuse the LTE TBCC with a preferred code rate of 1/3 to ensure reliable transmission and reduce specification complexity - Source [R1-10321-123-17, Panasonic] states that, for Device C, LTE TBCC with a coding rate of at least 1/3 could be beneficial to support for better coverage. - Source [R1-10321-123-24, Qualcomm] states that, LTE TBCC with a coding rate of 1/3 could achieve better performance than 3 repetitions. - Source [CATT] states that, the coding rate of TBCC should be selected between R=1/2 and R=1/3 based on the R2D coverage target. - Source [Samsung] states that, if FEC is supported for R2D, R=1/3 can be supported because LTE TBCC with code rate 1/3 is a well-established coding scheme and is already used in Rel-19 D2R. - Source [Xiaomi] states that additional code rates can be realized by LTE bit collection scheme based on 1/3 TBCC, and the puncturing and repetition ways are not flexible for realizing multiple values of cade rates. For code rate R=1/2, achieved by puncturing 3rd branch - For Code rate R=1/2, sources [R1-10321-123-3, vivo], [R1-10321-123-4, Nokia], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT] and [R1-10321-123-19, Apple] report that R=1/2 can be supported. - Source [R1-10321-123-3, vivo] reports, 1/2 code rate can improve spectrum efficiency and reduce power consumption for decoding. - Source [R1-10321-123-9, CATT] states that ,1/2 coding rate is preferable for TBCC, as it reduces device power consumption while achieving the same spectral efficiency as the Manchester coding in Rel-19 R2D. [CATT] states that, without LTE interleaving, TBCC codes with coding rate of 1/2 have more than 5dB coding gain over the Manchester code in TDL-C channel with 300ns delay spread, for OOK-4 with M=2, 6, 12 at BLER=10-1. The BLER performance of TBCC with a code rate of 1/3 is significantly better than that of TBCC with a code rate of 1/2, with a gain ranging from 1.14 to 2.32 dB at a BLER of 10⁻¹. - Sources [R1-10321-123-19, Apple] and [R1-10321-123-8, NEC] state that, 1/2 and 1/3 coding rate as baseline. - Sources [R1-10321-123-1, FUTUREWEI] state that, the performance difference between R=1/2 and R=1/3 is about 2 dB. - Source [R1-10321-123-6, Huawei] states that, the necessity of supporting other code rates apart from 1/3 for TBCC, which was supported in D2R for Rel-19, is not clear, because a 1/2 code rate performs worse than a 1/3 code rate - Source [Samsung] states that, 1/2 coding rate should not be supported because its coding gain is lower than 1/3 and it may even degrade R2D coverage, while its benefit is unclear For code rate R=1/4, source [R1-10321-123-25, NTT DOCOMO] reports that Code rate of R=1/4 can be achieved by repetition and puncturing based on LTE TBCC with 1/3 code rate. For code rate R=1/4, obtained by adding a new polynomial for the fourth branch - Sources [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO] and [R1-10321-123-19, Apple] report that, Code rate R=1/4 obtained by adding a new polynomial for the fourth branch, can be supported. - Source [R1-10321-123-19, Apple] states that, 1/4 coding rate can be considered for extreme coverage scenarios. - Source [R1-10321-123-24, Qualcomm] states that, 1/4 coding rate can further improve the performance than that of 1/3 coding rate. The extra polynomial [133 171 165 137], nested to the existing LTE CC (K=7 and R=1/3), is used - Source [R1-10321-123-1, FUTUREWEI] states that, the performance difference between R=1/3 and R=1/4 is about 1.25 dB. - Source [R1-10321-123-3, vivo] states that, 1/4 mother code rate increases power consumption and larger buffer for decoding, and it requires large standard effort to determine new polynomial for the 4th branch. The coverage enhancement can also be achieved by 1/3 mother code rate with block-level repetition. [vivo] states that, 1/2 code rate with 2 repetitions achieves similar performance of 1/4 mother code rate or even better performance than 1/4 code rate by repetition of bits from virtual circular buffer based on mother code rate 1/3. - Source [R1-10321-123-4, Nokia] states that, 1/4 code rate should be deprioritized since it increases complexity. - Source [CATT] states that, due to its higher decoding complexity and negligible coding gain, 1/4 coding rate should not be considered. - Source [Samsung] states that, 1/4 coding rate derived by repeating TBCC branch outputs should not be supported, because its benefit largely overlaps with R2D block-level repetition. R=1/4 achieved by adding a new coding polynomial should not be supported, because it would impose additional implementation burden at the gNB and require extra specification effort - Source [NEC] states that if code rate 1/4 is adopted in both R2D and D2R, same generation method should be applied. Source [R1-10321-123-1, FUTUREWEI] provide views for above code rates using tailed convolutional code with the same polynomial as LTE TBCC; while other sources provide views for above code rates using LTE TBCC.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| Xiaomi |  | For FEC coding rate, we provide two observations for realizing additional coding rate in our contribution. Observation 7: Based on the existing 1/3 TBCC encoder, one straightforward but not flexible way to achieve a specific additional code rate like 1/2 is adopting repetition and/or puncturing operations. Observation 8: Based on the existing 1/3 TBCC encoder, another flexible way to achieve additional code rates of various values is using the existing LTE bit collection scheme. So, please add our view as below: Source [Xiaomi] states that additional code rates can be realized by LTE bit collection scheme based on 1/3 TBCC, and the puncturing and repetition ways are not flexible for realizing multiple values of cade rates. |
| CATT | Y |  |
| NEC |  | For FEC coding, we provide the following proposal: Proposal 3: For Device 2b and Device C, TBCC with {1/2, 1/3} code rate can be the starting point for R2D FEC. - R2D transmission without FEC (i.e., code rate=1) can also be considered; - Further study the necessity to introduce other code rate (i.e., 1/4) according to the evaluation results for coverage performance in AI 9.3.1; if code rate 1/4 is adopted in both R2D and D2R, same generation method should be applied. Hence, please add our view on code rate 1/4 Source [NEC] states that if code rate 1/4 is adopted in both R2D and D2R, same generation method should be applied. |
| FL |  | Update based on reply from Xiaomi and NEC. |

  - R2D Scrambling

In previous meetings, companies discussed the feasibility and necessity of scrambling for R2D transmission without consensus, and companies continue discussion in their tdocs in this meeting.

Regarding the feasibility, OPPO, ERICSSON state it is feasible to do descrambling because it requires a small LFSR and simple binary addition/XOR operation, which is supportable by device 2b/C.

Regarding the necessity, three potential benefits proposed by companies includes

(1) interference randomization,

(2) reader identification, and

(3) preventing continuous 1s or 0s and improving clock recovery, synchronization and spectrum efficiency, if no Manchester coding is not applied.

Since RAN1 already concluded that Manchester is not removed, the benefit of (3) does not hold anymore. In the following, companies’ views on (1) & (2) are summarized as follows:

  - OPPO, IIT and Qualcomm states that, R2D scrambling enables interference randomization to reduce intra-frequency inter-cell/reader interference

  - vivo states that, reader identification relying on scrambling is infeasible for OOK modulation with presence of strong.

  - xiaomi states that, with cell frequency reuse factor of 3 per RAN4 agreement, the need of scrambling to reduce inter-cell/inter-reader interference may not be needed.

FL notes:

- Based on companies’ input, FL observes different views on the effectiveness of interference randomization and reader differentiation by R2D scrambling, considering OOK characteristic.

- Considering still quite diverged views on the necessity of R2D scrambling for the entire SI, FL suggests to drop the discussion.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Similar as other controversial topics, it’s worthy to capture companies’ observation on the R2D scrambling pros/cons. |
| FL |  | I will provide a TP later |

- R2D signal

  - R2D signal design for CFO/LO calibration

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Issue 4.1-1: CFO calibration signal design/type

Spreadtrum, Ericsson, CATT, Nokia, Xiaomi, ZTE, vivo, Qualcomm, Apple, LGE, TCL, Samsung , NEC, Sequans, Lenovo, InterDigital and NTT DOCOMO discussed the CFO calibration signal design or type in their contributions for this meeting

Companies provided their views on the 3 options.

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

Company views in the contributions for thies meeting is summarized as follows.

Note that, some of the company views provided in the contributions has already been capture in TR. Company views which has not been captured in TR, is marked with Grey Color.

Option a: unmodulated sinusoid single tone

[Positive views]

- Qualcomm provide evaluation results and show that, By using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=8dB and 1% error at SNR=18dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB.

- Spreadtrum states that, for earning a better calibration performance, CFO calibration signal/part is better a RF single-tone sine wave. Sequans states that it enables highly reliable frequency offset estimation even under large oscillator mismatches (hundreds to thousands of ppm), while maintaining minimal receiver complexity and energy consumption.

- Ericsson states that, for CFO calibration, single-carrier (a.k.a. unmodulated sinusoid single tone) method is simple but needs additional periodic signal. RAN1 recommends only CFO calibration based on a single-tone signal (a.k.a. unmodulated sinusoid single tone signal) which is common (i.e., agnostic) to device 2b and C.

- Xiaomi, Nokia, Apple, ZTE, Sharp, Quetel states that unmodulated sinusoid single tone feasible and required low complexity at AIoT device for CFO calibtation. ZTE states that it allows CFO correction to be implemented using hardware circuits (e.g., AFC loop), making it well-suited for A-IoT devices. And Sequans states that the constant-amplitude single-tone allows simple phase-based CFO estimation without correlation or symbol detection

- vivo states that unmodulated Single-tone signal can be used to calibrate CFO based on injection lock mechanism, and the frequency of LO is calibrated to the frequency of the single-tone signal. This method has been proposed for tens of years, which has clear working principle, and still widely used as effective CFO calibration method in low power devices in recent years.

- vivo states that unmodulated Single-tone signal has been supported in NB-IoT, i.e., NPRACH, there is no coexistence issue.

- Panasonic and TCL states that either Option a or Option b is can be considerer and it require less device complexity than option c.

- Quectel states that, the envelope of unmodulated signal for CFO calibration is constant, which provides higher calibration precision than modulated signal.

[Negative views]

- CATT states that, injection lock mechanism will consume huge energy of the device, which is unacceptable even for active devices.

- CATT states that, single-tone RF signal is not reliable and it is vulnerable to deep fading, which will reduce the reliability of calibration.

- CATT states that, for Options a sinusoidal signals for CFO calibration signals transmitted on NR DL spectrum, it will create the inter-channel interference to neighbouring NR sub-bands.

Option b: unmodulated multiple sinusoid single tones

[Positive views]

- Panasonic and TCL states that either Option a or Option b is can be considerer and it require less device complexity than option c.

[Negative views]

- Spreadtrum, ZTE, NEC and Quectel states that, unmodulated multiple sinusoid single tones will increase the complexity for device to perform CFO/LO calibration.

- Xiaomi and Quectel states that unmodulated multiple sinusoid single tones will result in unexpected frequency components when mixed with the local RF frequency signal.

- ZTE states that the benefit of using multiple sinusoid single tones for CFO calibration is unclear compared with single tone.

- vivo state that Option b (unmodulated multiple sinusoid single tones), based on counting of number of cycles of envelop of frequency components of |f1 – f2|, is not feasible for achieving 10 ppm accuracy. It requires either an extremely long duration or an excessive transmission bandwidth, both of which are impractical for R20 AIoT.

- Sequas state that multiple unmodulated tones increase PAPR and require FFT-based tone separation, raising both transmitter and receiver complexity.

- CATT states that, for Options b sinusoidal signals for CFO calibration signals transmitted on NR DL spectrum, it will create the inter-channel interference to neighbouring NR sub-bands.

Option c: Modulated multiple tones

[Positive views]

- CATT states that, modulated multiple tones, can be used for CFO calibration based on pre-compensation, R2D synchronization signal and M-sequence can be used for CFO calibration signal.

  - With the CFO pre-compensation method, if R2D CFO calibration signal adopts M-sequence with M value equals to 4 and its length equals to 7 bits, it will occupy only two OFDM symbols, the 90% CDF residual CFO after calibration at the device side is no more than 5ppm for Device C and 20ppm for Device 2b.

  - The pre-compensated CFO received signals should be used for CFO calibration to reduce the degradation of received signal detection and decoding performance due to frequency errors for Device 2b and Device C.

[Negative views]

- Spreadtrum states that, modulated multiple tones will increase the complexity for device to perform CFO/LO calibration. Active devices will use analog circuits, e.g., PLL or FLL, to achieve frequency tracking and locking.

- Ericsson states that, using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm), whereas the CFO calibration of device C may require using single-tone (a.k.a. unmodulated sinusoid single tone) to meet a residual CFO of 10 ppm.

- Quectel states that the random changes in the amplitude and phase of the modulated signal will introduce "noise", which contaminates the estimation of frequency and phase, thereby affects the calibration results at device side.

- Sequans states that modulated multiple tones further complicate synchronization and demands symbol-level demodulation. such a composite, chopped by ON/OFF gating, multi-tone chunk faces multi-tone beating, edge transients at every ON/OFF (phase jitter), and unknown per-ON phase contributions from mapping/windowing and the channel. Knowing the envelope transitions will not give the per-subcarrier phase reference needed.

- ZTE and vivo states that, based on multi-tone LP-SS discussion, the LP-WUR lacks the capability to acquire good frequency calibration. vivo further states that, according to agreements for LP-SS OOK sequence in LP-WUS WI (RAN1#118 meeting), the design metric only include time synchronization accuracy and RRM measurement accuracy, no design consideration and metric for frequency error calibration in LP-SS design.

- vivo states that it is not feasible to use option c, modulated multiple tones, i.e., OOK sequence for R2D CFO calibration, as that in D2R receiver.

  - Each CFO hypothesis or CFO compensation should be constructed by multiplying a phase rotation sequence to local sequence, and each phase rotation sequence corresponds to a CFO hypothesis. However, only envelop/amplitude of the Rx signal can be obtained in IF-ED detector, and there is no I/Q path in receiver, phase information which reflects frequency offset is lost in receiver.

  - Estimation or compensation using R2D OOK sequence is still not feasible, since the R2D waveform generation is not specified, which means the sample level amplitude (with ripples) and phase at transmitter is unknown to receiver when constructing local sequence, because OOK signal generation totally up to Reader implementation.

  - It is not feasible to use RSRP-like metric to determine CFO at receiver. To achieve 10ppm calibration accuracy, several filter covering different frequency point should be used which is too complex for AIoT devices. Even if using so many IF filter, the received power captured into each filter would be very close. The right hypothesis cannot be reliably determined, the accuracy of CFO calibration cannot be guaranteed.

- NEC states that, since the CFO calibration is mainly applied to the LO in RF level, which cannot be based on the baseband clock, for the signal design, Option c is not feasible.

FL notes:

- The situation on 3 signal designs for CFO calibration signal based on inputs in RAN1#123 and RAN1#124, is summarized as follows.

| Options | Companies views |
| --- | --- |
| For Option a: unmodulated sinusoid single tone | FUTUREWEI, Spreadtrum, vivo, CMCC, Huawei, Ericsson, NEC, CTC, Xiaomi, Samsung, ZTE, LGE, Lenovo, Apple, MediaTek, Sharp, Quectel, Qualcomm, NTT DOCOMO, TCL, IITK, CEWiT, Sequans, InterDigital, Sony, Nokia Panasonic |
| Option b: unmodulated multiple sinusoid single tones | LGE, Qualcomm, TCL, Panasonic |
| Option c: Modulated multiple tones | CATT, NTT Docomo, MediaTek |

- Clear majority companies support option a, unmodulated sinusoid single tone as CFO calibration signal, and the principle of CFO calibration is clear, with references and evaluation results provided by proponents.

- Although there are few proponents for option B. The principle on how option b works is still not provided by any of the proponents, until this very last meeting, this option is less likely to be supported even we postpone the decision in WI.

- The signal design and principle of CFO calibration are diverse from supporters of option c. Clear majority companies does not support option c, and technical concerns are not fully addressed for any of the 3 working principles. This option is less likely to be supported even we postpone the decision in WI.

- Some of the company views summarized above, has already been capture in TR, the new information provided by companies are reflected in the updated TP.

Given the situation summarized above, FL suggest the following proposal.

[TP][U] FL Proposal 4.1-1

FL Proposal 4.1-1-v1

Update TR 38.769 for signal design for CFO calibration

For the signal design, the following options are studied: Option A: Unmodulated single tone sinusoid. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson], [R1-10321-123-8, NEC], [R1-10321-123-10, CTC], [R1-10321-123-11, Xiaomi], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-18, Lenovo], [R1-10321-123-19, Apple], [R1-10321-123-21, MediaTek], [R1-10321-123-22, Sharp], [R1-10321-123-23, Quectel], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-26, TCL], [R1-10321-123-27, IITK], [R1-10321-123-28, CEWiT], [R1-10321-123-29, Sequans], [R1-10321-123-20, InterDigital], and [R1-10321-123-30, Sony], [Nokia] and [Panasonic] state that Option A is feasible, and can be considered for CFO calibration signal. The following design principles for this option are described according to sources: - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei] and [R1-10321-123-25, NTT DOCOMO], report with references, and state that, the CFO calibration is based on injection lock mechanism, and the working principle of injection lock is to leverage the phase interaction between an external single-tone signal and the internal oscillation signal of an oscillator to lead the oscillator’s frequency lock to that of the unmodulated sinusoid single-tone signal. - Source [R1-10321-123-5, CMCC] report with simulation result, and states that, CFO calibration using unmodulated sinusoid is performed at intermediate frequency, where the reader transmits the CFO calibration signal with a pre-defined duration, the device mixes the CFO calibration signal with its internally generated wave, can then count the number of periodicities of the residual frequency within the duration to estimate and calibrate the CFO. The following sources provide reasons to support this option: - Source [R1-10321-123-5, CMCC] report that with a duration of 1 OFDM symbol, the CFO after calibration can be reduced to less than 10 ppm. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-7, Ericsson], [R1-10321-123-10, CTC], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-24, Qualcomm], [R1-10321-123-18], and [R1-10321-123-27, IITK], [Xiaomi], [Nokia], [Apple], [Sharp], [Panasonic], [TCL], [Sequans] and [Quetel] state that unmodulated sinusoid single tone can achieve CFO calibration with low complexity. Source [ZTE] states that it allows CFO correction to be implemented using hardware circuits (e.g., AFC loop), making it well-suited for A-IoT devices. And Source [Sequans] states that the constant-amplitude single-tone allows simple phase-based CFO estimation without correlation or symbol detection. - Source [R1-10321-123-3, vivo] states that single tone signal is already supported in NB-IoT, (e.g., NPRACH), there is no co-existence issue. - Source [Qualcomm] provide evaluation results and show that, By using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=20dB and 1% error at SNR=30dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB. - Source [Qualcomm] provide evaluation results and show that Device 2b requires a higher CNR than Device C because of its larger SFO/CFO before/after calibration, e.g., by using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=8dB and 1% error at SNR=18dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB. - Source [Quectel] states that, the envelope of unmodulated signal for CFO calibration is constant, which provides higher calibration precision than modulated signal. The following sources provide reasons to not support this option: - Source [R1-10321-123-9, CATT] states that it will create the inter-channel interference to neighboring NR sub-bands. - Source [R1-10321-123-24, Qualcomm] and [CATT] states that this method may be vulnerable to deep fading, reducing reliability. - Source [CATT] states that, injection lock mechanism will consume huge energy of the device, which is unacceptable even for active devices. Option B: Unmodulated multiple single tone sinusoids. - Sources [R1-10321-123-16, LGE ], [R1-10321-123-24, Qualcomm], and [R1-10321-123-26, TCL] and [Panasonic] report that Option B can be considered for CFO calibration The following reasons were provided to support this option: - Source [R1-10321-123-16, LGE ] states that, Option B provides frequency diversity gain while maintaining low complexity; considered as optional feature. It is preferable that the tones are sufficiently separated in frequency, e.g., by at least the Rx filter bandwidth. - Source [R1-10321-123-24, Qualcomm] states that, using multiple sinusoidal tones can improve robustness against fading by providing frequency diversity. - Source [R1-10321-123-26, TCL] states that, although Option B is slightly more complex than a single tone, it is still a very simple waveform. The following reasons were provided to not support this option: - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-2, Spreadtrum], [R1-10321-123-30, Sony], [R1-10321-123-26, TCL], and [R1-10321-123-23, Quectel]. [Spreadtrum], [ZTE], [NEC] and [Quectel] state that Option B increases in detection complexity, and it may also complicate frequency resource allocation. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-6, Huawei] raised feasibility concerns on Option B - Source [R1-10321-123-3, vivo] states that even with increased complexity, it is not clear how does it work, since frequency of each tone cannot be accurately determined, due to only frequency range of each can be determined if captured by Rx filter, and the target frequency for LO calibration cannot be accurately determined. - Sources [R1-10321-123-6, Huawei], and [R1-10321-123-8, NEC], [Xiaomi] and [Quectel] state that multiple frequency components would occur when mixed with the local RF frequency signal and it is difficult to lock the LO frequency. - Source [R1-10321-123-29, Sequans] state that multiple unmodulated tones increase PAPR and require FFT-based tone separation - Source [ZTE] states that the benefit of using multiple sinusoid single tones for CFO calibration is unclear compared with single tone. - Source [vivo] states that Option b (unmodulated multiple sinusoid single tones), based on counting of number of cycles of envelop of frequency components of ¦f1 – f2¦, is not feasible for achieving 10 ppm accuracy. It requires either an extremely long duration or an excessive transmission bandwidth, both of which are impractical for R20 AIoT. - Source [CATT] states that, for Options b sinusoidal signals for CFO calibration signals transmitted on NR DL spectrum, it will create the inter-channel interference to neighbouring NR sub-bands. Option C: Multiple modulated tones, where the CFO calibration signal may be the same as or different from the synchronization signal. The following three different design principles for this option are described according to sources: - Source [R1-10321-123-7, Ericsson] state that the unmodulated multiple tones are transmitted in different frequency locations in different time resources The reader transmits the synchronization signal using a centered bandwidth sweeping method, where each centered bandwidth covers a portion of the overall synchronization signal bandwidth The device detects the synchronization signal power at different strengths on each of the subsets and based on the subset with the strongest synchronization signal power, the device applies a frequency correction - Sources [R1-10321-123-9, CATT] and [R1-10321-123-21, MediaTek] report that, CFO calibration is done in baseband domain, and CFO Calibration is OOK sequence (not R19 SIP or CAP). The CFO pre-compensation method is given as follows: Step 1: The received signals with pre-compensated CFO ±Δf is added in the received R2D signals to limited the frequency offset within ±1/2 Δf. Step 2: The correlation processing of the CFO calibration signal with the received signals, the precompensated received signals with additional Δf, and the pre-compensated received signals with additional -Δf. Step 3: The peak of the correlation values from the three correlation outputs would be selected as the received signals for the R2D signal processing. - Source [R1-10321-123-25, NTT DOCOMO] states that, the CFO calibration can be achieved by measuring chip length, and that one possible implementation is devices calibrate CFO based on SFO calibration by counting the number of samples of 0s and 1s and comparing with the defined duration of a pattern. The following reasons were provided to support this option: - Source [R1-10321-123-7, Ericsson] state that using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm) - Source [R1-10321-123-9, CATT] state that the OFDM waveform-based CFO calibration signals would be orthogonal to neighboring NR channels - Sources [R1-10321-123-9, CATT] and [R1-10321-123-21, MediaTek] state that R2D synchronization signal can be used for CFO calibration signal - Source [R1-10321-123-25, NTT DOCOMO] state that may be feasible if same clock (LO) is used for both carrier frequency generation and sampling. - Source [CATT] states that, modulated multiple tones, can be used for CFO calibration based on pre-compensation, R2D synchronization signal and M-sequence can be used for CFO calibration signal. - With the CFO pre-compensation method, if R2D CFO calibration signal adopts M-sequence with M value equals to 4 and its length equals to 7 bits, it will occupy only two OFDM symbols, the 90% CDF residual CFO after calibration at the device side is no more than 5ppm for Device C and 20ppm for Device 2b. - The pre-compensated CFO received signals should be used for CFO calibration to reduce the degradation of received signal detection and decoding performance due to frequency errors for Device 2b and Device C. The following reasons were provided to not support this option: - Source [R1-10321-123-7, Ericsson] state that the CFO calibration of device C may require using unmodulated sinusoid single tone to meet a residual CFO of 10 ppm. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC] and [R1-10321-123-28, CEWiT] state that the CFO hypothesis at AIoT receiver cannot be constructed in baseband due to phase information for received signal is not kept in envelop detector, and no I/Q path in receiver baseband. - Source [R1-10321-123-14, ZTE] and [vivo] report that, in multi-tone LP-SS discussion, the LP-WUR doesn’t have the capability to acquire good frequency calibration, similarly this method is not reliable. And source [vivo] further states that, according to agreements for LP-SS OOK sequence in LP-WUS WI (RAN1#118 meeting), the design metric only include time synchronization accuracy and RRM measurement accuracy, no design consideration and metric for frequency error calibration in LP-SS design. - Sources [R1-10321-123-6, Huawei] and [R1-10321-123-8, NEC] state that the LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure and the calibration signal should be utilized for the RF-end clock. - Source [R1-10321-123-3, vivo] state that using ON chip duration to calibration Clock it not feasible, since according to RAN4 requirements on pulse width, Table 6.4.2-1 in section 6.4.2 of Ts 389.194, the pulse of a chip, the allowed chip length can be <=1.3 Tc when the nominal chip length is Tc, in this case, it is not feasible to use chip length/pulse width for CFO calibration. - Source [R1-10321-123-18, Lenovo] states that pulse width distortion analysis is possible to be used for A-IoT device due to simplicity, but timer resolution at device is required to be much less than the difference between estimated pulse width and the transmitted pulse width. - Source [vivo] states that it is not feasible to use option c, modulated multiple tones, i.e., OOK sequence for R2D CFO calibration, as that in D2R receiver. - Each CFO hypothesis or CFO compensation should be constructed by multiplying a phase rotation sequence to local sequence, and each phase rotation sequence corresponds to a CFO hypothesis. However, only envelop/amplitude of the Rx signal can be obtained in IF-ED detector, and there is no I/Q path in receiver, phase information which reflects frequency offset is lost in receiver. - Estimation or compensation using R2D OOK sequence is still not feasible, since the R2D waveform generation is not specified, which means the sample level amplitude (with ripples) and phase at transmitter is unknown to receiver when constructing local sequence, because OOK signal generation totally up to Reader implementation. - It is not feasible to use RSRP-like metric to determine CFO at receiver. To achieve 10ppm calibration accuracy, several filter covering different frequency point should be used which is too complex for AIoT devices. Even if using so many IF filter, the received power captured into each filter would be very close. The right hypothesis cannot be reliably determined, the accuracy of CFO calibration cannot be guaranteed. - Source [NEC] states that, since the CFO calibration is mainly applied to the LO in RF level, which cannot be based on the baseband clock, for the signal design, Option c is not feasible.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | We have updated our tdoc as R1-2601481, where the CFO calibration evaluation results are updated. - Source [Qualcomm] provide evaluation results and show that Device 2b requires a higher CNR than Device C because of its larger SFO/CFO before/after calibration, e.g., by using Option a, the CFO calibration signal with 2 OFDM symbol can achieve CFO calibrated from 1000ppm to 100ppm for Device 2b with 10% error at SNR=8dB and 1% error at SNR=18dB, and to achieve CFO calibrated from 50ppm to 10ppm for Device C with 10% error at SNR=0dB and 1% error at SNR=10dB. |
| LGE | Y |  |
| CATT | Y | OK to capture it the TR and make the down-selection from the 3 options in the WI phase. |
| NEC | Y |  |
| FL |  | Updated based on reply from Qualcomm |

[H] FL Proposal 4.1-1a

FL Proposal 4.1-1a-v1

For CFO calibration signal, Option a: unmodulated sinusoid single tone is considered for R20 AIoT, and following options is not considered

- Option b:unmodulated multiple sinusoid single tones

- Option c: Modulated multiple tones

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| [OPPO] | Y |  |
| Spreadtrum | Y |  |
| Panasonic | Y |  |
| LGE | Y |  |
| Lenovo | Y |  |
| Xiaomi | Y |  |
| DOCOMO | Y |  |
| Samsung | Y |  |
| Huawei, HiSilicon | Y | We support the use of an unmodulated single tone sinusoid for the CFO calibration signal because it is meant to be used for the LO frequency calibration for active devices where the LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure. It should be utilized for the RF-end clock, for which, we propose to use a single-tone signal for the analog circuit tracking and locking its RF frequency. If the CFO calibration signal is a baseband multiple single tones or multiple tones, multiple frequency components would occur when mixed with the local RF frequency signal and it is difficult to lock the LO frequency. |
| ZTE, Sanechips | Y |  |
| CATT | N | As we mention in the tdoc, Option a has the following critical issues which need to be addressed before making a decision, the down-selection is too early in this stage since our following critical technical concerns are not addressed by the proponent of Option a: - Concern#1: injection lock mechanism will consume huge energy of the device, which is unacceptable even for active devices. - Concern#2: single-tone RF signal is not reliable and it is vulnerable to deep fading, which will reduce the reliability of calibration. - Concern#3: single-tone RF signal transmits on NR DL spectrum, it will create the inter-channel interference to neighbouring NR sub-bands. |
| NEC | Y |  |
| FL |  | It seems CATT still have concern on option a and prefer option C. I think we can first exclude option b, and further down-select between option a and option C in WI phase. The principle on how option b works is still not provided by any of the proponents, until this very last meeting, this option is less likely to be supported even we postpone the decision in WI. Hence, the proposal is updated as follows FL Proposal 4.1-1a-v2 For CFO calibration signal, following options is not considered - Option b:unmodulated multiple sinusoid single tones |

Issue 4.1-2: Periodic or aperiodic for CFO calibration signal

Huawei, ZTE, CATT, vivo, OPPO, Xiaomi, CMCC, Apple, DOCOMO, Ericsson, Nokia, LGE, Panasonic, Samsung, Sharp, Lenovo, Qualcomm, Quectel, TCL, Sequans, and NEC provide views on Periodic or aperiodic for CFO calibration signal.

Regarding selection between periodic or aperiodic transmission for CFO calibration signal. Company views are summarized as follows

  - Huawei states that

- For a periodic CFO calibration signal, it is transmitted in relation to the synchronization signal, i.e., periodically after the periodic synchronization signal and the MIB-like broadcast nformation.

- For an aperiodic CFO calibration signal, it is transmitted in relation to the data payload of the PRDCH, i.e., after the PRDCH carrying the paging message for DT/DO-DTT transmissions, with its presence indicated by the L1 R2D control information.

  - OPPO states that, CFO calibration signal to be transmitted periodically with synchronization signal, and immediately after the SIP.

  - CATT states that, Since R2D reception and D2R transmission may be either periodic or aperiodic, both periodic and aperiodic CFO calibration signals should be supported.

  - CMCC states that, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

  - Xiaomi States that

- For the purpose of power saving at reader side, a large periodicity can be considered for CFO calibration signal transmission.

- Meanwhile, to guarantee the calibration precision at Device side, additional on demand CFO calibration signal can be also considered.

  - ZTE states that

- At least for D2R traffic triggered by PRDCH, aperiodic CFO calibration signals transmitted as needed can be considered.

  - vivo suggests that, transmitted after priodic sync signal or broadcast information.

  - Panasonic states that, the synchronization signal transmission can be quasi periodic at reader if these are within the limit on variance and the device is assumed not to use the relation between synchronization signals.

  - Samsung suggest to study the conditions under which the CFO calibration signal should be transmitted.

- Option 1. in every R2D transmission

- Option 2. only for specific R2D message types (e.g.,paging)

- Option 3. with a specific periodicity

  - Apple propose periodic transmission

FL notes

  - Views from Panasonic and Samsung are not included in current TR, while views from other companies seems already captured in TR. Hence, the update is proposed for the delta information.

[TP][U] FL Proposal 4.1-2

FL Proposal 4.1-2-v1

Update TR 38.769 for periodic or aperiodic CFO calibration signal transmission

6A.1.x.3 Transmission Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE], [R1-10321-123-19, Apple], [R1-10321-123-20, InterDigital], [R1-10321-123-21, MediaTek], [R1-10321-123-23, Quectel], [R1-10321-123-29, Sequans], [R1-10321-123-19, Apple], [R1-10321-123-24, Qualcomm], [R1-10321-123-26, TCL], [R1-10321-123-18, Lenovo] and [R1-10321-123-28, CEWiT] state that the CFO calibration signal is periodically transmitted. - Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-10, CTC] and [R1-10321-123-30, Sony] state that, CFO calibration signal with hundreds ms or 1 second periodicity is already sufficient. Aperiodic CFO calibration signal, e.g., on demand transmission with PRDCH is not needed due to slow frequency drift. Source [R1-10321-123-7, Ericsson] also states that the CFO calibration signal can be transmitted in very sparse manner. - Sources [R1-10321-123-5, CMCC], [R1-10321-123-6, Huawei], [R1-10321-123-10, CTC], [R1-10321-123-18, Lenovo] and [R1-10321-123-23, Quectel] states that, to support DO-A traffic, periodic CFO calibration signal is needed. - Source [R1-10321-123-5, CMCC] states that, periodic CFO calibration is needed to avoid a device from keep monitoring the CFO calibration signal before transmitting the very first D2R transmission for DOA traffic. - Source [R1-10321-123-21, MediaTek] and [R1-10321-123-2, Spreadtrum] states that it is beneficial to provide regular opportunities for devices to perform frequency calibration and maintain accurate synchronization with the reader. - Source [Samsung] suggest to study the condition that CFO calibration signal transmitted with a specific periodicity. Sources [R1-10321-123-6, Huawei], [R1-10321-123-9, CATT], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-21, MediaTek], [R1-10321-123-23, Quectel], [R1-10321-123-2, Spreadtrum], [R1-10321-123-18, Lenovo], [R1-10321-123-19, Apple], [R1-10321-123-22, Sharp], [R1-10321-123-24, Qualcomm], [R1-10321-123-27, IITK], [R1-10321-123-13, Samsung] and [R1-10321-123-28, CEWiT] state that the CFO calibration signal can be transmitted aperiodically. In which, Sources [R1-10321-123-6, Huawei], [R1-10321-123-10, CTC], and [R1-10321-123-23, Quectel] state that an aperiodic CFO calibration signal can be considered with periodic CFO calibration signal as baseline. - Source [R1-10321-123-6, Huawei] state that with the DRX configuration, the device will miss several instances of the periodic CFO calibration signal. In this case, the CFO calibration signal can be considered to be transmitted as needed, e.g., transmitted after the PRDCH carrying the paging message. Source [Samsung] suggest to study the condition that CFO calibration signal can be together with specific R2D message types (e.g.,paging) - Source [R1-10321-123-9, CATT] states that the R2D calibration signal should be transmitted before R2D reception and D2R transmission. Since R2D reception and D2R transmission may be either periodic or aperiodic, both periodic and aperiodic CFO calibration signals should be supported. - Sources [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-18, Lenovo], and [R1-10321-123-21, MediaTek] states that, at least for D2R triggered by PRDCH, the on-demand transmission of CFO calibration signals is beneficial to keep the interval between the CFO calibration signal and the subsequent D2R transmission short for better CFO calibration accuracy. Source [Qualcomm] states that aperiodic CFO calibration signal may be needed in addition to periodic CFO calibration to ensure the device's CFO calibration accuracy for D2R transmission. - Source [R1-10321-123-3, vivo] states that if CFO calibration totally relying on aperiodic signal, the CFO calibration accuracy cannot be guaranteed, which depends on traffic. - Source [Samsung] suggest to study the condition that CFO calibration signal transmitted in every R2D transmission. Source [Panasonic] states that, the synchronization signal transmission can be quasi periodic at reader if these are within the limit on variance and the device is assumed not to use the relation between synchronization signals.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic | Y |  |
| LGE | Y |  |
| CATT | Y | OK |
| FL |  | Add Qualcomm’s view, based on comment for proposal 4.1-4 |

Issue 4.1-3: Time locations of CFO calibration signal

There are five options per previous agreements

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

Huawei, Futurewei, Ericsson, ZTE, vivo, Apple, Samsung, Qualcomm, CATT, CMCC, Spreadtrum, OPPO, Sequans, Docomo and TCL provide views on the time location of CFO calibration signal.

Company views are summarized in the following table.

| Option | Company Views (Support / Opposition) | Detailed Rationale |
| --- | --- | --- |
| Option 1: In relation to L1 R2D control info | Before L1 R2D control Supported by: - CATT, LGE, Panasonic, Docomo Not supported by: - Huawei, vivo, TCL, ZTE | [Pros] - Correction before control/data (CATT, LGE, Docomo); - Benefits FDM (LGE); - Easier for other devices to skip (Panasonic). [Cons] - Not necessary if already calibrated (Huawei); - Time drift makes start unreliable (vivo); Switching delay causes interruption (vivo); - Challenge for AIoT devices (TCL); - Resource wastage (ZTE). |
|  | After L1 R2D control Supported by: N/A Not supported by: - Vivo | [Cons] - Interruption to subsequent R2D data payload due to calibration delay (vivo). |
| Option 2: In relation to data payload of PRDCH | Before data payload of PRDCH Supported by: - LGE Not supported by: - vivo, CATT | [Pros] - Update CFO estimate immediately prior to R2D transmission (LGE). [Cons] - Interruption to payload reception (vivo); - Cannot apply to detection of L1 control (CATT). |
|  | After data payload of PRDCH Supported by: - Samsung, Huawei, TCL, vivo, Qualcomm, Sequans, Lenovo, NEC, Spreadtrum, Xiaomi, ZTE, Docomo Not supported by: - N/A | [Pros] - Sleep earlier, avoid interference (Samsung); - Indicated by L1 control (Huawei, Samsung); - Minor impact on timeliness (TCL); - Avoid interruption (vivo); - Efficient (Qualcomm, Sequans); - Improve PDRCH (Lenovo, NEC, Spreadtrum, Xiaomi, Docomo). [Cons] - Availability depends on traffic (vivo); |
| Option 3: In relation to PDRCH | Prior to PDRCH Supported by: - ZTE, LGE, Apple, Docomo Not supported by: - CATT, vivo, TCL, ZTE, Sequans, Huawei, Ericsson, CMCC | [Pros] - Frequency accuracy for D2R (ZTE, LGE); - Viable but extra wakeup (Apple). [Cons] - No gain due to slow drift (Huawei, Ericsson, vivo, CMCC); - Resource fragmentation (vivo); Time drift issues (vivo, TCL); - Power consumption/double wakeup (ZTE); - Inefficient (Sequans); - Cannot calibrate for PRDCH (CATT). |
| Option 4: In relation to sync signal | Before periodic sync signal Supported by: - N/A Not supported by: - Vivo | [Pros] - N/A [Cons] - Interruption to synchronization signal reception (vivo). |
|  | After periodic sync signal Supported by: - vivo, Apple, Qualcomm, LGE, CMCC, TCL, OPPO, ETRI, InterDigital, NEC, Docomo | [Pros] - Stable accuracy, common reference (vivo); - Efficiency (Qualcomm, vivo, ETRI); - utilize wakeup time (Apple) - Reduce complexity (InterDigital); - Beneficial to D2R (NEC, ETRI). - Limit the flexibility of CFO calibration signal transmission, and Periodicity may not be suitable for D2R (ZTE). - Power saving (ZTE). |
| Option 5: In relation to broadcast info | Before broadcast information Supported by: - CATT, ZTE, Docomo Not supported by: - Huawei | [Pros] - Improve PRDCH/Broadcast info (CATT, Docomo); - Reliable broadcast reception (ZTE). [Cons] - Not necessary, increases overhead/complexity (Huawei). - Limit the flexibility of CFO calibration signal transmission (ZTE). - Periodicity may not be suitable for D2R (ZTE). |
|  | After broadcast information Supported by: - Huawei, vivo, CMCC, NEC, Docomo Not supported by: - N/A | [Pros] - Stable accuracy, common reference (vivo); - Block transmission (CMCC, Huawei); - Beneficial to D2R (NEC, Docomo). [Cons] - N/A |

FL notes

- Above Table summarizes company views for better understanding the situation.

- FL suggest to capture company views in TR 38.769.

[TP][N] FL proposal 4.1-3

FL proposal 4.1-3-v1

For the time location of the CFO calibration signal, capture following in TR38.769

Option 1: In relation to the L1 R2D control information

- For CFO calibration signal preceding L1 R2D control information

[Positive observations]

  - Source [CATT] and [LGE] states that R2D CFO calibration signal should be located before the L1 R2D control information to enable frequency offset correction before reception of control and data, enhancing decoding performance.

  - Source [LGE] states that option 1 provides benefits for potential R2D FDM operation, as the device can perform R2D processing with reduced residual CFO.

  - Source [Panasonic] states that, if the PRDCH / L1 R2D control information is device specific, option 1 could be easier as other devices are not required to determine the TBS comparing with option 2.

  - Source [Docomo] states that CFO calibration signal transmitted before L1 R2D control can be applied for CFO calibration for L1 R2D control (if needed), PRDCH (if needed), aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

[Negative observations]

  - Source [Huawei] states that, it is not necessary to transmit an CFO calibration signal prior to an R2D transmission after the periodic block since the device has already calibrated its LO using the CFO calibration signal transmitted along with the periodic synchronization signal and the MIB-like broadcast information, and can take advantage of the residual CFO and the minimal frequency drift.

  - Source [vivo] states that, the start of CFO calibration signal cannot be reliably determined due to time drift if it is located before L1 control information and before SIP.

  - Sourve [vivo] states that, CFO calibration signal immediately preceding L1 control could cause interruption to subsequent control or data payload reception due to calibration/RF switching delay.

  - Source [TCL] states that option 1 may require the device's RF/baseband to have a high-speed frequency offset estimation loop that can be adjusted within a few symbol times, which would be a challenge for AIoT devices.

  - Source [vivo] states that, the CFO calibration accuracy is unstable if PRDCH with L1 control transmission is aperiodic.

Option 2: In relation to the data payload of the PRDCH

- For CFO calibration signal preceding data payload of PRDCH

[Positive observations]

  - Source [LGE] states that transmitting the CFO calibration signal before PRDCH reception allows the device to update its CFO estimate immediately prior to the R2D transmission, and provides benefits for potential R2D FDM operation, as the device can perform R2D processing with reduced residual CFO, when CFO calibration signal is transmitted before the data payload of the PRDCH.

  - Source [Docomo] states that CFO calibration signal transmitted after PRDCH can be applied for CFO calibration for aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift.

[Negative observations]

  - Source [CATT] states that R2D CFO calibration signal will not be able to apply to the detection of L1 R2D control information prior to time location of CFO calibration signal.

  - Source [vivo] states that CFO calibration signal immediately before R2D data payload would cause interruption to the payload reception.

- For CFO calibration signal after data payload of PRDCH at the end of a R2D transmission

[Positive observations]

  - Source [Samsung] states that the CFO calibration signal be placed right after the R2D data payload. This allows devices not requiring calibration to ignore the signal and enter sleep earlier, while avoiding interference with ongoing payload decoding. Besides, the impact of padding can be further studied.

  - Source [Huawei], [Samsung] and [ETRI] state that an aperiodic CFO calibration signal transmitted after the PRDCH (e.g., paging message), with its presence indicated by L1 control.

  - Source [TCL] states that placing the calibration signal after receiving the PRDCH has minor impact on timeliness and does not delay acquisition of control info.

  - Source [vivo] observes that interruption of R2D transmission can be avoided if CFO calibration signal is transmitted at the end of the R2D block (after the data payload).

  - Source [Qualcomm] supports aperiodic LO calibration using CFO calibration signal transmitted together with the PRDCH.

  - Source [Sequans] and [ETRI] state that it is efficient to include CFO calibration signal together with PRDCH transmission.

  - Source [Lenovo], [NEC], [Spreadtrum] and [Xiaomi] states that it is beneficial to place the CFO calibration signal after the PRDCH to improve performance of PDRCH.

[Negative observations]

  - Source [vivo] states that the availability of CFO calibration signal depends on traffic if not transmitted along with broadcast information.

Option 3: In relation to the PDRCH (D2R transmission),

- the following observations are made assuming the CFO calibration signal transmitted prior to the PDRCH (D2R transmission)

[Positive observations]

  - Source [ZTE], [LGE] and [Docomo] state that the periodic CFO calibration signal is transmitted before PDRCH/DO-A transmission, which is beneficial for ensuring frequency accuracy for D2R transmissions.

  - Source [Apple] notes that independent transmission of the calibration signal (Option 3) can be considered as a viable but it requires additional device wakeup.

[Negative observations]

  - Sources [CATT] states that R2D CFO calibration signal is transmitted before the PDRCH, and it cannot be used for the calibration of the device clock for LO for carrier frequency for the reception of PRDCH.

  - Source [vivo] states that, standalone CFO calibration signal in relation to PDRCH (not transmitted with R2D) causes resource fragmentation, and leads to high overhead of transmitted in relation to all PDRCH transmissions.

  - Source [vivo] and [TCL] states that the start of CFO calibration signal cannot be reliably determined due to accumulated time drift after the previous synchronization.

  - Source [ZTE] states that device power consumption may increase in this option, since the device needs to wake up twice to receive common signals.

  - Source [Sequans] states that having a separate CFO calibration signal transmission will be inefficient for ultra-low-power AIoT devices due to extra monitoring and wake/sleep overhead.

  - Sources [Huawei], [Ericsson], [vivo] and [CMCC] find that this location offers no remarkable difference or gain for D2R reception performance due to slow CFO drift. Source [Huawei] further states that periodic CFO calibration signal with a predefined period will always help to improve the D2R performance by maintaining the residual CFO level with very small additional frequency drift.

Option 4: In relation to the synchronization signal

- For CFO calibration signal transmitted preceding the synchronization signal

[Negative observations]

  - Source [vivo] states that CFO calibration signal preceding synchronization signal would cause interruption to synchronization signal reception.

- For CFO calibration signal transmitted after the synchronization signal in a R2D block

[Positive observations]

  - Source [vivo] states that CFO calibration signal can be transmitted immediately after the periodic synchronization signal (if not with broadcast). This ensure a stable accuracy due to periodic transmission.

  - Source [vivo], [ZTE] and [ETRI] state that this option minimizes resource overhead, and achieves higher resource efficiency.

  - Source [ZTE] state that placing common signals within the same time period is beneficial for device power saving.

  - Source [Huawei] states that periodic CFO calibration signals can be transmitted immediately after a periodic synchronization signal and MIB-like part.

  - Source [Apple] recommends that the CFO calibration signal be transmitted at the end of the periodic synchronization signal as a baseline, efficiently utilizing device wake-up periods.

  - Source [Qualcomm], [LGE], [OPPO] and [TCL]states that CFO calibration signal as part of the periodic synchronization signal. Source [TCL] states that periodic synchronization signals are very suitable for also performing frequency offset calibration at the same time.

  - Source [CMCC] states that AIoT device can first search the periodic synchronization signal for time synchronization and frequency acquisition due to it is robust to CFO, and then receive the CFO calibration signal.

  - Source [interDigital] states that periodic R2D synchronization signal contains at least one sequence for timing synchronization and one sequence for CFO calibration, which would reduce monitoring complexity.

  - Source [NEC] and [ETRI] state that the CFO calibration is transmitted immediately after periodic synchronization signal is beneficial to D2R performance.

  - Source [Docomo] states that CFO calibration signal is transmitted after synchronization signal and can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx.

[Negative observations]

  - Source [ZTE] states that the periodicity of CFO calibration signal along with periodic synchronization signal may not suitable for D2R.

Option 5: In relation to broadcast information

- For CFO calibration signal transmitted preceding broadcast information

[Positive observations]

  - Source [CATT] and [Docomo] state that R2D CFO calibration signal be received before broadcast information for better detection/decoding of PRDCH signals.

  - Source [ZTE] states that periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information, is beneficial for reliable reception of broadcast information, and also beneficial for power saving. And there is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information.

[Negative observations]

  - Source [ZTE] states that CFO calibration signal is scheduled after the synchronization signal and before the broadcast information limits the flexibility for transmission of CFO calibration signal.

  - Source [ZTE] states that the periodicity of CFO calibration signal along with broadcast information may not suitable for D2R.

  - Source [Huawei] states that it is not necessary to transmit an CFO calibration signal prior to an MIB-like transmission of the periodic block since the device may need additional delay to calibrate its LO frequency and it will potentially introduce time gap between the CFO calibration signal, which will increase the resource overhead of the system and the implementation complexity of the device.

- For CFO calibration signal transmitted after broadcast information at the end of the R2D block

[Positive observations]

  - Source [CMCC] states that the synchronization signal, CFO calibration signal, and MIB-like broadcast information can be transmitted as a block, with CFO calibration signal immediately after the synchronization signal or the MIB part, AIoT device can first search the periodic synchronization signal for time synchronization and frequency acquisition due to it is robust to CFO, and then receive the CFO calibration signal.

  - Source [Huawei] states that periodic CFO calibration signals can be transmitted immediately after a periodic synchronization signal and MIB-like part, this enable the device calibrated its LO on receiving the CFO calibration signal initially along with the synchronization signal.

  - Source [vivo] supports CFO calibration signal transmitted at the end of the R2D block with broadcast information (MIB-like or SIB1-like), ensuring stable calibration accuracy, and it provides common reference accessible to all devices, and resource overhead is minimized.

  - Source [NEC] and [Docomo] state that the CFO calibration is transmitted along and immediately after broadcast information is beneficial to D2R performance.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic | Y |  |
| ETRI |  | We have provided our positive views on Option 2 in our contribution with proposal 1 & 2. Therefore, we would like to update the TP as follows. Option 2: In relation to the data payload of the PRDCH - For CFO calibration signal after data payload of PRDCH at the end of a R2D transmission [Positive observations] - Source [Samsung] states that the CFO calibration signal be placed right after the R2D data payload. This allows devices not requiring calibration to ignore the signal and enter sleep earlier, while avoiding interference with ongoing payload decoding. Besides, the impact of padding can be further studied. - Source [Huawei], [Samsung] and [ETRI] state that an aperiodic CFO calibration signal transmitted after the PRDCH (e.g., paging message), with its presence indicated by L1 control. - Source [TCL] states that placing the calibration signal after receiving the PRDCH has minor impact on timeliness and does not delay acquisition of control info. - Source [vivo] observes that interruption of R2D transmission can be avoided if CFO calibration signal is transmitted at the end of the R2D block (after the data payload). - Source [Qualcomm] supports aperiodic LO calibration using CFO calibration signal transmitted together with the PRDCH. - Source [Sequans] and [ETRI] state that it is efficient to include CFO calibration signal together with PRDCH transmission. - Source [Lenovo], [NEC], [Spreadtrum] and [Xiaomi] states that it is beneficial to place the CFO calibration signal after the PRDCH to improve performance of PDRCH. |
| LGE | Y |  |
| Futurewei |  | Editorial For the time location of the CFO calibration signal, capture following in TR38.769 Option 1: In relation to the L1 R2D control information - For CFO calibration signal preceding L1 R2D control information Preceeding preceding several times benifical -> beneficial several times provices -> provides several times |
| NTT Docomo |  | We would like to add following observation in our Tdoc to the TP. Option1 positive For CFO calibration signal preceeding L1 R2D control information - Source [Docomo] states that CFO calibration signal transmitted before L1 R2D control can be applied for CFO calibration for L1 R2D control (if needed), PRDCH (if needed), aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift. Option2 positive For CFO calibration signal after data payload of PRDCH at the end of a R2D transmission - Source [Docomo] states that CFO calibration signal transmitted after PRDCH can be applied for CFO calibration for aperiodic D2R Tx triggered by the R2D, and first X occasions of periodic D2R Tx triggered by the R2D where X depending on periodicity and CFO drift. Option3 positive. The following observations are made assuming the CFO calibration signal transmitted prior to the PDRCH (D2R transmission) - Source [ZTE] and [LGE] and [Docomo] states that the periodic CFO calibration signal is transmitted before PDRCH/DO-A transmission, which is beneficial for ensuring frequency accuracy for D2R transmissions. Option4 positive For CFO calibration signal transmitted after the synchronization signal in a R2D block Source [Docomo] states that CFO calibration signal is transmitted after synchronization signal and can be applied for CFO calibration for broadcast information Rx (if needed), aperiodic D2R Tx and periodic D2R Tx. |
| Huawei, HiSilicon | Y |  |
| CATT | N | One clarification: we don’t want to support Option 5, please remove CATT from the group of [Positive observations]of Option 5. |
| FL |  | Update based on reply from ETRI, Futurewei, Docomo and CATT |

Issue 4.1-4 Regarding other details of CFO calibration signal

Previous agreements also says that other details for CFO calibration signal can be studied.

Agreement For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects: - Whether the signal is - Option a: unmodulated sinusoid single tone - Option b: unmodulated multiple sinusoid single tones - Option c: modulated multiple tones - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported) - Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically) - The time location(s) of CFO calibration signal. - Option 1: in relation to the L1 R2D control information - Option 2: in relation to the data payload of the PRDCH - Option 3: in relation to the PDRCH - Option 4: in relation to the synchronization signal (if supported) - Option 5: in relation to broadcast information (if supported) - Combination of Options is not precluded - Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Company inputs on CFO calibration signal are summarized as follows.

Periodicty of CFO calibration signal

CMCC states that, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

ZTE states that, the periodicity of CFO calibration signal should be same as or multiple of the periodicity of DO-A resources.

DOCOMO states that, considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient.

Xiaomi states that for the purpose of power saving at reader side, a large periodicity can be considered for CFO calibration signal transmission.

Sequans states that, support periodic transmission with sparse periodicity for CFO calibration signal.

Frequency location of the CFO calibration signal

NEC state that exact frequency of CFO calibration signal may need to be explicitly indicated from reader and then determined by the device.

vivo state that in predefined frequency location within the transmission BW of the associated R2D transmission, e.g., center frequency of the periodic sync signal (AIoT-SSB) or one subcarrier adjacent to the center frequency of AIoT-SSB.

Samsung states that, it is preferred that frequency positions of the CFO calibration signal start with same frequency domain resource as of R2D transmission.

Lenovo suggest to study the following frequency-allocation alternatives for the CFO calibration signal:

Alt 1. Transmit the CFO calibration signal on the initial frequency used to provide initial access to A-IoT devices.

Alt 2. Transmit the CFO calibration signal on the frequency that provides resources for D2R data payload transmission.

Triggering method for CFO calibration signal

Huawei states that the presence of CFO calibration signal can be indicated e.g., in L1 control signal for flexibility and overhead reduction.

Samsung states that one possible approach is to make the presence of the CFO calibration signal dependent on the R2D message type or on an explicit indicator within the PRDCH.

Xiaomi states that additional on demand CFO calibration signal can be also considered.

FL notes

- Previous agreements also indicate that other details for CFO calibration signal can be studied. Hence, FL suggest to capture following TP to reflect company views in TR 38.769.

[TP][N] FL proposal 4.1-4

FL proposal 4.1-4-v1

Regarding other details of CFO calibration signal, capture following in TR 38.769

[Periodicty]

For Periodicty of CFO calibration signal

Source [CMCC] states that, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point.

Source [ZTE] states that, the periodicity of CFO calibration signal should be same as or multiple of the periodicity of DO-A resources.

Source [DOCOMO] states that, considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient.

Source [Xiaomi] states that for the purpose of power saving at reader side, a large periodicity can be considered for CFO calibration signal transmission.

Source [Sequans] states that, support periodic transmission with sparse periodicity for CFO calibration signal.

[Frequency location]

For Frequency location of the CFO calibration signal

Source [NEC] state that exact frequency of CFO calibration signal may need to be explicitly indicated from reader and then determined by the device.

Source [vivo] state that in predefined frequency location within the transmission BW of the associated R2D transmission, e.g., center frequency of the periodic sync signal (AIoT-SSB) or one subcarrier adjacent to the center frequency of AIoT-SSB.

Source [Samsung] states that, it is preferred that frequency positions of the CFO calibration signal start with same frequency domain resource as of R2D transmission.

Source [Lenovo] suggest to study the following frequency-allocation alternatives for the CFO calibration signal, Alt 1. Transmit on the initial frequency used to provide initial access to A-IoT devices, and Alt 2. Transmit on the frequency that provides resources for D2R data payload transmission.

[CFO calibration signal presence/absence]

Regarding CFO calibration signal presence/absence

sources [Huawei], [Xiaomi] and [Samsung] state that the presence of CFO calibration signal can be indicated e.g., in L1 control signal, for flexibility and overhead reduction. Source [Huawei] additionally state that this indication is used in the case of aperiodic CFO calibration signals.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Pls also add our views as below [CFO calibration signal presence/absence] Regarding CFO calibration signal presence/absence source [Qualcomm] states that aperiodic CFO calibration signal may be needed in addition to periodic CFO calibration to ensure the device's CFO calibration accuracy for D2R transmission. |
| Huawei, HiSilicon |  | For the observation on the presence/absence of the CFO calibration signal, we intend for this indication to be used only in the case of aperiodic CFO calibration signals. Hence, we would like to make the following update: Regarding CFO calibration signal presence/absence sources [Huawei], [Xiaomi] and [Samsung] state that the presence of CFO calibration signal can be indicated e.g., in L1 control signal, for flexibility and overhead reduction. Source [Huawei] additionally state that this indication is used in the case of aperiodic CFO calibration signals. |
| CATT | N | For the first part of this proposal, we prefer the following update: [Periodicity and aperiodic CFO calibration signal] For Periodicty of and aperiodic CFO calibration signal Source [CMCC] states that, the periodicity of the CFO calibration signal can be the same as the periodic synchronization signal, i.e., use 160 ms as a starting point. Source [ZTE] states that, the periodicity of CFO calibration signal should be same as or multiple of the periodicity of DO-A resources. Source [DOCOMO] states that, considering low CFO drift rate, i.e., 1ppm/s or 0.1 ppm/s, and hundreds of ms periodicity, CFO calibration via periodic CFO calibration signal can be sufficient. Source [Xiaomi] states that for the purpose of power saving at reader side, a large periodicity can be considered for CFO calibration signal transmission. Source [Sequans] states that, support periodic transmission with sparse periodicity for CFO calibration signal. Source [CATT] states both periodic and aperiodic CFO calibration signals should be supported. |
| FL |  | Updated based on reply from Huawei. Views from Qualcomm is related to whether to support periodic CFO calibration signal or Aperiodic CFO calibration signal, and I add Qualcomm’s view to Proposal 4.1-2. Hence, not updated in this TP. Views from CATT has already been captured in TR since last meeting, and can be found in proposal 4.1-2. Hence, not updated in this TP. |

  - R2D signal design for start indication

Following agreements are made in previous meeting.

Agreement For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP: - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence). - Option 2: Reuse the Rel-19 SIP. Agreement For the study of R2D binary sequence-based SIP, study the following aspects: - Sequence type - Option 1: m-sequence - Option 2: Golay sequence - Length of sequence - Number of sequence(s) - FFS: whether to apply Manchester coding to the sequence - Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

Issue 4.2-1 Binary sequence-based SIP vs Reuse the Rel-19 SIP

Regarding binary sequence-based SIP vs Reuse the Rel-19 SIP, companies are continuing the discussion in their contributions in this meeting.

8 companies support sequence-based SIP, and not support Rel-19 SIP

vivo states that

- Using Rel-19 SIP detection at AIoT device cannot provide power saving benefits.

  - In R19, the reason of low power consumption for SIP detection originated from the inherent low power consumption for device 1, in which there is no LNA, no mixer, no IF Amplifier, no IF-filter, and no BB amplifier in envelop detector, thus the SIP can be detection with low power consumption.

  - For R20 Active device, only (Z)IF-ED receiver is considered, in which LNA, mixer, IF amplifier, IF filters and BB amplifiers are enabled in detector of active device, and these components are most power consuming components in receiver. Device should enable all these power-hungry components to obtain the transition edge for Rel-19 SIP detection.

- Complexity of sequence correlation in binary-sequence based detection is limited,

  - The multiplications in sequence correlation are simplified to add operations thanks to binary sequence.

- R19 SIP cannot achieve coverage of 500 meters in UMa Scenario

  - MDR of Rel-19 SIP using edge detection method (with LNA, mixer, IF amplifier/filter, BB amplifier enabled, no power saving benefits) is not even close to meet coverage of 500 meters at 10% MDR.

  - Even if sequence correlation is used for Rel-19 SIP detection, the coverage of 500meters cannot be met at the 10% MDR when BS Tx power is 33dBm.

Xiaomi states that

- Sequence-based SIP outperforms Rel-19 SIP with ~6 dB gain at BLER of 10% and ~3dB gain at BLER of 1%.

- For the functionality of indicating the start of the R2D transmission that contains PRDCH, option 1 can be studied.

  - Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence).

NEC states that, the Rel-19 SIP design may be no longer feasible since Rel-20 A-IoT is target for outdoor scenario different from Rel-19, a sequence-based SIP should be designed.

Nokia states that, RAN1 captures that binary sequence-based SIP is beneficial and necessary to provide better coverage performance for device 2b/C.

Apple states that Binary sequence-based (m-sequence or Golay, length 15 or 31)

Sharp states that, Device 2b/C does not erroneously detect R2D transmissions for Device 1. Option 2 above should not be supported. Support Option 1: binary sequence-based SIP

interDigital states that, use of Rel-19 SIP for outdoor coverage can impair Rx sensitivity due to high probability of missed transmission detection

Sony states that

- There is a need to enhance the preamble design so that its functionality fulfills the requirement for outdoor scenarios as well as being suitable for devices with better reception capability as in device 2b and C.

- Binary sequence-based SIP, m-sequence or Golay sequence, has an advantage over release 19 SIP design as it can be designed such that it can also provide clock acquisition and is used for synchronization purposes, allowing for a one-part low-complexity preamble design.

- Such a design allows for a lower complexity of the preamble design and lower processing and therefore lower power consumption at the device side.

3 companies support reusing Rel-19 SIP

CATT states that

- For binary sequence-based SIP, the insertion of CP in the binary sequence will seriously affect the detection performance of SIP.

- Energy/edge detection-based Rel-19 SIP can significantly reduce the complexity and power consumptions of SIP detection, compared with sequence-based SIP. Moreover, reusing Rel-19 SIP will not introduce the CP handling issue.

- R-TAS SIP defined in Rel-19 A-IoT can satisfy the detection requirements of target MDR of 1% for the FAR up to 1% in outdoor scenarios.

Spreadtrum states that the Rel-19 SIP can be reused in Rel-20 without any enhancement. Sequence-based design will increase the complexity for device to perform coherent detection.

Sequans states that it is reasonable to expect equal or better detection performance by re-using the existing SIP design, considering that Rel-20 Device 2b/C types will feature improved oscillator stability and processing capability.

FL notes

- Following update TP is provided to reflect company view.

[TP][U] FL Proposal 4.2-1

FL Proposal 4.2-1-v1

Update TR 38.769 for R2D signal for start indication.

6A.1.x R2D timing 6A.1.x.y Start of R2D [omit unchanged part] Option 1 was found to be feasible and it can be used for timing synchronization/tracking. - Source [Samsung] states, due to CFO/SFO drift, sequence-based SIP may not always achieve ideal detection performance. - Source [CATT] states for Option 1, the CP handling issue should be considered for the design of R-TAS SIP. The insertion of CP in the binary sequence will seriously affect the detection performance of binary sequence-based SIP if the CP is not removed before correlation-based SIP detector. When the device detects the binary sequence-based SIP, it does not know the location of CP and cannot remove it. For binary sequence-based SIP, it occupies more than a dozen or even dozens of OFDM symbols, it will seriously affects the detection performance of the binary sequence without CP removal and leading to a degradation in the detection performance of the start of the R2D transmission. Option 2: A SIP as per TS 38.291 [228] - Source [R1-10321-123-9, CATT] [CATT] provides evaluation results show that R-TAS SIP defined in Rel-19 A-IoT can satisfy the detection requirements of target MDR of 10% for the FAR up to 1% in outdoor scenarios - Reusing Rel-19 SIP will not introduce the CP handling issue. For binary sequence-based SIP, the insertion of CP in the binary sequence will seriously affect the detection performance of SIP. - Source [Spreadtrum] states that, the Rel-19 SIP can be reused in Rel-20 without any enhancement. Sequence-based design will increase the complexity for device to perform coherent detection. - Source [Sequans] states that, it is reasonable to expect equal or better detection performance by re-using the existing SIP design, considering that Rel-20 Device 2b/C types will feature improved oscillator stability and processing capability. - Sources [R1-10321-123-3 vivo], [R1-10321-123-6 Huawei], [R1-10321-123-24 Qualcomm], [R1-10321-123-14 ZTE], [Xiaomi] and [vivo] provide evaluation results to show R19 SIP cannot be reliably detected. - Source [R1-10321-123-3, vivo] and [vivo] shows that R19 SIP does not outperform sequence-based SIP in MDR, even if R19 SIP uses is using sequence correlation detection instead of edge detection in R19. Source [vivo] show that R19 SIP cannot achieve coverage of 500 meters in UMa Scenario. MDR of Rel-19 SIP using edge detection method (with LNA, mixer, IF amplifier/filter, BB amplifier enabled, no power saving benefits) is not even close to meet coverage of 500 meters at 10% MDR. Even if sequence correlation is used for Rel-19 SIP detection, the coverage of 500meters cannot be met at the 10% MDR when BS Tx power is 33dBm. - Source [R1-10321-123-24, Qualcomm] shows that, for Rel19 SIP using energy detection (Option 2), the required CNR at BLER=10% is about 8dB higher than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control), indicating that Rel19 SIP cannot be reused and enhancement is needed. - Source [R1-10321-123-6, Huawei] shows that if the same R-TAS design is used for a device C using correlation detection, the required SNR is 17.0 dB to achieve MDR ≤ 1% and FAR ≤ 1%, which is also not sufficient for the coverage enhancement for outdoor scenarios. - Source [R1-10321-123-14, ZTE] provides evaluation results shows that the Release 19 SIP sequence performance at BLER=1% cannot satisfy the coverage requirements, while the performance at BLER=10% can satisfy the coverage requirements. - Source [Xiaomi] states that Sequence-based SIP outperforms Rel-19 SIP with ~6 dB gain at BLER of 10% and ~3dB gain at BLER of 1%. - Source [NEC], [Apple], [Sharp] and [Nokia] states that, binary sequence-based SIP is beneficial and necessary to provide better coverage performance for device 2b/C. In addition to detection performance, observations for power consumption and complexity for these two options were provided. - Source [CATT] states that energy/edge detection-based Rel-19 SIP can significantly reduce the complexity and power consumptions of SIP detection, compared with sequence-based SIP. Source [Samsung] states, sequence-based SIP may introduce increased complexity and higher power consumption compared to Rel-19 SIP. - Source [vivo] states that using Rel-19 SIP detection at AIoT device cannot provide power saving benefits. - In R19, the reason of low power consumption for SIP detection originated from the inherent low power consumption for device 1, in which there is no LNA, no mixer, no IF Amplifier, no IF-filter, and no BB amplifier in envelop detector, thus the SIP can be detection with low power consumption. - For R20 Active device, only (Z)IF-ED receiver is considered, in which LNA, mixer, IF amplifier, IF filters and BB amplifiers are enabled in detector of active device, and these components are most power consuming components in receiver. Device should enable all these power-hungry components to obtain the transition edge for Rel-19 SIP detection. - Complexity of sequence correlation in binary-sequence based detection is limited, and the multiplications in sequence correlation are simplified to add operations thanks to binary sequence. - Source [Sony] states that binary sequence-based SIP design allows for a lower complexity of the preamble design and lower processing and therefore lower power consumption at the device side - Binary sequence-based SIP, m-sequence or Golay sequence, has an advantage over release 19 SIP design as it can be designed such that it can also provide clock acquisition and is used for synchronization purposes, allowing for a one-part low-complexity preamble design.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| Samsung |  | We also prefer to reusing Rel-19 SIP, and discuss the cons of sequence-based SIP. It is preferred to be captured in TP: Source [Samsung] states, sequence-based SIP may introduce increased complexity and higher power consumption compared to Rel-19 SIP. In addition, due to CFO/SFO drift, sequence-based SIP may not always achieve ideal detection performance. |
| CATT | N | Our statement on CP handling issue for sequence-based SIP detection should also be captured in the part of Option 1 of the proposal as follows: [omit unchanged part] Option 1 was found to be feasible and it can be used for timing synchronization/tracking. Source [CATT] states for Option 1, the CP handling issue should be considered for the design of R-TAS SIP. The insertion of CP in the binary sequence will seriously affect the detection performance of binary sequence-based SIP if the CP is not removed before correlation-based SIP detector. When the device detects the binary sequence-based SIP, it does not know the location of CP and cannot remove it. For binary sequence-based SIP, it occupies more than a dozen or even dozens of OFDM symbols, it will seriously affects the detection performance of the binary sequence without CP removal and leading to a degradation in the detection performance of the start of the R2D transmission.. |
| FL |  | Update based on reply from Samsung and CATT. |

Issue 4.2-2 Sequence type/length/number

The company inputs regarding Sequence type/length/number for R2D SIP is summarized as follows.

- On Squence number

- Nokia states that, whether support for multiple sequences for sequence-based SIP is beneficial and necessary can be discussed during the WI phase.

- NEC states that, for Rel-20 R2D SIP design, a single binary sequence with single length should be defined.

- ZTE states that to maintain low decoding complexity and fast detection, support for multiple sequences for the SIP should not be considered

- LGE states that multiple sequences are needed for R2D SIP is for reader differentiation.

- Sony state that a single length is benificial for low complexity, while multiple sequence can be considered for reader differentiation, different coverage range and chip duration indication.

FL notes:

- the views from NEC has already captured in TR, add ZTE and LGE views in the TP.

- The first sentence for number of sequence for R2D SIP seems misssing in current TR, added it back.

[TP][U] FL Proposal 4.2-2

FL Proposal 4.2-2-v1

Update TR 38.769 for sequence number for binary sequence based R2D SIP, as follows

#### 6A.1.x.y Start of R2D [Omit Unchanged part] For number of sequences for binary sequence based R2D SIP - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-4, Nokia], [R1-10321-123-6, Huawei], [R1-10321-123-11, Xiaomi], [R1-10321-123-17, Panasonic], [R1-10321-123-18, Lenovo], and [R1-10321-123-24, Qualcomm] and [LGE] report that multiple sequences can be used for a given length for R2D SIP. - Sources [R1-10321-123-3, vivo], [R1-10321-123-17, Panasonic], [R1-10321-123-18, Lenovo], [Sony] and [LGE] report the purpose of multiple sequences is for reader differentiation, Sources [R1-10321-123-3, vivo] and [R1-10321-123-17, Panasonic] suggest to consider 3 sequences. - Source [R1-10321-123-6, Huawei] report that different length sequences are mapped to different coverage distances, in order to maximize the performance for each of the distances for outdoor scenarios while maintaining MDR and FAR to be ≤ 1%. Source [Sony] state that multiple sequences allow for different required coverage range. - Sources [R1-10321-123-11, Xiaomi], [R1-10321-123-18, Lenovo] report the purpose of multiple sequences is to indicate parameters of upcoming R2D transmission. [R1-10321-123-11, Xiaomi] [R1-10321-123-18, Lenovo]report that presence of CFO calibration signal can be indicated by SIP sequences, and [R1-10321-123-18, Lenovo] also report that inclusion or exclusion of L1 control information, synchronization signal, or PRDCH, can be indicated by SIP sequences. - Source [R1-10321-123-8, NEC] and [ZTE] report that single length and single sequence is beneficial for lower complexity and power consumption. - Source [ZTE] report that due to the use of OOK modulation in the downlink, multiple sequences for SIP will not reduce inter cell interference. - Source [R1-10321-123-25, NTT DOCOMO] report that device complexity on correlation-based detection for multiple sequences should be considered. [Omit Unchanged part]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| ZTE, Sanechips |  | The following update(green text) is proposed. #### 6A.1.x.y Start of R2D [Omit Unchanged part] For number of sequences for binary sequence based R2D SIP - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-4, Nokia], [R1-10321-123-6, Huawei], [R1-10321-123-11, Xiaomi], [R1-10321-123-17, Panasonic], [R1-10321-123-18, Lenovo], and [R1-10321-123-24, Qualcomm] and [LGE] report that multiple sequences can be used for a given length for R2D SIP. - Sources [R1-10321-123-3, vivo], [R1-10321-123-17, Panasonic], [R1-10321-123-18, Lenovo], [Sony] and [LGE] report the purpose of multiple sequences is for reader differentiation, Sources [R1-10321-123-3, vivo] and [R1-10321-123-17, Panasonic] suggest to consider 3 sequences. - Source [R1-10321-123-6, Huawei] report that different length sequences are mapped to different coverage distances, in order to maximize the performance for each of the distances for outdoor scenarios while maintaining MDR and FAR to be ≤ 1%. Source [Sony] state that multiple sequences allow for different required coverage range. - Sources [R1-10321-123-11, Xiaomi], [R1-10321-123-18, Lenovo] report the purpose of multiple sequences is to indicate parameters of upcoming R2D transmission. [R1-10321-123-11, Xiaomi] [R1-10321-123-18, Lenovo]report that presence of CFO calibration signal can be indicated by SIP sequences, and [R1-10321-123-18, Lenovo] also report that inclusion or exclusion of L1 control information, synchronization signal, or PRDCH, can be indicated by SIP sequences. - Source [R1-10321-123-8, NEC] and [ZTE] report that single length and single sequence is beneficial for lower complexity and power consumption. - Source [ZTE] report that due to the use of OOK modulation in the downlink, multiple sequences for SIP will not reduce inter cell interference. - Source [R1-10321-123-25, NTT DOCOMO] report that device complexity on correlation-based detection for multiple sequences should be considered. [Omit Unchanged part]. |
| CATT | N | We prefer single length and single ON-OFF pattern SIP. The following part can be added in the proposal: Source [CATT] report that single length and single ON-OFF pattern is beneficial for lower complexity and power consumption.. |
| FL |  | Updated based on ZTE reply. For comments from CATT, since this TP is about the binary sequence based SIP, not for reusing R19 SIP. And FL did not find corresponding discussion in CATT Tdoc, and the following text is found in CATT Tdoc similar to above text - For Option 2, Rel-19 SIP is reused in outdoor scenarios for Rel-20 A-IoT. Rel-19 SIP is not binary sequencebased and it is energy/edge detection-based multi-ON-OFF transmission with pre-defined duration for each of the Ons-OFFs. Hence, energy/edge detection-based Rel-19 SIP can significantly reduce the complexity and power consumptions of SIP detection, compared with sequence-based SIP. Moreover, reusing Rel-19 SIP will not introduce the CP handling issue. CATT seems discussing on benefit of reusing R19 SIP, and CATT view is already captured in Proposal 4.2-1. |

- On sequence type and length

- Xiaomi: the sequence type can be m-sequence and the sequence length can be 7.

- NEC: For Rel-20 R2D sequence-based SIP design, RAN1 can further down-selection b/w the sequence types of m-sequence and Golay sequence. Other SIP designs are not pursued during Rel-20.

- interDigital: m-sequences and Golay sequences offer comparable performance with regard to PDRCH BLER, and SFO estimation, and Prioritize the design for an m-sequence based R2D SIP.

- LGE: For the sequence type of binary sequence-based SIP, the following observations are made:

  - M-sequence enables simple implementation and ideal autocorrelation, but its poor crosscorrelation may limit applicability in multi-device environments.

  - Golay sequence achieves zero autocorrelation sidelobes and good cross-correlation, though it requires sequence pairing and involves higher implementation complexity.

  - Support R2D chip padding immediately after binary sequence-based SIP when the M value of SIP differs from that of the following R2D transmission.

- ZTE:

  - Different sequences with same length have similar performance @BLER = 0.01.

  - 16-length sequence has 2-4dB performance gain @BLER = 0.1 compared with 8-length sequence.

  - Determine the target SNR and corresponding maximum sequence length of SIP based on R20 coverage target.

FL notes:

- ZTE and Xiaomi views on sequence length are added to the TP.

- Xiaomi, NEC and ZTE views on sequence type are added to the TP.

- Some views have already been captured in TP, hence not handled.

- Some of the views may belong to WI details e.g., padding at end of SIP, hence not handled.

[TP][U] FL Proposal 4.2-3

FL Proposal 4.2-3-v1

Update TR 38.769 for sequence length for binary sequence based R2D SIP, as follows

6A.1.x.y Start of R2D [omit unchanged part] For sequence length for binary sequence based R2D SIP - Source [R1-10321-123-3, vivo] report that: - At least 31-length m-sequence with M=2 and with Manchester coding (i.e., 31 OFDM symbols), is needed for R2D SIP, assuming 38 dBm Tx power and 20 dB penetration. - At least 127-length m-sequence with M=2 and with Manchester coding (i.e., 127 OFDM symbols), is needed for R2D SIP, assuming 33 dBm Tx power and 20 dB penetration. - Source [R1-10321-123-6, Huawei] report that: - 256, 64 and 16-length Golay sequence-based SIP can achieve an SNR performance of 4.0 dB, 8.2 dB and 12.0 dB, respectively, with the MDR ≤ 1%, FAR ≤ 1% and M=1 without Manchester coding. - Source [R1-10321-123-19, Apple] report that: - Consider m-sequence based SIP with length 15 or 31 to improve detection performance while maintaining reasonable overhead - Source [R1-10321-123-24, Qualcomm] report that SIP duration with at least 7 OFDM symbols by using 15-length and M=2 without Manchester coding or 31-length and M=2 with Manchester coding may be needed to achieve required SINR for PRDCH. - Source [R1-10321-123-8, NEC] report that single length and single sequence is beneficial for lower complexity and power consumption. - Source [R1-10321-123-14, ZTE] report that the Release 19 SIP sequence with a length of 8 can achieve a BLER of less than 10% at an SNR of 4 dB, and BLER of less than 1% at an SNR of 10 dB. Source [ZTE] report that 16-length sequence has 2-4dB performance gain @BLER = 0.1 compared with 8-length sequence, and the maximum sequence length of R2D SIP is determined based on R20 coverage target. - Source [Xiaomi] report that the sequence type can be m-sequence and the sequence length can be 7. [omit unchanged part]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |

[TP][U] FL Proposal 4.2-4

FL Proposal 4.2-4-v1

Update TR 38.769 for sequence type for binary sequence based R2D SIP, as follows

6A.1.x.y Start of R2D For the functionality of indicating the start of the R2D transmission containing PRDCH, the following options are studied. Option 1: A binary sequence-based SIP, where the candidate sequence types are as follows: Option 1-1: m-sequence - Sources [R1-10321-123-3, vivo], [R1-10321-123-12, OPPO], [R1-10321-123-19, Apple], [R1-10321-123-20, InterDigital], [R1-10321-123-24, Qualcomm] and [R1-10321-123-16, LGE] [Xiaomi] and [NEC]report that m-sequence can be considered as binary sequence for R2D SIP, with the following observations: - Source [R1-10321-123-3, vivo] states that m-sequences with same length but different cyclic shifts/initial states can be considered to achieve good cross-correlation between different sequences. - Sources [R1-10321-123-20, InterDigital], [R1-10321-123-16, LGE ] and [R1-10321-123-24, Qualcomm] state that m-sequence design can be expected to be implemented with lower complexity than a Golay sequence. Source [R1-10321-123-24, Qualcomm] further state that m-sequences can be generated by storing the initialization state of the polynomial, reducing memory requirements. Option 1-2: Golay sequence - Sources [R1-10321-123-6, Huawei], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-16, LGE ], [NEC] report that Golay sequence can be considered as binary sequence for R2D SIP, with the following observations: - Source [R1-10321-123-6, Huawei] states that Golay sequence achieves lower operational and correlation complexity compared with m-sequence. - Source [R1-10321-123-25, NTT DOCOMO] states that Golay sequence with flexible sequence length may be beneficial for alignment with the OFDM symbol boundary. - Sources [R1-10321-123-24, Qualcomm] and [R1-10321-123-16, LGE ] states that Golay-sequence offers slightly better cross-correlation than m-sequence, and source [R1-10321-123-24, Qualcomm] further states cross-correlation depends on the choice of Golay pairs, and Golay sequences may be programmed into devices. - Sources [R1-10321-123-24, Qualcomm] and [R1-10321-123-20, InterDigital] provide evaluation results showing that m-sequence and Golay sequence shows similar MDR/FAR and correlation property. [R1-10321-123-25, NTT DOCOMO] states that both m-sequence and Golay sequence have good auto-correlation property. Source [ZTE] report that Different sequences with same length have similar performance @BLER = 0.01. Source [Docomo] states that the sequence of SIP should be distinguishable from other R2D in case the sequence length is short. [omit unchanged part]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| NTT Docomo |  | We would like to add following observation in our Tdoc to the TP Source [Docomo] states that the sequence of SIP should be distinguishable from other R2D in case the sequence length is short. |
| FL |  | Update based on Docomo reply |

Issue 4.2-3 whether to apply Manchester code on SIP

For R2D SIP, following FFS bullet is raised during previous meeting.

- FFS: whether to apply Manchester coding to the sequence

In last meeting, company views on whether to apply manchester code to R2D SIP is discussed and captured in TR. Companies are continuing discussing this issue in the contributions this meeting, as follows.

Huawei states that

- Golay sequence with Manchester encoding with M = 2 yield ~1 dB performance loss when compared with same length Golay sequence without Manchester encoding with M = 1.

- Golay sequence with Manchester encoding with M = 2 has very close residual timing error performance with the same length Golay sequence without Manchester encoding with M = 1.

- Removing Manchester encoding from the R2D SIP before performing correlation with theoriginal sequence will increase the complexity of the device.

Xiaomi states that for a same sequence, applying Manchester coding to the sequence-based SIP will consume twice the time domain resources to without Manchester.

LGE states that the disadvantages of applying Manchester coding include increasing complexity and decreasing the number of sequences.

ZTE states that

- 8-length sequence with Manchester coding has similar performance as 16-length sequence without Manchester coding.

- Manchester coding is not used in SIP. If correlation-based detection is used for SIP, Manchester coding provides no benefit in decoding performance and limits the selection of sequences.

FL notes

- FL suggest the following TP for TR update.

[TP][U] FL Proposal 4.2-5

FL Proposal 4.2-5-v1

Update TR 38.769 on whether to apply manchester coding to binary sequence based R2D SIP

6A.1.x.y Start of R2D [omit unchanged part] Regarding whether to apply Manchester coding to the binary sequence-based SIP, sources [R1-10321-123-3, vivo], [R1-10321-123-10, CTC], [R1-10321-123-24, Qualcomm] and [R1-10321-123-30, Sony] report that Manchester coding can be applied to binary sequence for R2D SIP - Source [R1-10321-123-3, vivo] and [R1-10321-123-24, Qualcomm] state that sequence Manchester coding achieves similar MDR and timing performance. - Source [R1-10321-123-3, vivo] provide evaluation results show that for 31-bit m-sequence with Manchester coding, 31-bit m- sequence without Manchester coding, and 63-bit without Manchester coding, these three cases have comparable durations (~15 OFDM symbols), and shows similar MDR performance. - Source [R1-10321-123-24, Qualcomm] provide evaluation results show that the 15-bit m-sequence with Manchester coding and the 31-bit m sequence without Manchester coding have comparable durations (~7.5 OFDM symbol), and both show similar MD performance. - Source [R1-10321-123-30, Sony] states that applying Manchester coding to the sequence is important in OOK modulation with low data rate where a long number of zeros and ones may result in the clock drift. - Source [R1-10321-123-3, vivo] also states that timing performance for sequence with Manchester coding is slightly better compared with longer sequence w/o Manchester coding (same overall duration), due to narrower main lobe of auto-correlation can be achieved with Manchester coding. Sources [Huawei] [Xiaomi] [LGE] and [ZTE] report that Manchester coding is not applied to binary sequence for R2D SIP - Source [Huawei] state that Golay sequence with Manchester encoding with M = 2 yield ~1 dB performance loss when compared with same length Golay sequence without Manchester encoding with M = 1; and Golay sequence with Manchester encoding with M = 2 has very close residual timing error performance with the same length Golay sequence without Manchester encoding with M = 1. - Source [Huawei] state that Removing Manchester encoding from the R2D SIP before performing correlation with theoriginal sequence will increase the complexity of the device. - Xiaomi states that for a same sequence, applying Manchester coding to the sequence-based SIP will consume twice the time domain resources to without Manchester. - ZTE states that, if correlation-based detection is used for SIP, Manchester coding provides no benefit in decoding performance and limits the selection of sequences, although 8-length sequence with Manchester coding has similar performance as 16-length sequence without Manchester coding.. - Source [R1-10321-123-1, FUTUREWEI] and [LGE] states that the disadvantages of applying Manchester coding include increasing complexity and decreasing the number of sequences. [omit unchanged part]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |

  - R2D signal for SFO calibration

In last meeting, following agreements are made for the functionality of SFO calibration.

Agreement Following update is applied to the agreement made at RAN1#122bis. Agreement For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options: - Option 1: Reuse CAP and/or Manchester encoding - Option 2: sequence-based SIP with or without R2D midamble(s) - Option 3: using the CFO calibration signal. Agreement Regarding the functionality of SFO calibration at device: capture following in TR 38.769 - It is reported by sources [Futurewei], [Spreadtrum], [vivo], [Huawei], [CTC], [CATT], [Samsung], [ZTE], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device 2b. And it is reported by sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device C. - Sources [Spreadtrum], [Huawei], [OPPO], [Samsung], [ZTE], [Apple], and [Quectel] state that the SFO calibration is feasible and can be achieved by Manchester coding for PRDCH. Additionally, source [Huawei] state that this method applies to device 2b. - Sources [CATT], [Samsung], [Apple], [Quectel] and [TCL] report that SFO is feasible and can be achieved by reusing CAP. - Sources [vivo], [NEC], [CTC] and [ZTE] report that SFO calibration is feasible and can be achieved by receiving sequence-based SIP. - Sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [OPPO], [Apple], [Qualcomm], [Quectel], [Sony] and [Docomo] report that, SFO calibration is feasible and can be achieved by CFO calibration signal. Additionally, source [Huawei] state that this method applies to device C.

In this meeting, company inputs for the functionality of SFO calibration are summarized as follows.

- Spreadtrum states that Manchester coding can be used for roughly SFO calibration, and CFO calibration signal can also be used for SFO calibration.

- Ericsson states that use the synchronization-signal to perform SFO calibration. An initial SFO calibration can be provided by the synchronization signal, which at a later stage can be complemented (i.e., strengthened) by the CAP and/or Manchester code that may be embedded in a subsequent PRDCH transmission.

- CATT states that R-TAS CAP defined in Rel-19 A-IoT and CFO calibration signal can be used for SFO estimation in outdoor scenarios for Rel-20 A-IoT.

- LG states that

- SFO calibration is necessary for device 2b and device C

- SFO is feasible and can be achieved by reusing CAP.

- SFO calibration is feasible and can be achieved by Manchester coding for PRDCH.

- SFO calibration is feasible and can be achieved by receiving sequence-based SIP.

- Samsung states that

- support Option 1: Reuse CAP and/or Manchester encoding.

- Option 2 with sequence-based SIP can be open for further discussion after SIP pattern design being agreed.

- It is preferred to remove Option 3: using CFO calibration signal.

- Apple states that using CAP and Manchester encoding to do SFO calibration.

- Quectel states that whether Option 1, Option 2 or Option 3, if supported, is utilized for SFO calibration can be determined by device’s implementation for device 2b/C.

FL notes:

- FL believes it is unnecessary to debate which options are (not) suitable for SFO calibration,

  - As all candidate options on the table serve multiple purposes, not intended for SFO calibration only, while it can be used for SFO calibration by implementation, as stated by Quectel.

- Views from Ericsson and LGE on the options for SFO calibration are added in the TP

- Views from some companies are already captured in TR, hence not handled.

[TP][U] FL Proposal 4.3-1

FL Proposal 4.3-1-v1

Update TR 38.769 on whether to apply manchester coding to binary sequence based R2D SIP

6A.1.x.y SFO calibration It is reported by sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-10, CTC], [R1-10321-123-9, CATT], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-26, TCL], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-23, Quectel] and [LGE] that SFO calibration is necessary for device 2b. And it is reported by sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-6, Huawei], [R1-10321-123-9, CATT], [R1-10321-123-26, TCL], [R1-10321-123-24, Qualcomm], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-23, Quectel] and [LGE] that SFO calibration is necessary for device C. The functionality of SFO calibration using the following options is studied: Option 1: CAP defined in TS 38.291 [228] and/or Manchester encoding. - Sources [R1-10321-123-9, CATT], [R1-10321-123-13, Samsung], [R1-10321-123-19, Apple], [R1-10321-123-23, Quectel], and [R1-10321-123-26, TCL], [LGE] and [Ericsson] report that SFO is feasible and can be achieved by reusing CAP. - Sources [R1-10321-123-2, Spreadtrum], [R1-10321-123-6, Huawei], [R1-10321-123-12, OPPO], [R1-10321-123-13, Samsung], [R1-10321-123-14, ZTE], [R1-10321-123-19, Apple], and [R1-10321-123-23, Quectel] [LGE] and [Ericsson] state that the SFO calibration is feasible and can be achieved by Manchester coding for PRDCH. Additionally, source [R1-10321-123-6, Huawei] state that this method applies to device 2b. Option 2: A sequence-based SIP. - Sources [R1-10321-123-3, vivo], [R1-10321-123-8, NEC], [R1-10321-123-10, CTC], and [R1-10321-123-14, ZTE] and [LGE] report that SFO calibration is feasible and can be achieved by receiving sequence-based SIP. Option 3: A CFO calibration signal. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-6, Huawei], [R1-10321-123-9, CATT], [R1-10321-123-12, OPPO], [R1-10321-123-19, Apple], [R1-10321-123-24, Qualcomm], [R1-10321-123-23, Quectel], [R1-10321-123-30, Sony] and [R1-10321-123-25, NTT DOCOMO] report that SFO calibration is feasible and can be achieved by CFO calibration signal. Additionally, source [R1-10321-123-6, Huawei] state that this method applies to device C.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| CATT | Y |  |

  - R2D signal design for periodic sync signal

Following agreements are made last meeting

Agreement Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking. - Alt1: periodic synchronization signal - Alt2: aperiodic synchronization signal Agreement At least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization, the periodic synchronization signal transmission is necessary. - Default transmission periodicity is predefined - FFS (in SI or WI): whether synchronization signal transmission is strictly periodic at reader Agreement R2D periodic synchronization signal is a binary sequence in time domain - The binary sequence reuses Rel-19 R2D waveform - The binary sequence has a pre-defined chip length - Device assumes that the binary sequence has a predefined length - The number of binary sequence(s) is less than or equal to 4 - If more than 1 sequence is supported, it is for potential reader identification/differentiation - Study whether Manchester coding is applied to the binary sequence - Study necessity of other aspects, including overlaid sequence

Issue 4.4-1 whether synchronization signal transmission is strictly periodic

Regarding whether synchronization signal transmission is strictly periodic at reader, company views are summarized as follows

Futurewei states that For the R2D link, do not support a requirement that synchronization signal transmission is strictly periodic at the reader.

- “Strictly periodic at the reader” imposes unnecessary scheduling constraints for the reader.

- Devices may not be able to benefit from a strictly periodic transmission

Samsung states that For the functionality of initial frequency and timing synchronization, the synchronization signal transmission is not necessary to be strictly periodic at reader.

- If other signals which can be utilized for synchronization functionality are transmitted before the expected time position of periodic synchronization signal, it is feasible to cancel/postpone the transmission of periodic synchronization signal with no noticeable performance loss.

- Based on current resource determination methods, strict periodic transmission of synchronization signal would overlap with other signals and introduces blocking/collision. It is necessary to support reader side flexibility to adjust the position of periodic synchronization signal.

- Prefer up to reader determination of resource position of periodic synchronization signal, which ensure at least one synchronization signal being transmitted within the time interval corresponding to synchronization periodicity

Huawei states that the use of a window-based periodic transmission of the synchronizations signal will have a negative impact on the device’s ability to perform the first step of initial frequency acquisition, since it cannot know when to expect these signals from the reader.

Spreadtrum states that For the reader, there is no additional cost/complexity for transmitting the sync signal periodic strictly. For the device, strictly periodic sync signal is benefit to monitor and for perform energy saving operation

Ericsson states that the sync-signal should be strictly periodic because new devices can attempt to access the reader anytime, therefore it is an unpredictable event that requires a synchronization signal to be transmitted periodically as to deal with this non-deterministic event.

CATT state that, to avoid collisions, periodic R2D synchronization signal transmission is not strictly periodic at reader. In order to prevent continuous 1s or 0s during the transmission of R2D synchronization signal and improving clock recovery, Manchester coding should be applied to the binary sequence of R2D periodic synchronization signal.

CMCC states that, at least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization, the periodic synchronization signal transmission is strictly periodic at reader.

- If the R2D synchronization signal is not strictly periodic, it will lead to ambiguity for a device to determine the timing information of the system. Without timing information, Rel-20 A-IoT system is difficult to configure RACH resources to support DO-A traffic, and DRX-like configuration for device power saving, etc.

- The periodicity of the R2D synchronization signal can be hundreds of ms, e.g., 160 ms, the overhead is quite low from network perspective.

vivo states that, the transmission occasion for periodic sync signal is strictly periodic, and it is up to reader whether to transmit or omit the periodic transmission occasion, AIoT device assumes the sync signal is periodically transmitted.

- If the time position of periodic sync signal is flexible within a transmission window(option-1), which is up to reader

  - the AIoT device have to perform blind searching within the transmission window to locate the periodic sync signal, thereby increasing the device power consumption.

  - the actual time location of detected sync signal within the transmission window should be indicated to device

- Current NR specification logic does not mandate BS to transmit sync signal(SSB) periodically, the flexibility of omit SSB transmission can be achieved by network implementation, and not forbidden by specification.

- If transmission periodicity for periodic sync signal is long enough, e.g., longer than several hundred ms, there is no clear motivation to have flexibility for BS to adjust the transmission occasion within a window or omit for periodic sync signal.

  - For time synchronization, the SIP part before each PRDCH can also be used to timing re-sync, and the functionality of timing synchronization is not totally relying on periodic sync signal, which does not require the periodic sync signal frequently transmitted.

Sony states that, for R2D synchronization signal for frequency acquisition/synchronization and timing synchronization/tracking, periodic transmission should be considered as baseline.

NEC states that, RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D sync signal. It is unnecessary to further discuss whether R2D sync signal transmission is strictly periodic at reader in RAN1, it can be totally up to reader’s implementation.

FL suggest that capture caompany views in TR as follows

[TP][N] FL Proposal 4.4-1

FL Proposal 4.4-1-v1

Regarding whether synchronization signal transmission is strictly periodic at reader, capture following in TR 38.769

Option 1: Synchronization signal transmission is strictly periodic at reader

[Positive observations]

- Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition since it cannot know when to expect the signals.

- Source [Huawei] state that if the synchronization signal is periodic with respect to the reader, it will be transmitted with a pre-defined period and the device would be able to detect it periodically, enabling it to perform the timing synchronization every time in order to access to the network and maintain the timing tracking to obtain the accurate timeline. The synchronization signal is periodic in order to ensure that the device can perform time and frequency synchronization periodically in order to be able to access the network in a timely manner.

- Source [Spreadtrum] states that for the reader, there is no additional cost/complexity for transmitting the sync signal strictly periodic, and for the device, it is beneficial for monitoring and energy-saving operations.

- Source [Ericsson] states that the sync-signal should be strictly periodic to handle new devices attempting access at any time, which is a non-deterministic event.

- Source [CMCC] states that strictly periodic transmission is necessary to avoid ambiguity in determining system timing, which is critical for RACH resources and DRX configurations. The overhead is low from the network perspective.

- Source [Sony] and [Docomo] states that periodic transmission should be considered as baseline for frequency acquisition and timing synchronization/tracking.

- Source [vivo] states that strictly periodic occasions help avoid blind searching within a transmission window, which would increase device power consumption.

- Source [Lenovo] states that R20 AIoT systems require precise synchronization, with periodic synchronization signals playing a critical role.

[Negative observations]

- Source [Futurewei] states that it imposes unnecessary scheduling constraints for the reader and devices may not benefit from it.

- Source [Samsung] states that strict periodicity could lead to overlaps with other signals and introduce blocking/collisions based on current resource determination methods.

Option 2: Synchronization signal transmission is not strictly periodic at reader (e.g., window-based or flexible)

[Positive observations]

- Source [Samsung] states that prefer up to reader determination of resource position of periodic synchronization signal, which ensure at least one synchronization signal being transmitted within the time interval corresponding to synchronization periodicity. It is necessary to support reader side flexibility to adjust the position of periodic synchronization signal. If other signals which can be utilized for synchronization functionality are transmitted before the expected time position of periodic synchronization signal, it is feasible to cancel/postpone the transmission of periodic synchronization signal with no noticeable performance loss.

- Source [CATT] and [Panasonic] states that R2D synchronization signal transmission should not be strictly periodic at reader to avoid collisions.

[Negative observations]

- Source [Huawei] states that window-based transmission impacts the device's ability to perform frequency acquisition due to timing uncertainty.

- Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition due to timing uncertainty since it cannot know when to expect the signals.

- Source [vivo] states that a flexible time position within a window requires blind searching, increasing power consumption, and requires additional indication of the actual location.

- Source [vivo] states that the flexibility to omit transmissions (while keeping occasions strictly periodic) is consistent with NR specification logic where BS is not mandated to transmit SSB periodically.

- Source [CMCC] states that if the R2D synchronization signal is not strictly periodic, it will lead to ambiguity for a device to determine the timing information of the system. Without timing information, Rel-20 A-IoT system is difficult to configure RACH resources to support DO-A traffic, and DRX-like configuration for device power saving, etc.

- Source [vivo] states that if transmission periodicity for periodic sync signal is long enough, e.g., longer than several hundred ms, there is no clear motivation to have flexibility for BS to adjust the transmission occasion within a window or omit for periodic sync signal.

Source [NEC] and [Qualcomm] state that, RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D sync signal. It is unnecessary to further discuss whether R2D sync signal transmission is strictly periodic at reader in RAN1, it can be totally up to reader’s implementation.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Pls also add our view as below: Source [NEC][Qualcomm] state that, RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D sync signal. It is unnecessary to further discuss whether R2D sync signal transmission is strictly periodic at reader in RAN1, it can be totally up to reader’s implementation. |
| Panasonic |  | In our TDoc R1-2600705, we have “Proposal 3: The synchronization signal transmission can be quasi periodic at reader if these are within the limit on variance and the device is assumed not to use the relation between synchronization signals.” Please add our view to option 2. |
|  |  | Panasonic2) As below. [Positive observations] - Source [CATT] [Panasonic] states that R2D synchronization signal transmission should not be strictly periodic at reader to avoid collisions. |
| Lenovo |  | Please add our view as below (see R1-2601020): Option 1: Synchronization signal transmission is strictly periodic at reader [Positive observations] Source [Lenovo] states that R20 AIoT systems require precise synchronization, with periodic synchronization signals playing a critical role. |
| NTT Docomo |  | We would like to add following observation in our Tdoc to the TP Option 1: Synchronization signal transmission is strictly periodic at reader [Positive observations] - Source [Sony][Docomo] states that periodic transmission should be considered as baseline for frequency acquisition and timing synchronization/tracking. |
| Huawei, HiSilicon |  | There are a few changes that need to be made based on our TDocs: Option 1: Synchronization signal transmission is strictly periodic at reader [Positive observations] - Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition since it cannot know when to expect the signals. - Source [Huawei] state that if the synchronization signal is periodic with respect to the reader, it will be transmitted with a pre-defined period and the device would be able to detect it periodically, enabling it to perform the timing synchronization every time in order to access to the network and maintain the timing tracking to obtain the accurate timeline. The synchronization signal is periodic in order to ensure that the device can perform time and frequency synchronization periodically in order to be able to access the network in a timely manner. … Option 2: Synchronization signal transmission is not strictly periodic at reader (e.g., window-based or flexible) [Positive observations] - … [Negative observations] - Source [Huawei] states that window-based transmission impacts the device's ability to perform frequency acquisition due to timing uncertainty. - Source [Huawei] states that the use of a window-based periodic transmission (not strictly periodic) will have a negative impact on the device's ability to perform initial frequency acquisition due to timing uncertainty since it cannot know when to expect the signals. |
| CATT | Y | OK |
| FL |  | Updated based on comments from Qualcomm, Panasonic, Lenovo, Docomo, and Huawei. |

Issue 4.4-2 Time domain resource for periodic synchronization signal

In contributions, companies also discuss signal/channel design for the periodic sync signal.

For periodicity and time resource for Periodic sync signal

- vivo: >=320ms <=1.28s, and short periodicity may lead to collision with other R2D transmissions.

- CMCC, Ericsson: 160ms

- Sequans: in order of 100s ms

- ZTE: A periodicity of 40ms as starting point and transmitted at a specific frame boundary.

- Docomo states that for time synchronization/tracking, additional synchronization signal without corresponding broadcast information can be beneficial for reducing accumulated time error due to SFO while avoiding additional overhead of broadcast information

FL notes:

- This TP is proposed last meeting, and it was commented by some companies that this belongs WI details and TP not needed.

- There are still 5 companies discussed the periodicity of periodic synchronization, the Text proposal is provided again.

[TP][N] FL Proposal 4.4-2

FL Proposal 4.4-2-v1

Capture following in TR 38.769

Periodicity of R2D periodic synchronization signal is studied, with following observations

- Source [vivo] states that it is beneficial to consider a long periodicity for periodic sync signal block (AIoT-SSB) to avoid collision between R2D/D2R transmissions and AIoT-SSB. The periodicity can be in between [320ms, 1.28s], and short periodicity may lead to collision with other R2D transmissions.

- Source [CMCC] states that frequency acquisition and reader identification latency, the timing drift due to SFO within the periodicity of the synchronization signal and the overhead of synchronization signal should be considered when determine the periodicity of the R2D periodic synchronization signal. And periodicity can be hundreds of ms, e.g., 160ms, as a starting point.

- Source [Ericsson] states that dense periodic synchronization signal can accelerate reader/cell search and improve timing synchronization at the cost of increased resource consumption (and energy consumption at the device side) and signaling overhead, whereas sparse transmission may reduce resource usage but require extra preamble overhead for reliable PRDCH demodulation. The periodicity can be 160ms, and other value can be studied.

- Source [Sequans] states that, periodicity in the order of 100s of ms can be considered, to support calibration for DO-A traffic case (i.e., device initiates D2R transmission after fast fix from such periodic R2D) without introducing large latency penalty.

- Source [ZTE] states that 40ms can be a starting point , and the Periodic R2D synchronization signal can be transmitted at a specific frame boundary.

- Source [Docomo] states that for time synchronization/tracking, additional synchronization signal without corresponding broadcast information can be beneficial for reducing accumulated time error due to SFO while avoiding additional overhead of broadcast information

- Source [CATT] states that the transmission period of the periodic synchronization signals should be longer than 160 ms.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| CATT | N | Our proposal in the tdoc should be added in the TP: Source [CATT] states that the transmission period of the periodic synchronization signals should be longer than 160 ms. |

Issue 4.4-3 Sequence design for Periodic sync signal

This section summarizes the input for sequence design aspects, including, sequence number, sequence length, and sequence type

Regarding sequence number

- CATT states that, the reader identification information, which indicates the identifier of the reader, should be carried by R2D synchronization signal. The number of binary sequence(s) of R2D synchronization signal should be equal to 4 for reader identification.

- ZTE states that, the blind decoding using multiple 8-length sequences has ~2dB performance loss compared with single-sequence detection. The necessity of synchronization signal with multiple sequences needs to be discussed

- vivo states that, it is benificial to consider m-sequence with ~3 sequences for reader differentiation

- Docomo states that, for the number of sequences of R2D synchronization signal for frequency synchronization, device complexity on correlation-based detection should be considered assuming that device performs correlation-based detection to detect the signal on multiple candidate frequencies at least for initial frequency acquisition.

Regarding sequence type

- vivo states that m sequence can be considered as sequence type for periodic sync signal.

- Ericsson states a set of unique patterns of the binary sequences can be used for periodic sync signal

- Apple states that M-sequence with length 31 should be considered as baseline for periodic synchronization signal, with Manchester coding applied for SFO tracking.

- Docomo states that, for sequence type of R2D synchronization signal for frequency synchronization, m sequence or Golay sequence with good auto-correlation property should be considered.

Regarding sequence length

vivo states that, for sequence length of periodic sync sequence, to achieve target coverage for outdoor scenario.

- The m-sequence with length 31 with Manchester coding using M=2, occupied 31 OFDM symbols can be considered for periodic sync sequence, assuming 38 dBm Tx power and 20 dB penetration.

- The m-sequence with length 127 with Manchester coding using M=2, occupied 127 OFDM symbols can be considered for periodic sync sequence, assuming 33 dBm Tx power and 20 dB penetration.

Qulcomm states that, for R2D synchronization signal (SS), Device 2b requires a longer length than Device C because of its larger initial SFO/CFO.

- For Device C with initial SFO/CFO of 50ppm

  - The SS sequence with 7.5 symbols (with 63-length m-sequence and M=2) can achieve MD of 10% at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control).

- For Device 2b with an initial SFO/CFO of 1000ppm,

  - The SS sequence with 31.5 symbols (with 15-length m-sequence and M=2) can achieve MD of 10% at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control).

Besides, distinguish between sequence for R2D periodic synchronization signal and binary sequence for R2D SIP are also discussed

- NEC states that whether the sequence of the newly introduced R2D sync signal and R2D SIP is same or not should be considered.

- vivo states that the m-sequence should be differentiated from that used for R2D SIP for PRDCH.

FL notes

- For sequence number for periodic synchronization signal, we already have agreements as SI outcome, hence no further observation is needed

  - “The number of binary sequence(s) is less than or equal to 4”, and “If more than 1 sequence is supported, it is for potential reader identification/differentiation”

- For sequence length and sequence, the TP is provided for TR capturing, as follows.

[TP][N] FL Proposal 4.4-3

FL Proposal 4.4-3-v1

For sequence design for periodic synchronization signal, capture the following in TR38.769

Regarding sequence type for periodic synchronization signal

- Source [vivo] and [CATT] state, that m sequence can be considered as sequence type for periodic sync signal.

- Source [Ericsson] states, a set of unique patterns of the binary sequences can be used for periodic sync signal

- Source [Apple] states that, M-sequence with length 31 should be considered as baseline for periodic synchronization signal, with Manchester coding applied for SFO tracking.

- Source [Docomo] states that, for sequence type of R2D synchronization signal for frequency synchronization, m sequence or Golay sequence with good auto-correlation property should be considered.

- Source [R1-10321-123-6, Huawei] state that the periodic synchronization signal is binary sequence-based, using a Golay sequence with good auto-correlation characteristic and low correlation complexity, with a fixed maximum length, in order to reduce the blind detection effort by the device for the time and frequency synchronization.

Regarding sequence length for periodic synchronization signal

- Source [vivo] states that, for sequence length of periodic sync sequence, to achieve target coverage for outdoor scenario.

  - The m-sequence with length 31 with Manchester coding using M=2, occupied 31 OFDM symbols can be considered for periodic sync sequence, assuming 38 dBm Tx power and 20 dB penetration.

  - The m-sequence with length 127 with Manchester coding using M=2, occupied 127 OFDM symbols can be considered for periodic sync sequence, assuming 33 dBm Tx power and 20 dB penetration.

- Source [Qualcomm] states that, for R2D synchronization signal (SS), Device 2b requires a longer length than Device C because of its larger initial SFO/CFO.

  - For Device C with initial SFO/CFO of 50ppm

    - The SS sequence with 7.5 symbols (with 63-length m-sequence and M=2) can achieve MD of 10% at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control).

  - For Device 2b with an initial SFO/CFO of 1000ppm,

    - The SS sequence with 31.5 symbols (with 15-length m-sequence and M=2) can achieve MD of 10% at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control).

- Source [R1-10321-123-6, Huawei] state that the periodic synchronization signal should be the maximum length e.g., 256 for the binary sequence to guarantee all the devices with the maximum coverage distance can access the network.

Regarding sequence number for periodic synchronization signal

- Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-7, Ericsson], [R1-10321-123-9, CATT] and [R1-10321-123-24, Qualcomm] report that a predefined number, e.g., 3~4 sequences can be considered for reader differentiation

  - Source [CATT] states that, the reader identification information, which indicates the identifier of the reader, should be carried by R2D synchronization signal. The number of binary sequence(s) of R2D synchronization signal should be equal to 4 for reader identification.

  - Source [vivo] states that, it is benificial to consider m-sequence with ~3 sequences for reader differentiation

- Sources [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-5, CMCC] and [R1-10321-123-21, MTK] report that Single sequence can be considered.

  - Source [Huawei] further state that if multiple sequences of the periodic synchronization signal are used, it will increase the complexity and power consumption of the device.

- Source [ZTE] states that, the blind decoding using multiple 8-length sequences has ~2dB performance loss compared with single-sequence detection. The necessity of synchronization signal with multiple sequences needs to be discussed.

- Source [Docomo] states that, for the number of sequences of R2D synchronization signal for frequency synchronization, device complexity on correlation-based detection should be considered assuming that device performs correlation-based detection to detect the signal on multiple candidate frequencies at least for initial frequency acquisition.

[Sequence number]

Regarding sequence number for periodic synchronization signal, both single sequence and multiple sequences are discussed,

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We would like to add our company view based on our TDoc from the previous meeting: Regarding sequence type for periodic synchronization signal Source [Huawei] state that the periodic synchronization signal is binary sequence-based, using a Golay sequence with good auto-correlation characteristic and low correlation complexity, with a fixed maximum length, in order to reduce the blind detection effort by the device for the time and frequency synchronization. Regarding sequence length for periodic synchronization signal Source [Huawei] state that the periodic synchronization signal should be the maximum length e.g., 256 for the binary sequence to guarantee all the devices with the maximum coverage distance can access the network. |
| ZTE, Sanechips |  | We are wondering how to capture our observation. ZTE states that, the blind decoding using multiple 8-length sequences has ~2dB performance loss compared with single-sequence detection. The necessity of synchronization signal with multiple sequences needs to be discussed. |
| CATT | N | Our proposal in the tdoc should be added in the TP: Source [CATT] states that m sequence should be adopted as sequence type for periodic sync signal. |
| FL |  | Update based on Huawei and CATT reply ZTE suggest to also capture views on number of sequences although we already have agreements on up to 4 sequences for periodic sync signal, hence I add one more paragragh to reflect company views. Considering more companies provide their views in the last meeting, the new TP also includes input from the last meeting. |

Issue 4.4-4 Whether to apply manchester coding to periodic synchronization signal

Huawei, Ericsson, OPPO, CATT, vivo, Qualcomm provided views on whether to apply manchester coding to periodic synchronization signal, the views are asummarized as follows.

| View | Sources | Technical Justification |
| --- | --- | --- |
| Apply Manchester coding | OPPO, CATT | - Ensures continuous clock embedding - Mitigates timing drift - Reduces DC offset issues - Facilitates accurate signal detection with lower complexity - Prevents continuous 1s or 0s during transmission - Improves clock recovery |
| Not apply Manchester coding | Huawei | - Results in marginal performance loss - Similar residual timing error performance as without coding - Increases device complexity (requires Manchester decodingbefore correlation with original sequence) |
| Neutral | Ericsson | - Requires verification/evaluation whether it outperforms binarysequences with good auto-correlation and cross-correlationproperties |
| Evaluation | vivo | - Comparable MDR performance among 31-bit m-sequencewith/without Manchester and 63-bit without (similar ~15 OFDMsymbols duration) - Timing performance slightly better with Manchester due tonarrower main lobe of auto-correlation |
| Mixed | Qualcomm | - No major impact on miss detection and timing performance - Prevents consecutive 1s or 0s - Could lead to increased false alarm rate due toresemblance to PRDCH |

FL Notes

- Views are split, and FL suggest the following TP for TR capturing.

[TP][N] FL Proposal 4.4-4

FL Proposal 4.4-4-v1：

Capture the following in TR38.769

Regarding whether to apply manchester coding to periodic synchronization signal

[Negative to apply manchester coding]

- Source [Huawei] states that, applying Manchester coding to the sequence-based synchronization signal would result in a marginal performance loss with similar residual timing error performance, while increasing the complexity of the device if it is expected to remove the Manchester encoding before performing correlation with the original sequence.

- Source [Qualcomm] states that, applying Manchester coding to synchronization signal could lead to an increased false alarm rate due to its resemblance to the PRDCH.

[FFS to apply manchester coding]

- Source [Ericsson] states that, before applying Manchester coding to the binary sequence, it needs to be verified/evaluated whether it outperforms or not the performance of a binary sequence designed with good auto-correlation and cross-correlation properties.

[Positive to apply manchester coding]

- Source [OPPO] states that, applying Manchester coding to the binary sequence of synchronization signal should be considered, and the application of Manchester coding to the binary sequence ensures continuous clock embedding, which mitigates timing drift, reduces DC offset issues, and facilitates accurate signal detection with lower complexity.

- Source [CATT] states that, in order to prevent continuous 1s or 0s during the transmission of R2D synchronization signal and improving clock recovery, Manchester coding should be applied to the binary sequence of R2D periodic synchronization signal.

- Source [vivo] provide evaluation results show that for 31-bit m-sequence with Manchester coding, 31-bit m- sequence without Manchester coding, and 63-bit without Manchester coding, these three cases have comparable durations (~15 OFDM symbols), and shows similar MDR performance.Besides, timing performance for sequence with Manchester coding is slightly better compared with longer sequence w/o Manchester coding (same overall duration), due to narrower main lobe of auto-correlation can be achieved with Manchester coding.

- Source [Qualcomm] states that, there is no major impact on miss detection and timing performance if Manchester coding to synchronization signal, while this approach helps prevent consecutive 1s or 0s, it could also lead to an increased false alarm rate due to its resemblance to the PRDCH.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Our views for pros/cons can be captured separately. [Negative to apply manchester coding] - Source [Qualcomm] states that, applying Manchester coding to synchronization signal could lead to an increased false alarm rate due to its resemblance to the PRDCH. [Positive to apply manchester coding] - Source [Qualcomm] states that, there is no major impact on miss detection and timing performance if Manchester coding to synchronization signal, while this approach helps prevent consecutive 1s or 0s. |
| LGE |  | We have a similar view as QC. When Manchester coding is applied to the synchronization signal, it may increase the false alarm rate since a Manchester-encoded sequence may not be distinguishable from the PRDCH waveform. |
| Huawei, HiSilicon | Y |  |
| FL |  | Update based on Qualcomm’s view |

Issue 4.4-5 Overlaid sequence to periodic synchronization signal

Regarding whether to apply overlaid sequence to periodic synchronization signal,

OPPO states that whether to apply overlaid sequence, as a device cannot detect any information conveyed by the overlaid sequence, whether it is applied or not should be transparent to device and up to reader implementation.

Ericsson states that an “overlaid” design is not deemed as essential as the previous bullets therefore it can be down prioritized.

NEC does not think overlaid sequence can be decoded by the A-IoT devices which will exceed the capability of such a low-end type of device

vivo states that overlaid sequence on sequence of periodic sync signal does not bring benefit for MDR performance and timing accuracy, which is different from that in LP-WUS

- Evaluation results are provided shows that overlaid sequence on sequence of periodic sync signal does not bring benefit for MDR performance and timing accuracy

- The transmission bandwidth of LP-WUS is 5MHz, and frequency diversity can be obtained by applying overlaid sequence. However, considering Ambient IoT with 1PRB Tx BW as baseline, no frequency diversity gain can be achieved even if overlaid sequence is applied.

- The overlaid sequence will introduce more ripples for the ON/OFF states in time domain waveform, and lead to lower time synchronization accuracy compared with that using OOK binary sequence without applying overlaid sequence.

- If overlaid sequence is introduced, RAN4 requirements on OOK waveform quality for periodic R2D sync signal with overlaid sequence should be separately defined, which increase the workload

Qualcomm states that it is beneficial to be achieve frequency diversity and robust again fast fading by using larger transmission bandwidth for transmission.

Company views are summarized as follows

1. Performance Impact: vivo's evaluation shows no benefit for MDR performance and timing accuracy when applying overlaid sequence to periodic sync signal, contrasting with LP-WUS where it provides advantages.

2. Fundamental Difference from LP-WUS:

- LP-WUS: 5MHz transmission bandwidth allows frequency diversity gain from overlaid sequence

- Ambient IoT: 1PRB baseline transmission bandwidth means no frequency diversity gain even with overlaid sequence

3. Technical Drawbacks:

- Degraded time synchronization accuracy due to increased waveform ripples (vivo)

- Decoding complexity exceeds low-end A-IoT device capability (NEC)

- Additional RAN4 specification workload for waveform quality requirements (vivo)

4. Majority view: (NEC, vivo, Ericsson, OPPO) indicates not to apply overlaid sequence to periodic sync signal, either due to technical limitations or non-essential priority.

FL notes

- FL suggest to follow majority views, and overlaid sequence on periodic synchronization signal is not consider for R20 AIoT.

[H] FL Proposal 4.4-5

FL Proposal 4.4-5-v1

Overlaid sequence on R2D periodic synchronization signal is not consider for R20 AIoT.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| [OPPO] | Y |  |
| Spreadtrum | Y |  |
| Panasonic | Y |  |
| LGE | Y |  |
| Lenovo | Y |  |
| Samsung | Y |  |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |
| CATT | Y |  |
| NEC | Y |  |

Issue 4.4-6 Aperiodic synchronization signal

In last neeting, it is agreed that periodic synchronization signal with a default pre-defined transmission periodicity is neessary at least for the functionality of frequency synchronization, including frequency acquisition, and initial timing synchronization.

In this meeting, following companies suggest to also consider aperiodic synchronization signal, as follows

- ZTE states that aperiodic R2D synchronization signal can also be considered to increase the synchronization accuracy for DO-A traffic transmission.

- CATT states that Both periodic and aperiodic R2D synchronization signals could be supported with the presumption that the A-IoT devices do not require to maintain synchronization with the reader in Rel-20 to support A-IoT device synchronization to the reader.

FL Notes

- Views from CATT and ZTE are added in the TP below.

[TP][U] FL Proposal 4.4-6

FL Proposal 4.4-6-v1

Update TR 38.769 on periodic synchronization signal and aperiodic synchronization signal

6A.1.x.y Synchronization signal A periodic synchronization signal with a default pre-defined transmission periodicity is neessary at least for the functionality of frequency synchronization, including frequency acquisition, and initial timing synchronization. It is a binary signal in the time domain, reusing the Rel-19 waveform and having a pre-defined chip length. The number of binary sequence(s) is less-than-or-equal to 4 and if more than 1 sequence is supported, the purpose of doing so is for potential reader identification/differentiation. A device assumes that the binary sequence has a pre-defined length. Inaddition to periodic synchronization signal, - Source [ZTE] states that aperiodic R2D synchronization signal can also be considered to increase the synchronization accuracy for DO-A traffic transmission. - Source [CATT] and [Panasonic] states that Both periodic and aperiodic R2D synchronization signals could be supported with the presumption that the A-IoT devices do not require to maintain synchronization with the reader in Rel-20 to support A-IoT device synchronization to the reader.

| Company | Y/N | Comments |
| --- | --- | --- |
| Panasonic |  | In our TDoc R1-2600705, we have “Proposal 4: CFO calibration signal can be transmitted as a combination of periodic and aperiodic signal to improve the system performance for a A-IoT system” and “Proposal 5: The aperiodic signal can be together with other signal(s) / channel(s), and can be common or device specific”. Please add our view as below: Source [CATT] [Panasonic] states that Both periodic and aperiodic R2D synchronization signals could be supported with the presumption that the A-IoT devices do not require to maintain synchronization with the reader in Rel-20 to support A-IoT device synchronization to the reader |
| FL |  | Update based on reply from Panasonic |

  - How to transmit Broadcast information

Agreement For R2D, study at least the following aspects for the study of broadcast information: - Information included in the broadcast information - Whether to use PRDCH to carry the broadcast information - Transmission of broadcast information: - Option 1: periodic - Option 2: aperiodic - Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal. Agreement - The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception - On how A-IoT devices acquire the broadcast information, study the following two cases: - Case B1: Single-step acquisition of the broadcast information - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information) - On whether to use PRDCH to carry the broadcast information, - For Case B1, PRDCH is used to carry the broadcast information. - For Case B2 - PRDCH is used to carry the SIB1-like broadcast information. - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information Agreement Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C. - FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader - FFS (in SI or WI): periodicity of the transmission of broadcast information

Regarding whether to use PRDCH to carry the broadcast information, it will be handled in AI 9.3.2.3.

Issue# 4.5-1: Time/Frequency resources for broadcast information

For time location of broadcast information, companies provided views on the relative timing between synchronization signals and broadcast information, for MIB-like and SIB1-like broadcast information separately.

- Immediate after periodic sync signal vs. separated timing location.

  - Immediate: Majority companies (including Huawei, vivo, Xiaomi, and Apple) support transmission immediately after the periodic sync signal to facilitate quick device identification and simplify implementation. vivo proposes a continuous "AIoT-SSB" block containing sync sequences and broadcast data.

  - Separated transmission or with time gap: Ericsson, Samsung, and others suggest that broadcast information can have a different periodicity or a fixed gap/offset from the sync signal, potentially with L1 control signaling in between.

- For SIB1-like Information, Huawei, CMCC, and vivo suggest that time resource of SIB1-like resources can be indicated within the MIB-like part, scheduled via L1 control, or fixed relative to the sync signal.

For whether strictly periodic transmission of broadcast information, the debate centers on the balance between regularity and flexibility.

- Periodic Baseline: Periodic transmission is generally considered the baseline (vivo, NTT DOCOMO). While Samsung prefer not to limit the broadcast information being strictly periodic at reader side.

- For transmission periodicity, NTT DOCOMO suggests a periodicity of up to a few hundred milliseconds is feasible, depending on energy storage assumptions.

For frequency Location of Broadcast Information,

- Same frequency: Most companies (including FUTUREWEI, Huawei, Ericsson and OPPO) support using the same frequency resource for broadcast information as the sync signal, as the device is already tuned to that raster. vivo highlights the need for a narrow bandwidth (e.g., within 1 RB).

- Alternatives: Samsung suggests frequency could be derived from the sync signal or use FDM. For SIB1-like information, Huawei suggests the frequency location can be indicated in the MIB or follow a fixed relationship.

FL Notes

- Based on company inputs, FL provide following TP which reflects company views for TR capturing.

[TP][N] FL Proposal 4.5-1

FL Proposal 4.5-1-v1

Capture following in TR38.769 regarding time and frequency resources for broadcast information:

Time and frequency resources for broadcast information is studied, and the following observations are provided.

For time location of broadcast information

[immediately after periodic synchronization signal]

Source [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [vivo], [Apple], [CMCC], [ETRI], [Samsung], [Lenovo], [ZTE] and [Docomo] state that broadcast information can be transmitted immediately after periodic synchronization signal.

- Source [Huawei], [Spreadtrum] and [vivo] states that this ensures the device can obtain system information (e.g., reader ID) as part of the reader identification/differentiation procedure. Source [Huawei] and [vivo] state that this applies to MIB-like broadcast information, and [CMCC] state that this can be one potential option.

- Source [Xiaomi] states that the broadcast information can be transmitted at the end of the periodic synchronization signal with fixed time domain resources to simplify device implementation.

- Source [vivo] state that it is beneficial to support periodic R2D sync signal block, aka., AIoT-SSB, which can be composed of periodic sync sequence, PRDCH for MIB-like broadcast information, and CFO calibration signal, and these parts are continuous in time.

- Source [ZTE] states that periodic CFO calibration signal and broadcast information is placed after different synchronization signals.

[Separated or have Gap from periodic synchronization signal]

Source [Ericsson], [Samsung], [CMCC], [ZTE] and [Docomo] states that, the broadcast information can be transmitted not immediately after periodic synchronization signal.

- Source [Ericsson] states that broadcast information can be transmitted with a different periodicity from the periodic sync signal and with pre-defined offset to the sync signal when transmitted. Source [CMCC] also state that, MIB-like part is transmitted separately with the synchronization signal

- Source [Samsung] state that time resource can be derived by periodic synchronization signal with a fixed gap.

- Source [Docomo] state that time resource of the broadcast information can be immediately after the R2D signal or with an L1 R2D control in between.

- Source [ZTE] state that periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information. There is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information.

[For SIB1-like broadcast information]

Source [Huawei], [CMCC] and [vivo] further provide views on time domain resource for SIB1-like broadcast information

- Source [Huawei] state that, for time resources to receive the SIB-like broadcast information, the start time resource information can either be indicated in the MIB-like broadcast information, or be fixed in relation to the periodic synchronization signal/MIB-like broadcast transmission.

- Source [CMCC] state that, for the SIB1-like broadcast information, the time resources can be indicated by MIB-like part or scheduled by the corresponding L1 R2D control information

- Source [vivo] state that, it is beneficial to support periodic SIB1-like transmitted separately from MIB-like broadcast information. The periodicity of SIB1-like broadcast information can be equal to or larger than that for AIoT-SSB.

For whether the broadcast information transmission is strictly periodic

Source [vivo] states that the transmission occasion for broadcast information is strictly periodic.

Source [Docomo] states that periodic transmission should be considered as baseline, and aperiodic transmission can provide flexibility for reader while it may lead to additional workload, e.g., on how much time tolerance can be allowed for broadcast information transmission. Source [Docomo] further state that the periodicity of broadcast information, up to a few hundreds of ms can be feasible depending on energy storage assumption.

Source [Samsung] states that it is preferred not to limit the broadcast information being strictly periodic at reader side.

Source [Qualcomm] states that RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D broadcast information. It is unnecessary to further discuss whether R2D broadcast information is strictly periodic at reader in RAN1.

For frequency location of broadcast information

[Same frequency as the periodic synchronization signal]

Source [FUTUREWEI], [Huawei], [Spreadtrum], [Ericsson] [Xiaomi], [vivo], [OPPO], [NTT DOCOMO] [Qualcomm], [Samsung] and [Apple] state that the same frequency resource is used for broadcast information as the periodic synchronization signal.

- Source [Huawei] states that the device has only acquired the frequency synchronization of the sync raster on which it detected the periodic synchronization signal, and same frequency resource as periodic synchronization signal applies to MIB-like broadcast information.

- Source [OPPO] states that a device can assume the same center frequency for receiving both broadcast information and the periodic sync signal, as long as IF/ZIF receiver requirements are met.

- Souce [vivo] state that M value should be ≤6 for periodic sync signal and PRDCH for broadcast to ensure applicability to deployment in 1 RB Tx BW.

[Potential different frequency as the periodic synchronization signal]

Source [Samsung] states that, frequency resource for broadcast information can be same as or derived by periodic synchronization signal, and potential FDM possibility and whether transmissions occur in narrowband or wideband should be clarifyed before determining the frequency resource.

Source [Samsung] suggest to study whether system supports multiple frequency positions for sync signal (FDM among sync signals), and whether system supports multiple frequency positions for broadcast information (FDM among broadcast information), e.g. to support a raster-based search like sync/MIB acquisition.

Source [Huawei] state that, for frequency resources to receive the SIB-like broadcast information, the frequency resource information can either be indicated in the MIB-like broadcast information, or be fixed in relation to the periodic synchronization signal/MIB-like broadcast transmission.

Source [Lenovo] states that, upon synchronizing via signals broadcast on initial frequency, devices acquire system information that enumerates available frequencies for further reception/transmission.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Regarding the time location of broadcast information, whether it is immediately after periodic synchronization signal, it is not clear yet whether/where the calibration signal is transmitted relative to periodic sync signal and periodic broadcast information. Pls add our views as below: For whether the broadcast information transmission is strictly periodic Source [Qualcomm] states that RAN1 may only need to specify a default periodicity assumption from device perspective for the R2D broadcast information. It is unnecessary to further discuss whether R2D broadcast information is strictly periodic at reader in RAN1. [Same frequency as the periodic synchronization signal] Source [FUTUREWEI], [Huawei], [Spreadtrum], [Ericsson] [Xiaomi], [vivo], [OPPO], [NTT DOCOMO], [Qualcomm] and [Apple] state that the same frequency resource is used for broadcast information as the periodic synchronization signal. |
| ETRI | Y | Fine with the TP. |
| LGE |  | We also think that it is desirable for the broadcast information to be transmitted immediately after the periodic synchronization signal, as this allows the device to utilize the timing acquired from the periodic synchronization signal without requiring additional search or monitoring. Regarding frequency location of broadcast information, we also think that the broadcast information should use the same frequency location as the periodic synchronization signal, since the device achieves frequency synchronization based on the periodic synchronization signal. Using a common frequency resource avoids additional signaling overhead needed to explicitly indicate a separate frequency resource for broadcast information and simplifies device implementation. |
| Lenovo |  | Please add our view as below (see R1-2601022): For time location of broadcast information [immediately after periodic synchronization signal] Source [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [vivo], [Apple], [CMCC], [ETRI], [Samsung] and [Docomo], and [Lenovo] state that broadcast information can be transmitted immediately after periodic synchronization signal. For frequency location of broadcast information [Potential different frequency as the periodic synchronization signal] Source [Lenovo] states that, upon synchronizing via signals broadcast on initial frequency, devices acquire system information that enumerates available frequencies for further reception/transmission. |
| Samsung |  | For time location of broadcast information, we consider both immediately after sync signal as “Time resource can be derived by periodic synchronization signal, e.g., back-to-back or have a fixed gap”. It is preferred to capture our position in both [immediately after periodic synchronization signal] and [Separated or have Gap from periodic synchronization signal]. For frequency location, we would like to clarify our view. At first, we also proposed same frequency position as sync signal, so please add us to [Same frequency as the periodic synchronization signal] bullet. Regarding the FDM bullet, our view is not to propose whether sync signal is FDMed with broadcast information. Our clarification questions is that whether system supports multiple frequency positions for sync signal (FDM among sync signals), and whether system supports multiple frequency positions for broadcast information (FDM among broadcast information), e.g. to support a raster-based search like sync/MIB acquisition. We consider it would impact the frequency resource allocation of broadcast information and should be clarified at first. |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips |  | We states time location of broadcast information in our tDoc as following. For the location of periodic CFO calibration signal, the following three options can be considered: - Option#1: Periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information. There is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information. - Option#2: Periodic CFO calibration signal is scheduled before DO-A resources. Broadcast information is transmitted right after a synchronization signal. - Option#3: Periodic CFO calibration signal and broadcast information is placed after different synchronization signals - Therefore, the following update(green text) is proposed. For time location of broadcast information [immediately after periodic synchronization signal] Source [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [vivo], [Apple], [CMCC], [ETRI], [Samsung], [ZTE] and [Docomo] state that broadcast information can be transmitted immediately after periodic synchronization signal. - Source [Huawei], [Spreadtrum] and [vivo] states that this ensures the device can obtain system information (e.g., reader ID) as part of the reader identification/differentiation procedure. Source [Huawei] and [vivo] state that this applies to MIB-like broadcast information, and [CMCC] state that this can be one potential option. - Source [Xiaomi] states that the broadcast information can be transmitted at the end of the periodic synchronization signal with fixed time domain resources to simplify device implementation. - Source [vivo] state that it is beneficial to support periodic R2D sync signal block, aka., AIoT-SSB, which can be composed of periodic sync sequence, PRDCH for MIB-like broadcast information, and CFO calibration signal, and these parts are continuous in time. - Source [ZTE] states that periodic CFO calibration signal and broadcast information is placed after different synchronization signals. [Separated or have Gap from periodic synchronization signal] Source [Ericsson], [Samsung], [CMCC], [ZTE], and [Docomo] states that, the broadcast information can be transmitted not immediately after periodic synchronization signal. - Source [Ericsson] states that broadcast information can be transmitted with a different periodicity from the periodic sync signal and with pre-defined offset to the sync signal when transmitted. Source [CMCC] also state that, MIB-like part is transmitted separately with the synchronization signal - Source [Samsung] state that time resource can be derived by periodic synchronization signal with a fixed gap. - Source [Docomo] state that time resource of the broadcast information can be immediately after the R2D signal or with an L1 R2D control in between. - Source [ZTE] state that periodic CFO calibration signal is scheduled after the synchronization signal and before the broadcast information. There is a gap between the synchronization signal and the CFO calibration signal, as well as another gap between the CFO calibration signal and the broadcast information.. |
| FL |  | Update based on comments from Qualcomm, Lenovo, Samsung and ZTE. |

Issue# 4.5-2: Transmission parameter for broadcast information

Futurewei, Huawei, Spreadtrum, OPPO, Xiaomi, ZTE, vivo, Apple, ETRI, Ericsson, DOCOMO and Qualcomm provide views on transmission parameters for broadcast information.

Company views are summarized as follows

- Using fixed and predefined parameters, e.g., M value, payload size, repetition, and FEC, for transmission of broadcast information are supported by 9 companies (Futurewei, Huawei(for MIB), Spreadtrum, OPPO, Xiaomi, ZTE, vivo(for MIB), Apple, ETRI(for MIB), Panasonic). L1 R2D control information is not needed for broadcast information transmission. Spreadtrum and Xiaomi state that this simplifies device implementation.

- Variable TBS is raised by Ericsson and Qualcomm for flexbility and TBS can be indicated via L1 control.

- For SIB1-like broadcast information, Huawei, vivo and ETRI suggest flexible parameters can be indicated.

- ZTE and Docomo suggest to study both fixed parameter (w/o L1 control) and parameters indicated by L1 control

Details can be found in the TP below.

FL Notes

- Based on company inputs, FL provide following TP which reflects company views for TR capturing.

[TP][N] FL Proposal 4.5-2

FL Proposal 4.5-2-v1

Capture following in TR38.769 regarding transmission parameter for broadcast information.

Transmission parameter for broadcast information is studied, and the following observations are provided.

For transmission parameters for broadcast information,

[Fixed and predefined parameters]

Source [Futurewei], [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [ZTE], [vivo], [Apple], [ETRI], [Lenovo], [Samsung] and [Panasonic] state that L1 R2D control information is not needed for broadcast information transmission, and parameters (e.g., M value, payload size, repetition, FEC related parameters) can be fixed and predefined. Hence, L1 control information is not needed.

- Source [Huawei], [vivo] and [ETRI] state that fixed and predefined parameter applies for MIB-like broadcast information for the case broadcast information is splitted into MIB and SIB.

- Source [Spreadtrum] and [Xiaomi] state that fixed and predefined parameter simplifies device implementation.

- Source [Ericsson] also state that the M value, the number of repetitions, and the code rate are strictly not necessary and can cater to coverage-edge cases, i.e., corresponding to the values that provide the best coverage.

- Source [Ericsson] state that the need for variable TBS can be discussed, e.g., considering potential needs to increase the contents of broadcast information in a future release. If needed, the TBS can be indicated via L1 R2D control information or postamble. Otherwise, the TBS can be fixed and predefined.

For parameters for SIB1-like broadcast information when broadcast information is split into MIB-like broadcast information and SIB1-like broadcast information

- Source [Huawei] state that the L1 R2D control information for the PRDCH carrying the SIB-like broadcast information is used to indicate information such as the TBS, FEC code rate and repetition number to save the indication overhead of the MIB-like broadcast information.

- Source [vivo] state that L1 R2D control information is needed for SIB1-like broadcast information, and the parameters (including parameters of L1 control information of SIB1-like broadcast information, the start time offset and periodicity) can be predefined and/or indicated by MIB-like broadcast information.

- Source [ETRI] state that if broadcast information is split into MIB and SIB, the transmission parameters of SIB may be fixed and predefined or indicated via MIB.

[Potential flexibility for transmission parameters]

Source [Qualcomm] state that for system information broadcast messages, L1 control is useful to indicate the transmission parameters, such as TBS and relevant fields for subsequent system information (SIB) within the same PRDCH. Device can perform SIB monitoring with a predefined periodicity and detect SIB scheduled by the L1 control in the same PRDCH, which is more straightforward and potentially more efficient than using a separate MIB.

Source [NTT DOCOMO] state that for each parameter of the PRDCH with broadcast information, such as M value, TBS, etc., how much flexibility should be ensured can be further studied. Opt.1) fixed value is applied, Opt.2) indicated by R2D L1 control with/without restriction on applicable values.

Source [ZTE] state that both broadcast information with or without L1 control can be studied. The code method, code rate, chip duration, and repetition can be a fixed and L1 control is not needed. The flexibility can be improved if L1 control is available for broadcast information, the chip duration of L1 control part in broadcast information, the chip duration of L1 control part is a fixed value.

[End of TP]

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Our study on MIB/SIB is in R1-2601262. [Potential flexibility for transmission parameters] Source [Qualcomm] state that for system information broadcast messages, L1 control is useful to indicate the transmission parameters, such as TBS and relevant fields for subsequent system information (SIB) within the same PRDCH. Device can perform SIB monitoring with a predefined periodicity and detect SIB scheduled by the L1 control in the same PRDCH, which is more straightforward and potentially more efficient than using a separate MIB. |
| ETRI | Y | Fine with the TP. |
| LGE |  | We also think that fixed/predefined transmission parameters can be used for broadcast information. |
| Lenovo |  | Please add our view as below (see R1-2601022): [Fixed and predefined parameters] Source [Futurewei], [Huawei], [Spreadtrum], [OPPO], [Xiaomi], [ZTE], [vivo], [Apple], [ETRI] [Panasonic], and Lenovo state that L1 R2D control information is not needed for broadcast information transmission, and parameters (e.g., M value, payload size, repetition, FEC related parameters) can be fixed and predefined. Hence, L1 control information is not needed. |
| Samsung |  | We support to use fixed parameters with a condition that transmission resource of broadcast information is bundled with the periodic synchronization signal. We suggest to capture our view in [Fixed and predefined parameters]. |
| Huawei, HiSilicon |  | Some editorial changes: - Source [Huawei], [vivo] and [ETRI] state that fixed and predefined parameter applies for MIB-like broadcast information for the case broadcast information is splitted into MIB and SIB. … For pamameters parameters for SIB1-like broadcast information when broadcast information is split into MIB-like broadcast information and SIB1-like broadcast information |
| FL |  | Update based on reply from Qualcomm, Lenovo, Samsung, and Huawei. |

- R2D L1 control Aspects

This section discusses R2D L1 control aspects other than chip length determination (which is summarized in section 4.5).

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted Agreement For the functionality of indicating the end of the PRDCH, study the following option: - Option 1: Use the L1 R2D control information to indicate the TBS. Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

  - L1 R2D control transmission

Regarding information to be included in L1 R2D control information, and how the control information is transmitted. Following agreements are made last meeting.

Agreement Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects - Necessary information to be included in the L1 R2D control - Details on how the L1 R2D control information is transmitted

Issue#5.1-1 How the L1 R2D control information is transmitted

Regarding how the L1 R2D control information is transmitted, including FEC/repetition, payload size, and CRC attachment, following information are summarized from company input for this meeting.

Regarding FEC and repetition for L1 R2D control information

CATT states that the separate FEC applied for L1 control information should be considered for error protection to against the multi-path fading channel in outdoor deployment scenarios

Huawei states that it is necessary to apply FEC and block repetition for L1 R2D control information to maintain a similar performance with the subsequent data part of PRDCH.

- The FEC and repetition parameters of the L1 R2D control information of the SIB-like broadcast information is predefined or indicated by the MIB-like broadcast information.

- The FEC and repetition parameters of the L1 R2D control information after the SIB-like broadcast information is indicated by the SIB-like broadcast information.

Regarding payload for L1 R2D control information

- Huawei state that the size of the L1 control information is 10~12 bits considering at least the FEC code rate, repetition number and TBS.

- ZTE state that L1 control occupies one or multiple full OFDM symbols.

- Samsung state that R2D L1 control information is variable, and within 24 bits for 6-bit CRC attachment

- Qualcomm propose two different sizes for L1 control can be considered for unicast and non-unicast.

- Xiaomi, Apple, LGE propose only one bit size of L1 R2D control information

For separated CRC for L1 control information

- Huawei, Xiaomi, LG suggest to always use 16 bit CRC to ensure high reliability

- Samsung suggest to always use 6 bits (assuming no larger than 24 bits payload for L1)

- Ericsson, vivo, Qualcomm and TCL suggest to use the same rule as R19, 6 bits for < 25 TBS and 16 bits otherwise.

FL notes

- Company view are added to the following update TP for TR capturing.

- Most companies support both FEC and repetition for L1 control, , according to contributions last meeting. While only FEC is reflected in the first sentence in section, ‘6A.1.x.4 FEC and repetition’, and company views on repetition is not correct summarized in the TP provided last meeting. Hence, update to correctly reflect company view in last meeting.

[TP][U] FL Proposal 5.1-1

6A.1.x.4 FEC and repetition Regarding FEC and repetition for L1 R2D control part, sources [R1-10321-123-3, vivo], [R1-10321-123-12, OPPO], [R1-10321-123-5, CMCC], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-14, ZTE], and [R1-10321-123-24, Qualcomm], and [Huawei] report that FEC and repetition can also be applied to L1 R2D control information. Source [R1-10321-123-6, Huawei] state that repetition is needed for L1 R2D control information. Source [R1-10321-123-1, FUTUREWEI] and [R1-10321-123-9, CATT] report that FEC can be applied to L1 R2D control information. Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-8, NEC], [R1-10321-123-12, OPPO], [R1-10321-123-10, CTC], [R1-10321-123-14, ZTE] and [R1-10321-123-24, Qualcomm] report methods to determine FEC, repetition related parameters for L1 R2D control information, if FEC/repetition is supported for L1 control. - Sources [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-8, NEC], [R1-10321-123-14, ZTE] and [R1-10321-123-24, Qualcomm] report that the parameters for repetition and FEC for L1 R2D control information can be pre-configured, e.g., by broadcast information. [R1-10321-123-12, OPPO] report that these parameters should be pre-known by device. - Sources [R1-10321-123-8, NEC] and [R1-10321-123-10, CTC] states that parameters for repetition and FEC for L1 R2D control information can be fixed and pre-defined. Source [R1-10321-123-8, NEC] also state that these parameters can be indicated by a set of different binary sequences with fixed or variable length for indication. Source [R1-10321-123-8, NEC] report that a separate interleaving is needed for L1 R2D control. - Source [Huawei] state that The FEC and repetition parameters of the L1 R2D control information of the SIB-like broadcast information is predefined or indicated by the MIB-like broadcast information. The FEC and repetition parameters of the L1 R2D control information after the SIB-like broadcast information is indicated by the SIB-like broadcast information. 6A.1.x.5 Payload size of control Sources [R1-10321-123-3, vivo], [R1-10321-123-12, OPPO], [R1-10321-123-7, Ericsson], and [R1-10321-123-19, Apple], [Xiaomi] and [LGE] report that device only detects one payload size at a given time. And [R1-10321-123-3, vivo] also report that up to two payload sizes can be considered and detected by preamble. Source [Qualcomm] state that two different sizes for L1 control can be considered for unicast and non-unicast. Source [Huawei] state that the size of the L1 control information is 10~12 bits considering at least the FEC code rate, repetition number and TBS. Source [Samsung] state that R2D L1 control information is variable, and within 24 bits for 6-bit CRC attachment. Source [ZTE] state that L1 control occupies one or multiple full OFDM symbols. [Omit unchanged part] 6A.1.x.7 CRC R2D control information with a separate CRC from data is studied. Source [R1-10321-123-1, FUTUREWEI] states that, for separate CRC on the L1 R2D control information, when the size of the L1 R2D control information is 24 bits or less, the existing CRC6 specification can be reused, if the size of the L1 R2D control information is between 25 and 57 bits, the existing CRC6 polynomial can be reused without impact to the undetected error rate. Sources [R1-10321-123-11, Xiaomi], and [R1-10321-123-19, Apple], [Huawei] and [LGE] state that how to attach the separate CRC for L1 R2D control information needs to be considered, and a fixed length (equal to or larger than 16) of CRC is required to ensure the reliability and avoid unnecessary blind detection for L1 R2D control information. Source [Ericsson], [vivo], [Qualcomm] and [TCL] suggest to use the same rule as R19, 6 bits for < 25 TBS and 16 bits otherwise. Source [vivo] state that this provides sufficient CRC protection and reasonable overhead. Source [Samsung] suggest to always use 6 bits, assuming no larger than 24 bits payload for L1 control information.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Futurewei | Y |  |

  - Relation between L1 R2D control and PRDCH

Following agreements related time/frequency resource and channel designs to transmit L1 R2D control information.

Agreement For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

Issue 5.2-1 whether L1 control and data payload of PRDCH are contiguous in time

Regarding whether L1 control and data payload of PRDCH are contiguous in time, following information are summarized from company input for this meeting.

Xiaomi, Nokia, interDigital, Sony, Quectel suggest time gap between L1 R2D control and data payload of PRDCH, to simplify implementation by avoiding buffering PRDCH part and cocurrent decoding. interDigtal and Quectel mentions the benefit of flexibility in time. And Quectel mentioned another purpose is to support different frequency between L1 R2D control and data payload in PRDCH

Spreadtrum, CATT, vivo and NEC propose L1 control and data payload of PRDCH are contiguous in time.

- CATT, vivo and NEC do not agree the purpoes of reserving time gap for different frequency between L1 control and R2D data payload, and frequency retuning should be avoided.

- vivo provide the analysis with reference show that the decoding latency of L1 control signaling (assuming ~30bits payload size) is a few tens of us, less than one OFDM symbol. The memory required for buffer signal during one OFDM symbol is limited(~2000bits), and feasible even for passive device, according to product datasheet, and also feasible for active device. NEC also share similar view.

- vivo state that different chip length for L1 control and data part does not require clock rate change; R2D signal can be received properly without filter adjustment for different M values/chip duration.

- vivo did not see the necessity of cross Slot scheduling for R2D data, as this is not even deployed for NR UE, and not practical for AIoT device either.

FL notes:

- Views from Nokia, interDigital, Sony, Quectel, Speadtrum, CATT and vivo are further add to the TP.

- Views from Xiaomi and NEC have already relected in TR, hence not handled.

[TP][U] FL Proposal 5.2-1

FL Proposal 5.2-1-v1

Update TR 38.768 on whether L1 R2D control information and data payload of PRDCH are contiguous in time

6A.1.x.2 Time location L1 control information precedes the data payload of the PRDCH. Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Sources [R1-10321-123-13 Samsung] and [R1-10321-123-19, Apple] state that, transmit L1 R2D control information and data payload in the same PRDCH with contiguous time resources and same frequency resource is beneficial for low device complexity. [R1-10321-123-19, Apple] further report that this simplifies the timeline design. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-8, NEC] report that the time gap is not long enough to introduce gap between L1 R2D control information and data payload. Sources [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC] further states that decoding of L1 R2D control information the device can parallelly store the data payload in buffer, and receive L1 R2D control information, hence time gap is not needed. Source [vivo] provide the analysis with reference show that the decoding latency of L1 control signaling (assuming ~30bits payload size) is a few tens of us, less than one OFDM symbol. The memory required for buffer signal during one OFDM symbol is limited(~2000bits), and feasible even for passive device, according to product datasheet, and also feasible for active device. - Sources [R1-10321-123-3, vivo], and [R1-10321-123-14, ZTE], [Spreadtrum] and [CATT] report that L1 R2D control transmitted in the same frequency as data payload in PRDCH, and time gap is not needed. - Source [R1-10321-123-16, LGE] report that no time gap needed if the chip length for L1 R2D control information is the same as that for data payload of PRDCH. Source [vivo] state that different chip length for L1 control and data part does not require clock rate change. Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-7, Ericsson] and [R1-10321-123-11, Xiaomi] report that a short time gap is needed if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH. - Source [R1-10321-123-5, CMCC] state that, L1 R2D control and data payload are transmitted in separated physical channel, hence they are not contiguous. - Source [R1-10321-123-12, OPPO] state that, time gap is needed to switch the frequency if L1 R2D control is transmitted in different frequency from data payload. - Source [R1-10321-123-1, FUTUREWEI], [R1-10321-123-16] report that time gap maybe needed if the chip length for L1 R2D control information is different from that for data payload of PRDCH. - Source [R1-10321-123-24, Qualcomm] report that, if there is a gap, midamble can be inserted between L1 R2D control information and data payload. - Source [interDigital] and [Quectel] time gap is needed for flexibility for R2D data payload transmission in time. Factors affecting the choice between the above options were reported as follows: - Whether device buffers data part before finishing L1 control decoding, as reported by sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-2, Spreadtrum], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-7, Ericsson], [R1-10321-123-24, Qualcomm], [R1-10321-123-8, NEC], [R1-10321-123-4, Nokia], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE] and [R1-10321-123-19, Apple]. - Sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-2, Spreadtrum], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-8, NEC] and [R1-10321-123-14, ZTE] state that gap between L1 R2D control and data is not needed, assuming device can buffer data part before finishing L1 R2D control decoding. Source [R1-10321-123-6, Huawei] further state that the time gap would increase the power consumption of the device. - Sources [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], and [R1-10321-123-11, Xiaomi], [Nokia], [interDigital], [Sony] and [Quectel] state that time gap is needed to reduce/avoid buffer at AIoT device. - Whether frequency resource is different for L1 R2D control information and data payload, as reported by sources [R1-10321-123-12, OPPO], and [R1-10321-123-4, Nokia], and [Quectel]. - Whether chip duration (M value) is different for L1 R2D control information and data payload, which is reported by sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-6, Huawei] and [R1-10321-123-16, LGE ] - Whether L1 R2D control information and data payload are separate channel, as reported by source [R1-10321-123-5, CMCC] - Whether CFO calibration signal is placed between L1 R2D control information and data payload, as reported by [R1-10321-123-18, Lenovo]. - Whether flexibility for time location for PRDCH data payload in relation to L1 R2D control information is necessary, as reported by [R1-10321-123-18, Lenovo], [vivo], [interDigital] and [Quectel].

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| CATT | Y |  |

Issue 5.2-2 Frequency resource for L1 R2D control information

Companies provide views on frequency resource for the L1 R2D control information

Xiaomi states that it is straightforward to transmit L1 R2D control and data payload of the subsequent PRDCH on the same frequency resource. If L1 R2D control information is transmitted on different frequency resource to data payload of the subsequent PRDCH, frequency retuning will be required, which is more complicated and increase the Device power consumption.

LGE report that the same frequency resource for R2D preamble, L1 control and data part should be used.

interDigital state that little benefit is expected in a PRDCH center frequency different from the center frequency of the L1 R2D control channel that configured it

CATT state that, for L1 R2D control information and data payload of PRDCH not contiguous in time, it mainly considers that the device can prepare reception of the FDMed transmission of data payload during the gap. However, the FDMed transmission of data payload would increase device processing complexity, such as adjusting the filter bandwidth and/or IF centre frequency

FL notes

- Clear majority company report that, frequency resource of L1 R2D control information is same as R2D preamble and L1 control.

- FL suggest following proposal and TP for capturing.

- FL also suggest to converge to same frequency between L1 control and data payload, considering the situation of 13 support this direction while 1 company have different view.

[TP][U] FL Proposal 5.2-2

FL Proposal 5.2-2-v1

Update TR 38.768 on frequency location of L1 R2D control information

Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-8, NEC], [R1-10321-123-14, ZTE], [R1-10321-123-19, Apple], and [R1-10321-123-24, Qualcomm], [Xiaomi], [LGE], [interDigital] and [CATT] report that the same frequency resource for R2D preamble, L1 control and data part should be used. - Source [R1-10321-123-1, FUTUREWEI] states that different frequency resource will cause additional latency as the transmission carrying control information must be first received followed by receiving the transmission carrying the payload. And unclear of the necessity of this option if FDM is not supported - Sources [R1-10321-123-3, vivo], and [R1-10321-123-6, Huawei], [Xiaomi] and [CATT] state that frequency retuning can be avoided if L1 control information and the corresponding data payload are located with same frequency locations. Otherwise, it will lead to additional adjustment delay and power consumption. - Sources [R1-10321-123-2, Spreadtrum] and [R1-10321-123-3, vivo] state that the motivation of transmitting L1 control and data payload in different frequency is not clear. Source [interDigital] state that little benefit is expected in a PRDCH center frequency different from the center frequency of the L1 R2D control channel that configured it. Source [R1-10321-123-12, OPPO] reports that frequency resource for R2D preamble, L1 control and data part can be different such as to reduce the load in the frequency resource used for L1 control.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |

[H] FL Proposal 5.2-3

FL Proposal 5.2-3-v1

R-TAS, L1 R2D control information and data payload of PRDCH are transmitted at the same frequency resource.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| [OPPO] |  | Regarding “frequency resource”, is it referring to center frequency only or also referring bandwidth? If referring to both, the proposal is too restrictive as it means L1 R2D control and the associated data payload always use same M value. If referring to center frequency only, our suggestion is to rephase the proposal from device perspective, as the device is not sensitive to center frequency shift, it does not impact reception if the reader shift the center frequency for several sub-carriers. For receiving R-TAS, L1 R2D control information and data payload of PRDCH, the device assumes same centre frequency are transmitted at the same frequency resource. |
| Spreadtrum | Y |  |
| Panasonic |  | We have same question as OPPO on “frequency resource”. It’s better to clarify it |
| LGE | Y |  |
| Lenovo | Y |  |
| Futurewei |  | Please clarify if the intention is center frequency. This also may impact chip duration for data |
| DOCOMO | Y |  |
| Huawei, HiSilicon | Y | We are supportive of having the R-TAS, L1 R2D control information and the data payload to be transmitted at the same frequency because otherwise, the device needs to retune its LO frequency and the frequency retuning may need additional handling delay and additional power consumption. It would also complicate the time relationship at the device and would require it to buffer the L1 control information for a significant time which is hard for the device to implement. Receiving both of them in the same frequency is simple for device implementation. |
| ZTE, Sanechips | Y |  |
| CATT | Y |  |
| FL |  | FL Comments - OPPO and Panasonic raise the question that whether both center frequency and bandwidth are same for these parts, and prefer to keep flexibility for different BW for these parts to keep flexibility for different BW for these parts. - OPPO suggest to change the proposal from device perspective, to keep more flexibility at reader side. - The proposal is updated as follows to address above concern, as a small step forward. Whether the BW for L1 control and data payload is same or not can be discussed in WI phase. FL Proposal 5.2-3-v2 AIoT device assumes the same centre frequency for receiving R-TAS, L1 R2D control information and corresponding data payload of PRDCH. |

Issue 5.2-3 L1 control in the same PRDCH with data, or Separate PRDCH

Spreadtrum, vivo, NEC, interDigital propose L1 control and data payload are transmitted in the same PRDCH.

- vivo state that Introducing two different PRDCH for L1 control and data payload adds additional standard effort burden, e.g., different channel structure for different PRDCH with or without preceding preamble, the relation between two consecutive R2D reception

- NEC state that the coding/repetition scheme for the control information is like the data payload, and aslo no specific reference RS would be dedicatedly introduced for the control information.

- interDigital state that a single framework would limit spec impact and avoid complex device implementation.

Xiaomi and Panasonic propose L1 control and data payload are transmitted in separated PRDCH.

- Xiaomi state that separated transmission shoud be adopted if option 2(non-contiguous) for time location of L1 R2D control information and R2D data payload.

- Panasonic state that control part would require higher reliability, also one control information could be used for multiple PRDCHs, Method 2 would be preferred that control and data are in different PRDCHs for simpler design.

FL notes

- Views from vivo, interDigital and Panasonic is alreaddy captured in TR.

- Views from Spreadtrum, Xiaomi and NEC are further added to the update TP.

[TP][U] FL Proposal 5.2-4

FL Proposal 5.2-4-v1

Update TR 38.769 for whether L1 control and data payload is transmitted in the same PRDCH, or Separate PRDCH.

6A.1.x.6 Physical channel No new R2D physical channel for L1 R2D control information is introduced. Sources [R1-10321-123-3, vivo], [R1-10321-123-24, Qualcomm], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-8, NEC], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], and [R1-10321-123-20, InterDigital] and [Spreadtrum]state that L1 R2D control is transmitted in the same PRDCH carries data payload for simplicity for standard and device. - [R1-10321-123-6, Huawei] states if L1 control and data are in different PRDCH, the time relationship design is complicated and the device needs to buffer the L1 control information for a long time which is hard for the device to implement. - Source [NEC] state that the coding/repetition scheme for the control information is like the data payload, and also no specific reference RS would be dedicatedly introduced for the control information. Sources [R1-10321-123-14, ZTE], [R1-10321-123-5, CMCC], [R1-10321-123-15, Transsion], [R1-10321-123-17, Panasonic], and [R1-10321-123-21, MediaTek] and [Xiaomi] state that L1 R2D control information is transmitted in a separate PRDCH - Source [R1-10321-123-14] report that, L1 R2D control information is encoded separately, it is more appropriate to transmit L1 R2D control information on a separate PRDCH - Sources [R1-10321-123-5, CMCC] and [R1-10321-123-15, Transsion] state that, defining a new PRDCH format to carry L1 R2D control information only is more efficient for scheduling and resource utilization. - Source [R1-10321-123-17, Panasonic] state that, L1 control information is transmitted in separated PRDCH since control part would require higher reliability, and one control information could be used for multiple PRDCH transmissions. - Source [Xiaomi] state that separated transmission should be adopted if option 2(non-contiguous) for time location of L1 R2D control information and R2D data payload.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| Futurewei |  | Editorial corrections Source [Xiaomi] state that separated transmission should be adopted if option 2(non-contiguous) for time location of L1 R2D control information and R2D data payload. - Source [NEC] state that the coding/repetition scheme for the control information is like the data payload, and also no specific reference RS would be dedicatedly introduced for the control information. |
| FL |  | Update based on Futurewei’s correction |

  - R2D Chip length determination

Agreement For the functionality of chip duration determination of the data payload in the PRDCH, study the following options: - Option 1: same as the chip duration of the corresponding L1 R2D control information - Option 2: indicated by the corresponding L1 R2D control information Agreement For the functionality of chip duration determination of the L1 R2D control information, study at least the following options: - Option 1: Fixed chip duration. No use of CAP for chip duration determination. - Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination. - Option 3: Use CAP - Alt.3-1 Rel-19 CAP pattern - Alt.3-2 Enhanced CAP - Alt.3-2a: Rel-19 CAP pattern with repetition - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information - Option 4: Use a set of different binary sequences with fixed or variable length - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination. - Alt.4-2: using binary sequences for CAP - Option 5: Use broadcast information (if any) to indicate the chip duration

Issue 5.3-1: Chip Length determination for data payload in the PRDCH

For the functionality of chip duration determination of the data payload in the PRDCH,

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Following views are further provided in contribution

- Xiaomi state that for the functionality of chip duration determination of the data payload in the PRDCH, option 2 can be studied.

- ZTE state that the chip duration of an R2D transmission that includes L1 control is indicated by the corresponding L1 R2D control information. The chip duration of an R2D transmission without L1 control, if present, is a fixed value.

- Ericsson state that For the functionality of chip duration determination of the data payload in the PRDCH, the chip duration is fixed for paging and broadcast PRDCH and indicated by the corresponding L1 R2D control information for other PRDCH such as Msg2 and Msg4.

FL notes

- Views from Xiaomi and Ericsson are already captured in TR

- New point from ZTE is added to the TP below.

[TP][U] FL Proposal 5.3-1

FL Proposal 5.3-1-v1

Update TR38.769 for determine chip duration for data payload

6A.1.x.y.1 Chip duration of data payload The chip duration determination of the data payload in the PRDCH and the L1 control information are studied. The following options are studied for the functionality of the chip duration determination of the data payload in the PRDCH: Option 1-data: The chip duration of the corresponding L1 R2D control information is used. [omit unchanged part] Option 2-data: The chip duration indicated by the corresponding L1 R2D control information is used. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-5, CMCC], [R1-10321-123-7, Ericsson], [R1-10321-123-8, NEC], [R1-10321-123-10, CTC], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE], [R1-10321-123-16, LGE ], [R1-10321-123-17, Panasonic],[R1-10321-123-25, NTT DOCOMO], [R1-10321-123-19, Apple], [R1-10321-123-24, Qualcomm], and [R1-10321-123-28, CEWiT], and [CATT] report that, chip duration of the data payload in the PRDCH indicated by the corresponding L1 R2D control information, have the following benefits: [omit unchanged part] - Sources [R1-10321-123-19, Apple] and [R1-10321-123-7, Ericsson] state that this option provides flexibility to adapt to different coverage conditions or message types, with [R1-10321-123-7, Ericsson] states this option provides benefits for PRDCH like Msg2 and Msg4. Source [ZTE] state that the chip duration of an R2D data payload without L1 control (if supported), if present, is a fixed value.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| Nokia | Comment | Regarding ZTE’s view (last sentence): the condition “if present” seems a bit unclear, in our understanding “if supported” would be more appropriate since the question is whether the case “R2D data without L1 control” will be supported. |
| CATT |  | We support option2, as described in proposal 39 in our contribution. Please add CATT under option 2. |
| FL |  | Update based on comments from Nokia and CATT. |

Issue 5.3-2: Chip duration determination for L1 control part

Huawei, Ericsson, CATT, vivo, ZTE, Nokia, NEC, LGE, Xiaomi, Qualcomm, Samsung, Docomo, Apple, OPPO, and Sony provide views on this topic in their contributions. Company views are provided as follows.

For option 1, Fixed chip duration. No use of CAP for chip duration determination.

- CATT state that the chip duration of L1 R2D control information could be different from that of data payload, in which the chip duration of L1 R2D control information is fixed for reliability

- Xiaomi: most straightforward to implement but may not be able to support link adaptation for the L1 R2D control information.

- ZTE: The chip duration of L1 control part in broadcast information, the chip duration of L1 control part is a fixed value,

- Sony: while simple, it is less flexible as it cannot support devices with different coverage range

For option 2, Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- NEC and Xiaomi: increases detection complexity for Devices.

- Sony: performing blind detection on a predefined set of the chip durations leads to unnessary complex processing and power consumption

For option 3, Use CAP

- NEC: exclude option 3, in Rel-20 A-IoT, the R2D reception operation is not edge detection in Rel-19

- vivo provide evaluation result show that it is not feasible to use R19 CAP (Alt 3-1) and R19 CAP with repetitions (Alt 3-2a) to determine chip length for L1 control part. The BLER performance is not even close (more than 10dB performance gap) to meet coverage of 500 meters at 10% BLER in UMa scenario. Propose to exclude Rel-19 CAP pattern (Alt.3-1) and R19 CAP with repetitions (Alt 3-2a).

- Sony: using a CAP for chip length determination requires a two-part preamble design which is not necessary with a binary sequence based SIP design.

- Docomo: options 3 have an advantage as flexible chip duration of L1 R2D control.

For Option 4, Use a set of different binary sequences with fixed or variable length

- NOKIA: only limited flexibility is needed, e.g., up to 2, so 2 Sequence is sufficient. Sequence-based SIP can be effectively used to indicate chip duration of a PRDCH for L1 control with limited indication flexibility to keep low device complexity.

- Xiaomi: For option 4-1, on top of sequence-based SIP, it is reasonable to use feasible multiple sequences to indicate multiple values of the L1 R2D control information.

- Sony: Option 4-1: single-part preamble where a sequence-based signal can not only provide start and synchronization functionality, but it can also be used to carry information with regard to chip length of the subsequent control and data payload, has an advantage over the other approaches in terms of low-complexity and power consumption.

- Docomo: Options 4 have an advantage as flexible chip duration of L1 R2D control. And Alt.4-1 has a disadvantage that device needs to blind detect multiple sequences for SIP detection, which leads to higher device complexit

- Sony: Option 4-2: selection of sequences with variable lengths for determination of the chip length in control and data payload may result in unnecessary complexity and power consumption.

For option 5, Use broadcast information (if any) to indicate the chip duration

- Xiaomi: As a common indication method, option 5 is an efficient way for chip duration indication of the L1 R2D control information but loses some flexibility for the L1 R2D control information transmission towards different Devices.

- ZTE: the chip duration of L1 control part in R2D transmission other than broadcast information is indicated by the broadcast information.

FL notes

- Views from companies are further added to the update TP.

[TP][U] FL Proposal 5.3-2

FL Proposal 5.3-2-v1

Update TR 38.769 for Chip duration determination of L1 R2D control information.

6A.1.x.y.2 Chip duration of L1 R2D control information The following options are identified for the functionality of the chip duration determination of the L1 R2D control information: Option 1-control: A fixed chip duration is used, with no use of CAP. Option 2-control: A set of pre-defined chip durations, blindly detected by a device with no use of CAP. Option 3-control: A CAP is used, where multiple alternatives are studied: Option 3-1-control: Using the pattern of CAP defined in TS 38.291 [228]. Option 3-2-control: Enhancing CAP by the following options: Option 3-2a-control: Using the pattern of CAP defined in TS 38.291 [228] with repetitions, Option 3-2b-control: Using a pattern that associates the chip duration of the clock acquisition part to different chip durations of the L1 R2D control information. Option 4-control: A set of binary sequences with fixed or variable length is used, where multiple alternatives are studied: Option 4-1-control: Use of the binary sequence-based SIP, with no use of CAP. Option 4-2-control: Use of binary sequences for the CAP. Option 5-control: Broadcast information is used to indicate the chip duration. For Option 1-control, the following points were reported as advantages: - Sources [R1-10321-123-7, Ericsson], [R1-10321-123-9, CATT], [R1-10321-123-16, LGE ] and [R1-10321-123-10, CTC] states that fixed chip duration for L1 R2D control information lowers the complexity and power consumption of chip duration detection. - Source [R1-10321-123-9, CATT] states that fixed chip duration for L1 R2D control information minimize the ambiguity of the chip duration detection, and thus enhance the success rate of L1 R2D control information reception. - Source [R1-10321-123-7, Ericsson] states that using fixed chip duration (using the smallest M value for optimal coverage) can achieve higher reliability for L1 R2D control information. - Source [ZTE] state that the chip duration of L1 control part for broadcast information, is a fixed value. For Option 1-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] and [R1-10321-123-13, Samsung] states that to achieve the largest coverage, L1 control would need to use the lowest M, which increases overhead for each R2D transmission. - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-13], [R1-10321-123-14], [R1-10321-123-6, Huawei], [R1-10321-123-3, vivo], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-5, CMCC], [Sony] and [Xiaomi] states that, if chip duration for L1 control information is fixed, the flexibility or transmission efficiency is restricted. For Option 2-control, the following points were reported as advantages: - Sources [R1-10321-123-5, CMCC] and [R1-10321-123-17, Panasonic] state that blind detection within configured set provide better flexibility for chip length for L1 R2D control information. - Source [R1-10321-123-5, CMCC] states that blind detection is viable if the number of predefined chip duration candidates is restricted. Additionally, [R1-10321-123-5, CMCC] states that Option 2 can be combined with device-specific configuration signaling to enhance flexibility without excessive complexity. For Option 2-control, the following points were reported as disadvantages: - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-13, Samsung], [R1-10321-123-3, vivo], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-7, Ericsson], and [R1-10321-123-14, ZTE], [NEC], [Sony] and [Xiaomi] state that Option 2 increases detection complexity. - Source [R1-10321-123-24, Qualcomm] states that it may be impractical due to constraints on power consumption and memory needed for buffering the incoming R2D L1 control information samples. - Sources [R1-10321-123-6, Huawei] and [R1-10321-123-16, LGE ] state that option 2 would require additional time gap between the L1 control information and the data payload of the PRDCH to allow for decoding delay. - Sources [R1-10321-123-3, vivo] and [R1-10321-123-7, Ericsson] state that reliability is doubtful due to blind detection. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway to achieve flexibilities in coverage and resource overhead, no need to leave chip duration for blind detection. For Option 3-control, the following points were reported as advantages: - For Alt.3-1 Rel-19 CAP pattern, sources [R1-10321-123-13, Samsung] and [R1-10321-123-29, Sequans] state that Rel-19 CAP pattern to determine chip length of L1 R2D control part offer a balance between detection complexity and performance. - For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, sources [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple] and [R1-10321-123-24, Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Source [R1-10321-123-19, Apple] further states that this option maintains consistency with Release-19 design principles. Source [R1-10321-123-24, Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility. Source [Qualcomm] also provides simulation results that the CAP with 2bits and fixed length (fixed M=2 chip duration) can be detected with 10% false detection at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control). - Source [Docomo] state that options 3 have an advantage as flexible chip duration of L1 R2D control. For Option 3-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] states option 3 requires blind detection; and MD/FA rates and CAP overhead needs to be considered. - Source [R1-10321-123-14 ZTE] states option 3 results in additional overhead due to transmission of CAP. - Sources [R1-10321-123-5, CMCC], [R1-10321-123-3, vivo], and [R1-10321-123-6, Huawei] and [NEC] state that Option 3 is based on edge detection to determine the chip duration by using CAP, so the performance is limited or unfeasible due to the use of edge detection to detect the CAP pattern. Sources [R1-10321-123-14, ZTE] and [R1-10321-123-7, Ericsson] also report the performance of this option should be further evaluated. - Source [vivo] provide evaluation result show that it is not feasible to use R19 CAP (Alt 3-1) and R19 CAP with repetitions (Alt 3-2a) to determine chip length for L1 control part. The BLER performance is not even close (more than 10dB performance gap) to meet coverage of 500 meters at 10% BLER in UMa scenario. - Source [R1-10321-123-7, Ericsson] states that option 3 would make the decoding of the L1 R2D control information more complicated for the device. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection using CAP. - Source [Sony] state that using a CAP for chip length determination requires a two-part preamble design which is not necessary with a binary sequence based SIP design. For Option 4-control, the following points were reported as advantages: For Alt 4-1, using binary sequences for SIP. No use of CAP for chip duration determination - Sources [R1-10321-123-4, Nokia], [R1-10321-123-6, Huawei], [R1-10321-123-8, NEC], [R1-10321-123-16, LGE ], [R1-10321-123-25, NTT DOCOMO], and [R1-10321-123-17, Panasonic] and [Xiaomi] state that this method can provide flexibility for R2D chip duration. - Source [R1-10321-123-6, Huawei] state that a restricted set of binary sequences of differing lengths are mapped to different chip durations and coverage distances, which can be detected with limited blind detection complexity. Parallel processing for the blind detection of different sequence lengths avoids and processing delay. Besides, R-TAS design is simplified, which consist only sequence-based SIP.Alt.4-2: using binary sequences for CAP. - Sources [R1-10321-123-4, Nokia] and [R1-10321-123-17, Panasonic] report that flexibility on chip length of L1 R2D control can be achieved. Source [Nokia] state that sequence-based SIP can be effectively used to indicate chip duration of a PRDCH for L1 control with limited indication flexibility to keep low device complexity, and only limited flexibility is needed, e.g., up to 2 sequence is sufficient. - Source [Sony] state that single-part preamble where a sequence-based signal can not only provide start and synchronization functionality, but it can also be used to carry information with regard to chip length of the subsequent control and data payload, has an advantage over the other approaches in terms of low-complexity and power consumption. For Option 4-control, the following points were reported as disadvantages: - Sources [R1-10321-123-3, vivo], [R1-10321-123-7, Ericsson], and [R1-10321-123-24, Qualcomm] and [Docomo] states that option 4 increases detection complexity. And [R1-10321-123-7] states that option 4 would make the decoding of the L1 R2D control information more complicated for the device. - Sources [R1-10321-123-14, ZTE] and [R1-10321-123-7, Ericsson] report that the performance of this option should be further evaluated. - Source [R1-10321-123-3, vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection; source [R1-10321-123-3] further reports that it leader more challenges on preamble design, requiring larger number of sequences to indicate four M values in addition to reader/cell ID differentiation; For Option 5-control, the following points were reported as advantages: - Sources [R1-10321-123-3, vivo], [R1-10321-123-14, ZTE] and [R1-10321-123-24, Qualcomm] state that better flexibility can be achieved without blind detection, and sources [R1-10321-123-3, vivo] and [R1-10321-123-14, ZTE] further state that the chip duration for L1 can be indicated together with other parameters for control information, e.g., repetition and FEC related parameters. Source [R1-10321-123-5, CMCC] state that this method can be used for L1 R2D control information common for devices, for example, the L1 R2D control information for scheduling paging message. - Source [R1-10321-123-12, OPPO] state that the chip length should be pre-known by device to minimize complexity and power consumption at AIoT device. For Option 5-control, the following points were reported as disadvantages: - Source [R1-10321-123-24, Qualcomm] states that it requires system info acquisition and memorization. - Source [R1-10321-123-6, Huawei] and [R1-10321-123-13, Samsung] states that only to indicate chip duration in broadcast information limits flexible time resource allocation from system perspective. - Source [R1-10321-123-7, Ericsson] states that this option is a simple solution but not applicable for paging and broadcast PRDCH. - Source [R1-10321-123-25, NTT DOCOMO] states that state that option 5 has less flexibility on chip duration of L1 R2D control compared to option 2/3/4. - Source [Xiaomi] state that As a common indication method, option 5 is an efficient way for chip duration indication of the L1 R2D control information but loses some flexibility for the L1 R2D control information transmission towards different Devices.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| Qualcomm |  | Pls also add our observation as below: - For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, sources [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple] and [R1-10321-123-24, Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Source [R1-10321-123-19, Apple] further states that this option maintains consistency with Release-19 design principles. Source [R1-10321-123-24, Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility. Source [Qualcomm] also provides simulation results that the CAP with 2bits and fixed length (fixed M=2 chip duration) can be detected with 10% false detection at an SNR lower than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control). |
| LGE | Y |  |
| NTT Docomo |  | We would like to add following observation in our Tdoc to the TP. For Option 4-control, the following points were reported as disadvantages: Source [Docomo] states that alt.4-1 has a disadvantage that device needs to blind detect multiple sequences for SIP detection, which leads to higher device complexity. |
| CATT | Y |  |
| FL |  | Update based on comments from Qualcomm and Docomo. |

FL notes, the company views on Chip duration determination of L1 R2D control information, considering company inputs in last meeting and this meeting, is summarized in the following table.

- This Table is for brief illustration of update TP, please DO NOT revise this table if the information is not reflected in TP.

| Options |  | Situation |
| --- | --- | --- |
| Option-1 Fixed chip duration |  | Supported by Ericsson, CATT, CTC, LGE, ZTE(for L1 control for broadcast info) |
| Option-2 Blind detection in Predefined Set |  | Supported by CMCC, Panasonic |
| Option-3 Use CAP |  |  |
| Alt.3-1 Rel-19 CAP pattern |  | Alt 3-1: Samsung, LGE, Sequans |
| Alt.3-2 Enhanced CAP | Alt.3-2a: R19 CAP pattern with repetition | N/A |
|  | Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations for L1 control | Alt 3-2b: LGE, Apple, Qualcomm |
| Opt-4 Use a set of different binary sequences with fixed or variable length | Alt.4-1: use SIP | Nokia, Huawei (sequence with different length), NEC, LGE, Docomo, Panasonic , Xiaomi |
|  | Alt.4-2:use CAP | Supported by Nokia, Panasonic |
| Opt-5: Use broadcast info |  | Supported by vivo, CMCC, OPPO, ZTE, Qualcomm |

FL Notes

- Considering nothing is written specifically for Option 3-2a in TR and in the update TP, and more negative comments are made for option 2 then others options, suggest to exclude these two options.

[H] FL Proposal 5.3-3

FL Proposal 5.3-3-v1

Following options are NOT considered for for Chip duration determination for L1 R2D control information.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Alt.3-2a: R19 CAP pattern with repetition

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| [OPPO] | Y |  |
| Panasonic |  | We are ok to compromise. |
| LGE | Y |  |
| Futurewei | Y | Duplicate ‘for’ Following options are NOT considered for for Chip duration determination for L1 R2D control information. |
| Huawei, HiSilicon | Y | We are supportive of not considering option 2 and 3-2a. For option 2, not only would it increase the complexity of the device since it would have to blindly search for different chip durations to be able to decode the L1 R2D control information, it would also increase the processing delay. For option 3, the Rel-19 CAP relies of edge detection which is expected to provide a worse performance when compared to correlation detection and the performance is limited to the high SNR region even if the CAP length is extended by repeating the CAP patterns. |
| ZTE, Sanechips | Y |  |
| CATT | Y |  |

  - Necessary information in L1 R2D control

In last meeting the content for L1 control information has been discussed, and company views are captured in TR. In this meeting, companies further provide potential necessary information to be contained in L2 R2D control information, summarized in the following table.

| Control information elements | Mentioned companies and views. |
| --- | --- |
| TBS for data payload (Already agreed) | Aready agreed Samsung: if message type is included in L1 R2D control information, TBS is not necessary to be explicitly indicated in L1 R2D control information. |
| M value for data part chip duration | [Ericsson][CATT][Xiaomi][NEC][interDigital] FL: All of these inputs has already been captured in TR. No update made to TR. |
| Repetition and FEC information | [ERICSSON][CATT][NEC][Spreadtrum][interDigital][Quectel] [ZTE]: whether or not bit collection or interleaver is performed when FEC is supported. FL: views from Quectel and ZTE are added to the update TP. Other inputs has already been captured in TR. No update made to TR. |
| Timing related indication | Nokia: time-domain resource for the subsequent PRDCH for data payload FL: views from Nokia has already been captured in TR. No update made to TR. |
| Frequency resource allocation | ERICSSON] Quectel FL: views from Ericsson and Quectel has already been captured in TR. No update made to TR. |
| Indication of next R2D transmission | Quectel: the time start of PRDCH Panasoinc: starting time for PRDCH for multi PRDCH scheduling |
| D2R resources to send ACK/NACK feedback for unicast R2D command reception. | Qualcomm |
| Frequency adjustment information to improve CFO accuracy. | vivo |
| Msg types /functional indication | Qualcomm: types of broadcast information, used to indicate which system information is transmitted. Xiaomi: message types for early termination of R2D transmission FL: views from Xiaomi has already been captured in TR. No update made to TR. views from Qualcomm are added to the update TP. |
| DOA triggering | Nokia: an indicator relevant to DO-A access. |
| Indication of round number in access trigger | Qualcomm |

FL notes

- FL do not plan to open discussion for this part at the beginning of this meeting, and may handle this later this meeting or next meeting.

[TP][U] FL Proposal 5.4-1

FL Proposal 5.4-1-v1

Update TR 38.769 for content in L1 R2D control information.

6A.1.x R2D control information 6A.1.x.1 Content In addition to TBS for PRDCH data payload, the following L1 R2D control information, which potentially can be indicated to the device, is studied: - M value for PRDCH data payload chip duration, as reported by [R1-10321-123-3, vivo], [R1-10321-123-24, Qualcomm], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-7, Ericsson], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-4, Nokia], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-17, Panasonic], [R1-10321-123-19, Apple], [R1-10321-123-20, InterDigital], [R1-10321-123-26, TCL] and [R1-10321-123-28, CEWiT]. - On the other hand, sources [R1-10321-123-2, Spreadtrum], [R1-10321-123-6, Huawei], [R1-10321-123-7, Ericsson] and [R1-10321-123-29, Sequans] suggest to use same chip duration between L1 R2D control information and data payload, hence M value indication is not needed in L1 R2D control. - Repetition/FEC related parameters for PRDCH data payload, as reported by sources [R1-10321-123-3, vivo], [R1-10321-123-24, Qualcomm], [R1-10321-123-6, Huawei], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-7, Ericsson], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-8, NEC], [R1-10321-123-9, CATT], [R1-10321-123-10, CTC], [R1-10321-123-14, ZTE], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-19, Apple], [R1-10321-123-20, InterDigital], [R1-10321-123-26, TCL], and [R1-10321-123-28, CEWiT], and [Quectel], if Repetition/FEC is supported for R2D. - Timing related information for PRDCH data payload, as reported by sources [R1-10321-123-24, Qualcomm], [R1-10321-123-5, CMCC], [R1-10321-123-12, OPPO], [R1-10321-123-4, Nokia], [R1-10321-123-15, Transsion], [R1-10321-123-16, LGE ], [R1-10321-123-20, InterDigital] and [R1-10321-123-28, CEWiT], considering potential need of gap between L1 R2D control and corresponding data payload. - On the other hand, sources [R1-10321-123-3, vivo], [R1-10321-123-6, Huawei], [R1-10321-123-13, Samsung], [R1-10321-123-2, Spreadtrum], [R1-10321-123-25, NTT DOCOMO], [R1-10321-123-8, NEC] and [R1-10321-123-14, ZTE] state that, this information is not included in L1 R2D control, because of no need of gap between L1 R2D control and corresponding data payload. - Timing gap between R2D and the corresponding D2R transmission, as reported by [R1-10321-123-24, Qualcomm] - Frequency resource for PRDCH data payload, as reported by sources [R1-10321-123-7, Ericsson], [R1-10321-123-23, Quectel] and [R1-10321-123-28, CEWiT], if FDM for R2D is supported. - Message types/Functional indication, as reported by Sources [R1-10321-123-24, Qualcomm], [R1-10321-123-3, vivo], [R1-10321-123-12, OPPO], [R1-10321-123-13, Samsung], [R1-10321-123-1, FUTUREWEI], [R1-10321-123-4, Nokia], [R1-10321-123-11, Xiaomi], [R1-10321-123-14, ZTE] and [R1-10321-123-5, CMCC], for the purpose of early termination of R2D detection. Source [Qualcomm] state that it can be used to indicate which system information is transmitted. Besides, source [Samsung] state that if message type is included in L1 R2D control information, TBS is not necessary to be explicitly indicated in L1 R2D control information. - ID related information, which potentially include at least one of reader ID, device ID or Transaction ID related information - ID related information associated with device(s) - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-3, vivo], [R1-10321-123-24, Qualcomm], [R1-10321-123-12, OPPO], [R1-10321-123-13, Samsung] and [R1-10321-123-26, TCL] state that, ID related information associated with device(s), indicating for which device or group of devices the upcoming data is intended, can be included in L1 R2D control for early termination. - Reader ID related information, if needed, as reported by source [R1-10321-123-24, Qualcomm]. - Indication of next R2D transmission, as reported by source [R1-10321-123-1, FUTUREWEI], [Quectel] and [Panasonic], used to indicate when the next R2D transmission. Source [R1-10321-123-1, FUTUREWEI] state that the indication can help non-intended devices have the opportunity to harvest energy. Source [Panasonic] states that starting time for PRDCH for multi PRDCH scheduling should be indicated. - D2R resources to send ACK/NACK feedback for unicast R2D command reception, as reported by source [Qualcomm]. - Frequency adjustment information indicated to AIoT device for CFO calibration, as reported by source [vivo]. - Indicator relevant to DO-A access, as reported by source [Nokia]. And Indicator relevant to first D2R of scheduling request for DO-A, as reported by source [Lenovo]. - Indication of round number in access trigger, as reported by [Qualcomm].

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE | Y |  |
| Lenovo |  | Please add our view as below (see R1-2601022): Indicator relevant to first D2R of scheduling request for DO-A, as reported by source [Lenovo]. |
| Futurewei | Y |  |
| FL |  | Update based on Lenovo comments |

- R2D Multiplexing

Agreement (RAN1#122bis) Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects: - complexity of device to support FDM - how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Company views on R2D multiplexing are provided as follows.

Issue 6-1 Necessity feasibility, complexity and frequency resource for R2D FDM

For R2D FDM, following views are provided by companies.

- Ericsson think if multiplexing UEs using FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW (15-PRBs) and 5 MHz CBW (25-PRBs) can be considered.

- Sharp states that Regarding R2D FDM, a Device 2b or Device C is not required to receive more than one of the FDM’ed R2D transmissions.

- Vivo think that when AIoT R2D bandwidth is less than 4 RBs, it is not feasible to support R2D FDM for device 2b with large residual CFO, i.e., 100ppm.

- Xiaomi states that to determine the frequency resource for receiving one of the FDMed R2D transmissions, pre-configuration and dynamic indication can be considered. And to support FDMA, both implementation complexity and hardware costs for device will increase.

- ZTE states that FDM will not introduce significant additional complexity and R2D FDM is feasible for Device 2b and for Device C, considering device equipped with IF/ZIF filters at the receiver.

- CATT proposes the complexity of Device 2b and Device C to support R2D FDM transmission is too high. The necessity and feasibility needs further study.

- NEC states that R2D FDM does not introduce obvious complexity to the device due to the proper IF/ZIF receiver architecture for device 2b and C in Rel-20. And provide potential options on how a device determines the frequency resource for receiving one of the FDMed R2D transmissions.

FL Note:

- Views from CATT, Xiaomi, ZTE, NEC on the device complexity have already been captured in TR.

- Views from NEC on the Determines the frequency resource has already been captured in TR.

- And corresponding views from Ericsson, vivo, ZTE and Xiaomi are added in the updated TP.

[TP][U] FL Proposal 6.1-1

FL Proposal 6.1-1-v1

Update TR38.769, on R2D multiplexing, as follows

FDM for R2D from the same reader is studied, considering the complexity of a device required to support FDM, and how a device determines the frequency resource to receive the FDMed R2D transmissions. In assessing feasibility of such FDM: - Sources [R1-10321-123-1, FUTUREWEI], [R1-10321-123-2, Spreadtrum], [R1-10321-123-6, Huawei, HiSilicon] , [ZTE] and [R1-10321-123-25, NTT DOCOMO] state that R2D FDM is feasible for Device 2b and for Device C, considering device equipped with IF/ZIF filters at the receiver. - Source [R1-10321-123-7, Ericsson] states that the total number of PRBs required to multiplex devices using FDM needs to be defined. Source [Ericsson] further states that if multiplexing UEs using FDM requires a larger transmission bandwidth (ergo channel bandwidth) than in Rel-19, then to minimize the specification impacts, the smallest channel bandwidths available in NR i.e., 3 MHz CBW (15-PRBs) and 5 MHz CBW (25-PRBs) can be considered. - Source [vivo] states when AIoT R2D bandwidth is less than 4 RBs, it is not feasible to support R2D FDM for device 2b with large residual CFO, i.e., 100ppm. [Omit Unchanged part] Regarding how a device determines the frequency resource for receiving one of the FDMed R2D transmissions: - Sources [R1-10321-123-1 FUTUREWEI], [R1-10321-123-6 Huawei, HiSilicon] [R1-10321-123-2 Spreadtrum] and [R1-10321-123-24 Qualcomm] state that the frequency resource of synchronization signal, CFO calibration signal or broadcast information need to be fixed or predefined. - Sources [R1-10321-123-6 Huawei, HiSilicon], [R1-10321-123-2 Spreadtrum], [R1-10321-123-24 Qualcomm], [R1-10321-123-25 NTT DOCOMO], [R1-10321-123-8 NEC], [R1-10321-123-14 ZTE], [Xiaomi] and [R1-10321-123-12 OPPO] state that some of R2D transmissions e.g., L1 control information or a R2D transmission other than sync signal or broadcast information can determine their frequency resources based on the indication. - Source [R1-10321-123-12 OPPO] state that the frequency resource for data payload is indicated by corresponding L1 R2D control. - Source [R1-10321-123-8 NEC] and [R1-10321-123-25 NTT DOCOMO] state that the frequency resource of a R2D transmission can be associated with the previous corresponding D2R transmission. - Source [R1-10321-123-8 NEC] state that the exact frequency resource/channel for a R2D transmission can be associated with a specific coverage level from a set of candidates R2D frequency resources preconfigured. - Source [Sharp] states that a Device 2b or Device C is not required to receive more than one of the FDM’ed R2D transmissions.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |
| LGE |  | We also share a similar view with Ericsson. If R2D FDM is supported, the definition or limitation on the total number of RPBs available for multiplexing devices needs to be specified. |

Issue 6-2 R2D FDM (non) applicable cases

[NEC] [Samsung] report applicable R2D transmissions that can support R2D FDM.

- [NEC] proposes that Access Trigger message, Random ID Response message (Msg2 in CBRA), NACK Feedback message, any combination of them to perform FDM is not precluded.

- [Samsung] propose R2D FDM can be support after clock synchronization or calibration. And whether to support R2D FDM before clock synchronization/calibration, considering the impact of initial CFO at least for device 2b, needs to be considered.

FL notes:

- As raised by Samsung and NEC, under the condition that R2D FDM is supported, R2D FDM (non) applicable cases are further discussed. From FL’s view, the discussion on R2D FDM (non) applicable cases should happens in R20 AIoT WI, if R2D FDM is agreed to be specified. Hence, in the last meeting, we will focus on the study on the aspects of Necessity feasibility, complexity and frequency resource for R2D FDM.

- Other issues

Some of company views which are not summarized in the sections above. FL think these views may belong to WI details or other working groups, hence it seems not necessary to be handled in this meeting.

Sharp suggest that All R2D channel bandwidths (and corresponding transmission bandwidth configurations) as specified in Rel-19, i.e. 200 kHz (1 PRB), 400 kHz (2 PRBs), 600 kHz (3 PRBs) and 800 kHz (4 PRBs), are supported in Rel-20.

TCL state that Support R2D measurement by using the existing R2D signals or the new R2D signals, such as A-IoT RSRP measurement

Ericsson discussed designs for sync rasters. During reader/cell search, the signal pass filter bandwidth should take the initial CFO into account to avoid miss detection, and adjacent carrier interference grows as the carrier frequency increases (This also implies that the A-IoT sync-raster points must be sufficiently far apart from each other as to take the initial CFO into account

FL Notes

- The issues above seems not urgent, and these issues can be done in WI phase or should be addressed by other working groups, hence FL does not plan to treat these issues, in this meeting.

FL Question:

- If there is any issue missed in this summary, please point out in the following table. If the missing issue belongs to WI details, FL suggest to postpone discussion to the WI stage.

Please provide you views in the table below.

| Company | Y/N | Comments |
| --- | --- | --- |

References

| [1] | R1-2600069 | R2D Air Interface for Device 2b/C | FUTUREWEI |
| --- | --- | --- | --- |
| [2] | R1-2600088 | Study on R2D signals, channels, waveform and procedures | Huawei, HiSilicon |
| [3] | R1-2600101 | Discussion on R2D signals, channels, waveform and procedures for Ambient IoT | Spreadtrum, UNISOC |
| [4] | R1-2600179 | Discussion on necessary and feasible change to R2D for Rel-20 A-IoT | OPPO |
| [5] | R1-2600210 | R2D signals, channels, waveform, and procedures | Ericsson |
| [6] | R1-2600327 | Study of R2D signals, channels, and waveform and procedure of A-IoT enhancement for device 2b/C | CATT |
| [7] | R1-2600360 | Discussion on AIoT R2D signals, channels, and procedures | Tejas Network Limited |
| [8] | R1-2600368 | AIoT R2D signals, channels, waveform and procedures | Nokia |
| [9] | R1-2600378 | Discussion on R2D signals, channels, waveform and procedures | CMCC |
| [10] | R1-2600417 | Discussion on R2D signals, channels, waveform and procedures for Device 2b/C | Xiaomi |
| [11] | R1-2600451 | Discussion on R2D design for active Ambient IoT device | ZTE Corporation, Sanechips |
| [12] | R1-2601466 | Discussion on R2D Aspects for R20 AIoT | vivo |
| [13] | R1-2600545 | R2D air interface for Device 2b/C | LG Electronics |
| [14] | R1-2600659 | Study on R2D signals, channels, waveform and procedures | NEC |
| [15] | R1-2600705 | Discussion on A-IoT Air Interface for R2D | Panasonic |
| [16] | R1-2600743 | Study on R2D aspects of air interface for Device 2b/C | Samsung |
| [18] | R1-2600815 | On R2D design details for device 2b/C | Apple |
| [19] | R1-2600965 | Discussion on R2D signals, channels, waveform and procedures | Sharp |
| [20] | R1-2600992 | Discussion on R2D signals, channels, waveform and procedures | ETRI |
| [21] | R1-2601020 | Discussion on R2D for R20 Ambient IoT | Lenovo |
| [22] | R1-2601074 | R2D Design for Active AIoT Devices | InterDigital, Inc. |
| [23] | R1-2601123 | R2D signals, channels, waveform and procedures | Sony |
| [24] | R1-2601167 | Study on R2D design and procedures for Ambient IoT outdoor for active device | NTT DOCOMO, INC. |
| [25] | R1-2601481 | Study of R2D designs for Device 2b/C | Qualcomm Incorporated |
| [27] | R1-2601292 | Discussion on R2D signals, channels, waveform and procedures | Quectel |
| [28] | R1-2601300 | Discussion on R2D signals, channels, waveform and procedures for Ambient IoT | China Telecom |
| [29] | R1-2601338 | Discussion on R2D signals, channels, waveform and procedures for Ambient IoT | TCL |
| [30] | R1-2601368 | On AIoT R2D design for Rel.20 Device 2b/C | Sequans Communications |

Annex A: WID objectives (RP-252964)

Objectives

- Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:

- Deployment scenario 4 with topology 1 with no external carrier wave.

- Traffic types DO-DTT, DT, DO-A.

- Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.

- FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

- Assumption of the same air interface for Device 2b and Device C.

  - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769

- Positioning for outdoor scenarios is under objective 2.

This objective begins after RAN#108 (June 2025)

RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work.

This objective includes:

RAN1-led

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios

  - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios

- Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

- While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming

  - Maximum distance between Reader and Device of 50-500 m

  - Maximum transmit power between -3 dBm and +5 dBm for device C

    - RAN1 to recommend feasible values within this range

  - Maximum transmit power of -20 dBm and -10 dBm for device 2b

Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources.

- Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations.

- Define link budget calculation for coverage.

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

- Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1].

- RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement

- Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work.

- The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement

- This objective does not include the study of dedicated Device procedures for Device localization

- The objective assumes no dedicated positioning architecture will be specified in Rel-20

- The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20

- This objective will start at RAN1#124

Annex B: RAN1 Agreements

Agreements related to R2D Aspects for R20 made in previous meetings are summarized in this section.

R2D waveform and modulation

RAN1#122

Agreement

For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values:

- Addition of M=1

- Addition of M=32

- Removal of M=24

RAN1#122bis

Agreement

The minimum R2D transmission bandwidth of 1 PRB is the baseline for Rel-20.

RAN1#123

Agreement

Regarding removal of M=24 for R2D: Capture following observations in TR 38.769

Sources [Huawei], [Ericsson], [Panasonic], [Xiaomi] and [vivo] state that it is not necessary to keep M value of 24 for R2D.

- Source [Huawei], [Ericsson] and [Panasonic] state and provide evaluation results show that, M=24 cannot work well for outdoor scenarios as its chip length is too short to resist large multipath delay and channel fading.

  - Source [Huawei] shows that M=24 cannot work for outdoor scenarios with BLER close to 1 under different SNRs

  - Source [Ericsson] shows that 10% BLER cannot be achieved with M=24 (even with repetitions)

  - Source [Panasonic] shows that, when M=24 (and M≥12), both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB or larger (even without SFO/CFO)

- Source [vivo] states and provides evaluation results show that, R2D transmission with M=24 cannot achieve 1% BLER even with 2 block-level repetitions, and under similar data rates, the performance of M=24 (with 2 repetitions) is worse than that of M=12 (without repetition)

Sources [Docomo], [Futurewei], [OPPO], [Samsung], [ZTE], [Qualcomm], [CATT] and [Nokia] state that it is necessary to keep M value of 24 for R2D.

- Source [Docomo] states that M=24 can be supported for a device which is close to reader and beneficial for device power saving to reduce PRDCH reception duration with higher peak data rate

- Source [Futurewei] provides evaluation results show that M=24 can still achieve 10% BLER with a reasonable SNR

- Sources [Futurewei], [OPPO] and [Samsung] state that M=24 is needed to achieve data rates similar as R19.

- Source [Nokia] states that, M=24 should not be removed, since it is still useful for indoor scenario, and the air interface for Device 2b/C studied for outdoor scenarios will be reused in indoor scenarios.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

Regarding addition of M=32 for R2D: Capture following observations in TR 38.769

Sources [Futurewei], [Qualcomm], [LGE], [Spreadtrum], [Xiaomi], [Transsion] state it is beneficial to add M value of 32 for R2D.

- Source [Futurewei] states and provide evaluation results shows that, for the TDL-C 300 ns environment, the BLER of M=32 is below 10% when CP handling method is used with 8RB resource allocation;

- Source [Futurewei], [Qualcomm], [LGE], [Xiaomi], [Transsion] and [Spreadtrum] state that, the addition of M=32 could be considered for the purpose of achieving higher data rate. Qualcomm also states that it can increase peak data rate and reduce latency, and active devices can support it for TDL-A fading or LOS channel conditions, with the decision up to reader scheduling.

- Source [Xiaomi] states that, from the energy consumption perspective, M = 32 can enable short transmission duration to avoid interruption.

Sources [vivo], [Huawei], [OPPO], [CATT], [Samsung], [ZTE], [Panasonic], [Docomo] and [Ericsson] state that it is not necessary to add M value of 32 for R2D.

- Sources [vivo], [Huawei], [Samsung], [CATT], [ZTE], [Docomo], [Panasonic], and [OPPO] state concern on feasibility of M=32 due to too short chip duration.

  - Source [Huawei] provide evaluation results show that, M=32 cannot work for outdoor scenarios with BLER close to 1 under different SNRs, as its chip length is too short to resist large multi-path delay and channel fading, so addition of M=32 is not supported.

  - Source [CATT] provide evaluation results show that, for Device 2b, M=32 cannot satisfy the 500m coverage requirement at 10% BLER, and for Device C, though it can reach 10% BLER, it has an error floor above 1% BLER.

  - Source [vivo] provide evaluation results show that R2D transmission with M=32 cannot achieve 1% BLER even with 3 block-level repetitions, and under similar data rates, the performance of M=32 (with 3 repetitions) is worse than that of M=12 (without repetition)

  - Source [Panasonic] provide evaluation results show that, when M =32, both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB and larger due to smaller chip duration.

  - Source [Ericsson] provide evaluation results show that, for Device 2b and Device C, a 10% BLER cannot be achieved with M=32.

- Sources [OPPO] and [Samsung] state that, higher data rate with M=32 lacks motivation.

- [Docomo] states that, it would increase device complexity as devices need to blindly detect and differentiate M=24 and M=32 with certain FDR.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

Regarding Addition M=1 for R2D: Capture following observations in TR 38.769

- Sources [Futurewei], [Spreadtrum], [Nokia], [CMCC], Huawei], [Ericsson], [Xiaomi], [CATT], [Transsion], [LGE], [Panasonic], [Apple], [TCL], [IIT Kanpur], and [Sequans] states that addition of M=1 is beneficial to improve R2D coverage.

  - Source [Huawei] shows that, M = 1 provides ~3 dB performance gain compared with M = 2 using correlation detection with TBS 20 bits and 96 bits, and ~2.5dB performance gain with TBS 400 bits, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to section 4.3.2 of TR 38.769.

  - Source [Panasonic] shows that, M = 1 shows performance gain, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to section 4.3.2 of TR 38.769.

  - Source [CATT] states that, For M=1, the larger chip duration could reduce the DS impact and improve the detection performance for Device 2b/C in outdoor deployment scenario.

- Sources [vivo], [ZTE], [OPPO], [Qualcomm], [DOCOMO] states that addition of M=1 is not necessary.

  - Sources [vivo], [ZTE], [OPPO], [DOCOMO] states that, OOK with M=2 using together with repetition achieve the same data rates as M=1.

  - Source [ZTE] further observed that OOK with M=2 using together with repetition can achieve better time-domain diversity gain and power boosting gain than OOK with M=1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to section 4.3.2 of TR 38.769, OFDM symbol with zero power is not counted.

  - Source [ZTE] states that, OOK4 with M =2 and M=4 have better detection performance than OOK4 with M =1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Section 4.3.2 of TR 38.769, OFDM symbol with zero power is not counted.

  - Source [Qualcomm] shows that, M=1 cannot provide better performance than M=2, if the CINR/CNR for OFDM symbol with non-zero power is calculated according to Section 4.3.2 of TR 38.769, and OFDM symbol with zero power is not counted. Source [Qualcomm] also shows that M=1 provide less than 2dB performance gain than M=2, if the CINR/CNR, for OOK4 signal across all OFDM symbols, is calculated according to Section 4.3.2 of TR 38.769.

R2D channel coding

RAN1#122

Agreement

Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects:

- Coding schemes: convolutional code (e.g TBCC), RM code

  - Note: focus on coding schemes already specified in LTE or NR

- FEC code rates: at least 1/2, 1/3

- Interleaver or LTE bit collection scheme for TBCC

RAN1#122bis

Agreement

Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects:

- Coding schemes

  - LTE TBCC

  - Tailed convolutional code with the same polynomial as LTE TBCC

  - RM code

- FEC code rates: 1/2, 1/3, 1/4

Agreement

For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

RAN1#123

Agreement

Regarding the order of channel coding (with interleaver, if supported) and block-level repetition: Capture following observations in TR 38.769

- Sources [vivo], [Huawei], [NEC], [CATT], [ZTE], [Futurewei], [Apple] and [Qualcomm] report that channel coding before block-level repetition should be supported, with following justifications

  - Sources [vivo], [NEC], [CATT] and [Qualcomm] state that less memory is required by the channel coding before block-level repetition, compared with channel coding after block-level repetition.

  - Source [Huawei] states that the order of the channel coding first, followed by block-level repetition should be supported because of the diversity gain achieved due to the increased distance of a given parity bit in the output stream when compared to the reverse order.

  - Source [CATT] states that block-level repetition before FEC scheme requires Rblock times more interleaver memory as well as higher implementation complexity and processing delay for both devices and reader.

- Sources [OPPO] and [LGE] state that channel coding after block-level repetition should be supported.

Agreement

Regarding R2D FEC schemes, Capture following observations in TR 38.769

If FFC is supported for R2D,

- Sources [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC], [Transsion], [LGE], [Apple], [TCL], [Sequans] [OPPO], [Panasonic], [Qualcomm] report LTE TBCC should be supported.

- Sources [Futurewei], [Transsion] report tailed CC with same polynomial as LTE TBCC can be supported.

- Source [Qualcomm] reports Reed-Muller coding can be supported. Source [CATT] reports Reed-Muller coding can be supported for L1 control, if L1 control is no larger than 11 bits, otherwise, TBCC is supported.

The following are observations regarding comparison of LTE TBCC, Tailed CC with same polynomial as LTE TBCC and Reed-Muller coding

For comparison between LTE TBCC and Tailed CC with same polynomial as LTE TBCC.

- Source [Futurewei] states, for the same number of information bits N, a tailed convolutional code encodes L-1 additional bits (tail bits) to ensure the final state returns to 0. With tail bits, a typical decoder can operate on N+L-1 bits. With TBCC, no additional bits are needed but a typical decoder operates on at least 2N bits. For moderate and larger number of information bits, tailed convolutional codes and TBCC have the same performance. For smaller number of information bits, with additional decoding iterations, the performance of TBCC can approach the performance of tailed convolutional codes.

- Source [vivo] states, compared with LTE TBCC, tailed convolutional code can reduce decoding complexity while increase the overhead.

- Source [Huawei] states tailed convolutional codes with the same polynomial as LTE TBCC has a performance similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits.

- Source [TCL] states, the decoding for Tailed convolutional code maybe simpler, but the tail bits need to be sent, and the actual effective code rate slightly decreases

For comparison between LTE TBCC and RM

- Source [vivo] states RM code suffers performance degradation and larger complexity at least for larger transport block size, without power reduction benefit

- Source [CMCC] states, RM code is deprioritized due to the restricted operational range

- Source [Huawei] states RM codes have a 4-5 times higher power consumption than convolutional codes due to the complicated fast Hadamard transform calculation for the FEC decoder, and has a 3-11 bit TBS restriction in the NR baseline design.

- Source [TCL] states the combinations of code length and code rate for RM code are limited, which is less flexible than convolutional codes. And the evaluation for the performance of the RM code is needed under low signal-to-noise ratio conditions.

- Source [Qualcomm] states RM codes performs worse than TBCC. RM with k=8 or 10, n=32 has lower decoding complexity, memory cost, hardware area cost and lower power consumption than Viterbi decoding for TBCC with 1/2 or 1/3 coding rate.

- Source [Docomo] states decoding complexity of RM code is expected to be lower than Viterbi decoding for TBCC. RM is not supported for payload size larger than 11.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

Regarding interleaving and LTE bit collection schemes for R2D, capture following observations in TR 38.769

Sources [vivo], [Huawei], [Qualcomm], [Docomo] and [ZTE] state that it is beneficial to support interleaving to improve coverage by obtaining time diversity gain.

Sources [vivo], [Huawei] and [Docomo] state it is feasible to support LTE subblock interleaving and LTE bit collection.

- Source [vivo] report with references to show that the power consumption for LTE de-interleaving for turbo decoding is tens of uW, even with multiple parallel (de)interleaving operation, and each operation with max number of bits of 6144. It can be expected that the power consumption for TBCC with LTE bit collection and with or without LTE sub-block interleaving can be even simpler. Hence, both LTE TBCC with bit collection with or without LTE sub-block interleaving are feasible.

- Source [Huawei] and [Docomo] report that the additional power consumption can be supported by active devices due to higher peak power consumption, allowing them to support larger memory. Source [Huawei] further report that larger memory would enable these devices to support the use of a LTE de-interleaver, and can follow the same hardware as used for the LTE sub-block interleaver with LTE bit collection scheme in D2R.

The following are observations regarding comparison of three alternatives.

For Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Sources [Futurewei], [Huawei], [OPPO], [CATT], [LGE] and [Panasonic] report LTE sub-block interleaver and LTE bit collection scheme can be supported, if FEC is supported.

  - Source [Futurewei] states that, reusing both LTE sub-block interleaver and LTE bit collection scheme enables a device to eliminate the repetition block and associated memory as the bit collection scheme can use the memory for the sub-block interleaver for repeated bits. Another benefit is the various coding rates are achievable without modifying the encoder specification.

  - Source [Huawei] states that, reusing the LTE sub-block interleaver and LTE bit collection scheme should be supported due to its performance gain especially for larger TBS transmissions, providing 1dB and 2.8 dB performance gain compared with Alt 2 for BLER 10% and BLER 1% respectively, for a TBS of 400 bits.

  - Source [LGE] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort

  - Source [Panasonic] states that, LTE sub-block interleaver could be beneficial for R2D to reduce the burst error caused by fading, especially for the longer message size with smaller values of M

  - Source [Qualcomm] states that, Alt 1 and Alt 2 can achieve similar BLER performance, and both require buffering full-length coded bits.

For Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Sources [vivo], [NEC], [ZTE], [LGE], [Apple] and [Qualcomm] report LTE bit collection scheme without LTE sub-block interleaver can be supported, if FEC is supported.

  - Source [vivo] states that, Alt 1 and Alt 2 presents similar performance; thus Alt 2 is sufficient.

  - Source [ZTE] states that, since bit collection does not require interleaving, the complexity of Alt 2 is lower than interleaving. FEC with bit collection and FEC with sub-block interleaver has similar performance compared with FEC with bit collection.

  - Source [LGE] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort

  - Source [Apple] states that, Alt 2 provides a good balance between performance and complexity for device 2b/C, reusing existing 3GPP designs while minimizing memory requirements.

  - Source [Qualcomm] states that, for small number of coded bits, Alt2 can be used to improve interleaving gain

For Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Source [Qualcomm] and [NEC] report that, LTE bit collection scheme with segment can be supported, if FEC is supported.

  - Source [NEC] states that, Alt 3 can be adopted if memory size limitation exists in devices

  - Source [Qualcomm] states that, Alt3 with two segmentations requires half of the buffering size versus Alt2 at price of 2.5dB performance loss, but still outperforms the no-FEC approach using three block-level repetitions. For large number of coded bits, Alt3 can be applied to reduce buffering requirements

  - Source [vivo] and [Huawei] state that, Alt 3 degrades performance due to lower time diversity gain compared with Alt 1/2, and [vivo] observes that, Alt 3 with two segments for 1000 bits TBS also suffers loss compared with block-level repetition.

  - Sources [vivo] and [Huawei] state that, since memory/power consumption by Alt 1 and Alt 2 is feasible for AIoT device, it is unnecessary to further reduce memory/power consumption by Alt 3.

  - Source [LGE] states that, Alt 3 is potentially advantageous in terms of memory reduction for A-IoT devices, but is considered less favourable due to the extensive discussion required to evaluate the performance of various bit collection schemes

Agreement

Capture following observations in TR 38.769:

For LTE TBCC Sources [vivo], [Huawei], [Qualcomm], [NOKIA], [CATT], [ZTE] and [Apple] report that the power consumption for TBCC decoding, as follows

- Source [vivo] states power consumption is tens of uW to few hundreds of uW

- Sources [Huawei] and [ZTE] state power consumption is less than 200uW.

- Source [NOKIA] states power consumption can be less than 100uW.

- Source [CATT] states power consumption is about 100uW

- Source [Qualcomm] states power consumption is tens of percent higher than a few mWs or less, dependent on the clock frequency, with constraint length K=7.

- Source [Apple] states power consumption is tens of uW.

Sources [Futurewei] and [Qualcomm] also provide complexity and memory cost analysis for LTE TBCC

- Source [Futurewei] states that, the decoding complexity depends on number of bits, constraint length K and number of iterations.

- Source [Qualcomm] states that, the decoding complexity and memory depend on constraint length K and trackback depth D.

For Tailed convolutional code with the same polynomial as LTE TBCC.

- Source [Futurewei] states the complexity is no larger than LTE TBCC.

- Source [Huawei] states that its performance is similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits.

For RM

- Sources [Qualcomm], [Docomo] and [Xiaomi] state memory and power consumption for RM is lower than TBCC, and it is feasible.

  - Source [Qualcomm] further state that, the decoding complexity, power consumption, memory cost and hardware area cost of RM code is lower than that of LTE TBCC, and suitable for ultra-low power A-IoT devices.

- Sources [Huawei] and [Transsion] state that, RM decoding based on fast Hadamard Transform incurs higher power consumption, compared with the short-constraint length convolutional codes. Source [Transsion] states that, the power consumption for RM decoding may not meet power consumption limit of device 2b/C.

Agreement

Regarding the code rate for FEC: Capture following observations in TR 38.769

For R2D FEC, code rate 1/2, 1/3 and 1/4 are studied

For Code rate R=1/3

- Sources [vivo], [NOKIA], [CMCC], [Huawei], [NEC], [LGE], [Panasonic], [Apple], [ZTE], [Qualcomm] and [TCL] state R=1/3 can be supported.

  - Source [CMCC] states that, 1/3 code rate is supported when interleaving is not applied.

  - Source [Huawei] states that, 1/3 code rate is necessary and feasible, and should be supported due to its performance gain and an increased power consumption for active devices.

  - Source [LGE] states that, it is beneficial to reuse the LTE TBCC with a preferred code rate of 1/3 to ensure reliable transmission and reduce specification complexity

  - Source [Panasonic] states that, for Device C, LTE TBCC with a coding rate of at least 1/3 could be beneficial to support for better coverage.

  - Source [Qualcomm] states that, LTE TBCC with a coding rate of 1/3 could achieve better performance than 3 repetitions.

For Code rate R=1/2, achieved by puncturing 3rd branch

[Positive views]

- For Code rate R=1/2, sources [vivo], [NOKIA], [NEC], [CATT] and [Apple] report that R=1/2 can be supported.

  - Source [vivo] reports, 1/2 code rate can improve spectrum efficiency and reduce power consumption for decoding.

  - Source [CATT] states that ,1/2 coding rate is preferable for TBCC, as it reduces device power consumption while achieving the same spectral efficiency as the Manchester coding in Rel-19 R2D.

  - Sources [Apple] and [NEC] state that, 1/2 and 1/3 coding rate as baseline.

  - Sources [Futurewei] state that, the performance difference between R=1/2 and R=1/3 is about 2 dB.

[Negative views]

- Source [Huawei] states that, the necessity of supporting other code rates apart from 1/3 for TBCC, which was supported in D2R for Rel-19, is not clear, because a 1/2 code rate performs worse than a 1/3 code rate

For Code rate R=1/4, source [Docomo] reports that Code rate of R=1/4 can be achieved by repetition and puncturing based on LTE TBCC with 1/3 code rate.

For Code rate R=1/4, obtained by adding a new polynomial for the fourth branch

[Positive views]

- Sources [Qualcomm], [Docomo] and [Apple] report that, Code rate R=1/4 obtained by adding a new polynomial for the fourth branch, can be supported.

  - Source [Apple] states that, 1/4 coding rate can be considered for extreme coverage scenarios.

  - Source [Qualcomm] states that, 1/4 coding rate can further improve the performance than that of 1/3 coding rate. The extra polynomial [133 171 165 137], nested to the existing LTE CC (K=7 and R=1/3), is used

  - Source [Futurewei] states that, the performance difference between R=1/3 and R=1/4 is about 1.25 dB.

[Negative views]

- Source [vivo] states that, 1/4 mother code rate increases power consumption and larger buffer for decoding, and it requires large standard effort to determine new polynomial for the 4th branch. The coverage enhancement can also be achieved by 1/3 mother code rate with block-level repetition.

- Source [NOKIA] states that, 1/4 code rate should be deprioritized since it increases complexity.

[Coding scheme assumption]

Source [Futurewei] provide views for above code rates using tailed convolutional code with the same polynomial as LTE TBCC; while other sources provide views for above code rates using LTE TBCC.

Note: whether/how to capture blue parts can be up to editor

R2D repetition

RAN1#122

Agreement

Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types:

- Block-level repetitions

- Bit-level Type-2 repetitions

- Chip-level repetitions (in case of M=1 with FEC if applied)

RAN1#123

Agreement

For R2D Repetition, block-level repetition is considered for R20 AIoT.

- bit-level and chip level repetition are not considered for further study.

Agreement

Regarding R2D Repetition, Capture following observations in TR 38.769

Regarding R2D repetition, it is reported by sources [Futurewei], [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC],  [OPPO], [Transsion], [LGE], [Apple], [Qualcomm], [CEWiT], [Sequans], [Panasonic], [Quectel], [NTT DOCOMO] and [IIT Kanpur] state that R2D repetition is feasible and can be considered. Sources [vivo], [Huawei], [CMCC], [OPPO], [CATT], [Transsion], [Apple], and [IIT Kanpur] report that repetition schemes including block-level repetition are feasible.

R2D Manchester coding

RAN1#123

Conclusion:

There is no need to remove Manchester coding for Rel-20 AIoT PRDCH.

R2D signal functionalities

RAN1#122

Agreement

For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed):

- R2D chip duration determination

- indication of the start of R2D

- indication of the end of PRDCH

- frequency synchronization (including frequency acquisition)

- CFO estimation / LO calibration

- SFO calibration

- timing synchronization/tracking

- measurement (e.g., signal strength)

- reader identification/differentiation

R2D signal for start indication

RAN1#122bis

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1: binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

Agreement

For the study of R2D binary sequence-based SIP, study the following aspects:

- Sequence type

  - Option 1: m-sequence

  - Option 2: Golay sequence

- Length of sequence

- Number of sequence(s)

- FFS: whether to apply Manchester coding to the sequence

- Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

RAN1#123

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH,

- At least, it is feasible to use binary sequence-based SIP (i.e., m-sequence or Golay sequence), and the binary sequence-based SIP can be used for timing synchronization/tracking.

- Regarding Rel-19 SIP,

  - Source [CATT] provide evaluation results show that R-TAS SIP defined in Rel-19 A-IoT can satisfy the detection requirements of target MDR of 10% for the FAR up to 1% in outdoor scenarios

  - Source [vivo, Huawei, Qualcomm, ZTE] provide evaluation results to show R19 SIP cannot be reliably detected.

    - Source [vivo] show that R19 SIP does not outperform sequence-based SIP in MDR, even if R19 is using sequence correlation detection instead of edge detection in R19.

    - Source [Qualcomm] shows that, for Rel19 SIP using energy detection (Option 2), the required CNR at BLER=10% is about 8dB higher than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control), indicating that Rel19 SIP cannot be reused and enhancement is needed.

    - Source [Huawei] show that if the same R-TAS design is used for a device C using correlation detection, the required SNR is 17.0 dB to achieve MDR ≤ 1% and FAR ≤ 1%, which is also not sufficient for the coverage enhancement for outdoor scenarios

    - Source [ZTE] provide evaluation results shows that the Release 19 SIP sequence performance at BLER=1% cannot satisfy the coverage requirements, while the performance at BLER=10% can satisfy the coverage requirements.

Agreement

Capture following observations for sequence type for R2D SIP in the TR 38.769, that

For binary sequence based R2D SIP, m-sequence and Golay sequence are studied.

- Sources [Qualcomm] and [interdigital] provide evaluation results show that m-sequence and Golay sequence shows similar MDR/FAR and correlation property. [Docomo] states that both m-sequence and Golay sequence have good auto-correlation property.

- Sources [vivo], [OPPO], [Apple], [interdigital], [Qualcomm] and [LGE] report that m-sequence can be considered as binary sequence for R2D SIP, with the following observations:

  - Source [vivo] states that m-sequences with same length but different cyclic shifts/initial states can be considered to achieve good cross-correlation between different sequences.

  - Sources [interdigital], [LGE] and [Qualcomm] state that m-sequence design can be expected to be implemented with lower complexity than a Golay sequence. Source [Qualcomm] further state that m-sequences can be generated by storing the initialization state of the polynomial, reducing memory requirements.

- Sources [Huawei], [Qualcomm], [Docomo], [LGE] report that Golay sequence can be considered as binary sequence for R2D SIP, with the following observations:

  - Source [Huawei] states that Golay sequence achieves lower operational and correlation complexity compared with m-sequence.

  - Source [Docomo] states that Golay sequence with flexible sequence length may be beneficial for alignment with the OFDM symbol boundary.

  - Sources [Qualcomm] and [LGE] states that Golay-sequence offers slightly better cross-correlation than m-sequence, and source [Qualcomm] further states cross-correlation depends on the choice of Golay pairs, and Golay sequences may be programmed into devices.

Agreement

Capture following observations for number of sequences for binary sequence based R2D SIP in the TR 38.769, that

For number of sequences for binary sequence based R2D SIP

- Sources [Futurewei], [vivo], [Nokia], [Huawei], [Xiaomi], [Panasonic], [Lenovo] and [Qualcomm] report that multiple sequences can be used for a given length for R2D SIP.

  - Sources [vivo], [Panasonic], [Lenovo] report the purpose of multiple sequences is for reader differentiation, Sources [vivo] and [Panasonic] suggest to consider 3 sequences.

  - Source [Huawei] report that different length sequences are mapped to different coverage distances, in order to maximize the performance for each of the distances for outdoor scenarios while maintaining MDR and FAR to be ≤ 1%.

  - Sources [Xiaomi], [Lenovo] report the purpose of multiple sequences is to indicate parameters of upcoming R2D transmission. [Xiaomi] [Lenovo]report that presence of CFO calibration signal can be indicated by SIP sequences, and [Lenovo] also report that inclusion or exclusion of L1 control information, synchronization signal, or PRDCH, can be indicated by SIP sequences.

- Source [NEC] report that single length and single sequence is beneficial for lower complexity and power consumption.

- Source [Docomo] report that device complexity on correlation-based detection for multiple sequences should be considered.

Agreement

Regarding whether to apply Manchester coding to the binary sequence-based SIP, Capture following in TR38.769.

Sources [vivo], [CTC], [Qualcomm] and [Sony] report that Manchester coding can be applied to binary sequence for R2D SIP

- Source [vivo] and [Qualcomm] state that sequence Manchester coding achieves similar MDR and timing performance.

  - Source [vivo] provide evaluation results show that for 31-bit m-sequence with Manchester coding, 31-bit m- sequence without Manchester coding, and 63-bit without Manchester coding, these three cases have comparable durations (~15 OFDM symbols), and shows similar MDR performance.

  - Source [Qualcomm] provide evaluation results show that the 15-bit m-sequence with Manchester coding and the 31-bit m sequence without Manchester coding have comparable durations (~7.5 OFDM symbol), and both show similar MD performance.

- Source [Sony] states that applying Manchester coding to the sequence is important in OOK modulation with low data rate where a long number of zeros and ones may result in the clock drift.

- Source [vivo] also states that timing performance for sequence with Manchester coding is slightly better compared with longer sequence w/o Manchester coding (same overall duration), due to narrower main lobe of auto-correlation can be achieved with Manchester coding.

Source [Futurewei] states that the disadvantages of applying Manchester coding include increasing complexity and decreasing the number of sequences.

Agreement

Capture following observations for sequence length for binary sequence based R2D SIP in the TR 38.769, that

For sequence length for binary sequence based R2D SIP

- Source [vivo] report that

  - At least 31-length m-sequence with M=2 and with Manchester coding (i.e., 31 OFDM symbols), is needed for R2D SIP, assuming 38 dBm Tx power and 20 dB penetration.

  - At least 127-length m-sequence with M=2 and with Manchester coding (i.e., 127 OFDM symbols), is needed for R2D SIP, assuming 33 dBm Tx power and 20 dB penetration.

- Source [Huawei] report that

  - 256, 64 and 16-length Golay sequence-based SIP can achieve an SNR performance of 4.0 dB, 8.2 dB and 12.0 dB, respectively, with the MDR ≤ 1%, FAR ≤ 1% and M=1 without Manchester coding.

- Source [Apple] report that

  - Consider m-sequence based SIP with length 15 or 31 to improve detection performance while maintaining reasonable overhead

- Source [Qualcomm] report that SIP duration with at least 7 OFDM symbols by using 15-length and M=2 without Manchester coding or 31-length and M=2 with Manchester coding may be needed to achieve required SINR for PRDCH.

- Source [NEC] report that single length and single sequence is beneficial for lower complexity and power consumption.

- Source [ZTE] report that the Release 19 SIP sequence with a length of 8 can achieve a BLER of less than 10% at an SNR of 4 dB, and BLER of less than 1% at an SNR of 10 dB.

R2D signal for chip duration determination

RAN1#122bis

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

Agreement

Regarding whether to apply same or different chip duration for L1 R2D control and data payload, Capture following in TR38.769.

Sources [Spreadtrum], [Huawei], [Ericsson], [Samsung], [LGE], [Apple] and [Sequans] report that, the use of the same chip duration for L1 R2D control information and data payload in the PRDCH, have the following benefits:

- Sources [Spreadtrum], [Huawei], [Samsung], [Sequans] and [Apple] state that the use of the same chip duration simplifies device implementation, and [Huawei] further states that the L1 R2D control information can avoid any decoding delay and avoid the need for a time gap between the L1 R2D control information and the data payload of the PRDCH.

- Sources [Ericsson] states that, for broadcast PRDCH and paging PRDCH, the R2D parameters, including chip length for data payload of PRDCH, can be fixed for cater to the cell-edge scenarios.

- Sources [Samsung], [Sequans] and [LGE] state that using the same M value for the L1 R2D control part and the data payload avoids additional signaling overhead, and [Samsung] [Sequans] further states that this ensures consistent coverage for both control and data, and no clear benefit to use different chip length for L1 control part and data part.

Sources [Futurewei], [vivo], [CMCC], [Ericsson], [NEC], [CTC], [Xiaomi], [ZTE], [LGE], [Panasonic],[Docomo], [Apple], [Qualcomm] and [CEWiT] report that, chip duration of the data payload in the PRDCH indicated by the corresponding L1 R2D control information, have the following benefits:

- Sources [Futurewei], [vivo], [CMCC], [NEC], [CTC], [Xiaomi], [LGE], [ZTE], [Panasonic], [Docomo] and [CEWiT] state that chip duration indication through L1 control information provides better flexibility to meet diverse reliability and data rate needs. [ZTE] further states that it better coordinates with TB size, repetition rate, and code rate for improved link adaptation; [Futurewei] states that it allows tailoring data rates for specific devices and can improve efficiency;

- Source [vivo] state that with better coverage for L1 control part using longer chip length, unnecessary data reception can be avoided due to more robust L1 control;

- Sources [Apple] and [Ericsson] state that this option provides flexibility to adapt to different coverage conditions or message types, with [Ericsson] states this option provides benefits for PRDCH like Msg2 and Msg4.

Agreement

Capture following in TR 38.769

For the functionality of chip duration determination of the L1 R2D control information, the following options are studied

For option 1, Fixed chip duration. No use of CAP for chip duration determination.

[positive observations]

- Sources [Ericsson], [CATT], [LGE] and [CTC] states that fixed chip duration for L1 R2D control information lowers the complexity and power consumption of chip duration detection.

- Source [CATT] states that fixed chip duration for L1 R2D control information minimize the ambiguity of the chip duration detection, and thus enhance the success rate of L1 R2D control information reception.

- Source [Ericsson] states that using fixed chip duration (using the smallest M value for optimal coverage) can achieve higher reliability for L1 R2D control information.

[Negative observations]

- Source [Qualcomm] and [Samsung] states that to achieve the largest coverage, L1 control would need to use the lowest M, which increases overhead for each R2D transmission.

- Sources [FUTUREWEI], [Samsung], [ZTE], [Huawei], [vivo], [Docomo] and [CMCC] states that, if chip duration for L1 control information is fixed, the flexibility or transmission efficiency is restricted.

For option 2, Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

[positive observations]

- [CMCC] and [Panasonic] state that blind detection within configured set provide better flexibility for chip length for L1 R2D control information.

- [CMCC] states that blind detection is viable if the number of predefined chip duration candidates is restricted. Additionally, [CMCC] states that Option 2 can be combined with device-specific configuration signaling to enhance flexibility without excessive complexity.

[Negative observations]

- Sources [FUTUREWEI], [Samsung], [vivo], [DOCOMO], [Ericsson] and [ZTE] state that Option 2 increases detection complexity.

- Source [Qualcomm] states that it may be impractical due to constraints on power consumption and memory needed for buffering the incoming R2D L1 control information samples.

- Sources [Huawei] and [LGE] state that option 2 would require additional time gap between the L1 control information and the data payload of the PRDCH to allow for decoding delay.

- Sources [vivo] and [Ericsson] state that reliability is doubtful due to blind detection.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway to achieve flexibilities in coverage and resource overhead, no need to leave chip duration for blind detection.

For option 3, Use CAP

[positive observations]

- For Alt.3-1 Rel-19 CAP pattern, Sources [Samsung] and [Sequans] state that Rel-19 CAP pattern to determine chip length of L1 R2D control part offer a balance between detection complexity and performance.

- For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, Sources [LGE], [Apple] and [Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Apple further states that this option maintains consistency with Release-19 design principles. Source [Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility.

[Negative observations]

- Source [Qualcomm] states option 3 requires blind detection; and MD/FA rates and CAP overhead needs to be considered.

- Source [ZTE] states option 3 results in additional overhead due to transmission of CAP.

- Sources [CMCC], [vivo] and [Huawei] state that Option 3 is based on edge detection to determine the chip duration by using CAP, so the performance is limited or unfeasible due to the use of edge detection to detect the CAP pattern. Sources [ZTE] and [Ericsson] also report the performance of this option should be further evaluated.

- Source [Ericsson] states that option 3 would make the decoding of the L1 R2D control information more complicated for the device.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection using CAP.

For Option 4, Use a set of different binary sequences with fixed or variable length

[positive observations]

- For Alt 4-1, using binary sequences for SIP. No use of CAP for chip duration determination

  - Sources [Nokia], [Huawei], [NEC], [LGE], [Docomo] and [Panasonic] state that this method can provide flexibility for R2D chip duration.

  - Source [Huawei] state that a restricted set of binary sequences of differing lengths are mapped to different chip durations and coverage distances, which can be detected with limited blind detection complexity. Parallel processing for the blind detection of different sequence lengths avoids and processing delay. Besides, R-TAS design is simplified, which consist only sequence-based SIP.Alt.4-2: using binary sequences for CAP.

  - Sources [Nokia] and [Panasonic] report that flexibility on chip length of L1 R2D control can be achieved.

[Negative observations]

- Sources [vivo], [Ericsson] and [Qualcomm] states that option 4 increases detection complexity. And Ericsson states that option 4 would make the decoding of the L1 R2D control information more complicated for the device.

- Sources [ZTE] and [Ericsson] report that the performance of this option should be further evaluated.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection; vivo further report that it leader more challenges on preamble design, requiring larger number of sequences to indicate four M values in addition to reader/cell ID differentiation;

For option 5, Use broadcast information (if any) to indicate the chip duration

[positive observations]

- Sources [vivo], [ZTE] and [Qualcomm] state that better flexibility can be achieved without blind detection, and sources [vivo] and [ZTE] further state that the chip duration for L1 can be indicated together with other parameters for control information, e.g., repetition and FEC related parameters. Source [CMCC] state that this method can be used for L1 R2D control information common for devices, for example, the L1 R2D control information for scheduling paging message.

- Source [OPPO] state that the chip length should be pre-known by device to minimize complexity and power consumption at AIoT device.

[Negative observations]

- Source [Qualcomm] states that it requires system info acquisition and memorization.

- Source [Huawei] and [Samsung] states that only to indicate chip duration in broadcast information limits flexible time resource allocation from system perspective.

- Source [Ericsson] states that this option is a simple solution but not applicable for paging and broadcast PRDCH.

- Source [Docomo] states that state that option 5 has less flexibility on chip duration of L1 R2D control compared to option 2/3/4.

Note: whether/how to capture blue parts can be up to editor

R2D signal for end indication

RAN1#122bis

Agreement

For the functionality of indicating the end of the PRDCH, study the following option:

- Option 1: Use the L1 R2D control information to indicate the TBS.

R2D signal for CFO estimation/LO calibration at device

RAN1#122bis

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

- Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

RAN1#123

Agreement

For Option a: unmodulated sinusoid single tone

Sources [Futurewei], [Spreadtrum], [vivo], [CMCC], [Huawei], [Ericsson], [NEC], [CTC], [Xiaomi], [Samsung], [ZTE], [LGE], [Lenovo], [Apple], [MTK], [Sharp], [Quectel], [Qualcomm], [Docomo], [TCL], [IIT Kanpur], [CEWiT], [Sequans], [interDigital], and [Sony] state that, option-a unmodulated sinusoid single tone is feasible, and can be considered for CFO calibration signal.

[Working principle]

- Sources [vivo] and [Huawei] and [Docomo], report with references, and state that, the CFO calibration is based on injection lock mechanism, and the working principle of injection lock is to leverage the phase interaction between an external single-tone signal and the internal oscillation signal of an oscillator to lead the oscillator’s frequency lock to that of the unmodulated sinusoid single-tone signal.

- Source [CMCC] report with simulation result, and states that, CFO calibration using unmodulated sinusoid is performed at intermediate frequency, where the reader transmits the CFO calibration signal with a pre-defined duration, the device mixes the CFO calibration signal with its internally generated wave, can then count the number of periodicities of the residual frequency within the duration to estimate and calibrate the CFO.

[Positive views]

- Source [CMCC] report that with a duration of 1 OFDM symbol, the CFO after calibration can be reduced to less than 10 ppm.

- Sources [Futurewei], [Ericsson], [CTC], [Samsung], [ZTE], [LGE], [Qualcomm], [Lenovo] and [IIT Kanpur] state that unmodulated sinusoid single tone can achieve CFO calibration with low complexity.

- Source [vivo] states that single tone signal is already supported in NB-IoT, (e.g., NPRACH), there is no co-existence issue.

[Negative views]

- Source [CATT] states that it will create the inter-channel interference to neighboring NR sub-bands.

- Source [Qualcomm] states that this method may be vulnerable to deep fading, reducing reliability.

For option b, unmodulated multiple sinusoid single tones

Sources [LGE], [Qualcomm] and [TCL] report that option b con be considered for CFO calibration

[Positive views]

- Source [LGE] states that, option b provides frequency diversity gain while maintaining low complexity; considered as optional feature. It is preferable that the tones are sufficiently separated in frequency, e.g., by at least the Rx filter bandwidth.

- Source [Qualcomm] states that, using multiple sinusoidal tones can improve robustness against fading by providing frequency diversity.

- Source [TCL] states that, although option b is slightly more complex than a single tone, it is still a very simple waveform.

[Negative views:]

- Sources [Futurewei], [vivo], [Spreadtrum], [Sony], [TCL] and [Quectel] state that option b increases in detection complexity, and it may also complicate frequency resource allocation.

- Sources [vivo] and [Huawei] raised feasibility concerns on option b

  - Source [vivo] states that even with increased complexity, it is not clear how does it work, since frequency of each tone cannot be accurately determined, due to only frequency range of each can be determined if captured by Rx filter, and the target frequency for LO calibration cannot be accurately determined.

  - Sources [Huawei] and [NEC] state that multiple frequency components would occur when mixed with the local RF frequency signal and it is difficult to lock the LO frequency.

- Source [Sequans] state that multiple unmodulated tones increase PAPR and require FFT-based tone separation

For Option c, modulated multiple tones, 3 different designs are reported by companies

[Working principle]

- Source [Ericsson] state that the unmodulated multiple tones are transmitted in different frequency locations in different time resources

  - The reader transmits the synchronization signal using a centered bandwidth sweeping method, where each centered bandwidth covers a portion of the overall synchronization signal bandwidth

  - The device detects the synchronization signal power at different strengths on each of the subsets and based on the subset with the strongest synchronization signal power, the device applies a frequency correction

- Sources [CATT] and [MTK] report that, CFO calibration is done in baseband domain, and CFO Calibration is OOK sequence (Not R19 SIP or CAP). The CFO pre-compensation method is given as follows:

  - Step 1: The received signals with pre-compensated CFO ±Δf is added in the received R2D signals to limited the frequency offset within ±1/2 Δf.

  - Step 2: The correlation processing of the CFO calibration signal with the received signals, the precompensated received signals with additional Δf, and the pre-compensated received signals with additional-Δf.

  - Step 3: The peak of the correlation values from the three correlation outputs would be selected as the received signals for the R2D signal processing.

- Sources [Docomo] states that, the CFO calibration can be achieved by measuring chip length

  - Source [Docomo] states that one possible implementation is devices calibrate CFO based on SFO calibration by counting the number of samples of 0s and 1s and comparing with the defined duration of a pattern.

[Positive views]

- Source [Ericsson] state that using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm)

- Source [CATT] state that the OFDM waveform-based CFO calibration signals would be orthogonal to neighboring NR channels

- Sources [CATT] and [MTK] state that R2D synchronization signal can be used for CFO calibration signal

- Source [Docomo] state that may be feasible if same clock (LO) is used for both carrier frequency generation and sampling.

[Negative views]

- Source [Ericsson] state that the CFO calibration of device C may require using unmodulated sinusoid single tone to meet a residual CFO of 10 ppm.

- Sources [Futurewei], [vivo], [CMCC] and [CEWiT] state that the CFO hypothesis at AIoT receiver cannot be constructed in baseband due to phase information for received signal is not kept in envelop detector, and no I/Q path in receiver baseband.

- Source [ZTE] report that, in multi-tone LP-SS discussion, the LP-WUR doesn’t have the capability to acquire good frequency calibration, similarly this method is not reliable.

- Sources [Huawei] and [NEC] state that the LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure and the calibration signal should be utilized for the RF-end clock.

- Source [vivo] state that using ON chip duration to calibration Clock it not feasible, since according to RAN4 requirements on pulse width, Table 6.4.2-1 in section 6.4.2 of Ts 39.194, the pulse of a chip, the allowed chip length can be <=1.3 Tc when the nominal chip length is Tc, in this case, it is not feasible to use chip length/pulse width for CFO calibration.

- Source [Lenovo] states that pulse width distortion analysis is possible to be used for A-IoT device due to simplicity, but timer resolution at device is required to be much less than the difference between estimated pulse width and the transmitted pulse width.

Note: whether/how to capture blue parts can be up to editor

Agreement

Regarding whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically), capture following in TR38.769

[Periodic transmission CFO calibration signal]

Sources [Futurewei], [vivo], [CMCC], [Huawei], [CATT], [CTC], [Xiaomi], [ZTE], [Apple], [interdigital], [MTK], [Quectel], [Sequans], [Apple], [Qualcomm], [TCL], [Lenovo] and [CEWiT] state that the CFO calibration signal is periodically transmitted.

[Positive observations]

- Sources [vivo], [CMCC], [CTC] and [Sony] state that, CFO calibration signal with hundreds ms or 1 second periodicity is already sufficient. Aperiodic CFO calibration signal, e.g., on demand transmission with PRDCH is not needed due to slow frequency drift. Source [Ericsson] also states that the CFO calibration signal can be transmitted in very sparse manner.

- Sources [CMCC], [Huawei], [CTC], [Lenovo] and [Quectel] states that, to support DO-A traffic, periodic CFO calibration signal is needed.

  - Source [CMCC] states that, periodic CFO calibration is needed to avoid a device from keep monitoring the CFO calibration signal before transmitting the very first D2R transmission for DOA traffic.

- Source [MTK] and [Spreadtrum] states that it is beneficial to provide regular opportunities for devices to perform frequency calibration and maintain accurate synchronization with the reader.

[Aperiodic transmission CFO calibration signal]

Sources [Huawei], [CATT], [Xiaomi], [ZTE], [LGE], [MTK], [Quectel], [Spreadtrum], [Lenovo], [Apple], [Sharp], [Qualcomm], [IIT Kanpur], [Samsung] and [CEWiT] state that the CFO calibration signal can be transmitted aperiodically. In which, Sources [Huawei], [CTC], and [Quectel] state that Aperiodic CFO calibration signal can be considered with periodic CFO calibration signal as baseline.

[Positive observations]

- Source [Huawei] state that with the DRX configuration, the device will miss several instances of the periodic CFO calibration signal. In this case, the CFO calibration signal can be considered to be transmitted as needed, e.g., transmitted after the PRDCH carrying the paging message.

- Source [CATT] states that the R2D calibration signal should be transmitted before R2D reception and D2R transmission. Since R2D reception and D2R transmission may be either periodic or aperiodic, both periodic and aperiodic CFO calibration signals should be supported.

- Sources [ZTE], [LGE], [Lenovo], and [MTK] states that, at least for D2R triggered by PRDCH, the on-demand transmission of CFO calibration signals is beneficial to keep the interval between the CFO calibration signal and the subsequent D2R transmission short for better CFO calibration accuracy.

[Negative views]

- Source [vivo] states that if CFO calibration totally relying on aperiodic signal, the CFO calibration accuracy cannot be guaranteed, which depends on traffic.

Note: whether/how to capture blue parts can be up to editor

R2D signal for frequency and timing synchronization

RAN1#122bis

Agreement

Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking.

- Alt1: periodic synchronization signal

- Alt2: aperiodic synchronization signal

RAN1#123

Agreement

At least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization, the periodic synchronization signal transmission is necessary.

- Default transmission periodicity is predefined

- FFS (in SI or WI): whether synchronization signal transmission is strictly periodic at reader

Agreement

R2D periodic synchronization signal is a binary sequence in time domain

- The binary sequence reuses Rel-19 R2D waveform

- The binary sequence has a pre-defined chip length

- Device assumes that the binary sequence has a predefined length

- The number of binary sequence(s) is less than or equal to 4

  - If more than 1 sequence is supported, it is for potential reader identification/differentiation

- Study whether Manchester coding is applied to the binary sequence

- Study necessity of other aspects, including overlaid sequence

R2D signal for SFO calibration

RAN1#122bis

Agreement

For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options:

- Option 1: Reuse CAP and/or Manchester encoding

- Option 2: sequence-based SIP with or without R2D midamble(s)

- Option 3: using the CFO calibration signal

RAN1#123

Agreement

Following update is applied to the agreement made at RAN1#122bis.

Agreement For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options: - Option 1: Reuse CAP and/or Manchester encoding - Option 2: sequence-based SIP with or without R2D midamble(s) - Option 3: using the CFO calibration signal

Agreement

Regarding the functionality of SFO calibration at device: capture following in TR 38.769

- It is reported by sources [Futurewei], [Spreadtrum], [vivo], [Huawei], [CTC], [CATT], [Samsung], [ZTE], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device 2b. And it is reported by sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device C.

- Sources [Spreadtrum], [Huawei], [OPPO], [Samsung], [ZTE], [Apple], and [Quectel] state that the SFO calibration is feasible and can be achieved by Manchester coding for PRDCH. Additionally, source [Huawei] state that this method applies to device 2b.

- Sources [CATT], [Samsung], [Apple], [Quectel] and [TCL] report that SFO is feasible and can be achieved by reusing CAP.

- Sources [vivo], [NEC], [CTC] and [ZTE] report that SFO calibration is feasible and can be achieved by receiving sequence-based SIP.

- Sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [OPPO], [Apple], [Qualcomm], [Quectel], [Sony] and [Docomo] report that, SFO calibration is feasible and can be achieved by CFO calibration signal. Additionally, source [Huawei] state that this method applies to device C.

Scheduling aspects

RAN1#122bis

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

Agreement

For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects:

- Time location of L1 R2D control information:

  - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

  - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

- Study frequency resource of L1 R2D control information

- Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

RAN1#123

Agreement

Do not introduce a new R2D physical channel for L1 R2D control information, and update previous agreements as follows

Agreement (in RAN1#122bis) For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

Agreement

Capture following in TR38.769

Company reports that whether a time gap between L1 R2D control information and data payload in PRDCH, depends on following factors

- Whether device buffers data part before finishing L1 control decoding, as reported by sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [CMCC], [OPPO], [Ericsson], [Qualcomm], [NEC], [Nokia], [Xiaomi], [ZTE] and [Apple].

  - Sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [NEC] and [ZTE] state that gap between L1 R2D control and data is not needed, assuming device can buffer data part before finishing L1 R2D control decoding. Source [Huawei] further state that the time gap would increase the power consumption of the device.

  - Sources [CMCC][OPPO] and [Xiaomi] state that time gap is needed to reduce/avoid buffer at AIoT device.

- Whether frequency resource is different for L1 R2D control information and data payload, as reported by sources [OPPO] and [NOKIA]

- Whether chip duration (M value) is different for L1 R2D control information and data payload, which is reported by sources [Futurewei], [Huawei] and [LGE]

- Whether L1 R2D control information and data payload are separate channel, as reported by source [CMCC]

- Whether CFO calibration signal is placed between L1 R2D control information and data payload, as reported by [Lenovo].

For Option 1, L1 R2D control information and data payload of PRDCH are contiguous in time,

- Sources [Samsung] and [Apple] state that, transmit L1 R2D control information and data payload in the same PRDCH with contiguous time resources and same frequency resource is beneficial for low device complexity. [Apple] further report that this simplifies the timeline design.

- Sources [vivo] and [NEC] report that the time gap is not long enough to introduce gap between L1 R2D control information and data payload. Sources [Huawei], [NEC] further states that decoding of L1 R2D control information the device can parallelly store the data payload in buffer, and receive L1 R2D control information, hence time gap is not needed.

- Sources [vivo] and [ZTE] report that L1 R2D control transmitted in the same frequency as data payload in PRDCH, and time gap is not needed.

- Source [LGE] report that no time gap needed if the chip length for L1 R2D control information is the same as that for data payload of PRDCH.

For Option 2, L1 R2D control information and data payload of PRDCH are NOT contiguous in time,

- Sources [FUTUREWEI], [Ericsson] and [Xiaomi] report that a short time gap is needed if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH.

- Source [CMCC] state that, L1 R2D control and data payload are transmitted in separated physical channel, hence they are not contiguous.

- Source [OPPO] state that, time gap is needed to switch the frequency if L1 R2D control is transmitted in different frequency from data payload.

- Source [FUTUREWEI], [LGE] report that time gap maybe needed if the chip length for L1 R2D control information is different from that for data payload of PRDCH.

- Source [Qualcomm] report that, if there is a gap, midamble can be inserted between L1 R2D control information and data payload.

Agreement

Regarding frequency resource of L1 R2D control information

- Sources [Futurewei], [Spreadtrum], [vivo], [Huawei], [Samsung], [NEC], [ZTE], [Apple] and [Qualcomm] report that same frequency resource for R2D preamble, L1 control and data part.

  - Source [Futurewei] state that different frequency resource will cause additional latency as the transmission carrying control information must be first received followed by receiving the transmission carrying the payload. And unclear of the necessity of this option if FDM is not supported

  - Sources [vivo] and [Huawei] state that frequency retuning can be avoided if L1 control information and the corresponding data payload are located with same frequency locations. Otherwise, it will lead to additional adjustment delay and power consumption.

  - Sources [Spreadtrum] and [vivo] state that the motivation of transmitting L1 control and data payload in different frequency is not clear.

- [OPPO] report that frequency resource for R2D preamble, L1 control and data part can be different such as to reduce the load in the frequency resource used for L1 control.

Agreement

Capture following in TR 38.769

For the functionality of chip duration determination of the L1 R2D control information, the following options are studied

For option 1, Fixed chip duration. No use of CAP for chip duration determination.

[positive observations]

- Sources [Ericsson], [CATT], [LGE] and [CTC] states that fixed chip duration for L1 R2D control information lowers the complexity and power consumption of chip duration detection.

- Source [CATT] states that fixed chip duration for L1 R2D control information minimize the ambiguity of the chip duration detection, and thus enhance the success rate of L1 R2D control information reception.

- Source [Ericsson] states that using fixed chip duration (using the smallest M value for optimal coverage) can achieve higher reliability for L1 R2D control information.

[Negative observations]

- Source [Qualcomm] and [Samsung] states that to achieve the largest coverage, L1 control would need to use the lowest M, which increases overhead for each R2D transmission.

- Sources [FUTUREWEI], [Samsung], [ZTE], [Huawei], [vivo], [Docomo] and [CMCC] states that, if chip duration for L1 control information is fixed, the flexibility or transmission efficiency is restricted.

For option 2, Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

[positive observations]

- [CMCC] and [Panasonic] state that blind detection within configured set provide better flexibility for chip length for L1 R2D control information.

- [CMCC] states that blind detection is viable if the number of predefined chip duration candidates is restricted. Additionally, [CMCC] states that Option 2 can be combined with device-specific configuration signaling to enhance flexibility without excessive complexity.

[Negative observations]

- Sources [FUTUREWEI], [Samsung], [vivo], [DOCOMO], [Ericsson] and [ZTE] state that Option 2 increases detection complexity.

- Source [Qualcomm] states that it may be impractical due to constraints on power consumption and memory needed for buffering the incoming R2D L1 control information samples.

- Sources [Huawei] and [LGE] state that option 2 would require additional time gap between the L1 control information and the data payload of the PRDCH to allow for decoding delay.

- Sources [vivo] and [Ericsson] state that reliability is doubtful due to blind detection.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway to achieve flexibilities in coverage and resource overhead, no need to leave chip duration for blind detection.

For option 3, Use CAP

[positive observations]

- For Alt.3-1 Rel-19 CAP pattern, Sources [Samsung] and [Sequans] state that Rel-19 CAP pattern to determine chip length of L1 R2D control part offer a balance between detection complexity and performance.

- For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, Sources [LGE], [Apple] and [Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Apple further states that this option maintains consistency with Release-19 design principles. Source [Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility.

[Negative observations]

- Source [Qualcomm] states option 3 requires blind detection; and MD/FA rates and CAP overhead needs to be considered.

- Source [ZTE] states option 3 results in additional overhead due to transmission of CAP.

- Sources [CMCC], [vivo] and [Huawei] state that Option 3 is based on edge detection to determine the chip duration by using CAP, so the performance is limited or unfeasible due to the use of edge detection to detect the CAP pattern. Sources [ZTE] and [Ericsson] also report the performance of this option should be further evaluated.

- Source [Ericsson] states that option 3 would make the decoding of the L1 R2D control information more complicated for the device.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection using CAP.

For Option 4, Use a set of different binary sequences with fixed or variable length

[positive observations]

- For Alt 4-1, using binary sequences for SIP. No use of CAP for chip duration determination

  - Sources [Nokia], [Huawei], [NEC], [LGE], [Docomo] and [Panasonic] state that this method can provide flexibility for R2D chip duration.

  - Source [Huawei] state that a restricted set of binary sequences of differing lengths are mapped to different chip durations and coverage distances, which can be detected with limited blind detection complexity. Parallel processing for the blind detection of different sequence lengths avoids and processing delay. Besides, R-TAS design is simplified, which consist only sequence-based SIP.Alt.4-2: using binary sequences for CAP.

  - Sources [Nokia] and [Panasonic] report that flexibility on chip length of L1 R2D control can be achieved.

[Negative observations]

- Sources [vivo], [Ericsson] and [Qualcomm] states that option 4 increases detection complexity. And Ericsson states that option 4 would make the decoding of the L1 R2D control information more complicated for the device.

- Sources [ZTE] and [Ericsson] report that the performance of this option should be further evaluated.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection; vivo further report that it leader more challenges on preamble design, requiring larger number of sequences to indicate four M values in addition to reader/cell ID differentiation;

For option 5, Use broadcast information (if any) to indicate the chip duration

[positive observations]

- Sources [vivo], [ZTE] and [Qualcomm] state that better flexibility can be achieved without blind detection, and sources [vivo] and [ZTE] further state that the chip duration for L1 can be indicated together with other parameters for control information, e.g., repetition and FEC related parameters. Source [CMCC] state that this method can be used for L1 R2D control information common for devices, for example, the L1 R2D control information for scheduling paging message.

- Source [OPPO] state that the chip length should be pre-known by device to minimize complexity and power consumption at AIoT device.

[Negative observations]

- Source [Qualcomm] states that it requires system info acquisition and memorization.

- Source [Huawei] and [Samsung] states that only to indicate chip duration in broadcast information limits flexible time resource allocation from system perspective.

- Source [Ericsson] states that this option is a simple solution but not applicable for paging and broadcast PRDCH.

- Source [Docomo] states that state that option 5 has less flexibility on chip duration of L1 R2D control compared to option 2/3/4.

Note: whether/how to capture blue parts can be up to editor

Agreement

Regarding whether to apply same or different chip duration for L1 R2D control and data payload, Capture following in TR38.769.

Sources [Spreadtrum], [Huawei], [Ericsson], [Samsung], [LGE], [Apple] and [Sequans] report that, the use of the same chip duration for L1 R2D control information and data payload in the PRDCH, have the following benefits:

- Sources [Spreadtrum], [Huawei], [Samsung], [Sequans] and [Apple] state that the use of the same chip duration simplifies device implementation, and [Huawei] further states that the L1 R2D control information can avoid any decoding delay and avoid the need for a time gap between the L1 R2D control information and the data payload of the PRDCH.

- Sources [Ericsson] states that, for broadcast PRDCH and paging PRDCH, the R2D parameters, including chip length for data payload of PRDCH, can be fixed for cater to the cell-edge scenarios.

- Sources [Samsung], [Sequans] and [LGE] state that using the same M value for the L1 R2D control part and the data payload avoids additional signaling overhead, and [Samsung] [Sequans] further states that this ensures consistent coverage for both control and data, and no clear benefit to use different chip length for L1 control part and data part.

Sources [Futurewei], [vivo], [CMCC], [Ericsson], [NEC], [CTC], [Xiaomi], [ZTE], [LGE], [Panasonic],[Docomo], [Apple], [Qualcomm] and [CEWiT] report that, chip duration of the data payload in the PRDCH indicated by the corresponding L1 R2D control information, have the following benefits:

- Sources [Futurewei], [vivo], [CMCC], [NEC], [CTC], [Xiaomi], [LGE], [ZTE], [Panasonic], [Docomo] and [CEWiT] state that chip duration indication through L1 control information provides better flexibility to meet diverse reliability and data rate needs. [ZTE] further states that it better coordinates with TB size, repetition rate, and code rate for improved link adaptation; [Futurewei] states that it allows tailoring data rates for specific devices and can improve efficiency;

- Source [vivo] state that with better coverage for L1 control part using longer chip length, unnecessary data reception can be avoided due to more robust L1 control;

- Sources [Apple] and [Ericsson] state that this option provides flexibility to adapt to different coverage conditions or message types, with [Ericsson] states this option provides benefits for PRDCH like Msg2 and Msg4.

Agreement

Regarding how the L1 R2D control information is transmitted, capture following in TR 38.769.

[FEC and repetition for L1 R2D control information]

Regarding FEC and repetition for L1 R2D control part, sources [vivo], [OPPO], [CMCC], [Futurewei], [NEC], [CATT], [CTC], [ZTE] and [Qualcomm] report that FEC can also be applied L1 R2D control information. Source [Huawei] state that repetition is needed for L1 R2D control information.

[Parameters for transmission of L1 R2D control information]

Sources [vivo], [CMCC], [NEC], [OPPO], [CTC], [ZTE] and [Qualcomm] report methods to determine FEC, repetition related parameters for L1 R2D control information, if FEC/repetition is supported for L1 control.

- Sources [vivo], [CMCC], [NEC], [ZTE] and [Qualcomm] report that the parameters for repetition and FEC for L1 R2D control information can be pre-configured, e.g., by broadcast information. [OPPO] report that these parameters should be pre-known by device.

- Sources [NEC] and [CTC] states that parameters for repetition and FEC for L1 R2D control information can be fixed and pre-defined. Source [NEC] also state that these parameters can be indicated by a set of different binary sequences with fixed or variable length for indication. Source [NEC] report that a separate interleaving is needed for L1 R2D control.

[information size]

Regarding bit size for L1 control information

- Sources [vivo], [OPPO], [Ericsson] and [Apple] report that device only detects one TBS at a given time. And [vivo] also report that up to two TBS can be considered and detected by preamble.

[CRC Attachment]

Regarding separate CRC for L1 control information

- Source [Futurewei] state that, for Separate CRC on the L1 R2D control information, when the size of the L1 R2D control information is 24 bits or less, the existing CRC6 specification can be reused, if the size of the L1 R2D control information is between 25 and 57 bits, the existing CRC6 polynomial can be reused without impact to the undetected error rate.

- Source [Xiaomi] and [Apple] state that, how to attach the separate CRC for L1 R2D control information needs to be considered, and a fixed length (equal to or larger than 16) of CRC is required to ensure the reliability and avoid unnecessary blind detection for L1 R2D control information.

Note: whether/how to capture blue parts can be up to editor

Agreement

Capture following in TR38.769

Regarding whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH

- Sources [vivo], [Qualcomm], [Huawei], [Samsung], [Futurewei], [Docomo], [NEC], [LGE], [Apple], and [Interdigital] state that L1 R2D control is transmitted in the same PRDCH carries data payload for simplicity for standard and device.

  - [Huawei] states if L1 control and data are in different PRDCH, the time relationship design is complicated and the device needs to buffer the L1 control information for a long time which is hard for the device to implement.

- Sources [ZTE], [CMCC], [Transsion], [Panasonic] and [MTK] state that L1 R2D control information is transmitted in a separate PRDCH

  - Source [ZTE] report that, L1 R2D control information is encoded separately, it is more appropriate to transmit L1 R2D control information on a separate PRDCH

  - Sources [CMCC] and [Transsion] state that, defining a new PRDCH format to carry L1 R2D control information only is more efficient for scheduling and resource utilization.

  - Source [Panasonic] state that, L1 control information is transmitted in separated PRDCH since control part would require higher reliability, and one control information could be used for multiple PRDCH transmissions.

Agreement

Regarding necessary information to be included in the L1 R2D control: capture following observations in TR 38.769

In addition to TBS for PRDCH data payload, the following L1 R2D control information, which potentially can be indicated to the device, is studied:

- M value for PRDCH data payload chip duration, as reported by [vivo], [Qualcomm], [CMCC], [OPPO], [Futurewei], [Ericsson], [Docomo], [NOKIA], [NEC], [CATT], [CTC], [Xiaomi], [ZTE], [Transsion], [LGE], [Panasonic], [Apple], [Interdigital], [TCL] and [CEWiT].

  - On the other hand, sources [Spreadtrum], [Huawei], [Ericsson] and [Sequans] suggest to use same chip duration between L1 R2D control information and data payload, hence M value indication is not needed in L1 R2D control.

- Repetition/FEC related parameters for PRDCH data payload, as reported by sources [vivo], [Qualcomm], [Huawei], [CMCC], [OPPO], [Futurewei], [Spreadtrum], [ERICSSON], [Docomo], [NEC], [CATT], [CTC], [ZTE], [Transsion], [LGE], [Apple], [Interdigital], [TCL] and [CEWiT], if Repetition/FEC is supported for R2D.

- Timing related information for PRDCH data payload, as reported by sources [Qualcomm], [CMCC], [OPPO], [NOKIA], [Transsion], [LGE], [Interdigital] and [CEWiT], considering potential need of gap between L1 R2D control and corresponding data payload.

  - On the other hand, sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [NEC] and [ZTE] state that, this information is not included in L1 R2D control, because of no need of gap between L1 R2D control and corresponding data payload.

- Timing gap between R2D and the corresponding D2R transmission, as reported by [Qualcomm]

- Frequency resource for PRDCH data payload, as reported by sources [Ericsson], [Quectel] and [CEWiT], if FDM for R2D is supported.

- Message types/Functional indication, as reported by Sources [Qualcomm], [vivo], [OPPO], [Samsung], [Futurewei], [NOKIA], [Xiaomi], [ZTE] and [CMCC], for the purpose of early termination of R2D detection.

- ID related information, which potentially include at least one of reader ID, device ID or Transaction ID related information

  - ID related information associated with device(s)

    - Sources [Futurewei], [vivo], [Qualcomm], [OPPO], [Samsung] and [TCL] state that, ID related information associated with device(s), indicating for which device or group of devices the upcoming data is intended, can be included in L1 R2D control for early termination.

  - Reader ID related information, if needed, as reported by Source [Qualcomm].

- Indication of next R2D transmission, as reported by Source [Futurewei], used to indicate when the next R2D transmission, can help non-intended devices have the opportunity to harvest energy.

R2D FDM

RAN1#122bis

Agreement

Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects:

- complexity of device to support FDM

- how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

RAN1#123

Agreement

Capture following observations in TR 38.769

For R2D FDM from the same reader for Device 2b and for Device C, feasibility and necessity are studied

[Feasibility]

- Sources [FUTUREWEI], [Spreadtrum], [Huawei] and [DOCOMO] states that R2D FDM is feasible for Device 2b and for Device C, considering device equipped with IF/ZIF filters at the receiver.

- Source [Ericsson] states that the total number of PRBs required to multiplex devices using FDM needs to be defined.

[Necessity]

- Source [vivo] reports that the transmission resource efficiency by using R2D FDM cannot be improved compared with R2D non-FDMA, when AIoT R2D bandwidth is no larger than 4 RBs.

- Source [LGE] states that:

  - If allows all devices to receive the initial paging message without FDM, while enabling efficient sub-band utilization using FDM for following messages, it will introduce complexity in managing mode transitions between FDM and non-FDM, and coordinating sub-band usage.

  - If enables device-specific optimization and parallel reception across sub-bands, it will increase transmission resource usage due to duplication of paging messages.

- Sources [vivo], [Ericsson], [NEC], [Samsung] and [TCL] state that guard band between FDMed transmissions need to be allocated to compensate the impact of CFO.

- Source [Docomo] states that for R2D, FDM can be beneficial to extend multiplexing capacity. Sources [ZTE] and [Qualcomm] state that in A-IoT systems, the capacity bottleneck lies on the R2D link. And to enhance system capacity, FDM is a potential solution.

[Device complexity]

Regarding device complexity to support R2D FDM from the same reader for Device 2b and for Device C

- Sources [CMCC], [CATT], [Xiaomi] and [Apple] state that R2D FDM will increase the device complexity since the adjustment of filter bandwidth and/or IF center frequency is required.

- Source [Samsung] raises the related issue that whether different FDMed frequency resources can be supported by the same device, and if so, how the Rx filter architecture of IF/ZIF receivers can support such flexible determination.

- Sources [ZTE], [Huawei] and [NEC] state that additional complexity is limited since device with IF/ZIF receiver can already support to distinguish different frequency resources.

[Determines the frequency resource]

Regarding how a device determines the frequency resource for receiving one of the FDMed R2D transmissions:

- Sources [FUTUREWEI], [Huawei] [Spreadtrum] and [Qualcomm] state that the frequency resource of synchronization signal, CFO calibration signal or broadcast information need to be fixed or predefined.

- Sources [Huawei], [Spreadtrum], [Qualcomm], [DOCOMO], [NEC], [ZTE] and [OPPO] state that some of R2D transmissions e.g., L1 control information or a R2D transmission other than sync signal or broadcast information can determine their frequency resources based on the indication.

- Source [OPPO] state that the frequency resource for data payload is indicated by corresponding L1 R2D control.

- Source [NEC] and [DOCOMO] state that the frequency resource of a R2D transmission can be associated with the previous corresponding D2R transmission.

- Source [NEC] state that the exact frequency resource/channel for a R2D transmission can be associated with a specific coverage level from a set of candidates R2D frequency resources preconfigured.

Note: whether/how to capture blue parts can be up to editor