3GPP TSG-RAN WG1 Meeting #124	R1-2601544

# Gothenburg, Sweden, 9 – 13 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRC parameter alignment for HD-FDD Ues in NR over NTN in TS38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3 |  |  |  |  |  | Date: |  |  | 2026-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RRC parameter alignment for UE feature group for supporting the HD-FDD collision handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Parameter CollisionHandlingOfHDFDDOperation in text is replaced by the correct UE feature name which is ntn-Collision-RedCap-r19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 17.2	Half-Duplex UE in paired spectrum

A half-duplex UE (HD-UE) in paired spectrum is not capable of simultaneous transmissions and receptions on a serving cell with paired spectrum. This clause is applicable for communication of a HD-UE on a serving cell with paired spectrum. Procedures for a HD-UE are same as described for a UE in all other clauses of this document unless stated otherwise.

A HD-UE that operates on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate ntn-Collision-RedCap-r19, does not expect to detect a DCI format scheduling a reception in a set of symbols and detect a DCI format scheduling a transmission in any symbol from the set of symbols. A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates ntn-Collision-RedCap-r19, is scheduled to receive a PDSCH or CSI-RS in a set of symbols based on an indication by a first DCI format, and is scheduled to transmit a PUSCH, PUCCH, or PRACH that overlap with any symbol from the set of symbols based on an indication by a second DCI format, determines

-	to either receive the PDSCH or the CSI-RS or transmit the PRACH based on the HD-UE implementation;

-	to either receive the PDSCH or transmit the PUSCH or PUCCH based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic, and the transmission of the PUCCH or the PUSCH would not start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the PUSCH or PUCCH, if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntn-RedcapPrioritizeUL-Dynamic, or

-	the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic and the transmission of the PUCCH or the PUSCH would start before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format.

A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates ntn-Collision-RedCap-r19, is scheduled to receive a PDSCH or CSI-RS based on an indication by a first DCI format, and is scheduled to transmit an SRS based on an indication by a second DCI format and the transmission of the SRS in a set of symbols overlaps with the PDSCH reception or CSI-RS reception, determines

-	to either receive the PDSCH or transmit the SRS based on the HD-UE implementation, if the first DCI format is provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set;

-	to receive the PDSCH or the CSI-RS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic, and the first symbol from the set of symbols is not before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format;

-	to transmit the SRS if the first DCI format is not provided by a PDCCH the HD-UE received according to a Type0/0A/1/2-PDCCH CSS set, and

-	the HD-UE is provided ntn-RedcapPrioritizeUL-Dynamic, or

-	the HD-UE is not provided ntn-RedcapPrioritizeUL-Dynamic and any symbol from the set of symbols is before Tproc,2 after the last symbol of the PDCCH reception providing the first DCI format

When a PDCCH reception by a HD-UE includes two PDCCH candidates from corresponding search space sets, as described in clause 10.1, the end of the PDCCH reception is the end of the PDCCH candidate that ends later.

If a HD-UE is configured by higher layers to receive a PDCCH, or PDSCH, or CSI-RS, or DL PRS in a set of symbols, the HD-UE receives the PDCCH, or PDSCH, or CSI-RS, or DL PRS if the HD-UE does not detect a DCI format that indicates to the HD-UE to transmit a PUSCH, or PUCCH, or PRACH, or SRS in at least one symbol of the set of symbols; otherwise, the HD-UE does not receive the PDCCH, or PDSCH, or CSI-RS, or DL PRS in the set of symbols.

If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214].

-	the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols.

Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH.

A HD-UE operating on a non-NTN serving cell, or a HD-UE that operates on an NTN serving cell and does not indicate ntn-Collision-RedCap-r19, does not expect to receive both dedicated higher layer parameters configuring transmission in a set of symbols and dedicated higher layer parameters configuring reception in the set of symbols. The HD-UE does not expect to receive both a Type-0/0A/0B/1/2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring transmission in the set of symbols, except a Type-2-PDCCH CSS set configuration for PDCCH reception in a set of symbols and dedicated higher layer parameters configuring configured-grant based PUSCH transmission as described in clause 19.1 in the set of symbols for which case the HD-UE follows the procedure as in clause 5.1B.2.6 of [10, TS 38.133]. The HD-UE expects to be configured with a Type-2-PDCCH CSS set configuration for PDCCH reception such that there is at least one paging occasion that does not overlap with configured-grant based PUSCH transmission as described in clause 19.1 per SI modification period.

A HD-UE that operates on an NTN serving cell in the RRC_CONNECTED state, indicates ntn-Collision-RedCap-r19, would receive a PDCCH or would receive a PDSCH or CSI-RS or PRS based on a configuration by higher layers in a set of symbols, and would transmit a PUSCH, PUCCH, or SRS based on a configuration by higher layers that overlap with any symbol from the set of symbols, determines

-	to either receive the PDCCH or transmit the PUSCH/PUCCH/SRS based on the HD-UE implementation, if the PDCCH reception is according to a Type0/0A/1/2-PDCCH CSS set

-	to receive the PDCCH, if the PDCCH reception is not according to a Type0/0A/1/2-PDCCH CSS set, or the PDSCH, or the CSI-RS, or the PRS if the HD-UE is not provided ntn-RedcapPrioritizeUL-Semistatic

-	to transmit the PUSCH, the PUCCH, or the SRS, if the PDCCH reception is not according to a Type0/0A/1/2-PDCCH CSS set, and the HD-UE is provided ntn-RedcapPrioritizeUL-Semistatic

A HD-UE that operates on an NTN serving cell in the RRC_INACTIVE state, would receive a PDCCH in a set of symbols and would transmit a PUSCH or SRS that overlap with any symbol from the set of symbols, determines to either receive the PDCCH or transmit the PUSCH or SRS based on the HD-UE implementation.

< Unchanged text omitted >