3GPP TSG RAN WG1 #124			R1-2601646

Gothenburg, SE, Feb 9th – 13th, 2026

Agenda item:	8.1

Source:	Moderator (Samsung)

Title:	FL summary #2 for AI/ML in beam management

Document for:	Discussion and Decision

- Introduction

This contribution summarised the remaining issues on AI/ML based beam management.

(FL0) Question 0

Please enter/update contact info below to facilitate discussion.

| Company | Contact | Email address |
| --- | --- | --- |
| OPPO | Jeffrey (Jianfei) Cao | caojianfei@oppo.com |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| CMCC | Yi Zheng Jiazhen Zhang | zhengyi@chinamobile.com zhangjiazhen@chinamobile.com |
| NTT DOCOMO | Xin Wang | wangx@docomolabs-beijing.com.cn |
| vivo | Hao Wu | hao.wu@vivo.com |
| NEC | Peng Guan Pravjyot Deogun Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM y-jiang_ct@nec.com |
| ETRI | Yongjin Kwon | yjkwon@etri.re.kr |
| Xiaomi | Mingju | limingju@xiaomi.com |
| ZTE | Wenfeng LIU Xingguang WEI | liu.wenfeng@zte.com.cn wei.xingguang@zte.com.cn |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| Panasonic | Henry Tran | xuantuong.tran@sg.panasonic.com |
| CATT | Min Zhu | zhumin@catt.cn |
| Google | Yushu Zhang | yushuzhang@google.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| LG | Seongwon Go | sw.go@lge.com |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |
| Fraunhofer HHI | Baris Göktepe | Baris.goektepe@hhi.fraunhofer.de |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |
| SONY | Chen Sun Yingshuang Bai | chen.sun@sony.com yingshuang.bai@sony.com |
| Hw/HiSi | Yuan Li | liyuan3@huawei.com |
| Apple | Weidong Yang | wyang23@apple.com |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |

- Remaining issues for UE-side AI/ML model

2.1 APU/CPU

ZTE

Reason for change: CPU duration for beam management related UE-side data collection is not captured correctly. In current specification, periodic CSI report for UE-side data collection for beam management is not supported.

Agreement For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported. Agreement For UE-sided model, regarding a CSI-ReportConfig for data collection, OCPU=1 - Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured

Summary of change: Reflect the RAN1 agreements accurately in Section 5.2.1.6 of TS 38.214. Include periodic CSI report for UE-side data collection for beam management.

Consequence if not approved: Incorrect UE behaviour for CPU duration for UE-side data collection for training.

Text Proposal 1: To adopt the following changes in section 5.2.1.6, TS 38.214.

5.2.1.6 CSI processing criteria < Unchanged parts are omitted > For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-csi-r19' or 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement. where (Z3,Z3') are defined in the table 5.4-2. < Unchanged parts are omitted >

Sharp

Proposal 3: Adopt the following TP#3 in TS 38.214 to clarify that “OCPU”, rather than “OCPU,1and/or OCPU,x”, should be used to describe CPU occupation time for monitoring report.

- Reason for change: The new symbols “OCPU,1and/or OCPU,x” are introduced in TS38.214 to define the CSI reports for inference which requires CPU and/or APU. However, the symbols “OCPU,1and/or OCPU,x” are mistakenly applied to describe the CPU occupation time for the CSI reports for monitoring.

- Summary of change: Clarify that OCPU is the appropriate symbol for CSI reports with reportQuantity set to ‘csi-pai-r19’, or ‘rs-pai-r19’, when describing CPU occupation time.

- Consequences if not approved: The description of CPU occupation time for CSI reports for monitoring is not accurate.

TP#3 5.2.1.6 CSI processing criteria <unchanged parts are omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter [RRC_name-r19], and OCPU for CSI reports with reportQuantity set to ‘csi-pai-r19’ or ‘rs-pai-r19’) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report, a semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19', and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used. - An initial semi-persistent CSI report on PUSCH after the PDCCH trigger occupies CPU(s) from the first symbol after the PDCCH until the last symbol of the scheduled PUSCH carrying the report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the CPU occupation duration, the PDCCH candidate that ends later in time is used. - A semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' occupies CPU(s) from the first symbol of KP-th latest consecutive periodic/semi-persistent CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where the value of KP∈{1,2,4} is indicated by UE capability. - A semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19' occupies CPU(s) from the first symbol of the latest consecutive periodic/semi-persistent CSI-RS occasion of a corresponding linked CSI report (linked by inferenceReportConfigId-r19) no later than CSI reference resource of the linked CSI report, until the last symbol of the PUSCH carrying the CSI-PAI report. <unchanged parts are omitted>

Proposal 4: Adopt the following TP#4 in TS 38.214 to clarify the determination of updated AI/ML CSI reports based on CPU and APU.

- Reason for change: The current specification may result in a situation where the UE is prevented from updating a higher-priority CSI report, while it is still allowed to update a lower-priority CSI report, even though both CSI reports require CPUs from a same PU pool. For example, as illustrated in the Figure below, the CSI report #4 (higher priority) is not updated, while the CSI report #5 (lower priority) is updated.

- Summary of change: Add descriptions “with nonzero reported OCPU,1 and nonzero reported OCPU,x” to clarify that, for a CSI report whose required OCPU,1 and OCPU,2 are both nonzero, if any one of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not updated.

- Consequences if not approved: For CSI reports that require OCPU from the same CPU pool, it is possible that a lower priority CSI report may be updated instead.

TP#4 5.2.1.6 CSI processing criteria <unchanged parts are omitted> The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter csi-InferencePrediction-r19 corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, OCPU,1 and OCPU,x are considered for determining M and Mx, and if a CSI report with nonzero reported OCPU,1 and nonzero reported OCPU,x is not considered within any of M and Mx, the CPUs corresponding to OCPU,1 and OCPU,x of the CSI report are not occupied, for the procedure previously described in this clause and the UE is not required to update the CSI report. <unchanged parts are omitted>

Proposal 5: Adopt the following TP#5 in TS 38.214 to clarify that only Set A is considered for determination of CPU occupation time for UE-side data collection.

- Reason for change: For UE-sided data collection, both Set A and Set B are configured for channel measurement. When determining CPU occupation time, it is specified that “each transmission occasion for channel measurement” is referred. “Transmission occasion” is defined per resource set (including Set A and Set B), which means there would be separate “transmission occasions” in Set A and Set B, respectively. Therefore, it would lead to over-counting for CPU occupation, e.g. CPU occupation corresponding to transmission occasion of Set B, if Set B is a subset of Set A, is counted twice.

- Summary of change: Clarify that only Set A is considered for determination of CPU occupation time for UE-sided data collection.

- Consequences if not approved: CPU occupation time for UE-side data collection is incorrectly determined.

TP#5 5.2.1.6 CSI processing criteria <unchanged parts are omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation of the second Resource Setting, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion of the second Resource Setting. <unchanged parts are omitted>

Proposal 6: Adopt the following TP#6 in TS 38.214 to specify (Z3, Z3') as CSI computation time for model monitoring CSI report.

- Reason for change: CSI computation time for monitoring report is not defined yet in current specs.

- Summary of change: Clarify that (Z3, Z3') is the corresponding CSI computation time for model monitoring CSI report.

- Consequences if not approved: UE is unable to determine CSI computation time for monitoring report.

TP#6 5.4 UE CSI computation time <unchanged parts are omitted> Z, Z' and µ are defined as: Z=maxm=0,…,M-1(Z(m))⁡ and Z'=maxm=0,…,M-1(Z'(m)), where M is the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as - (Z1,Z1') of the table 5.4-1 if max{ µPDCCH, µCSI-RS, µUL} ≤ 3 and if the CSI is triggered without a PUSCH with either transport block or HARQ-ACK or both when L = 0 CPUs are occupied (according to Clause 5.2.1.6) and the CSI to be transmitted is a single CSI and corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the CSI corresponds to at most 4 CSI-RS ports in a single resource without CRI report and where CodebookType is set to 'typeI-SinglePanel' or where reportQuantity is set to 'cri-RI-CQI', or - (Z1,Z1') of the table 5.4-2 if the CSI to be transmitted corresponds to wideband frequency-granularity where the reportQuantity is set to 'ssb-Index-SINR', 'cri-SINR', 'ssb-Index-SINR- Index ', or 'cri-SINR- Index ', or - (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'cri-RSRP', 'ssb-Index-RSRP', 'cri-RSRP- Index', 'L1-SRS-RSRP', 'L1-CLI-RSSI' or, 'ssb-Index-RSRP- Index ' or 'rs-pai-r19', where Xµ is according to UE reported capability beamReportTiming and KBl is according to UE reported capability beamSwitchTiming as defined in [13, TS 38.306], or if the CSI report is configured with ltm-CSI-ReportConfig for L1-RSRP measurement, or if the CSI report is configured with eventType, or - (Z3+d,Z3'+d'), with (Z3,Z3') of the table 5.4-2 if reportQuantity is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', where d and d' are according to UE reported capability as defined in [13, TS 38.306] and different values for d and d' may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report, or <unchanged parts are omitted>

Nokia

Reason for change: CPU duration for Beam prediction related UE-sided data collection is not captured correctly.

Summary of change: Reflect the RAN1 agreements accurately in Section 5.2.1.6 of TS 38.214.

Consequence if not approved: Incorrect behaviour for CPU duration.

Proposal 3: For UE-sided data collection, to clarify CPU duration for UE-sided data collection, endorse the following text proposal to 38.214.

5.2.1.6 CSI processing criteria < Unchanged parts are omitted > For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-csi-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement in each transmission occasion. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. where (Z3,Z3') are defined in the table 5.4-2. < Unchanged parts are omitted >

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.1.1# Correction on CPU occupation time for data collection

Several companies [2,11] provided TPs to clarify the CPU occupation time for data collection.

Proposal. 2.1.1

Adopt the following TP for TS 38.214 Clause 5.2.1.6.

Reason for change: CPU occupation time for beam prediction related UE-sided data collection is not captured correctly.

Summary of change: Reflect the RAN1 agreements accurately in Clause 5.2.1.6 of TS 38.214.

Consequence if not approved: Incorrect behaviour for CPU occupation time for beam prediction related UE-sided data collection.

5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-csi-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-BM-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of each the transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter eventType configured, the CPU is occupied from when CSI-ReportConfig is configured and until CSI-ReportConfig is released. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | The proposal is drafted based on Nokia’s TP. Some changes are made in blue on the top of Nokia’s TP such that the wording is exactly the same as legacy wording (Rel-15) and the report quantity parameter name is aligned with the latest 38.331. Please check and share your view. |
| LGE | Is it the intention to preclude the bullet regarding aperiodic CSI report? I think the current spec is nothing wrong based on the below captured agreement. Agreement For UE-sided model, regarding a CSI-ReportConfig for data collection, - Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured. |
| vivo | Support, To LGE, aperiodic CSI report for data collection purpose is not supported Agreement For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported. |
| Huawei, HiSilicon | OK |
| ZTE | OK |
| QC | Share similar views as LGE |
| FL2 | It seems that majority company are ok with the proposal. The key controversial part is whether to include the description of periodic CSI report. |

Issue 2.1.2# Correction on the occupied CPU for inference reporting

Sharp [9] provided discussion or TPs to correct the notation of occupied CPU in the description of CPU occupation time for inference reporting.

Proposal. 2.1.2

Adopt the following TP for TS 38.214 Clause 5.2.1.6.

Reason for change: The notation of “OCPU,1and/or OCPU,x” is mistakenly used to describe occupied CPU for CPU occupation time for CSI report for monitoring.

Summary of change: Adding correct notation of occupied CPU for CPU occupation time for CSI report for monitoring.

Consequences if not approved: The notation of occupied CPU for CPU occupation time for CSI report for monitoring is incorrect.

5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or ‘csi-pai-r19’, or ‘rs-pai-r19’, and CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, and OCPU for CSI report with reportQuantity set to ‘rs-PAI-r19’) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report, a semi-persistent CSI report on PUSCH configured with reportQuantity set to 'csi-pai-r19', and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | The TP is based on the TP in [9]. It seems technically correct to revise “OCPU,1and/or OCPU,x” as “OCPU” . Also, there is an editorial change to make the report quantity parameter aligned with the latest 38.331. Please check and share your view. |
| LGE | Not critical, but we can live with it. |
| vivo | Support |
| Huawei, HiSilicon | OK |
| ZTE | OK |
| QC | OK, but following the same logic ‘csi-pai-r19’ should also move below, i.e., “, and OCPU for CSI report with reportQuantity set to ‘csi-pai-r19’, or ‘rs-PAI-r19’”. |
| Sharp | Support. The modification from QC looks good. |
| FL2 | It seems that there is no objection on the proposal, For CSI prediction part, we can have a mirror agreement as long as the beam management part is agreed. |

Others

Sharp [9] proposed to only consider the CSI report with non-zero CPU occupation. However, this is an optimization rather than a critical correction.

Sharp [9] proposed to clarified that the second Resource Setting should be used for the determination of CPU occupation time. However, the current specification is clear since it says “CSI-RS/SSB resource for channel measurement”.

Sharp [9] proposed to add RS-PAI for Z3 description. However, this is unnecessary since only the value of Z3 is considered which is similar to the situation as scenario of “none”.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| Huawei, HiSilicon | We think the current spec is clear and there is no need to discuss. |
| Sharp | - Regarding the issue of CSI reports with non-zero CPU occupation, rather than treating this as an optimization, we would consider the correction necessary to align with legacy behavior. That is, a higher-priority CSI report should be allowed to be updated before a lower-priority CSI report. - For CPU occupation time of data collection, as FL said, CSI-RS/SSB for channel measurement is used. In 38.214, it explicitly specifies as “both the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) and the second Resource Setting (given by higher layer parameter resourcesForSetA-r19) are for channel measurement”, therefore, both Set A and Set B are used for determination and this leads to over-calculation of CPU occupation time, since Set B can be a subset of Set A. For CSI computation time, clearly it has not been specified yet for model monitoring CSI report, while for such report, it is necessary. We don’t agree with FL that this is to add description for Z3, it should be a missing case for model monitoring CSI report. |

2.2 CSI report for model inference

Proposals related to CSI report for model inference are summarized as follows.

Huawei

Table 1 Text proposal for relaxation of legacy rate matching rule

Reason for change: PDSCH rate-matching around virtual CSI-RS in Set A (for inference configuration or applicability configuration Option B) or Set B (for applicability configuration Option B) results in unnecessary waste of resource. Summary of change: Clarify that CSI-RS only configured in Set A (for inference configuration or applicability configuration Option B) or Set B (for applicability configuration Option B) is not used to perform PDSCH rate-matching. Consequences if not approved: The resource occupied by the virtual CSI-RS in Set A or Set B cannot be used for PDSCH transmission.

##### TS 38.211 Clause 7.3.1.5 Mapping to virtual resource blocks <omitted texts> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE and not only configured by any of the resourcesForChannelPrediction-r19 in CSI-ReportConfig with reportQuantity set to ‘p-CRI-r19’, ‘p-CRI-RSRP-r19’, ‘p-SSB-Index-r19’, or ‘p-SSB-Index-RSRP-r19’, resourcesForChannelMeasurement-r19 in ApplicabilitySetConfigCSI-r19, and resourcesForChannelPrediction-r19 in ApplicabilitySetConfigCSI-r19, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

Proposal 1: Consider the text proposal in Table 1.

Vivo

Proposal 1: Adopt the following TPs for TS 38.211 Clause 7.3.1.5

Reason for change: PDSCH rate-matching around virtual CSI-RS in Set A results in unnecessary waste of resource.

Summary of change: Clarify that CSI-RS only configured in Set A is not used to perform PDSCH rate-matching.

Consequences if not approved: The resource occupied by the virtual CSI-RS in Set A cannot be used for PDSCH transmission.

##### TS 38.214 Clause 7.3.1.5 Mapping to virtual resource blocks <omitted texts> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE and not only configured by resourcesForChannelPrediction IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. <omitted texts>

InterDigital

Proposal 1: Support virtual resource blocks mapping of PDSCH in TS38.211 based on the following:

- if CSI-ResourcePeriodicityAndOffset is configured for NZP CSI-RS configured in Set A, rate matching of the resource elements in the corresponding physical resource blocks is performed .

- if CSI-ResourcePeriodicityAndOffset is absent for NZP CSI-RS configured in Set A, rate matching of the resource elements in the corresponding physical resource blocks is not performed.

Proposal 2: Support the following TP:

Reason for change: Res for NZP CSI-RS resources in Set A are rate matched when the Res are not occupied by actual NZP CSI-RS transmissions.

Summary of change: For NZP CSI-RS resources in Set A are not rate matched when CSI-ResourcePeriodicityAndOffset is absent for NZP CSI-RS configured in Set A.

Consequences if not approved: Utilization of Res for NZP CSI-RS resources without actual NZP CSI-RS transmission is not possible.

##### 7.4.1.5.1 General Zero-power (ZP) and non-zero-power (NZP) CSI-RS are defined - for a non-zero-power CSI-RS without CSI-ResourcePeriodicityAndOffset configured by the NZP-CSI-RS-Resource IE configured by resourcesForSetA-r19, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are used for PDSCH transmission. - for a non-zero-power CSI-RS configured by the NZP-CSI-RS-Resource IE or by the CSI-RS-Resource-Mobility field in the CSI-RS-ResourceConfigMobility IE or by the TRS-ResourceSet IE, the sequence shall be generated according to clause 7.4.1.5.2 and mapped to resource elements according to clause 7.4.1.5.3 - for a zero-power CSI-RS configured by the ZP-CSI-RS-Resource IE, the UE shall assume that the resource elements defined in clause 7.4.1.5.3 are not used for PDSCH transmission subject to clause 5.1.4.2 of [6, TS 38.214]. The UE performs the same measurement/reception on channels/signals except PDSCH regardless of whether they collide with ZP CSI-RS or not.

Google

Proposal 2: Endorse the following TP for 38.214 to clarify that UE does not report the inference results if it fails to receive K transmission occasions within the DRX active time

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’ or ‘p-ssb-index-RSRP-r19’ and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability.

NEC

- Proposal 2: The configured P/SP NZP-CSI-RS resources for Set A (and not for Set B) should be available for other channel/signal (e.g., PDSCH) at least during model inference. Adopt the following TP#2 to TS 38.211.

- Reason for change: PDSCH rate-matching around virtual CSI-RS in Set A results in unnecessary waste of resource.

- Summary of change: Clarify that CSI-RS only configured in Set A is not used to perform PDSCH rate-matching.

- Consequences if not approved: The resource occupied by the virtual CSI-RS in Set A cannot be used for PDSCH transmission.

7.3.1.5 Mapping to virtual resource blocks <Unchanged part omitted> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE and not only configured by resourcesForChannelPrediction IE associated with CSI-ReportConfig with reportQuantity set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, or ‘p-ssb-index-RSRP-r19’, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. <Unchanged part omitted>

Proposal 3: If the applicability status of the corresponding CSI-ReportConfig changes from applicable to inapplicable, UE transmits a CSI report which contains an invalid value. Adopt the following TP#3 to TS 38.214

- Reason for change: For a P/SP CSI report, when the corresponding CSI-ReportConfig changes from applicable to inapplicable, and the inapplicable CSI-ReportConfig is not released, UE may be unable to perform model inference.

- Summary of change: If prediction is not performed by the UE, UE shall transmit a CSI report which contains an invalid value.

- Consequences if not approved: UE report is not clear after the functionality becomes inapplicable.

5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <Unchanged part omitted> For a CSI-ReportConfig with reportQuantity-r19 set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, ‘p-ssb-index-RSRP-r19’ or ‘none-bm-r19’, if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. For a CSI-ReportConfig with reportQuantity-r19 set to ‘p-cri-r19’, ‘p-cri-RSRP-r19’, ‘p-ssb-index-r19’, ‘p-ssb-index-RSRP-r19’ or ‘none-bm-r19’, after applicability reporting of the changes of applicability status of the CSI-ReportConfig from applicable to inapplicable, the UE shall transmit a CSI report which contains an invalid value. <Unchanged part omitted>

Samsung

Proposal 1: Adopt the following TP for TS 38.214 Clause 5.2.1.2

Reason for change: The configuration restriction of all resources in Set A with the same time domain behavior is too restrictive.

Summary of change: Relax the configuration restriction of all resources in Set A with the same time domain behavior.

Consequences if not approved: The configuration of Set A and Set B with different time domain behavior is not possible.

##### TS 38.214 Clause 5.2.2.3.1 NZP CSI-RS <omitted texts> The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration: - nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity. - periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. All the CSI-RS resources within one set not configured by resourcesForSetA-r19 are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources. <omitted texts>

Proposal 2: Adopt the following TP for TS 38.211 Clause 7.3.1.5

Reason for change: PDSCH rate-matching around virtual CSI-RS in Set A results in unnecessary waste of resource.

Summary of change: Clarify that CSI-RS only configured in Set A is not used to perform PDSCH rate-matching.

Consequences if not approved: The resource occupied by the virtual CSI-RS in Set A cannot be used for PDSCH transmission.

##### TS 38.214 Clause 7.3.1.5 Mapping to virtual resource blocks <omitted texts> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is only configured by resourcesForChannelPrediction-r19 IE associated with CSI-ReportConfig with reportQuantity set to ‘p-CRI-r19’, ‘p-CRI-RSRP-r19’, ‘p-SSB-Index-r19’, or ‘p-SSB-Index-RSRP-r19’; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. <omitted texts>

Proposal 3: Adopt the following TP for TS 38.214 Clause 5.2.1.4.2

Reason for change: There is a bracket related to Set A description. Report quantity name and RRC parameters name are not aligned.

Summary of change: Remove the bracket related to Set A description and align the report quantity name and RRC parameters name.

Consequences if not approved: The description related to Set A is unclear.

##### TS 38.214 Clause 5.2.1.4.2 Report quantity configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to ‘p-cri-r19p-CRI-r19’, ‘p-cri-RSRP-r19p-CRI-RSRP-r19’, ‘p-ssb-index-r19p-SSB-Index-r19’ or ‘p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19’, and with nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 and/or nroftimeinstance-r19nrofTimeInstance-r19, the following applies: - the UE is not required to update measurements for more than 64 CSI-RS or SSB resources given by resourcesForChannelMeasurement,[ and is not expected to predict for more than 64 CSI-RS or SSB resources given by resourcesForSetA-r19]resourcesForChannelPrediction-r19, - the UE shall report one of the following in a single report: - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting, if nroftimeinstance-r19nrofTimeInstance-r19 is not configured and reportQuantity-r19 is set to ‘ p-cri-r19p-CRI-r19’ or ‘ p-ssb-index-r19p-SSB-Index-r19’, - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting, with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19nrofTimeInstance-r19 is not configured and reportQuantity-r19 set to ‘ p-cri-RSRP-r19p-CRI-RSRP-r19’ or ‘ p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19’, - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting for each of nroftimeinstance-r19nrofTimeInstance-r19 time instance(s), if nroftimeinstance-r19nrofTimeInstance-r19 is configured and reportQuantity-r19 set to ‘p-cri-r19’ or ‘ p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19’, - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting for each of nroftimeinstance-r19nrofTimeInstance-r19 time instance(s), with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19nrofTimeInstance-r19 is configured and reportQuantity-r19 set to ‘ p-cri-RSRP-r19p-CRI-RSRP-r19’ or ‘ p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19’. <omitted texts>

Ericsson

Proposal 1. Clarify the UE assumptions for the applicability reporting of the NW configured predicted resources.

- Update specification: For the applicability report, the UE does not consider the NW time-frequency occupation of the predicted resources configured in resourcesForChannelPrediction-r19

Nokia

Proposal 1: For UE-sided BM Case-1 and 2, for CSI-RS resources in Set A, endorse the following text proposal to 38.214 Clause 5.2.2.3.1.

Reason for change: The same-periodicity requirement restricts the flexibility needed for AI/ML beam management, particularly when dealing with subsets of resource sets that require different transmission periodicities.

Summary of change: Allow different CSI-RS resources within a single set to have varied time-domain transmission behaviors when the resources belong to resourcesForSetA-r19, which are used for prediction.

Consequence if not approved: If this issue is not addressed, there could be ambiguity in determining the periodicity of CSI-RS resources in Set A.

5.2.2.3.1 NZP CSI-RS < Unchanged parts are omitted > The UE can be configured with one or more NZP CSI-RS resource set configuration(s) as indicated by the higher layer parameters CSI-ResourceConfig, and NZP-CSI-RS-ResourceSet. Each NZP CSI-RS resource set consists of K≥1 NZP CSI-RS resource(s). The following parameters for which the UE shall assume non-zero transmission power for CSI-RS resource are configured via the higher layer parameter NZP-CSI-RS-Resource, CSI-ResourceConfig and NZP-CSI-RS-ResourceSet for each CSI-RS resource configuration: - nzp-CSI-RS-ResourceId determines CSI-RS resource configuration identity. - periodicityAndOffset defines the CSI-RS periodicity and slot offset for periodic/semi-persistent CSI-RS. Except resources in resourcesForSetA-r19 used for predicted RS quantities reporting, aAll the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources. - resourceMapping defines the number of ports, CDM-type, and OFDM symbol and subcarrier occupancy of the CSI-RS resource within a slot that are given in Clause 7.4.1.5 of [4, TS 38.211]. - nrofPorts in resourceMapping defines the number of CSI-RS ports, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. - density in resourceMapping defines CSI-RS frequency density of each CSI-RS port per PRB, and CSI-RS PRB offset in case of the density value of 1/2, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. For density 1/2, the odd/even PRB allocation indicated in density is with respect to the common resource block grid. - cdm-Type in resourceMapping defines CDM values and pattern, where the allowable values are given in Clause 7.4.1.5 of [4, TS 38.211]. < Unchanged parts are omitted >

Based on the summary above, issues and corresponding proposals are provided as follows.

Issue 2.2.1# Relaxation of configuration restriction for Set A

It is proposed by several companies [2-6,9,11,13-15] that legacy configuration restriction for CSI Resource Settings shall be relaxed for Set A. Based on the discussion in RAN1#122bis, the following configuration restriction in TS38.214 are identified and aspect D was agreed:

- A) When a UE is configured with multiple CSI-ResourceConfigs consisting the same NZP CSI-RS resource ID, the same time domain behavior shall be configured for the CSI-ResourceConfigs.

- B) All CSI Resource Settings linked to a CSI Report Setting shall have the same time domain behavior.

- C) All the CSI-RS resources within one set are configured with the same periodicity, while the slot offset can be same or different for different CSI-RS resources

- (Agreed) D) UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within aperiodic resource setting.

- E) For virtual resource blocks mapping of PDSCH in TS38.211, rate matching of the resource elements in the corresponding physical resource blocks are not performed around the NZP CSI-RS only configured in Set A for CSI report for inference.

There are proposals from several companies [1,4,5,7,8] on the aspects E) and a TP is formulated based on the TP in [1].

Proposal. 2.2.1

Adopt the following TP for TS 38.211 Clause 7.3.1.5.

Reason for change: PDSCH rate-matching around virtual CSI-RS in Set A results in unnecessary waste of resource.

Summary of change: Clarify that CSI-RS only configured in Set A is not used to perform PDSCH rate-matching.

Consequences if not approved: The resource occupied by the virtual CSI-RS in Set A cannot be used for PDSCH transmission.

7.3.1.5 Mapping to virtual resource blocks <omitted texts> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE and not only configured by the resourcesForChannelPrediction-r19 in CSI-ReportConfig with reportQuantity set to ‘p-CRI-r19’, ‘p-CRI-RSRP-r19’, ‘p-SSB-Index-r19’, or ‘p-SSB-Index-RSRP-r19’, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | Proposal. 2.2.1 is necessary to justify the overhead reduction gain of AI/ML based beam prediction. |
| LGE | Not support, as we commented in the last meeting: “we still think that it is not needed. The reason is that NZP-CSI-RS resources in real deployment are mostly cell-specific CSI-RS(which is being basically transmitted), it is risky to say that NZP-CSI-RS resources configured in Set A for inference are not transmitted and don’t have to be rate-matched for PDSCH. NZP-CSI-RS resources configured in Set A for inference can also be used for RRM measurement. In short, even without the TP, system is not broken.” |
| Vivo | Support |
| Huawei, HiSilicon | Support the direction. In addition to Set A for inference, the CSI-RS in Set A/Set B for ApplicabilityReportConfig for Option B can also be virtual resources. Thus, the same rule can be applied to “resourcesForChannelMeasurement-r19 in ApplicabilitySetConfigCSI-r19 and resourcesForChannelPrediction-r19 in ApplicabilitySetConfigCSI-r19”. So, we suggest to update the TP as follows: 7.3.1.5 Mapping to virtual resource blocks <omitted texts> The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled Ues as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS, which is according to clause 7.4.1.5 and not configured by the TRS-ResourceSet IE and not only configured by any of the resourcesForChannelPrediction-r19 in CSI-ReportConfig with reportQuantity set to ‘p-CRI-r19’, ‘p-CRI-RSRP-r19’, ‘p-SSB-Index-r19’, or ‘p-SSB-Index-RSRP-r19’, resourcesForChannelMeasurement-r19 in ApplicabilitySetConfigCSI-r19, and resourcesForChannelPrediction-r19 in ApplicabilitySetConfigCSI-r19, if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI for multicast, G-CS-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as ‘not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. <omitted texts>. |
| NEC | We think it is critical for this AI/ML feature and we are OK with the version discussed in last meeting. The update from Huawei is also fine. |
| ZTE | The current wording looks a little confusing. Suggest to remove the new added sentence to the end of this paragraph, that is, ‘except if the non-zero-power CSI-RS is only configured by the resourcesForChannelPrediction-r19…’ |
| QC | This has been already debated for a few meetings now, and unless the modification is critical (which we believe it isn’t) we should not debate it at this maintenance stage. |
| FL2 | It seems that the proposal is still not stable yet. More discussion is needed in the second round. |

Issue 2.2.2# Correction on the description of inference reporting

Samsung [8] proposed several corrections in Clause 5.2.1.4.2 on the description of inference reporting.

Proposal. 2.2.2

Adopt the following TP for TS 38.214 Clause 5.2.1.4.2.

Reason for change: There is a bracket related to Set A description. Report quantity name and RRC parameters name are not aligned with the latest TS38.331.

Summary of change: Remove the bracket related to Set A description and align the report quantity name and RRC parameters name.

Consequences if not approved: The description related to Set A is unclear.

##### 5.2.1.4.2 Report quantity configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19p-CRI-r19', 'p-cri-RSRP-r19p-CRI-RSRP-r19', 'p-ssb-index-r19p-SSB-Index-r19' or 'p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19', and with nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 and/or nroftimeinstance-r19nrofTimeInstance-r19, the following applies: - the UE is not required to update measurements for more than 64 CSI-RS or SSB resources given by resourcesForChannelMeasurement,[ and is not expected to predict for more than 64 CSI-RS or SSB resources given by resourcesForSetA-r19]resourcesForChannelPrediction-r19, - the UE shall report one of the following in a single report: - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting, if nroftimeinstance-r19nrofTimeInstance-r19 is not configured and reportQuantity-r19 is set to ' p-cri-r19p-CRI-r19' or ' p-ssb-index-r19p-SSB-Index-r19', - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting, with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19nrofTimeInstance-r19 is not configured and reportQuantity-r19 set to ' p-cri-RSRP-r19p-CRI-RSRP-r19' or ' p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19', - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting for each of nroftimeinstance-r19nrofTimeInstance-r19 time instance(s), if nroftimeinstance-r19nrofTimeInstance-r19 is configured and reportQuantity-r19 set to 'p-cri-r19' or ' p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19', - nrofreportedpredictedrs-r19nrofReportedPredictedRS-r19 different P-CRIs or P-SSBRIs of the second Resource Setting for each of nroftimeinstance-r19nrofTimeInstance-r19 time instance(s), with corresponding predicted L1-RSRP(s), if nroftimeinstance-r19nrofTimeInstance-r19 is configured and reportQuantity-r19 set to ' p-cri-RSRP-r19p-CRI-RSRP-r19' or ' p-ssb-index-RSRP-r19p-SSB-Index-RSRP-r19'. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | Proposal. 2.2.2 is necessary to remove the bracket in specification related to Set A description and align the report quantity name. |
| LG | OK as an alignment CR. |
| vivo | Support |
| Huawei, HiSilicon | OK |
| ZTE | OK |
| QC | OK |

Others

Google [6] proposed to add additional restriction on the condition of CSI reporting for inference. However, based on the discussion in the previous meeting, the proposal has very limited support.

Several companies [7,10] proposed to consider specify behavior related to applicability reporting. However, it seems that those behavior are optimization rather than critical issues.

Nokia [11] proposed to consider the aspect B) on the relaxation of configuration restriction for Set A. However, based on the discussion in the previous meeting, the proposal has very limited support.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| Huawei, HiSilicon | For the issue proposed by Google, we think there is no need to add additional restriction. For the issue related to applicability reporting, we think the current UE behaviour is clear. For the issue proposed by Nokia, we think the relaxation is beneficial. Since the CSI-RS in Set A is virtual, there is no need to restrict other resource sets (e.g., Set B) to be the same resource type as Set A, which makes the configuration to be more flexible. |

2.3 CSI report for performance monitoring

Google

Proposal 2-3: Endorse the following TP for 38.214 to clarify that UE does not report the RS-PAI if it fails to receive the corresponding transmission occasions within the DRX active time

5.2.2.5 CSI reference resource definition <omitted text> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise.

NEC

Proposal 1: For a monitoring report, only if its linked inference report is activated, the UE reports RS-PAI; otherwise, the UE shall drop the monitoring report. Adopt the following TP to TS 38.214 V19.1.0.

- Reason for change: For a monitoring report, if its linked inference report is stopped during performance monitoring, the UE will no longer be able to obtain the inference results linked with the (remaining) monitoring instance(s). At this time, the corresponding monitoring behavior should be defined.

- Summary of change: For a monitoring report, if the linked inference report is activated, the UE may report the monitoring report; otherwise, the UE may drop the monitoring report.

- Consequences if not approved: wrong RS-PAI report after the linked inference report is stopped.

##### 5.2.1.4.3b RS-PAI Reporting <Unchanged part omitted> - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). - the UE reports a RS-PAI report of the second CSI Reporting Setting only if the report of the first CSI Reporting Setting is activated, and drops the report of the second CSI Reporting Setting otherwise. <Unchanged part omitted>

Sharp

Proposal 1: Adopt the following TP#1 in TS 38.214 to clarify that values 1 or 3 configured by nroftransmissionOccasion-r19, which would result in RS-PAI with 1 or 2 bits, should be prevented for P-CSI and SP-CSI on PUCCH.

- Reason for change: According to RAN1 agreements, periodic CSI reporting and semi-persistent CSI reporting on PUCCH are supported for reporting RS-PAI. The size of RS-PAI may be 1, 2, 3 or 4 bits, depending on the value configured by nroftransmissionOccasion-r19. However, NR specifications do not support PUCCH carrying 1 or 2 information bits other than HARQ-ACK/SR. In addition, details for reporting RS-PAI on PUCCH had not been discussed in RAN1. Therefore, to avoid undiscussed/unspecified RS-PAI reporting on PUCCH with 1 or 2 bits, configurations (i.e., nroftransmissionOccasion-r19) that would result in RS-PAI with 1 or 2 bits should be prevented in specifications for P-CSI and SP-CSI on PUCCH.

- Summary of change: Clarify that when a UE is configured with P-CSI on PUCCH or SP-CSI on PUCCH for RS-PAI reporting, the UE shall expect that the value M provided by nroftransmissionOccasion-r19 is not set to 1 or 3.

- Consequences if not approved: Unspecified UE and base station behaviours to report RS-PAI on PUCCH with 1or 2 bits when nroftransmissionOccasion-r19 is configured to 1 or 3.

TP#1 ##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on L1-RSRP(s) measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s) if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). When a UE is configured with a periodic or semi-persistent reporting setting on PUCCH for the CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', the UE shall expect that the value M given by nroftransmissionOccasion-r19 is not set to 1 or 3.

Based on the summary above, there is no proposal formulated at this stage.

Google [6] proposed to add additional restriction on the condition of CSI reporting for performance monitoring. However, based on the discussion in the previous meeting, the proposal has very limited support.

NEC [7] proposed to stop monitoring report depending on the associated inference report. However, this seems to be an optimization rather than an essential issue.

Sharp [9] proposed to restrict the number of transmission occasions such that the payload size of PUCCH is always more than 2 bits. However this is unnecessary since zero padding is done in TS38.212 in this case to ensure the payload size is no less than 3 bis. Related texts in TS38.212 are cited below.

##### TS 38.212. 6.3.1.1.2 CSI only Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence , without two-part CSI report(s) UCI bit sequence; CSI report number. CSI report #1 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. CSI report #2 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. …. CSI report #n as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, all CSI sub-reports within the CSI report #i are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId.. If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated, and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| vivo | Share the same view as FL, the TP is unnecessary |
| Huawei, HiSilicon | We agree that zero padding is a better solution. However, the current description in TS38.212 is for CSI part 2. Similar descriptions should be added for reporting RS-PAI in CSI part 1, dedicatedly for RS-PAI. |
| Sharp | Firstly, we agree with Huawei that Zero padding is only applied to CSI part 2 to ensure its length is at least 3 bits. This operation does not apply to CSI part 1 or to CSI reports without two parts, since a CSI report without two parts or with part 1 is typically longer than 2 bits in legacy CSI reporting and therefore would not be that small. Second, zero-padding could be an option to solve the issue. It needs to also apply the zero padding to a single CSI without two parts. Lastly, we think restricting M to large values for P-CSI and SP-CSI on PUCCH could be fine. One-shot or quick monitoring evaluation by using M=1 or 3 can be performed by SP-CSI on PUSCH or AP-CSI. |
| FL2 | Thanks for HW and Sharp to point out the issue. This will be discussed in the next round. |

2.4 CSI processing Timeline

Sharp

Proposal 2: Adopt the following TP#2 in TS 38.214 to clarify that a maximum value Z and Z’, among M updated CSI reports and Mx updated CSI reports, are used to decide time locations Zref and Z'ref , respectively.

- Reason for change: In TS38.214, the UE determines the time locations Zref  and Z'ref based on selecting a maximum value Z and Z’ among the M updated CSI reports, respectively. In Rel-19, AI/ML-based CSI reports can be configured and triggered. Such AI/ML-based CSI reports have their Z and Z’ computation delay requirements and should be also taken into consideration when  determining the time locations Zref  and Z'ref .

- Summary of change: Clarify that a maximum value Z and Z’, among M updated CSI reports and Mx updated CSI reports, are used to decide time locations Zref  and Z'ref , respectively.

- Consequences if not approved: UE may determine Zref  and Z'ref based on a subset of updated CSI reports except the AI/ML-based updated CSI reports, resulting in insufficient time for CSI computation and leading to the invalid CSI reporting.

TP#2 ## 5.4 UE CSI computation time When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, the UE shall provide a valid CSI report for the n-th triggered report, - if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts no earlier than at symbol Zref, and - if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts no earlier than at symbol Z'ref(n), where Zref is defined as the next uplink symbol with its CP starting after the end of the last symbol of the PDCCH triggering the CSI report(s), and where Z'ref(n), is defined as the next uplink symbol with its CP starting after the end of the last symbol in time of the latest of: aperiodic CSI-RS resource for channel measurements, aperiodic CSI-IM used for interference measurements, and aperiodic NZP CSI-RS for interference measurement for a CSI-ReportConfig, or for all triggered sub-configurations if CSI-ReportConfig contains multiple sub-configurations, when aperiodic CSI-RS is used for channel measurement for the n-th triggered CSI report, and where Tswitch is defined in clause 6.4 and is applied only if Z1 of table 5.4-1 is applied. If the PUSCH indicated by the DCI is overlapping with another PUCCH or PUSCH, then the CSI report(s) are multiplexed following the procedure in clause 9.2.5 of [6, TS 38.213] and clause 5.2.5 when applicable, otherwise the CSI report(s) are transmitted on the PUSCH indicated by the DCI. When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the corresponding CSI report(s) including the effect of the timing advance, starts earlier than at symbol Zref, - the UE may ignore the scheduling DCI if no HARQ-ACK or transport block is multiplexed on the PUSCH. When the CSI request field on a DCI triggers a CSI report(s) on PUSCH, if the first uplink symbol to carry the n-th CSI report including the effect of the timing advance, starts earlier than at symbol Z'ref(n), - the UE may ignore the scheduling DCI if the number of triggered reports is one and no HARQ-ACK or transport block is multiplexed on the PUSCH - Otherwise, the UE is not required to update the CSI for the n-th triggered CSI report. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the last symbol of the PDCCH triggering the CSI report(s), the PDCCH candidate that ends later in time is used. Z, Z' and µ are defined as: Z=max⁡{maxm=0,…,M-1Zm,maxm=0,…,Mx-1Zm}⁡ and Z'=max⁡{maxm=0,…,M-1(Z'(m)),maxm=0,…,Mx-1Z'm}, where M and Mx is are the number of updated CSI report(s) according to Clause 5.2.1.6, (Z(m),Z'(m)) corresponds to the m-th updated CSI report and is defined as <omit>

Based on the summary above, there is no proposal formulated at this stage.

NEC [7] proposed to clarify the calculation of Z timeline in case of CSI multiplexing. However, the specification says that the updated CSI report(s) are used for the calculation which is nothing wrong (although the notation may cause confusion but seems ok).

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| LGE | Open to discuss. |
| vivo | The existing specification seems clear enough |
| Huawei, HiSilicon | The current spec is clear, no need to update. |
| Sharp | We do not think the current spec is clear that M updated CSI reports can include Mx updated CSI reports. According to Clause 5.2.1.6, "M " and "Mx" have their specific definitions and refer to distinct CSI reports, which should be treated as separate. It is important to clearly specify that in spec with accurate notations; otherwise, ambiguity arise, such as misunderstanding that AI-based CSI reports are not necessarily considered when determining CSI computation time. |

2.5 Associated ID

vivo

- Proposal 2. Adopt the following TPs for TS 38.214 Clause 5.2.1.4.1

Reason for change: The UE assumption with same associated ID configured to be associated with different resource sets is unclear.

Summary of change: Clarify that the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets with same associated ID.

Consequences if not approved: UE may not know the meaning of the similar properties.

< Start of change request >

##### 5.2.1.4.1	Resource Setting configuration

<Unchanged part omitted>

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig:

-	if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting.

-	if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released.

<End of change request>

< Start of change request >

##### 5.2.1.4.1	Resource Setting configuration

<Unchanged part omitted>

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', the UE may be configured with one or two Associated ID(s) in CSI-ReportConfig:

-	if only associatedIDforSetA-r19 is configured, it is associated with the resource sets of the second Resource Setting, and the UE expects that all CSI-RS resources or SS/PBCH block resources in the resource set of the first Resource Setting are among the CSI-RS resources and/or SS/PBCH block resources in the resource set of the second Resource Setting.

-	if associatedIDforSetA-r19 and associatedIDforSetB-r19 are configured, they are associated with the resource set of the second Resource Setting and of the first Resource Setting, respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter reportQuantity-r19 is set to 'none-bm-r19', 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', if the same Associated ID is configured to be associated with different resource sets, the UE may assume similar properties of downlink spatial domain transmission filter for the CSI-RS resources and/or SS/PBCH block resources among those different resource sets, irrespective of when the corresponding Resource Setting(s) is configured by higher layer signalling or released.

<End of change request>

Based on the summary above, there is no proposal formulated at this stage.

vivo [7] proposed to clarify “similar property” in the description of associated ID. However, this issue has been discussed for several meetings and lack of consensus.

| Company | Comments |
| --- | --- |
| FL | Please share your comments and suggestions, if any. |
| LGE | Support. |
| vivo | Support |
| Huawei, HiSilicon | The current spec is clear and no need to discuss. |

- Remaining issues for NW-side AI/ML model

ZTE

Proposal 1: For NW-side data collection, support the following logging contents, where Set A and Set B are only used for discussion purpose and does not necessarily known by UE.

- Opt 1: L1-RSRPs of all measured beams (Set A and Set B).

- Opt 2: L1-RSRPs of all measured beams in Set B, and Top-K beams based on the measured beams (beam index and L1-RSRP) in Set A.

The above proposal was discussed in the past several meetings but seems difficult to reach consensus.

| Company | Comments |
| --- | --- |
| FL | Please share your view or suggestion. |
| LGE | Not support. RAN2 already decided about the contents of NW-side data collection and it is already implemented in RAN2 spec. |
| Huawei, HiSilicon | Seems not needed. There are already agreements on this in RAN2. |
| ZTE | According to RAN2 LS, the logging content for data collection is ‘L1-RSRP and/or Beam ID’. Whether to include both L1-RSRP/Beam ID or only one is still a pending issue which needs to be further confirmed by RAN1. |

- Second round Discussion

Proposal. 2.1.1

Adopt the following TP for TS 38.214 Clause 5.2.1.6.

Reason for change: CPU occupation time for beam prediction related UE-sided data collection is not captured correctly.

Summary of change: Reflect the RAN1 agreements accurately in Clause 5.2.1.6 of TS 38.214.

Consequence if not approved: Incorrect behaviour for CPU occupation time for beam prediction related UE-sided data collection.

5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-csi-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-BM-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of each the transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter eventType configured, the CPU is occupied from when CSI-ReportConfig is configured and until CSI-ReportConfig is released. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | Per chair’s guidance, this proposal will have a quick check in the next online. Please share your comments if you have concern. |
| QC | As discussed before, the added part in red does NOT capture the CPU occupation time for AP, and initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report. So the red part needs to be rewritten to incorporate those aspects. |

Proposal. 2.3.1-A

Adopt the following TP for TS 38.214 Clause 5.2.1.4.3b.

Reason for change: According to RAN1 agreements, periodic CSI reporting and semi-persistent CSI reporting on PUCCH are supported for reporting RS-PAI. The size of RS-PAI may be 1, 2, 3 or 4 bits, depending on the value configured by nroftransmissionOccasion-r19. However, NR specifications do not support PUCCH carrying 1 or 2 information bits other than HARQ-ACK/SR. In addition, details for reporting RS-PAI on PUCCH had not been discussed in RAN1. Therefore, to avoid undiscussed/unspecified RS-PAI reporting on PUCCH with 1 or 2 bits, configurations (i.e., nroftransmissionOccasion-r19) that would result in RS-PAI with 1 or 2 bits should be prevented in specifications for P-CSI and SP-CSI on PUCCH.

Summary of change: Clarify that when a UE is configured with P-CSI on PUCCH or SP-CSI on PUCCH for RS-PAI reporting, the UE shall expect that the value M provided by nroftransmissionOccasion-r19 is not set to 1 or 3.

Consequences if not approved: Unspecified UE and base station behaviours to report RS-PAI on PUCCH with 1or 2 bits when nroftransmissionOccasion-r19 is configured to 1 or 3.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on L1-RSRP(s) measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s) if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). When a UE is configured with a periodic or semi-persistent reporting setting on PUCCH for the CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', the UE shall expect that the value M given by nroftransmissionOccasion-r19 is not set to 1 or 3.

Proposal. 2.3.1-B

Adopt the following TP for TS 38.214 Clause 6.3.1.1.2.

Reason for change: For P/SP CSI reporting on PUCCH for performance monitoring, it is possible that the number of information bits of RS-PAI becomes less than 3 bits. However, NR specifications do not support PUCCH carrying 1 or 2 information bits other than HARQ-ACK/SR.

Summary of change: For P/SP CSI reporting on PUCCH for performance monitoring, the number of information bits of related CSI is zero padded to 3 bits.

Consequences if not approved: For P/SP CSI reporting on PUCCH for performance monitoring, 1-bit or 2-bit payload size of RS-PAI is not supported.

##### 6.3.1.1.2 CSI only <omitted texts> If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. If the length of UCI bit sequence a0,a1,a2,a3,...,aA-1 is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3. Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence , without two-part CSI report(s) UCI bit sequence; CSI report number. CSI report #1 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. CSI report #2 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. …. CSI report #n as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, all CSI sub-reports within the CSI report #i are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId.. If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated, and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3. <omitted texts>

| Company | Comments |
| --- | --- |
| FL | Base on the comments from the first round, two TPs are provided for P/SP CSI reporting on PUCCH for performance monitoring. Proposal. 2.3.1-A is imposing restriction on the configuration of M to avoid the payload size of RS-PAI to be less than 3 bits. Proposal. 2.3.1-B reuses the similar operation of CSI part 2 (highlighted in yellow) for the payload size of CSI including RS-PAI to be less than 3 bits (i.e., zero padding is performed). It seems more natural to go for the direction of Proposal. 2.3.1-B to address the issue. Please share you view. |
| Sharp | Thank you FL for providing two TPs. We are fine with either option. For the Proposal. 2.3.1-B, we think the correction below on the field size of the RS-PAI may be necessary in light of the zero-padding operation. - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a max(log2(M+1),3)-bit field (M is given by nroftransmissionOccasion-r19). |

- Remaining issues for next meeting

5.1 Issue of 1-bit or 2-bit RS-PAI on PUCCH

Companies are encouraged to consider the issues of 1-bit or 2-bit RS-PAI on PUCCH. Proposal. 2.3.1-A and Proposal. 2.3.1-B represent two directions (configuration restriction vs. zero padding) to resolved the issue.

Proposal. 2.3.1-A

Adopt the following TP for TS 38.214 Clause 5.2.1.4.3b.

Reason for change: According to RAN1 agreements, periodic CSI reporting and semi-persistent CSI reporting on PUCCH are supported for reporting RS-PAI. The size of RS-PAI may be 1, 2, 3 or 4 bits, depending on the value configured by nroftransmissionOccasion-r19. However, NR specifications do not support PUCCH carrying 1 or 2 information bits other than HARQ-ACK/SR. In addition, details for reporting RS-PAI on PUCCH had not been discussed in RAN1. Therefore, to avoid undiscussed/unspecified RS-PAI reporting on PUCCH with 1 or 2 bits, configurations (i.e., nroftransmissionOccasion-r19) that would result in RS-PAI with 1 or 2 bits should be prevented in specifications for P-CSI and SP-CSI on PUCCH.

Summary of change: Clarify that when a UE is configured with P-CSI on PUCCH or SP-CSI on PUCCH for RS-PAI reporting, the UE shall expect that the value M provided by nroftransmissionOccasion-r19 is not set to 1 or 3.

Consequences if not approved: Unspecified UE and base station behaviours to report RS-PAI on PUCCH with 1or 2 bits when nroftransmissionOccasion-r19 is configured to 1 or 3.

##### 5.2.1.4.3b RS-PAI Reporting When the UE is configured with a first CSI Reporting Setting for reporting P-CRI, P-SSBRI, and/or P-L1-RSRP, based on a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', and a second CSI Reporting Setting for reporting RS-PAI, based on a CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', and the second Reporting Setting is linked to the first Reporting Setting by inferenceReportConfigId-r19, the reporting of RS-PAI corresponding to the second CSI Reporting Setting shall consider the following: - for each of the nroftransmissionOccasion-r19 latest transmission occasion(s), where the latest transmission occasion of nroftransmissionOccasion-r19 latest transmission occasion(s), and the latest report of the first CSI Reporting Setting considered for linking, are no later than the CSI reference resource corresponding to the CSI report of the second CSI Reporting setting, the UE shall: - perform L1-RSRP measurements for the configured CSI-RS resources, or SS/PBCH Block resources, associated with the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - determine the best nrofBestBeamforMonitoring-r19 CSI-RS resources, or SS/PBCH Block resources based on L1-RSRP(s) measured CSI-RS resources, or SS/PBCH Block resources of the corresponding Resource Set; - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s) if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a log2(M+1)-bit field (M is given by nroftransmissionOccasion-r19). When a UE is configured with a periodic or semi-persistent reporting setting on PUCCH for the CSI-ReportConfig with reportQuantity-r19 set to 'rs-pai-r19', the UE shall expect that the value M given by nroftransmissionOccasion-r19 is not set to 1 or 3.

Proposal. 2.3.1-B

Adopt the following TP for TS 38.214 Clause 6.3.1.1.2.

Reason for change: For P/SP CSI reporting on PUCCH for performance monitoring, it is possible that the number of information bits of RS-PAI becomes less than 3 bits. However, NR specifications do not support PUCCH carrying 1 or 2 information bits other than HARQ-ACK/SR.

Summary of change: For P/SP CSI reporting on PUCCH for performance monitoring, the number of information bits of related CSI is zero padded to 3 bits.

Consequences if not approved: For P/SP CSI reporting on PUCCH for performance monitoring, 1-bit or 2-bit payload size of RS-PAI is not supported.

##### 6.3.1.1.2 CSI only <omitted texts> If none of the CSI reports for transmission on a PUCCH is of two parts, the CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-12, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. If the length of UCI bit sequence a0,a1,a2,a3,...,aA-1 is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3. Table 6.3.1.1.2-12: Mapping order of CSI reports to UCI bit sequence , without two-part CSI report(s) UCI bit sequence; CSI report number. CSI report #1 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. CSI report #2 as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. …. CSI report #n as in Table 6.3.1.1.2-7/7A/7B/8/8A/8B/8C/8D/8E/8F/8G/8H. NOTE: For a CSI report #i containing Nisub CSI sub-reports, where i∈{1,2,…,n}, all CSI sub-reports within the CSI report #i are mapped to the corresponding segment of the UCI bit sequence of CSI report #i, from upper part to lower part of the segment, in increasing order of CSI sub-report number. CSI sub-report #1, CSI sub-report #2, …, CSI sub-report #Nisub correspond to the CSI sub-reports in increasing order of reportSubConfigId.. If at least one of the CSI reports for transmission on a PUCCH is of two parts, two UCI bit sequences are generated, and . The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-13, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to. The CSI fields of all CSI reports, in the order from upper part to lower part in Table 6.3.1.1.2-14, are mapped to the UCI bit sequence starting with . The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to . If the length of UCI bit sequence is less than 3 bits, zeros shall be appended to the UCI bit sequence until its length equals 3. <omitted texts>

5.2 Issue of CPU occupation time for data collection

Companies are encouraged to consider the issue of CPU occupation time for data collection. Proposal. 2.1.1 can be the starting point.

Proposal. 2.1.1

Adopt the following TP for TS 38.214 Clause 5.2.1.6.

Reason for change: CPU occupation time for beam prediction related UE-sided data collection is not captured correctly.

Summary of change: Reflect the RAN1 agreements accurately in Clause 5.2.1.6 of TS 38.214.

Consequence if not approved: Incorrect behaviour for CPU occupation time for beam prediction related UE-sided data collection.

5.2.1.6 CSI processing criteria <omitted texts> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none' and CSI-RS-ResourceSet with higher layer parameter trs-Info not configured, or reportQuantity set to 'none-bm-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - A semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report) occupies CPU(s) from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. - An aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol between Z3 symbols after the first symbol after the PDCCH triggering the CSI report and Z3' symbols after the last symbol of the latest one of each CSI-RS/SSB resource for channel measurement for L1-RSRP computation. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-csi-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3' symbols after the transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to 'none-BM-r19', the CPU(s) are occupied for a number of OFDM symbols as follows: - CSI report occupies CPU(s) from the first symbol of each transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation, until Z3' symbols after the last symbol of the latest one of each the transmission occasion of periodic or semi-persistent CSI-RS/SSB resource for channel measurement for L1-RSRP computation in each transmission occasion. where (Z3,Z3') are defined in the table 5.4-2. For a CSI report with CSI-ReportConfig with higher layer parameter eventType configured, the CPU is occupied from when CSI-ReportConfig is configured and until CSI-ReportConfig is released. <omitted texts>

References

R1-2600092	Maintenance of Rel-19 AI/ML for air interface	Huawei, HiSilicon

R1-2600267	Discussion on maintenance of AI for Air Interface       ZTE Corporation, Sanechips

R1-2600287	Maintenance on Rel-19 AI/ML for NR air interface	CATT

R1-2600472	Maintenance on AI/ML for NR Air Interface		vivo

R1-2600528	Maintenance on Rel-19 AI/ML	InterDigital, Inc.

R1-2600624	Maintenance for AI/ML for NR Air Interface	Google

R1-2600654	Remaining Issues on AIML for NR Air Interface	NEC

R1-2600723	Remaining issue on AI/ML for NR Air Interface	Samsung

R1-2600982	Maintenance on AI/ML for NR Air Interface	Sharp

R1-2601043	Remaining Issues of Rel-19 AI/ML for NR Air Interface	Ericsson

R1-2501082	Maintenance on AI/ML for NR Air Interface	Nokia

Appendix A. Previous agreements

RAN1#116

Agreement

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

  - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

    - FFS on the quantization method of probability information

    - Probability information is the probability of the beam to be the Top 1 or Top K beam

  - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

    - FFS on definition of reported RSRP

    - FFS on the definition and quantization method of confidence information

  - Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement

- For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework

- FFS on whether/how potential enhancement is needed

Conclusion

For UE sided model at least for inference, for measurement, the configuration of Set B,

- take the current CSI framework as the starting point

RAN1#116b

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Agreement

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Agreement

For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

  - Other necessary configuration are not precluded.

Agreement

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt 2: Performance monitoring based

  - FFS details

- Other options are not precluded.

RAN1#117

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

Agreement

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support

- Support differential L1-RSRP reporting with legacy quantization step and range

  - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP

  - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Agreement

Following Working Assumption is confirmed.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

- Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

RAN1#118

Agreement

For UE sided model in beam management, introduce support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

RAN1#118bis

Agreement

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

Agreement

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Agreement

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

RAN1#119

Agreement

- For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

RAN1#120

Agreement

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

Agreement

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

Agreement

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances,

- the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Agreement

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

RAN1#120b

Agreement

For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported.

Agreement

For BM-Case1, for the Top K beam(s) report as the inference results

- K is configured in inference report configuration to the UE.

Agreement

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

RSRP #1

differential RSRP #2

…

differential RSRP #K

- CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K

- RSRP #1 is absolute predicted RSRP

- Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Agreement

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

Time instance indicator

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

RSRP #1 of time instance #1

Differential RSRP #2 of time instance #1

…

Differential RSRP #K of time instance #1

Differential RSRP #1 of time instance #2

Differential RSRP #2 of time instance #2

…

Differential RSRP #K of time instance #2

…

Differential RSRP #1 of time instance #N

Differential RSRP #2 of time instance #N

…

Differential RSRP #K of time instance #N

  - Time instance indicator exist if N > 1

  - The size of CSI field for time instance indicator is

  - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances

  - Time instance #1 corresponds to the time instance indicated by the time instance indicator

  - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances

    - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s)

  - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K

  - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Agreement

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

Agreement

For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Agreement

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

Agreement

For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as:

- At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

  - Where K is the number of predicted beam(s) in the corresponding inference report per time instance

  - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring

    - M= 1, 2

  - FFS: detailed rule or signalling

Agreement

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model:

  - Support the size of a set for monitoring is the same as the size of Set A,

    - The n-th resource in the set for monitoring is linked to the n-th resource in Set A.

  - Support the size of a set for monitoring is smaller than the size of Set A

Working Assumption

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Working Assumption

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

- wherein N (N>=1) is configured in CSI-ReportConfig

- FFS on additional rule for counting N linked pair

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Conclusion

For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Agreement

- For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection:

  - Option 1: only dedicated AI/ML PU is occupied,  is reported by UE.

    - And

  - Option 2: only legacy CPU is occupied,  it is reported by UE.

  - Option 3: both dedicated AI/ML PU and legacy CPU are occupied,  is reported by UE.

    - And

Note: The supported option by UE is reported by UE capability, if multiple options are supported.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

  - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Working Assumption

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

RAN1#121

Agreement

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A,

- support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support

  - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring

    - The x-th MSB of the bitmap corresponds to x-th resource in Set A

    - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring.

  - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

- Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately.

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement

For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”.

Agreement

For UE-sided model, for inference report for BM-Case2, a time instance for prediction is defined as a slot.

Conclusion

For NW sided model for L1-RSRP report in L1 signaling, legacy quantization steps and range are reused.

Agreement

For the determination of CSI report priority value of a CSI report for inference, the existing  is reused.

- k = 0 for the CSI report for inference.

For the determination of CSI report priority value of a CSI report for monitoring, the existing  is reused.

- k = 0 for the CSI report for monitoring.

Agreement

For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference,

- For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively.

  - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively.

  - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed.

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively.

- Detailed values of d and d’ can be further discussed in UE feature.

Agreement

For UE-sided model, regarding a CSI-ReportConfig for data collection,

- Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured.

Agreement

For NW-sided model, for inference, when M<the size of measurement resource set, the beam information is CRI/SSBRI

Note: The purpose, such as above “For NW-sided model, for inference” will not be specified in RAN1 specifications.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1,

- Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report.

- Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report.

- Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report).

Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is (0 ≤≤ N)

- Where  is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig the size of CSI field associated with the RS-PAI is

Agreement

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Agreement

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring

  - The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

    - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring.

    - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

  - Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement

For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

Agreement

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU.

- If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused

- If the CSI report is semi-persistent or periodic,

  - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report.

Agreement

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

  - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

  - TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present

RAN1#122

Agreement

Adopt the following TP for the clarification of ranking information.

Reason for change: The description of ranking information for P-CRIs or P-SSBRIs is unclear.

Summary of change: Clarify that the ranking information is for P-CRIs or P-SSBRIs.

Consequences if not approved: UE cannot generate the ranking information.

TP for TS 38.214 Clause 5.2.1.4.3a P-CRI, P-SSBRI, and P-L1-RSRP reporting <omitted texts> For a CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', if nroftimeinstance-r19 is configured, the UE is configured with TimeGap-r19 indicating a time gap between a reference time and the earliest time instance of nroftimeinstance-r19 time instance(s) (defined in slot(s)). If nroftimeinstance-r19 is greater than 1, the same time gap is considered between two consecutive time instances. The UE considers the reference time to be the slot of the latest one of each CSI-RS/SSB resource, for channel measurement, respective latest CSI-RS/SSB transmission occasion no later than the corresponding CSI reference resource of the CSI report. For P-CRI or P-SSBRI reporting without P-L1-RSRP, the ranking information of the nrofreportedpredictedrs-r19 P-CRIs or P-SSBRIs (per time instance, if nroftimeinstance-r19 is configured) is conveyed by the order of the P-CRIs or P-SSBRIs reported in the CSI report, where the first reported P-CRI or P-SSBRI ranks first. <omitted texts>

Agreement

Adopt the following TP for CSI reporting for monitoring.

Reason for change: The condition associated with CSI reporting for RS-PAI is unclear.

Summary of change: Clarify that the timeinstanceformonitoring-r19-th time instance is used in case of BM-Case2 for condition check. Adding the description on resource mapping methods depending on the size of the set for monitoring is smaller than or the same as the size of Set A.

Consequences if not approved: gNB and UE may have different understanding on the condition associated with CSI reporting for RS-PAI.

TP for TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting ##### 5.2.1.4.3b RS-PAI Reporting <omitted texts> - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources is mapped one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), if nroftimeinstance-r19 is not configured, or for the timeinstanceformonitoring-r19-th time instance if nroftimeinstance-r19 is configured, of the linked report of the first CSI Reporting Setting, wherein the mapping between CSI-RS resources, or SS/PBCH Block resources of Resource Set for channel measurement of the second CSI Reporting Setting and CSI-RS resources, or SS/PBCH Block resources of Resource Set given by resourcesForSetA-r19 of the first CSI Reporting Setting is provided by the higher layer parameter RSMappingtoSetA in the second CSI Reporting Setting; - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance; - determine RS-PAI as the total count of accurate reference signals prediction instance(s), and the UE shall report RS-PAI for the second Reporting Setting, with a -bit field (M is given by nroftransmissionOccasion-r19). <omitted texts>

Agreement

Adopt the following TP in section 5.2.1.6, TS 38.214.

Reason for change: The CPU occupation time of the monitoring report is unclear.

Summary of change: For UE-sided model, regarding a CSI report with CSI-ReportConfig for monitoring, Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report for all types of CSI reports (i.e., AP/SP/P CSI report).

Consequence if not approved: The CPU occupation time of the monitoring report is not aligned between the gNB and UE, leading to inefficient CSI report scheduling.

#### 5.2.1.6 CSI processing criteria #### <Unchanged part is omitted> For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including and/or , for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' or 'rs-pai-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report. #### <Unchanged part is omitted>

Agreement

Adopt the following TP for Clause 6.3.1.1.2 in TS 38.212.

Reason for change: For NW-sided model, for a CSI-ReportConfig with the number of reported RS is the same as the size of the corresponding resource set for channel measurement, the mapping between the reported RSRPs and the resources within the resource set is unclear.

Summary of change: Clarify the mapping between the reported RSRPs and the resources within the resource set when the number of reported RS of a CSI-ReportConfig is the same as the size of the corresponding resource set for channel measurement.

Consequence if not approved: The understanding of resources corresponding to reported RSRPs is not aligned between the gNB and UE.

Table 6.3.1.1.2-8G: Mapping order of CSI fields of one report for CRI/RSRP or SSBRI/RSRP reporting, if nrofReportedRS is configured CSI report number; CSI fields. CSI report #n; CRI or SSBRI #1 as in Table 6.3.1.1.2-6. CRI or SSBRI #2 as in Table 6.3.1.1.2-6, if reported. …. CRI or SSBRI # M as in Table 6.3.1.1.2-6, if reported. RSRP #1 as in Table 6.3.1.1.2-6. Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported. …. Differential RSRP # M as in Table 6.3.1.1.2-6, if reported. NOTE: The value of M is configured by the higher layer parameter nrofReportedRS. If M is equal to the number of SSB/CSI-RS resources in the corresponding resource set for channel measurement, Differential RSRP #m, m = 2, …, M, corresponds to the (m-1)-th SSB/CSI-RS resources among the remaining M-1 SSB/CSI-RS resources other than the resource corresponding to CRI/SSBRI#1 in the resource set..

Agreement

For one CSI resource setting for Set A for CSI report for inference for UE-side model

- Only a single resource set can be configured for the CSI resource setting.

Agreement

Support the following on the reporting condition of CSI reporting for inference

- For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

Agreement

Support the following on the reporting condition of CSI reporting for monitoring

- After the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for monitoring only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion for each of the RS resources in the resource set for monitoring no later than the corresponding CSI reference resource.

Agreement

For AP CSI report for inference, regarding the associated ID for aperiodic CSI resource setting with multiple resource sets for Set B

- Associated ID in CSI report configuration is for the resource set(s) for Set B

  - Note: This does not have impact on RAN1 specifications

Agreement

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting CSI report for inference for BM-Case2 is unclear.

Summary of change: Adding the condition of reporting CSI report for inference for BM-Case2.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting CSI report for inference for BM-Case2.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19' and with nroftimeinstance-r19 is configured, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least 𝐾BM latest consecutive transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. The value of 𝐾BM is indicated by UE capability. <omitted texts>

Agreement:

Adopt the following TP for TS 38.214 Clause 5.2.2.5.

Reason for change: The condition of reporting RS-PAI is unclear.

Summary of change: Adding the condition of reporting RS-PAI.

Consequences if not approved: gNB and UE may have different understanding on the condition of reporting RS-PAI.

5.2.2.5 CSI reference resource definition <omitted texts> For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'etypeII-Doppler-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only after receiving at least one aperiodic or 𝐾𝑝 consecutive periodic/semi-persistent CSI-RS transmission occasion(s) for each of the CSI-RS resources in each CSI-RS resource group in the corresponding CSI-RS resource set for channel measurement and at least one CSI-RS and/or CSI-IM resource transmission occasion for each of the CSI-RS and/or CSI-IM resources in the corresponding resource set for interference measurement no later than the CSI reference resource and within the same DRX active time, when DRX is configured, and drops the report otherwise. The value of 𝐾𝑝∈{1,2,4} is indicated by UE capability, as defined in clause 5.2.1.6. For a CSI-ReportConfig configured with the higher layer parameter reportQuantity set to 'rs-pai-r19', after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, the UE reports a CSI report only if receiving at least nroftransmissionOccasion-r19 latest transmission occasion(s) for each of the CSI-RS resources or SS/PBCH Block resources in the corresponding resource set for channel measurement no later than the CSI reference resource, and drops the report otherwise. <omitted texts>

Agreement:

Adopt the following TP in section 5.2.1.6, TS 38.214 in principle subject to the outcome of the discussion in Proposal below.

Proposal

- For TS38.214 Section 5.2.1.6, use the original notion of CPU (instead of CPU, 1) to represent the legacy CPU, and replace all the text of “CPU, 2” with “APU” to correctly represent AI/ML-based PU.

Reason for change: To capture the agreement to support two AI/ML PU pools.

Summary of change: Adding the description to support two AI/ML PU pools.

Consequences if not approved: two AI/ML PU pools are not supported.

#### 5.2.1.6 CSI processing criteria <Unchanged part is omitted> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,2x simultaneous CSI calculations it is said to have NCPU,2x CSI processing units for processing CSI reports. If L Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,2x-LLx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,2x-L Lx CPUs are unoccupied, where each CSI report corresponds to OCPU,2x(n), the UE is not required to update the N-M2x requested CSI reports with lowest priority (according to Clause 5.2.5), where is the largest value such that Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2,3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19] if a CSI report is not considered within any of M and M2x, the values for OCPU,1 and OCPU,2x are considered to be 0, for the procedure previously described in this clause and the UE is not required to update the CSI report. <Unchanged part is omitted> - for a CSI report with CSI-ReportConfig with reportQuantity-r19 set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', - if nroftimeinstance-r19 is not configured, OCPU,1=X1 and OCPU,2x=X2x, where the value of X1and X2x are reported by UE capability. - if nroftimeinstance-r19 is configured, OCPU,1=Y1 and OCPU,2x=Y2x, where the value of Y1and Y2x are reported by UE capability. - OCPU,1=X1 and OCPU,2x=X2x, for a CSI report configured with the higher layer parameter [RRC_name-r19], where the values of X1 and X2x are reported by UE capability. For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', or a CSI report with LTM-CSI-ReportConfig, or reportQuantity not set to 'none-bm-r19' or 'none-csi-r19', the CPU(s) (including OCPU,1and/or OCPU,2x, for CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19]) are occupied for a number of OFDM symbols as follows: <Unchanged part is omitted>

Agreement:

Support the configuration of following parameters in a CSI-ReportConfig with the higher layer parameter reportQuantity set to'none-bm-r19':

- TimeGap-r19, i.e., the time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of predicted time instances for BM-Case 2.

RAN1#122bis

Agreement

The following TP for TS 38.214 Clause 5.2.1.4.3b is endorsed.

Reason for change: The slot of transmission occasion for monitoring is unclear for the linkage of the corresponding CSI report for inference.

Summary of change: Clarify the first slot of transmission occasion for monitoring is used for the linkage.

Consequences if not approved: gNB and UE may have different understanding on the linkage.

##### TS 38.214 Clause 5.2.1.4.3b RS-PAI Reporting <omitted texts> - check a condition : - for the transmission occasion of the second CSI Reporting Setting, there is a linked report of the first CSI Reporting Setting. When nroftimeinstance-r19 is not configured in the first Reporting Setting, the linking is determined if CSI reference resource of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting. When nroftimeinstance-r19 is configured in the first Reporting Setting, timeinstanceformonitoring-r19 configured in the second CSI Reporting Setting indicates the timeinstanceformonitoring-r19-th time instance among nroftimeinstance-r19 time instance(s), and the linking is determined if the slot corresponding to the time instance indicated by timeinstanceformonitoring-r19 of a report of the first CSI Reporting Setting has a minimal slot offset, no larger than 64 slots, from the first slot of the transmission occasion of the corresponding Resource Set for channel measurement of the second CSI Reporting Setting; <omitted texts>

Agreement

The following TP for 38.214 Clause 6.2.1.3 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the prioritization rules between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the prioritization rules.

Consequences if not approved: the prioritization rules between a transmission of SRS and transmission of a physical signal/channel with P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1.3 UE sounding procedure between component carriers <omitted text> The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier c1 and transmission of a physical signal/channel over a carrier of a serving cell in set Sc2 - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI/P-CRI/P-SSBRI/RS-PAI and/or PRACH on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sc2 happen to overlap in the same symbol - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/P-L1-RSRP, and/or SRS transmission on a carrier of a serving cell in set Sc2 configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR/TDCP/cjtc-Dd/cjtc-F/cjtc-Dd-F/cjtc-P/P-L1-RSRP on a carrier of a serving cell in set Sc2 whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell c1 happen to overlap in the same symbol. <omitted text>

Agreement

The following TP for 38.214 Clause 6.2.1 is endorsed.

Reason for change: P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is missing in the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel.

Summary of change: Add P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP in the collision handling procedure.

Consequences if not approved: the collision handling procedure between a transmission of SRS and transmission of a physical signal/channel P-CRI/P-SSBRI/RS-PAI/P-L1-RSRP is unclear.

6.2.1 UE sounding procedure <omitted text> For PUCCH and SRS on the same carrier, a UE shall not transmit SRS when semi-persistent or periodic SRS is configured in the same symbol(s) with PUCCH carrying only CSI report(s), or only L1-RSRP report(s), or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s). A UE shall not transmit SRS when semi-persistent or periodic SRS is configured or aperiodic SRS is triggered to be transmitted in the same symbol(s) with PUCCH carrying HARQ-ACK, link recovery request (as defined in clause 9.2.4 of [6, 38.213]) and/or SR. In the case that SRS is not transmitted due to overlap with PUCCH, only the SRS symbol(s) that overlap with PUCCH symbol(s) are dropped. PUCCH shall not be transmitted when aperiodic SRS is triggered to be transmitted to overlap in the same symbol with PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s), or only P-CRI/P-SSBRI/P-L1-RSRP report(s), or only RS-PAI report(s) and the PUCCH starts no earlier than Tproc,2 after the last symbol of the PDCCH carrying the triggering command for the aperiodic SRS, where Tproc,2 is the PUSCH preparation time for the corresponding UE processing capability assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the triggering command and the SCS configuration of the PUCCH. <omitted text>

Agreement

Regarding the RAN2 LS from R1-2506715, for the UE-side candidate data collection configuration for BM-Case2, include the following IEs along with the already agreed IEs in RAN2:

- TimeGap-r19, i.e., the expected time gap between two consecutive predicted time instances and between the reference time and the earliest predicted time instance.

- nroftimeinstance-r19, i.e., number of expected predicted time instances.

Agreement

Regarding the RAN2 LS R1-2506720, the answer to the question in the LS is as below:

From RAN1 perspective, Option 2 is best option and can resolve the issue. - No additional RAN 1 impact was identified for Option 2.

Agreement

The following TP for TS 38.214 Clause 5.2.1.4.2 is endorsed.

Reason for change: The configuration restriction of AP RS resource set with no more than 16 resources is too restrictive for Set A for inference.

Summary of change: For Set A for inference, relax the configuration restriction of AP RS resource set with no more than 16 resources.

Consequences if not approved: The configuration of more than 16 resource within Set A for inference is not possible.

##### TS 38.214 Clause 5.2.1.4.2 Report quantity configurations <omitted texts> If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-LI-PMI-CQI', UE does not expect the CSI-ReportConfig to be configured with higher layer parameter codebookType set to 'typeII-r16' or 'typeII-PortSelection-r16', 'typeII-PortSelection-r17', 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', 'typeII-Doppler-PortSelection-r18', 'eTypeII-r19', 'TypeII-FePortSelection-r19' or 'typeII-Doppler-r19'. If the UE is configured with a CSI-ReportConfig with higher layer parameter reportQuantity set to 'cri-RSRP', 'cri-SINR', 'none', 'cri-RSRP- Index', 'cri-SINR- Index, 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', 'p-ssb-index-RSRP-r19', or ‘rs-pai-r19', and the CSI-ReportConfig is linked to a resource setting not configured by resourcesForSetA-r19 and configured with the higher layer parameter resourceType set to 'aperiodic', then the UE is not expected to be configured with more than 16 CSI-RS resources in a CSI-RS resource set contained within the resource setting. <omitted texts>

Agreement

The draft response LS in R1-2508121 is endorsed. Final LS in R1-2508122.

RAN1#123

Agreement

Adopt the following TPs for TS 38.214 Clause 5.2.1.6.

Reason for change: The release of CPU1 and CPUx for a CSI report not considered within any of M and Mx is unclear.

Summary of change: Clarify that the CPU1 and CPUx for a CSI report not considered within any of M and Mx are not occupied.

Consequences if not approved: gNB and UE may have different understanding on the release of CPU1 and CPUx for a CSI report not considered within any of M and Mx.

TP for TS 38.214 Clause 5.2.1.6 CSI processing criteria <omitted texts> For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, â€¦, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0â‰¤Mxâ‰¤N is the largest value such that n=0Mx-1OCPU,x(n)â‰¤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter the higher layer parameter csi-InferencePrediction-r19 corresponds to either x = 2 or x = 3 subject to UE capability. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter csi-InferencePrediction-r19, OCPU,1 and OCPU,x are considered for determining M and Mx, and if a CSI report is not considered within any of M and Mx, the values for the CPUs corresponding to OCPU,1 and OCPU,x of the CSI report are not occupied considered to be 0, for the procedure previously described in this clause, and the UE is not required to update the CSI report. <omitted texts>

Appendix B. Topic specific agreements

NW-side AI/ML model

Agreement (RAN1#116)

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement (RAN1#116bis)

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above "For NW-sided model, for BM-Case1 and BM-Case2" and "Set A" and "Set B", will not be specified in RAN 1 specifications

Agreement (RAN1#117)

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support

- Support differential L1-RSRP reporting with legacy quantization step and range

  - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP

  - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Agreement (RAN1#117)

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

- Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

Conclusion (RAN1#121)

- For NW sided model for L1-RSRP report in L1 signaling, legacy quantization steps and range are reused.

Agreement (RAN1#121)

For NW-sided model, for inference, when M<the size of measurement resource set, the beam information is CRI/SSBRI

Note: The purpose, such as above “For NW-sided model, for inference” will not be specified in RAN1 specifications.

#### Beam indication

Agreement (RAN1#116)

For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework

- FFS on whether/how potential enhancement is needed

UE-side AI/ML model

#### Data collection

Agreement (RAN1#120)

For UE-sided model, for configuring the resource for data collection purpose, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report.

  - One CSI-ResourceConfigId is configured for Set A.

  - One CSI-ResourceConfigId is configured for Set B.

  - Note: UE performs measurement on all resources

  - One or two associated IDs can be configured in CSI-ReportConfig

    - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

    - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: whether/how to support 'aperiodic' CSI RS

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Agreement (RAN1#120bis)

For UE-sided model, regarding the resource type for data collection purpose, only always-on SSB and P/SP CSI-RS are supported.

Agreement (RAN1#121)

For data collection for UE-sided model, in CSI-report configuration, reportQuantity is set to “none-BM-r19”

#### Inference (BM-Case1)

Model consistency (Associated ID)

Agreement (RAN1#116bis)

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt 2: Performance monitoring based

  - FFS details

- Other options are not precluded.

Agreement (RAN1#118)

For UE sided model in beam management, support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement (RAN1#120)

For UE-sided model, in CSI-ReportConfig for inference

- One or two associated IDs can be configured in CSI-ReportConfig

  - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured,

  - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B

- FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

Resource/Reporting Configuration

Conclusion (RAN1#116)

For UE sided model at least for inference, for measurement, the configuration of Set B,

- take the current CSI framework as the starting point

Agreement (RAN1#116bis)

For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

  - Other necessary configuration are not precluded.

Agreement (RAN1#118)

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement (RAN1#118bis)

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement (RAN1#118bis)

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement (RAN1#118bis)

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Conclusion (RAN1#119)

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement (RAN1#119)

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- two CSI-ResourceConfigId s are configured for Set A and Set B separately.

Report contents

Agreement (RAN1#116)

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

  - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

    - FFS on the quantization method of probability information

    - Probability information is the probability of the beam to be the Top 1 or Top K beam

  - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

    - FFS on definition of reported RSRP

    - FFS on the definition and quantization method of confidence information

  - Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement (RAN1#116bis)

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP.

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.

- Where the predicted RSRP is based on AI/ML output.

- Note: Support both Option A and Option B is not precluded.

Working Assumption (RAN1#116bis)

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement (RAN1#117)

Following Working Assumption is confirmed.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement (RAN1#119)

For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Agreement (RAN1#120)

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

Agreement (RAN1#120)

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

Agreement (RAN1#120bis)

For BM-Case1, for the Top K beam(s) report as the inference results

- K is configured in inference report configuration to the UE.

Agreement (RAN1#120bis)

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

CRI or SSBRI #1

CRI or SSBRI #2

…

CRI or SSBRI #K

RSRP #1

differential RSRP #2

…

differential RSRP #K

- CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K

- RSRP #1 is absolute predicted RSRP

- Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

Agreement (RAN1#121)

For UE-sided model, for BM-Case 1 and BM-Case 2, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information.

#### Inference (BM-Case2)

Agreement (RAN1#116bis)

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1.

  - Note: overhead reduction is not precluded.

- FFS on details

Agreement (RAN1#118)

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement (RAN1#118)

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

Agreement (RAN1#118bis)

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement (RAN1#120)

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances,

- the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Agreement (RAN1#120)

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement (RAN1#120bis)

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances, and also represents the time gap between the reference time and the first future time instance for prediction.

Agreement (RAN1#120bis)

For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case2, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

Time instance indicator

CRI or SSBRI #1 of time instance #1

CRI or SSBRI #2 of time instance #1

…

CRI or SSBRI #K of time instance #1

CRI or SSBRI #1 of time instance #2

CRI or SSBRI #2 of time instance #2

…

CRI or SSBRI #K of time instance #2

…

CRI or SSBRI #1 of time instance #N

CRI or SSBRI #2 of time instance #N

…

CRI or SSBRI #K of time instance #N

RSRP #1 of time instance #1

Differential RSRP #2 of time instance #1

…

Differential RSRP #K of time instance #1

Differential RSRP #1 of time instance #2

Differential RSRP #2 of time instance #2

…

Differential RSRP #K of time instance #2

…

Differential RSRP #1 of time instance #N

Differential RSRP #2 of time instance #N

…

Differential RSRP #K of time instance #N

  - Time instance indicator exist if N > 1

  - The size of CSI field for time instance indicator is

  - The value of time instance indicator n (n≥0) corresponds to the (n+1)-th earliest time instance in the N time instances

  - Time instance #1 corresponds to the time instance indicated by the time instance indicator

  - Time instance #2~#N are mapped to the remaining N-1 time instance(s) other than time instance #1 based on the time domain order of the time instances

    - where time instance #2 is mapped to the earliest time instance from the N-1 time instance(s)

  - CRI or SSBRI #k is mapped to RSRP #k with the same time instance, where k = 1,2,…,K

  - RSRP #1 of time instance #1 is absolute RSRP; and the remaining RSRP are differential RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1 of time instance #1

Conclusion (RAN1#120bis)

For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Agreement (RAN1#121)

For UE-sided model, for inference report for BM-Case2, a time instance for prediction is defined as a slot.

#### Performance monitoring

Agreement (RAN1#117)

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

Agreement (RAN1#118)

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement (RAN1#118bis)

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement (RAN1#118bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement (RAN1#119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Agreement (RAN1#120bis)

For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

Agreement (RAN1#120bis)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |
| P report | Support | Support | Support |

Agreement (RAN1#120bis)

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

Agreement (RAN1#120bis)

For UE-sided AI/ML model for beam management, for Option 2 (UE-assisted performance monitoring), the performance metric of Top 1 or Top K beam prediction accuracy is defined as:

- At least one of the Top M beam(s) of the resource set(s) for monitoring is among Top-K predicted beam(s) of Set A (e.g., linked to at least one of the Top-K predicted beam(s) of Set A based on certain rule or signalling)

  - Where K is the number of predicted beam(s) in the corresponding inference report per time instance

  - Where Top M beam(s) is the best M beam(s) based on L1-RSRP measurements of the resource set(s) for monitoring

  - M is configured by NW in CSI report configuration for monitoring

    - M= 1, 2

  - FFS: detailed rule or signalling

Agreement (RAN1#120bis)

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model:

  - Support the size of a set for monitoring is the same as the size of Set A,

    - The n-th resource in the set for monitoring is linked to the n-th resource in Set A.

  - Support the size of a set for monitoring is smaller than the size of Set A

Working Assumption (RAN1#120bis)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X for the minimal slot offset, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

Working Assumption (RAN1#120bis)

For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring

- wherein N (N>=1) is configured in CSI-ReportConfig

- FFS on additional rule for counting N linked pair

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Working Assumption (RAN1#120bis)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- FFS on whether to configure more than one time instance

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring

- N (N>=1) is configured in the CSI-ReportConfig

- FFS on additional rule for counting N linked pair

- measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

Agreement (RAN1#121)

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is (0 ≤≤ N)

- Where  is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is

Agreement (RAN1#121)

For calculation the performance metric of Type 1 Option 2 performance monitoring for UE-sided model, when the size of the set for monitoring is smaller than the size of Set A,

- support the mapping of the resources in the set for monitoring to resources in Set A is configured via RRC, support

  - A X-bit bitmap with Y non-zero bits is configured by the RRC in CSI Report Config for monitoring, where X is the size of Set A and Y is the size of the set for monitoring

  - The x-th MSB of the bitmap corresponds to x-th resource in Set A

  - The y-th nonzero bit of the bitmap corresponds to the y-th entry of associated nzp-CSI-RS-Resources or csi-SSB-ResourceList in the set for monitoring, 1≤y≤Y

Agreement (RAN1#121)

- At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for calculation of metric for monitoring,

- for BM-Case 1, the measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with an inference report, where the CSI reference resource of the corresponding inference report has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference report, and the transmission occasion of the CSI-RS/SSB resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - Predefines a threshold X = 64 for the minimal slot offset, which is configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked inference report.

- Note: CSI reference resource corresponding to the CSI report for monitoring and inference is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP, considering discussing associated timeline separately

- The associated working assumption made in RAN1#120b will not be confirmed.

Agreement (RAN1#121)

For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring,

- only one resource set is configured in the CSI-ReportConfig

- the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring

- the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource, no later than CSI reference resource corresponding to the CSI report for monitoring

- The measurement result of nth (n = 1,..,N) latest transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the nth transmission occasion of the CSI-RS/SSB resources for monitoring.

  - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, are no later than the CSI reference resource corresponding to the CSI report for monitoring

  - Predefines a threshold X =64, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance.

- Note: CSI reference resource corresponding to the CSI report for monitoring is determined based on legacy for all types (P/SP/AP) of CSI report carrying L1-RSRP

The associated working assumption made in RAN1#120b will not be confirmed.

Agreement (RAN1#121)

For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

#### CPU/APU

CSI report for data collection

Agreement (RAN1#121)

For UE-sided model, regarding a CSI-ReportConfig for data collection,

- Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured

CSI report for inference

Agreement (RAN1#118bis)

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2.

Agreement (RAN1#120bis)

- For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection:

  - Option 1: only dedicated AI/ML PU is occupied,  is reported by UE.

    - And

  - Option 2: only legacy CPU is occupied,  it is reported by UE.

  - Option 3: both dedicated AI/ML PU and legacy CPU are occupied,  is reported by UE.

    - And

Note: The supported option by UE is reported by UE capability, if multiple options are supported.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

  - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement (RAN1#121)

For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference,

- For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

  - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively

  - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively

  - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature.

  - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

  - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1,

  - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report

  - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report

  - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report)

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU.

- If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused

- If the CSI report is semi-persistent or periodic,

  - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report.

CSI report for monitoring

Agreement (RAN1#120bis)

For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, .

Note: the occupation duration is a separate discussion.

#### Z/Z’ timeline

Agreement (RAN1#121)

For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively

- Detailed values of d and d’ can be further discussed in UE feature.

#### CSI priority value

Agreement (RAN1#121)

For the determination of CSI report priority value of a CSI report for inference, the existing  is reused

- k = 0 for the CSI report for inference

For the determination of CSI report priority value of a CSI report for monitoring, the existing  is reused

- k = 0 for the CSI report for monitoring

RAN2 LS related

Agreement (RAN1#118bis)

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement (RAN1#118bis)

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement (RAN1#118bis)

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“ for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing.

Agreement (RAN1#118bis)

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Agreement (RAN1#119)

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Conclusion (RAN1#119)

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Agreement (RAN1#119)

Send LS to RAN2 with below information.

RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

RAN 1 made the following agreements related to the Questions from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848:

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Q3: Is NW-side additional condition functionality specific?

Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2.

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

Answer to Q4: And please refer to the agreements related to the Questions from RAN 2.

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2.

Q5: What is the content of applicable functionality reporting in Step 4?

Answer to Q5: Please refer to the agreements related to the Questions from RAN 2.

Q6: What is the content of inference configuration in Step 5?

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

Answer to Q7: Please refer to the agreements related to the Questions from RAN 2.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2.

Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

Answer to Q9: Please refer to the agreements related to the Questions from RAN 2.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

Answer to Q10:  Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

Final reply LS is approved in R1-2410898.

Agreement (RAN1#121)

For option B of applicability check, RAN 1 assumes that at least the following RRC parameters are to be reused:

- For both BM-Case 1 and BM-Case 2:

  - associatedIDforSetA-r19, resourcesForSetA-r19, resourcesForChannelMeasurement, associatedIDforSetB-r19, reportQuantity-r19, reportConfigType, nrofreportedpredictedrs-r19

- For BM-Case 2:

  - TimeGap-r19, nroftimeinstance-r19,

- Note: this doesn’t imply the associated ID is always present