3GPP TSG RAN WG1 #122bis			R1-250XXXX

Prague, Czech, Oct 13th – 17th, 2025

Agenda Item:	10.1

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session notes for AI 10.1 (AI/ML for NR Air Interface Enhancements)

Document for:	Discussion, Decision

## Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements

Please refer to RP-252445 for detailed scope of the WI.

[122bis-R20-AI/ML] Email discussion on Rel-20 AI/ML – Chenxi (Qualcomm)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### CSI spatial/frequency compression without temporal aspects (“Case 0”)

#### Inference related aspects

Including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports.

R1-2506743	Discussion on inference related aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2506774	Discussion on inference related aspects of CSI compression	ZTE Corporation, Sanechips

R1-2506781	Discussion on interference related aspects for CSI compression	TCL

R1-2506792	AI/ML CSI Spatial/Frequency Compression: Inference Aspects	InterDigital, Inc.

R1-2506803	Discussion on AIML for CSI compression inference related aspects	Spreadtrum, UNISOC, BUPT

R1-2506832	Inference related aspects of AI/ML for CSI compression	Ericsson

R1-2506887	Discussion on inference related aspects for CSI compression	vivo

R1-2506931	Discussion on Inference related aspects for CSI compression	Huawei, HiSilicon

R1-2506978	Discussion on inference related aspects of two-sided AI/ML model based CSI feedback	Xiaomi

R1-2507006	Discussion on inference related aspects of CSI compression	CMCC

R1-2507108	Discussion on inference related aspects of AI/ML-based CSI compression	CATT

R1-2507165	Inference related aspects for AI/ML CSI compression	OPPO

R1-2507183	Discussion on inference related aspects for CSI compression	KT Corp.

R1-2507205	Inference related aspects for CSI compression	HONOR

R1-2507242	Views on inference related aspects of CSI compression	Samsung

R1-2507280	Discussion on inference related aspects in CSI compression with AI/ML	Fujitsu

R1-2507301	Discussion on inference aspects of CSI compression	NEC

R1-2507389	CSI Compression: Inference Related Aspects	Nokia

R1-2507396	Discussion on inference related aspects for CSI compression	LG Electronics

R1-2507415	Discussion on inference aspects for AI/ML-based CSI compression	Panasonic

R1-2507443	Inference related aspects for CSI compression	Lenovo

R1-2507462	Discussion on Inference related aspects for AI CSI compression	Ofinno

R1-2507496	Discussion on inference related aspects of CSI compression	ETRI

R1-2507517	Inference for AI/ML based CSI Compression	Google

R1-2507552	Discussion on Inference Related Aspects for CSI Compression	Sharp

R1-2507616	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Inference related aspects			MediaTek Inc.

R1-2507630	Discussion on inference related aspects of CSI compression	Transsion Holdings

R1-2507663	On inference related aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2507710	Specification of inference aspects of AIML CSI compression	Qualcomm Incorporated

R1-2507733	Inference Related Aspects of AI/ML for CSI Compression	TOYOTA Info Technology Center

R1-2507802	Discussion on the inference-related aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2507897	Discussion on AI/ML for CSI compression inference related aspects	CEWiT

R1-2508021

Agreement:

Support precoding matrix with port-subband domain representation as target CSI type.

- For a certain layer l, a precoding matrix on N_sb subband and P CSI-RS ports is Winl=[Win,0l, Win,1l,⋯,Win, Nsb-1l] with Win,kl being a P×1 vector.

- Note: Only sub-band PMI reporting is supported.

Agreement:

For determining the payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and ranks, consider following general principles for precoding matrix-based target CSI

- Rank-common and layer-common model with payload-scalable design is considered as the baseline implementation for discussion purpose.

  - Note: layer-specific / rank-specific (if supported) payload parameters are supported by this rank-common and layer-common model with payload-scalable design.

- Limiting the total number of (per-layer) payload parameters combinations dl,ν,Ql,ν or dl,ν,L,Ql,ν  across all layers and all ranks.

- At least for medium to high per-layer payload regime, the total payload size of rank 2, rank 3 and 4 are comparable to each other.

- Spectral efficiency or system throughput and trade-off with feedback overhead are considered as the performance metric.

R1-2508022

Agreement:

When the Target CSI is precoder matrix, in order to down-select to N per-layer payload parameter combinations (PC) of dl,ν,Ql,ν or dl,ν, L,Ql,ν based on performance, overhead and complexity trade-off results, consider the following candidate values:

- Candiate values for dl,ν: 32, 64, 96, 128, 192

- Candidate values for Ql,ν: 1, 2, 3, 4, (8, 10 only for VQ)

  - 2 is considered as the baseline assumption for SQ.

- Candidate values for L: L=2, 4,  dl,ν

- Candidate values for N: 4, 5, 6

- Note: These N payload PCs of dl,ν,Ql,ν or dl,ν, L,Ql,ν are considered as the pool for the payload PC for each layer and each rank.

- Note: The above is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: Rank-common and layer-common model with payload scalable design is considered.

- Note: The above does not mean support of both SQ and VQ.

- Note: The number of parameters combinations configurations (Y) does not exceed 8.

Agreement:

For payload parameter combinations (PC), consider followings

- Support Y configurable PCs (across layers and ranks) as below, where X_n, Xn_31, Xn_32, Xn_33, Xn_41, Xn_42, Xn_43, Xn_44 represent a pair of dl,ν,Ql,ν or a triplet of dl,ν, L,Ql,ν  chosen from the per-layer PC pool.

|  | Rank1 | Rank2 | Rank3 | Rank4 |
| --- | --- | --- | --- | --- |
| PC1 | X1 | X1, X1 | X1_31, X1_32, X1_33 | X1_41, X1_42, X1_43, X1_44 |
| PC2 | X2 | X2, X2 | X2_31, X2_32, X2_33 | X2_41, X2_42, X2_43, X2_44 |
| … |  |  |  |  |

- For rank 1 and rank 2, layer-common and rank-common payload is supported.

- For rank 3, consider alternatives of {Xn_31, Xn_32, Xn_33} = {a, a, a}, {a, a, b}, {a, b, b} for potential down selection.

- For rank 4, consider alternatives of {Xn_41, Xn_42, Xn_43, Xn_44} = {a, a, a, a}, {a, a, b, b}, {a, b, b, c}, {a, a, b, c} or {a, b, c, c} for potential down selection.

- Note: The table is for discussion purpose, how to configure the PCs (across layers and ranks) is a separate discussion.

- Note: a, b values in rank3 can be different from that in rank 4.

R1-2508023

Agreement:

Consider following for evaluation:

- For down-selection of per-layer pairs of dl,ν,Ql,ν or a triplets of dl,ν, L,Ql,ν , average SGCS across per layer can be considered as performance metric.

- For down-selection of the rank-specific and layer-specific patterns (e.g., {Xn_31, Xn_32, Xn_33} and {Xn_41, Xn_42, Xn_43, Xn_44}), spectral efficiency or throughput should be considered.

Agreement:

For CSI feedback via two-sided AIML models, reuse legacy CSI reporting framework

- Support CSI report based on periodic, semi-persistent and aperiodic CSI-RS resources with legacy measurement resource configurations.

- Support aperiodic report and semi-persistent CSI report on PUSCH and reuse legacy triggering mechanism.

#### Other aspects

Including NW and UE data collection for training, performance monitoring, as well as model pairing related issues.

R1-2506744	Discussion on other aspects for CSI spatial/frequency compression without temporal aspects (“Case 0”)	FUTUREWEI

R1-2506775	Discussion on other aspects of CSI compression	ZTE Corporation, Sanechips

R1-2506782	Discussion on other aspects for CSI compression	TCL

R1-2506793	AI/ML CSI Spatial/Frequency Compression: Other Aspects	InterDigital, Inc.

R1-2506804	Discussion on AIML for CSI compression other aspects	Spreadtrum, UNISOC

R1-2506833	Other aspects of AI/ML for CSI compression	Ericsson

R1-2506888	Discussion on other aspects for CSI compression	vivo

R1-2506932	Discussion on other aspects for CSI compression	Huawei, HiSilicon

R1-2506979	Discussion on other aspects of CSI spatial/frequency compression	Xiaomi

R1-2507007	Discussion on other aspects of CSI compression	CMCC

R1-2507109	Discussion on other aspects of AI/ML-based CSI compression	CATT

R1-2507166	Other aspects for AI/ML CSI compression	OPPO

R1-2507184	Discussion on other aspects of CSI compression	KT Corp.

R1-2507206	Other aspects for CSI compression	HONOR

R1-2507243	Views on other aspects of CSI compression	Samsung

R1-2507281	Discussion on other aspects of CSI compression	Fujitsu

R1-2507303	Discussion on other aspects of CSI compression	NEC

R1-2507390	CSI Compression: Other Aspects	Nokia

R1-2507397	Discussion on other aspects for CSI compression	LG Electronics

R1-2507416	Discussion on other aspects for AI/ML-based CSI compression	Panasonic

R1-2507444	Other aspects for CSI compression	Lenovo

R1-2507478	Other aspects	Tejas Network Limited

R1-2507497	Discussion on other aspects of CSI compression	ETRI

R1-2507518	Other Aspects for AI/ML based CSI Compression	Google

R1-2507587	Discussion on other aspects for CSI compression	Sony

R1-2507617	CSI spatial/frequency compression without temporal aspects (“Case 0”)- Other aspects	MediaTek Inc.

R1-2507664	On other aspects for AI based CSI spatial/frequency domain compression	Apple

R1-2507711	Specification of other aspects of AIML CSI compression	Qualcomm Incorporated

R1-2507775	Discussions on other aspects for NR air interface enhancements	Sharp

R1-2507778	Discussion on performance monitoring for CSI compression	Shanghai Jiao Tong University, Toyota

R1-2507803	Discussion on other aspects of AI/ML CSI compression	NTT DOCOMO, INC.

R1-2507868	Discussion on AIML based CSI compression	ASUSTeK

R1-2508028

Agreement:

For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation:

- Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2.

  - Note: Companies to report whether  real value and imaginary value are quantized, or amplitude value and phase value are quantized.

- Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed.

  - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts.

- Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values.

- Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation.

R1-2508030

Agreement:

For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection.

- L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits

  - Company to report the quantization step for reference amplitude for 6bits.

- No restriction to the total number of non-zero coefficients for a specific rank.

- Other combinations can be considered and reported by companies.

Agreement:

For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered:

- Benchmark#1: FP32 (k1=k2=32 bits)

- Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2)

- CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer.

- Tx port: 32, 64, or 128 up to companies.

- Subband number: 13, or 19 up to companies.

- Performance metric:

  - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option.

  - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution

  - Overhead per sample and per layer SGCS results are reported per layer

- Note: Complexity should be analysed.

- For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32

- Other assumptions follow R18 study EVM

R1-2508031

Agreement

For CPU counting of UE side data collection,

- OCPU=1

- FFS: The CPU occupancy duration

### Inter-vendor training collaboration for two-sided AI/ML models

Including specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”), as well as RAN4-triggered issues.

R1-2506745	Discussion on inter-vendor training collaboration for two-sided AI/ML models	FUTUREWEI

R1-2506776	Discussion on inter-vendor training collaboration for two-sided AI/ML models	ZTE Corporation, Sanechips

R1-2506783	Discussion on inter-vendor collaboration for CSI compression	TCL

R1-2506794	AI/ML CSI Spatial/Frequency Compression: Inter-vendor Collaboration	InterDigital, Inc.

R1-2506805	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Spreadtrum, UNISOC

R1-2506834	Inter-vendor training collaboration for two-sided AI/ML models	Ericsson

R1-2506889	Discussion on inter-vendor training collaboration for two-sided AI/ML models	vivo

R1-2506933	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	Huawei, HiSilicon

R1-2506980	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Xiaomi

R1-2507008	Discussion on inter-vendor training collaboration for CSI compression	CMCC

R1-2507110	Inter-vendor training collaboration for CSI compression	CATT

R1-2507167	Inter-vendor training collaboration for AI/ML CSI compression	OPPO

R1-2507244	Views on inter-vendor training collaboration for two-sided AI/ML models	Samsung

R1-2507282	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Fujitsu

R1-2507324	Discussion on Inter-vendor training collaboration for two-sided AI/ML models	China Telecom

R1-2507391	Inter-vendor training collaboration for two-sided AI/ML models	Nokia

R1-2507398	Discussion on Inter-vendor training collaboration for two-sided models	LG Electronics

R1-2507414	Discussion on Inter-vendor Training Collaboration for two-sided AI/ML models	NEC

R1-2507417	Discussion on inter-vendor training collaboration for two-sided AI/ML models	Panasonic

R1-2507487	Inter-vendor training collaboration for two-sided AI/ML models	Lenovo

R1-2507498	Discussion on inter vendor training collaboration for two-sided AI/ML models	ETRI

R1-2507519	Inter-Vendor Collaboration for AI/ML based CSI Compression	Google

R1-2507553	Discussions on Inter-vendor training collaboration	Sharp

R1-2507588	Inter-vendor training collaboration for two-sided AI/ML models	Sony

R1-2507618	Inter-vendor training collaboration for two-sided AI/ML models	MediaTek Inc.

R1-2507665	Discussion on inter-vendor training collaboration for two sided AI/ML models	Apple

R1-2507666	FL summary # 1 for inter-vendor training collaboration	Moderator (Apple)

R1-2507667	FL summary # 2 for inter-vendor training collaboration	Moderator (Apple)

R1-2507668	FL summary # 3 for inter-vendor training collaboration	Moderator (Apple)

R1-2507712	Inter-vendor training collaboration for two-sided CSI compression use case	Qualcomm Incorporated

R1-2507732	Discussion on inter-vendor training collaboration for two-sided models	Indian Institute of Tech (M)

R1-2507764	Discussion on inter-vendor training collaboration	Continental Automotive

R1-2507804	Discussion on the inter-vendor training collaborations for two-sided AI/ML models	NTT DOCOMO, INC.

R1-2507828	Views on inter-vendor training collaboration for two-sided AI/ML models	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application

R1-2507666

Conclusion:

For Option 4-1 under Direction A in AI/ML based CSI compression, there is no consensus to further study the necessity of sharing assisted information to align model design aspects.

Agreement:

- For Option 4-1 under Direction A in AI/ML based CSI compression, both average SGCS and average NMSE as performance target in the exchanged dataset are supported. Both average SGCS and average NMSE are provided.

  - NW side can optionally send NMSE and/or SGCS values at X-percentiles.

R1-2507668

Agreement:

For Option 4-1 under Direction A in AI/ML based CSI compression, for the association between target CSI and CSI feedback in the exchanged dataset.

- Support 1:M mapping between one target CSI sample/set of samples to M CSI feedback samples/sets of samples associated with different CSI payload size configurations

- FFS: Association between Target CSI and CSI feedback, including scalability related information for different number of Tx port, sub bands related information.

- FFS: Whether/how Target CSI samples/set of samples within an exchanged dataset, with different Tx port and sub-band related information are organized.

- FFS: Whether the Target CSI is per layer or per rank when Target CSI is precoder matrix.

- FFS: Whether and how to support CSI feedback structure with different payload scalability options

- FFS: Other details