3GPP TSG RAN WG1 #122bis							               R1-2508004

Prague, Czech, October 13th – 17th, 2025

Agenda item:	        11.6

Source: 	Samsung (Moderator)

Title:	Moderator summary #3 on AI/ML for 6GR

Document for: 	Decision

# Introduction

In RAN 1 #122, the following agreement was made for use case identification:

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., no collaboration/interaction UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

This paper summarized the proposed use cases for 6GR study on AI/ML.

# Use cases

## CSI prediction and CSI-RS overhead reduction

#### Proposed observation 2.1 v1:

For 6GR AI/ML use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

- [6 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 source] assumed differently. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [4 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/band/frequency block with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/training type) and initial analysis can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- Besides, one source provided preliminary simulation results and analysis on, Tokenized CSI prediction (Huawei), and time domain CSI prediction combining CSI-RS and DMRS measurements (MediaTek).

Note: whether/how to capture the observation in the TR is a separate discussion.

Table A

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML |
| --- | --- |
| Reported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12 |
| Model input (for decoder for 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 2. Received RS signal7,8,9 3. Reported CSI for NW-sided model3,4,5 |
| Model output (for decoder for 2-sided model, when applicable) | 1. Full channel matrix (majority) 2. Eigenvector 3 for NW-sided model 3. Channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density(majority) 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Training types | Offline training(majority) Online finetuning for UE-sided model (for NW-sided model + UE sided model without training collaboration)4 |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS |
| Model location for inference | UE-sided model, NW-sided model2,3, 4,5,6, Two-sided model3, NW-sided model + UE-sided model without training collaboration4, |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR As two-sided model for CSI compression4 in NR |
| Potential spec impact | 1.Sparse CSI-RS design and corresponding feedback (especially for NW-sided model) 2. Signalling/ procedure related to LCM including the support of online finetuning, when applicable 3. Inter-vendor collaboration for two-sided model, when applicable |

Table B

| Sub-use case | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams |
| --- | --- | --- | --- |
| Reported Companies | (6) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1 | (4) Samsung, Apple, LGE, DoCoMo1 | (2) Samsung, vivo1 |
| Model input | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with 20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 | Channel matrix of carrier/band/frequency block A | Channel matrix of Set B of beams |
| Model output | 1. Channel matrix of future instances 2. Interference in future instances1 | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams |
| Label | Measurement in future time occasions. | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams |
| Training types assumption | offline training | offline training | offline training |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS, NMSE, throughput, ratio of CSI-RS overhead |
| Benchmark |  | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams |
| Model location for inference | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 Two-sided model1 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR in NR As NW-sided model in NR1 | As UE-sided model in NR in NR | As UE-sided model in NR in NR |
| Potential spec impact | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams 2. signalling/ procedure related to LCM |

#### Proposed observation 2.1 v2:

For 6GR AI/ML use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead/no CSI-RS with AI/ML.

- [6 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 source] assumed differently.

- Besides, [2 sources (Samsung, vivo)] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. One source provided preliminary simulation results and analysis on, Tokenized CSI prediction (Huawei), and time domain CSI prediction combining CSI-RS and DMRS measurements (MediaTek).

- Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table A

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead/no CSI-RS with AI/ML | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) |
| --- | --- | --- |
| Reported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung13, Fujitsu, Apple14, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12 | (6) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1 |
| Model input (for decoder for 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 2. Received RS signal7,8,9 3. Reported CSI for NW-sided model3,4,5 4. Channel matrix of carrier/band/frequency block A10,11,13,14 | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with 20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 |
| Model output (for decoder for 2-sided model, when applicable) | 1. Full channel matrix (majority) 2. Eigenvector 3 for NW-sided model 3. Channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 4. Channel matrix of carrier/band/frequency block B10,11,13,14 | 1. Channel matrix of future instances 2. Interference in future instances1 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density(majority) 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 4. Channel matrix of carrier/band/frequency block B10,11,13,14 | Measurement in future time occasions. |
| Training types | Offline training(majority) Online finetuning for UE-sided model (for NW-sided model+UE sided model without training collaboration)4 | offline training |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS 3. Ground truth of target frequency block or sample and hold from the measurement of carrier/band/frequency A10,11,13,14 |  |
| Model location for inference | UE-sided model, NW-sided model2,3, 4,5,6, Two-sided model3 NW-sided model + UE-sided model without training collaboration4, | UE-sided model NW-sided model1 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR As two-sided model for CSI compression4 in NR | As UE-sided model in NR in NR As NW-sided model in NR1 |
| Potential spec impact | 1.Sparse CSI-RS design and corresponding feedback (especially for NW-sided model) 2. Signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable 4. Cross carrier/frequency switching procedure enhancement based on predicted CSI10,11,13,14 | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Intention is to agree on the above as observation. In addition, I suggest to delete the part marked as grey, because no evaluation results provided. Please share your view and comment |
| NTT DOCOMO |  | The boundary of Sub case E and Sub case A is not clear to us. Both of them are considering the spatial domain CSI prediction. Our suggestion is to merge these two sub-cases. |
| QC |  | In Table A, please add Model input: CSI information (e.g., preferred CSI-RS ports subset) = > FL: unclear model input Labe: Ideal CSI information (e.g., preferred CSI-RS ports subset) => FL: unclear model output KPI: Accuracy of predicted CSI information In Table B, sub-Case B, please add Model input: Interference measurements via IMR and DMRS. Decoding information. Model output: Predicted CSI (e.g., RI, CQI) for future time occasions Label: Measurement of channel/interference and CSI (e.g., RI, CQI) in the future time occasions Not clear what “the part marked as grey” is referring to. =>FL: I think I already deleted it.(hopefully) |
| Huawei, HiSilicon |  | 1, “CSI-RS overhead reduction” – whether this benefit can be achieved or not, is questionable, since there is anyway non-AI/ML UE existing; it is not likely to reduce the cell-specific CSI-RS just due to the existing of AI/ML UEs. On the other hand, if the model is NW-side, it is possible to achieve cell specific OH reduction (if UE feedback is mandatory); even though CSI-RS OH reduction cannot be achieved, it can reduce UE CSI complexity and UL overhead. Therefore, the key word is not “overhead reduction”, but “CSI prediction” (or CSI reconstruction). [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML. - [23 sources] provided preliminary simulation results and analysis on sparse CSI-RS overhead reduction in frequency and/or spatial domain with AI/ML CSI prediction. Details can be found in Table A. 2, Huawei’s LT-CI aided CSI prediction can be merged into Table A, CSI-RS overhead reduction in frequency and/or spatial domain with AI/ML. Besides estimated channel matrix over sparse CSI-RS as model input, we also adopt additional long-term multi-path information as assistance information. In addition, we also consider NW-side model in evaluation. For Training types of Table A, we also adopt online fine-tuning in evaluation. Model input: 1. Estimated channel matrix over sparse CSI-RS 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 2. Received RS signal7,8,9 3 Eigenvector2,3 4 Reported CSI for NW-sided model4, 5,10 Training types assumption: Offline training, online finetuning4 =>FL: this is up to implementation. And this is why I highlighted “assumption” Model location for inference: UE-sided model, NW-sided model2,3,4,5,6,10, Two-sided model3,4 3. Regarding potential spec impact, new CSI type like explicit channel matrix is added. Potential spec impact: 1.Sparse CSI-RS design 2. signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable 4. Feedback of new CSI type to NW for NW side model |
| Xiaomi |  | We share same comments as DOCOMO that Sub case E can be merged to Sub case A. |
| Fujitsu |  | Similar question as NTT DoCoMo and Xiaomi regarding sub case E and sub case A. |
| CATT, CICTCI | Y | Understand the confusion from DOCOMO and other companies. Maybe better to Alt 1 (keep the purpose of NES): change the title of sub use case E into antenna port (so-called Type 1 spatial domain NES) or pattern on/off (so-called type 2 spatial domain NES). For type 2, the # of ports between model input and output are still the same, but the associated antenna elements are different due to the shut-down of some antenna elements. Alt 2 is to totally merge sub case E (both port and pattern on/off) into sub use case A. In this case, the following change is needed: Observation: - [23 sources] provided preliminary simulation results and analysis on CSI-RS overhead reduction prediction in frequency and/or spatial domain with AI/ML. Details can be found in Table A. Name: Sub-Case A: sparse CSI-RS prediction in frequency and/or spatial domain with AI/ML Label: 1. Estimated/ideal channel matrix based on full CSI-RS density 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns (not port on/off patterns). PS: We provide simulation results for both cases. (2) We believe an ID (e.g. associated ID) to implicit show the additional condition at NW sided is needed for UE-sided model. Is it considered as part of ‘LCM’? (3) BTW, is ‘the part marked as grey’ in FL comment means the part in excel file? |
| SPRD |  | We believe that sub use case E is specific to the NES scenario. If this is the case, we suggest clarifying it. For example, Sub-Case E: On/off of antenna port for NES. If not, we suggest merging it with sub-case A as a case for spatial domain overhead reduction. |
| ZTE |  | Regarding the two-sided model, based on our understanding on companies’ contribution, it seems there are at least the following different understandings: - Joint CSI-RS pattern design and CSI prediction; - Joint CSI prediction and CSI compression These two-sided model can be separated from the table A. Regarding the label of sub-Case B, it seems this use case is exactly the same as 5G-A CSI prediction, but the label is different. More clarification is needed, or we can directly reuse what we have in 5G-A. |
| Ofinno |  | Regarding potential specification impact, AI-enabled RS subset activation needs to be considered, since RRC and MAC/PHY layers require new IEs or indicators to signal which subset of ports is active and whether AI inference is applied for full-port channel estimation/prediction based on partial-port CSI-RS. On training type, online training (at UE-side for re-training the AI model based on the performance monitoring) can be considered as similar to HW. |
| Nokia |  | First, we suggest focusing only on Table A for now. It is clear that Table A contains the majority supported direction. #### Proposed observation 2.1: [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML. - [23 sources] provided preliminary simulation results and analysis on CSI-RS overhead reduction in frequency and/or spatial domain with AI/ML. Details can be found in Table A. Under Table A, - It is good to simplify the model input and output discussions without listing lot of variants. Suggest focusing on the main option. E.g., Model input : received signal on sparse CSI-RS. Model output : full size channel matrix - Same applies to Label. Please focus on the main option. - It is not important to mention results in the Table. - Model location for inference : this shall be mainly a UE-sided model. That is the majority direction. - Collaboration : UE-side model from NR is the main assumption. Spec impacts : High-level suggestion from FL is sufficient for now. |
| MediaTek |  | We support sub-case A and sub- case B. Are the listed companies only those providing evaluation results? We have provided evaluation results for sub-case B. Also, which is the part marked in grey? |
| CMCC |  | For Sub-Case E, there can be two different types when considering combination with AI and Rel-18 NES CSI. If it is CSI prediction for NES Type 1 CSI (TXRU on/off), we think it can be merged to Sub-Case A while the model output is CSI in another sub-configuration with different port pattern, not CSI in full ports. If it is CSI prediction for NES Type 2 CSI (antenna element on/off), the number of CSI ports of model input and output are same, but the associated antenna element pattern of CSI is different, then it can be also merged into Sub-Case A. Model input: 1. Estimated channel matrix over sparse CSI-RS 2. Received RS signal7,8,9 3 Eigenvector2,3 4 Reported CSI for NW-sided model5,10 Model output: 1. Estimated channel matrix including different CSI-RS ports from model input 2. Received RS signal7,8,9 |
| LGE |  | In Table A, the UE-side model is considered more suitable for inference in sub-case A, and the corresponding simulation results are presented. Please see above with our edits for “Model input” and “Model location for inference”: Model input: 4. Reported CSI for NW-sided model5,10 Model location for inference: NW-sided model2,3,5,6,10 In Table A, please add Model output: Sparse Channel matrix of inactive port/frequency region Label: Estimated/ideal sparse channel matrix based on inactive CSI-RS density - FL: no need to go such level of details It is not clear whether the eigenvector used as the model input is derived from the full CSI matrix or from a sparse CSI matrix. In addition, it should be explicitly stated that the PMI at the model output is derived from the full-dimension channel matrix. =>FL: 1 is for explicit channel, 3 is eigenvectors In Table B, regarding the sub-case B, since we provided the evaluation results on cross-frequency band/range CSI prediction, we don’t support to mark the sub-case B as grey and please add out company (LGE) to ‘supported company’ Also, we are not clear what ‘frequency block’ is meant, so we prefer to change as following: - Sub-use case: CSI prediction cross carrier/frequency block CSI prediction cross carrier/frequency-band - Model input: Channel matrix of frequency block A Channel matrix/CSI/measurement of carrier/frequency-band A - Model output: Channel matrix of frequency block B Predicted Channel matrix/CSI/measurement of carrier/frequency-band B - Label: Channel matrix of frequency block B Ideal Channel matrix/CSI/measurement of carrier/frequency-band B For the Sub-Case E, we support to study CSI prediction with different antenna on-off patterns for both current and future time instances. |
| TCL |  | Agree |
| IIT Kanpur |  | We are generally fine with the observation. For Table A, we also share QC’s view in adding the CSI information related to preferred CSI-RS ports subset to model input and label sections. |
| NEC |  | Studying all sub use cases seems too much work load. We support to have a limited number of use cases, e.g., sub use A. |
| Vivo |  | Three comments; - Support QC’s comment to add the following sub-case into Table B subcase B: - Model input: Interference measurements via IMR and/or DMRS. - Model output: Predicted CSI (e.g., RI, CQI) for future time occasions - Label: Measurement of channel/interference and CSI (e.g., RI, CQI) in the future time occasions - Model location: UE sided or NW sided. - =>let me check whether both vivo and Qc submitted results further - For Table B Sub-Case D, CSI interpolation across analog beams can also be NW sided model. And for interaction between UE and gNB, similar mechanisms can be applied for both one sided model and two sided model. Thus the model location should be revised as following; - Model location for inference: UE sided model, NW sided model, two sided model - Collaboraiton/interaction between UE and NW: As CSI prediction for UE-sided model in NR, As two-sided model for CSI compression4 in NR - FL: let me check whether evaluation results provided - For Table B sub-case E, we are also fine to merge it into sub-case A. If not, we would also to make similar revision as sub-case D for both UE sided model, NW sided model and two sided model. |
| OPPO |  | Regarding the title of the use case, it seems that CSI-RS overhead reduction is more generic than CSI prediction and reflects its original purpose, otherwise we are somehow confused that we are trying to save CSI-RS OH or predicting CSI. Considering the fact that Table A is with majority support, CSI-RS OH reduction in spatial and/or frequency domain can be with higher priority. |
| Ericsson |  | Is “the part marked as grey” referring to the greyed columns in the excel document? In Table A, sub-case A includes several further sub-use cases. For instance, CSI-RS overhead reduction in frequency and/or spatial domain with AI/ML includes the following further sub-cases: i) CSI-RS overhead reduction in spatial domain ii) CSI-RS overhead reduction in frequency domain iii) CSI-RS overhead reduction in spatial and frequency domains. Furthermore, the table includes UE-sided, NW-sided, and two-sided model. To limit the sub-use cases within CSI-RS overhead reduction, further splitting and prioritization of these sub-sub use cases is needed. For the preliminary result in Table B and also in some columns in the excel sheet corresponding to Table A, we suggest to use more quantitative description. For instance, ‘Decent SGCS’ does not provide much information. Suggest to quantify percentile gain over the baseline used in the simulations. In Table B, sub-Case B, please add Label: Measurements of channel in future time occasions. Benchmark: Rel-18 CSI prediction and Rel-19 AI CSI prediction using periodic CSI-RS with a large periodicity (e.g., 20 ms) |
| InterDigital |  | We agree with DCM and others that cases A and E should be merged. At this point, there is not much value in being too specific based on subtle nuances or detailed approaches for different sub-cases. It is clear from the contributions that companies listed for a sub-use case have various differences in terms of details. While it may be reasonable to acknowledge different approaches that companies have used, we should not spend time on the detailed classification, capturing potential spec impact, etc. The task at hand is to identify use cases and subsequently, different approaches can be studied in detail after the conclusion of the current phase. |
| Samsung |  | Support the observation. We believe that the difference between sub use case A and sub use case E needs to be clarified. |
| CATT2 |  | Due to the incident in offline discussion, some alignment change seems missed from the latest draft version, e.g. name of the Table A… I take the liberty to modify the latest table a bit, please FL have a check whether the edit mark change in the latest table is correct/acceptable. |
| MediaTek |  | For Table B, sub-case B, following are the details from our evaluation: Model input; Past CSI samples obtained from CSI-RS measurements. Model output; Future CSI at time granularity finer than CSI-RS periodicity. Label; Measurement in future time occasions at finer time resolution.. Training types assumption; offline training. KPI; NMSE, SGCS, throughput. Benchmark; Sample and hold. Model location for inference; UE-sided model or NW-sided. Collaboration/interaction between UE and NW; For UE-sided model: UE to initiate data collection and report prediction.. Potential spec impact; Signaling for data collection and inference. |
| vivo |  | For sub-case B, the following information is updated. Model output. Channel matrix of future instances, Interference in future instances Model location for inference. UE-sided model, NW sided model Collaboration/interaction between UE and NW. As UE-sided model in NR, As NW-sided model in NR Potential spec impact As CSI prediction in NR for Rel-19 CSI prediction Reporting content, signalling and procedure for LCM for extension cases |

#### (3rd round)Proposed observation 2.1 A with extra table:

Observation

For 6GR AI/ML use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

- [6 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 sources] assumed differently. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [4 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/band/frequency block with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/training type) and initial analysis can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [ 1source] provided preliminary simulation results and analysis on, Tokenized CSI prediction.Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table B

| Sub-use case | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams | Sub-Case E: Tokenized CSI prediction |
| --- | --- | --- | --- | --- |
| Reported Companies | (6) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1 | (4) Samsung, Apple, LGE, DoCoMo1 | (2) Samsung, vivo1 | (1) Huawei |
| Model input | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with 20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 | Channel matrix of carrier/band/frequency block A | Channel matrix of Set B of beams | Past CSI tokens |
| Model output | 1. Channel matrix of future instances 2. Interference in future instances1 | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams | Predicted CSI tokens |
| Label | Measurement in future time occasions. | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams | Estimated/ideal CSI tokens |
| Training types assumption | offline training | offline training | offline training | Online finetuning |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS |
| Benchmark |  | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams | Non-AI based CSI prediction or AI-based CSI prediction with offline trained model. |
| Model location for inference | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 Two-sided model1 | UE-sided model |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR1 | As UE-sided model in NR | As UE-sided model in NR | As UE-sided model in NR |
| Potential spec impact | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams 2. signalling/ procedure related to LCM | On top of NR CSI prediction, signalling/ procedure related to LCM for online finetuning |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Comments or questions. I have a question: What is CSI token? Can Huawei define this in existing 3GPP terminology or define it with wireless communication language? I didn’t use “similar to xxx”, because this use case is some extension of NR. But if you have some concern, please let me know. |

## DMRS design with AI receiver(closed)

Observation

For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver.

- [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver.

- [11 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver.

- [5 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver.

- Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table for detailed assumptions of each category will be added.

#### Proposed observation 2.2-1:

Table C

| Sub-use case | Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain | Sub-case B: Superimposed pilot | Sub-case C: DMRS free |
| --- | --- | --- | --- |
| Reported companies | (23) Nokia1, Futurewei2, Ericsson3, ZTE4, {Spreadtrum, UNISOC}5, Interdigial6, vivo7, xiaomi8, CMCC9, {CATT, CICTCI}10, Fujitsu11, Apple12, Samsung13, Kyocera14, Lenovo15, Huawei16, Qualcomm 17, Ofinno18, NVIDIA19, MediaTek20, Lekha21, LGE22, DocoMo23 | (12) vivo 1, CMCC2, ZTE3, Lenovo4, Huawei5, OPPO6, NVIDIA7, LGE8，Xiaomi9 , InterDigital10 , DocoMo11 Kyocera12 | (5) InterDigital1, Huawei2, NVIDA3, MediaTek4, Lenovo5 |
| Model input | 1. Received signal/estimated channel at DMRS and received signal on data 1, 13, 22,15,3,17,10,4, 20,6,18,23 1a. additionally noise variance 1,13 2. Received signal/estimated channel at DMRS2,7, 8,11,12,13,16,5 | 1. Received signal of target REs (superimposed signal) (Majority) 2. Estimated channel (in delay doppler domain) from the received signal of target REs (superimposed signal) 1 For Tx side of two-sided model: modulated symbols and DMRS symbol5 | Received signal on target REs For Tx side of two-sided model: coded bit2 |
| Model output | 1. Estimated channel at target data and/or DMRS REs2,4,5,7,8,9,11, 12, 13,16,17,18,19,21,22,23 1a. Estimated noise variance 12 2. LLRs1,2,3, 4, 6,10, 13,15,19,20, 22 3. Filtering coefficients for channel estimation 7 | 1. Estimated channel at target REs1,3,4,5,6,8 2.LLR2,3, 5,6,7,8,11,12 3.Estimated modulation symbols9 For Tx side of two-sided model: superimposed signal5 | 1.LLR (majority) 2.Estimated channel2 For Tx side of two-sided model: modulated data symbols 5 |
| Label | 1. Ideal channel information 2,5,7,8,9,11,12,13,15,16,17,18,22,23 2. Known sequence/data1,2,3, 4,10, 13,15,16,20,22 3. Label free (unsupervised)6, 21 4. Estimated channel using legacy DMRS pattern with legacy receiver8, 5. Estimated channel of adjacent RE (self-supervised)13 | 1. Reference transmission of known sequence/data 2,3,11,12 2. Ideal channel information1,8 3.Transmitted modulation symbols9 | 1. Known sequence/data 2 Genie-aided channel2 3. Label free1 |
| Training types assumption | offline training | offline training | offline training |
| KPI | MSE, BLER, throughput | MSE, BLER, throughput | MSE, BLER, throughput |
| Benchmark | With ideal channel information With conventional receiver with sparse or legacy DMRS | With ideal channel informal With conventional receiver with legacy DMRS overhead | With ideal channel information With conventional receiver with legacy DMRS overhead |
| Model location for inference | UE-sided model for DL or NW-sided model for UL | UE-sided model for DL NW-sided model for UL Two-sided model5 | UE-sided model for DL1 NW-sided model for UL3,4 Two-sided model2,5 |
| Collaboration/interaction between UE and NW | As UE-sided or NW-sided model in NR | As UE-sided or NW-sided model in NR As for two-sided model in NR5 | As for UE-sided model NR As for NW-sided model in NR As for two-sided model in NR |
| Potential spec impact | 1. DMRS pattern design 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model | 1. DMRS pattern design 2. RAN 4: Demod requirement 3. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable | 1. RAN 4: Demod requirement 2. Signalling/ procedure related to LCM for UE and/or NW sided model or two-sided model (including inter-vendor calibration), when applicable |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide the potential spec impact on DMRS free with AI receivers. |
| NTT DOCOMO |  | We also provide the simulation results for Sub-case 1 and Sub-case 2 and support the further study and down-selection for these two sub-use cases. Please add DCM to sub-case 1 and 2. Thank you! |
| Futurewei |  | We have done simulation whose output may be either predicted channels for all REs or LLRs. Please see above with our edits for “Model output”: 1. Estimated channel at all REs2,5,8,9, 12, 13,16,17,18,19,21 |
| QC |  | For sub-case A, please add Model output: Estimated Rnn |
| Huawei, HiSilicon |  | 1, For Sub-use case C, for model output, we also adopt estimated channel, other than LLR LLR, estimated channel2 2, For potential spec impact, what does “RAN 4: Demo requirement” mean? |
| Xiaomi |  | we have identified a few issues regarding our schemes, as outlined below: - Sparse DMRS: as stated in our paper, the input to the AI model should be the Estimated channel at DMRS, rather than Received signal of REs. The output of the AI model can include not only the estimated channel at all REs, but also estimated channel at partial REs. Moreover, the label does not necessarily need to be the [approximately] ideal channel; it can also be the real estimated channel using legacy DMRS pattern with legacy receiver. Note: we can also fine with using [approximately] ideal channel as a label. - SIP: It seems that Xiaomi’s solution was not accounted for in this sub-case B. However, our paper provided detailed preliminary evaluation results and analysis on SIP. As discussed above, we have revised the Proposed observation 2.2 using Track Changes. |
| CATT, CICTCI |  | We provide simulation results with output of demodulate bits. Add our mark ‘10,’ above LLR in the model output row, and also bullet 2 of label row. Additionally, For sub case A, the model output will be quite different between ‘estimated channels’ and ‘LLR’. The spec impact may be different, e.g. in terms of data collection. Should we split them into different sub use cases?=> the difference is just different format of collected data. It should be ok |
| ZTE |  | The model input of Sub-case A, does “1. Received signal of target REs [and transmitted target REs] ” refer to the received signal of the DMRS and the received signal of data channel? Some clarification is needed. The label of Sub-case C, it seems to be the bit sequence instead of “Constellation points of data channel”. |
| Ofinno |  | Regarding training type, online training (at UE-side for re-training the AI model based on the performance monitoring) can be considered. |
| Nokia |  | Suggest focusing on the majority supported direction. #### Proposed observation 2.2: [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver. - [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver. On Table C, we suggest focusing only on Sub-case A. - For model input, 1 and 2 variants should be enough. - For model output, 1 and 2 variants should be enough. - For Label, 1 and 2 variants should be enough. - Benchmark, no need of using ideal CSI. Better to us 1) conventional receiver with sparse DMRS 2) conventional receiver with legacy DMRS - No need to list results at this stage. - It is unclear what is meant by “No” in collaboration. We think similar considerations as Rel-19 for UE-sided and NW-sided model shall be considered. |
| MediaTek |  | We support the study of AI-receiver only in the UL. AI-receiver is too complex to be implemented at the UE for DL. For sub-case C, the labels are the transmit data bits (0 or 1), and not the constellation points. Although the model inference for sub-case B is taking place at the NW, the model input could also be considered as the transmit bits, instead of the received signal, since the constellation mapping applied to the bits is part of the model. Spec impact of DMRS-free UL transmission includes: - Data collection framework for offline training - Signalling for indicating - transmit constellation(s) to be used by UE, - performance monitoring, and - change(s) in DMRS configuration |
| CMCC |  | For Sub-Case A and B, except offline training, we think online training also can be studied to alleviate generalization issue and model complexity. Training types assumption: offline training online training =>FL: need feasibility analysis for online training. |
| LGE |  | In Table C, regarding the sub-case A, we have done simulation whose output may be LLRs. We also support considering the channel over all REs as the model output. Signaling-based collaboration between the UE and the network, such as UE capability reporting, is required. Please edit Model input: 1. Received signal of target REs1,2,3,8,9,12,14,15,16,18 ,20,21, 22[and transmitted target REs9,13,19] Model output: 1. Estimated channel at all REs5,8,9, 12, 13,16,17,18,19,21, 22 2. LLRs1,2,3, 13,15,19,20, 22 Label: 1. [Approximately] ideal channel 5,7,8,9,12,13,17,18, 22 2. Reference transmission of known sequence/data1,2,3,13,15, 22 Collaboration/interaction between UE and NW: As Channel prediction for UE/NW-sided model in NR In the case of the label-free approach, the definitions of the model input and output, as well as the formulation of the loss function, should be clearly specified. In Table C, regarding the sub-case B, We consider that the model output may include both the estimated channel and the LLR values, while in some cases only the LLR values are predicted. Signaling-based collaboration between the UE and the network, such as UE capability reporting, is required. Please edit Model output: 2. LLR2,5,6,7,8 Label: 1. Reference transmission of known sequence/data 2,3,8 2. [Approximately] ideal channel 1,8 Collaboration/interaction between UE and NW: As Channel prediction for UE/NW-sided model in NR |
| TCL |  | Support Sub-case A and B. |
| vivo |  | Ok with the FL original version. |
| OPPO |  | It is premature to narrow down sub-cases for DMRS OH reduction at early stage of Rel-20. In our view, all three sub cases deserve to be carefully studied in Rel-20. Meanwhile, in our evaluation, we did identify performance differences under different scenarios, e.g. high-speed scenario with 120km/h. It can be considered as a hint to move on to the study and evaluation. |
| Ericsson |  | For sub Case A, for “Collaboration/interaction between UE and NW”, please add - NW-sided model: Potential collaboration/interaction for data collection via the UE transmission of bit sequences known to both transmitter and receiver. - UE-sided model: UE-sided data collection and LCM-related processes (e.g., functionality activation/deactivation, monitoring, fallback, etc.) For “Potential spec impact”, please add: - The definition of sequences of bits known to both receiver and transmitter - => FL: covered by data collection under LCM |
| InterDigital |  | For sub-use case B, we have provided simulation results as well and have added our name in the Table above as the 10th source. For sub-use cases A and C, we added some clarification on label and training type for the evaluations we’ve performed. For sub-use case A, perhaps it should say “lower MSE” for “preliminary result”? |
| Samsung |  | Support the observation. Some newly added model output can be merged. For example, Estimated channel at DMRS RE and Estimated channel at partial REs can be merged to the same bullet. |
| CMCC |  | For the label of Sub-Case A, we are using 1. genie-aided channel estimation. Table C has captured our input well. |
| MediaTek |  | For both sub-case A and C, in our evaluations, the labels are the known bit sequence, and the model outputs are the LLRs after demodulation. For sub-case C, the model input could also be considered as the transmitted bit sequence, since the learned constellation at the UE may also be considered part of the model. |
| NTT DOCOMO |  | We have updated the model input/output/label for the sparse DMRS and SIP. For model input of both schemes, our assumptions are the same as the majority. |
| Samsung |  | Regarding sub-case A, we provide the following additional input. The model input requires the DMRS sequence information, if received signal is input without channel estimation. 1. Received signal of target REs (DMRS and/or data ) (majority) 1.a) DMRS sequence for the target REs 2. Estimated channel at DMRS7,8，11 |
| Apple |  | For sub-case A, Apple is also estimated channel at target REs, in addition to noise estimation. For terminology, suggest changing Rnn to noise variance estimation, where noise variance includes the channel estimation error noise \sigma_e^2. LLR calculation includes both channel estimation and noise estimation. |
| Nokia |  | Few comments/updates in blue for some boxes. Label; 1. Genie-aided channel estimation 5,7,8,9,12,13,17,18,22,23 2. Reference transmission of known sequence/data1,2,3, 10, 13,15,22 3. Label free6, 13,21 4. Real estimated channel using legacy DMRS pattern with legacy receiver8. As we understood, IDC still use pseudo-labels ? with the known repetition mapping serving as prior knowledge to assist decoding. It resembles supervised learning because it relies on "labeled-like information", guided decoding, and doesn’t involve discovering hidden patterns like in unsupervised learning. Did not find much details on Samsung and Lekha on label-free method. Shall we delete this blue highlighted part ? |
| Samsung |  | We used self-supervised learning for denoising. Please find some update in the table. |

## CSI compression and feedback

#### Proposed observation 2.3:

[13 sources] provided preliminary simulation results and analysis on CSI compression and feedback with AI/ML receiver.

- Alt 1:

  - [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC), and/or modulation (JSCM) with two-sided model.

  - [3 sources] provided preliminary simulation results and analysis on CSI compression with linear projection and NW-sided model.

- Alt 2:

  - [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC) with two-sided model.

  - [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM) with two-sided model or NW-sided model (i.e. with linear projection).

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook with NW-sided model.

- [xx sources] provided preliminary simulation results (or cite to NR AI/ML for beam management simulation results) and analysis on CSI restriction with CSI feedback with SRS with two-sided model (assuming separate source and channel coding).

- Besides, one source provided preliminary simulation results and analysis on joint CSI prediction and compression (BJTU), joint CSI Estimation and Compression with DMRS overhead reduction (BJTU), Multi-beam CSI compression for HBF (vivo), JSCC with legacy modulation constellation with NW-sided model ({Pengcheng, ZGC}), SRS fusion (Qualcomm), Vector quantization codebook enhanced CSI compression (BUPT, ZGC).

- SSCC + SRS

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D.

Note: whether/how to capture the observation in the TR is a separate discussion.

#### Proposed observation 2.3A:

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on CSI compression and feedback with AI/ML receiver.

- [10 sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC)

- [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM) including compression via a projection matrix

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook with NW-sided model.

- [xx sources] provided preliminary simulation results (or cite to NR AI/ML for CSI compression simulation results) and analysis on CSI reconstruction with CSI feedback with SRS with two-sided model (assuming separate source and channel coding).

- Besides, one source provided preliminary simulation Multi-beam CSI compression for HBF (vivo), JSCC with legacy modulation constellation with NW-sided model ({Pengcheng, ZGC}), SRS fusion (Qualcomm), Vector quantization codebook enhanced CSI compression (BUPT, ZGC).

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D.

Note: whether/how to capture the observation in the TR is a separate discussion.

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  |  |
| QC |  | For sub-case B: Why limit this sub-case to “linear projection”? QC is also interested in studying simpler UE-part model (of which linear projection is a special case) enabled by SRS fusion at the NW-side, as we indicated in our tdoc. If the sub-case B is relaxed to include QC interest, please add Model location for inference: two-sided model Collaboration/interaction between UE and NW: As for two-sided model in NR |
| Huawei, HiSilicon |  | 1, For Sub-case A, “[and estimated channel based on SRS2,3]” with Explicit H as input is an individual sub-use case, not necessarily belonging to JSCC/JSCCM - SSCC can also adopt explicit H. Please make it as a separate sub-case. In addition, from ZTE, MTK, vivo Tdocs, we understand they do not bring Explicit H as model input/output for sub-use case A. 2, what is the difference between Sub-case B and Sub-case C? |
| CATT, CICTCI |  | For sub use case A, should we split JSCC and JSCM into different sub use cases? The spec impacts are expected to be different in terms of inference report and also data collection. |
| ZTE |  | Regarding the Sub-case B, the spec impact “2. Signalling/ procedure related to LCM with NW-sided model” seems not needed because there is no model at the UE side. Regarding the Sub-case C, the model input may need the selected basis as well. |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| MediaTek |  | We have strong concerns on JSCC and JSCM for CSI compression, especially with two-sided models; we believe more detailed study of the effect of UL control channel errors on 5G NR-type CSI compression is required. |
| CMCC |  | 1. We think [estimated channel based SRS] is not necessarily coupled with JSCC/JSCCM, it can be also used for SSCC based Rel-20 CSI compression. 2. What is the difference between Sub-Case B and C? If the DLable basis/codebook can be also seen as one kind of linear projection, and the amplitudes and phases based on feedback CSI bits in Sub-Case C is further post-process on compressed CSI information. =>FL: C still need quantization, but B doesn’t |
| TCL |  | Support Sub-Case A |
| OPPO |  | It seems sub-case A and sub-case B are close to each other for CSI compression. However, for sub-case C, the downloadable codebook can be designed in an offline manner, there seems no inference at all. From this sense, sub-case C should be observed separately. On performance evaluations, for the eTypeII benchmark, how to evaluate its performance when considering non-ideal uplink feedback should be considered in the initial stage of EVM. |
| BUPT |  | Support Sub-Case A. We think that JSCC/JSCM-based approaches provide performance advantages, and Sub-case B can be adopted when users lack AI capability. |
| Samsung |  | We think it is better to separate sub-case A vs Sub-Case B considering very different spec impact for NW-sided model and two-sided model. regarding on whether to further separate JSCC vs JSCM with 2-sided model. we don’t have strong view. However, the difference of spec impact is limited. Therefore, it is better to keep them together. |
| LGE |  | Minor comment on the Excel file. In the Excel file, our proposal for the TDD extension of CSI compression seems to have been placed on the wrong sheet. It has to be moved from the “UL precoding” sheet to the “CSI compression” sheet. Accordingly, I made the change and uploaded a new Excel file (v005). |
| QC |  | We filled in Sub-case D. |
| MediaTek |  | We have filled in Sub-case E |

Observation For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on CSI compression and feedback. - [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC) - [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM) - [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook. - [3 sources] provided preliminary simulation results (or cite to NR AI/ML for CSI compression simulation results) and analysis on CSI reconstruction with CSI feedback with SRS (assuming separate source and channel coding). - [1 source] provided preliminary simulation results and analysis on CSI feedback without SRS. - Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D. Note: whether/how to capture the observation in the TR is a separate discussion.

#### (3rd round)Proposed observation 2.3B:

Table D

| Sub-use case | Sub-case A: CSI compression with JSCC | Sub-case B: CSI compression with JSCM | Sub-case C: DLable basis/codebook | Sub-case D: CSI reconstruction with CSI feedback with SRS (assuming SSCC) |
| --- | --- | --- | --- | --- |
| Reported companies | (10) ZTE1, Samsung2, vivo3, {Pengcheng, ZGC}, Lenovo, OPPO, MediaTek4, CMCC, Fujitsu, Apple, Qualcomm5 | (6) BJTU1, Samsung2, OPPO3,{Pengcheng, ZGC}4,vivo, Qualcomm | (2) ZTE1, Samsung | (3) Qualcomm, xxx, xxx |
| Model input of decoder [or model output of encoder, when applicable] | 1. Compressed CSI bits 1a. additionally estimated channel based on SRS2,3 | 1. Compressed CSI complex values via UE-sided model 2. Compressed CSI complex values via a projection matrix1,2,3 3. Received signal at sparse CSI-RS and CSI-RS sequence 1,4 | 1.Amplitudes and phases obtained by a look up table based on feedback CSI bits 2. Selected basis1 | 1. Compressed CSI bits 2. Estimated channel based on SRS |
| Model output of decoder [or model input of encoder, when applicable] | 1.Eigenvectors 2.Explicit H1,2,3,4 | 1.Eigenvectors 2.Explicit H2 | Eigenvectors | Eigenvectors |
| Label | 1.Eigenvectors 2.Explicit H1,2,3,4 | 1.Eigenvectors 2.Explicit H2 | Eigenvectors | Eigenvectors |
| Training types | Offline training | Offline training | Offline training | Offline training |
| KPI | SGCS, NMSE, SE, UPT, UE complexity | SGCS, NMSE, SE, UPT, UE complexity | UPT vs overhead | SGCS, UPT |
| Benchmark | eType II NR separate source and channel coding | eType II NR separate source and channel coding JSCM with two-sided model1,2,3 | eType II | eType II NR AI/ML CSI compression without SRS SRS without CSI feedback |
| Model location for inference | Two-sided model | Two-sided model NW-sided model1,2,3 | NW-sided model | Two-sided model |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR | Similar to two-sided model in NR For NW-sided model: no collaboration or Similar to NW-sided model in NR and/or DLable/generated matrix | No collaboration Or Similar to NW-sided model in NR and/or DLable basis | Similar to two-sided model in NR |
| Potential specification impact | 1. Necessary signalling/ procedure to support JSCC 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Necessary signalling/ procedure to support JSCM 2. Projection matrix related signalling for NW-sided model, when applicable 3. Signalling/ procedure related to LCM with NW-sided model, when applicable 4. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration, when applicable | 1. Downloadable basis related signalling/ procedure 2. Signalling/ procedure related to LCM with NW-sided model | 1. Necessary signaling/procedure to support lower overhead and/or simpler CSI feedback |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Please directly fill in or modified the table |

## AI for beam management and extension

#### Proposed observation 2.4:

Observation Xxxx

Table E-1 AI/ML for beam management and extension

| Sub-use case | Sub-case A: Inter-Cell/TRP beam prediction and management | Sub-Case B: Cross frequency beam prediction | Sub-Case C: Tx-Rx beam pair prediction | Sub-Case D: Beam prediction for initial access | Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Sub-Case F: reinforcement learning-based approach beam selection |
| --- | --- | --- | --- | --- | --- | --- |
| Reported companies | (5) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, Qualcomm, Lenovo | (4+) Futurewei1, xiaomi2, Apple3, Qualcomm4 | (2) Ericsson, Nokia | (2) Huawei, vivo, | (1) Huawei | (1) Nokia |
| Model input | Measurements from Set B of one or more TRPs/Cells | 1. Measurement for one or multiple FR1 cells1,3 2. L1 measurements in one frequency2,4 |  | Measurements from Set B of SSB | Measurements from Set B And additional local UE information (moving direction and speed) as UE side model input | Measurements from Set C, DL Tx beam scheduling stats (at the NW), Cross corelation among DL Tx beams |
| Model output | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A of multiple TRPs/Cells or from Set A of each TRP/Cell [of current or future time instance] | 1. FR2 cell/beam related information1,3 2. L1 beam-related information of another frequency2,4 [of current or future time instance] | Best beam pair indexes (probability of each Tx-Rx beam pair in Set A to be the Top-1 Tx-Rx beam) and/or Predicted measurements from Set A [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Selected beam index for scheduling UE(s) |
| Label | Measurements [or Top beams] of Set A | Measurements on the target frequency | Measurements [or Top beams pairs] of Set A | Measurements [or Top beams] of Set A | As NR study on AI for BM | label-free (online learning) |
| Training types assumption | offline training | offline training | offline training | offline training | offline training; online finetuning | Online learning |
| KPI | Prediction beam/measurement accuracy Throughput RS overhead reduction, Complexity. | Prediction beam/measurement accuracy RS overhead reduction, |  | SGCS, NMSE, throughput, ratio of CSI-RS overhead, Prediction beam accuracy | As NR study on AI for BM | Throughput, End to end packet latency |
| Benchmark | Based on Set A Based on Set B | Based on measurements on the target frequency |  | Based on Set A Based on Set B | As NR study on AI for BM | Beam with largest RSRP (from Set C) consider as the scheduling beam |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model |  | UE-sided model or NW-sided model | NW-sided model + UE-sided model without training collaboration | NW-sided model |
| Collaboration/interaction between UE and NW | As NR AI for BM | As NR AI for BM | As NR AI for BM | As NR AI for BM | As NR AI for BM | None |
| Potential spec impact | 1. Inter-Cell/TRP beam management related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. SCell/frequency range activation procedure(of any) based on prediction beam 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model |  | 1. Initial access related procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. As NR AI for BM; 2. Signalling/ procedure for online finetuning | 1. The enhancements required for data collection. 2. Signalling/ procedure related to exploration phase (to mitigate the impact of exploration). |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | For BM, the proposed use cases that can be derived based on NR AI/ML for beam management study are also considered, if the companies reported detailed information according to the agreement in last meeting. many other companies also support this use case, which can be found in excel. |
| NTT DOCOMO |  | We also support sub-case B as a sub-case of cross-frequency prediction. =>FL: let me check whether evaluation results is there or not |
| QC |  | Sub-case A: - We also support Sub-case A. - Model output: Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted … best beam indices and probabilities of each Tx beam in Set Ato be the Top-1 Tx beam are not equivalent, although the former can be derived from the latter. =>FL: follow Rel-18 study Sub-case B: - Model input: why only limit to CSI-RS at this stage? We believe it should be general, e.g., L1 measurements in first frequency - Model output: Suggest not limiting the scope now to a specific FR. Second frequency could be FR3. A good inclusive output can be L1 beam-related measurements of second frequency. Sub-case C: - Does this sub-case imply adopting 5GA spatial and temporal beam predictions into 6G. This is categorized alongside the BM extensions, so want to check. - Model output: Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted … best beam indices and probabilities of each Tx beam in Set Ato be the Top-1 Tx beam are not equivalent, although the former can be derived from the latter. Sub-case D: - Model output: Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted … best beam indices and probabilities of each Tx beam in Set Ato be the Top-1 Tx beam are not equivalent, although the former can be derived from the latter. - Not clear why this one has “decent prediction accuracy” compared to Sub-Case A which has higher/good prediction accuracy. |
| Huawei, HiSilicon |  | Our case can be categorized into Sub-case C, with enhancement of location UE side information (moving direction and speed) as UE side model input, and online finetuning. Not positioning assisted. Sub-Case C: Time/Spatial domain DL Tx beam prediction Model input: Measurements from Set B. Additional local UE information (moving direction and speed) as UE side model input [Huawei] Training types assumption: offline training, online finetuning |
| Xiaomi |  | We suggest to divide ‘Inter-cell/TRP beam prediction’ into two separate sub use case since M-TRP beam prediction means group-based beam prediction. The output of M-TRP beam prediction can be ‘beam indexes in best beam groups (probability of each Tx beam group in Set A to be the Top-1 Tx beam group) and/or Predicted measurements from Set A of multiple TRPs or from Set A of each TRP [of current or future time instance’. And the table for sub case E M-TRP beam group prediction and management can be seen as below: Sub-case E: M-TRP beam group prediction and management. (5+) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, [LGE], Lenovo. Measurements from Set B of multiple TRPs or of some TRPs. beam indexes in best beam groups (probability of each Tx beam group in Set A to be the Top-1 Tx beam group) and/or Predicted measurements from Set A of multiple TRPs or from Set A of each TRP [of current or future time instance’. Measurements [or Top beams] of Set A. offline training. Prediction beam group/measurement accuracy Throughput RS overhead reduction, Complexity.. Based on Set A Based on Set B. Higher/good prediction accuracy RS overhead reduction. UE-sided model or NW-sided model. As NR AI for BM. 1. M-TRP beam management related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model. |
| Fujitsu |  | We also support Sub-case A and Sub-case B |
| ZTE |  | Regarding the Sub-case D, there is no need to further do simulation for it as it is the same as the 5G-A. The only difference is it is now targeted for RRC IDLE state. |
| Nokia |  | Direction is OK, but let’s come-back to this after 2.1 and 2.2. |
| CMCC |  | Generally OK for these extensions of AI based BM. Just one clarification on Sub-Case C, it seems it is only adopting 5G-A spatial and temporal beam predictions into 6G? The spec impact is also same as 5G-A. |
| LGE |  | We support Sub-case B and Sub-case D, and please remove square brackets on our company name. Also, the term ‘decent prediction accuracy’ needs to be more clarified. Regarding Sub-case A, similar view as Xiaomi. In our view, inter-cell beam prediction and management can be more related with LTM procedure, while M-TRP beam prediction is more likely relevant to MIMO M-TRP schemes, e.g., M-TRP joint transmission, TRP switching, etc. So, we also prefer to split Sub-case A to inter-cell beam prediction and M-TRP beam prediction. Regarding Sub-case C, we have similar comment as QC, is this just reusing Rel-19 AI/ML BM features? Lastly, we support BFR prediction for 6G AI beam management extension, and we think that several companies also support this. |
| TCL |  | Support all Sub-use cases. |
| NEC |  | We are supportive to have sub cases A and D. Sub case C seems a combination of BM-Case1 and BM-Case2 we already had. |
| OPPO |  | It is okay to consider the BM-Case1 and BM-Case2 studied in Rel-18 as feasible for 6GR, but for new BM-related use cases, or new variants, from our view, it should be re-considered. |
| Ericsson |  | We suggest to report the preliminary results more quantitatively, specially for use cases that are not extensions of Rel-19 AI/ML BM. For instance, descriptions such as ‘Decent SGCS’ ‘good/decent prediction accuracy’ do not provide much information. Suggest to quantify percentile gain over the baseline used in the simulations. |
| InterDigital |  | On preliminary results, we would suggest avoiding phrases like “Decent SGCS” as they are not commonly interpretable and too vague. We have concerns regarding directly citing Rel-18/19 results from 5GA since in this case, it corresponds to a 5GA use case – not a new 6GR use case or an extension. If it is an extension of a 5GA use case, there should be clarity on the extension and that should be reflected in the evaluations as well as the details of the use case. For instance, for sub-use case D, on interaction between UE and network, we cannot simply say “As NR AI for BM” since handling LCM aspects and applicability determination, etc., when involving UE in Idle state is significantly different from what has been considered so far in 5GA AI/ML for BM. Thus, we cannot claim it is same as NR beam prediction but only for initial access. For results for sub-use case D, it would be necessary to clarify the real benefit from the “decent prediction accuracy”. Reading the column for sub-use case D, it is not clear what exactly is the use case! Sub-use case C seems same as 5GA use case and if there are enhancements companies have considered for their evaluations for 6GR studies, that goes in the direction of details of a solution to address a sub-use case. They should not be listed in the present context of use case identification as it only adds more confusion. |
| Samsung |  | We are supportive of Sub-case A. In our understanding, the specification impact on Sub-case A can include LTM procedure (e.g., beam reporting for candidate cells). |
| Apple |  | We support sub-case B and sub-case D. It is noted Sub-case D is built over Rel-18/Rel-19 study and specification development, and the rationale for its support should be intuitive given the evaluation/specification work. We suggest for use cases leveraging previous releases’ work, the interest level can be gauged by the total number of supporting companies. |
| Nokia |  | Added sub-case E above. |
| Huawei, HiSilicon |  | We made some changes with change marks to the table for Sub-Case D and Sub-Case E. |
| QC |  | - For sub-Case A, change model input to: Measurements from Set B of multiple TRPs/Cells or of some one or more TRPs/Cells. - For sub-Case B, we believe the second listed input/output is already inclusive of the first one, so suggest removing them (i.e., remove “1. Measurement for one or multiple FR1 cells” from model input and “1. FR2 cell/beam related information” from model output). - The KPI for sub-case D needs to be updated. - For sub-cases A and B, even though we have expressed interest in our contribution about the use cases, we have not provided evaluation results, so QC can be crossed out from reported companies for those two use cases. - For sub-cases A, D, and E, Model output: Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted … best beam indices and probabilities of each Tx beam in Set Ato be the Top-1 Tx beam are not equivalent, although the former can be derived from the latter. |

#### (3rd round)Proposed observation 2.4-1:

Observation Xxxx

Table E-1 AI/ML for beam management and extension

| Sub-use case | Sub-case A: Inter-Cell/TRP beam prediction and management | Sub-Case B: Cross frequency beam prediction | Sub-Case C: Tx-Rx beam pair prediction | Sub-Case D: Beam prediction for initial access | Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information | Sub-Case F: reinforcement learning-based approach beam selection |
| --- | --- | --- | --- | --- | --- | --- |
| Reported companies | (5) Nokia, ZTE, xiaomi, CEWiT, DoCoMo, Qualcomm, Lenovo | (4+) Futurewei1, xiaomi2, Apple3, Qualcomm4 | (2) Ericsson, Nokia | (2) Huawei, vivo, | (1) Huawei | (1) Nokia |
| Model input | Measurements from Set B of one or more TRPs/Cells | 1. Measurement for one or multiple FR1 cells1,3 2. L1 measurements in one frequency2,4 |  | Measurements from Set B of SSB | Measurements from Set B And additional local UE information (moving direction and speed) as UE side model input | Measurements from Set C, DL Tx beam scheduling stats (at the NW), Cross corelation among DL Tx beams |
| Model output | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A of multiple TRPs/Cells or from Set A of each TRP/Cell [of current or future time instance] | 1. FR2 cell/beam related information1,3 2. L1 beam-related information of another frequency2,4 [of current or future time instance] | Best beam pair indexes (probability of each Tx-Rx beam pair in Set A to be the Top-1 Tx-Rx beam) and/or Predicted measurements from Set A [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Best beam indexes (probability of each Tx beam in Set A to be the Top-1 Tx beam) and/or Predicted measurements from Set A [of current or future time instance] | Selected beam index for scheduling UE(s) |
| Label | Measurements [or Top beams] of Set A | Measurements on the target frequency | Measurements [or Top beams pairs] of Set A | Measurements [or Top beams] of Set A | As NR study on AI for BM | label-free (online learning) |
| Training types assumption | offline training | offline training | offline training | offline training | offline training; online finetuning | Online learning |
| KPI | Prediction beam/measurement accuracy Throughput RS overhead reduction, Complexity. | Prediction beam/measurement accuracy RS overhead reduction, |  | SGCS, NMSE, throughput, ratio of CSI-RS overhead, Prediction beam accuracy | As NR study on AI for BM | Throughput, End to end packet latency |
| Benchmark | Based on Set A Based on Set B | Based on measurements on the target frequency |  | Based on Set A Based on Set B | As NR study on AI for BM | Beam with largest RSRP (from Set C) consider as the scheduling beam |
| Model location for inference | UE-sided model or NW-sided model | UE-sided model or NW-sided model |  | UE-sided model or NW-sided model | NW-sided model + UE-sided model without training collaboration | NW-sided model |
| Collaboration/interaction between UE and NW | As UE-sided or NW-sided mode in nRl | As UE-sided or NW-sided mode in nRl | As UE-sided or NW-sided mode in nRl | As UE-sided or NW-sided mode in nRl | As UE-sided or NW-sided mode in nRl | No collaboration |
| Potential spec impact | 1. Inter-Cell/TRP beam management related singling/procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. SCell/frequency range activation procedure(of any) based on prediction beam 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model |  | 1. Initial access related procedure 2. Signalling/ procedure related to LCM for NW-sided model or UE-sided model | 1. As NR AI for BM; 2. Signalling/ procedure for online finetuning | 1. The enhancements required for data collection. 2. Signalling/ procedure related to exploration phase (to mitigate the impact of exploration). |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Please fill in and modify |

## AI based (De-)Modulation

#### Proposed observation 2.5:

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on (de)modulation related design with help of AI/ML or AI/ML receiver.

- [4 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI and with non-AI or AI receiver.

- [3 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table F.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on (de)modulation. - [5 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI, and with non-AI or AI receiver. - [3 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model. - Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table F. Note: whether/how to capture the observation in the TR is a separate discussion.

#### Proposed observation 2.5

Table F

| Sub-use case | Sub-use case A: AI-based (de)modulation | Sub-use case B: AI-based modulation and precoding |
| --- | --- | --- |
| Reported companies | (5)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5 | (3)ZTE, OPPO, Lenovo |
| Model input | 1. Information bits1,2,5 2. Received signal3 3. Channel characterization [and modulation order] 3,4 | Encoder: information bits Decoder: Estimated symbols |
| Model output | 1. Constellation 1,2,5 2. LLR3 3. Probability 4 | Encoder: modulated symbols after layer mapping Decoder: soft LLR |
| Label | 1. Information bit1,2,5 2. Label free4 | Information bit |
| Training types | Offline training | Offline |
| KPI | BLER | BLER |
| Benchmark | Conventional constellations with uniform spacing | NR modulation and layer maping |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,5 2.NW-sided model3 | Two-sided |
| Collaboration/interaction between UE and NW | NA | As for two-sided model in NR |
| Potential specification impact | 1. Constellation design [and downloading mechanism] 2. Signaling/ procedure related to LCM for NW-sided model | 1. Modulation design and layer mapping design 2. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| QC |  | For sub-case A, can FL or other companies clarify how this is different from geometric shaping being discussed in the 6G modulation agenda? Given that the gain of geometric shaping is already well known and being considered for 6G, shouldn’t companies take the conventional geometric shaping as a benchmark and assess if the AI-designed constellation outperforms the benchmark? |
| Xiaomi |  | For the training type, we think both offline training and online training is possible. Our original intention is that preliminary constellation can be obtained by offline. But considering the constellation quite rely on channel characterization, online fine-tuning is necessary to obtain the optimal constellation for a given channel. Once the optimal constellation is obtained , then the online training will stop. Can the FL change the training type of Xiaomi’s scheme as “offline training+online fine tuning” => finetune maybe implementation. |
| Fujitsu |  | For Sub-use case A, for AI-trained constellation, it may require performance monitoring, since the constellation is also data driven. There should be scheme to check whether the AI-trained constellation works well or not. The following change is suggested for the potential spec impact of Sub-use case A. 1. Constellations desgin [and downloading mechanism, performance monitoring] => FL: covered by 2 |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| MediaTek |  | Most of the simulation results provided are with AWGN channels. For constellation shaping, non-AI approaches are being explored in other 6G agendas. Hence it would be premature to have an observation on this topic now. |
| vivo |  | Our results also include both AWGN and fading channel. Thus it should be clarified as: - 0.11~0.8dB gain with different modulation order in AWGN and fading channel 3,5 @QC, our understanding is that the baseline should be what NR has already specified. Otherwise all the cases may need complicated baseline discussion. |
| OPPO |  | To reply @QC, we adopt AI-based demodulation in our evaluations, i.e. AI-based model at Rx and non-uniform constellations at Tx. For the study, we would like to share some observation that with the same GS-based constellations, AI-based demodulations can still achieve more than 1dB SNR gains. That’s the major reason why we would bring it up here for AI/ML use case. For sub-use case A, we have additional comments - For the benchmark, it should be conventional constellations with uniform spacing at the transmitter side, and legacy demodulation (e.g., ML, Reduced-ML) at receiver side. - For the model location for inference with AI receiver, it should be NW-sided model for uplink transmission and UE-sided model for downlink transmission. - For the collaboration/interaction between UE and NW, the AI learned constellation should be aligned with the AI receiver or pre-known at receiver side with legacy demodulation method. |
| InterDigital |  | We share similar views as QC that considering the ongoing discussions on modulation for 6GR, we should not simply go with uniform modulation of NR as the benchmark. AI/ML-based solutions should be able to outperform relevant non-AI/ML benchmarks for consideration. |
| ZTE |  | We suggest updating the table F for sub-use case B regarding the decoder model input and model output. Updated Table F Sub-use case; Sub-use case A: AI-based (de)modulation; Sub-use case B: AI-based modulation and precoding. Reported companies; (5)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5; (3)ZTE, OPPO, Lenovo. Model input; 1. Information bits1,2,5 2. Received signal3 3. Channel characterization [and modulation order] 3,4; Encoder: information bits Decoder: Soft LLR estimated symbols. Model output; 1. Constellation 1,2,5 2. LLR3 3. Probability 4; Encoder: modulated symbols after layer mapping Decoder: information bits soft LLR. Label; 1. Information bit1,2,5 2. Label free4; Information bit. Training types; Offline training; Offline. KPI; BLER; BLER. Benchmark; Conventional constellations with uniform spacing; NR modulation and layer maping. Model location for inference; 1.NA (AI for constellation design with legacy receiver) 1,2,5 2.NW-sided model3; Two-sided. |

#### (3rd round) Proposed observation 2.5-1:

Table F For (de)modulation

| Sub-use case | Sub-use case A: AI-based (de)modulation | Sub-use case B: AI-based modulation and precoding |
| --- | --- | --- |
| Reported companies | (5)ZTE1, OPPO2, vivo3, Lenovo4, Xiaomi5 | (3) ZTE, OPPO, Lenovo |
| Model input | 1. Information bits1,2,5 2. Received signal3 3. Channel characterization [and modulation order] 3,4 | Encoder: information bits Decoder: Estimated symbols |
| Model output | 1. Constellation 1,2,5 2. LLR3 3. Probability 4 | Encoder: modulated symbols after layer mapping Decoder: Soft LLR |
| Label | 1. Information bit1,2,5 2. Label free4 | Information bit |
| Training types | Offline training | Offline |
| KPI | BLER | BLER |
| Benchmark | Conventional constellations with uniform spacing | NR modulation and layer mapping |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 1,2,5 2.NW-sided model3 | Two-sided |
| Collaboration/interaction between UE and NW | NA or Similar to NW-sided model in NR | Similar to two-sided model in NR |
| Potential specification impact | 1. Constellation design [and downloading mechanism] 2. Signaling/ procedure related to LCM for NW-sided model | 1. Modulation design and layer mapping design 2. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please double check |

## AI for DPoD/DPD/none-linearity handling (Closed)

#### Proposed observation 2.6:

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver.

- [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver.

- [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver. - [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver. - [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter. - Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G. Note: whether/how to capture the observation in the TR is a separate discussion.

#### Proposed observation 2.6-1:

Table G

| Sub-use case | Sub-use case A: AI-based DPoD/None-linearity compensation | Sub-use case B: AI-based DPD |
| --- | --- | --- |
| Reported companies | (5) Samsung1, Ericsson2, OPPO3, vivo4, Huawei5 | (2) vivo, Huawei |
| Model input | 1. Information bits2 2. Received signal1,3,5 | Time domain samples before pre-distortion |
| Model output | 1. Compensated signal in time domain1,2,5 2. Soft bits2,3 | Time domain samples after pre-distortion |
| Label | 1. DMRS1 2. Known bit sequence2,3 3. time domain samples from known sequence5 | Time domain samples |
| Training types | Online training/finetune1 Offline training | Offline |
| KPI | BLER, MPR, EVM, throughput | BLER, EVM, MPR |
| Benchmark | Without compensation | No DPoD |
| Model location for inference | 1.NA (AI for constellation design with legacy receiver) 2 2.NW-sided model1,2,3,5 | UE-sided |
| Collaboration/interaction between UE and NW | NA | As for two-sided model in NR |
| Potential specification impact | 1. RAN 4 requirements 2. UE capability report 3. DMRS design/selection, Tx power determination 4. Data collection and performance monitoring for offline training. | 1. RAN4 requirements 2. Dedicated signal (RS/data) for training or monitoring data collection. |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| CATT, CICTCI |  | For the potential specification impact, is it a typo in sub use case B for ‘2. Dedicated signal (RS/data) for training or monitoring data collection for AI/ML DPoD.’? This sub use case is AI/ML-based DPD. Or, the spec impacts of these two sub-use cases are reversed incorrectly (because DPoD may not need UE capability report)? |
| ZTE |  | We propose to study these two sub cases in RAN4 instead of RAN1. |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| MediaTek |  | We may be open to studying AI-based DPoD; however we do not support AI-based DPD at the UE. |
| vivo |  | Yes, the specification impact for sub case B should be AI/ML DPD. And for AI-based DPD, it is not two sided model. It should be “as for UE-sided model in NR”. |
| OPPO |  | Fine to study DPoD, specifically for UL, where NW can handle the PA non-linearity more easily with acceptable complexity. |
| Ericsson |  | For Ericsson, the model location for inference is NW-sided. |
| InterDigital |  | For evaluations with online finetuning/training, since this has not been used in 5GA evaluations, it would be good for proponent companies to share how online training/finetuning was modelled and evaluated. |
| Samsung |  | For model input, we used the received signal related items (i.e., the time domain sample and high order item) for model input, so it is better put ours in option 2. We also have the model deployed at the network side. So some change in “Model input” and “Model location for inference”. |

## AI for TMPI

#### Proposed observation 2.7:

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI-based UL TMPI with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table H.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI-enabled UL precoder indication with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table H. Note: whether/how to capture the observation in the TR is a separate discussion.

#### (3rd round)Proposed observation 2.7-1:

Table H AI-enabled UL precoder indication

| Use case | AI-enabled UL precoder indication |
| --- | --- |
| Reported companies | (3) vivo1, Fujitus2, Samsung3 |
| Model input of decoder or model output of encoder | Multi-bit payload |
| Model output of decoder or model input of encoder | (Reconstructed) eigenvectors of UL channel |
| Label | Estimated eigenvectors of UL channel based on SRS measurement |
| Training types | offline training online finetune1 |
| KPI | SCGS, BLER |
| Benchmark | NR TPMI codebook |
| Model location for inference | N/A [downloadable UL codebooks are used in model-transparent manner] 1,3 Two-sided model1,2 |
| Collaboration/interaction between UE and NW | No collaboration for DLable UL codebook Similar as two-sided model in NR |
| Potential specification impact | 1.The procedure related to the download of UL codebooks 2. LCM procedure to facilitate the training of the downloadable UL codebooks or for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| Fujitsu |  | Which part is highlighted as yellow? |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| vivo |  | The table is fine from our side. |
| Samsung |  | Fine with the table. |

## AI for SRS

#### Proposed observation 2.8:

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and analysis in Table I.

Besides, one source provided preliminary simulation results and initial analysis on low PAPR SRS sequence design with help of AI/ML.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML [1 source] provided preliminary simulation results and initial analysis on low PAPR SRS sequence design with help of AI/ML Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and analysis in Table I. Note: whether/how to capture the observation in the TR is a separate discussion.

#### (3rd round)Proposed observation 2.8-1:

Table I

| Use case | Low overhead SRS with AI/ML | Low PAPR SRS sequence design |
| --- | --- | --- |
| Reported companies | (3) {Spreadtrum, UNISOC}, vivo, Huawei | (1)vivo |
| Model input | Received SRS | RS index (as input to specified/DLable/ULable look-up-table for SRS sequences) |
| Model output | Estimated channel | Learn sequences (as output from input to specified/DLable/ULable look-up-table for SRS sequences) |
| Label | Ideal channel information | Label free |
| Training types | Offline training | Offline training |
| KPI | SCGS, throughput | PAPR, SGCS, Cross-correlation between SRS sequences |
| Benchmark | With legacy SRS With ideal channel information | Legacy SRS sequence |
| Model location for inference | NW-sided model | NW-sided model or NA |
| Collaboration/interaction between UE and NW | No collaboration or Similar to NW-sided model in NR | No collaboration |
| Potential specification impact | 1.Sparse SRS design 2. Signalling/ procedure related to LCM for NW-sided model | SRS design, or signaling/procedure related to DLable/ULable SRS sequence. |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| LG Electronics |  | Please include LG as a supporting company. =>FL: let me check whether LGE provided evaluation results or not. |
| vivo |  | More is less: Similar SGCS with more less overhead. |
| Kyocera |  | Please include Kyocera as a supporting company for “Low overhead SRS with AI/ML”. We provided sims results on CSI-RS overhead reduction that is assisted with SRS fusion. Our simulation procedure can also be applied for low SRS overhead by adjusting the fusion process to exclude CSI-RS. |
| FL |  | Please check the yellow highlight |

## AI for PAPR reduction for waveform

#### Proposed observation 2.9:

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI/ML for PAPR reduction with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table J.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI/ML based waveform for PAPR reduction with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table J. Note: whether/how to capture the observation in the TR is a separate discussion.

#### Proposed observation 2.9-1:

Table J

| Use case | AI/ML based waveform for PAPR reduction |
| --- | --- |
| Reported companies | (3)vivo1, Samsung2, Huawei3 |
| Model input of decoder or model output of encoder | transformed symbols symbol in frequency domain |
| Model output of decoder or model input of encoder | the modulated symbol in frequency domain |
| Label | Label free2,3 LLR1,3 |
| Training types | offline training |
| KPI | BLER, CCDF of PAPR(UL), throughput (DL) |
| Benchmark | DFT-s-OFDM HPE waveform |
| Model location for inference | Two-sided model |
| Collaboration/interaction between UE and NW | As for two-sided model in NR |
| Potential specification impact | 1. Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| Huawei, HiSilicon |  | As a clarification, for two-sided model, our Rx side model output is Estimated channel/LLR, i.e., the Tx side model and Rx side model are not symmetric (though it is NW implementation on the Rx side model output). Accordingly, the loss function considers both minimizing PAPR (Label free) and optimizing LLR; thus LLR can also considered as label. Label: Label free2,3 HPE waveform LLR3 |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| vivo |  | For label part, the “HPE waveform1” can be deleted. Our results are also based on LLR for the NW side and PAPR or DCM reduction at the Tx side. Label: Label free2,3 HPE waveform LLR3 |
| Samsung |  | Generally fine. We suggest the following modifications - “Biased symbols” to “transformed symbols” for decoder input - Delete SGCS in the KPI as it is not applicable |

#### (3rd round)Proposed observation 2.9-2:

Table J AI/ML based waveform for PAPR reduction

| Use case | AI/ML based waveform for PAPR reduction |
| --- | --- |
| Reported companies | - vivo1, Samsung2, Huawei3 |
| Model input of decoder or model output of encoder | Transformed symbols symbol in frequency domain |
| Model output of decoder or model input of encoder | The modulated symbol in frequency domain |
| Label | Label free2,3 LLR1,3 |
| Training types | offline training |
| KPI | BLER, CCDF of PAPR(UL), throughput (DL) |
| Benchmark | DFT-s-OFDM |
| Model location for inference | Two-sided model |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR |
| Potential specification impact | Signaling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

## AI for Sequence based HARQ-ACK

#### Proposed observation 2.10:

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML for sequence based HARQ-ACK feedback with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table K.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML based HARQ-ACK feedback with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table K. Note: whether/how to capture the observation in the TR is a separate discussion.

#### (3rd round)Proposed observation 2.10-1:

Table K. AI/ML based HARQ-ACK feedback

| Use case | AI/ML based HARQ-ACK feedback |
| --- | --- |
| Reported companies | - Qualcomm1, vivo2 |
| Model input | HARQ ACK/NACK bit sequence (as input to specified/downloadable look-up table1) |
| Model output | Learn sequences modulated symbols (as output from specified/downloadable look-up-table1) |
| Label | HARQ ACK/NACK bit sequence |
| Training types | Offline training |
| KPI | BLER, SNR requirement for target BLER |
| Benchmark | NR RM code for short block length with Maximum Likelihood (ML) receiver |
| Model location for inference | NW sided model or NA |
| Collaboration/interaction between UE and NW | No collaboration1 Learned sequence or downloadable sequence for HARQ-ACK2 |
| Potential specification impact | Learned sequence (HARQ-ACK codebook) design or signaling for downloadable Learned sequence or downloadable sequence for HARQ-ACK Monitoring related procedure, if needed |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| QC |  | For model output, add modulated symbols. Also, not clear to us what the “highlighted yellow” above is referring to. |
| CATT, CICTCI |  | Is the order of the model input and model output reversed (assuming NW-sided model)? The label and model output should be consistent. |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| vivo |  | Ok for the FL version. @CATT this is from the model for training sequences perspective. |
| InterDigital |  | The description for the use case is not clear to us – similar questions as CATT on model output, label, etc. across the two sources. On model location for inferencing, it is not clear which results correspond to “NA” case and which ones are for NW-sided model. |
| QC |  | We made updates with change marks in Table K. |
| FL |  | Some further updates. Please check. |

## RAN for token traffic

#### Proposed observation 2.11:

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on RAN for token traffic with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table L. Besides, one resource provided preliminary simulation results and analysis on RAN assisted Token communication considering spatial-temporal correlation.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation For 6GR AI/ML related service, for 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on improved scheduling/HARQ for token traffic Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table L. Note: whether/how to capture the observation in the TR is a separate discussion.

#### (3rd round) Proposed observation 2.11-1:

Table L.

| Use case | RAN for token traffic |
| --- | --- |
| Reported companies | (2) Huawei1, OPPO2 |
| Model input | UE side: • Input: Raw data (e.g., image/video/audio, etc.) NW or OTT server side: • Input: Tokens or filtered tokens with stamps by RAN |
| Model output | UE side: • Output: Tokens (e.g., tokenized image/video/audio) NW or OTT server side: • Output: Inference results for downstream tasks/Raw data of multiple modalities |
| Label | Label free2 Training at OTT, transparent to RAN1 |
| Training types | Offline training at OTT, transparent to RAN |
| KPI | Supported number of UEs, achievable throughput |
| Benchmark | NR scheduling/HARQ mechanism |
| Model location for inference | The tokenizer model is at UE (e.g., an encoder). The de-tokenizer model is at NW or OTT server side (e.g., a decoder). |
| Collaboration/interaction between UE and NW | NA |
| Potential specification impact | • Service awareness in RAN • Token error identification, new scheduling and HARQ |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check the highlighted yellow |
| Lenovo | Support. | We support to take token traffic as a RAN use case for evaluation and study. |
| Huawei, HiSilicon |  | Since the AI/ML model is at application layer, the Label is 3GPP transparent. Label: Label free training at OTT, transparent to RAN |
| ZTE |  | AI for RAN is an interesting use case. But it seems this sub-Case is more like an application layer model, not sure RAN1 is the right place to discuss this use case. |
| Nokia |  | We think it is too early to have an observation on this item. Prefer to focus on 2.1 and 2.2 for now. |
| LG Electronics |  | Agree with Nokia. |
| TCL |  | Here only UL token communication is considered, please explain why there is no DL case. |
| FL |  | Question: How can PHY knows or define “image/video/audio”? |

## Other use cases

#### Proposed observation 2.12:

For 6GR AI/ML use cases identification/categorization,

[one source] provided preliminary simulation results and analysis on pathloss prediction,

[one source] provided preliminary simulation results and analysis on AI based UL closed-loop power control,

[one source] provided preliminary simulation results and analysis on prior-information-aided DCI decoding

[one source] provided preliminary simulation results and analysis on lossless DCI compression

[one source] provided preliminary simulation results and analysis on early contention resolution in RACH,

[one source] provided preliminary simulation results and analysis on sensing based RAN digital twin construction with NW-side AI/ML model ,

[one source] provided preliminary simulation results and analysis on AI/ML-enabled RAN digital twin with distributed model

[one source] provided preliminary simulation results and analysis on AI/ML based SRS power imbalance compensation,

[[one source] provided preliminary simulation results and analysis on site specific learning for AI/ML and RAN digital twin (Deepsig).]

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table M

Note: whether/how to capture the observation in the TR is a separate discussion.

Table M

| Use Case | Prior-Information-Aided DCI Decoding | Lossless DCI Compression | Sub-Case X: UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE. |  | Sub-Case X: Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL (PUSCH/PUCCH/PRACH/SRS) power control. | … |
| --- | --- | --- | --- | --- | --- | --- |
| Reported companies | CMCC | CMCC | (1) Nokia |  | (1) Nokia |  |
| Model input | LLR after demodulation at current transmission, and historical AI/ML based predicted LLR | Historical DCI payload | UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size |  | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). input can consider history of measurements |  |
| Model output | Decoded DCI payloads, and predicted LLR for next transmission | Predicted DCI payload | Predicted TPC command index |  | Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). output can consider future instances |  |
| Label | DCI payload sequences | DCI payload sequences | Optimal TPC command index (offline learning) label-free (online learning) |  | Pathloss value(s) (or L1-RSRP(s)) for a set of RSs/beams (Set A) |  |
| Training types | Offline training at the UE side | Offline training at the NW side, and model delivery to UE side | Offline and Online learning |  | Offline training |  |
| KPI | BLER performance | BER and sample-level prediction accuracy; DCI overhead reduction | UL throughput. |  | Pathloss prediction accuracy, throughput, RS overhead reduction, Complexity. |  |
| Benchmark | - Traditional DCI decoder | - Traditional DCI design | 1. UL Power control with optimized OLPC parameters - 2. UL Power control with optimized OLPC parameters and possibly legacy CLPC algorithms (with 5G TPC tables). |  | Pathloss estimation based on Set A - Pathloss estimation based on Set B |  |
| Model location for inference | UE-sided model | Two-sided model | NW-sided model |  | UE-sided model NW-sided model |  |
| Collaboration/interaction between UE and NW | No collaboration | Model transfer from NW to UE | None |  | As NR AI for BM |  |
| Potential specification impact | 1. Signalling/configuration design for prior-information-aided DCI decoder. 2. Signalling/ procedure related to LCM for UE-sided model | 1. Signalling/configuration design for Lossless DCI Compression. 2. Signalling/ procedure related to LCM including model transfer | Configurability of the values in TPC command tables or an extended TPC command table (compared to NR). |  | 1. Pathloss prediction related signalling/procedure 2. Signalling/ procedure related to LCM for UE-sided or NW-sided model 3. RAN4 performance requirements and test cases, including defining new requirements related to pathloss reference signal (PL-RS) measurement and activation delays of TCI state(s). |  |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Anything I missed. Pls note that, this is only for the one with evaluation results. |
| Huawei, HiSilicon |  | Considering the sources of companies may rise in the next/future meetings, we should add a condition that “till RAN1#122bis”. Till RAN1#122bis, The following use cases were proposed by one source with preliminary simulation results and analysis |
| ZTE |  | We were wondering what’s the plan for some use cases that are not included above, e.g., BSR prediction. |
| Ofinno |  | To my understanding, based on PRACH enhancement, some companies mentioned the approach while the details seem different. =>FL: let them speak for themselves |
| CMCC |  | We don’t think to categorize the results based on the company number only. It should also take other technical aspects, e.g., performance results, complexity into account. We don’t have a common rule in the last meeting that the categorization is based on company number. So, it should be fair enough to allow companies to submit results in the next two meetings as Huawei also commented. Since we don’t have a chance to discuss the performance details of each case, it is unfair to deprioritize the DCI cases now. As you can see from the simulation results, the performance gain is quite significant (3dB), and as stated in our contribution, the results are clear. We propose that it should be listed as one category with capturing the following table (from R1-2507020). Table 12. Summary of Prior-Information-Aided DCI Decoding Use Case; Prior-Information-Aided DCI Decoding. Model input; LLR after demodulation, and the predicted LLR from the DCI prediction network. Extra assistant information can also be incorporated, such as CSI (including CQI, PMI, and RI), HARQ-ACK feedback, and SRS measurement results.. Model output; Decoded DCI payloads, and predicted LLR for the next transmission. Label; Label free. Training types; Offline training at the UE side.. KPI; BLER performance. Benchmark; Traditional DCI decoder: - For convolutional codes: BCJR decoder - For Polar codes: traditional BP decoder, and CA-SCL decoder. Preliminary result; - For enhanced Convolutional decoder: achieves >5 dB improvement at BLER@1% and ~1.5 dB at BLER@0.1% compared to the baseline BCJR decoder, where the dataset is from Redcap scenario. - For enhanced Polar decoder: achieves about 5 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from Redcap scenario. - For enhanced Polar decoder: achieves about 4 dB improvement at BLER@1% compared to the traditional BP decoder, and about 3 dB gain compared to the CA-SCL decoder, where the dataset is from eMBB scenario.. Model location for inference; UE-sided model. Collaboration/interaction between UE and NW; No collaboration. Potential specification impact; - DCI dataset collection for AI/ML-based DCI prediction network training. - Alt 1: The transmitter collects the DCI dataset and distributes it to receivers for model training. - Alt 2: Each receiver independently gathers its own dataset for training. - AI decoder/non-AI decoder fallback mechanism: - Define criteria for switching between the prior-information-aided DCI decoder and the conventional non-AI decoder. - Signaling/configuration design for prior-information-aided DCI decoder. - Which DCI fields for prediction as model input. Model backbone; LSTM based or Transformer based. Table 14. Summary of Lossless DCI Compression. Use case; Lossless DCI Compression: DCI payload predicator as AI task, and Lossless DCI Compression as downstream application. Model input; DCI payload predicator: Historical DCI payloads. Model output; DCI payload predicator: Predicated DCI payload. Label; Label free: i.e., using recorded DCI payload sequences as label. Training types; DCI payload predicator: Offline training can be conducted at the BS side, with the trained predictor subsequently delivered to the UE via model delivery.. KPI; DCI payload predicator: BER and sample-level predication accuracy Lossless DCI Compression: Compression ratio after CRC attachment, and Aggregate overhead reduction. Benchmark; Traditional DCI design. Preliminary result; - (Dataset 1) DL grant DCI from field test: Normalized information entropy = 22.5%, BER = 0.15%, sample-level predication accuracy = 95.62%, overhead reduction = 85.9%. - (Dataset 2) UL grant DCI from field test: Normalized information entropy = 25.0%, BER = 0.03%, sample-level predication accuracy = 99.22%, overhead reduction = 88.6%. - (Dataset 3) DL grant DCI from SLS: Normalized information entropy = 11.2% (mean), BER = 0.01%, sample-level predication accuracy = 99.81%, overhead reduction = 86.3%.. Model location for inference; DCI payload predicator: Model inference is executed at both BS side and UE side. - Note: The predictor is offline-trained at BS and deployed to the UE via model delivery. This ensures both BS and UE share identical predictors, guaranteeing aligned inference outputs.. Collaboration/interaction between UE and NW; DCI payload predicator: Model delivery from BS to UE is required to maintain inference alignment.. Potential specification impact; - Introduction of a UE capability report to indicate support for Lossless DCI Compression. - Standardization of model delivery procedures to ensure predictor consistency between BS and UE. - Definition of a new short DCI format to indicate the use of AI-predicted legacy DCI content. - A mechanism to synchronize the historical DCI payload records between BS and UE.. Model backbone; DCI payload predicator: Feature Classifier (Transformer based) + Bit-Level Predictor (Bi-LSTM + Transformer based). |
| vivo |  | Interference prediction can be merged into CSI prediction case. Moreover, we believe it should be clear that use cases even with single company may still deserve some categorization for the future discussion. |
| Nokia |  | AIML-based pathloss prediction Sub-use case; Sub-Case X: Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL (PUSCH/PUCCH/PRACH/SRS) power control.. Reported companies; (1) Nokia. Model input; L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). - input can consider history of measurements. Model output; Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). - output can consider future instances. Label; Pathloss value(s) (or L1-RSRP(s)) for a set of RSs/beams (Set A). Training types; Offline training. KPI; Pathloss prediction accuracy, throughput, RS overhead reduction, Complexity.. Benchmark; Pathloss estimation based on Set A Pathloss estimation based on Set B. Model location for inference; UE-sided model NW-sided model. Collaboration/interaction between UE and NW; As NR AI for BM. Potential spec impact; 1. Pathloss prediction related signalling/procedure 2. Signalling/ procedure related to LCM for UE-sided or NW-sided model 3. RAN4 performance requirements and test cases, including defining new requirements related to pathloss reference signal (PL-RS) measurement and activation delays of TCI state(s).. AI/ML-based UL closed-loop power control Sub-use case; Sub-Case X: UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE.. Reported companies; (1) Nokia. Model input; UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size. Model output; Predicted TPC command index. Label; Optimal TPC command index (offline learning) label-free (online learning). Training types; Offline and Online learning. KPI; UL throughput.. Benchmark; 1. UL Power control with optimized OLPC parameters 2. UL Power control with optimized OLPC parameters and possibly legacy CLPC algorithms (with 5G TPC tables).. Model location for inference; NW-sided model. Collaboration/interaction between UE and NW; None. Potential spec impact; Configurability of the values in TPC command tables or an extended TPC command table (compared to NR).. |
| Huawei, HiSilicon |  | Please see some updates as in below. Note that one use case of “AI/ML based SRS power imbalance compensation” from our Tdoc (R1-2507064) is missed. For 6GR AI/ML use cases identification/categorization, [one source] provided preliminary simulation results and analysis on pathloss prediction (Nokia), [one source] provided preliminary simulation results and analysis on AI based UL closed-loop power control (Nokia), [one source] provided preliminary simulation results and analysis on prior-information-aided DCI decoding [one source] provided preliminary simulation results and analysis on lossless DCI compression [one source] provided preliminary simulation results and analysis on early contention resolution in RACH(Ofinno), [one source] provided preliminary simulation results and analysis on sensing based RAN digital twin construction with NW-side AI/ML model (Huawei), [one source] provided preliminary simulation results and analysis on AI/ML-enabled RAN digital twin with distributed model (Huawei), [one source] provided preliminary simulation results and analysis on AI/ML-enabled RAN digital twin with distributed model (Huawei), [one source] provided preliminary simulation results and analysis on AI/ML based SRS power imbalance compensation (Huawei), [one source] provided preliminary simulation results and analysis on site specific learning for AI/ML and RAN digital twin (Deepsig). Sub-use case; Sub-Case A: sensing based RAN digital twin construction with NW-side AI/ML model. Reported Companies; (1) Huawei. Model input; Point cloud sensed by the BS with mono-static sensing and sensed/reported by UEs with bi-static sensing. Model output; 3D point cloud representing the static environment. Label; Ground truth point cloud. Training types assumption; Offline training. KPI; Sensing accuracy metric: root mean square error (RMSE) of point cloud. RMSE=1ni=1nxi-xi2 is the square root of the average of the squared errors between each sensed point (xi in forms of coordinates) and ground truth point (xi in forms of coordinates) in the point cloud including n points with {x, y, z} dimensions.. Benchmark; BS side mono-static sensing only to construction RAN digital twin. Model location for inference; NW-side model. Collaboration/interaction between UE and NW; As NW-sided model in NR. Potential spec impact; Specify the type/format of bi-static sensing results reported from UE and the corresponding UL signaling.. Sub-use case; Sub-case B: AI/ML-enabled RAN digital twin with distributed model. Reported Companies; (1) Huawei. Model input; UE-part models: local sparse point cloud NW-part model: latent space information from multiple UEs. Model output; UE-part models: compressed latent space information NW-part model: global point cloud. Label; Ground truth point cloud. Training types assumption; Offline training (adopted in simulation) Online finetuning (can be optionally considered). KPI; 1) Overhead metric: Feedback bits per point 2) Sensing accuracy metric: intersection-over-union (IoU), edge detection probability. Benchmark; 1) Single UE sensing (to justify sensing accuracy metric of using distributed model). 2) Raw data transmission (to justify overhead metric of using distributed model).. Model location for inference; Distributed model: a NW-side model paired with multiple UE-side models.. Collaboration/interaction between UE and NW; Similar as two-sided model: UE reporting of compressed sensing results for inference. Inter-vendor training collaboration between NW side and UE side.. Potential spec impact; 1) Data collection and management procedure 2) Sensing results reported from UE in forms of compressed latent message 3) Inter-vendor training collaboration 4) Online finetuning related impact. Sub-use case; Sub-case C: AI/ML based SRS power imbalance compensation. Reported Companies; (1) Huawei. Model input; UL measured channel matrix from SRS with IL imbalance. Model output; DL channel matrix with IL compensated. Label; UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement. Training types assumption; offline training. KPI; SGCS. Benchmark; 1) SRS without IL imbalance; 2) non-AI based SRS IL imbalance compensation. Model location for inference; NW-sided model. Collaboration/interaction between UE and NW; UE reports for the inference, data collection, and monitoring of NW-side model.. Potential spec impact; 1. Inference: UE reporting on the IL range for ensuring generalization 2. Data collection/monitoring: UE reporting of channel matrix with DL CSI-RS measurement. |

#### (3rd round)Proposed observation 2.12-1:

Observation xxxx

Table M -1

| Use Case | Prior-Information-Aided DCI Decoding | Lossless DCI Compression | UL closed-loop power control with an NW-sided AI/ML model, where the model predicts the optimal power adjustment (or TPC command index) for the UE. | Pathloss prediction in the spatial, temporal, and/or frequency domain, to use the predicted pathloss in UL(PUSCH/PUCCH/PRACH/SRS) power control. |
| --- | --- | --- | --- | --- |
| Reported companies | (1)CMCC | (1)CMCC | (1) Nokia | (1) Nokia |
| Model input | LLR after demodulation at current transmission, and historical AI/ML based predicted LLR | Historical DCI payload | UL SINR measurement, UE Tx power estimate (derived from Pcmax, P0, PL alpha, pathloss measurement), and PUSCH allocation size | L1-RSRP measurements from a sub-set/set of RSs/beams (Set B). input can consider history of measurements |
| Model output | Decoded DCI payloads, and predicted LLR for next transmission | Predicted DCI payload | Predicted TPC command index | Predicted pathloss value(s) (or predicted L1-RSRP(s)) for a set of RSs/beams (Set A). output can consider future instances |
| Label | DCI payload sequences | DCI payload sequences | Optimal TPC command index (offline learning) label-free (online learning) | Pathloss value(s) (or L1-RSRP(s)) for a set of RSs/beams (Set A) |
| Training types | Offline training at the UE side | Offline training at the NW side, and model delivery to UE side | Offline and Online learning | Offline training |
| KPI | BLER performance | BER and sample-level prediction accuracy; DCI overhead reduction | UL throughput. | Pathloss prediction accuracy, throughput, RS overhead reduction, Complexity. |
| Benchmark | - Traditional DCI decoder | - Traditional DCI design | 1. UL Power control with optimized OLPC parameters - 2. UL Power control with optimized OLPC parameters and possibly legacy CLPC algorithms (with 5G TPC tables). | Pathloss estimation based on Set A - Pathloss estimation based on Set B |
| Model location for inference | UE-sided model | UE-sided model + NW-sided model | NW-sided model | UE-sided model NW-sided model |
| Collaboration/interaction between UE and NW | No collaboration | Model transfer from NW to UE | None | As UE-sided or NW-sided mode in nRl |
| Potential specification impact | 1. Signalling/configuration design for prior-information-aided DCI decoder. 2. Signalling/ procedure related to LCM for UE-sided model | 1. Signalling/configuration design for Lossless DCI Compression. 2. Signalling/ procedure related to LCM including model transfer | Configurability of the values in TPC command tables or an extended TPC command table (compared to NR). | 1. Pathloss prediction related signalling/procedure 2. Signalling/ procedure related to LCM for UE-sided or NW-sided model 3. RAN4 performance requirements and test cases, including defining new requirements related to pathloss reference signal (PL-RS) measurement and activation delays of TCI state(s). |

Table M-2

| Sub-use case | Sensing based RAN digital twin construction with NW-side AI/ML model | AI/ML-enabled RAN digital twin with distributed model | AI/ML based SRS power imbalance compensation |
| --- | --- | --- | --- |
| Reported Companies | (1) Huawei | (1) Huawei | (1) Huawei |
| Model input | Point cloud sensed by the BS with mono-static sensing and sensed/reported by UEs with bi-static sensing | UE-part models: local sparse point cloud NW-part model: latent space information from multiple UEs | UL measured channel matrix from SRS with IL imbalance |
| Model output | 3D point cloud representing the static environment | UE-part models: compressed latent space information NW-part model: global point cloud | DL channel matrix with IL compensated |
| Label | Ground truth point cloud | Ground truth point cloud | UL SRS measurement without IL (assuming it is compensated by UE at certain conditions) or DL CSI-RS measurement |
| Training types assumption | Offline training | Offline training (adopted in simulation) Online finetuning (can be optionally considered) | offline training |
| KPI | Sensing accuracy metric: root mean square error (RMSE) of point cloud. RMSE=1ni=1nxi-xi2 is the square root of the average of the squared errors between each sensed point (xi in forms of coordinates) and ground truth point (xi in forms of coordinates) in the point cloud including n points with {x, y, z} dimensions. | 1. Overhead metric: Feedback bits per point 2. Sensing accuracy metric: intersection-over-union (IoU), edge detection probability | SGCS |
| Benchmark | BS side mono-static sensing only to construction RAN digital twin | 1. Single UE sensing (to justify sensing accuracy metric of using distributed model). 2. Raw data transmission (to justify overhead metric of using distributed model). | 1. SRS without IL imbalance; 2. non-AI based SRS IL imbalance compensation |
| Model location for inference | NW-side model | Distributed model: a NW-side model paired with multiple UE-side models. | NW-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR | Similar to two-sided model: UE reporting of compressed sensing results for inference. Inter-vendor training collaboration between NW side and UE side. | Similar to NW-sided model in NR |
| Potential spec impact | 1. Signaling/procedure related to bi-static sensing results reported from UE 2. Signalling/procedure related to LCM for NW-sided model | 1. Sensing results reported from UE in forms of compressed latent message 2. Signalling/procedure related to LCM for two-sided model including inter-vendor collaboration | 1. Inference: UE reporting on the IL range for ensuring generalization 2. Signalling/procedure related to LCM for NW-sided model |

| Company | Support or not | Comment |
| --- | --- | --- |
| FL |  | Proponent companies, please provide check my updates |

## Overall

#### Observation 2.13(new):

For 6GR AI/ML use cases identification/categorization,

[xx sources] proposed to consider NR AI/ML for beam management as 6GR use case for study

- Supported by: {Spreadtrum, UNISOC}, Transsion, ETRI, KT, VDF(energy saving),Rakuten, NEC, LGE, Ofinno, Google, Deepsig?, {CATT, CICTCI}, Fujitsu, Qualcomm, Nokia, ZTE, xiaomi, CEWiT, DoCoMo, Lenovo, Samsung, CMCC ….

[xxx sources] proposed to consider NR AI/ML for CSI prediction as 6GR use case for study

- Supported: NTU, Transsion, ATT, ETRI, NEC, KT,SKT, Hanbat National University, Sony, Panasonic, google, NAVIDA,  Ericsson, BJTU, Samsung, MediaTek, LGE, vivo, CMCC….

[xxx sources] proposed to consider NR AI/ML for CSI compression as 6GR use case for study

- Supported: TCL, VDF, Rakuten, Ofinno, CMCC , Huawei/HiSilicon….

| Company | Comment |
| --- | --- |
| FL | Please add your name |
| CMCC | We support 5G-A AI/ML based BM, CSI prediction and CSI compression for 6GR use case for study and added our company name above. |
| Huawei, HiSilicon | Add Huawei/HiSilicon to CSI compression |
| MediaTek | We are not sure of the implication of this proposal. The meaning of “consider NR AI/ML for xxx as 6GR use case for study” is not clear. If the intention is to study the NR AI/ML use cases with (potentially) new assumptions/baseline resulting from the 6GR study in other agendas, then we would like to propose a general wording as follows: “While NR use cases are not intended to be evaluated further for the purpose of understanding their feasibility, it does not preclude underlying 6G design changes compared to NR and it is understood that such underlying enhancements may warrant updates to the past AI/ML modelling/evaluations/assumptions for the NR use cases at a later stage of the 6G AI/ML study.” |

# Framework and evaluation

## Evaluation and KPIs

Several companies discussed aspect on EVM and KPIs. Several companies proposed for comprehensive evaluation of AI/ML use cases by considering KPIs including system performance, system and model complexity, inter-vendor collaboration complexity, power consumption. In addition to intermediate and system KPIs that were adopted in 5G NR, companies proposed new KPIs such power consumption and inference latency to be considered in 6GR.

#### Proposal 3.1 :

For evaluation of AI/ML use cases in 6GR, consider

- Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead)

  - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training

- AI/ML Model related metrics, including model/computational complexity, inference latency, training latency (when applicable),

  - FFS on whether/how to measure power consumption

- Inter-vendor collaboration when applicable

- Generalization performance under a wide range of conditions

  - FFS on whether and how to consider realistic deployment scenarios

Note: Detailed metrics to be discussed per use case.

| Company | Comment |
| --- | --- |
| FL | Leftover from last meeting. The red part may need to explained by proponent companies |
| Lenovo | Model storage requirement should also be considered in the second bullet. And just a wording suggestion: - Inter-vendor collaboration complexity analysis when applicable |
| NTT DOCOMO | We suggest a further clarification for the Note in the last line, Note: Which types of KPI (intermediate, link-level, system-level) will be adopted and detailed metrics to be discussed per use case. |
| Futurewei | In the case that the model requires online fine-tuning, the overhead should also include the fine-tuning aspects. We suggest the following modification in the sub-bullet of the first main bullet: - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training and fine-tuning (if any). |
| Huawei, HiSilicon | 1, the system level KPI should also consider the metric to evaluate the efficiency of serving AI/ML traffic type (e.g., tokens) For evaluation of AI/ML use cases in 6GR, consider - Performance related metrics, including intermediate (model) performance KPIs, link level KPIs (e.g., BLER) and system level KPIs (e.g., throughput, overhead) - Where the overhead including the overhead associated with inferencing, performance monitoring and (re)training - FFS performance metric for AI/ML services such as token communication |
| CATT, CICTCI | (1) Training latency is difficult to evaluate as it is highly implementation or even hardware dependent. And it does not matter for most use cases unless they rely on fast online training. Suggest to remove it. (2) For two-sided model use cases (e.g. CSI compression related), we think ‘scalability’ should also be looked into, just the same as CSI compression study in NR. |
| SPRD | For the last bullet, what is the meaning of “a wide range of conditions”? |
| Xiaomi | - For the AI/ML related metrics, it is unclear how to measure the inter-vendor collaboration - For the power consumption, with the increase of AI use cases, the power consumption will be increased accordingly. From that sense, guranteeing the power efficiency is critical to the user throughput. Thus, power consumption should be one criteria for the use case evaluation. As for the how to measure the power consumption, we don’t think it could be simply reflected by the model complexity, it also affected by the inference frequency. Even when there is no task, certain base electronic current is also needed to maintain the AI engine on, that would also cause some energy consumption - As for the training latency, we share similar view with CATT |
| Nokia | Direction is fine. Better to discuss in the offline session for wording changes. |
| SK Telecom | For the third bullet, the wording needs to be improved like: “Complexity for inter-vendor collaboration when applicable”. For the last bullet, it is a bit unclear what it implies. What should be considered for generalized performance? |
| CMCC | If model (re)training or model fine-tuning is adopted, then model can be changed when UE moved into different areas. So, generalization performance is not needed under a wide range of conditions, maybe generalization under one or several cells is enough. - Generalization performance under a wide range of conditions |
| Tejas | Fine with the direction |
| IIT Kanpur | We agree with Futurewei’s comment regarding the inclusion of overhead aspects during fine-tuning, should the model require fine-tuning. |
| OPPO | Fine with this direction. But for some details, such as inference latency or training latency, we don’t really know how to evaluate it. Btw, training latency in Rel-18 study, it can be considered as not time critical, so why would we study it again for 6GR. |
| Ericsson | Regarding performance related metrics bullet, suggest adding a sub-bullet: - Conclusions shall be drawn based on final performance metric (e.g., throughput) instead of intermediate KPIs In addition, please add a bullet - RAN4 interoperability testing feasibility and complexity |
| InterDigital | We support the sub-bullet in red. The OH considering inferencing, training/retraining, and performance monitoring are critical aspects that determine the overall benefits of specifying an AI/ML-based solution for a functionality. In addition, we also would like to second the suggestion from Ericsson on consideration of final performance metrics. |

## LCM framework

Many companies proposed enhancement on NR’s LCM, encompassing aspects such as data and model management, including model transfer, applicability of the associated ID, support for localized models, advanced training methods, e.g., online and federated learning, meta-learning for handling network-side additional conditions. Moreover, a number of companies proposed to 5G NR’s LCM framework including functionality-based LCM as a starting point. Enhancement on AI/ML processing unit framework was proposed by a few companies, e.g., 1 company (Samsung) proposed to introduce AI/ML memory unit (MU) on the concurrently activated AI/ML feature/models

#### Proposal 3.2:

For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point.

- Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include:

  - Data and model management

  - Handling of additional conditions

  - Enablers for continuous (online) on-device model training/finetuning

  - framework for AI/ML processing and memory

[Note: the study strived to provide a unified LCM across use cases]

| Company | Comment |
| --- | --- |
| FL | Some modification according to Ericsson’s proposal |
| Lenovo | Generally fine with the proposal. One suggestion is to list some aspects, e.g., data collection for online training, model validation, under the third subbullet. |
| NTT DOCOMO | We are generally fine with directions in the main bullet, but feel the examples are not clearly defined currently. We also suggest keeping the last note, which should be an essential enhancement of 6GR AI/ML compared with 5GA. |
| Futurewei | We are ok with the proposal in general with a small comment regarding online model fine-tuning. We believe this should be applicable on either UE (side) or NW, so we suggest removing the wording of “on-device”: - Enablers for continuous (online) on-device model training/finetuning |
| Fujitsu | If the examples to study are listed in the proposal, then performance monitoring should be included which is important part of LCM. The following change is suggested. - Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include: - Data and model management - Performance monitoring - Handling of additional conditions - Enablers for continuous (online) on-device model training/finetuning - framework for AI/ML processing and memory |
| CATT, CICTCI | We think ‘continuity of UE-side AI/ML features across cells’ should be studied. This is to guarantee the UE-side AI/ML model/functionality can be smoothly functioning/switching among different cells around handover time. |
| SPRD | For the last example, it is necessary to clarify what components are included in the framework for AI/ML processing. |
| Huawei, HiSilicon | We do not think RAN1 can determine the 6GR AI/ML LCM framework on behalf of RAN2 – notice that TS38.331 spec is RAN2 spec, they are more appropriated to determine whether to reuse 5G or make a new architecture. For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications (e.g., TS38.331) as a starting point. |
| Ofinno | Generally fine. On model management, is it correct understanding to include performance monitoring aspects also? |
| Xiaomi | - For the LCM Framework, we share similar view with Huawei. The determination of LCM framework is RAN2 work. 2. For the third sub bullet, we share similar view with Futurewei, the “on-device” should be removed 3. For the Note part, we suggest to remove the bracket, it is quite important principle for the LCM framework |
| Nokia | Ok with the proposal with Futurewei update. Mentioning of e.g., TS38.331 is not needed. |
| SK Telecom | We think defining the unified AI/ML framework across use cases is very important in order to avoid any excessive framework, and thus prefer to keep the last note. Regarding the examples, it seems still necessary to be further discussed, and maybe the wording would be better to be open a bit more. |
| CMCC | We are generally supportive of this proposal. But if we consider two-sided model, 5G Rel-20 specifications are also needed as a starting point, and the 5G NR AI/ML LCM framework is not only described in TS 38.331, but also TS 38.214, TS 38.212, etc. For 6G LCM framework for AI/ML for air interface, consider the 5G NR AI/ML LCM framework as adopted in 5G Rel-19/20 specifications (e.g., TS38.331) as a starting point. |
| LG Electronics. | We are generally fine, but we may need to further study on efficient and faster model switching by considering fall back operation to non-AI and allowing performance monitoring for inactive model. So, we suggest following: - Study the necessity of potential enhancements for LCM, and if justified, the enhancement details. The examples to study include: - Data and model management - Handling of additional conditions - Enablers for continuous (online) on-device model training/finetuning - Faster and more efficient model/functionality switching (e.g. via fall back operation to non-AI, performance monitoring for inactive model) - framework for AI/ML processing and memory |
| TCL | Agree |
| Tejas | We are supportive to this proposal |
| IIT Kanpur | We are fine with the proposal. |
| NEC | “5G NR AI/ML LCM framework as adopted in 5G Rel-19 specifications” is use case dependent and is very CSI framework dependent. It may not fit 6G use cases especially those non-MIMO use cases. We think it should be removed for this early disucssion. |
| vivo | @Futurewei, online model training/finetuning is different from on device. The major difference is the latency required. For online model training, it may require stringent timeline but on device training/finetuning may not necessarily have stringent timeline. If you want to add NW side online model training/finetuning, it can be explicitly added. But on device finetuning should not be removed. |
| OPPO | The LCM of 5G-A supports CSI use case and positioning. If we take the CSI report configured in TS 38.331 as a starting point, it may not be applicable to other non-CSI use cases, such as DMRS OH reduction. At the beginning of 6G, it would be a good chance to strive for a unified LCM framework. |
| InterDigital | We support the suggested change in the first bullet from Huawei. In fact, we suggest to simplify the first and second bullets as: - For 6G LCM framework for AI/ML for air interface, study potential enhancements relative to 5G NR AI/ML LCM framework. We are fine to keep the examples (including performance monitoring bullet suggested by Fujitsu) but do not think they are critical since they encompass all of LCM aspects. |
| Samsung | Support |
| MediaTek | We wish to add a bullet point: “leveraging cross-use case similarities” e.g., CSI-related use cases could have same model input data (e.g., CSI-RS measurements) and some parts of the model architecture could also be reused across CSI-related use cases. |

## Data collection framework

A number of companies discussed data collection framework in their contribution. The following summarizes the discussion points

- Enhancement in the data collection framework for future-proof and unified (across working groups) design.

- Scope and restrictions, e.g., whether to restrict data collection to use cases or to support generic purpose data collection.

- Whether to introduces a new AI/ML data management plane

Some of the proposals may not be under the realm of RAN1. However, RAN1 may identify requirements which may consequently suggest enhancement in the relevant working group. With this in mind, the RAN1 study may focus in identifying requirements that may lead to data collection framework enhancement.

#### Conclusion 3.3:

For AI/ML study in 6GR, RAN1 to study on the content and format for data collection for each use case.

| Company | Comment |
| --- | --- |
| Lenovo | It should be clarified that the data collection in this conclusion at least includes the date collection for model training (at least for offline training). Both NW-side and UE-side data collection should be considered. |
| NTT DOCOMO | Support. |
| Fujitsu | Is this proposal applicable to offline training, online training and finetuning? |
| CATT, CICTCI | Support. But another question is that, does RAN1 need to analyze data collection framework in physical layer in high-level? For example, should RAN1 analyze the pros and cons if CSI related AI/ML data collection is imbedded in CSI reporting framework in physical layer? |
| SPRD | OK |
| Nokia | Let RAN2 discuss this. |
| SK Telecom | Support in principle for studying this but not sure if this would be the scope of RAN1. |
| CMCC | Support. When we discuss the requirements of data collection, both offline training, online training can be considered subject to each use case. |
| LG Electronics | Agree in principle, but what is the motivation of this conclusion. Anyway, when we discuss each use case, this part can be also studied. |
| TCL | Agree |
| vivo | Support |
| OPPO | Open to discuss. At least the content of data collection of RAN1 studied use cases should fall into RAN1 territory. |
| Ericsson | Support in general. Suggesting updating “each use case” to “each use case led by RAN1” |
| Samsung | Support |

# Proposal for online

# Appendix

## RAN 1 #122

Agreement

For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following:

- Definition of each (sub-)use case, including at least AI/ML model input/output

- The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results

- Assumption on training types, e.g.,

  - offline training, online training/finetuning

  - Label construction (if applicable), including whether/how to obtain label data for model training

- Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model

- Collaboration/interaction between UE and NW, e.g.,

no collaboration/interaction

UE/Network collaboration targeting at separate or joint ML operation

- High level potential specification impact

## RAN1 #122bis

Observation

For 6GR AI/ML use cases identification/categorization, [24 sources] provided preliminary simulation results and analysis on low overhead CSI-RS or CSI prediction with AI/ML.

- [23 sources] provided preliminary simulation results and analysis on frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table A.

- [6 sources] provided preliminary simulation results (or by citing to NR study for CSI time domain prediction) and analysis on CSI time domain prediction with AI/ML wherein [3 sources] assumed Rel-19 CSI prediction while [3 sources] assumed differently. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- [4 sources] provided preliminary simulation results and analysis on CSI prediction cross carrier/band/frequency block with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI/training type) and initial analysis can be found in Table B.

- [2 sources] provided preliminary simulation results and analysis on CSI prediction across analog beams with AI/ML. Detailed evaluation assumptions (model input/output/label/benchmark/KPI training type) and initial analysis can be found in Table B.

- Besides, one source provided preliminary simulation results and analysis on, Tokenized CSI prediction (Huawei), and time domain CSI prediction combining CSI-RS and DMRS measurements (MediaTek).

Note: whether/how to capture the observation in the TR is a separate discussion.

Table A

| Sub-use case | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML |
| --- | --- |
| Reported companies | (23) Ericsson1, ZTE2, vivo3, OPPO, Xiaomi, CMCC, Huawei4, Samsung, Fujitsu, Apple, Qualcomm5, Kyocera6, Nokia7, {Spreadtrum, UNISOC}8, Interdigital9, Lenovo, LGE10, DoCoMo11, CEWiT, IITM, IIT Kanpur, Tejas, {CATT, CICTCI}12 |
| Model input (for decoder of 2-sided model, when applicable) | 1. Measurement of channel with sparse/low overhead CSI-RS (majority) 1a. Additional long-term multi-path power/angle/delay info information as assistance information4 2. Reported CSI for NW-sided model3,4,5 |
| Model output (for decoder of 2-sided model, when applicable) | 1. Full channel matrix (majority) 2. Eigenvector 3 for NW-sided model 3. Channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Label | 1. Estimated/ideal channel matrix based on full CSI-RS density(majority) 2. Ideal precoding matrix with full dimension3 3. Estimated/ideal channel matrix/eigenvector with different/targeted antenna on/off patterns3, 12 |
| Training types | Offline training(majority) Online finetuning for UE-sided model (for NW-sided model + UE sided model without training collaboration)4 |
| KPI | NMSE, SGCS, throughput, ratio of CSI-RS overhead |
| Benchmark | 1. non-AI based on full CSI-RS 2. non-AI based on sparse CSI-RS |
| Model location for inference | UE-sided model NW-sided model2,3, 4,5,6 Two-sided model3 NW-sided model + UE-sided model without training collaboration4 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR As NW-sided model in NR As two-sided model for CSI compression4 in NR |
| Potential spec impact | 1.Sparse CSI-RS design and corresponding feedback (especially for NW-sided model) 2. Signalling/ procedure related to LCM 3. Inter-vendor collaboration for two-sided model, when applicable |

Table B

| Sub-use case | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams |
| --- | --- | --- | --- |
| Reported Companies | (6) Ericsson2, BJTU, Samsung, MediaTek3, LGE, vivo1 | (4) Samsung, Apple, LGE, DoCoMo1 | (2) Samsung, vivo1 |
| Model input | 1. Channel matrix over K CSI-RS occasions 2. Measurements of interference over K CSI-RS occasions1 3. Channel matrix over K CSI-RS occasions with 20ms periodicity3 4 Channel matrix with one P CSI-RS with 20ms periodicity and K-1 AP CSI-RS2 | Channel matrix of carrier/band/frequency block A | Channel matrix of Set B of beams |
| Model output | 1. Channel matrix of future instances 2. Interference in future instances1 | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams |
| Label | Measurement in future time occasions. | Channel matrix of carrier/band/frequency block B | Channel matrix of Set A of beams |
| Training types assumption | offline training | offline training | offline training |
| KPI | NMSE, SGCS, throughput, [ratio of CSI-RS overhead] | SGCS, NMSE, throughput, ratio of CSI-RS overhead | SGCS, NMSE, throughput, ratio of CSI-RS overhead |
| Benchmark |  | 1.Ground truth of target frequency block 2. Sample and hold | Ground truth of Set A of beams |
| Model location for inference | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 | UE-sided model NW-sided model1 Two-sided model1 |
| Collaboration/interaction between UE and NW | As UE-sided model in NR in NR As NW-sided model in NR1 | As UE-sided model in NR in NR | As UE-sided model in NR in NR |
| Potential spec impact | 1. As AI based CSI prediction in NR 2. Reporting content, signalling and procedure for LCM for extension cases1 | 1. Cross carrier/frequency switching procedure enhancement based on predicted CSI 2. signalling/ procedure related to LCM | 1.CSI-RS configuration for predicted beams 2. signalling/ procedure related to LCM |

Table for single company will be added.

Observation

For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver.

- [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver.

- [11 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver.

- [5 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver.

- Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C.

Note: whether/how to capture the observation in the TR is a separate discussion.

Table for detailed assumptions of each category will be added.

Observation

For 6GR AI/ML use cases identification/categorization, [13 sources] provided preliminary simulation results and analysis on CSI compression and feedback.

- [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source and channel coding (JSCC)

- [xx sources] provided preliminary simulation results and analysis on CSI compression with joint source, channel coding and modulation (JSCM)

- [2 sources] provided preliminary simulation results and analysis on CSI feedback with downloadable basis/codebook.

- [3 sources] provided preliminary simulation results (or cite to NR AI/ML for CSI compression simulation results) and analysis on CSI reconstruction with CSI feedback with SRS (assuming separate source and channel coding).

- [1 source] provided preliminary simulation results and analysis on CSI feedback without SRS.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in in Table D.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on (de)modulation.

- [5 sources] provided preliminary simulation results and analysis on modulation constellation design with the help of AI, and with non-AI or AI receiver.

- [3 sources] provided preliminary simulation results and analysis on AI-based modulation and precoding with two-sided model.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table F.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver.

- [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver.

- [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter.

- Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on low overhead SRS with AI/ML

[1 source] provided preliminary simulation results and initial analysis on low PAPR SRS sequence design with help of AI/ML

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and analysis in Table I.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI-enabled UL precoder indication with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table H.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [3 sources] provided preliminary simulation results and analysis on AI/ML based waveform for PAPR reduction with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table J.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on AI/ML based HARQ-ACK feedback with detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table K.

Note: whether/how to capture the observation in the TR is a separate discussion.

Observation

For 6GR AI/ML related service, for 6GR AI/ML use cases identification/categorization, [2 sources] provided preliminary simulation results and analysis on improved scheduling/HARQ for token traffic

Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis in Table L.

Note: whether/how to capture the observation in the TR is a separate discussion.

## Contact information

| Company | Delegate(s) | Email address |
| --- | --- | --- |
| Moderator | Feifei Sun | Feifei.sun@samsung.com |
| Google | Yushu Zhang | yushuzhang@google.com |
| Ofinno | Jaehoon Chung | jchung@ofinno.com |
| Sharp | Yinan Zhao | Yinan.zhao@cn.sharp-world.com |
| Fainity | Chia-Hung Lin | chlin@fainnov.com |
| Lenovo | Bingchao Liu Vahid Pourahmadi Srinivas Kothapalli | liubc2@lenovo.com vpourahmadi@lenovo.com vkothapalli@lenovo.com |
| CATT | Yongqiang FEI | feiyongqiang@catt.cn |
| SK Telecom | Hyunho Lee | hho.lee@sk.com |
| CMCC | Xiaodong Shen Yi Zheng Yuhua Cao | shenxiaodong@chinamobile.com zhengyi@chinamobile.com caoyuhua@chinamobile.com |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |
| ZTE | Xingguang, Wenfeng, Yunqi | wei.xingguang@zte.com.cn liu.wenfeng@zte.com.cn sun.yunqi@zte.com.cn |
| Huawei, HiSilicon | Yuan Li | liyuan3@huawei.com |
| Ericsson | Yufei Blankenship Jingya Li Siva Muruganathan | yufei.blankenship@ericsson.com jingya.li@ericsson.com siva.muruganathan@ericsson.com |
| NEC | Peng Guan Pravjyot Deogun Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM y-jiang_ct@nec.com |
| Panasonic | Henry Tran Tetsuya Yamamoto Hidetoshi Suzuki | xuantuong.tran@sg.panasonic.com yamamoto.tetsuya001@jp.panasonic.com suzuki.hidetoshi@jp.panasonic.com |
| NTT DOCOMO | Kosuke Shima Wang Xin Zhang Zhibo | kousuke.shima.nr@nttdocomo.com wangx@docomolabs-beijing.com.cn zhangzb@docomolabs-beijing.com.cn |
| Xiaomi | Qin MU | muqin@xiaomi.com |
| Qualcomm | Hamed Pezeshki Taesang Yoo | hamedp@qti.qualcomm.com taesangy@qti.qualcomm.com |
| OPPO | Jeffrey Cao Wendong Liu | caojianfei@oppo.com liuwendong1@oppo.com |
| ETRI | Youngjoon Yoon Minhyun Kim | youngjoon.yoon@etri.re.kr minhyun.kim@etri.re.kr |
| Spreadtrum | Shijia shao Zhe yu Mimi chen | Shijia.shao@unisoc.com Zhe.yu@unisoc.com Mimi.chen@unisoc.com |
| CEWiT | Dhivagar Baskaran Shiv Shankar | dhivagar.b@cewit.org.in shivshankar@cewit.org.in |
| TCL | Pu Yuan Tianqi Wu | pu.yuan@tcl.com tianqi1.wu@tcl.com |
| IITK | Chethan Ranganatha | chethanr@iitk.ac.in |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |
| IIT Madras | Anil Kumar Yerrapragada Jeeva Keshav S | anilkumar@5gtbiitm.in jeevak@5gtbiitm.in |
| LGE | Haewook Park | haewook.park@lge.com |
| InterDigital | Fumihiro Hasegawa | fumihiro.hasegawa@interdigital.com |
| AT&T | Salam Akoum | Salam.akoum@att.com |
| Rakuten | Awn Muhammad Abhinav Kishore Mattewada | awn.muhammad@rakuten.com abhinavkis.mattewada@rakuten.com |
| Samsung | Ameha Abebe Zhe Chen Qi Xiong | amehat.abebe@samsung.com tom.chenzhe@samsung.com q1005.xiong@samsung.com |
| Orange | Raphael Visoz | raphael.visoz@orange.com |
| Vivo | Peng Sun | sunpeng@vivo.com |
| MediaTek | Reubengeorge Stephen | Reubengeorge.Stephen@mediatek.com |
| Deepsig | James Lansford | jameslansford@deepsig.io |
| Lekha | Ipshita Panda | ipshita.panda@Lekhawireless.com Sashiganth M sashiganth.m@Lekhawireless.com Sreenath Ramanath sreenath@lekhawireless.com |
| Apple | Huaning Niu Weidong Yang | huaning_niu@apple.com wyang23@apple.com |
| Kyocera | Omar Sleem | omar.sleem@kyocera.com |

# Reference

- R1-2506742	Discussion on AI/ML in 6GR interface	FUTUREWEI

- R1-2506757	Views on AI/ML Operation and Use Cases for 6G Radio Air Interface	Nokia

- R1-2506762	AI/ML Use Cases for 6GR Air Interface	Ericsson

- R1-2506778	Discussion on AI-based Smart Radio for 6G Air Interface	ZTE Corporation, Sanechips

- R1-2506780	Discussion on AI/ML-driven use cases for 6GR	BJTU

- R1-2506784	Discussion on AI/ML in 6GR air interface	TCL

- R1-2506820	Discussion on AIML in 6GR interface	Spreadtrum, UNISOC

- R1-2506821	AI/ML for 6G Air Interface	InterDigital, Inc.

- R1-2506904	Discussion on AI/ML in 6GR interface	vivo

- R1-2506951	AI/ML in 6GR interface	Kyocera

- R1-2506995	Discussion on AI/ML in 6GR interface	Xiaomi

- R1-2507020	Discussion on AI/ML in 6GR interface	CMCC

- R1-2507029	Discussion on AI/ML in 6GR interface	Lekha Wireless Solutions

- R1-2507064	Views on AI/ML in 6GR air interface	Huawei, HiSilicon

- R1-2507107	Views on AI/ML in 6GR interface	CATT, CICTCI

- R1-2507182	AIML use cases for 6GR air interface	OPPO

- R1-2507203	AI/ML in 6GR interface	Tejas Network Limited

- R1-2507259	AI/ML Use cases and framework for 6GR	Samsung

- R1-2507288	Discussion on AI/ML in 6GR	Fujitsu

- R1-2507305	Discussion on AIML in 6GR interface	NEC

- R1-2507377	Discussion on AI/ML in 6GR interface	Panasonic

- R1-2507378	AI/ML Use Cases for 6G	NTU

- R1-2507400	Discussion on AI/ML in 6GR interface	LG Electronics

- R1-2507409	Views on AI/ML in 6GR air interface	SK Telecom

- R1-2507433	AI and ML in 6GR air interface	NVIDIA

- R1-2507471	Views on AI/ML in 6GR interface	Ofinno

- R1-2507488	Discussion on AI/ML Use-cases in 6GR	Lenovo

- R1-2507492	Use cases for AI/ML in 6GR interface	KT Corp.

- R1-2507512	Discussion on AI/ML in 6GR interface	ETRI

- R1-2507522	AI/ML in 6GR Air Interface	Google

- R1-2507525	New use cases for AI/ML in 6GR interface	Pengcheng Laboratory, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507547	Discussion on AI/ML in 6GR interface	Continental Automotive

- R1-2507601	Discussion on the potential AI/ML use cases for 6GR interface	Sony

- R1-2507604	Discussion on AI/ML for 6GR interface	Ruijie Networks Co. Ltd

- R1-2507605	Discussion on AI/ML Use Cases for 6GR	DeepSig Inc

- R1-2507613	AI/ML Framework and Use Cases for 6GR Air Interface	MediaTek Inc.

- R1-2507633	Discussion on AI/ML in 6GR interface	Transsion Holdings

- R1-2507683	Discussion on 6G AI/ML use cases	Apple

- R1-2507727	AI/ML in 6GR air interface	Qualcomm Incorporated

- R1-2507752	AI/ML use cases and framework for 6GR Air Interface	AT&T

- R1-2507776	Discussions on AI/ML in 6GR interface	Sharp

- R1-2507779	Discussion on AI/ML in 6GR	Shanghai Jiao Tong University, Toyota

- R1-2507821	Discussion on AI/ML for 6GR air interface	NTT DOCOMO, INC.

- R1-2507826	AI/ML in 6GR air interface	KAIST

- R1-2507827	Discussion on AI/ML-enabled use cases for 6GR	BUPT, ZGC Institute of Ubiquitous-X Innovation and Application

- R1-2507842	Discussion on AI/ML in 6GR air interface	Hanbat National University

- R1-2507872	Discussion on AI/ML in 6GR Interface	Indian Institute of Tech (M)

- R1-2507883	On new use cases for AI/ML in 6GR interface	Vodafone, Deutsche Telekom

- R1-2507907	AI/ML in 6GR Interface	CEWiT

- R1-2507910	Discussion on AI/ML in 6GR -Physical Layer	Rakuten Mobile, Inc

- R1-2507940	Discussion on AI/ML in 6GR air interface	IIT Kanpur