3GPP TSG RAN WG1 #122bis									           R1-25xxxxx

Prague, Czech Republic, Oct. 13th-17th 2025

Agenda item:	7

Source:	Moderator (NEC)

Title:              	Summary #2 on discussion for RACH-less HO

Document for:	Discussion and Decision

- Introduction

RAN1 received the LS on RACH-less handover from RAN2 [1] in RAN1#122 meeting.

In this meeting, the submitted discussion papers and draft CRs ([2] from vivo, [3][4] from NEC, [5] [6][7] from Ericsson, and [8] from OPPO) provided corresponding changes on the support of RACH-less HO in NTN, mobile-IAB and TN in TS 38.213 and TS 38.214.

Section 2 is for details of changes. Section 3 is for collection companies’ views.

- Discussion

The identified issues and proposed changes are as follows.

Change 1:

Both [2] from vivo, [4] from NEC and [8] from OPPO think section 22 should be updated to support of RACH-less handover in TN and mobile-IAB. However different solutions are proposed. Details can be found in [2] [4][8].

- Direction a [2][8]: Delete the term “NTN” in the title of clause 22, and add the case where the beam indication is provided via a TCI state to extend the support of RACH-less handover to TN in section 22.2. The corresponding TP#1 as follows:

# 22 PUSCH transmission in RACH-less handover ## 22.2 Dynamic-grant PUSCH transmission If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS configured by a TCI state via tci-StateID are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.

- Direction b [4]: Delete the term “NTN” in the title of clause 22, add the term “in NTN” in title of clause 22.2, and, add the new case where beam indication is provided to support of RACH-less handover in mIAB in section 22.3 or in TN in section 22.4. The corresponding TP#2 as follows:

# 22 PUSCH transmission in RACH-less handover ## 22.2 Dynamic-grant PUSCH transmission in NTN ## 22.3 Dynamic-grant PUSCH transmission in mIAB If tci-StateID is provided in RACH-LessHO, the UE may assume that DM-RS antenna ports associated with the PDCCH receptions for scheduling initial PUSCH transmission are quasi co-located with the SS/PBCH block or the TRS in the TCI state indicated by tci-StateID with respect to average gain and quasi co-location 'typeA' and 'typeD' properties. ## 22.4 Dynamic-grant PUSCH transmission in TN If ssb-Index is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties. If tci-StateID is provided in RACH-LessHO, the UE may assume that DM-RS antenna ports associated with the PDCCH receptions for scheduling initial PUSCH transmission are quasi co-located with the SS/PBCH block or the TRS in the TCI state indicated by tci-StateID with respect to average gain and quasi co-location 'typeA' and 'typeD' properties.

In addition, in [5][6][7] from Ericsson, only changes corresponding to the behaviour of TN UEs and mobile-IAB UEs in Sec. 7.1.1 of TS 38.213 is introduced to limit the specification changes. However, from the moderator perspective, the section 7 is for uplink power control, only the changes in section 7.1.1 cannot support the PUSCH transmission in RACH-less HO of TN UEs and mobile-IAB UEs. The changes in section 22 are needed.

Companies are encouraged to provide comments or views in Section 3 on Q1.

Change 2:

As proposed in [2] from vivo and [8] from OPPO, according to the current spec, downlink pathloss estimate in power control for RACH-less handover is determined only via a RS resource from the SSB index provided by RACH-less HO, which cannot support the case where the pathloss is estimated based on a RS indicated via the TCI state for TN and mobile-IAB. In [5][6][7] from Ericsson, the changes corresponding to the behaviour of TN UEs and mobile-IAB UEs in Sec. 7.1.1 physical uplink shared channel for uplink power control of TS 38.213 is also introduced.

The details of pathloss is estimated based on a RS indicated via the TCI state can be found in [2] [8][6][7].

Companies are encouraged to provide comments or views in Section 3 on Q2.

Change 3:

In [2] from vivo, it proposed that DMRS antenna port for RACH-less handover is determined only via a RS resource from the SSB index provided by RACH-LessHO, which cannot support the case that the DMRS antenna port for RACH-less handover is determined via the TCI state provided by RACH-LessHO for RACH-less handover in TN in TS 38.214. Therefore, the changes for TS 38.214 is provided as follows:

6.2.2 UE DM-RS transmission procedure ============== omitted =============== When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure, - for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213]. - for a configured-grant based PUSCH transmission in RACH-less handover or in RACH-less LTM cell switch, the UE is provided with a set of DM-RS port(s) by rrc-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) or the DL RS configured by a TCI state and a PUSCH occasion and the associated DM-RS resource as described in Clause 22.1 or clause 21.1 of [6, TS 38.213], respectively. ============== omitted ===============

Companies are encouraged to provide comments or views in Section 3 on Q3.

- Collecting companies’ views for round 1

Q1: Please provide your views or suggestions on the changes in section 22 of TS 38.213. In addition, you may provide preference on following directions and corresponding TPs.

- Direction a [2][8]: Delete the term “NTN” in the title of clause 22, and add the description where the beam indication is provided via a TCI state in section 22.2. The corresponding TP#1 is provided in section 2 Change 1.

- Direction b [4]: Delete the term “NTN” in the title of clause 22, add the term “in NTN” in title of clause 22.2, and, add the new section 22.3 and 22.4 to support the RACH-less handover in mIAB or in TN. The corresponding TP#2 is provided in section 2 change 1.

| Company | Which direction you prefer? [a/b] | Comments |
| --- | --- | --- |
| Moderator (NEC) | b | Based on the content of LS as follows, the target beam used by the NTN UEs, mobile IAB UEs and TN UEs are different. So we think that the RAN1 spec should reflect this. - Agreements RAN2#130: - RAN2 confirms that current specification follows this behaviour - For NTN, only SSB index is configured - For Mobile IAB only TCI state index is configured - for TN, SSB index or TCI State can be configured. In addition, if companies think that description of TP#2 is redundant, we suggest that the following TP#3 can also be considered: TP#3: # 22 PUSCH transmission in RACH-less handover ## 22.2 Dynamic-grant PUSCH transmission For RACH-less handover, only the ssb-Index can be configured for NTN UEs, only the tci-StateID can be configured for mobile IAB UEs, while ssb-Index or tci-StateID can be configured for TN UEs. If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS configured by a TCI state via tci-StateID are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.. |
| OPPO | a | The restriction of target beam that can be configured for the NTN UE, the IAB UE, and the TN UE can be captured in TS38.331, so there is no need to capture it in TS28.213. |
| Samsung | a | Share similar view with OPPO. Direction a provides a much cleaner change to the spec. We suggest the following TP for consideration. 22.2 Dynamic-grant PUSCH transmission ---------------------- unchanged parts are omitted ------------------------------ If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS associated with tci-StateID, respectively, are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties. ---------------------- unchanged parts are omitted ------------------------------ |

Q2: Do you agree to update the related description as follows in TS 38.213 to support the pathloss estimate based on a RS indicated via the TCI state. Or you can provide the description that you preferred.

7.1.1 UE behaviour ============== omitted =============== -- PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB, or using the SS/PBCH block the UE acquired the time and frequency synchronization for a secondary cell. - if the UE is provided RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource configured by a TCI state or from an SS/PBCH block with same SS/PBCH block index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync ============== omitted ===============

| Company | Agree or not | Comments |
| --- | --- | --- |
| Moderator (NEC) | Agree | The TCI state may be configured for TN UEs and mobile-IAB UEs in RACH-less HO. It is necessary to update the specification to support the pathloss estimate based on a RS indicated via the TCI state for RACH-less HO. |
| OPPO | Not agree | The QCL relationship of PDCCH reception in controlResourceSetZero can only be determined by SSB index. If the TCI state configures a CSI-RS for pathloss calculation, then the corresponding CORESET which is QCLed with the CSI-RS for PDCCH reception cannot be the CORESET0. So, we prefer the following TP in [8]: ---------------------- unchanged parts are omitted ------------------------------ - if the UE is provided RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index or a CSI-RS with same CSI-RS resource index as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero or commonControlResourceSet provided in ServingCellConfigCommon of ReconfigurationWithSync ---------------------- unchanged parts are omitted ------------------------------ |
| Samsung | Not agree | We agree that specification needs update to reflect pathloss estimate based on a RS indicated via the TCI state. However, we suggest the following TP for consideration. ---------------------- unchanged parts are omitted ------------------------------ - is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index for the active DL BWP, as described in clause 12, of carrier of serving cell - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates • - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB, or using the SS/PBCH block the UE acquired the time and frequency synchronization for a secondary cell. • - if the UE is provided RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index or from pathlossReferenceRS-Id associated with tci-StateId as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero provided in ServingCellConfigCommon of ReconfigurationWithSync ---------------------- unchanged parts are omitted ------------------------------ |

Q3: Do you agree to update the related description as follows in TS 38.214 support the case that the DMRS antenna port for RACH-less handover is determined via the TCI state? Or you can provide the description that you preferred.

6.2.2 UE DM-RS transmission procedure ============== omitted =============== When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure, - for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213]. - for a configured-grant based PUSCH transmission in RACH-less handover or in RACH-less LTM cell switch, the UE is provided with a set of DM-RS port(s) by rrc-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) or the DL RS configured by a TCI state and a PUSCH occasion and the associated DM-RS resource as described in Clause 22.1 or clause 21.1 of [6, TS 38.213], respectively. ============== omitted ===============

| Company | Agree or not | Comments |
| --- | --- | --- |
| Moderator (NEC) | Agree | The TCI state may be configured for TN UEs and mobile-IAB UEs in RACH-less HO. It is necessary to update the specification to cover case that the DMRS antenna port for RACH-less handover is determined via the TCI state provided by RACH-LessHO for RACH-less handover. |
| OPPO | Agree |  |
| Samsung | Not agree | We agree that specification needs update to reflect the DMRS antenna port for RACH-less handover determined via the TCI state. We suggest the following TP for consideration. ---------------------- unchanged parts are omitted ------------------------------ When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure, - for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213]. - for a configured-grant based PUSCH transmission in RACH-less handover or in RACH-less LTM cell switch, the UE is provided with a set of DM-RS port(s) by rrc-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) or the DL RS associated with tci-StateID, and a PUSCH occasion and the associated DM-RS resource as described in Clause 22.1 or clause 21.1 of [6, TS 38.213], respectively. ---------------------- unchanged parts are omitted ------------------------------ |

- Collecting companies’ views for round 2

Q1: Do you agree to update the related description as follows in TS 38.213 to support the RACH-less handover in NTN, mobile IAB and TN. If not, please provide the reason and the description that you preferred.

# 22 PUSCH transmission in RACH-less handover ============== unchanged parts are omitted =============== ## 22.2 Dynamic-grant PUSCH transmission If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS configured by a TCI state indicated by tci-StateID are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.

| Company | Agree or not | Comments |
| --- | --- | --- |
| Vivo3 | yes |  |
| Huawei, HiSilicon | Not agree | We think the RACH-less handover for mIAB and TN should be a separate clause from NTN (not in clause 22). The current version do not reflect the difference on beam indication across different features. In addition, clause 22.1 (configured grant) only applies for FDD as there is only FDD in MSS band. We think mIAB and TN may also be applicable in TDD bands, where the PO validation is different. In a TCI state, two RS can be configured one for QCL type A and the other for QCL type D. the current wording “the DL RS configured by a TCI state” excluding such case. In addition, the wording of “quasi co-location 'typeA' or 'typeD' properties” at the end is also not accurate. QCL D cannot work individually. Usually，we use “quasi co-location 'typeA' and 'typeD' properties when applicable” in LTM and MIMO. |
| Ericsson | Not agree | Proposed change does not accurately capture the RAN2 agreement mentioned in the LS. We do understand that there exist RRC parameter description provides clarifications on the applicability of ssb-Index and tci-StateID with respect to TN, NTN and mobile-IAB. It would be good to clarify these aspects in RAN1 specification at least once based on the RAN2 LS. Based on the current description, it is highly likely that one could misinterpret that ssb-Index or tci-StateID applicable to all operations of TN, NTN and mobile-IAB. It would be good to clarify in text separately the applicability of TN, NTN and mobile-IAB with respect each operation of TN, NTN and mobile-IAB, especially, when NTN is removed in heading of Section 22. |
| NEC | Agree, and see comments | Reply to Huawei and Ericsson The UE behaviour follows the configuration of beam indication, i.e., the restriction of target beam that can be configured for the NTN UE, the IAB UE, and the TN UE can be captured in TS 38.331, then UE only needs to determine the beam index based on the configuration for dynamic-grant PUSCH transmission. In addition, if companies still deems it necessary to clarify on the applicability of ssb-Index and tci-StateID with respect to TN, NTN and mobile-IAB, I think we can add one sentence as follows in clause 22.2 to clarify it. # 22 PUSCH transmission in RACH-less handover ## 22.2 Dynamic-grant PUSCH transmission For RACH-less handover, only the ssb-Index can be configured for NTN UEs, only the tci-StateID can be configured for mobile IAB UEs, while ssb-Index or tci-StateID can be configured for TN UEs. If ssb-Index or tci-StateID is provided in RACH-LessHO, the UE may assume that the DM-RS antenna port associated with the PDCCH receptions for scheduling initial PUSCH transmission and the SS/PBCH block indicated by ssb-Index or the DL RS configured by a TCI state via tci-StateID are quasi co-located with respect to average gain and quasi co-location 'typeA' or 'typeD' properties.. Reply to Huawei I think there is no restriction that clause 22.1 only applies for FDD, and PO validation check is not specified in this clause. For the wording of “quasi co-location 'typeA' or 'typeD' properties” at the end, it is legacy description, and we can see that other clause, e.g., clause 19, also have the same description. We do not need to modify it. |
| Huawei, HiSilicon |  | We disagree with NEC comments. When you compared 19.1 and 22.1, NTN intentionally not copy the following blue paragraph for PUSCH occasion validation because NTN is not for TDD For unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion …. However, when we draft CG for LTM we added the paragraph back in 21.1 for TDD. For unpaired spectrum and for SS/PBCH blocks with indexes provided by rrc-SSB-Subset in cg-LTM-Configuration, or ssb-PositionsInBurst in ServingCellConfigCommon if rrc-SSB-Subset is not provided - if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion …. So the current 22.1 is only for FDD. As for the wording of “quasi co-location 'typeA' or 'typeD' properties”, we think clause 19 made a mistake as well. Please further check there are a lot of places in MIMO and LTM using “quasi co-location 'typeA' and 'typeD' properties when applicable” |

Q2: Do you agree to update the related description as follows in TS 38.213 to support the pathloss estimate based on a RS indicated via the TCI state. If not, please provide the reason and the description that you preferred.

7.1.1 UE behaviour ============== unchanged parts are omitted =============== -- PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in clause 12, of carrier f of serving cell c - If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPL-ForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) - using a RS resource from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB, or using the SS/PBCH block the UE acquired the time and frequency synchronization for a secondary cell. - if the UE is provided RACH-LessHO in ReconfigurationWithSync [12. TS 38.331], using a RS resource from an SS/PBCH block with same SS/PBCH block index or the DL RS configured by a TCI state indicated by tci-StateID as the one with same quasi co-location properties as for PDCCH receptions for scheduling an initial PUSCH transmission, as described in Clause 10.1, in controlResourceSetZero or commonControlResourceSet provided in ServingCellConfigCommon of ReconfigurationWithSync. ============== unchanged parts are omitted ===============

| Company | Agree or not | Comments |
| --- | --- | --- |
| Vivo3 | Not agree with the second change | Reply to OPPO For initial access, only SSB can be the QCLed source for CORESET0. But for handover, the TCI for QCLed source for CORESET0 can be SSB or CSI-RS as indicated by TCI state, thus the original TP is correct. ==38.213== For a CORESET with index 0, - if the UE is provided TCI-State and followUnifiedTCI-State for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214] ==38.321== #### 6.1.3.15 TCI State Indication for UE-specific PDCCH MAC CE The TCI State Indication for UE-specific PDCCH MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields: - Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits. If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 as specified in TS 38.331 [5], this MAC CE applies to all theServing Cells in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively; - CORESET ID: This field indicates a Control Resource Set identified with ControlResourceSetId as specified in TS 38.331 [5], for which the TCI State is being indicated. In case the value of the field is 0, the field refers to the Control Resource Set configured by controlResourceSetZero as specified in TS 38.331 [5]. The length of the field is 4 bits; - TCI State ID: This field indicates the TCI state identified by TCI-StateId as specified in TS 38.331 [5] applicable to the Control Resource Set identified by CORESET ID field. If the field of CORESET ID is set to 0, this field indicates a TCI-StateId for a TCI state of the first 64 TCI-states configured by tci-StatesToAddModList and tci-StatesToReleaseList in the PDSCH-Config in the active BWP. If the field of CORESET ID is set to the other value than 0, this field indicates a TCI-StateId configured by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList in the controlResourceSet identified by the indicated CORESET ID. The length of the field is 7 bits. Figure 6.1.3.15-1: TCI State Indication for UE-specific PDCCH MAC CE Regarding whether there is a need to extend the sentence to commonControlResourceSet, it was discussed in question7 in R1-2401535 in RAN1#116, but it was not agreed because companies thought this is not a critical issue. |
| Huawei, HiSilicon | Not agree | The “DL RS configured by a TCI state” can be interpreted as QCL RS or pathloss RS. for power control purpose, it should be pathlossReferenceRS-Id as mentioned by Samsung. |
| Ericsson | Not agree | We share the view of Samsung in Round 1 with respect to power control. |
| OPPO |  | Reply to vivo After offline discussion, it is clarified that in handover procedure, CORESET0 can only be associated with SSB, as TCI state is configured by RRC instead of indicated by MAC CE. ===== TS38.213 ====== For a CORESET with index 0, - if the UE is provided TCI-State and followUnifiedTCI-State for the CORESET, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET and a DM-RS antenna port for PDSCH receptions scheduled by DCI formats provided by PDCCH receptions in the CORESET are quasi co-located with the reference signals provided by the indicated TCI-State [6, TS 38.214] - else if the UE is provided dl-OrJointTCI-StateList and is indicated a first TCI-State and a second TCI-State, and applyIndicatedTCI-State for the CORESET <<omitted>> - else, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with - the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or - the one or more DL RS configured by a TCI state provided by CandidateTCI-State, where the TCI state is indicated by an LTM Cell Switch Command MAC CE that triggers a RACH-less or RACH-based LTM cell switch, if any, or - a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure, or a SS/PBCH block the UE identified during a most recent configured grant PUSCH transmission as described in clause 19 or 22.1. |

Q3: Do you agree to update the related description as follows in TS 38.214 to support the case that the DMRS antenna port for RACH-less handover is determined via the TCI state? If not, please provide the reason and the description that you preferred.

6.2.2 UE DM-RS transmission procedure ============== unchanged parts are omitted =============== When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure, - for a configured-grant based PUSCH transmission in RRC_INACTIVE state, the UE is provided with a set of DM-RS port(s) by sdt-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 19.1 of [6, TS 38.213]. - for a configured-grant based PUSCH transmission in RACH-less handover or in RACH-less LTM cell switch, the UE is provided with a set of DM-RS port(s) by rrc-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) or the DL RS configured by a TCI state indicated by tci-stateID and a PUSCH occasion and the associated DM-RS resource as described in Clause 22.1 or clause 21.1 of [6, TS 38.213], respectively. ============== unchanged parts are omitted ===============

| Company | Agree or not | Comments |
| --- | --- | --- |
| Vivo3 | yes |  |
| Huawei, HiSilicon | Not agree | The mapping of DMRS resource on the PUSCH occasion is only related to SSB. For UE indicated TCI state with CSI-RS as QCL RS, it will check the root SSB. Similar behavior is defined for LTM. So, the 2nd change is not necessary. For the first change, we prefer not to mix RACH-less HO for NTN, mIAB and TN together. Suggest to use “RACH-less handover for NTN, TN or mIAB”. |
| Ericsson | Not agree | We also prefer to distinguish description with respect to RACH-less HO for TN, NTN and mobile-IAB. |
| OPPO |  | After offline discussion, we think remove “NTN” in the second bullet is enough: ============== unchanged parts are omitted =============== - for a configured-grant based PUSCH transmission in RACH-less handover or in RACH-less LTM cell switch, the UE is provided with a set of DM-RS port(s) by rrc-DMRS-Ports. The DM-RS port for the PUSCH is determined by the mapping between SS/PBCH block(s) and a PUSCH occasion and the associated DM-RS resource as described in Clause 22.1 or clause 21.1 of [6, TS 38.213], respectively. ============== unchanged parts are omitted =============== |

- Conclusion

According to the company's input, the summaries are as follows.

TBD

- References

- R1-2505120	LS on RACH-less handover	RAN2, NEC

- R1-2506861	Discussion on the support of RACH-less handover	vivo

- R1-2507309	Discussion on RAN2 LS on RACH-less handover	NEC

- R1-2507310	Draft CR for TS 38.213 on RACH-less handover LS	NEC

- R1-2507890	On RAN2 LS on RACH-less handover	Ericsson

- R1-2507893	Draft Rel-18 CR for RACH-less handover	Ericsson

- R1-2507894	Draft Rel-19 CR for RACH-less handover	Ericsson

- R1-2507133	Draft CR for RACH-less handover	OPPO