3GPP TSG RAN WG1 #125			R1-26abcde

Dalian, PR China, May 18th-22nd 2026

Agenda Item:	8.1

Source:	Moderator (Ericsson)

Title:	Draft summary of R19 NES maintenance for adaptation of common signaling

Document for:	Discussion

Introduction

This document provides summary for the R19 NES maintenance for adaptation of common signalling based on the relevant contributions listed in Appendix A.

Discussion

[1] proposes grammatical corrections to subclause 8.1 of 38.213 with no change to UE behavior.

| 1 | R1-2603893 | Maintenance on NES for NR | CATT |
| --- | --- | --- | --- |

Moderator suggests proponent of [1] to directly inform 38.213 editor as it is simply grammatical correction without any UE behavior change.

[2] proposes correction to subclause 8.1 of TS 38.213 to reflect that the RO validation should also consider partial overlapping case which was agreed in RAN1#120bis. Relevant discussion from [2] is captured in Appendix B.

| 2 | R1-2604251 | Maintenance on enhancements of network energy savings for NR | Sharp |
| --- | --- | --- | --- |

- Issue 1 : Please provide your view on below TP ([2] R1-2604251) for subclause 8.1 of TS 38.213.

TP for subclause 8.1 of TS 38.213-j30

Change reason: The RO validation for adaptation fails to take partial overlap scenarios into account.

Change summary: Add the definition of partial overlap to the specification to align with the existing agreement.

Consequence if not approved: Under partial overlap conditions, the UE cannot correctly determine the validity of RO.

-----------------------omitted text----------------------- ## 8.1 Random access preamble Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following: - A configuration for PRACH transmission on the cell [4, TS 38.211]. - A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell. - A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions. A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions. For Type-1 random access procedure, a UE can be provided, by rach-Config-Adapt in RACH-ConfigCommon, parameters for determining time resources and frequency resources for PRACH transmission [4, TS 38.211]. When a PRACH occasion associated with rach-Config-Adapt is overlapped with has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with rach-Config-Adapt, the former PRACH occasion is not valid and is not considered in the procedures in this clause. -----------------------omitted text-----------------------

| Company | Comment (if any) |
| --- | --- |
| Samsung | We think the CR is incorrect by replacing the description with simpliy “overlapped”, because it will missing the importance of overlap in both time and frequency domain, e.g., overlapped in only F domain will also count. We think it’s better to align the wording in agreement, like following: When a PRACH occasion associated with rach-Config-Adapt is overlapped in both time and frequency domain with has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with rach-Config-Adapt, the former PRACH occasion is not valid and is not considered in the procedures in this clause. |
| LG | We do not see a need for this CR, but we can accept Samsung’s proposal if the change is necessary. |

Conclusion

TBU

Appendix A (Contributions)

| 1 | R1-2603893 | Maintenance on NES for NR | CATT |
| --- | --- | --- | --- |
| 2 | R1-2604251 | Maintenance on enhancements of network energy savings for NR | Sharp |

Appendix B (Issue 1 related discussion from [2] R1-2604251)