3GPP TSG RAN WG1 #125			R1-26abcde

Dalian, PR China, May 18th-22nd 2026

Agenda Item:	8.1

Source:	Moderator (Ericsson)

Title:	Draft summary of R19 NES maintenance for adaptation of common signaling

Document for:	Discussion

# Introduction

This document provides summary for the R19 NES maintenance for adaptation of common signalling based on the relevant contributions listed in Appendix A.

# Discussion

[1] proposes grammatical corrections to subclause 8.1 of 38.213 with no change to UE behavior.

| 1 | R1-2603893 | Maintenance on NES for NR | CATT |
| --- | --- | --- | --- |

Moderator suggests proponent of [1] to directly inform 38.213 editor as it is simply grammatical correction without any UE behavior change.

[2] proposes correction to subclause 8.1 of TS 38.213 to reflect that the RO validation should also consider partial overlapping case which was agreed in RAN1#120bis. Relevant discussion from [2] is captured in Appendix B.

| 2 | R1-2604251 | Maintenance on enhancements of network energy savings for NR | Sharp |
| --- | --- | --- | --- |

### Issue 1 : Please provide your view on below TP ([2] R1-2604251) for subclause 8.1 of TS 38.213.

TP for subclause 8.1 of TS 38.213-j30

Change reason: The RO validation for adaptation fails to take partial overlap scenarios into account.

Change summary: Add the definition of partial overlap to the specification to align with the existing agreement.

Consequence if not approved: Under partial overlap conditions, the UE cannot correctly determine the validity of RO.

-----------------------omitted text----------------------- ## 8.1 Random access preamble Physical random access procedure for a UE is triggered upon request of a PRACH transmission by higher layers or by a PDCCH order or LTM Cell Switch Command MAC CE in clause 6.1.3.75 [11, TS 38.321] for a cell. A configuration by higher layers for a PRACH transmission includes the following: - A configuration for PRACH transmission on the cell [4, TS 38.211]. - A preamble index, a preamble SCS, PPRACH,target, a corresponding RA-RNTI when applicable [11, TS 38.321], and a PRACH resource for the cell. - A number of Npreamblerep>1 preamble repetitions for the PRACH transmission if the UE would transmit the PRACH with repetitions. A UE transmits a PRACH on a cell using the selected PRACH format with transmission power PPRACH,b,f,c(i), as described in clause 7.4, on the indicated PRACH resource or on a determined set of Npreamblerep resources using a same spatial filter in case of Npreamblerep preamble repetitions. For Type-1 random access procedure, a UE can be provided, by rach-Config-Adapt in RACH-ConfigCommon, parameters for determining time resources and frequency resources for PRACH transmission [4, TS 38.211]. When a PRACH occasion associated with rach-Config-Adapt is overlapped with has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with rach-Config-Adapt, the former PRACH occasion is not valid and is not considered in the procedures in this clause. -----------------------omitted text-----------------------

| Company | Comment (if any) |
| --- | --- |
| Samsung | We think the CR is incorrect by replacing the description with simpliy “overlapped”, because it will missing the importance of overlap in both time and frequency domain, e.g., overlapped in only F domain will also count. We think it’s better to align the wording in agreement, like following: When a PRACH occasion associated with rach-Config-Adapt is overlapped in both time and frequency domain with has same frequency resource and same time resource as a PRACH occasion in RACH-ConfigCommon that is not associated with rach-Config-Adapt, the former PRACH occasion is not valid and is not considered in the procedures in this clause. |
| LG | We do not see a need for this CR, but we can accept Samsung’s proposal if the change is necessary. |
| Sharp | We can also accept Samsung’s proposal. |
| Ofinno | Same view as LG. Not sure that the proposed CR really changes anything. |
| Nokia | We share LG and Ofinno's view that CR is unnecessary. If it is concluded that the change is needed, we would like to point out that the usual wording in 38.213 is “overlaps with,” rather than “is overlapped with.” |

# Conclusion

TBU

# Appendix A (Contributions)

| 1 | R1-2603893 | Maintenance on NES for NR | CATT |
| --- | --- | --- | --- |
| 2 | R1-2604251 | Maintenance on enhancements of network energy savings for NR | Sharp |

# Appendix B (Issue 1 related discussion from [2] R1-2604251)