3GPP TSG RAN WG1 #122bis	                                   					       							                     R1-2507741

Prague, Czech Republic, October 13th –17th, 2025

Agenda Item:	9.6

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR mobility enhancements Phase 4

Document for:	Discussion/Decision

- Introduction

This document presents the summary of email discussion [122bis-R19-UE_features] during RAN1 #122bis. According to the Chair’s Notes:

[122bis-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #122bis within the scope of [122bis-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [1].

- Summary of Contributions Submitted to RAN1 #122bis

The following is the moderator’s summary of contributions submitted to RAN1 #122bis in this agenda item.

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource | 45-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] | For CSI-RS based beam measurement, the UE measures the CSI-RS resource for beam management of one or more candidate cells. Thus, the maximum number of CSI-RS resources for L1-RSRP measurement of each candidate cell shall be up to UE capability. And the UE is configured to measure multiple candidate cells simultaneously. Thus, the maximum number of CSI-RS resources of all candidate cells shall be UE capability too. Therefore, the FGs 63-1 and 63-2 shall be updated as follows: Proposal 1: Add two more items in FGs 63-1 and 63-2 as follows: 63. NR_Mob_Ph4; 63-1; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource. 5. Maximum number of periodic CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 6. Maximum total number of periodic CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement.; FFS; Yes; NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported. |
| Samsung [7] |  |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: - Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-1, 63-2, 63-7, 63-7a: These feature-groups are currently defined for intra-frequency measurements on NZP CSI-RS and CSI-IM. Given the RAN4 definition for inter/intra-frequency measurements, we propose to generalize their definitions to include both intra-frequency and inter-frequency NZP CSI-RS and CSI-IM. Additionally, we propose to add a new component to each of the feature groups to indicate the UE capability for measurements on inter-frequency resources, see Appendix for the details. 63-1; NW triggered L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 3. Maximum number of RRC configured candidate cells for L1-RSRP measurement on CSI-RS resource 4. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for L1-RSRP measurement 5. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource; 45-1; NW triggered L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; Per BC; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 5 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4}. |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource | 63-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {0,1,2,3,4} Note: For component 4, the UE must support a non-zero value for at least one of aperiodic and semi-persistent | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] | For CSI-RS based beam measurement, the UE measures the CSI-RS resource for beam management of one or more candidate cells. Thus, the maximum number of CSI-RS resources for L1-RSRP measurement of each candidate cell shall be up to UE capability. And the UE is configured to measure multiple candidate cells simultaneously. Thus, the maximum number of CSI-RS resources of all candidate cells shall be UE capability too. Therefore, the FGs 63-1 and 63-2 shall be updated as follows: Proposal 1: Add two more items in FGs 63-1 and 63-2 as follows: 63. NR_Mob_Ph4; 63-2; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource 3. Maximum number of semi-persistent CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 4. Maximum total number of semi-persistent CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement.; 63-1; Yes; NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported. |
| Samsung [7] |  |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-1, 63-2, 63-7, 63-7a: These feature-groups are currently defined for intra-frequency measurements on NZP CSI-RS and CSI-IM. Given the RAN4 definition for inter/intra-frequency measurements, we propose to generalize their definitions to include both intra-frequency and inter-frequency NZP CSI-RS and CSI-IM. Additionally, we propose to add a new component to each of the feature groups to indicate the UE capability for measurements on inter-frequency resources, see Appendix for the details. 63-2; NW triggered L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure; 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Support of inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 3. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource; 63-1; NW triggered L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported; Per BC; Component 3 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {0,1,2,3,4}. |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-3 | CSI-RS as Type-D QCL source RS in the indicated joint LTM TCI state | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states | 45-3 | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | 45-3a, 63-3 | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-4 | CSI-RS as Type-D QCL source RS in the indicated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | 45-4 | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS in the indicated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | 45-4a, 63-4 | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS | Yes | No | Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG63-6 and FG63-6a, the prerequisite feature groups have not been defined in last meeting. Considering CSI measurement is performed after receiving LTM CSC MAC CE for these FGs. Thus, UE supporting 63-6 should also report the capability of basic LTM procedure, i.e. RAN2 FG for LTM in Rel-18 (at least one of ltm-MCG-r18 and ltm-SCG-r18). For FG63-6a, the prerequisite should be FG63-6. Proposal 1: For FG63-6, support RAN2 FG for LTM in Rel-18 (at least one of ltm-MCG-r18 and ltm-SCG-r18) as prerequisite. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS RAN2 FG for LTM in Rel-18; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per Band; n/a; n/a; n/a; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| ZTE Corporation/Sanechips [3] | - For FG-63-6, the prerequisite FG 2-36 (Type I single panel codebook) needs to be added. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 2-36; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band; n/a; n/a; n/a; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| Nokia [4] | - Since FG 63-6 includes support for both CSI-RS and CSI-IM, FG 2-33 on basic CSI using both CSI-RS and CSI-IM reception for CSI feedback should be defined as its prerequisite FG. In addition, since the UE starts measurements only after receiving the LTM CSC MAC CE, the UE should also support the basic LTM capability, i.e., the RAN2 FG for Rel-18 LTM. - The prerequisite FG for FG 63-6a should be 63-6. - The FG description and the description of Component 1 of FG 63-6 currently only mention “based on periodic CSI-RS resource.” However, it should also include “periodic CSI-IM resource.” - The FG description and the description of Component 1 of FG 63-6a currently only mention “based on semi-persistent CSI-RS resource.” However, it should also include “semi-persistent CSI-IM resource.” - The component numbering for components 3 to 7 should be corrected for both 63-6 and 63-6a. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS and CSI-IM resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5 4. Maximum number of ports in one NZP CSI-RS resource 6 5. Max rank for CSI reporting for a candidate cell 7 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 2-33, RAN2 FG for LTM in Rel-18; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band; n/a; n/a; n/a; Component 32 candidate values: {1,2,3,4,5,6,7,8} Component 43 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 54 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 65 candidate values: {1,2,3,4,5,6,7,8} Component 76 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| CATT [5] | - For FG 63-6, since LTM CSC MAC CE has to be received and CSI reporting is needed, both RAN2 FG for LTM in Rel-18 (FG 45-1) and basic CSI feedback feature (FG 2-32) should be supported as prerequisite. 63. NR_Mob_Ph4; 63-6; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE [2. Maximum number of the RRC configured candidate cells] 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 45-1, 2-32; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS Per Band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: FFS {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-6, 63-6a: These FGs are defined for intra-frequency measurements. However, they concern measurements after reception of LTM CSC MAC CE. At that point in time there is no serving cell or candidate cell, hence it is not possible to speak of inter- or intra-frequency measurements according to the RAN4 definitions. See Appendix for our proposed updates. 63-6; CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 4. Maximum number of ports in one NZP CSI-RS resource 5. Max rank for CSI reporting for a candidate cell 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS; Periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 5 candidate values: {1,2,3,4,5,6,7,8} Component 6 candidate values: {1,2,3,4,5,6,7,8}. |
| NTT DOCOMO, INC. [9] | 63-6 - FG45-1 should be prerequisite FG since this feature is LTM-specific. |

| 63. NR_Mob_Ph4 | 63-6a | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS | Yes | No | Intra-frequency semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG63-6 and FG63-6a, the prerequisite feature groups have not been defined in last meeting. Considering CSI measurement is performed after receiving LTM CSC MAC CE for these FGs. Thus, UE supporting 63-6 should also report the capability of basic LTM procedure, i.e. RAN2 FG for LTM in Rel-18 (at least one of ltm-MCG-r18 and ltm-SCG-r18). For FG63-6a, the prerequisite should be FG63-6. Proposal 2: For FG63-6a, support FG63-6 as prerequisite. 63. NR_Mob_Ph4; 63-6a; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 63-6; Yes; No; Intra-frequency semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per Band; n/a; n/a; n/a; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| ZTE Corporation/Sanechips [3] | - For FG-63-6a, the prerequisite FG 63-6 needs to be added. 63. NR_Mob_Ph4; 63-6a; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 63-3; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,…64}; Optional with capability signaling. |
| Nokia [4] | - Since FG 63-6 includes support for both CSI-RS and CSI-IM, FG 2-33 on basic CSI using both CSI-RS and CSI-IM reception for CSI feedback should be defined as its prerequisite FG. In addition, since the UE starts measurements only after receiving the LTM CSC MAC CE, the UE should also support the basic LTM capability, i.e., the RAN2 FG for Rel-18 LTM. - The prerequisite FG for FG 63-6a should be 63-6. - The FG description and the description of Component 1 of FG 63-6 currently only mention “based on periodic CSI-RS resource.” However, it should also include “periodic CSI-IM resource.” - The FG description and the description of Component 1 of FG 63-6a currently only mention “based on semi-persistent CSI-RS resource.” However, it should also include “semi-persistent CSI-IM resource.” - The component numbering for components 3 to 7 should be corrected for both 63-6 and 63-6a. 63. NR_Mob_Ph4; 63-6a; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS and CSI-IM resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5 4. Maximum number of ports in one NZP CSI-RS resource 6 5. Max rank for CSI reporting for a candidate cell 7 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 63-6; Yes; No; Intra-frequency semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band; n/a; n/a; n/a; Component 32 candidate values: {1,2,3,4,5,6,7,8} Component 43 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 54 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 65 candidate values: {1,2,3,4,5,6,7,8} Component 76 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| CATT [5] | - For FG 63-6a, support FG 63-6 as prerequisite. 63. NR_Mob_Ph4; 63-6a; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE [2. Maximum number of the RRC configured candidate cells] 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of CSI-RS ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of Tx ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS 63-6; Yes; No; Intra-frequency semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; FFS Per Band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: FFS {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| OPPO [6] |  |
| Samsung [7] | First of all, we support FG 63-6 as a pre-requisite for FG 63-6a. Proposal 1. Incorporate FG 63-6 as a pre-requisite for FG 63-6a. |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-6, 63-6a: These FGs are defined for intra-frequency measurements. However, they concern measurements after reception of LTM CSC MAC CE. At that point in time there is no serving cell or candidate cell, hence it is not possible to speak of inter- or intra-frequency measurements according to the RAN4 definitions. See Appendix for our proposed updates. 63-6a; CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 4. Maximum number of ports in one NZP CSI-RS resource 5. Max rank for CSI reporting for a candidate cell 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell; FFS; Semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported; Per band; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 5 candidate values: {1,2,3,4,5,6,7,8} Component 6 candidate values: {1,2,3,4,5,6,7,8}. |
| NTT DOCOMO, INC. [9] | 63-6a - FG63-6 should be prerequisite FG. - For component 1, the following update should be applied to correct typo. - Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of cell indicated in CSC MAC CE |

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cells | 1. Support of CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE | 63-6 | Yes | No | Intra-frequency periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,…64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] | - Similar to FG 63-6 and 63-6a, the description of the FG 63-7 and FG 63-7a should also include “periodic CSI-IM resource” and “semi-persistent CSI-RS resource”, respectively. 63. NR_Mob_Ph4; 63-7; Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells; 1. Support of CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE; 63-6; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported; Per BC; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,…64}; Optional with capability signaling. |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] | Regarding FG 63-7 and FG 63-7a, it should be clarified that the candidate cells in Component 3 should correspond to those RRC configured for CSI measurement before receiving LTM CSC MAC CE, which is more aligned with the corresponding RAN1’s agreement(s) – otherwise, it is unclear how these candidate cells are obtained/determined. Hence, we propose the following updates highlighted in red to Component 3 in FG 63-7 and FG 63-7a. Proposal 2. Adopt the following updates highlighted in red to Component 3 in FG 63-7 and FG 63-7a. “3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE” Similarly, for Component 6 in FG 63-7 and FG 63-7a, we propose the following updates in red: Proposal 3. Adopt the following updates highlighted in red to Component 6 in FG 63-7 and FG 63-7a. “6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE” |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-1, 63-2, 63-7, 63-7a: These feature-groups are currently defined for intra-frequency measurements on NZP CSI-RS and CSI-IM. Given the RAN4 definition for inter/intra-frequency measurements, we propose to generalize their definitions to include both intra-frequency and inter-frequency NZP CSI-RS and CSI-IM. Additionally, we propose to add a new component to each of the feature groups to indicate the UE capability for measurements on inter-frequency resources, see Appendix for the details. 63-7; CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) of candidate cells; 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 2. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 3. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 4. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 5. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 7. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE; 63-6; Periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported; Per BC; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,...64} Component 5 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 6 candidate values: {1,2,4,8,12,16,24,32} Component 7 candidate values: {1,2,…64}. |
| NTT DOCOMO, INC. [9] | 63-7/7a - For component 4/5/6, these should be removed since prerequisite FG includes the value. |

| 63. NR_Mob_Ph4 | 63-7a | Intra-frequency CSI-RS and CSI-IM measurement for candidate celbefore reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells | 1. Support of CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE | 63-6a | Yes | No | Intra-frequency semi-persistent CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,...64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] | - Similar to FG 63-6 and 63-6a, the description of the FG 63-7 and FG 63-7a should also include “periodic CSI-IM resource” and “semi-persistent CSI-RS resource”, respectively. 63. NR_Mob_Ph4; 63-7a; Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells; 1. Support of CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE; 63-6a; Yes; No; Intra-frequency semi-persistent CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported; Per BC; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,...64}; Optional with capability signaling. |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] | Regarding FG 63-7 and FG 63-7a, it should be clarified that the candidate cells in Component 3 should correspond to those RRC configured for CSI measurement before receiving LTM CSC MAC CE, which is more aligned with the corresponding RAN1’s agreement(s) – otherwise, it is unclear how these candidate cells are obtained/determined. Hence, we propose the following updates highlighted in red to Component 3 in FG 63-7 and FG 63-7a. Proposal 2. Adopt the following updates highlighted in red to Component 3 in FG 63-7 and FG 63-7a. “3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE” Similarly, for Component 6 in FG 63-7 and FG 63-7a, we propose the following updates in red: Proposal 3. Adopt the following updates highlighted in red to Component 6 in FG 63-7 and FG 63-7a. “6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE” |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FGs 63-1, 63-2, 63-7, 63-7a: These feature-groups are currently defined for intra-frequency measurements on NZP CSI-RS and CSI-IM. Given the RAN4 definition for inter/intra-frequency measurements, we propose to generalize their definitions to include both intra-frequency and inter-frequency NZP CSI-RS and CSI-IM. Additionally, we propose to add a new component to each of the feature groups to indicate the UE capability for measurements on inter-frequency resources, see Appendix for the details. 63-7a; CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells; 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 2. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 3. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 4. Maximum number of RRC configured CSI-RS resources across candidate cells for CSI measurement before LTM CSC MAC CE 5. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 7. Maximum number of RRC configured CSI-IM resources across candidate cells for CSI measurement before LTM CSC MAC CE; 63-6a; Semi-persistent CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported; Per BC; Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,...64} Component 5 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 6 candidate values: {1,2,4,8,12,16,24,32} Component 7 candidate values: {1,2,...64}. |
| NTT DOCOMO, INC. [9] | 63-7/7a - For component 4/5/6, these should be removed since prerequisite FG includes the value. |

| 63. NR_Mob_Ph4 | 63-8 | Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s) | 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s) | 63-1 or 63-2 | Yes | No | UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s) | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] | Regarding FG 63-8, the FG functionality description is inaccurate. Support of “always” inclusion is somewhat misleading, which does not exist in the corresponding RAN1’s agreement(s) and should be removed. Proposal 4. Adopt the following updates in red to FG 63-8. 63. NR_Mob_Ph4; 63-8; Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s); 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s); 63-1 or 63-2; Yes; No; UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s); Per BC; n/a; n/a; n/a; Optional with capability signalling. |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

| 63. NR_Mob_Ph4 | 63-10 | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception | 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration | 63-6 or 63-6a or 63-7 or 63-7a | Yes | No | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] | 63. NR_Mob_Ph4; 63-10; Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception; 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration; 63-6 or 63-6a or 63-7 or 63-7a, or 63-X1; Yes; No; Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Nokia [4] | 63. NR_Mob_Ph4; 63-10; Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception; 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration; 63-6 or 63-6a or 63-7 or 63-7a; Yes; No; Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] | In the LS RAN4 informs RAN1 of the following agreement on definition of CSI-RS based L1 intra/inter-frequency measurements: Definition on CSI-RS based intra-frequency L1 measurement Agreement: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the SCS of active DL BWP, and - the CP type of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is the same as the CP type of active DL BWP, and - It is applied for SCS = 60KHz - at least 48 RBs of the CSI-RS resource of LTM candidate cell(s) configured for L1 measurement is included within the active DL BWP. Otherwise, a measurement is defined as a CSI-RS based inter-frequency L1 measurement. - RAN4 will not define requirements for CSI-RS based inter-frequency L1 measurement on LTM candidate cell(s) in R19. This agreement has implications on RAN1 feature-groups: - FG 63-10: This FG is defined for intra-frequency measurement and reporting of CSI without measurements on CSI-IM. We propose that this FG should not depend on the frequency location of the NCP-CSI-RS, i.e. it should apply to both the inter- and the intra-frequency case, see Appendix for details. 63-10; CSI-RS-RS measurement and CSI reporting without CSI-IM reception; 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration; 63-6 or 63-6a or 63-7 or 63-7a; CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported; Per band. |
| NTT DOCOMO, INC. [9] |  |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] |  |
| ZTE Corporation/Sanechips [3] | - Capturing a new FG 63-X1 on “supporting early CSI acquisition of L3 handover” based on the following RAN2 LS [3], where RAN2 discussed on the intention to support early CSI acquisition for L3 handover by re-using the early CSI acquisition framework for LTM. RAN2 discussed the RRC CR (R2-2506450) and agreed that the RRC CR is technically correct. However, RAN2 postponed the CR pending confirmation from RAN1. RAN2 identified at least the following potential impacts to RAN1: - TS 38.214: Describe CSI reporting for L3 handover which will be similar to the description in clause “5.2.4a CSI Reporting for LTM”. - TS 38.212: Add a reference to TS 38.214 in clause “6.3.2.1.2 CSI”. To RAN1 ACTION: RAN2 respectfully asks RAN1 to check if it is OK to support early CSI acquisition for L3 handover and update the RAN1 spec if needed.. - The following components are included: - 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after Handover Command based on periodic CSI-RS(s) of a target cell - 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a target cell - Component 2 candidate values: {1,2,3,4,5,6,7,8} - 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a target cell - Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} - 4. Maximum number of ports in one NZP CSI-RS resource - Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} - 5. Max rank for CSI reporting for a target cell - Component 5 candidate values: {1,2,3,4,5,6,7,8} - 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a target cell - Component 6 candidate values: {1,2,3,4,5,6,7,8} - The prerequisite RAN2 related FG needs to be added. - The granularity should be per band. 63. NR_Mob_Ph4; 63-X1; Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command based on periodic CSI-RS resource; 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after Handover Command based on periodic CSI-RS(s) of a target cell 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a target cell 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a target cell 4. Maximum number of ports in one NZP CSI-RS resource 5. Max rank for CSI reporting for a target cell 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a target cell; RAN2 related FG; Yes; No; Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command is not supported; Per band; n/a; n/a; n/a; Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 5 candidate values: {1,2,3,4,5,6,7,8} Component 6 candidate values: {1,2,3,4,5,6,7,8}; Optional with capability signaling. |
| Nokia [4] |  |
| CATT [5] |  |
| OPPO [6] |  |
| Samsung [7] |  |
| Ericsson [8] |  |
| NTT DOCOMO, INC. [9] |  |

- Discussion Items during RAN1 #122bis

After review of contributions submitted to RAN1 #122bis in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #122bis.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

  - FG 63-1

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 1a. Support of inter-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using periodic CSI-RS as measurement resource 5. Maximum number of periodic CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 6. Maximum total number of periodic CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | 45-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: L: {1, 2,3,4} M: {1, 2,3,4} M L: {1,2,3,4, 6, 8, 9, 12, 16} Component 4 candidate values: Aperiodic: {0,1,2,3,4} Periodic: {1,2,3,4} Semi-persistent: {0,1,2,3,4} Component 5 candidate values: FFS Component 6 candidate values: FFS | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | We support the proposed changes except for the new components 5 and 6, as these components correspond to RAN4-related UE capabilities. |
| ZTE | We understand from RAN4’s LS that inter-frequency measurement is not actually supported in Rel-19 LTM. So we don’t think there should be any changes to the current FG. |

  - FG 63-2

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 1a. Support of inter-frequency L1- RSRP measurement and reporting based on semi-persistent CSI-RS(s) of candidate cell(s) 2. Maximum number of LTM CSI report configs using semi-persistent CSI-RS as measurement resource 3. Maximum number of semi-persistent CSI-RS resources of one candidate cell configured for intra-frequency L1-RSRP measurement 4. Maximum total number of semi-persistent CSI-RS resources of all candidate cells configured for intra-frequency L1-RSRP measurement. | 63-1 | Yes | No | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: Aperiodic: {0,1,2,3,4} Semi-persistent: {0,1,2,3,4} Component 3 candidate values: FFS Component 4 candidate values: FFS Note: For component 4, the UE must support a non-zero value for at least one of aperiodic and semi-persistent | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support the changes except the new components 3 and 4. The component 3 and 4 are RAN4 related UE capabilities. |
| ZTE | We understand from RAN4’s LS that inter-frequency measurement is not actually supported in Rel-19 LTM. So we don’t think there should be any changes to the current FG. |

  - FG 63-6

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on periodic CSI-RS and CSI-IM resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS RAN2 FG for LTM in Rel-18, 2-32, 2-33, 2-36, 45-1 | Yes | No | Intra-frequency Periodic CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | For the prerequisite FGs, 45-1 should be removed, as 45-1 relates to intra-frequency SSB-based L1-RSRP measurement and has no relation to 63-6. In addition, only the RAN2 FGs for LTM in Rel-18 and 2-33 are sufficient; however, it may also be acceptable to include 2-32 and 2-36. |
| ZTE | “intra-frequency” should not be removed since inter-frequency measurement is not supported in Rel-19 LTM according to LS from RAN4. |

  - FG 63-6a

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-6a | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE based on semi-persistent CSI-RS and CSI-IM resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic semi-persistent CSI-RS(s) and CSI-IM resources of cell indicated in CSC MAC CE 3. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a candidate cell 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource 6. Max rank for CSI reporting for a candidate cell 7. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a candidate cell | FFS 63-6 | Yes | No | Intra-frequency Semi-persistent CSI-RS and CSI-IM measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {1,2,3,4,5,6,7,8} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 6 candidate values: {1,2,3,4,5,6,7,8} Component 7 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support |
| ZTE | “intra-frequency” should not be removed since inter-frequency measurement is not supported in Rel-19 LTM according to LS from RAN4. |

  - FG 63-7

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells | 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) and CSI-IM resources of candidate cells 1a. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on periodic CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE | 63-6 | Yes | No | Intra-frequency Periodic CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,…64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | In component 1a: it should be “….based on periodic CSI-RS(s) and CSI-IM resources of candidate cells”. Better to keep the components 4, 5, and 6; otherwise, please also remove component 3 as that can also be derived based on pre-requisite FG 63-6. |
| ZTE | Firstly, “intra-frequency” should not be removed since inter-frequency measurement is not supported in Rel-19 LTM according to LS from RAN4. Besides, for component 4,5,6, we have the same view as Nokia. |

  - FG 63-7a

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-7a | Intra-frequency CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells | 1. Support of intra-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells 1a. Support of inter-frequency CSI-RS and CSI-IM measurement before reception of CSC MAC CE based on semi-persistent CSI-RS(s) of candidate cells 2. Maximum number of RRC configured candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of RRC configured CSI-RS resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE 4. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a candidate cell 5. Maximum number of ports in one NZP CSI-RS resource associated with a CSI report configuration for CSI reporting for a candidate cell 6. Maximum number of RRC configured CSI-IM resources across candidate cells RRC configured for CSI measurement before LTM CSC MAC CE | 63-6a | Yes | No | Intra-frequency Semi-persistent CSI-RS and CSI-IM measurement for candidate cell before reception of LTM CSC MAC CE is not supported | Per BC | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,...64} Component 4 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 5 candidate values: {1,2,4,8,12,16,24,32} Component 6 candidate values: {1,2,...64} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | In component 1a: it should be “….based on semi-persistent CSI-RS(s) and CSI-IM resources of candidate cells”. Better to keep the components 4, 5, and 6; otherwise, please also remove component 3 as that can also be derived based on pre-requisite FG 63-6a. |
| ZTE | Same comments as FG 63-7 |

  - FG 63-8

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-8 | Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s) | 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s) | 63-1 or 63-2 | Yes | No | UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s) | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | This issue has been discussed in RAN1#122 meeting and majority disagree with removing “always” since such term is also used for Rel-18 LTM. |

  - FG 63-10

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-10 | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception | 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration | 63-6 or 63-6a or 63-7 or 63-7a or 63-11 | Yes | No | Intra-frequency CSI-RS-RS measurement and CSI reporting without CSI-IM reception is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-11 | Intra-frequency CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command based on periodic CSI-RS resource | 1. Support of CSI-RS and CSI-IM measurement and CSI reporting after Handover Command based on periodic CSI-RS(s) of a target cell 2. Maximum number of CSI-RS resources for CMR associated with CSI report configuration for a target cell 3. Max number of ports of CSI-RS resource(s) associated with a CSI report configuration for CSI reporting for a target cell 4. Maximum number of ports in one NZP CSI-RS resource 5. Max rank for CSI reporting for a target cell 6. Maximum number of CSI-IM resources for interference measurement associated with CSI report configuration for a target cell | RAN2 related FG | Yes | No | Intra-frequency periodic CSI-RS and CSI-IM measurement and CSI reporting for a target cell after reception of Handover Command is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4,5,6,7,8} Component 3 candidate values: {1,2,4,8,12,16,24,32,48,64,128} Component 4 candidate values: {1, 2, 4, 8, 12, 16, 24, 32} Component 5 candidate values: {1,2,3,4,5,6,7,8} Component 6 candidate values: {1,2,3,4,5,6,7,8} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia | Support the changes in 63-10. 63-11 is related to TEI19 related to early CSI acquisition for L3 HO. |
| ZTE | For FG 63-10, We disagree with removing “intra-frequency”. For FG 63-11, we can discuss it after whether to support early CSI for L3 HO is confirmed in RAN1. |

- Conclusion

Agreements reached during RAN1 #122bis as part of this agenda item are summarized in [10].

- References

- R1-2506627, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2506943, UE features for NR mobility enhancements phase 4, Huawei/HiSilicon

- R1-2507039, Discussion on UE features for NR mobility enhancements Phase 4, ZTE Corporation/Sanechips

- R1-2507076, NR mobility enhancements Phase 4 UE features, Nokia

- R1-2507128, Discussion on UE features for NR mobility enhancements Phase 4, CATT

- R1-2507162, Discussion on UE features for NR mobility enhancements, OPPO

- R1-2507273, Remaining issues on UE features for Rel-19 LTM, Samsung

- R1-2507475, UE features for NR mobility enhancements phase 4, Ericsson

- R1-2507799, Discussion on UE features for NR mobility enhancemens Phase4, NTT DOCOMO, INC.

- R1-25nnnnn, Session Notes of AI 9.6, Ad-Hoc Chair (AT&T)