3GPP TSG RAN WG2#131	R2-2505027

Bengaluru, India, 25th - 29thAugust 2025

3GPP TSG RAN WG1 #121	R1-2505008

St Julian’s, Malta, May 19th – 23th, 2025

Title:	LS on Draft CR on TS38.300 for Rel-19 MIMO

Response to:	-

Release:	Rel-19

Work Item:	NR_MIMO_Ph5

Source:	RAN WG1

To:	RAN WG2

CC:

Contact Person:

#### Name:	Youngrok Jang

#### Email Address:	yr.jang@samsung.com

1. Overall Description:

As part of Rel-19 MIMO WI, the following TP to 38.300 has been endorsed from RAN1 perspective.

## ----------------------------------------- start TP for 38.300 v.18.5.0 ------------------------------------------- ## 6.12 Multiple Transmit/Receive Point Operation In Multiple Transmit/Receive Point (multi-TRP) operation, a serving cell can schedule the UE from two TRPs, providing better coverage, reliability and/or data rates for PDSCH, PDCCH, PUSCH, and PUCCH. There are two different operation modes to schedule multi-TRP PDSCH transmissions: single-DCI and multi-DCI. For both modes, control of uplink and downlink operation can be done by physical layer and MAC layer, within the configuration provided by the RRC layer. In single-DCI mode, the UE is scheduled by the same DCI for both TRPs and in multi-DCI mode, the UE is scheduled by independent DCIs from each TRP. <unchanged parts are omitted> For inter-TRP delay, frequency, and phase calibration for coherent joint transmission (CJT), CSI-RS per TRP is transmitted and the UE measures the CSI-RS(s) and estimates the reporting quantity including inter-TRP delay offset(s), frequency offset(s), and phase offset(s). A CSI report associated with the reporting quantity is carried on a PUSCH. - 9.2.3 Mobility in RRC_CONNECTED - 9.2.3.1 Overview <unchanged parts are omitted> Beam Level Mobility does not require explicit RRC signalling to be triggered. Beam level mobility can be within a cell, or between cells, the latter is referred to as inter-cell beam management (ICBM). For ICBM, a UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell, while non-UE-dedicated channels/signals can only be received via a TRP associated with a PCI of the serving cell. The gNB provides via RRC signalling the UE with measurement configuration containing configurations of SSB/CSI resources and resource sets, reports and trigger states for triggering channel and interference measurements and reports. In case of ICBM, a measurement configuration includes SSB resources associated with PCIs different from the PCI of a serving cell. Beam Level Mobility is then dealt with at lower layers by means of physical layer and MAC layer control signalling, and RRC is not required to know which beam is being used at a given point in time. SSB-based Beam Level Mobility is based on the CD-SSB associated to the initial DL BWP and can be configured for the initial DL BWPs, for DL BWPs containing the CD-SSB associated to the initial DL BWP, and if supported, for DL BWPs not containing the CD-SSB associated to the initial DL BWP. SSB-based Beam Level Mobility can be also performed based on an NCD-SSB, if configured for the active DL BWP. Beam Level Mobility can be also performed based on CSI-RS, if configured for the active DL BWP. For UE-initiated beam reporting, Mode A and Mode B are supported, where upon detection of an event, UE transmits a UE Initiated Report Indicator (UEIRI) indication in a PUCCH resource to indicate transmission of a beam report. Corresponding to the UEIRI indication, in Mode A, the UE transmits the beam report on a PUSCH indicated by a DCI, while in Mode B, the UE transmits the beam report on a type1 CG PUSCH associated to the PUCCH resource. <unchanged parts are omitted> ## 6.X Asymmetric Downlink Single-TRP and Uplink Multi-TRP For asymmetric DL single-TRP and UL multi-TRP scenario where UL TRP may reduce or even turn off DL transmission, pathloss offset can be configured by RRC and indicated by TCI state for PUSCH, PUCCH, and SRS transmission, and can be indicated by a PDCCH order for the PDCCH order triggered PRACH toward UL TRP facilitating pathloss calculation. In addition, up to two closed loop power control adjustment states can be supported for SRS separate with PUSCH. Also, up to two TAGs can be configured in the serving cell without coresetPoolIndex configured, and a TCI state for PUSCH, PUCCH, and SRS transmission can be associated with any one of two TAGs. ## ----------------------------------------- end TP-------------------------------------------

RAN1 respectfully asks RAN2 to take the above into account in their future work.

2. Actions:

To RAN2

ACTION:  RAN1 respectfully asks RAN2 to take the above into account in their future work.

3. Date of Next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #122	25th – 29th August 2025		India, IN

TSG RAN WG1 Meeting #122-bis	13th– 17th October 2025		Czech, CZ