3GPP TSG-RAN WG2 Meeting #131	R2-2505073

Bangaluru, India, 25-29 August 2025

Agenda item:	8.7.6

Source:	Qualcomm Incorporated

Title:	Discussion on XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the remaining issues for uplink rate control.

# Discussion

[UE capability-5] Maximum number of QoS flows that a UE supports for rate control

In 5G, each PDU session can include up to 64 QoS flows, and a UE can support up to 15 PDU sessions. Therefore, in theory, there can be up to 15x64=960 QoS flow in a UE.

This number obviously is way more than the typical number of QoS flows that need rate control. To simplify UE implementation, it is therefore desirable to define a maximum for this number.

The value of this maximum clearly depends on a UE’s capabilities. For example, a premium tier UE with more processing power and more memory can support more QoS flows than a low-tier UE (e.g. a RedCap UE). Therefore, it makes more sense to define this maximum as a UE capability instead of a constant in the spec for all types of UEs.

For most XR applications, it is safe to assume that the number of QoS flows subject to rate control is at most 64. This assumption was also the consensus among companies during the post-RAN2#130 RRC running CR review. We therefore propose using this number to set the range of the above UE capability.

Proposal 1.	[UE capability-5] The maximum number of QoS flows subject to rate control is a UE capability, and its range is 1~64.

[MAC-3] Which type of ID should be used to identify a QoS flow

In our understanding, it is true that a QFI is unique only within a PDU session. Therefore, PDU session ID and QFI have to be used together to uniquely identify a QoS flow.

According to TS 24.007, PDU session ID is 8 bits long, and QFI is 6 bits long (a maximum of 64 QoS flows per PDU session). Therefore, if PDU session ID and QFI are used together, the length of the QoS flow identifier is 14 bits.

We have observed that for most XR applications, the number of active traffic flows is in the range of 6~10. If we assume each traffic flow is supported by a dedicated QoS flow, then the above design implies the total number of bits required by the QoS flow identifiers in a UL Rate Control MAC CE can be up to 10x14 = 140 bits = 17.5 bytes. This is rather excessive, in our view.

Observation 1.	Using both PDU session ID and QFI to identify QoS flows in the UL Rate Control MAC CE can result in high overhead.

Since PDU session IDs and QFIs are relatively static and the typical number of QoS flows is only a very small percentage of the total ID space, we believe it is much more efficient to use RRC to configure a shorter identifier. That is feasible, because only 6 bits are needed to encode a maximum of 64 flows. Procedure wise, after core network provides the set of QoS flows subject to rate control, RAN configures this 6-bit identifier for each of these QoS flows for the UE. It then is used in the UL Rate Control MAC CE.

Now let us compare this approach with the option of using a bitmap.

If the maximum number of QoS flows is 64, then a 64-bit (or 8-byte) bitmap is needed. In addition, this bitmap is a fixed overhead, regardless of the number of QoS flows included in a Rate Control MAC CE.

On the other hand, since the typical number of QoS flows is 6~10, the total number of bits required by the identifiers is 36~60 bits. This overhead clearly is lower than that of the bitmap, and the saving is more if there are less QoS flows.

- Observation 2.	Using a 6-bit identifier configured by RRC is more efficient than a fixed-size bitmap.

We therefore propose using a 6-bit identifier configured by RRC to indicate a QoS flow in the UL Rate Control MAC CE.

Proposal 2.	[MAC-3] Use a 6-bit identifier configured by RRC to indicate a QoS flow in the UL Rate Control MAC CE.

[MAC-4] Format of the UL Rate Control MAC CE

If companies agree to adopt the design with identifier instead of bitmap for identifying QoS flows, then an extension bit is needed to indicate whether an additional QoS flow is included in the MAC CE.

Proposal 3.  	[MAC-4] An extension bit is used to indicate whether an additional QoS flow is included in the UL Rate Control MAC CE.

[MAC-5] Handling of triggered UL rate queries

It was agreed at the last RAN2 meeting that each QoS flow capable of rate adaptation can be configured with its own prohibit timer. As a result, it is possible that multiple rate queries can be triggered and remain pending at the same time.

If an available UL grant is not large enough to accommodate all the pending rate queries, it is to be discussed whether the UE can transmit some of the queries first or it must wait until it can send all pending queries together.

In our view, this decision depends on the types of traffic flow that triggered the queries. For example, some traffic flows, like videos from different sensors, should adapt together, while others, such as audio and video, can adapt by themselves independently. In addition, different types of traffic flows may have different priorities too. For example, flows with higher bit rates may adapt first.

Since this decision depends on the types of traffic, we think it should be left to UE implementation.

Proposal 4.	[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, it is up to UE implementation whether to transmit some of the queries first or wait until all of them can be sent together.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	[UE capability-5] The maximum number of QoS flows subject to rate control is a UE capability, and its range is 1~64.

Observation 1.	Using both PDU session ID and QFI to identify QoS flows in the UL Rate Control MAC CE can result in high overhead.

- Observation 2.	Using a 6-bit identifier configured by RRC is more efficient than a fixed-size bitmap.

Proposal 2.	[MAC-3] Use a 6-bit identifier configured by RRC to indicate a QoS flow in the UL Rate Control MAC CE.

Proposal 3.  	[MAC-4] An extension bit is used to indicate whether an additional QoS flow is included in the UL Rate Control MAC CE.

Proposal 4.	[MAC-5] If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, it is up to UE implementation whether to transmit some of the queries first or wait until all of them can be sent together.