3GPP TSG-RAN WG2 Meeting #131 	 R2-2505079

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.8.2

Source:	vivo

Title:	Remaining Issues on Repetition Enhancements for NTN

Document for:	Discussion and Decision

# Introduction

To improve NR downlink coverage in NTN, the following repetition enhancements are supported for FR1-NTN:

- Repetition of PDCCH CSS.

- Repetitions of PDSCH carrying Msg4.

- Repetitions within 20 ms duration of PDSCH carrying SIB1.

In this contribution, we would like to provide our consideration of some remaining RAN2 issues regarding repetition enhancements.

# Discussion

## 2.1	On barring the UEs not supporting DL CE

In the RAN2#129bis meeting, the following agreements regarding barring the UEs not supporting DL CE were achieved [1]. There remains a remaining issue of whether to introduce a DL-CE-specific bar bit.

Agreements: - From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits; - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring.

In our understanding, the key point in determining whether to bar UEs not supporting DL CE from the repetition enhancement perspective lies in whether the UE can successfully receive the common DL message prior to RRC reconfiguration (Note that UL coverage is not a concern, as the already specified UL coverage enhancements can be reused). Specifically, it involves the reception of SIB1/SIB19/Msg2/Msg4. Let’s examine how the legacy UE operates.

For SIB1, based on the following RAN1 agreement, it can be concluded that the NTN cell with repetition enhancements (e.g., repetition of PDCCH CSS Type0) will repeat the same PDCCH and its associated PDSCH across two consecutive slots n0 and n0+1 associated with the same SSB index. The Rel-19 DL CE capable UEs decode the PDCCH and the associated PDSCH through soft combining. The legacy UE will also detect and decode two consecutive slots n0 and n0+1, thereby achieving the TDM diversity gain based on the existing UE procedure for monitoring Type0-PDCCH CSS sets (see section 13 of 38.213). Even though the TDM diversity gain is somewhat less than that from soft combining, legacy UEs are likely able to acquire SIB1 and successfully access the cell when PDCCH and SIB1 are transmitted with repetition. This observation is also noted in the RAN1 agreement highlighted below.

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

For SIB19, it can be scheduled via the common search space #0 or common search space #1 (CSS for SI-RNTI). In the case of common search space #0, the implementation for SIB1 is reused. That is the legacy UE is independently detecting and decoding two consecutive slots n0 and n0+1 associated with the same SSB index for SIB19. In the case of common search space #1, intra-slot PDCCH repetition is used. That is, the legacy UE will monitor 2 PDCCH occasions from 2 linked search spaces respectively. As a result, the legacy will naturally decode two PDDCH and associated PDSCH carrying SIB19. Regardless of which case, the legacy can obtain the TDM diversity gain, similar to the SIB1 case. According to the RAN1 evaluation provided in the Annex, it can be observed that the coverage gap regarding SIB19 transmission is smaller than that of SIB1 (i.e., 3.5 dB vs 4.1 dB). To a certain extent, if a legacy UE has successfully acquired SIB1, it will generally have no difficulty receiving SIB19. In other words, the legacy UE may still successfully access the NTN cell with repetition enhancement.

Meanwhile, there is no coverage gap for Msg2. As for Msg4, we assume its scenario is similar to that of Msg2. Specifically, the network may either schedule a Msg4 with a small TBS (note that Msg4 repetition is introduced under the assumption that the TBS of Msg4 is 1040 bits) or retransmit Msg4 at the cost of increased PDCCH overhead due to additional HARQ retransmission scheduling. If a cell-edge legacy UE encounters a severe coverage issue, the network can reject the UE via Msg4 (ensuring the UE still retains the chance to monitor PWS and Paging) or the UE may directly camp on another cell because it fails to successfully acquire SIB1/19.

Therefore, the network has no motivation to bar all legacy UEs within its coverage, especially considering that the biggest bottleneck lies in the repetition of SIB1 and SIB19. Once a legacy UE can read SIB1, it indicates that downlink coverage is not a major issue. Why, then, should the network still set a bar for legacy UEs in SIB1?

Observation 1: Legacy UEs may successfully access an NTN cell with repetition enhancements through 2-slot decoding of SIB1/SIB19 and Msg4 retransmission, with the additional consideration that no coverage issues are identified for Msg2.

Observation 2: The biggest bottleneck of DL repetition lies in the repetition of SIB1. Once a legacy UE can read SIB1, it indicates that downlink coverage is not a major issue. Then, it does not make sense for the network to still set a DL-CE-specific bar bit for legacy UEs in SIB1.

Proposal 1: RAN2 confirms there is no need to introduce new barring bits regarding DL CE.

Another remaining issue is whether to further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE. The proponent [2] thinks that introducing an excluded cell list for legacy UE (while the R19 DL CE capable UE will ignore the list) helps to prevent the legacy UE from reading SIB1/19 and trying to access the NTN cells with DL CE, considering that the UE may fail to acquire the SIB1/19 due to poor radio link quality. We believe the excluded cell list solution is overly heavy-handed. As analyzed earlier, legacy UEs have the potential to access NTN cells with DL CE. There may also be scenarios where only the NTN cell with DL CE can provide coverage for a legacy UE, making it advisable to allow the UE to attempt access. Furthermore, if the network aims to guide legacy UEs toward more suitable cells, it would be better to prioritize the corresponding frequencies rather than preventing these UEs from attempting to access the NTN cell with DL CE.

Proposal 2: RAN2 does not consider any enhancement to allow UEs not supporting DL CE to down-prioritize re-selection to cells operating with DL CE.

## 2.2	Support of Msg4 PDSCH repetition

Agreement For the activation/deactivation of Msg4 PDSCH repetition: - Alt 1: UE specific PDSCH with Msg4 repetition activation indicated via DCI Format 1_0: - Signaling uses re-interpretation of 1 MSB in MCS field in DCI. - A UE capable of Msg4 PDSCH repetition may report its capability/request in Msg3 PUSCH. - Note: RAN1 considers there is no difference between capability and request - FFS: whether to specify condition(s) for the UE to report its capability/request. Such conditions may be discussed in RAN1 or other WGs. - The aggregation factor is configured in SIB1, with possible value 2 or 4 - When the aggregation factor is configured in SIB1, the PDSCH MSG4 repetition is enabled.

In the LS R2-2505024, RAN1 kindly requests RAN2 to consider the above agreement when designing the report included in Msg3 PUSCH. In our understanding, there are 2 remaining issues,

- whether to specify condition(s) for the UE to report its capability/request

- How to indicate UE’s capability/request for Msg4 repetition

In our understanding, the RSRP-based UE request mechanism employed for HARQ-ACK in Msg1/3/ Msg4 HARQ ACK is unnecessary in the scenario of Msg4 repetition. Specifically, Msg4 is transmitted only after the successful transmission of Msg2 and Msg3, which implies that the network can already perceive the radio conditions based on the transmission processes of Msg2 and Msg3. At this point, the network has obtained sufficient information to determine the scheduling of Msg4; for instance, it can decide on the scheduling scheme for Msg4 according to the performance gap between Msg2 and Msg4. Since the UE request fails to provide additional useful information, there is no need to introduce the RSRP-based UE request mechanism.

Proposal 3: RAN2 assumes there is no need to introduce RSRP-based request for Msg4 PDSCH repetition.

Regarding the indication of UE’s capability/request for Msg4 repetition in Msg3, we think the simplest and efficientest way is to use the reserved values (with 56 remaining reserved values) of LCID for UL-SCH, specified in the (LX)/R/LCID MAC subheader. Next, we need to consider feasible combinations involving (e)RedCap, PUCCH repetition for Msg4 ACK, and Msg4 PDSCH repetition. We maintain that all combinations are feasible (NOTE that both RedCap NTN UE and (e)RedCap UE are also taken into account). It is noted that [3] proposes coupling the Msg4 PDSCH repetition with the Msg4 HARQ ACK repetition. However, we are not convinced by this view. Msg4 repetition is primarily intended for Set1-3 scenarios, which arise due to satellite downlink power constraints. This does not imply that uplink coverage enhancement is always necessary. Instead, we should allow more flexibility for implementation to accommodate diverse use cases. Additionally, RAN1 has not provided such input either. Therefore, it is more prudent to consider a comprehensive set of combinations.

Proposal 4: The following LCID values are considered for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE.

Proposal 5: Add a NOTE in Table 6.2.1-2c of TS 38.321 that the MAC entity uses the code points corresponding to Msg4 PDSCH repetition for UE capable of Msg4 PDSCH repetition.

For Proposals 4 and 5, a corresponding TP is given below for reference.

Table 6.2.1-2c: Values of LCID for UL-SCH when the LX field is set to 1 [38.321]

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 | (216 + 320) | CCCH of size 48 bits for an eRedCap UE |
| 1 | (216 + 321) | CCCH of size 64 bits for an eRedCap UE |
| 2 | (216 + 322) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 3 | (216 + 323) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 4 | (216 + 324) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 5 | (216 + 325) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 6 | (216 + 326) | CCCH of size 48 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 7 | (216 + 327) | CCCH of size 64 bits for PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 8 | (216 + 328) | CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 9 | (216 + 329) | CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE |
| 10 | (216 + 330) | CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE |
| 11 | (216 + 331) | CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE |
| 12 | (216 + 332) | CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 13 | (216 + 333) | CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE |
| 14 | (216 + 334) | CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 15 | (216 + 335) | CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE |
| 16 | (216 + 336) | CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 17 | (216 + 337) | CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE |
| 18 | (216 + 338) | CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 19 | (216 + 339) | CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE |
| 20 to 63 | (216 + 340) to (216 + 383) | Reserved |
| NOTE 1: The MAC entity may use the code point corresponding to a given feature or feature combination in Table 6.2.1-2c only if network indicates support for the corresponding feature or feature combination. NOTE 2: CCCH of size 48 bits and CCCH of size 64 bits are referred to as CCCH and CCCH1, respectively, in TS 38.331 [5]. NOTE 3: For UE capable of PUCCH repetition of Msg4 HARQ-ACK, the MAC entity uses the code points corresponding to PUCCH repetition of Msg4 HARQ-ACK if numberOfMsg4HARQ-ACK-Repetitions is configured and rsrp-ThresholdMsg4HARQ-ACK is not configured, or if both are configured and the RSRP of the downlink pathloss reference is less than rsrp-ThresholdMsg4HARQ-ACK. NOTE 4: For UE capable of Msg4 PDSCH repetition, the MAC entity uses the code points corresponding to Msg4 PDSCH repetition. |  |  |

# 3 Conclusion

In this contribution, we have discussed the remaining issues regarding repetition enhancements in Rel-19 NTN. And we make the following observations and proposals,

On barring the UEs not supporting DL CE:

Observation 1: Legacy UEs may successfully access an NTN cell with repetition enhancements through 2-slot decoding of SIB1/SIB19 and Msg4 retransmission, with the additional consideration that no coverage issues are identified for Msg2.

Observation 2: The biggest bottleneck of DL repetition lies in the repetition of SIB1. Once a legacy UE can read SIB1, it indicates that downlink coverage is not a major issue. Then, it does not make sense for the network to still set a DL-CE-specific bar bit for legacy UEs in SIB1.

Proposal 1: RAN2 confirms there is no need to introduce new barring bits regarding DL CE.

Proposal 2: RAN2 does not consider any enhancement to allow UEs not supporting DL CE to down-prioritize re-selection to cells operating with DL CE.

Support of Msg4 PDSCH repetition:

Proposal 3: RAN2 assumes there is no need to introduce RSRP-based request for Msg4 PDSCH repetition.

Proposal 4: The following LCID values are considered for Msg4 PDSCH repetition:

- CCCH of size 48 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition, except for an (e)RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of a RedCap UE;

- CCCH of size 48 bits for Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 64 bits for Msg4 PDSCH repetition of an eRedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK, except for an (e)RedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of a RedCap UE;

- CCCH of size 48 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE;

- CCCH of size 64 bits for both Msg4 PDSCH repetition and PUCCH repetition of Msg4 HARQ-ACK of an eRedCap UE.

Proposal 5: Add a NOTE in Table 6.2.1-2c of TS 38.321 that the MAC entity uses the code points corresponding to Msg4 PDSCH repetition for UE capable of Msg4 PDSCH repetition.

# 4 Reference

[1] R2-2503302, RAN2#129bis Meeting Report, MCC.

[2] R2-2504079, Consideration on downlink coverage enhancements, ZTE Corporation, Sanechips.

[3] R2-2501798, Discussion on DL coverage enhancement, Xiaomi.

# 5 Annex

Observation

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is around 4 dB compared to CNR of -1.9 dB.

  - With parameter LEO600km Set1-3 FR1:

    - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB.

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH.

Observation

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 9 dB compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 1 dB on average compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg2.

Observation

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

      - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

        - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

      - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB

    - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB

Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4.

Observation

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

        - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB

    - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

        - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

        - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB

    - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

        - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

    - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

        - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB

- Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Observation

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB on average compared to CNR of -1.9 dB

  - With parameter LEO600km Set1-3 FR1:

    - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB on average compared to CNR of -9.9 dB

- Note: all the results above assumed no SIB19 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.