3GPP TSG-RAN WG2 Meeting #131		R2-2505086

Bangalore, India, 25th – 29th August 2025

Source:	vivo

Title:	(SRAP-1/RRC-13) Discussion on SRAP configuration and timer extension in multi-hop relay

Agenda Item:		8.13.3

Document for:	Discussion and Decision

- Introduction

This contribution is to discuss the identified open issues in control plane for multi-hop relay, as follows:

- Open issue SRAP-1: SRAP configuration of indirectly connected Remote UE;

- Open Issue RRC-13: Fine Tuning of Extended Timer Values Covering Both Uu and PC5 Hops.

- Discussion

  - SRAP-1: SRAP configuration of indirectly connected Remote UE

The issue is explained as follows[1]:

Issue description: Relevant agreement: SRAP configuration including remote UE ID and BEARER ID is configured by the network via dedicated RRC signalling as in R17. For the SRAP configuration at the relay UE (both last relay and intermediate relay UE), rely on network to provide SRAP configuration for at least the directly connected child UEs, based on the L2 IDs of the direct connected child UEs. For CR drafting, assume as a baseline that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID for next-hop determination. We will review next meeting if this explicit mapping functionality is needed. FFS whether to introduce reflective bearer mapping as an optional feature. Status in running CR： Based on the above RAN2 agreements, the running CR is drafted according to the baseline solution that the child UE’s SRAP configuration can include entries for indirect child UE with associated local ID. And related Editor’s Notes are added in clause 4.5 and 5.2.2.1 as follows: EN at clause 4.5: Editor’s Notes: FFS whether the UE ID for indirectly connected U2N Remote UE is needed. EN at clause 5.2.2.1: Editor’s Notes: FFS whether the SRAP configuration list for the indirectly connected Remote UE is needed. Main points of contention: whether to introduce reflective bearer mapping. And if so, how does the reflective bearer mapping mechanism work needs to be further clarified. Possible options: Option-1: Besides the baseline mechanism specified in the running CR, additionally support the reflective bearer mapping as optional feature. For this option, how does the reflective bearer mapping work needs to be clarified. Option-2: Not to additionally support the reflective bearer mapping.

This issue arises because in multi-hop relay, last relay UE and intermediate relay do not have direct PC5 connection with remote UE, so they cannot know the remote UE’s L2 ID. According to RAN2 #129bis meeting agreements as shown above[2], the current running CR is as follows[3]:

#### SL-L2RelayUE-Config The IE SL-L2RelayUE-Config is used to configure L2 U2N relay operation related configurations used by L2 U2N Relay UE, or L2 U2U relay operation related configurations used by L2 U2U Relay UE. SL-L2RelayUE-Config information element -- ASN1START -- TAG-SL-L2RELAYUE-CONFIG-START SL-L2RelayUE-Config-r17 ::= SEQUENCE { sl-RemoteUE-ToAddModList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-RemoteUE-ToAddMod-r17 OPTIONAL, -- Need N sl-RemoteUE-ToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-DestinationIdentity-r16 OPTIONAL, -- Need N ..., [[ sl-U2U-RemoteUE-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-U2U-RemoteUE-Config-r18 OPTIONAL, -- Need N sl-U2U-RemoteUE-ToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-DestinationIdentity-r16 OPTIONAL -- Need N ]] } SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE { sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M ..., [[ sl-SRAP-ConfigRelay-ToAddModList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-Config-ToAddMod-r19 OPTIONAL, -- Need R sl-SRAP-ConfigRelay-ToReleaseList-r19 SEQUENCE (SIZE (1..maxNrofRemoteUE-r17)) OF SL-SRAP-ConfigId-r19 OPTIONAL -- Need R ]] } SL-U2U-RemoteUE-Config-r18 ::= SEQUENCE { sl-L2IdentityRemoteUE-r18 SL-DestinationIdentity-r16, sl-SourceRemoteUE-ToAddModList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SourceRemoteUE-Config-r18 OPTIONAL, -- Need N sl-SourceRemoteUE-ToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-SourceIdentity-r17 OPTIONAL, -- Need N ... } SL-SourceRemoteUE-Config-r18 ::= SEQUENCE { sl-SourceUE-Identity-r18 SL-SourceIdentity-r17, sl-SRAP-ConfigU2U-r18 SL-SRAP-ConfigU2U-r18, ... } SL-SRAP-Config-ToAddMod-r19 ::= SEQUENCE { sl-SRAP-ConfigId-r19 SL-SRAP-ConfigId-r19, sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 ... } -- TAG-SL-L2RELAYUE-CONFIG-STOP -- ASN1STOP sl-SRAP-ConfigRelay-ToAddModList List of SRAP configuration for each indirectly connected child UE in the multi hop case. sl-SRAP-ConfigRelay-ToReleaseList List of SRAP configuration be released for each indirectly connected child UE in the multi hop case.

For current running CR, the SRAP configuration is working at e.g., intermediate Relay UE as follows:

- The network put the directly connected first relay UE L2 ID=1 in sl-L2IdentityRemote-r17;

- If first relay UE connects to remote UE L2 ID=2/3/4, then network put the remote UE local ID=x/y/z in sl-LocalIdentity-r17 inside SL-SRAP-Config-r17.

It can be seen that the current running CR already works well.

Observation 1: Current running CR can already work by introducing sl-SRAP-ConfigRelay-ToAddModList to include indirect UE’s SRAP configuration at U2N relay UEs.

Meanwhile the motivation for reflective mapping is as follows[4]:

Next hop UE ID determination: Regarding DL next hop UE ID, the intermediate relay records the node from which the UL remote UE’s packets are received. Then the intermediate relay forwards the remote UE’s packets to the same node in the downlink. RLC channel determination: For the downlink RLC channel determination, the relay UE forward the DL packet to same RLC channel on which the UL packet is received.

For this mechanism, the result is that the network will put indirect remote UE’s L2 ID in sl-L2IdentityRemote-r17 at e.g., intermediate Relay UE configuration. In our understanding, the main difference is that the signaling configuration is nested or not. As the current running CR already works, it is ok not to have further optimization as anyway we understand there may not be big overhead on this.

Observation 2: Reflective mapping mechanism is more about optimization on signalling to avoid nested configuration.

However, there are some concerns for how this reflective mapping can work, e.g., for indirect remote UE traffic, the intermediate relay UE cannot know the next hop local UE ID although it may know where to forward the packet, so error handling may not work well. In order to save the time to avoid discussion on how to make the reflective mapping work, and to avoid discussion on the concrete spec impact brought by reflective mapping, we suggest to just rely on current running CR for SRAP configuration for indirect remote UE configuration.

Observation 3: More time is needed to discuss whether the reflective mapping work for error handling, as well as the concrete spec impact brought by reflective mapping.

Proposal 1: (SRAP-1) For multi-hop SRAP configuration, confirm the signalling design in current running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE).

  - RRC-13: Fine Tuning of Extended Timer Values Covering Both Uu and PC5 Hops

This issue is explained as follows[5]:

In the RAN2 # 130 meeting, we agreed to extend the T300, T301 and T319 timers for multi-hop U2N relay operation such that the UE and gNB will multiply the signalled timer value by the hop count. In the current running CR, the Remote UE timer values, UE-TimersAndConstantsRemoteUE, which include components from both the Uu and PC5 hops, were considered to be multiplied by the hop count. However with this approach the value of the timer for the multi hop remote UE might be quite large as the Remote UE timer already considers one PC5 hop and one Uu hop and multiplying it with the hop count will result in higher values. Some of the companies think that the time required for the RRC Connection Setup may increase exponentially when the number of hops increases so we may need higher values of the timers. There are four possible options to fine tune the timer values. Option 1 - Instead of multiplying the timers in UE-TimersAndConstantsRemoteUE with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count. Option 2 – We introduce a new timer for a PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5 and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n. Option 3 – We multiply the timers in UE-TimersAndConstantsRemoteUE with the hop count as currently captured in the CR Option 4 - Further extend the timer T300 to cover setup times that grow exponentially with the number of hops in UE-TimersAndConstantsRemoteUE Option 1 seems simpler but less accurate while Option 2 seems to be more accurate but would require introducing new PC5 timers. Option 3 preserves the current implementation but may lead to unnecessarily prolonged timers in multi-hop configurations. Option 4 needs further extension of timers Proposal 7 – RAN2 to discuss if we need to further fine tune the timers value calculation for the multihop scenario.

This issue was discussed in RAN2 #130 and the current running CR based on agreements is as follows[3]:

UE-TimersAndConstantsRemoteUE information element

-- ASN1START

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-START

UE-TimersAndConstantsRemoteUE-r17 ::= SEQUENCE {

t300-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t301-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

t319-RemoteUE-r17                     ENUMERATED {ms100, ms200, ms300, ms400, ms600, ms1000, ms1500, ms2000} OPTIONAL, -- Need S

...

}

-- TAG-UE-TIMERSANDCONSTANTSREMOTEUE-STOP

-- ASN1STOP

UE-TimersAndConstantsRemoteUE field descriptions

t300-RemoteUE Indicates the timer value of T300 used by L2 U2N Remote UE. The effective T300 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T300 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t300 applies to L2 U2N Remote UE.

t301-RemoteUE Indicates the timer value of T301 used by L2 U2N Remote UE. The effective T301 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components, is obtained by multiplying the base T301 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t301 applies to L2 U2N Remote UE.

t319-RemoteUE Indicates the timer value of T319 used by L2 U2N Remote UE. The effective T319 value for the L2 U2N Remote UE, accounting for both the Uu and PC5 hop components,, is obtained by multiplying the base T319 timer value by the Hop Count. For a single-hop scenario involving one Relay UE, the Hop Count is 1. For multi-hop scenarios involving two or three Relay UEs, the Hop Count is 2 or 3, respectively. If the field is absent, the timer value indicated in t319 applies to L2 U2N Remote UE.

We understand it is true that the current value-multiplying handling is not so accurate.

Before we go for options proposed by rapporteur, we think it is better to first clarify whehter there would be problems that the if the final value after multiplying t300-RemoteUE with Hop Count by UE itself, execeed the maximum value 2000 ms. In our understanding, it is an abnormal case that the network configure UE with a timer with a value, but the UE multiply it with a count and use that as the new value for this timer, meanwhile this new value even exceeds the maximum value possibly configured for the original timer. We understand it is not clear whether it is aligned with the network configuration and should be clarified.

Observation 4: If the UE multiply t300 or t300-remoteUE with a hop count and use that as the new value for this timer, meanwhile this new value even exceeds the maximum value possibly configured for the original timer (2000 ms), it is not clear whether it is aligned with the network configuration and whether it needs new UE capability.

If the above issue is clarified, regarding the options on table, we think the simplest one is to adopt the current running CR, i.e., option-3. Although it is not accurate, it is safer to have a relative larger timer length for multi-hop, as the time required for the RRC Connection Setup may increase exponentially in multi-hop case. Also, the spec impact is minimized as we only changed the field description and it is already implemented in the running CR. Therefore, we propose:

Proposal 2: (RRC-13) RAN2 to clarify whether it is aligned with the network configuration and whether it needs new UE capability when the new T300 for multi-hop (after multiplying) exceeds the maximum value possibly configured for the original timer (2000 ms).

Proposal 3: (RRC-13) Option-3 is confirmed as in current running CR, i.e., multiply the timers in UE-TimersAndConstantsRemoteUE with the hop count.

- Conclusion

In this contribution, we discussed SRAP-1 and RRC-13 for control plane procedure.

We have the following Observations and Proposals:

SRAP-1

Observation 1: Current running CR can already work by introducing sl-SRAP-ConfigRelay-ToAddModList to include indirect UE’s SRAP configuration at U2N relay UEs.

Observation 2: Reflective mapping mechanism is more about optimization on signalling to avoid nested configuration.

Observation 3: More time is needed to discuss whether the reflective mapping work for error handling, as well as the concrete spec impact brought by reflective mapping.

Proposal 1: (SRAP-1) For multi-hop SRAP configuration, confirm the signalling design in current running CR (i.e., child UE’s SRAP configuration includes entries for indirect child UE).

RRC-13

Observation 4: If the UE multiply t300 or t300-remoteUE with a hop count and use that as the new value for this timer, meanwhile this new value even exceeds the maximum value possibly configured for the original timer (2000 ms), it is not clear whether it is aligned with the network configuration and whether it needs new UE capability.

Proposal 2: (RRC-13) RAN2 to clarify whether it is aligned with the network configuration and whether it needs new UE capability when the new T300 for multi-hop (after multiplying) exceeds the maximum value possibly configured for the original timer (2000 ms).

Proposal 3: (RRC-13) Option-3 is confirmed as in current running CR, i.e., multiply the timers in UE-TimersAndConstantsRemoteUE with the hop count.

- References

- R2-250XXXX - SRAP open issues for NR sidelink multi-hop relay_V02_Rapp;

- RAN2 #129bis meeting chair notes;

- R2-250XXXX Merged RRC CR for R19 Multihop_SL_Relay_v01_Rapp;

- RAN2 #130 meeting chair notes;

- R2-250XXXX Open issues for Multi hop Sidelink Relay in TS 38.331_V03_Rapp;