3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505093

Bangalore, India Aug 25th – 29th, 2025

Source:	CATT

Title:	Discussion on the A-IoT Data Transmission

Agenda Item:	8.2.4

Document for:	Discussion

1.	Introduction

This paper further discusses the data transmission related issues for Ambient IoT. In details, the following aspects are covered by our contribution,

- More data indication handling

- R2D TBS

Section 2 gives the detailed discussion, and followed by the Section 3 that summarizes the conclusion of this paper.

2.	Discussion

## 2.1	More data indication handling (Issue 3-7)

There is an FFS issue on more data indication at RAN2#130 post email discussion [1]:

| (New)Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - During CR implementation, the rapp identified there is no clear conclusion how to set the “more data indication” in case of no data available, i.e., zero SDU. - Status in running CR: captured as value FFS. | Companies are invited to input views for Q#9 |
| --- | --- | --- |

According to the online discussion among companies, there are the following two options on the table:

- Option 1: set More Data Indication field to value 1 in case of no NAS response available

- Option 2: set More Data Indication field to value 0 in case of no NAS response available

According to the current MAC running CR, the “More data indication” is a MAC-layer indication, which represents the segment of the MAC PDU due to not a large enough D2R TBS, but not to indicate upper layer data.

Observation 1:	According to the current MAC running CR, the “More data indication” is a MAC-layer indication for segmentation, but not to indicate upper layer data.

For Option 1, when this field is set to value 1, the expected reader behavior is to schedule a new D2R transmission, but there may be the case where device still does not complete the write operation.

For option 2, when this field is set to value 0, according to the current running CR implementation, i.e., zero SDU and all MAC padding in D2R MAC PDU, reader can understand that current situation belongs to no NAS response available case. Then it can be up to reader implementation to re-schedule a new D2R resource after receiving the D2R MAC PDU with zero SDU and all MAC padding. Therefore, we prefer to Option 2.

Proposal 1:	(3-7) Set “More Data Indication” field to value 0 in case of no NAS response available for the write command.

## 2.2	R2D TBS (Issue 3-8)

There is an FFS issue on R2D TBS at RAN2#130 post email discussion [1]:

| Subgroup: R2D message content for data transmission |  |  |
| --- | --- | --- |
| (New) Issue 3-8: R2D TBS | How to handle the R2D TBS, which may impact R2D padding, byte-alignment design. - Relevant agreements: - RAN1 LS in R1-2504915 - The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed) - Status in running CR: not captured yet. | Companies are invited to input views for Q#10.1, Q10.2 |

In RAN1#121 meeting, RAN1 sends LS in R1-2504915 [2] to inform RAN2 considering TBS information for R2D message via L2 control signalling.

RAN1#121 agreement: R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified.

Regarding to whether/how to add a R2D TBS field for the variable size R2D messages (i.e., Paging message, Access Trigger message, Random ID Response message, R2D Upper Layer Data Transfer message, NACK Feedback message), the rapporteur suggests to add a 7-bit R2D TBS field (in unit of byte) after R2D message type indication within the variable-length R2D messages.

In our opinion, before decoding the variable-size R2D messages, device should firstly check the CRC attached to the transport block carrying the R2D message, which ensures the reliable R2D message transmission over PRDCH. That is to say, the device needs to know the TB in early so that it can perform CRC checking. If the R2D TBS field is directly added into the variable-size R2D messages, we are not sure whether it meets the RAN1 intention.

Observation 2:	Taking the CRC checking into account, RAN2 should be cautious with adding the R2D TBS field into the variable-size R2D messages directly, which may not align with the RAN1 intention.

To solve above concern, from RAN2 perspective, benefited from the 7-bit TBS with a fixed size, a potential candidate solution is before sending a variable-size R2D message, the reader can firstly send a new R2D message containing this TBS field, i.e., a new R2D message with 3-bit R2D message type and 7-bit TBS field. By this way, the device can perform CRC checking based on the TBS indication before decoding any field of the variable R2D message.

However, LS to RAN1 is also fine to us to finally confirm the RAN2 design is aligned with the RAN1 original intention. Therefore, we propose that,

Proposal 2:	(3-8) LS to RAN1 whether it is feasible to include the R2D TBS field after the R2D message type indication field in the variable-size R2D messages.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

More data indication handling (Issue 3-7)

Observation 1:	According to the current MAC running CR, the “More data indication” is a MAC-layer indication for segmentation, but not to indicate upper layer data.

Proposal 1:	(3-7) Set “More Data Indication” field to value 0 in case of no NAS response available for the write command.

R2D TBS (Issue 3-8)

Observation 2:	Taking the CRC checking into account, RAN2 should be cautious with adding the R2D TBS field into the variable-size R2D messages directly, which may not align with the RAN1 intention.

Proposal 2:	(3-8) LS to RAN1 whether it is feasible to include the R2D TBS field after the R2D message type indication field in the variable-size R2D messages.

4. References

[1] Draft_A-IoT MAC open issues_v38_Rappv2

[2] R1-2504915 LS on Ambient IoT R2D control information