3GPP TSG-RAN WG2 Meeting #131	R2-2505116

Bangalore, India  Aug 25th – 29th , 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on NW-sided model

Agenda Item:	8.3.3

Document for:	Discussion and Decisions

- Introduction

During the RAN#108 meeting, the new WID on “Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The WID describes the objective of the work on NW-sided model as below,

- Specify support for RRM measurement prediction for both UE and network sided models [RAN2]:

- Specify signalling and protocol aspect to enable reporting inference outcome of UE sided model for RRM measurement prediction

- Specify signalling and protocol aspect to enable reporting information from UE for inference operation of network sided model for RRM measurement prediction

- At least the use cases studied so far in RAN2 SI are supported: Temporal domain prediction case A and case B, and Frequency domain prediction for collocated cases. Cell level predication is supported

Note 1: For UE sided model, spatial domain is not supported. Can check in RAN#109 whether for network sided model spatial domain is supported or not, based on the final outcome of the SI.

Note 2: Check in RAN#109 whether L3 beam level prediction is also supported, based on the final outcome of the SI.

- For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]:

- Network sided data collection, at least

    - UE is configured to log RRM measurement results.

    - UE supports periodical logging and L3 event triggered logging.

    - UE can indicate logged data availability.

This paper discusses the NW-sided model, including the scenarios/(sub)use cases to be supported, reporting information from UE for inference operation, performance monitoring, and data collection.

- Scenarios/(sub)use cases

At the RAN2 #130 meeting, the following agreements were achieved on scenarios/sub-use cases/domains:

Agreements

1 The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 assumes that a network-sided model requires no specification impacts.  Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for the same time instance of another cell(s).

2 The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

According to the RAN2 agreements and new WID, spatial domain prediction with UE-sided model in both cell dimension and beam dimension will not be supported in 5GA Rel-20.  In addition, the spatial domain prediction with NW-sided model in cell dimension requires no specification impacts. Therefore, the leftover issue to be discussed is whether spatial domain prediction in beam dimension with NW-sided model should be supported in Rel-20 specification or not. According to simulation evaluation results, it is observed that cluster-based prediction, i.e., measurement results from a set of cells are used as input to the model, could provide better performance than single-cell based spatial domain prediction. Besides, if the NW-sided model is assumed, the specification impact will only about L1 beam-level measurement results as the input for inference operation, which has a lot in common with the other scenarios.

Proposal 1: Support intra-frequency cluster-based spatial domain prediction in beam dimension with NW-sided model for Rel-20 specification.

Furthermore, the other issue is the identified scenarios and sub-use cases to be supported for NW-sided model. Firstly, we believe that NW could have higher computation capability, power resource and diverse information which could enable equivalent or even better performance than UE-sided model on measurement prediction. Therefore, we think all the identified scenarios(temporal domain, frequency domain) and sub-use cases(sub-use case 1~3) for UE-sided model should also be supported for NW-sided model. One new sub-use case brought up is the L3 beam-level prediction(sub-use case 4), and we believe the inference input of this sub-use case is the same as the sub-use case 1 for NW-sided model. Therefore, we have following proposal and summarize all the scenarios and sub-use cases that should be supported for Rel-20 specification in Table 1.

Proposal 2: Support the L3 beam-level prediction with NW-sided model for Rel-20 specification.

Proposal 3: Support the scenarios and sub-use cases in the Table.1 for Rel-20 specification.

Table.1 Supported scenarios and sub-use cases for Rel-20 specification

| Scenario |  | UE-sided model | NW-sided model |
| --- | --- | --- | --- |
| RRM prediction | Temporal domain Case A | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Temporal domain Case B | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Frequency domain | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Spatial domain in beam dimension | -- | Sub-use case 1,3,4 |
| Event prediction |  | Indirect, Direct | -- |

- Reporting information from UE for inference operation

At the RAN2 #130 meeting, the following agreements were achieved on inference input for NW-sided model:

Agreements

1 For network-side RRM measurement prediction, the legacy RRM measurement configuration and RRM measurement reporting framework can be used.   For L1-filtered beam-level RSRP reporting can be configured by setting co-efficient to zero.   FFS if there are specification impacts to support subcase 1 and 3 (if supported) and interference for cell level prediction.

2 To support cell/beam level prediction, one enhancements is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.

As described in Section 2 and Figure.1, for NW-sided model, sub-use case 1,3,4 should be specified for all the identified scenarios. Since L1 beam-level measurement results is the inference input for sub-use case 1,3,4, the L3 beam-level measurement values with co-efficient zero could be used as agreed at last meeting. In addition, since L3 beam-level measurement report is optionally configured, NW needs to configure the reportQuantityRS-Indexes/csi-RS-IndexResults/cri-RSRP / cri-RI-PMI-CQI accordingly to enable the corresponding L3 beam-level measurement.

The inference input of sub-use case 2 is L3 cell-level measurement results, which can be obtained by legacy RRM measurement framework. At the last meeting, the enhancement was agreed on multiple time instances in one measurement report for NW-sided model, which is mainly for overhead reduction for temporal domain prediction. When other scenarios are also considered, it would be beneficial to extend the enhancement in more domains.

Proposal 4: The following report information is configured for inference operation with NW-side model.

- Sub-use case 1,3,4: L1-filtered beam-level measurement results are reported as L3 beam-level measurement results by setting coefficient to zero, which is enabled by appropriate reportQuantityRS-Indexes/csi-RS-IndexResults/cri-RSRP / cri-RI-PMI-CQI parameter setting.

- Sub-use case 1~4: RRM measurement results at multiple beams/cells/time instances/frequencies.

Figure.1 Inference input for sub-use cases

- Performance monitoring

At the RAN2 #130 meeting, the following agreements were achieved on performance monitoring for NW-sided model:

Agreements

1 RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case

2  For performance monitoring of NW-side model, UE can provide the label data (i.e., actual measurement results) for gNB. The existing measurement/report configuration can be reused to configure UE to report the actual measurement

3 FFS on UE awareness and preference for NW sided model

According to the discussion and agreement for AI4PHY, the UE does not have any awareness and preference for NW-sided model. The gNB is responsible for monitoring its own performance and UE is just configured to provide the necessary label data. However, at the last meeting, some requirements had been raised for AI4MOB on the UE awareness of the on-going performance monitoring which needs the actual measurement results as the label data, and preference from UE that whether it has the capability or whether it would like to conduct measurement to assist NW-sided model performance monitoring. From operator point of view, NW should be able to configure the measurement report to guarantee the performance even without AI/ML model in legacy specification. In addition, comparing with AI4PHY, the measurement for label data is based on RRM measurement framework whose efforts is less than CSI framework, so that a clear motivation of requiring additional UE awareness and preference for possibly avoiding measurement report should be verified. Even if it is necessary that UE is aware of the NW on-going performance monitoring and UE is allowed to provide the preference on measurement, the final decision should still be made by NW.

Proposal 5:

- UE awareness and preference on measurements for performance monitoring for NW-sided model is NOT supported unless the motivation is verified.

- NW is responsible for the final decision on the measurement for performance monitoring for NW-sided model if UE awareness and preference is supported

- Data collection

At the RAN2 #130 meeting, the following agreements were achieved on NW-side data collection [2],

Agreements

For the collection of data for the training of a network-sided model for RRM measurement prediction:

- UE can be configured to log, at a certain logging periodicity, one or more of the following:

- L3 cell level measurements,

- L3 beam level measurements,

- L1-filtered beam level measurements (if sub-case 1 and 3 is supported)

- Cell ID (FFS CGI of serving cell.  If CGI is unavailable, or for neighboring cells the UE logs PCI-ARFCN as a fallback)

- Time info (if as agreed by AI/ML and/or if needed)

- The UE configuration is provided via the RRM measurement framework. Whether we reuse existing measconfig structure or we need a separate data logging configuration is for WI phase.   Required enhancements are FFS.

- The UE can be configured with a L3 event for determining when logging is to be performed. When the event conditions are fulfilled, it performs the logging with the logging periodicity.

- The UE sends availability indication of collected logged data via UAI or RRCReconfigurationComplete message for HO case.

- The availability indication can be triggered due to:

- full buffer being reached (if configured), (FFS if buffer is per use case)

- buffer threshold being reached (if configured), or (FFS if buffer is per use case)

- low power (if configured)

- Upon sending the availability indication, UE indicates:

- Data is available

- Reason for the triggering of the availability indication (full buffer. Threshold)

- Low power indication

- The UE sends the collected data upon explicit/on-demand request from the network (using UEInformationRequest/Response signaling)

- The UE keeps the collected data upon HO, unless explicitly indicated to release it by the network (e.g., during HO).

- The UE releases the collected data upon transitioning to IDLE/INACTIVE

- For RLF case, capture in the TR that for mobility keeping the data during RLF can be beneficial.   It can be up to WI phase if this is done depending on whether a simple solution can be found

For UE sided data collection – use AI/ML PHY request/configuration framework as baseline.  FFS enhancements/or differences

Since the amount and distribution of dataset may have significant impact on the AI/ML performance, the data collection for training the NW-sided model may require the measurement results of neighboring cells. If the Cell ID is defined as CGI for serving cell and neighboring cell in the data logging, the measurement effort of UE containing the SIB reading will be largely increased. In practice, the PCI and ARFCN is sufficient to identify a cell. Moreover, if both CGI and PCI-ARFCN are used, the content of logged data will be inconsistent and complicate the data processing for successive training. Therefore, regarding the content to be logged, we think PCI and ARFCN are enough for both serving cell and neighboring cell for consistent content of data logging.

Proposal 6: PCI-ARFCN is used as baseline for data logging of both serving cell and neighboring cells.

Although the event-triggered logging is supported for both AI4PHY and AI4Mob, the motivation is relatively different: The event for AI4PHY logging is to ensure the radio condition is good enough to get the valid dataset; however, the event for AI4Mob logging is to get the dataset with certain features. Therefore, the L3 event for determining the logging should reflect the data feature explicitly.

Proposal 7: The L3 event for data logging should reflect the feature of the dataset.

For AI4Mob, the data collected when HO or RLF is beneficial. Therefore, UE should be able to retain the logged data when RLF happens and retrieve the logged data after the RRC reestablishment procedure as configured by NW.

Proposal 8

- UE is able to retain the logged data when RLF happens and retrieve the logged data after the RRC reestablishment procedure as configured by NW.

  - UE may be configured to retain/release the logged data before RLF or during RRC reestablishment procedure.

  - UE may be able to send availability information during or after RRC reestablishment procedure.

- Conclusion

In this contribution, we have the following proposals on NW-sided model,

- Scenario and sub-use cases:

Proposal 1: Support intra-frequency cluster-based spatial domain prediction in beam dimension with NW-sided model for Rel-20 specification.

Proposal 2: Support the L3 beam-level prediction with NW-sided model for Rel-20 specification.

Proposal 3: Support the scenarios and sub-use cases in the Table.1 for Rel-20 specification.

Table.1 Supported scenarios and sub-use cases for Rel-20 specification

| Scenario |  | UE-sided model | NW-sided model |
| --- | --- | --- | --- |
| RRM prediction | Temporal domain Case A | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Temporal domain Case B | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Frequency domain | Sub-use case 1,2,3,4 | Sub-use case 1,2,3,4 |
|  | Spatial domain in beam dimension | -- | Sub-use case 1,3,4 |
| Event prediction |  | Indirect, Direct | -- |

- Reporting information from UE for inference operation:

Proposal 4: The following report information is configured for inference operation with NW-side model.

- Sub-use case 1,3,4: L1-filtered beam-level measurement results are reported as L3 beam-level measurement results by setting coefficient to zero, which is enabled by appropriate reportQuantityRS-Indexes/csi-RS-IndexResults/cri-RSRP / cri-RI-PMI-CQI parameter setting.

- Sub-use case 1~4: RRM measurement results at multiple beams/cells/time instances/frequencies.

- Performance monitoring:

Proposal 5:

- UE awareness and preference on measurements for performance monitoring for NW-sided model is NOT supported unless the motivation is verified.

- NW is responsible for the final decision on the measurement for performance monitoring for NW-sided model if UE awareness and preference is supported

- Data collection:

Proposal 6: PCI-ARFCN is used as baseline for data logging of both serving cell and neighboring cells.

Proposal 7: The L3 event for data logging should reflect the feature of the dataset.

Proposal 8

- UE is able to retain the logged data when RLF happens and retrieve the logged data after the RRC reestablishment procedure as configured by NW.

  - UE may be configured to retain/release the logged data before RLF or during RRC reestablishment procedure.

  - UE may be able to send availability information during or after RRC reestablishment procedure.

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #130 Chair Notes”, May, 2025.