3GPP TSG-RAN WG2 Meeting #131	R2-2505122

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.2.3

Source: 	Xiaomi

Title:  	Remaining open issues on access procedure for A-IOT

Document for:	Discussion and Decision

# 1	Introduction

In the email discussion [POST130][027][AIoT] MAC Running CR (Huawei), A-IoT MAC CR Rapporteur listed open issues for Paging, Random access and data transmission.

In addition, in RAN2#130, the following agreements were made:

- Agreements on RA - 1 Exclude the option of MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer) 2 A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below. A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary. - Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message - Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS) - Option A (the boundary being the subsequent paging only) is excluded. For option C, further discuss in terms of complexity at the device vs reader flexibility. 3 Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible. FFS Discuss further whether to include it.

In this contribution, we discuss the following RACH related open issues:

Issue 1: Issue 2-3: R2D trigger message byte alignment

Issue 2: MSG2 boundary to trigger CBRA failure

# 2	Discussion

## 2.1 Issue 2-3: R2D trigger message byte alignment

| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - FFS R2D byte alignment dependent on TBS size discussion - Status in running CR: not captured yet. | Companies are invited to input views for Q#11 |
| --- | --- | --- |

The issue has been discussed in the email discussion. Most companies agreed that Access trigger message byte aligned is not needed. Some companies commented that byte aligned is simpler, and with additional CRCs, bit alignment does not save overhead.

Based on the design in the MAC running CR, so far Access trigger message only contains R2D Message type (3bits).

And based on RAN1 agreements, 6 bits CRC is applied for Access trigger message, i.e., the total size from physical layer perspective is 9 bits. But RAN1 only counts the payload, i.e., the size of MAC message. If Access trigger message is byte aligned, additional 5 bits overhead are needed. Therefore we do agree with the statement mentioned by Rapporteur.

However, for Access Trigger message which appears extremely frequently, in order to make it byte-aligned, a large proportion of padding is required, which will waste a lot of transmission resources and have a significant impact on system performance.

Proposal 1: Access Trigger message is bit aligned instead of byte aligned.

## 2.2 MSG2 boundary to trigger CBRA failure

In previous meeting, companies had different view on whether K=1 (Option B) is enough or K>1 (Option C) should be allowed. In the following we analyse both options.

K=1 (Option B)

Pros: It is simple, and the most efficient in terms of reducing RN16 collisions.

Cons: A concern with K=1 is that the reader may not be able to collect enough RN16s to fully leverage Msg2/Msg3 multiplexing and therefore it may not be able to use the resources in efficiency way.

K>1 (Option C)

Pros: It enables the reader to collect more RN16s, hence to leverage the Msg2/Msg3 multiplexing. It can use the resources in efficient way.

Cons: There is an increased risk of RN16 collision. The device complexity is slightly increased, as it has to maintain a counter of Access Triggers to know when to stop monitoring Msg2. The reader complexity is also increased. It needs to collect more RN16s, and remember/associate a given RN16 with the slot in which it was transmitted, to know till when the Msg2 for the corresponding device can be sent. However from device perspective, the additional complexity is acceptable.

As it can be seen, both options have pros and cons. However, Option C is a good trade-off between system complexity and resources efficiency. Therefore it should be supported.

Regarding the K value, in general, we believe K should be set as low as possible, while allowing to make a full use of Msg2/Msg3 multiplexing.

Assuming a reader is capable and wants to use up to Y Msg3 multiplexing, then at least Y RN16s needs to be collected. In such case, we assume the reader would also configures NSFS at least to Y in the Paging procedure (for Msg1). At optimal slotted aloha operating point, success rate is 0.37. This means the number of collected RN16s per slot will be around X*Y*0.37.

If X=2, this translates into Y*0.74 per slot. This might be considered enough, but full use of Msg2/Msg3 multiplexing may benefit from K=2 slots. If X=1, the expected RN16s is twice less and seems to really impede the use of Msg2/Msg3 multiplexing. For that case at least, K>1 should be used, and a sensible value would be to use the double, i.e. K=4.

Proposal 2: Option C is used with K=4 when X=1, and K=2 when X=2

# 3	Conclusion

In this contribution, we discussed the RACH related issues and have corresponding proposals.

Proposal 1: Access Trigger message is bit aligned instead of byte aligned.

Proposal 2: Option C is used with K=4 when X=1, and K=2 when X=2

# Reference

[1] R2-2505521 Email discussion report: [POST130][027][AIoT] MAC Running CR (Huawei)