3GPP TSG-RAN WG2 Meeting #131	R2-2505123

Bangalore, India 25th –29th , 2025

Agenda Item:	8.2.4

Source: 	Xiaomi

Title:  	Remaining open issues on data transmission

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting, RAN2 discussed data transmission and the MAC PDU design. However following issues still need further discussion:

-	Subgroup: AS ID

  - Issue 3-3: AS ID release

-	Subgroup: D2R message content for data transmission

  - Issue 3-7: more data indication handling

- Subgroup: R2D message content for data transmission

  - Issue 3-8: R2D TBS

In this contribution, we continue the discussion on these remaining open issues and provide corresponding proposals.

# Discussion

## D2R message content for data transmission

### 2.1.1 Issue 3-7: more data indication handling

As listed by Rapporteur, the issue is

| (New)Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - During CR implementation, the rapp identified there is no clear conclusion how to set the “more data indication” in case of no data available, i.e., zero SDU. - Status in running CR: captured as value FFS. |
| --- | --- |

Based on the offline reply, companies have different understanding on how to set this value. Some companies argue that this bit should be set to “0” because the ‘more data indication’ is used for segmentation case. If ‘more data indication’ is set to “1”, reader may immediately send the D2R resource, but NAS response may still not be ready.

However, in our understanding, we think this “more bit indication” should be a mandatory field at least before we realize the NAS response issue, otherwise additional present bit shall be added. Then for zero SDU case, to indicate 1 is better than indicating 0 as

- The device still needs D2R resource for the response, indicating “1” at least allows the reader to know there will be follow-up data although not sure when it will be ready.

- With zero SDU/padding and the more data indication, the reader is aware of the situation and should not schedule a resource immediately, e.g., there is no requirement on the min/max time between the D2R and the next R2D. It is up to reader implementation to schedule a resource when the NAS message should be available or if the NAS message is still not available, then the device just indicates another zero SDU/padding.

Proposal 1: (Issue 3-7) RAN2 to agree that the more data indication is a mandatory field (without specification impact) and should be set to “1” for the case of zero SDU.

## R2D message content for data transmission

### 2.2.1 Issue 3-8: R2D TBS

As listed by Rapporteur, the issue is

| (New) Issue 3-8: R2D TBS | How to handle the R2D TBS, which may impact R2D padding, byte-alignment design. - Relevant agreements: - RAN1 LS in R1-2504915 - The length field inside MAC for SDU is not needed for R2D messages, assuming R2D MAC padding is not needed. FFS can come back if padding is needed depending on granularity of TBS (only if needed) - Status in running CR: not captured yet. |
| --- | --- |

We have already agreed that MAC PDU should be byte aligned, whether R2D trigger message is byte-aligned or not depends on the outcome of Q#11. Therefore, at least for all the variable size R2D message, including paging, Msg2, NACK feedback and R2D upper layer data message, byte alignment is required.

Proposal 2: (Issue 3-8) RAN2 to confirm R2D messages including paging, Msg2, NACK feedback and R2D upper date message are byte aligned.

Regarding how to achieve the byte-alignment, as captured in the current running CR, some fill bits at the end of each message can be utilized, which is similar as padding. However, length field is not needed considering the TBS field as agreed by RAN1 is included.

Proposal 3: (Issue 3-8) RAN2 to agree padding/fill bits at the end of the paging, Msg2, NACK feedback and R2D upper date message are utilized to achieve byte alignment. Length field is not needed.

## AS ID

### 2.3.1 New issue: AS ID release

According to the current MAC running CR, the device releases the AS ID only upon reception of the paging message.

Upon receiving the A-IoT Paging message, the A-IoT MAC entity shall: 1> if the Access Type field in the A-IoT Paging message indicates CBRA: 2> if the device has no stored Transaction ID; or 2> if the value of the Transaction ID field is different from the stored Transaction ID; or 2> if the value of the Transaction ID field is the same as the stored Transaction ID, and the previous procedure was determined as failed for this Transaction ID as specified in clause 5.5: 3> release the stored AS ID if any;

However, considering only NACK feedback is supported, access failure is declared upon reception of the NACK feedback with the corresponding AS ID, for which case re-access is determined as needed and the AS ID is not valid any more. Therefore, for the “access failure” case, it does not make sense to release the AS ID upon receiving the paging with the same transaction ID, but the AS ID should be released as long as NACK for this device is received due to:

- The earlier to release the AS ID, the less power to consume and the more storage to relieve. Especially when there is significant number of devices participating in the paging procedure and multiple trigger messages within the same paging round are transmitted, e.g., time interval between NACK and subsequent paging is quite long.

- If the AS ID can be released earlier, reader can still allocate this AS ID to other devices if needed and can reduce the AS ID collision.

Proposal 4: RAN2 to agree that device should release the AS ID upon reception of NACK message including the AS ID for the device.

### 2.3.2 New issue: AS ID for inventory only

According to the existing AO selection and RN16 generation scheme, it is possible RN16 collision may happen. For example, two devices select the AOs in the same/different access slot and generate the same RN16. To solve this issue, the most straightforward solution is to introduce a frequency index for each echoed RN16 in Msg2. In addition, to solve the collision, reader needs to assign a “AS ID” for each collide RN16 which is utilized as the identity for further transmission, e.g., NACK/command. For inventory only procedure, even there is no follow-up command, NACK message needs to be supported, therefore to identify the devices with collide RN16 in the NACK message, AS ID should be assigned. An example is shown in Figure1, assume device1 with RN16=X transmits Msg1 in the orange access occasion and receives Msg2 with its RN16=X in the same slot, i.e., RA succeed, while another device2 with the same RN16=X transmit Msg1 in the red access occasion triggered by next R2D trigger message and receives Msg2 with its RN16=X as well, in this case if Msg3 transmission for device1 fails and reader wants to indicate NACK to trigger re-access, without new assigned AS ID, this “X” cannot distinguish these two devices. Therefore, we suggest RAN2 to revert the previous agreement and apply AS ID for inventory only case as well. There should be no impact on MAC spec, simple update in stage 2 spec is sufficient.

Figure 1

Proposal 5: RAN2 to revert the agreement that “AS ID is applied for inventory+command case” to “AS ID is applied for inventory+command case and inventory only case” (no stage 3 specification impact)

# Conclusions

Based on the above discussion, we propose following proposals:

More data indication

Proposal 1: (Issue 3-7) RAN2 to agree that the more data indication is a mandatory field (without specification impact) and should be set to “1” for the case of zero SDU.

R2D TBS

Proposal 2: (Issue 3-8) RAN2 to confirm R2D messages including paging, Msg2, NACK feedback and R2D upper date message are byte aligned.

Proposal 3: (Issue 3-8) RAN2 to agree padding/fill bits at the end of the paging, Msg2, NACK feedback and R2D upper date message are utilized to achieve byte alignment. Length field is not needed.

AS ID release

Proposal 4: RAN2 to agree that device should release the AS ID upon reception of NACK message including the AS ID for the device.

AS ID for inventory only

Proposal 5: RAN2 to revert the agreement that “AS ID is applied for inventory+command case” to “AS ID is applied for inventory+command case and inventory only case” (no stage 3 specification impact)

# Reference

[1] RAN2#130 meeting agreement 
[2] R2-2505521 Remaining MAC open issues