3GPP TSG-RAN WG2 Meeting #131	R2-2505149

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		8.8.4 Support of broadcast service

Title:		Remaining issues on the support of broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behaviour is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

During the 129th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list. 2. It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH). 3. Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented).

During the 129bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts) 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. 4. The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

During the 130th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. Do not introduce service area information specific for FSAI in SIB21. 2. Do not introduce service area per neighbour cell in MCCH 3. RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session 4. The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In this paper, we would like to provide our views on remaining issues on support of broadcast service in NTN.

# 2. Discussion

2.1	Terminology: Broadcast Service Area vs. ISA

At the RAN2#130 meeting, it was briefly suggested (time constraints prevented discussion) that, for MBS, the term ‘broadcast service area’ should refer to the ‘intended service area’ (ISA), with the motivation to reuse the legacy UE behaviour for initiating the MII procedure upon entering or leaving the MBS ISA.

In this regard, RAN3 has already captured in CRs to TS 38.300 and TS 38.413 that broadcast service area information may include a Cell list, a TAI list, as well as an intended service area (ISA) list [1][2]. This indicates that the ISA does not have the same meaning as the broadcast service area; rather, ISA can be considered a subset of the broadcast service area.

Observation 1: In CRs of specifications maintained by RAN3, the ‘intended service area’ (ISA) is not identical to the ‘broadcast service area’ and can be considered a subset.

Accordingly, ETRI believes that it is inappropriate to align the term ‘intended service area’ with ‘broadcast service area’, since the latter may include additional scoping information (e.g., Cell list, TAI list) beyond a purely geographic description.

Proposal 1: RAN2 to retain the existing term ‘intended service area (ISA)’ as a subset of ‘broadcast service area’, and not redefine ‘broadcast service area’ to be identical to ISA.

Independent of the agreement on Proposal 1, ETRI does not see the need for any modification to the MII initiation behaviour when a UE enters or leaves the ISA of an MBS session. From Observation 1, the ‘broadcast service area’ of an MBS session may denote (i) the geographic area where the MBS session content is distributed and/or (ii) the area where broadcast signals can be received. Therefore, the existing wording in TS 38.331, “entering or leaving the broadcast service area”, already encompasses the case of entering or leaving the intended service area, and thus, no change is required.

Proposal 2: RAN2 not to change the legacy specifications regarding the initiation of the MII procedure upon entering or leaving the MBS intended service area; the existing TS 38.331 behaviour is sufficient.

2.2	Signalling formats for ISA

During the post-RAN2#130 email discussion, the Rapporteur proposed that RAN2 align the circular ISA format with SA4 and use Ellipsoid-PointWithUncertaintyCircle (TS 37.355) to define the circle in the new SIBXX. From ETRI’s perspective, the distanceRadius-based format (referenceLocation + distanceRadius) offers uniform quantization and a wider radius span, ensuring consistent accuracy from small to very large ISAs, whereas Ellipsoid-PointWithUncertaintyCircle employs non-linear (exponential) scaling and has a shorter effective range for large footprints. Therefore, for large-area MBS/NTN and consistent UE geofencing behaviour across scales, the distanceRadius-based format, which was already specified for other RAN-level signalling, provides more stable granularity and a broader range, enabling flexible network implementations optimized for diverse channel and deployment environments. (Interworking with SA4 might be maintained via a simple network-side mapping)

Proposal 3: RAN2 to retain (referenceLocation + distanceRadius) as the circular ISA format in SIBXX.

# 3. Conclusions

Observation 1: In CRs of specifications maintained by RAN3, the ‘intended service area’ (ISA) is not identical to the ‘broadcast service area’, and can be considered a subset.

Proposal 1: RAN2 to retain the existing term ‘intended service area (ISA)’ as a subset of ‘broadcast service area’, and not redefine ‘broadcast service area’ to be identical to ISA.

Proposal 2: RAN2 not to change the legacy specifications regarding the initiation of the MII procedure upon entering or leaving the MBS intended service area; the existing TS 38.331 behaviour is sufficient.

Proposal 3: RAN2 to retain (referenceLocation + distanceRadius) as the circular ISA format in SIBXX.

# 4. References

[1]	R3-253093, “(BL CR to 38.300) Support of Regenerative Payload and MBS broadcast in NR NTN,” Ericsson, Thales et al.

[2]	R3-253095, “Support of Regenerative Payload and MBS broadcast in NR NTN,” Ericsson, Thales et al.