3GPP TSG-RAN WG2 Meeting #131	                                                                                   R2-2505165

Bangalore, India, 25th – 30th August, 2025

Source:	CATT

Title:	Discussion on inter-CU LTM

Agenda Item:    8.6.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on inter-CU LTM [1]. The contribution is organized as the following. In section 2, we discuss the following aspects,

- Handling of SP CSI-RS/CSI-IM resource sets for target cell

- On the capture of L2 behavior for inter-CU SCG LTM

Our proposals are summarized in Section 3.

Discussion

Handling of semi-persistent CSI-RS/CSI-IM resource sets for target cell

According to the discussion in last meeting, it was agreed that,

UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch.

Regarding how to handle the activated SP CSI-RS for target cell for LTM cell switch, it still remains the following open issue,

| MAC-25 | Handling of the activated SP CSI-RS for target cell after LTM cell switch Editor’s NOTE: After reconfiguration with sync that is triggered by LTM, it is FFS how to handle the configured Semi-persistent CSI-RS/CSI-IM resource sets for target cell. Based on the following agreement: UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

Some companies think that after reconfiguration with sync that is triggered by LTM, UE should continue measure the SP CSI-RS for early CSI acquisition, according to the following RAN1 agreement,

Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2. Send an LS to RAN2 and RAN3 to inform this agreement. Agreement For target cell CSI acquisition, - After the reception of cell switch command, the UE may measure (depending on the timeline) CSI-RS resource(s) associated with determined CSI report configuration.

However, the intention is to acquire the target cell’s CSI report upon cell switch, due to the fact that it couldn’t be reported before/during cell switch. But after the cell switch, since the UE had applied the target configuration, the target cell may provide the same SP CSI-RS or periodic CSI-RS for CSI acquisition in the target cell, so there is no other concern to deactivate the SP CSI-RS for the target cell which is activated by the source cell before LTM cell switch.

Proposal 1 (MAC-25): UE deactivates the activated SP CSI-RS for the target cell after cell switch.

L2 behavior issue of inter-CU SCG LTM in RRC Running CR

L2 behavior for DRB

For DRB, now it is captured in the running CR as follows,

1> if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange: … 2> at the end of the procedure, for each drb-Identity value that is part of the current UE configuration: 3> if the LTM cell switch is triggered on the MCG; or 3> if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or 3> if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed: … 4> re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2; 4> re-establish the corresponding RLC entity as specified in TS 38.322 [4]; 3> else if LTM cell switch is triggered on the SCG and this DRB is using the master key: 4> if this DRB is an AM DRB: 5> after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC; 5> after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

As can be seen from above highlighted “else if” condition, i.e., UE performs inter-CU SCG LTM and the DRB is terminated in MN entity without termination point change, UE always performs RLC re-establishment and PDCP data recovery for the AM DRB,  even though the DRB is MN terminated with MCG bearer.

Observation 1: Upon inter-CU SCG LTM, UE always performs RLC re-establishment and PDCP data recovery for the AM DRB which is using the master key according to the RRC running CR, even though the DRB is only with MCG bearer. This is not aligned with the RAN2 agreement.

However, according to the following agreements,

Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell, - For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer. - For SN terminated bearer without change of termination point, UE performs PDCP re-establishment. If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

PDCP data recovery and RLC re-establishment of the SCG RLC entity is only needed if the MN terminated DRB is associated with a SCG RLC bearer, i.e., MN terminated SCG bearer or MN terminated split bearer. Therefore, the running CR should be updated based on above principle.

Proposal 2a: Update the current running CR based on the following principle for DRB upon inter-CU SCG LTM execution,

- Re-establish the SCG RLC entity if it is MN terminated SCG bearer or MN terminated split bearer;

- Perform PDCP data recovery if it is a MN terminated SCG bearer or MN terminated split bearer.

L2 behavior for SRB

For SRB, now it is captured in the running CR as follows,

1> if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange: … 2> at the end of the procedure, for each srb-Identity value that is part of the current UE configuration: … 3> re-establish the PDCP entity of this SRB as specified in TS 38.323 [5]; 3> re-establish the corresponding RLC entity as specified in TS 38.322 [4];

For inter-CU SCG LTM, according to the following agreements,

For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3. For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

SRB1/2, i.e., for the SRBs terminated in MN entity, does not perform PDCP re-establishment based on the NW configuration. For SRB3, the PDCP re-establishment is performed based on R19 set ID. Besides, for the SRB(s) terminated in MN entity but configured with split bear, the corresponding SCG RLC entity should be re-established. However, the current running CR only says “for each srb-Identity value” without differentiating the above mentioned cases, and thus should be updated.

Observation 2: Upon inter-CU SCG LTM, UE always performs RLC re-establishment and PDCP re-establishment for all the SRBs (including SRB1/2) that is part of the current UE configuration according to the RRC running CR. This is not aligned with the RAN2 agreement.

Proposal 2b: Update the current running CR based on the following principle for SRB upon inter-CU SCG LTM execution,

- For SRB(s) terminated in SN entity (i.e., SRB3), PDCP/RLC re-establishment is performed based on the Rel-19 set ID.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- For SRB(s) terminated in MN entity (i.e., SRB1/2), RLC re-establishment of SCG RLC entity is performed if this SRB is associated with SCG RLC bearer.

Proposal 2c: RAN2 to adopt Annex 1 for Proposal 2a and Proposal 2b.

Conclusion

Based on the previous analysis in Section 2, our observations and proposals are summarized as follows:

Observation 1: Upon inter-CU SCG LTM, UE always performs RLC re-establishment and PDCP data recovery for the AM DRB which is using the master key according to the RRC running CR, even though the DRB is MN terminated with MCG bearer. This is not aligned with the RAN2 agreement.

Observation 2: Upon inter-CU SCG LTM, UE always performs RLC re-establishment and PDCP re-establishment for all the SRBs (including SRB1/2) that is part of the current UE configuration according to the RRC running CR. This is not aligned with the RAN2 agreement.

Handling of SP CSI-RS/CSI-IM resource sets for target cell

Proposal 1 (MAC-25): UE deactivates the activated SP CSI-RS for the target cell after cell switch.

L2 behavior issue of inter-CU SCG LTM in RRC running CR

Proposal 2a: Update the current running CR based on the following principle for DRB upon inter-CU SCG LTM execution,

- Re-establish the SCG RLC entity if it is MN terminated SCG bearer or MN terminated split bearer;

- Perform PDCP data recovery if it is a MN terminated SCG bearer or MN terminated split bearer.

Proposal 2b: Update the current running CR based on the following principle for SRB upon inter-CU SCG LTM execution,

- For SRB(s) terminated in SN entity (i.e., SRB3), PDCP/RLC re-establishment is performed based on the Rel-19 set ID.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- For SRB(s) terminated in MN entity (i.e., SRB1/2), RLC re-establishment of SCG RLC entity is performed if this SRB is associated with SCG RLC bearer.

Proposal 2c: RAN2 to adopt Annex 1 for Proposal 2a and Proposal 2b.

Reference

[1] Draft_RAN2_130_Meeting_Report_v3

Annex 1

5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or upon the fulfilment of LTM cell switch execution conditions, the UE shall:

1>	if this procedure is triggered due to fulfilment of LTM cell switch execution conditions:

2> if more than one LTM candidate configuration has triggered this procedure:

3>	select one of the LTM candidate configurations as the selected cell for the LTM cell switch execution;

1>	if the LTM cell switch is triggered on the MCG; or

1>	if the LTM cell switch is triggered on the SCG and the LTM candidate configuration to be applied is configured via ltm-ConfigNRDC:

2>	release/clear all current dedicated and common radio configurations which have neither been received via SRB1 within mrdc-SecondaryCellGroup, nor via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID, and VarLTM-ServingCellNoSecurityChange;

-	the ltm-Config and ltm-ConfigNRDC (if configured);

-	the MCG C-RNTI;

-	the AS security configurations associated with the master key;

-	the logged measurement configuration;

1>	if the LTM cell switch is triggered on the SCG:

2>	release/clear all current dedicated and common radio configurations which have been received either via SRB1 within mrdc-SecondaryCellGroup, or via SRB3 except for the following:

-	the radio bearer configuration (configured via RadioBearerConfig IE)

-	the logicalChannelIdentity and logicalChannelIdentityExt of RLC bearers configured in RLC-BearerConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the bh-LogicalChannelIdentity of BH RLC channels configured in BH-RLC-ChannelConfig and the associated RLC entities, their state variables, buffers, and timers, except for triggering the associated RLC entities to reset the variable RETX_COUNT its initial value, as specified in TS 38.322 [4];

-	the UE variables VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID;

-	the ltm-Config;

-	the AS security configurations associated with the secondary key;

1>	for each SRB/DRB in the current UE configuration:

2>	if the LTM cell switch is triggered on the MCG and the SRB/DRB using the master key; or

2>	if the LTM cell switch is triggered on the SCG and the SRB/DRB using the secondary key:

3>	keep the associated PDCP and SDAP entities, their state variables, buffers and timers;

3>	release all fields related to the SRB/DRB configuration except for srb-Identity and drb-Identity;

1>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

1>	use the default values specified in 9.2.3 for timers T310, T311 and constants N310, N311 associated with the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure, where T310, N310, and N311 are for both MCG and SCG, and T311 is only for the MCG;

1>	apply the default MAC Cell Group configuration as specified in 9.2.2 for the cell group(s) for which the RRCReconfiguration message is applied due to the triggered LTM cell switch procedure;

1>	for each srb-Identity in the current UE configuration:

2>	apply the default SRB configuration defined in 9.2.1 for the corresponding SRB;

1>	if the value of ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChange within VarLTM-ServingCellNoSecurityChange:

2>	if the LTM cell switch is triggered on the MCG:

3>	update the master security key by performing the AS security key update procedure as specified in 5.3.5.7;

2> else if the LTM cell switch is triggered on the SCG:

3>	consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange associated to the the field ltm-NoSecurityChangeID as the selected sk-Counter value, and update the secondary key by performing security key update procedure as specified in 5.3.5.7;

3>	remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChange;

2>	at the end of the procedure, for each drb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this DRB is using the secondary key; or

3>	if the LTM cell switch is triggered on the SCG and the keyToUse for this DRB is changed:

4>	if the PDCP entity of this DRB is not configured with cipheringDisabled:

5>	configure the PDCP entity with the ciphering algorithm and KUPenc key associated with the master key (KgNB) or secondary key (S-KgNB), as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent PDCP PDUs received and sent by the UE;

4>	if the PDCP entity of this DRB is configured with integrityProtection:

5>	configure the PDCP entity with the integrity protection algorithms according to securityConfig and apply the KUPint key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse;

4>	if drb-ContinueROHC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueROHC is configured;

4>	if drb-ContinueEHC-DL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-DL is configured;

4>	if drb-ContinueEHC-UL is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueEHC-UL is configured;

4>	if drb-ContinueUDC is included in pdcp-Config:

5>	indicate to lower layer that drb-ContinueUDC is configured;

4>	re-establish the PDCP entity of this DRB as specified in TS 38.323 [5], clause 5.1.2;

4>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];

3>	else if LTM cell switch is triggered on the SCG and this DRB is using the master key:

4>	if this DRB is an AM DRB and this DRB is configured with MN terminated SCG bearer or MN terminated split bearer:

5>	after the end of this procedure, re-establish the corresponding SCG RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

5>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC;

2>	at the end of the procedure, for each srb-Identity value that is part of the current UE configuration:

3>	if the LTM cell switch is triggered on the MCG; or

3>	if the LTM cell switch is triggered on the SCG and this SRB is using the secondary key:

4>	configure the PDCP entity to apply the integrity protection algorithm and KRRCint key associated with the master key (KgNB) or the secondary key (S-KgNB), as indicated in keyToUse, i.e. the integrity protection configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	configure the PDCP entity to apply the ciphering algorithm and KRRCenc key associated with the master key (KgNB) or the secondary key (S-KgNB) as indicated in keyToUse, i.e. the ciphering configuration shall be applied to all subsequent messages received and sent by the UE, including the message used to indicate the successful completion of the procedure;

4>	re-establish the PDCP entity of this SRB as specified in TS 38.323 [5];

4>	re-establish the corresponding RLC entity as specified in TS 38.322 [4];

3> else if the LTM cell switch is triggered on the SCG and this SRB is using the master key:

4>	if this SRB is configured with MN terminated split bearer:

5>	re-establish the corresponding SCG RLC entity as specified in TS 38.322 [4];

2>	if the value of field ltm-NoSecurityChangeID contained in the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChange:

3>	replace the value of ltm-ServingCellNoSecurityChangeID in VarLTM-ServingCellNoSecurityChange with the value of ltm-NoSecurityChangeID in the LTM-Candidate in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3;