3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505170

Bangalore, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Adaptation of common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues on Paging adaptation and RACH adaptation.

Discussion

Paging adaptation

- Remaining issue 1

In the post email [1], following remaining issue was discussed:

- Q13a. Please comment on whether the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames: - at symbol level - at slot level - by restricting the maximum value range of pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter for different SCS corresponding to pagingAdaptationNAndPagingFrameOffset-r19 (i.e. N) - by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them. - other

In legacy paging configurations, the maximum value of Ns is 4 and the maximum number of bits for each PMO is 17, where the maximum offset of the first PDCCH monitoring occasion for paging of each PO of the PF is 16 radio frames. The maximum signaling overhead for PMO configuration is maximum value of Ns*bits for each PMO (4*17=68 bits).

For paging adaptation, the maximum number of Ns is extended to 8, the maximum PO per PF (i.e. maxPO-perPF) needs to be extended to 8 correspondingly. And a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging is introduced, the first PDCCH monitoring occasion for R19 paging of each PO of PF is extended to 32 radio frames. The maximum number of bits for each PMO needs to be extended to 18. With these extensions, the maximum signaling overhead for firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is 144 bits (8*18), which is acceptable to the limited massage size of SIB1. Hence, we propose:

Proposal 1: The maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames at symbol level.

If paging adaptation is enabled, it is possible that the separated firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent. I.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information, but the separated firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent. In this case, there are two possible understandings for R19 UE supporting paging adaption:

- Alt1: Legacy firstPDCCH-MonitoringOccasionOfPO if configured is used.

- Alt2: The first PDCCH monitoring occasion of each PO of the PF is not configured.

Considering that using legacy firstPDCCH-MonitoringOccasionOfPO is not sufficient when the number of PO per PF for paging adaptation is greater than that of legacy, we slightly prefer alt2

Proposal 2: If paging adaptation is enabled (i.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information) in the cell, but the firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent, UE considers that the first PDCCH monitoring occasion of each PO of the PF is not configured.

- Remaining issue 2

- Q14a. Please comment on whether the maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames: - at symbol level - at slot level - by restricting the maximum value range of Rel-19 pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for different SCS corresponding to pagingAdaptationNAndPagingFrameOffset-r19 (i.e. N) - by restricting the configuration to evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them. - other

Similar to pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19, the maximum number of PEI occasion per paging frame is extended to 8, while the maximum value of the offset, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O, is extended to 32 radio frames at symbol level.

Proposal 3: The maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames at symbol level.

Different from firstPDCCH-MonitoringOccasionOfPO, firstPDCCH-MonitoringOccasionOfPEI-O should be configured when PEI is configured. When pagingAdaptationPEI-Config is signaled in system information, it is unclear if the separate pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 can be absent. There are two possible understandings:

- Alt1: If pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 is absent, legacy firstPDCCH-MonitoringOccasionOfPEI-O is used for a UE supporting paging adaptation and PEI.

- Alt2: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured.

Similar as pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19, we slightly prefer alt2.

Proposal 4: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured.

- Remaining issue 3

In the post email [1], some companies pointed out the issue of whether Redcap and Paging adaptation can coexist.

| OPPO005 | [OPPO] Compared with the implementation in PDCCH-ConfigCommon, it seems the yellow part is the delta part, Question-1: Do we really need the implementation in PDCCH-ConfigCommon for PO, considering the following conclusion and the condition of “Cond OtherBWP”? Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED. [Samsung]: Its needed for BWP other than InitialDownlinkBWP (e.g. initialDownlinkBWP-RedCap) [OPPO] Thanks for the point from Samsung. After some offline, we reached the consensus that for non-Redcap case, the PO location configuration @ PDCCH-ConfigCommon is *not* needed. For Redcap, I agree it is feasible to configure PO location @ PDCCH-ConfigCommon. Yet we are not sure whether we can easily extend the applicable scenario for NES to (e)Redcap, which should be confirmed by R2 first. E.g., the capability for supporting PO bundling within initialDownlinkBWP-RedCap may not be covered by legacy (e)Redcap capability or the new NES capability for non-redcap UE. So we tend to avoid talking about Redcap + Paging adaptation scenario. [Apple] Our view is that RAN2 has not agreed to pursue optimization of Redcap+Paging adaptation. Unless RAN2 can explicitly agree to support this feature combination, we don’t think we need to capture text related to Redcap+Paging adaptation. | Open issue. there is no explicit agreement. However, in 3GPP, it is generally assumed that unless someone found something broken/needs particular changes but not addressed for some reason (due to lack of interest, time etc) it applies to all specified features. This should also apply in case of RedCap. Why should a network, which has enabled RedCap, configuring this NES feature have a limitation on whether/how to configure initial BWP for RedCap UEs? |
| --- | --- | --- |

Except new fields, i.e. pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19, need to be introduced, support of Redcap+Paging adaptation scenario doesn’t introduce additional complexity. Therefore, we propose to confirm.

Proposal 5: The scenario of Redcap+Paging adaptation is supported in Release 19.

- Open issue 4

The value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 and pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 is also an open issue:

| Samsung 003 | pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 CHOICE { sCS15KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS), sCS30KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS), sCS60KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS), sCS120KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS), sCS480KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPO-perPF-r19)) OF INTEGER (0..FFS) } OPTIONAL, -- Cond OtherBWP pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 CHOICE { sCS15KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS), sCS30KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS), sCS60KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS), sCS120KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS), sCS480KHZoneThirtySecondT SEQUENCE (SIZE (1..maxPEI-perPF-r19)) OF INTEGER (0..FFS) } OPTIONAL -- Cond InitialBWP-Paging ]] Values for T, T/2, T/4, T/8 and T16 are missing. These can also be configured for paging adaptation and network should be able to configure starting PDCCH monitoring occasion number of POs in this case for paging adaptation. [Apple] We agree with Samsung. We don’t think it is right way to only configure T32 in Rel-19 config, which requires Rel-19 UE to combine legacy configuration and new configuariton resulting in unnecessary UE complexity. We prefer that Rel-19 UE only need to check Rel-19 IE which include all cases (including legacy case of T, T/2, T/4, T/8 and T16). We provide an example of ASN.1 to Rapporteur under OPPO 005. |
| --- | --- |

In current running CR, pagingAdaptationNAndPagingFrameOffset-r19 including T, T/2, T/4, T/8, T/16 and T/32, the setting of pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 and pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 should be aligned with pagingAdaptationNAndPagingFrameOffset-r19. In addition, paging adaptation should be able to configure legacy configurations to provide flexibility, rather than just configuring T/32.

Proposal 6: The values for T, T/2, T/4, T/8 and T/16 should also be configured in paging adaptation.

- Open issue 5

The issue of whether pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset, and firstPDCCH-MonitoringOccasionOfPO are grouped together in one IE was raised in a post email [1].

| Samsung 004 | #### pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset and firstPDCCH-MonitoringOccasionOfPO should be grouped together in one IE. pagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset should be mandatory inside the IE. pagingAdaptation-r19 ::= SEQUENCE { pagingAdaptation-NS-r19 ENUMERATED {eight,four, two, one} pagingAdaptationNAndPagingFrameOffset-r19 CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15), oneThirtySecondT INTEGER (0..31) } firstPDCCH-MonitoringOccasionOfPO-r19 CHOICE { } OPTIONAL -- Need R } OPTIONAL -- Need R |
| --- | --- |

In our view, it is beneficial to clearly express the correlation between paging adaptive parameters, thus, we propose:

Proposal 7: PagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset and firstPDCCH-MonitoringOccasionOfPO can be grouped together in one IE.

RACH adaptation

In the last RAN2 #130 meeting, the issue that CE feature and RACH adaptation do not coexist was discussed, with the following agreement:

RAN2 will select one of the following options. (CE here means MSG1 repetition). - Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon. - Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17)

In our view, option-1b is ultimately controlled by the UE, which increases the complexity of the UE, while option-1c is controlled by the NW, fundamentally solving the problem of RACH adaptation and CE feature non coexistence, so option-1c is preferred. In order to support option-1c, separate rach-ConfigCommon may be configured for CE feature and non-CE feature, which is totally up to network implementation.

Proposal 8: Option-1c is used to avoid the use of additional RACH resource in RACH partitioning with CE.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Paging Adaptation

Proposal 1: The maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to 32 radio frames at symbol level.

Proposal 2: If paging adaptation is enabled (i.e., pagingAdaptation-NS and pagingAdaptationNAndPagingFrameOffset are signaled in system information) in the cell, but the firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is absent, UE considers that the first PDCCH monitoring occasion of each PO of the PF is not configured.

Proposal 3: The maximum offset value for pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter is extended to 32 radio frames at symbol level.

Proposal 4: When pagingAdaptationPEI-Config is signaled in system information, pagingAdaptationFirstPDCCH-MonitoringOccasionOfPEI-O-r19 should be configured.

Proposal 5: The scenario of Redcap+Paging adaptation is supported in Release 19.

Proposal 6: The values for T, T/2, T/4, T/8 and T/16 should also be configured in paging adaptation.

Proposal 7: PagingAdaptation-NS-r19, pagingAdaptationNAndPagingFrameOffset and firstPDCCH-MonitoringOccasionOfPO can be grouped together in one IE.

RACH Adaptation

Proposal 8: Option-1c is used to avoid the use of additional RACH resource in RACH partitioning with CE.

Reference

- R2-2505708, Report of [POST130][107][NES] (Ericsson)