3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505173

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT

Title:	Discussion on XR Rate Control

Agenda Item:	8.7.6

Document for:	Discussion and Decision

Introduction

In RAN2#130 meeting, regarding to XR rate control, the below agreements were agreed [1]:

Single or multiple QoS flows (MAC-05):

(MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify.

ID for flow identification (MAC-04):

We rule out explicit signalling of PDU session ID + QFI

We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead:

- Explicit DRB ID + QFI (FFS if QFI is explicit or implicit)

- Implicit, e.g. index or mapping

Procedure details and prohibit timer configuration (RRC-1, MAC-08, MAC-10):

(RRC-1) The granularity of bit rate query prohibit timer is QoS flow.

FFS The value of the prohibit timer is the same for all flows

(MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

- Available bit rate query (MAC-12):

(MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit.

(MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate.

Impact of Dual-Connectivity (MAC-9):

No optimization is needed to address DC case for XR rate control MAC CE

We capture in stage-2 that XR rate indication is for application and not for MAC entity.

DL rate control (MAC-11):

RAN2 will not specify DL rate control and DL rate control query

Except the issues that well handled in the post email discussion, in the current contribution, we discuss the below key leftover issues on XR rate control:

1）[MAC-3] ID for flow identification.

2）[MAC-4] Format of the UL Rate Control MAC CE.

3）[MAC-5] UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE.

Discussion

[MAC-3] ID for flow identification

In the last meeting, RAN2 firstly excluded the PDU session ID and QFI approach, the concern for this approach was mainly due to the overhead and cross-layer information delivery complexity. The next step is to down select between the below two options:

- Explicit DRB ID+ QFI(FFS if QFI is explicit or implicit).

- Implicit, e.g. index or mapping.

Besides the two options, there is also one guide for the way forward: considering the max number of flows we want to be able to indicate and trying to minimize overhead. Hence, we further discuss this issue based on the two aspects:

- For the issue that the max number of flows we want to be able to indicate, for a certain service, similar QoS requirement will be merged into one QoS flow and marked with 5QI parameter. For different 5QI parameter, there will be typical service. For example, the example service for 5QI value 1 is Conversation Voice, and the example service for 5QI value 5 is IMS signalling. In the TS23.501, there is a standardized 5QI to QoS characteristics mapping table. In 3GPP, cater to the XR service, the 5QI_80, 5QI_87, 5QI_88, 5QI_89 and 5QI_90 had been introduced. 5QI_80 is for NGBR service, and all the others are for delay critical GBR service. Next, considering the RAN resource limitation, different QoS flow can be mapped into one DRB. That is to say, the N:1 mapping from QoS flow to DRB is a very common configuration under the current RAN architecture. All in all, the max number of flows we want to be able to indicate should focus on the new introduced 5QI group, that is to say, the typical number of flows in use is 1~2 in most cases.

- For the issue about the overhead, the most consume overhead PDU session ID and QFI approach had been excluded, it is fair enough to use the explicit DRB ID plus QFI to identify one QoS flow in the new introduced MAC CE which more follows the legacy design. For the implicit approach, although it can further reduce the overhead in the MAC CE, but the cost is to further introduce RRC spec impacts which is the scope expansion compared to what RAN2 agreed in the previous meeting.

Hence, we propose that RAN2 uses the LCID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19.

Proposal 1: (MAC-3) RAN2 uses the LCID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19.

[MAC-4] Format of the UL Rate Control MAC CE

In RAN2#130 meeting, RAN2 agreed that XR rate control MAC CE can comprise multiple QoS flows, considering the use frequency of multiple QoS flows in single MAC CE, simple design is the first recommendation, hence, we prefer to use similar DSR enhancement approach to handle it:

Proposal 2: (MAC-4) One extension bit per pair to indicate the presence of the next pair of the different QoS flow.

In case that the multiple QoS flows belong to different LCID, to further reduce the overhead of MAC CE, one indication “I” field can be introduced to show that whether there will be LCID filed needed in the following pair of the different QoS flow. “I” filed set to 1 indicates there will be LCID filed in the following pair of the different QoS flow, that is to say, the following one belongs to different LCID compared to the previous one. “I” filed set to 0 indicates there will be no LCID field in the following pair of the different QoS flow, which means the following one belongs to the same LCID compared to the previous one.

Proposal 3: (MAC-4) One indication bit per pair to indicate the presence of the LCID field in the following pair of the different QoS flow.

The Figure 6.1.3.20-1 in 38.321[4] shows the format of recommended bit rate MAC CE in previous release. It has a fixed size and consists of two octets.

Figure 6.1.3.20-1: Recommended bit rate MAC CE

The related fields are:

1) “LCID” field indicates the identity of the logical channel for which the recommended bit rate or the recommended bit rate query is applicable. The length of the field is 6 bits.

2) “UL/DL” field indicates whether the recommended bit rate or the recommended bit rate query applies to uplink or downlink. The length of the field is 1 bit. The UL/DL field set to 0 indicates downlink. The UL/DL field set to 1 indicates uplink.

3) “Bit Rate” field indicates an index to Table 6.1.3.20-1 in 38.321[4]. The length of the field is 6 bits. For bit rate recommendation the value indicates the recommended bit rate. For bit rate recommendation query the value indicates the desired bit rate.

4) “X” is Bit rate multiplier, for UEs supporting recommended bit rate multiplier, when bitRateMultiplier is configured for the logical channel indicated by LCID field, X field set to 1 indicates the actual value of bit rate is the value corresponding to the index indicated by the Bit Rate field multiplied by bitRateMultiplier as specified in TS 38.331.

5) “R” is reserved bit. There are two R bits in the current format which are all set to 0.

Based on the current discussion progress, we propose the below new introduced Recommended bit rate MAC CE for Rel-19 XR: The indication to the related QoS flow in the MAC CE will cost 12 bits (6bits for LCID and 6 bits for QFI), Bit rate field will cost 8 bits, “Ext” field indicated the presence of the next pair of the different QoS flow will cost 1 bit, “I” filed indicated whether there will be LCID field in the next pair of the different QoS flow to save overhead in case that same LCID case will cost 1 bit, to further achieve the byte alignment requirement, 2 “R” reserved bit should be further introduced.

Figure 1. Proposed Recommended bit rate MAC CE for Rel-19 XR

Proposal 4: (MAC-4) RAN2 adapts 3 bytes format for the Recommended bit rate MAC CE for Rel-19 XR, including 12 bits QoS flow indication, 8 bits rate index, 1 extension field, 1 indication field and 2 reservations.

Proposal 5: (MAC-4) In case that the presence of the next pair of the different QoS flow has the same LCID compared to the previous one, the following pair can just only adapt 2 bytes format, including 6 bits QoS flow indication, 8 bits rate index, 1 extension field and 1 indication field.

[MAC-5] UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE

One leftover issue was raised during the post email discussion about UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together. This issue was introduced due to XR rate control MAC CE can comprise multiple QoS flows in a single MAC CE. To handle this issue, we prefer to follow the legacy limitation for the MAC CE transmission, that the UE doesn’t support transmitting some of the queries if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE. Since this suggestion has the least specification impacts.

Proposal 6: (MAC-5) If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, UE shall skip the current transmission occasion.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: (MAC-3) RAN2 uses the LCID and QoS Flow ID to indicate per QoS flow level recommended bit rate in Rel-19.

Proposal 2: (MAC-4) One extension bit per pair to indicate the presence of the next pair of the different QoS flow.

Proposal 3: (MAC-4) One indication bit per pair to indicate the presence of the LCID field in the following pair of the different QoS flow.

Proposal 4: (MAC-4) RAN2 adapts 3 bytes format for the Recommended bit rate MAC CE for Rel-19 XR, including 12 bits QoS flow indication, 8 bits rate index, 1 extension field, 1 indication field and 2 reservations.

Proposal 5: (MAC-4) In case that the presence of the next pair of the different QoS flow has the same LCID compared to the previous one, the following pair can just only adapt 2 bytes format, including 6 bits QoS flow indication, 8 bits rate index, 1 extension field and 1 indication field.

Proposal 6: (MAC-5) If available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, UE shall skip the current transmission occasion.

Reference

- R2-2504675	Report from session on R19 XR and LTE Broadcast	Session chair (Huawei)	Report