3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505176

Bengaluru, India, 25th – 29th Aug 2025

Source:	CATT

Title:	Intra-gNB Service Continuity for Multi-hop U2N Relay

Agenda Item:	8.13.4

Document for:	Discussion and Decision

Introduction

In this contribution, based on the agreements at RAN2#130 meeting as in [1], two issues on intra-gNB service continuity for multi-hop U2N relay will be further discussed, including whether to only report the Relay UEs in RRC_CONNECTED and whether to introduce measurement enhancement in scenario C/D.

Discussion

[RRC-14] whether to only report the Relay UEs in RRC_CONNECTED

The RRC-14 issue is proposed as follows:

Clarification is needed to explicitly state that Remote UEs shall only report target relay UEs that are in RRC_CONNECTED state when operating in Scenario C and Scenario D. This ensures consistent behavior across implementations and avoids ambiguity regarding the state of candidate relay UEs during reporting procedures. The Relay UE includes an indication of its RRC_CONNECTED status in the discovery message RRC container. The Remote UE can utilize this indication to determine the connection state of each candidate Relay UE and subsequently report only those candidate First /Intermediate Relay UEs that are in the RRC_CONNECTED state. Such clarification can be included in the Stage 2 specification, specifically in the description of the signalling flows for Scenario C and Scenario D or in RRC Specification. Proposal 8 – RAN2 should clarify in TS 38.300 or TS 38.331 that, for Scenario C and Scenario D, the Remote UE shall report only Relay UEs in RRC_CONNECTED state for path switching, based on the Relay State indication received in the discovery message RRC container.

In last meeting, RAN2 agreed the remote UE reports at least candidate first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. Based on above issue, the remote UE could receive the Relay UE status information in the discovery message. However, if the remote UE only report the measurement results of Relay UEs in RRC_CONNECTED state for path switching, considering that there is some time gap between UE reporting and path switching decision in scenario C/D, it is very possible that the remote UE will miss some candidate Relay UE, which could transfer to RRC_CONNECTED state when the network make decision to path switching. Besides, the network could select the relay UE in RRC_CONNECTED state by itself, it is not necessary to let remote UE only report the Relay UEs in RRC_CONNECTED state.

Proposal 1: (RRC-14) It is not necessary to let Remote UE report only Relay UEs in RRC_CONNECTED state for path switching, the network will select the relay UE in RRC_CONNECTED state by itself.

On Measurement in Scenario C/D

RAN2#130 Agreements: Event Y1 and Y2 can be reused in path switching Scenario C with the understanding that the “first relay UE” in a candidate multi-hop relay link is “candidate L2 U2N Relay UE”. Event X2, Y2 and Z1 can be reused in path switching Scenario D with the understanding that the “first relay UE” in a candidate multi-hop relay link is “candidate L2 U2N Relay UE”. Wording for the multihop case to be checked in CR implementation. For intra-gNB direct to multi-hop indirect path switch (scenario C), remote UE evaluates both relay link (for the link between remote UE and candidate first relay UE) and Uu link, where the sidelink relay measurement report shall include at least candidate first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. For intra-gNB single-hop indirect to multi-hop indirect path switch (scenario D), the remote UE reports serving single-hop relay UE and candidate first relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Based on above agreements, the legacy measurement event Y1/Y2 will be used in scenario C, and the legacy measurement event X2/Y2/Z1 will be used in scenario D, which means that the source Remote UE will only report the channel quality from source Remote UE to candidate First Relay UE without any information on target link quality between Relay UEs.

One remain issue is whether the Remote UE needs to report other information (e.g., the channel quality of each hop link) to gNB. Considering that the WID has already restrict the gNB will find a target indirect path consisting of the Last relay UE in “direct” RRC Connected mode and all the other Intermediate relay(s) in “indirect” RRC Connected mode to the same cell, hence the gNB could know the channel quality of each target hop link since it could configure the Last Relay UE and Intermediate Relay UE to perform measurement and report the measure results, then the gNB could compute the accumulated QoS information for each hop, it is sufficient to let source Remote UE only report the channel quality to the candidate First Relay UE, the network will try to find a suitable multi-hop link if available. Otherwise, the channel quality on each hop needs to be delivered until to the end child-hop, which is complexity and not necessary. Hence, it is suggested not to report the link quality or other information on target each hop link, it is network implementation to find a suitable multi-hop relay link.

Proposal 2: Since network could configure the target Relay UEs in direct/indirect RRC Connected mode to report measurement results on each hop link, it is not necessary for source Remote UE to report the link quality or other information on target each hop link, it is network implementation to find a suitable multi-hop relay link.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: (RRC-14) It is not necessary to let Remote UE report only Relay UEs in RRC_CONNECTED state for path switching, the network will select the relay UE in RRC_CONNECTED state by itself.

Proposal 2: Since network could configure the target Relay UEs in direct/indirect RRC Connected mode to report measurement results on each hop link, it is not necessary for source Remote UE to report the link quality or other information on target each hop link, it is network implementation to find a suitable multi-hop relay link.

Reference

- Report from session on positioning and sidelink relay, MediaTek, RAN2#130 meeting.