3GPP TSG-RAN WG2 Meeting #131	R2-2505184

Bangalore, India  Aug 25th – 29th , 2025

Agenda Item:	8.3.1

Source:	OPPO

Title:	Text proposal on simulation model complexity

Document for:	Discussion, Decision

Discussion

In RAN2#129, we agreed to capture something on model complexity and left a FFS for it.

Agreements on RRM prediction

- We will capture something about model complexity.  FFS how and what to capture it

In the current TR, this agreement remains unaddressed, and it is advisable to propose a solution as we approach the end of the project. Drawing inspiration from the format used in TR 38.843, we propose the following table to illustrate model complexity.

Table 5.2.2-1: AI/ML model complexity/computation complexity 
used in the evaluations for AI/ML in RRM measurement prediction

|  | Model complexity in number of model parameters | Model complexity in number of model size | Computational complexity (FLOPs) |
| --- | --- | --- | --- |
| FR1 to FR1 intra-frequency temporal domain case B | 16K to 1.51M majority reported less than 0.8M | 67Kbytes to 5.1Mbytes majority reported less than 1Mbytes | 0.12M to 23.86M majority reported less than 3M |
| FR1 to FR1 inter-frequency (frequency domain) | 0.22K to 1.84M majority reported less than 0.33M | 2.1Kbytes to 7.3Mbytes majority reported less than 1Mbytes | 5K to 66M majority reported less than 3M |
| FR2 to FR2 intra-frequency temporal domain case A | 4.5k to 1.38M majority reported less than 0.7M | 69Kbytes to 10.8Mbytes majority reported less 0.74 Mbytes | 20K to 33.1M majority reported less than 10M |

Proposal 1: Capture Table 5.2.2-1 for AI/ML model complexity into TR. The corresponding text proposal is provided in the annex.

Annex: Proposed text proposal

START OF CHANGES

- 5.2.2	Evaluation results

In the evaluation, model complexity in number of model parameters, model complexity in number of model size, and computational complexity in FLOPs are used for AI/ML complexity analysis. Table 5.2.2-1 illustrates the complexity results for high-priority scenarios.

Table 5.2.2-1: AI/ML model complexity/computation complexity 
used in the evaluations for AI/ML in RRM measurement prediction

|  | Model complexity in number of model parameters | Model complexity in number of model size | Computational complexity (FLOPs) |
| --- | --- | --- | --- |
| FR1 to FR1 intra-frequency temporal domain case B | 16K to 1.51M majority reported less than 0.8M | 67Kbytes to 5.1Mbytes majority reported less than 1Mbytes | 0.12M to 23.86M majority reported less than 3M |
| FR1 to FR1 inter-frequency (frequency domain) | 0.22K to 1.84M majority reported less than 0.33M | 2.1Kbytes to 7.3Mbytes majority reported less than 1Mbytes | 5K to 66M majority reported less than 3M |
| FR2 to FR2 intra-frequency temporal domain case A | 4.5k to 1.38M majority reported less than 0.7M | 69Kbytes to 10.8Mbytes majority reported less 0.74 Mbytes | 20K to 33.1M majority reported less than 10M |

END OF CHANGE