3GPP TSG-RAN WG2 Meeting #131	R2-2505198

Bengaluru, India, 25th – 29th Aug, 2025

Source:	vivo

Title:	Remaining issues on AIoT Data Transmission

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we will discuss the remaining open issues regarding AIoT data transmission aspects, which are listed in the email discussion [POST130][027][AIoT] [1].

# Discussion

# Issue 3-3: AS ID release

In the email discussion, open issue 3-3 on AS ID release is described as below [1]:

| Issue 3-3: AS ID release | Whether a release message is needed for AS ID release - Relevant agreements: - FFS other cases for release ASID to avoid keeping it indefinitely. - - For CBRA, to avoid AS ID being occupied for unnecessary time and to keep alignment between reader and device on AS ID release, device can release AS ID upon receiving paging message with different transaction ID, no matter the paging message is for it or not. FFS for CFRA - - FFS for need for release message - Status in running CR: captured as Editor’s Note in 5.2. |
| --- | --- |

And according to the company inputs, the rapporteur provides a proposal as below:

Proposal 5 (Issue 3-3 AS ID release): Explicit release message is not needed. (18/24). [If needed: New message can include no AS ID (to release all devices) or AS ID list (to release one or multiple devices).]

In this contribution, we would like to justify more on the need of the AS ID release message, which mainly takes into account of the following RAN3#128 agreement on the New A-IoT Session Release procedure [2].

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message.

Based on the RAN3 agreed TP, the A-IoT Session Release procedure is specified as follows [3].

************************************RAN3 TP Start*************************************

### 8.xx.4	A-IoT Session Release

#### 8.xx.4.1	General

The purpose of the A-IoT Session Release procedure is to enable the A-IoT CN node to terminate all activities related to the indicated A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

#### 8.xx.4.2	Successful Operation

Figure 8.xx.4.2-1: A-IoT Session Release, successful operation.

The A-IoT CN node triggers the procedure by sending a AIOT SESSION RELEASE COMMAND message to the NG-RAN node. Upon reception of AIOT SESSION RELEASE COMMAND the NG-RAN node shall terminate all activities and release the context and resources related to the A-IoT session denoted by the AIOTF Identifier IE and the Correlation ID IE and respond with an AIOT SESSION RELEASE COMPLETE message.

*************************************RAN3 TP End **************************************

It is observed that once the A-IoT Session Release procedure is triggered by the A-IoT CN node, it requires the NG-RAN node (i.e., the reader) to terminate all activities and release the context and resources related to the A-IoT session, see highlighted green TP above. Consequently, the related AS context (mainly refers to the stored AS ID) maintained at the device side is in fact no longer needed, which obviously causes redundant R2D message monitoring and bring energy waste to the device after the A-IoT Session Release procedure.

- Once the A-IoT session Release is completed at the CN side, the related AS context (e.g., the stored AS ID) maintained at the device side and the reader side are no longer needed and shall be released.

Therefore, we propose that:

- (Issue 3-3) In line with the CN triggered A-IoT Session Release procedure, RAN2 to introduce explicit release procedure to timely release the AS context (e.g., the stored AS ID) maintained at device side.

Regarding whether to rely on an existing R2D message or introduce a new R2D message to release the AS context at the device side. There are two existing R2D messages (i.e., NACK feedback message and Paging message) which are the candidates. However, we think a new R2D message is better for the following reasons:

- Firstly, for the Paging message, in current A-IoT MAC running CR it contains some mandatory fields (e.g., Access Type, D2R Scheduling Info) which are not needed by the device in case of A-IoT Session Release procedure. Therefore, reuse the Paging message would cause complexity to the Paging format design and also signaling overhead to make some fields optionally present.

- Secondly, for the NACK feedback message, in current A-IoT MAC running CR receiving the NACK feedback message would mandate the device to perform re-access which is not aligned with the CN intention to terminate all activities for the related the A-IoT session.

- Last but not the least, in current A-IoT MAC running CR, it can only rely on a Paging message with new transaction ID to release AS ID, which may be quite long-time interval after the old Session has been ended/released at the CN side, see Figure 1 as below. Therefore, we believe that the existing solution is not sufficient for the reader to efficiently manage the device AS context and resources.

Figure 1. A-IoT Session Release before New Service/Session

As above, introducing a new R2D message to release the stored AS ID is preferred. In generally, the A-IoT Session Release performed at the CN side involves all the devices which has previously responded to the Paging containing a Transaction ID corresponding to the A-IoT session. To address the concerns of large signalling overhead by containing AS ID list (to release one or multiple devices), no AS ID to be contained in the new R2D message but just that corresponding Transaction ID is further proposed.

- (Issue 3-3) Introduce a new R2D Release message which contains a Message Type, and a Transaction ID that associates to the A-IoT Session Release at the CN side (see Figure 2 as an example).

Figure 2. An Example of New R2D Release Message

In accordance with the above Proposal 2, the device behavior upon reception of the new R2D Release message is proposed as follows.

- (Issue 3-3) Upon reception of the new R2D Release message, the device releases the stored AS ID if any, if the value of the Transaction ID included in the received R2D Release message is matched with the stored Transaction ID.

# Issue 3-7: More Data Indication

According to the email discussion, there is one open issue on MDI setting in case NAS response is unavailable at the corresponding D2R transmission occasion [1]:

| (New)Issue 3-7: more data indication handling | How to set “more data indication” value in case of no NAS response available (i.e., zero SDU) - During CR implementation, the rapp identified there is no clear conclusion how to set the “more data indication” in case of no data available, i.e., zero SDU. - Status in running CR: captured as value FFS. |
| --- | --- |

According to the input from the companies, 13 companies thinks that MDI should only be used for segmentation purposes, 7 companies thinks that MDI should be set to 1 to indicate NAS response is not available yet, 2 companies think it does not matter much which value is used as anyway the reader can learn the situation from 0 SDU already, and 1 company wants to wait for CT1. Based on this, the rapporteur prefers to define MDI setting in a self-consistent way:

The rapp agrees with the above analyze, but prefer to specify the value setting in a self-consistent way in the spec, if possible.

As far as our understanding, the solutions should be able to indicate the NAS response unavailability to the reader so that the reader can schedule the D2R transmission(s) for the NAS response later on. In order to identify the preferred solutions, the following 3 options for indicating the NAS response unavailability are discussed in comparison:

- Option a: (MAC based) Set MDI to 1 and SDU length to 0 in D2R MAC PDU.

- Option b: (MAC based) Set MDI to 0 and SDU length to 0 in D2R MAC PDU.

- Option c: (NAS based) defines a specific NAS message to inform AMF/AIoTF that NAS response is unavailable yet.

The feasibility and the pros & cons of the above 3 options are provided in the following table:

|  | Feasibility evaluation | Pros & Cons |
| --- | --- | --- |
| Option a: (MAC based) Set MDI to 1 and SDU length to 0 in D2R MAC PDU. | Feasible Both MDI and SDU length are mandatory fields to be included in the D2R MAC PDU for upper layer data transmission. Upon the reception of the MAC PDU comprising MDI = 1 and SDU Length = 0, the reader can determine that the NAS response is delayed and may schedule D2R transmissions for the NAS response later on. Here“MDI = 1” indicates to reader that there will be NAS response later on and NAS response to the received cmd is delayed. | Pros: Low complexity; Zero additional overhead. |
| Option b: (MAC based) Set MDI to 0 and SDU length to 0 in D2R MAC PDU. | Feasible. Both MDI and SDU length are mandatory fields to be included in the D2R MAC PDU for upper layer data transmission. Upon the reception of the D2R MAC PDU, the reader should: - ignore “MDI = 0”, which is originally defined to indicate the upper layer data transmission in response to a cmd is completed, and - determine that the NAS response is delayed based on SDU length = 0 and schedule D2R transmissions for the NAS response later on. However, the original intention of “MDI = 0” is used to indicate the upper layer data transmission in response to a cmd is completed but here the upper layer data transmission in response to the cmd is not started actually. | Pros: Low complexity; Zero additional overhead; Cons: MDI setting is not self-consistent with the original intention of “MDI = 0”. |
| Option c: (NAS based) defines a specific NAS message to inform AMF/AIoTF that NAS response is unavailable yet. | Infeasible As the NAS message in D2R MAC PDU is transparent to MAC, upon reception of a D2R MAC PDU comprising a specific NAS message to indicate the NAS response unavailability to AMF/AIoTF, the reader may misunderstand the NAS message as the NAS response and determine the related cmd procedure is finished. If this is the case, the reader will not schedule D2R transmission for the NAS response later on. |  |

- With either of the following two MAC based options for indicating the NAS response unavailability, the reader can schedule D2R transmission(s) for the NAS response later on:

- Option a: (MAC based) Set MDI to 1 and SDU length to 0 in D2R MAC PDU.

- Option b: (MAC based) Set MDI to 0 and SDU length to 0 in D2R MAC PDU.

- For Option b，MDI value setting is not self-consistent, as MDI = 0 is defined to indicate the upper layer data transmission in response to a cmd is finished but in case NAS response unavailability the NAS response transmission is not started yet.

- (Issue 3-7) When NAS response message is unavailable at the corresponding D2R TX occasion, the AIoT device should transmit the D2R MAC PDU comprising the following contents (i.e., Option a):

- MDI = 1;

- SDU Length = 0;

- Padding bits.

# Conclusion

In this contribution, remaining issues on AIoT data transmission aspects are discussed, and we have the following observations:

- Once the A-IoT session Release is completed at the CN side, the related AS context (e.g., the stored AS ID) maintained at the device side and the reader side are no longer needed and shall be released.

- With either of the following two MAC based options for indicating the NAS response unavailability, the reader can schedule D2R transmission(s) for the NAS response later on:

- Option a: (MAC based) Set MDI to 1 and SDU length to 0 in D2R MAC PDU.

- Option b: (MAC based) Set MDI to 0 and SDU length to 0 in D2R MAC PDU.

- For Option b，MDI value setting is not self-consistent, as MDI = 0 is defined to indicate the upper layer data transmission in response to a cmd is finished but in case NAS response unavailability the NAS response transmission is not started yet.

Based the above discussions and observations, we have the following proposals:

- (Issue 3-3) In line with the CN triggered A-IoT Session Release procedure, RAN2 to introduce explicit release procedure to timely release the AS context (e.g., the stored AS ID) maintained at device side.

- (Issue 3-3) Introduce a new R2D Release message which contains a Message Type, and a Transaction ID that associates to the A-IoT Session Release at the CN side (see Figure 2 as an example).

Figure 2. An Example of New R2D Release Message

- (Issue 3-3) Upon reception of the new R2D Release message, the device releases the stored AS ID if any, if the value of the Transaction ID included in the received R2D Release message is matched with the stored Transaction ID.

- (Issue 3-7) When NAS response message is unavailable at the corresponding D2R TX occasion, the AIoT device should transmit the D2R MAC PDU comprising the following contents (i.e., Option a):

- MDI = 1;

- SDU Length = 0;

- Padding bits.

- Reference

- R2-2505521, Summary of A-IoT MAC open issues.

- RAN3#128 Chairman Notes.

- R3-253937, (TP to TS 38.413 BL CR) New Release procedures, Ericsson, Huawei, Xiaomi.