3GPP TSG-RAN WG2 Meeting #131                                                     R2-2505211

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	MRO Enhancements for LTM

Agenda Item:	8.10.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#130 meeting, RAN2 discussed MRO enhancements for LTM related issues and some agreements were achieved [1].

In this contribution, we continue to discuss the potential MRO enhancements for LTM.

2. Discussion

2.1 MRO for SCG LTM

In RAN2#125bis meeting, RAN2 achieved the following agreements about MRO for LTM [2].

RAN2 considers SHR, RA report and RLF for MCG LTM SON. RAN2 will start work on MCG LTM.

Based on the agreements above, RAN2 mainly focused on MRO for MCG LTM in the previous RAN2 meetings. Considering the limited time of Rel-19 and there are still some leftover issues for MCG LTM, we suggest MRO enhancements for SCG LTM is not considered in Rel-19 SON/MDT.

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

### 2.2 Enhance SHR for LTM MRO

In case of successful LTM execution but it’s still close to failure, whether legacy SHR trigger conditions (i.e., T310 threshold, T312 threshold, T304 threshold) can be applied to LTM should be discussed. From our perspective, the T310 threshold and T304 threshold trigger conditions can be reused for LTM, but for T312 threshold trigger condition, considering that T312 is configured in MeasObjectNR, i.e., L3 RRM framework, it is not sure whether it is applicable for DU triggered LTM.

Proposal 2: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

SHR should be enhanced to record the successful LTM information. Similar as L3 measurement of serving cell, target cell and other LTM candidate cell(s) are included in RLF report upon RLF or mobility failure, L3 measurement of serving cell, target cell and other LTM candidate cell(s) should also be recorded in SHR for successful LTM execution.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

### 2.3 Enhance RACH report for LTM

For the TA value in LTM cell switch command (network-based TA acquisition), the early RACH procedure is needed. The source cell needs to send the PDCCH order command to UE, then the UE sends the preamble towards the indicated candidate cell. After the candidate cell acquires the TA value when receives the preamble from UE, it sends the TA value to the source cell and source cell includes the TA value in the LTM cell switch command to UE. The RACH procedure can be triggered multiple times towards same or different candidate cells before LTM execution. A new raPurpose can be introduced to indicate the network-based early TA acquisition.

Proposal 4: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Based on current spec, the UE may include up to 8 RACH report in RACH report list. If the RACH report is enhanced to support logging the RACH information of network-based early TA acquisition, the RACH report entry will reach max number (i.e., maxRAReport) soon as the frequent network-based early TA acquisition, this will affect the legacy RACH information logging, e.g., RACH information of handover as the UE will stop the RACH report logging if the entry number reaches max number, which will cause the legacy RACH information loss and this is not expected as the legacy RACH information is important for RACH optimization.

#### 5.7.10.4 Actions for the Random Access report determination Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, or upon failed or successfully completed RA-SDT operation as specified in clause 5.3.13.5, the UE shall: - 1> if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report; or - 1> if the registered SNPN identity or the SNPN identity selected by upper layers (see TS24.501 [23]) from the list of SNPN(s) included in the NPN-IdentityInfoList in SIB1is not included in snpn-IdentityList stored in a non-empty VarRA-Report: 2> clear the information included in VarRA-Report; - 1> if the UE is not in SNPN access mode and if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport: 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is less than maxPLMN; or 2> if the number of PLMN entries in plmn-IdentityList in identityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report: 3> append the following contents associated to the random-access procedure as a new entry in the VarRA-Report: <omit part>

RAN2 needs to discuss how to log the RACH information of network-based early TA acquisition and avoid the impact on the legacy RACH information logging. Some solutions can be considered to address the issue, e.g., one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before one LTM execution or one entry in RACH report can be used to log the RACH information of all network-based early TA acquisition before LTM configuration release or log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Proposal 5: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

### 2.4 Other issues

The LTM configuration will not be released after successful LTM execution, which means subsequent LTM can continue to be performed. However, LTM execution is based on the L1 measurement results which vary very rapidly and may cause the frequent LTM execution. Considering that the legacy RLF report/SHR only includes the latest failure/successful handover information, for LTM, there is high probability that the old RLF report/SHR will be overrode by new RLF report/SHR due to the frequent LTM execution, this will cause the information loss for network optimization. In order to address the issue, recording multiple RLF report/SHR can be considered as a candidate solution.

Proposal 6: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

In Rel-18 mobility WID, L3 measurement based LTM is supported, i.e., the CU can request the DU to send MAC CE to trigger LTM execution based on UE layer 3 measurements. If L3 measurement based LTM execution failure/near failure, it should be the source CU to analyze and optimize the failure/near failure instead of source DU. However, considering that the source CU cannot know whether it is L3 measurement based LTM or L1 measurement based LTM execution when the source CU receives the RLF report/SHR, the source CU can send the RLF report/SHR to source DU by F1 interface to optimize the failure/near failure, but, the handover decision mechanism of the source DU may not be problematic. In order to address the issue, the indication information can be needed to be included in RLF report/SHR to indicate L1 measurement based LTM or L3 measurement based LTM to avoid the error optimization occurred.

Proposal 7: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

3. Conclusion

According to the discussion in section 2, we propose:

MRO for SCG LTM

Proposal 1: MRO enhancement for SCG LTM is not considered in Rel-19 SON/MDT.

Enhance SHR for LTM MRO

Proposal 2: The legacy SHR trigger conditions based on T310 threshold and T304 threshold can be applied to LTM.

Proposal 3: Include L3 measurement results of serving cell, target cell and other LTM candidate cell(s) in SHR for LTM MRO.

Enhance RACH report for LTM

Proposal 4: A new raPurpose in RACH report is introduced to indicate the network-based early TA acquisition.

Proposal 5: RAN2 to discuss how to log the RACH information of network-based early TA acquisition to avoid the impact on the legacy RACH information logging (e.g., RACH information for handover):

- Option 1: one entry in RACH report to log the RACH information of all network-based early TA acquisition before one LTM execution;

- Option 2: one entry in RACH report to log the RACH information of all network-based early TA acquisition before LTM configuration release;

- Option 3: log the RACH information of network-based early TA acquisition per candidate cell in one or multiple entries.

Other issues

Proposal 6: To address the issue of frequent overriding RLF report/SHR for subsequent LTM, recording multiple reports can be considered as a candidate solution.

Proposal 7: Indication information is needed in RLF report/SHR to indicate whether it’s L1 measurement based LTM or L3 measurement based LTM.

4. Reference

- R2-2505002  RAN2#130 Meeting Report

- R2-2404102 RAN2#125bis Meeting Report