3GPP TSG-RAN WG2 Meeting #131                                                                    R2-2505217

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT, Turkcell

Title:	Functionality management for RRM measurement event prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meeting, some agreements were made for spec impact analysis for measurement event prediction use case [1] [2]. In this contribution, we will further analyze the spec impact of measurement event prediction from the following aspects for UE-sided model:

- Applicability reporting;

- Inference configuration and reporting;

- Performance monitoring.

- Discussion

In this section, we discuss the spec impact for measurement event prediction for UE-sided model. Both indirect measurement event prediction and direct measurement event prediction are considered, and for indirect measurement event prediction, all the following 4 listed will be considered which have been agreed in previous meeting.

Agreements 1 Agree to listed 4 definitions of indirect measurement event prediction Indirect measurement event prediction for temporal domain case A: In indirect measurement event prediction, future measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for temporal domain case B: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case B at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for frequency domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in frequency domain at first. Afterwards, predicted and optional actual historical measurement result(s) of serving cell are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model. Indirect measurement event prediction for spatial domain: In indirect measurement event prediction, measurement result(s) is predicted by a RRM measurement prediction model in spatial domain at first. Afterwards, predicted measurement result(s) and actual measurement results are used to derive whether a measurement event at one time instance occurs, without further involvement of an AI/ML model

  - Applicability reporting

In RAN2#130 meeting, RAN2 discussed the applicability reporting related issues and achieved the following agreements.

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

For indirect measurement event prediction, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the applicability reporting of indirect measurement event prediction is same as RRM measurement prediction.

Proposal 1: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

For direct measurement event prediction, the applicability reporting can take the applicability reporting of AI/ML based beam management as baseline (as agreed). For applicability reporting for BM UE-sided model, the following steps were proposed.

Figure 1: signalling procedure on applicability reporting for BM UE-sided model

For step 3, RAN2 has agreed to consider Option A and Option B like scheme for AI mobility. Similar as BM use case, for direct measurement event prediction, for Option A, one or more of ReportConfigNR for full inference configuration can be configured in RRM framework, and for Option B, it can be further discussed whether otherConfig or RRM framework is used to configure one set or multiple sets of inference related parameters, which based on the discussion in AI PHY.

Proposal 2: For direct measurement event prediction, for applicability reporting, RAN2 agrees the following:

- For Option A, one or more of ReportConfigNR for full inference configuration can be configured in RRM framework;

- For Option B, one set or multiple sets of inference related parameters can be configured in otherConfig or RRM framework, which based on the discussion in AI PHY.

In BM use case, for option A, for applicable functionality activation, RAN2 agreed the following agreements.

Agreements - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). - Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: Semi-persistent reporting, activated by MAC CE/DCI Aperiodic CSI reporting, activated by DCI

For direct measurement event prediction, if Option A is configured in Step 3, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e., without need to wait RRCReconfiguration in step 5). If Optioin B is configured in Step 3, considering that there is no agreement in BM use case yet, it can be discussed when to active the applicable functionalities based on the progress in BM use case.

Proposal 3: For direct measurement event prediction, for applicable functionality activation, RAN2 agrees the following:

- if Option A is configured in Step 3, the UE autonomously activates the applicable functionalities upon reporting them via RRCReconfigurationComplete in step 4 (i.e., without the need to wait for RRCReconfiguration in step 5);

- if Option B is configured in Step 3,  it can be discussed when to active the applicable functionalities based on the progress in BM use case.

  - Inference configuration and reporting

As mentioned above, considering that the RRM measurement prediction model is used for indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference for indirect measurement event prediction should be same as RRM measurement prediction.

Proposal 4: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

For temporal domain case B, considering L3 filtering can be applied to the measurement results, the measurements satisfying A3 enter condition and triggering condition are based on the filtered results of both actual and predicted measurements. Apart from this, the measurement event can be triggered considering all the measurement results within TTT which also include both actual and predicted measurement results. Hence, it is complex to indicate the measurement result which is actual or predicted. Consequently, the legacy RRM measurement report is sufficient for measurement report for indirect prediction of temporal domain case B.

For frequency domain and spatial domain prediction, the measurement results are predicted based on another frequency or RS resource set. And the measurement event can be predicted totally based on the predicted measurement results. Hence, there is no need to differentiate whether the measurement result is actual or predicted. Therefore, the legacy RRM measurement reporting can be reused for indirect measurement event predication of frequency domain and spatial domain.

Proposal 5: For temporal domain case B, frequency domain, and spatial domain prediction, the legacy RRM measurement reporting can be reused for indirect measurement event predication.

In indirect measurement event prediction for intra-frequency temporal domain case A, measurement result(s) is predicted by a RRM measurement prediction model at first. Afterwards, predicted and optionally actual historical measurement result(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model. It is used for handover performance enhancement. The UE can report the predicted occurrence time for the indirect predicted measurement event.

Proposal 6: For temporal domain case A, the UE can report the predicted occurrence time for the indirect predicted measurement event.

In RAN2#130 meeting, the following agreement was agreed, which means direct or indirect measurement event prediction is unknown for network, it is up to UE implement to decide direct or indirect model.

Agreements - Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.

Therefore, we understand, for the full inference configuration and inference related parameters contents, it is the same for direct and indirect measurement event prediction, e.g., the following inference configuration contents as agreed in RAN2#130 meeting.

Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction)

Proposal 7: For direct measurement event prediction, the full inference configuration and the inference related parameters contents are the same as indirect measurement event prediction.

For direct measurement event prediction, in previous RAN2 meeting, we agreed the following agreement.

Agreements on inputs/outputs and KPIs - For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Similar as indirect measurement event prediction for temporal domain case A, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Proposal 8: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

  - Performance monitoring

In RAN2#130 meeting, the performance monitoring for UE-sided model was discussed and the following agreements were achieved.

Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

Based on the agreements above, it can be seen that for RRM measurement prediction, RSRP differences can be used as the performance metric for monitoring, it is also applicable for indirect measurement event prediction, and the F1 score can also be used as the performance metric for monitoring. For direct measurement event prediction, the F1 score can be used as the performance metric for monitoring.

Proposal 9: For measurement event prediction, for performance monitoring, RAN2 agrees the following:

- For indirect measurement event prediction, RSRP differences/F1 score can be used as the performance metric for monitoring;

- For direct measurement event prediction, F1 score can be used as the performance metric for monitoring.

- Conclusion

In this contribution, we provide our views on the specification impact for measurement event prediction, and the proposals are summarized as follows:

Applicability reporting

Proposal 1: For indirect measurement event prediction, the applicability reporting is the same as that for RRM measurement prediction.

Proposal 2: For direct measurement event prediction, for applicability reporting, RAN2 agrees the following:

- For Option A, one or more of ReportConfigNR for full inference configuration can be configured in RRM framework;

- For Option B, one set or multiple sets of inference related parameters can be configured in otherConfig or RRM framework, which based on the discussion in AI PHY.

Proposal 3: For direct measurement event prediction, for applicable functionality activation, RAN2 agrees the following:

- if Option A is configured in Step 3, the UE autonomously activates the applicable functionalities upon reporting them via RRCReconfigurationComplete in step 4 (i.e., without the need to wait for RRCReconfiguration in step 5);

- if Option B is configured in Step 3,  it can be discussed when to active the applicable functionalities based on the progress in BM use case.

Inference configuration and reporting

Proposal 4: For indirect measurement event prediction, the model inference configuration e.g., input and output configuration of model inference is the same as that for RRM measurement prediction.

Proposal 5: For temporal domain case B, frequency domain, and spatial domain prediction, the legacy RRM measurement reporting can be reused for indirect measurement event predication.

Proposal 6: For temporal domain case A, the UE can report the predicted occurrence time for the indirect predicted measurement event.

Proposal 7: For direct measurement event prediction, the full inference configuration and the inference related parameters contents are the same as indirect measurement event prediction.

Proposal 8: For direct measurement event prediction, enhanced RRM measurement reporting mechanism is needed for the inference result reporting, e.g. reporting the probability of event occurrence within a time window.

Performance monitoring

Proposal 9: For measurement event prediction, for performance monitoring, RAN2 agrees the following:

- For indirect measurement event prediction, RSRP differences/F1 score can be used as the performance metric for monitoring;

- For direct measurement event prediction, F1 score can be used as the performance metric for monitoring.

- References

- R2-2503302, RAN2#129bis  Meeting Report

- R2-2505002, RAN2#130 Meeting Report